imulation3GPP TSG-RAN WG4 Meeting #119				                                                      R4-2607305

Dalian, China, 18 ‒ 22, 2026

Agenda Item:	8.1.1

Source: 	Huawei, Hisilicon

Title: 	Simulation assumption for FR1 L3 frequency domain cell level prediction in non-collocated scenario

Document for:	Information

- Introduction

To study the AI-RRM use case that non-collocated inter-cell prediction, a comprehension simulation assumption is essential. In this contribution, we provide the simulation assumptions for FR1 L3 frequency domain cell level prediction in non-collocated scenario which are based on the NR. The simulation assumption will be updated during the study item state based on the agreements.

- Discussion

To decide the simulation assumption for measurement prediction in FR1, the assumption used in RAN4 Rel-20 AI mobility [1] and can be used as baseline.

2.1 System level simulation assumptions for FR1

Table 2.2-1 Simulation assumptions of inter-frequency measurement

| Parameters | Value for FR1 |
| --- | --- |
| Frequency Range | FR1@{4GHz,30KHz} as central frequency FR1@{2GHz, 30KHz} as another frequency |
| Deployment | 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Channel model | UMa With spatial consistency TR 38.901 Clause 7.6.3.1. Note: This channel can be updated based on the new agreement. |
| System BW | 20MHz |
| UE speed | 30,90km/h |
| UE distribution | 100% outdoor |
| BS Antenna Configuration | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Option 1: 1Tx beam is assumed for both 2GHz and 4GHz. Option 2: 1Tx beam is assumed for 2GHz and 4Tx beams are assumed for 4GHz. |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern, 8 dBi |
| UE Antenna Configuration | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) 1RX beam (i.e., no Rx beam sweeping is assumed) is assumed |
| UE Antenna radiation pattern | Omni-direction |
| BS Tx Power | 44dBm |
| Maximum UE Tx Power | 23dBm |
| BS receiver Noise Figure | 5dB |
| UE receiver Noise Figure | 9dB |
| Inter site distance | 500 |
| Inter site distance of serving cell and another frequency target cell | FFS |
| BS Antenna height | 25m |
| UE Antenna height | 1.5m |
| Spatial consistency | companies report one of the spatial consistency procedures: - Procedure A in TR38.901 - Procedure B in TR38.901 |
| Other channel related model | FFS |
| UE trajectory model | 3 options in 38.843 section 6.3.1, companies report which option is used. |
| UE trajectory boundary processing model | Companies report which of the following models they used: wrap-around model, circle-bouncing model, boundary-terminated model |
| Sampling period | FFS |

Table 2.2-2

| L3 filtering parameter | value |
| --- | --- |
| FR1 FilterCoefficient | 0 Note: equal to 0 is the starting point. |

Table 2.2-3

| Consolidation parameter | value |
| --- | --- |
| nrofSS-BlocksToAverage for FR1 | 1 |
| absThreshSS-BlocksConsolidation for FR1 | -110dbm |

Table 2.2-4 Other parameters

| Parameters | Value for FR1 |
| --- | --- |
| L1 filtering | 5 samples Option 1: sliding Option 2: non-sliding |
| Observation window and prediction length | Assuming sliding L1 filtering: Option 1: - The range for OW length: 40ms to 400ms - The PW length: 40ms Option 2: - OW 200ms and PW 40ms - OW 400ms and PW 40ms |

Table 2.2-5: 3-Sector BS antenna radiation pattern for above 6GHz (TR 38.802 Table A.2.1-6)

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 8dBi |

Table 2.2-6: UE antenna radiation pattern model 1 (TR 38.802 Table A.2.1-8)

| Parameter | Values |
| --- | --- |
| Antenna element radiation pattern in θ'' dim (dB) |  |
| Antenna element radiation pattern in φ'' dim (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5dBi |

Note: are in local coordinate system.

Table 2.2-7 UE trajectory model and boundary processing model

| Parameters | Values |
| --- | --- |
| UE trajectory model | Option 1: Linear trajectory model with random direction change. - UE moving trajectory: UE will move straight along the selected direction to the end of an time interval, where the length of the time interval is provided by using an exponential distribution with average interval length, e.g., 5s, with granularity of 100 ms. - UE moving direction change: At the end of the time interval, UE will change the moving direction with the angle difference A_diff from the beginning of the time interval, provided by using a uniform distribution within [-45°, 45°]. - UE moves straight within the time interval with the fixed speed. Option 2: Linear trajectory model with random and smooth direction change. - UE moving trajectory: UE will change the moving direction by multiple steps within an time internal, where the length of the time interval is provided by using an exponential distribution with average interval length, e.g., 5s, with granularity of 100 ms. - UE moving direction change: At the end of the time interval, UE will change the moving direction with the angle difference A_diff from the beginning of the time interval, provided by using a uniform distribution within [-45°, 45°]. - The time interval is further broken into N sub-intervals, e.g. 100ms per sub-interval, and at the end of each sub-interval, UE change the direction by the angle of A_diff/N. - UE moves straight within the time sub-interval with the fixed speed. Option 3: Random direction straight-line trajectories. - Initial UE location, moving direction and speed: UE is randomly dropped in a cell, and an initial moving direction is randomly selected, with a fixed speed. - The initial UE location should be randomly drop within the following blue area: where d1 is the minimum distance that UE should be away from the BS. - Each sector is a cell and that the cell association is geometry based. - During the simulation, inter-cell handover or switching should be disabled. |
| UE trajectory boundary processing model | Option 1: wrap-around model, Option 2: circle-bouncing model, Option 3: boundary-terminated model |
| Note 1: For training data generation: - For each UE moving trajectory: the total length of the UE trajectory can be set as T seconds if it is in time, or set as D meter if it is in distance. - The trajectory sampling interval granularity depends on UE speed. - UE can move straight along the entire trajectory, or - UE can move straight during the time interval, where the time interval is provided by using an exponential distribution with average interval length ΔT - UE may change the moving direction at the end of the time interval. UE will change the moving direction with the angle difference A_diff from the beginning of the time interval, provided by using a uniform distribution within [-45°, 45°] - If the UE trajectory hits the cell boundary (the red line), the trajectory should be terminated. - If the trajectory length (in time) is less than the length of observation window + prediction window, the trajectory should be discarded. - The length of observation window + prediction window is not fixed and companies can report their values. Note 2: For the wrap-around model (option 1), when the UE hit the simulation border (the wrap-around contour), it will wrap around and enter the simulation area from a different point on the wrap-around contour. For the bouncing-circle model (option 2), when the UE hit the simulation border (the bouncing-circle), it will bounce back with a random angle and hence only area within circle can be used. For option 3, UE trajectory is terminated when UE hits the simulation border. Then another UE will be dropped randomly. |  |

2.3 Link level simulation assumptions for baseband error

Table 2.3-1 General parameters for measurement prediction evaluations

| Parameter | Value for FR1 |
| --- | --- |
| Carrier frequency | 7GHz/4GHz/2GHz |
| Subcarrier spacing | 30kHz for 4GHz 30kHz for 2GHz |
| Channel bandwidth | 20MHz |
| Channel model | Fading channel - Baseline: - TDL-C, DS=100ns - Optional: - CDL-C, DS=100ns |
| BS antenna configurations | Same as SLS; |
| UE antenna configurations | Same as SLS; 2Rx is assumed |
| UE moving speed | 30, 90km/h |
| Reference signal | SSB |
| DRX | No |
| Number of samples in L1 filtering | 1 sample |

Table 2.3-2 Cell-specific parameters for FR1

| Parameter | Unit | Value for FR1 |
| --- | --- | --- |
|  |  | Cell 1 |
| RF channel | - | Channel 1 |
| PBCH and DMRS power offset with respect to NR-PSS and NR-SSS | dB | 0 |
| Data and control PSD relative to NR-PSS and NR-SSS | dB | 0 |
| SSB periodicity | ms | 40 |
| SSB bandwidth | RB | 20 |
| SSB SCS | kHz | 30 |
| RB Utilization | % | 100 |
| Data Modulation | - | QPSK |
| CP Length | - | Normal |

2.4 Simulation assumptions for RF error

The following assumptions can be used to generate RF errors.

  - For FR1, follows a Gaussian distribution with (μ=0,  σ2=2.5) and with the limits of ±2.5dB.

    - Note 1: Same RF error is assumed for all the Tx beams.

    - Note 2: For inter-frequency prediction, same RF error for both frequencies

2.5 Performance metrics

At least the following performance characteristics are to be provided. Other performance characteristics are not precluded.

KPI 1: Absolute accuracy for predicted L3-RSRP

- Max (abs (95%-tile L3-RSRP), abs (5%-tile L3-RSRP)) on the CDF curve of L3-RSRP difference between the predicted L3-RSRP and the ground truth of L3-RSRP for a certain cell

Note: the ‘ground truth’ underlined refers to:

- Case 1: the label data with baseband and RF error.

- Case 2: the label data without baseband and RF error.

- References

R4-2600945 Simulation assumption for measurement prediction for AI mobility