3GPP TSG-RAN WG4 Meeting #119	R4-2607508

Dalain, China, 18th ‒ 22nd May, 2026

Title: 	WF on [119][214] NR_AIML_air_Ph2

Agenda Item:	7.10.1

Source: 	Ericsson

Document for:	Approval

# Topic 1: Finalization of dataset assumptions

## TDL correlation matrix

Agreement:

- Based on the existing demodulation specification for cross polarized antennas

  - Medium correlation

  - For up to rank 2 for TDL

  - Use beam steering approach and randomized beam directions specified in TS 38.101-4 Annex B.2.3.2.3A

## Number of sub-bands

Agreement:

19 (i.e., 18 is replaced by 19) for the dataset, FFS for the model and requirement

## Number of sub-bands

Companies are encouraged to provide the “1st priority” dataset first and then follow up with the “second priority”.

1st priority:

- 30k SCS, Sub-band size 16, 18 sub-bands

  - PRB arrangement:

    - Sub-band 0: All PRB

    - Sub-band 1-16: All PRB

    - Sub-band 17: 1 PRB

- 15k SCS, Sub-band size 4, 19 sub-bands

  - PRB arrangement:

    - Sub-band 0: 2 PRBs used

    - Sub-band 1-17: All PRB

    - Sub-band 18: 2 PRBs used

2nd priority:

- 30k SCS, Sub-band size 8, 19 sub-bands

  - PRB arrangement:

    - Sub-band 0: 4 PRBs used

    - Sub-band 1-17: All PRB

    - Sub-band 18: 4 PRBs used

- 15k SCS, Sub-band size 8, 19 sub-bands

  - PRB arrangement:

    - Sub-band 0: 4 PRBs used

    - Sub-band 1-17: All PRB

    - Sub-band 18: 4 PRBs used

## Antenna virtualization

- Agreement:

  - 3x1 is adopted

## Dataset contents and filename convention

Agreement:

- Use the following dataset format for CSI compression: Number of samples, Real/Imaginary, Subbands, SCS, Sub-band size, Tx ports and MIMO layers.

  - For the link simulation dataset, the filename format should be “<CompanyID>_<MeetingName>_<SCS>_<subbandsize> _<ChannelModel>_<TxPorts>_<AntArray> _<SNR>_<Index>.<ext>”

  - For the system simulation dataset, the filename format should be “<CompanyID>_<MeetingName>_<SCS>_<subbandsize>_<ChannelModel>_<TxPorts>_<AntArray>_<Index>.<ext>”

The fields should be written as follows:

CompanyID: As provided by MCC

MeetingName: RAN4 meeting number (with bis for bis meetings)

SCS: Either SCS15 or SCS30

Subbandsize: Either SBSIZE4, SBSIZE8 or SBSIZE16

Channel model: Either TDLA, TDLB, TDLC, UMA, Umi, Rural, Sma

TX ports: Either 16TX, 32TX, 64TX, 128TX

AntArray (Antenna arrai dimension): Either AA812, AA422, AA822, AA442

Index: 0, 1, 2, 3 etc. if the file needs splitting

## Dataset size

Agreement:

- SLS: Per Scenario / TX port number / TX port configuration / SCS / company  - 25k Samples

- LLS: Per Scenario / TX port number / TX port configuration / SCS  / SNR / company –10k Samples

  - SNR points are revised to 5, 12.5dB only

## Training and testing split

Agreement:

Dataset should consist of training+validation, (90%) testing (10%).

# Topic 2: Model dimensioning

Agreement:

Encoder:

- D_model for the encoder is 64

- N_TF for the encoder

  - 4 is a baseline

  - FFS whether 3 could replace 4 if performance is evaluated with larger payload sizes etc.

  - FFS whether 2 could replace 4 if performance is evaluated with larger payload sizes etc.

- N_Head and d_head – 8

- D_FF for the encoder 256

Decoder

- Baseline: N_TF 4 and d_model 64

- Check whether increasing the d_model can yield a substantial performance increase considering: N_TF 4 and d_model 128

- Check whether increasing the N_TF can yield a substantial performance increase considering: N_TF 6 and d_model 64

- Complexity should be taken into account. TE vendors and NW vendors should provide a view on an acceptable number.

- N_Head is 8 or 16 (depends on d_model) d_head – 8

- D_FF for the decoder 256

Performance should compare encoder and decoder pairs based on these combinations

- Companies should check performance with the encoder baseline and decoder baseline as a minimum

- Companies recommended to use a dataset of at least 500k samples for checking hyperparameters

Agreement:

For the CSI compression encoder model alignment, companies provide the following information in RAN4#120:

- How to implement the Embedding block with zero-padding (Example R4-2606612 5.4.1).

- How to implement the Positional Embedding block (Example R4-2606612 5.4.2).

- How to calculate the total number of parameters

  - Parameters correspond to the trained parameters.

  - Total number of parameters should include input embedding layer and CSI generation blocks

- How to calculate the total number of FLOPs

To further facilitate discussion at RAN4#120, in addition to the above, companies are encouraged to present their complexity analysis based on the following spreadsheet (in addition to their own calculation if relevant):

## Results spreadsheet

Create spreadsheets for both reporting complexity and SGCS, for both the model dimension and the scalability studies. Companies should submit both a word document and their results in Excel format in the same .zip file when contributing.

# Topic 3: Scalability

## Relation between 16/32 and 64/128 ports

There was no agreement. The following options were discussed: (Other options not precluded)

Option 1: Separate model (vivo, QC, ZTE, Samsung, OPPO, Apple, E///, Nokia, MTK)

Option 2: adaptation layer (based on the same structure and parameters for 16/32 ports) (CATT)

Option 3: further discuss and decide in the next meeting (HW, CATT, CMCC)

## Scalability evaluation basis

There was no agreement. The following option was discussed:

- Option 1:

  - Discuss whether to agree that for scalability, scalability is confirmed if SGCS loss compared to individual models is <[5] %.

  - If SGCS loss is more than [5]%, there is a need to discuss whether to have different models for different scalability dimensions.

  - For the SGCS loss:

    - Option 1a: the SGCS loss is the average SGCS loss over the related configuration cases

    - Other option is not precluded.

- Other option is not precluded.

## Sub-band scaling

Agreement:

- It is assumed that training the final model needs to take place with at least low/minimal, medium, high/maximal number (for example [6, 13, 18], or [3, 13, 19]) of subbands to achieve scalability.

## CSI payload scaling

The following was agreed in the discussion. There was no agreement on whether to use zero padding or embedded layer.

The complexity calculation needs to be aligned.

The complexity of adding a linear adaptation layer is acceptable for N_TF < 4. For N_TF=4, might exceed the UE complexity limit. TBC complexity after the complexity calculation is clarified.

The performance difference between adaptation layer and truncation is not significant for small payload sizes

The SGCS performance difference between adaptation layer and truncation is in the range 3 – 10% for large payload sizes (depending on company results)

## Hyperparameter assumptions used for scalability evaluation

Agreement:

At RAN4#118bis, it was agreed that the following hyperparameter assumptions should be used for scalability evaluations:

  - Encoder: N_TF = 4, d_model = 64, d_FF = 256, N_head X d_head = 8 X 8

  - Decoder: N_TF = 6, Addd_model = 64, d_FF = 256, N_head X d_head = 8 X 8

IT was agreed to revise the assumptions to N_TF=4 for the decoder side:

  - Encoder: N_TF = 4, d_model = 64, d_FF = 256, N_head X d_head = 8 X 8

  - Decoder: N_TF = 4, Addd_model = 64, d_FF = 256, N_head X d_head = 8 X 8

# Topic 4: Other model design issues

## Payload quantization

Agreement:

  - Agree to support [1/8, 3/8, 5/8, 7/8] for Q=2

## Model parameter quantization

The method for model parameter quantization was discussed. During the discussion, a question was raised whether there is a need to consider quantization at all. Later on, some companies expressed the view that, since models will always need quantizing, quantization aware training should be used, which implies a quantized model.

The following was captured from the discussion (not agreements)

- Check whether quantized values are needed for TE to ensure reproducible testing across different vendors

  - If TE vendors do not need a quantized model and always get similar performance regardless of quantization then model parameters quantization discussion can be dropped.

  - In case quantization is needed, Capture 3 options on how to do quantization for further discussion; Vivo proposal, Ericsson proposal and Samsung proposal to use Pytorch.

    - Ericsson proposal:

Qn=S⋅Pmax+⋅10-D⋅n-1126n=1,2,…,1270n=0-S⋅Pmin-⋅10-D⋅n-1126n=-127,-126,…,-1

    - Vivo proposal: Quantized value = round (Float value / Scale factor) + Zero point. Different layers may have different Scale factor and Zero point.

    - Samsung proposal: Use Pytorch

- BS and UE can use the floating point model and do their own quantization in implementation.

- FFS whether the quantized encoder needs to be captured in 3GPP

- FFS whether float 16 or float 32 is stored in the spec

- Companies encouraged to show results with and without quantization to check how much difference it makes.

  - Quantization aware training should be used when quantized models are presented

# Summary of issues to consider for RAN4#120

The following is not intended as an agreement, but is a summary of topics for which contributions are invited for RAN4#120 in order to make progress:

- Dataset generation

    - Companies are encouraged to generate and upload the dataset according to the agreements

  - Dataset comparison

    - Depending on availability of datasets, companies are encouraged to begin the process of comparing datasets using models trained with their own dataset, as outlined in previous agreements.

- Number of TX ports:

    - For 64/128 ports, whether to create a different model or use an adaptation layer

  - Scalability procedure

    - How to achieve CSI payload scaling (whether zero padding or adaptation layer)

    - Scalability evaluation basis; whether <5% loss etc.

    - Analysis of scalability performance

      - Companies are encouraged to provide results using the provided spreadsheet according to the agreements on assumptions

  - Model structure and parameters

    - Model complexity

      - Companies are encouraged to provide complexity estimates according to the complexity calculation spreadsheet and also with justification of alternative calculations if relevant.

      - Complexity calculations should be provided in the results spreadsheet.

    - Analysis and proposals for the encoder hyperparameters

      - Companies are encouraged to provide results using the provided spreadsheet according to the agreements on assumptions

    - Analysis and proposals for the decoder hyperparameters

      - Companies are encouraged to provide results using the provided spreadsheet according to the agreements on assumptions

  - Training

    - In general, it is useful for companies to provide details of the training they use

    - Proposals for alignment of training if needed.

  - Mosel parameters quantization

    - Analysis of the need for model parameter quantization and the performance difference between quantization and no quantization

  - Further consideration of how to capture the model and dataset in 3GPP

    - Companies are encouraged to consider ONNX and the framework outlined in Annex D for a 3GPP internal method.

  - Further details on the dataset alignment process

- Whether to consider robustness when selecting the final model parameters

- Considerations on model generalization

- Performance requirement design

- LCM requirements in case there is sufficient RAN1/2 progress

# Annex A: Link level simulation assumptions summary for dataset

- Dataset consists of Eigenvectors

- Common procedure to select Eigenvectors:

  - Select the right eigenvectors corresponding to the top-maximum- layer-number eigen values

  - Perform column normalization by normalizing the L2 norm of each eigenvector in to 1

  - Perform phase normalization by subtracting the angle of the first element from the angles of all the elements in each eigenvector

- 15k SCS and 30k SCS

1st priority:

- 30k SCS, Sub-band size 16, 18 sub-bands

  - PRB arrangement:

    - Sub-band 0: All PRB

    - Sub-band 1-16: All PRB

    - Sub-band 17: 1 PRB

- 15k SCS, Sub-band size 4, 19 sub-bands

  - PRB arrangement:

    - Sub-band 0: 2 PRBs used

    - Sub-band 1-17: All PRB

    - Sub-band 18: 2 PRBs used

2nd priority:

- 30k SCS, Sub-band size 8, 19 sub-bands

  - PRB arrangement:

    - Sub-band 0: 4 PRBs used

    - Sub-band 1-17: All PRB

    - Sub-band 18: 4 PRBs used

- 15k SCS, Sub-band size 8, 19 sub-bands

  - PRB arrangement:

    - Sub-band 0: 4 PRBs used

    - Sub-band 1-17: All PRB

    - Sub-band 18: 4 PRBs used

- Aim to deliver in order 16, 32, 64, 128 ports if datasets are delivered sequentially

- Array configuration:

  - 16 TX ports: [8, 1, 2], [4, 2, 2]

  - 32 TX ports: [8, 2, 2], [4, 4, 2]

  - Vertical sub-array size: 3 elements. Horizontal sub-array size: 1 element.

- TDL channels to be included:

| Combination name | Model | Maximum Doppler frequency |
| --- | --- | --- |
| TDLA30-10 | TDLA30 | 10 Hz |
| TDLB100-400 | TDLB100 | 400 Hz |
| TDLC300-100 | TDLC300 | 100 Hz |

- Companies should ensure TDL samples are uncorrelated

- Up to rank 2 only for TDL

- Antenna correlation medium (Alpha = 0.3, Beta = 0.6, Gamma = 0.2)

- SNR points: 5 and 12.5 dB SNR

- The channel to use for requirements should be discussed separately in the performance part.

# Annex B: System level simulation assumptions summary for dataset

- Dataset consists of Eigenvectors

- Common procedure to select Eigenvectors:

  - Select the right eigenvectors corresponding to the top-maximum- layer-number eigen values

  - Perform column normalization by normalizing the L2 norm of each eigenvector in to 1

  - Perform phase normalization by subtracting the angle of the first element from the angles of all the elements in each eigenvector

- 15k SCS and 30k SCS

1st priority:

- 30k SCS, Sub-band size 16, 18 sub-bands

  - PRB arrangement:

    - Sub-band 0: All PRB

    - Sub-band 1-16: All PRB

    - Sub-band 17: 1 PRB

- 15k SCS, Sub-band size 4, 19 sub-bands

  - PRB arrangement:

    - Sub-band 0: 2 PRBs used

    - Sub-band 1-17: All PRB

    - Sub-band 18: 2 PRBs used

2nd priority:

- 30k SCS, Sub-band size 8, 19 sub-bands

  - PRB arrangement:

    - Sub-band 0: 4 PRBs used

    - Sub-band 1-17: All PRB

    - Sub-band 18: 4 PRBs used

- 15k SCS, Sub-band size 8, 19 sub-bands

  - PRB arrangement:

    - Sub-band 0: 4 PRBs used

    - Sub-band 1-17: All PRB

    - Sub-band 18: 4 PRBs used

- Aim to deliver in order 16, 32, 64, 128 ports if datasets are delivered sequentially

- Array configuration:

  - 16 TX ports: [8, 1, 2], [4, 2, 2]

  - 32 TX ports: [8, 2, 2], [4, 4, 2]

  - Vertical sub-array size: 3 elements. Horizontal sub-array size: 1 element.

- Use 38.901 and the following scenarios:

  - Urban macro

  - Urban micro

  - Rural

  - Suburban Macro

# Annex C: Assumptions and combinations for investigating scalability

TX ports scalability:

- 16 ports and 32 ports

Sub-band scalability:

- For SCS 15kHz,

  - 6 sub-bands, 4 PRB sub-band size;

  - 13 sub-bands, 4 PRB sub-band size.

- For SCS 30kHz,

  - 6 sub-bands, 8 PRB sub-band size;

  - 13 sub-bands, 8 PRB sub-band size;

  - 18 sub-bands, 16 PRB sub-band size.

Method for TX ports and sub-bands scalability:

- Zero padding is defined as the zeros being placed at the end of the array

- Train the model with 16 and 32 ports

- Train the model with at least low/minimum, medium and high/maximum number of sub-bands

CSI scalability:

| Rank1 | Rank2 | Rank3 | Rank4 |
| --- | --- | --- | --- |
| {32, 2} | {32, 2}, {32, 2} |  |  |
| {64, 2} | {64, 2}, {64, 2} | {32, 2}, {32, 2}, {64, 2} | {32, 2}, {32, 2}, {32, 2}, {32, 2} |
| {128, 2} | {128, 2}, {128, 2} | {64, 2}, {64, 2}, {128, 2} | {64, 2}, {64, 2}, {64, 2}, {64, 2} |
| {192, 2} | {192, 2}, {192, 2} | {96, 2}, {96, 2}, {192, 2} | {96, 2}, {96, 2}, {96, 2}, {96, 2} |

Use RAN1 structure for scalability

Layer common model

- For scalability evaluation only, use the following assumptions for hyperparameters in order to obtain comparable results between companies:

  - Encoder: N_TF = 4, d_model = 64, d_FF = 256, N_head X d_head = 8 X 8

  - Decoder: N_TF = 4, Addd_model = 64, d_FF = 256, N_head X d_head = 8 X 8

# Annex D: Potential proposal for capturing the model in the specification

This annex is copied from R4-2603858 and provides an example of capturing a model in the 3GPP specifications. Companies are encouraged to examine and consider feasibility and potential improvements to the description. (Of course, the description will need to be aligned to eventual agreements).

## Transformer block in Encoder

The input of Encoder Transformer block is the matrix with dimension (N_token, d_model), that is the sum of the Embedding layer outputs and Positional Embedding block. The Encoder Transformer block consists of Self-attention block and Feed-forward network. After we apply the multiple repetitions (N_TF) of Transformer processing, we apply ‘Latent generation’ block to generate latent information, according to the CSI payload size configuration.

| Type | Variables | Dimension | Notes |
| --- | --- | --- | --- |
| Inputs | E | R2×Ntoken×dmodel | Ntoken: Sub-band domain dmodel: Tx domain |
| Output | E | R2×Ntoken×dmodel |  |

for t=1, 2, …, NTF // Loop for the number of Transformer blocks for i=1,2 // For real and imaginary parts for j=1,2,…,Ntoken Ei,j,:=Norm(Ei,j,:,γ1(k), β1(k)) // 1st layer normalization end for end for E←E+Multi_Head_Attention( E,E,WQk,WKk,WVk,WOk) // Multi-head attention, followed by Dropout for i=1,2 // For real and imaginary parts for j=1,2,…,Ntoken Ei,j,:=Norm(Ei,j,:,γ2(k), β2(k)) // 2nd Layer normalization end for end E←E+FFN(E,Wffn1(k),bffn1(k),Wffn2(k),bffn2(k)) // Feedforward network, followed by Dropout end for

## Transformer block in Decoder

The input of Decoder Transformer block is the CSI payload from reported from the UE. Since the payload size depends on the configuration, we need to put ‘Input Linear layer’ to convert the CSI payload to Transformer input matrix with dimension (N_token, d_model). Decoder transformer block consists of Self-attention block, Cross-attention block, and Feed-forward block. After we apply the multiple repetitions (N_TF) of Transformer processing, we apply ‘Output Linear layer’ to generate the target CSI.

| Type | Variables | Dimension | Notes |
| --- | --- | --- | --- |
| Inputs | E | R2×Ntoken×dmodel | Ntoken: Sub-band domain dmodel: Tx domain |
| Inputs | Z | R2×Ntoken×dmodel | Ntoken: Sub-band domain dmodel: Tx domain |
| Output | E | R2×Ntoken×dmodel |  |

// Self-attention for t=1, 2, …, NTF for i=1,2 // For real and imaginary parts for j=1,2,…,Ntoken Ei,j,:=Norm(Ei,j,:,γ1(k), β1(k)) // 1st Layer normalization end for end for E←E+Multi_Head_Attention(E,E,WQk,WKk,WVk,WOk) // Multi-head attention, followed by dropout // Cross-attention for t=1, 2, …, NTF for i=1,2 // For real and imaginary parts for j=1,2,…,Ntoken Ei,j,:=Norm(Ei,j,:,γ1(k), β1(k)) // 1st Layer normalization end for end for E←E+Multi_Head_Attention( E,Z,WQk,WKk,WVk,WOk) // Multi-head attention, followed by dropout for k=1,2 // For real and imaginary parts for s=1,2,…,Ntoken Ek,s,:=Norm(Ek,s,:,γ2(k), β2(k)) // 2nd Layer normalization end end for E=E+FFN(E,Wffn1(k),bffn1(k),Wffn2(k),bffn2(k)) // Feedforward network, followed by dropout end for

## Muti-head attention and Feed-forward network

### Multi-head attention block

Figure 10:	Multi-head attention block

Algorithm: Y←Multi_Head_Attention(E,Z,WQh,WKh,WVh,WOh)

Inputs: E∈R2×Ntoken×dmodel Inputs: Z∈R2×Ntoken×dmodel Outputs: Y∈R2×Ntoken×dmodel Model parameters: WQh∈R2×dmodel×dhead Model parameters: WKh∈R2×dmodel×dhead Model parameters: WVh∈R2×dmodel×dhead Model parameters: WO(h)∈R2×dmodel×dmodel

Nhead=dhaad⋅dmodel for h=1,2,…,Nhead Q(h)=ZWQ(h)∈R2×Ntoken×dhead K(h)=ZWK(h)∈R2×Ntoken×dhead V(h)=EWK(h)∈R2×Ntoken×dhead S(h)=AttentionQ(h),K(h),V(h)∈R2×Ntoken×dhead end for Concatenate all the scores Z=S(1),S(2),…,S(Nhead)∈R2×Ntoken×dmodel Apply Linear transformation Y=ZWO

### Attention block

The Multi-head Attention block is further divided with multiple (single) Attention blocks where the number of Attentions blocks are d_head. The single Attention block first calculates three matrices, Query (Q), Key (K), and Value (V) from the projection matrices WQ, WK, and WV. In the case of Self-attention used in Encoder and Decoder, Q, K, and V are generated from the same input E. On the other hand, in the case of Cross-attention used in Decoder, Q and K are generated from the latent information. Please remember N_token is the number of subbands, and d_model is the TX ports. The outputs of the Attention blocks are concatenated and applied a linear transformation. Single Attention block is illustrated in Figure 11.

Figure 11	(Single) Attention block.

Algorithm: S←Attention(Q,K,V)

Inputs: Q∈R2×Ntoken×dhead Inputs: K∈R2×Ntoken×dhead Inputs: V∈R2×Ntoken×dhead Outputs: S∈R2×Ntoken×dhead

Y=QKT∈R2×Ntoken×Ntoken S=softmaxYNheadV∈R2×Ntoken×dhead

Algorithm: X←Softmax(X)

Inputs: X∈R2×Ntoken×dhead Outputs: X∈R2×Ntoken×dhead

for i=1,2 // For real and imaginary parts for j=1,2,…,Ntoken N[i,j]=j=1NtokenX[i,j,k] X[i,j,k]=expX[i,j,k]N[i,j] end for end for

### Feed-forward network

Outputs of the Multi-head Attention block are fed into the feed-forward block. Figure 12 illustrates the feed-forward block. As it is shown in the figure, the dimension of inputs/output vectors are same, (N_token, d_model), but the vector dimension is extended to d_FF inside the block.

Figure 12	Feed-forward block.

Algorithm: Y←FFN(X)

Inputs: X∈R2×Ntoken×dmodel Outputs: Y∈R2×Ntoken×dmodel Model parameters: Wffn1∈R2×dmodel×dFF Model parameters: bffn1∈R2×dFF×1 Model parameters: Wffn2∈R2×dFF×dmodel Model parameters: bffn2∈R2×dmodel×1

X=XWffn1+bffn1∈R2×Ntoken×dFF Y=X+ReLUXWffn2+bffn2

Algorithm: Y←ReLU(X)

Inputs: X∈R2×Ntoken×dFF Outputs: Y∈R2×Ntoken×dFF

Y=max⁡(0,X)

### Normalization block

Algorithm: E←NormE,γ,β

Inputs: E∈Rdmodel Outputs: E∈Rdmodel Model parameters: γ∈Rdmodel Model parameters: β∈Rdmodel

Take average and variance over dmodel dimension. m=i=1dmodelEi/dmodel v=i=1dmodelEi-m2/dmodel Perform normalization and scaling E=E-mv ⊙γ+β where ⊙ denotes element-wide multiplication (or Hadamard product).

## Other blocks

### Embedding block with zero-padding

This block generated inputs to encoder block E from the target CSI V.

| Type | Variables | Dimension | Notes |
| --- | --- | --- | --- |
| Inputs | V | R2×Nsb×PCSI-RS | Target CSI |
| Outputs | E | R2×Ntoken×dmodel | Ntoken: Sub-band domain dmodel: Tx domain |
| Model Parameters | We | R2×dmodel×dmodel | Token embedding matrix |

Target CSI for MIMO layer l is given as follows.

V(l)=vreal(l),vimag(l)∈R2×Nsb×PCSI-RS

where vreal(l) and vimag(l) is real and imaginary parts of target CSI, with RNsb×PCSI-RS.

vreal(l)=vreall1,1vreall1,2⋯vreall1,PCSI-RSvreall2,1vreall2,2⋯ vreall2,PCSI-RS⋮⋮⋱⋮vreallNsb,1vreallNsb, 2⋯vreallNsb, PCSI-RS

vimag(l)=vimagl1,1vimagl1,2⋯vimagl1,PCSI-RSvimagl2,1vimagl2,2⋯ vimagl2,PCSI-RS⋮⋮⋱⋮vimaglNsb,1vimaglNsb, 2⋯vimaglNsb, PCSI-RS

where vreal(l)sb,p and vreal(l)sb,p is real and imaginary parts of the target CSI for MIMO layer l, subband sb, Tx port. p.

Generate zero-padded matrix X∈R2×Ntoken×dmodel from the target CSI, that is,

X:, 1:Nsb, 1:PCSI-RS=V[:,:,:]

X:,Nsb+1:,PCSI-RS+1:=0

Apply the embedding block We to generate the inputs to Transformer block E, as follows:

E=XWe

### Positional encoding block

The positional embedding block adds the hard-coded position information corresponding token’s position.

| Type | Variables | Dimension | Notes |
| --- | --- | --- | --- |
| Inputs | E | R2×Ntoken×dmodel |  |
| Outputs | E | R2×Ntoken×dmodel |  |

for j=1,2,…,Ntoken for k=1,2,…,dmodel E1,j,k←E1,j,k+cosjNtoken2k/dmodel E2,j,k←E1,j,k+sinjNtoken2k/dmodel end for end for

### Quantized CSI feedback generation with truncation

| Inputs | E | R2×Ntoken×dmodel |  |
| --- | --- | --- | --- |
| Outputs | Z | R2×dzl,v | Quantized latent |
| Model parameters | WLL | R2×(Ntoken ⋅dmodel)×dmax | Weight matrix for Latent generation |
| Configuration parameter | dzl,v |  | CSI payload size for MIMO layer l in rank v |
| Configuration parameter | q |  | Number of quantized bits for latent message for each real/imaginary part. |

Flatten the matrix from R2×Ntoken×dmodel with Flatten procedure

Eflat=FlattenE

Note: Eflat a matrix with R2×Ntoken⋅dmodel after the Flatten procedure.

Generate latent message from a feed-forward network.

Y=EflatWLL

Note: Y∈R2×dmax is the latent message before truncation and quantization.

Truncate Y to the configured CSI payload size dzl,v, followed by quantizing Ytrunc with the configured number of quantization bits q.

Ytrunc:,:=Y[:,1:dzl,v]

Z=QuantizeYtrunc),q

Algorithm: Eflat←Flatten(E)

Inputs: E∈R2×Ntoken×dmodel Outputs: Eflat∈R2×Ntoken⋅Nmodel

for j=1,2,…,Ntoken for k=1,2,…,dmodel Eflat:,(j-1)⋅dmodel+k=E:,j,k end for end for

Algorithm: Yq←Qunatize(Y,q)

Inputs: Y∈R2×d Outputs: Yq∈R2×d Configuration parameter: q∈[1,2]

Generate Yq by quantizing the input matrix Y with q bit (q∈[1,2]).