3GPP TSG-RAN WG4 Meeting #115	R4-2507704

Malta, MT, 19-23 May 2025

Title:	LS on E-UTRA uplink configurations for intra-band ENDC MSD

Response to:

Release:	Rel-19

Work Item:	DC_R19_xBLTE_yBNR-Core

Source:	RAN4

To:	RAN5

Cc:

Contact Person:

#### Name:	Laurent Noel

E-mail Address:	laurent.noel@skyworksinc.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

RAN4 would like to share with RAN5 the agreements reached to address concerns that some E-UTRA uplink configurations specified as “N/A” for the intra-band contiguous EN-DC MSD requirements may be problematic for conformance testing. An example of such uplink configuration is illustrated in Table 1 for DC_(n)3AA.

Table 1: Example of E-UTRA “N/A” uplink configuration for intra-band contiguous EN-DC MSD - Table 7.3B.2.1-1.

| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_(n)3AA | 3 | N/A | 15 | N/A | 1842.5 | 6.2 | FDD |
|  | n3 | 1770.0 | 30 | 10 (RBstart = 150) | 1865.0 | N/A |  |

RAN4 has made the following agreements:

- E-UTRA uplink carrier frequency “Fc (UL)”: RAN4 recommends configuring the E-UTRA Fc (UL) according to the E-UTRA band’s default Tx-Rx frequency separation specified in TS 36.101 Table 5.7.4-1;

- E-UTRA uplink RB allocation “Lcrb”: RAN4 recommends configuring the E-UTRA UL RB allocation according to the UL configuration specified in TS 36.101 Table 7.3.1-2 of the corresponding E-UTRA DL CBW;

- E-UTRA transmission power: RAN4 recommends configuring the E-UTRA transmit power as anchor agnostic, or preferably at minimum output power.

2. Actions:

To RAN5

ACTION: 	RAN4 respectfully asks RAN5 to take the above into consideration.

3. Date of Next TSG-RAN4 Meetings:

3GPP TSG RAN WG4#116	25 - 29 August 2025		Bangalore, India

3GPP TSG RAN WG4#116-bis	13 - 17 October 2025		Prague, Czech Republic