3GPP TSG-RAN5 Meeting #108	R5-253755

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0672 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of 5GC IMS DC TC 6.11 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_IMS_dataCH-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TC 6.11 Re-Registration / IMS data channel is needed according to the WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new TC 6.11 Re-Registration / IMS data channel |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work Plan will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.11 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 34.229-2 CR 0393 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

6.10.3.3	Specific message contents

Table 6.10.3.3-1: REGISTER (step 2, table 6.10.3.2-1)

Derivation path: TS 34.229-1 [2], Table in subclause A.1.1, Condition A1

Table 6.10.3.3-3: REGISTER (step 4, table 6.10.3.2-1)

Derivation Path: TS 34.229-1 [2], Table in subclause A.1.1, Conditions A2, A17

Table 6.10.3.3-4: 200 OK for REGISTER (step 5, table 6.10.3.2-1)

Derivation Path: TS 34.229-1 [2], Table in subclause A.1.3, Condition A6

6.11	Re-Registration / IMS data channel

6.11.1	Test Purpose (TP)

(1)

with { UE supporting IMS data channel and being registered to IMS }

ensure that {

when { UE needs to perform re-registration procedure }

then { UE sends REGISTER message including the sip.app-subtype media feature tag with a value "webrtc-datachannel" }

}

(2)

with { UE having sent REGISTER request }

ensure that {

when { UE receiving a valid 200 OK response without a Feature-Caps header field with the "+g.3gpp.datachannel" header field parameter }

then { UE continues to proceed the register procedure and shall not determine that the Network supports the IMS data channel capability }

}

6.11.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-18 requirements.

[TS 24.186, clause 9.2.2.1]:

If the UE is allowed to use IMS data channel, on reception of Re-REGISTER request, for user-initiated reregistration, the UE supporting IMS data channel shall include the media feature tag defined in IETF RFC 5688 [5] for supported streaming media type. For the IMS data channel capability indication, the UE shall use +sip.app-subtype="webrtc-datachannel" as specified in 3GPP TS 26.114 [4].

NOTE:	The policy related to the IMS data channel allowed at the UE, can be provided by the network to the UE using e.g., OMA-DM with the management objects specified in 3GPP TS 24.275 [11] or UICC configuration, as specified in clause 9.2.1.1.

…

On receiving the 200 (OK) response to the REGISTER request, if the 200 (OK) response does not include a Feature-Caps header field containing feature-capability indicator "g.3gpp.datachannel", the UE shall keep established data channel media of the UE's existing IMS session.

6.11.3	Test description

6.11.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

-	SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE.

-	SS is listening to SIP default port 5060 for both UDP and TCP protocols.

-	SS is able to perform IMS AKA authentication for the IMPI, according to 3GPP TS 33.203 [16] clause 6.1.

-	UE provided with a second P-CSCF in the PDU SESSION ESTABLISHMENT ACCEPT according to Table 4.7.2-2 (conditions Additional_P-CSCF_IPv6 and Additional_P-CSCF_IPv4) of TS 38.508-1 [21].

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured to register for IMS data channel after switching on.

-	The UE is switched off.

Preamble:

-	None.

6.11.3.2	Test procedure sequence

Table 6.11.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is switched on. | - | - | - | - |
| 2 | UE sends a correctly composed initial registration request including the sip.app-subtype media feature tag with a value “webrtc-datachannel”. | --> | REGISTER | - | - |
| 3 | SS sends 401 Unauthorized. | <-- | 401 Unauthorized | - | - |
| 4 | UE sends a subsequent registration request including the sip.app-subtype media feature tag with a value “webrtc-datachannel”. | --> | REGISTER | - | - |
| 5 | SS sends a valid 200 OK response including a Feature-Caps header field for REGISTER. | <-- | 200 OK | - | - |
| - | EXCEPTION: In parallel to the events described in steps 6 to 9, the steps specified in Table 6.11.3.2-2 may take place. | - | - | - | - |
| 6 | UE subscribes to reg-event. | --> | SUBSCRIBE | - | - |
| 7 | SS sends 200 OK for SUBSCRIBE with 120 seconds expiration interval. | <-- | 200 OK | - | - |
| 8 | SS sends NOTIFY for reg-event package, containing full registration state information for the registered public user identity in the XML body. | <-- | NOTIFY | - | - |
| 9 | UE acknowledges reception of NOTIFY. | --> | 200 OK | - | - |
| 10 | Check: Does the UE send a correctly composed re-registration request including the sip.app-subtype media feature tag with a value “webrtc-datachannel” after 60 seconds? | --> | REGISTER | 1 | P |
| 11 | SS responds with 200 OK with 180 seconds expiration interval, but without Feature-Caps header field with the "+g.3gpp.datachannel" header field parameter. | <-- | 200 OK | - | - |
| 12 | Make the UE attempt an IMS voice call. | - | - | - | - |
| 13 | Check: Does UE send INVITE without IMS data channel in the SDP offer? | --> | INVITE | 2 | P |
| 14-24 | Steps 2-12 of Annex A.4.1a take place. | - | - | - | - |
| NOTE: The associated NR/5GC signalling is specified in TS 38.508-1 [21] generic procedure 4.5.2. |  |  |  |  |  |

Table 6.11.3.2-2: Parallel Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE sends a PUBLISH request. | --> | PUBLISH | - | - |
| 2 | SS sends a 503 Service Unavailable response | <-- | 503 Service Unavailable | - | - |

6.11.3.3	Specific message contents

Table 6.11.3.3-1: REGISTER (Step 2, Table 6.11.3.2-1)

Derivation path: TS 34.229-1 [2], Annex A.1.1, Conditions A1, A36

Table 6.11.3.3-2: REGISTER (Step 4, Table 6.11.3.2-1)

Derivation path: TS 34.229-1 [2], Annex A.1.1, Conditions A2, A17, A36

Table 6.11.3.3-3: 200 OK for REGISTER (Step 5, Table 6.11.3.2-1)

Derivation Path: TS 34.229-1 [2], Table in subclause A.1.3, Condition A6

Table 6.11.3.3-4: 200 OK for SUBSCRIBE (Step 7, Table 6.11.3.2-1)

| Derivation Path: TS 34.229-1 [2], Table in subclause A.1.5 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Contact |  |  |  |  |
| expires |  | 120 |  |  |

Table 6.11.3.3-5: REGISTER (Step 10, Table 6.11.3.2-1)

| Derivation path: TS 34.229-1 [2], Annex A.1.1, Conditions A2, A17, A36 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Security-Client |  |  |  |  |
| spi-c |  | new SPI number of the inbound SA at the protected client port, shall be different from previously used number |  |  |
| spi-s |  | new SPI number of the inbound SA at the protected server port, shall be different from previously used number |  |  |
| port-c |  | new protected client port, shall be different from previously used number |  |  |
| port-s |  | same value as in the previous REGISTER |  |  |
| Authorization |  |  |  | RFC 2617 [23] |
| nonce-count |  | 2 |  | TS 24.229 [7] |

Table 6.11.3.3-6: 200 OK for REGISTER (Step 11, Table 6.11.3.2-1)

| Derivation Path: TS 34.229-1 [2], Table in subclause A.1.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Contact |  |  |  |  |
| expires |  | 180 |  |  |

Table 6.11.3.3-7: INVITE (Step 13, Table 6.11.3.2-1)

Derivation Path: Step 1 of A.4.1a

[End of change]