3GPP TSG-RAN5 Meeting #108	R5-253757

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5035 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of ATSSS Phase 3 TC 11.9.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS_Ph3-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TC 11.9.5 Inter-system mobility with established MA PDU session/ATSSS/Dual-registration mode with N26/establishing a PDN connection as the user plane resource of an MA PDU session is supported/Handover from NR/5GC to E-UTRAN/EPC is needed according to the WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new TC 11.9.5 Inter-system mobility with established MA PDU session/ATSSS/Dual-registration mode with N26/establishing a PDN connection as the user plane resource of an MA PDU session is supported/Handover from NR/5GC to E-UTRAN/EPC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work Plan will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.9.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

Table 11.9.4.3.3-7: ATTACH REQUEST (step 7b3, table 11.9.4.3.2-1)

| Derivation Path: TS 36.508 [7], Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| IMSI | IMSI of the UE |  |  |
| ESM message container | PDN CONNECTIVITY REQUEST |  |  |

11.9.5	Inter-system mobility with established MA PDU session / ATSSS / Dual-registration mode with N26 / establishing a PDN connection as the user plane resource of an MA PDU session is supported / Handover from NR/5GC to E-UTRAN/EPC

11.9.5.1	Test Purpose (TP)

(1)

with { UE is registered over 3GPP access connected to 5GC and untrusted non-3GPP access connected to EPC and UE indicates its support of establishing a PDN connection as the user plane resource of an MA PDU session during the MA PDU session establishment procedure and MA PDU session is established }

ensure that {

when { UE detects a suitable EPC E-UTRA cell after the serving 5GC cell becomes not suitable }

then { UE performs a Inter-system change from N1 mode to S1 mode by initiating and successfully completing an ATTACH or a TAU procedure, mapped EPC context used, and performs a local release of the MA PDU session over untrusted non-3GPP access while MA PDU session with the user-plane resource with the PDN connection over 3GPP access network was kept }

}

11.9.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.193, clause 4.6. Unless otherwise stated these are Rel-18 requirements.

[TS 24.193, clause 4.6]

In the network supporting N26 interface:

…

c)	for an inter-system change from N1 mode to S1 mode:

…

3)	if the UE established an MA PDU session with the user-plane resource over 3GPP access and the user-plane resource with the PDN connection over untrusted non-3GPP access network, the UE follows the procedure as specified in clause 6.1.4.1 of 3GPP TS 24.501 [6], and

A)	if the MA PDU session is transferred to EPS as a PDN connection and:

…

-	the UE indicates its support of establishing a PDN connection as the user plane resource of an MA PDU session during the MA PDU session establishment procedure as specified in clause 6.4.1.2 of 3GPP TS 24.501 [6], the network can keep the MA PDU session with the user-plane resource with the PDN connection over 3GPP access network and locally release the user-plane resource over untrusted non-3GPP access. The UE performs a local release of the MA PDU session over untrusted non-3GPP access.

11.9.5.3	Test description

11.9.5.3.1	Pre-test conditions

System Simulator:

-	WLAN Cell 27 is configured according to 38.508-1 [4], clause 4.4.1.3.1.

-	NR Cell 1 is configured according to TS 38.508-1 [4].

-	E-UTRA Cell 1 is configured to TS 36.508 [7].

-	NR Cell 1, WLAN Cell 27 and E-UTRA Cell 1 belong to the same PLMN and both NR Cell 1 and WLAN Cell 27 are set to '' Serving cell'', E-UTRA Cell 1 is set to "non-suitable cell".

UE:

-	UE is registered over both 3GPP access to 5GC and non-3GPP access to EPC in the same PLMN.

Preamble:

-	UE is brought to state 3W-A and state 3N-A with UE test loop mode B active according to TS 38.508-1 [4].

11.9.5.3.2	Test procedure sequence

Table 11.9.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Cause the UE to request to establish an MA PDU session by MMI or AT command. | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in steps 2-3 below the events specified in Table 11.9.5.3.2-2 may take place. | - | - | - | - |
| 2 | The UE transmits a PDU SESSION ESTABLISHMENT REQUEST message and sets the request type to "MA PDU request" in the UL NAS TRANSPORT message, include the ATSSS request parameter in the Extended protocol configuration options IE and sets the ATSSS-ST bits in the 5GSM capability IE of the PDU SESSION ESTABLISHMENT REQUEST message according to the ATSSS functionality and steering mode that UE supports on NR Cell 1. Note: PDU SESSION ESTABLISHMENT REQUEST is included in UL NAS transport. UL NAS transport message is included in dedicatedNAS-Message of ULInformationTransfer message. | --> | 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST | - | - |
| 3 | The SS transmits a PDU SESSION ESTABLISHMENT ACCEPT on NR Cell 1. | <-- | 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT ACCEPT | - | - |
| 4 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| 5 | NR Cell 1 is set "Non-suitable cell”, E-UTRA Cell 1 is set to “Serving cell”. | - | - | - | - |
| 6 | The UE performs on E-UTRA Cell 1 the TAU procedure for Inter-system change from N1 mode to S1 mode in 5GMM/EMM-IDLE mode as described in TS 38.508-1 [4], Table 4.9.7.2.2-1, 'connected without release'. | - | - | - | - |
| 7 | The SS transmits one IP Packet on MA PDU session established on WLAN Cell 27. | <-- | IP packet | - | - |
| 8 | Check: Does the UE loop back the IP Packet? | - | - | 1 | F |
| 9 | The SS transmits one IP Packet on MA PDU session established on E-UTRA Cell 1. | <-- | IP packet | - | - |
| 10 | Check: Does the UE loop back the IP Packet? | - | - | 1 | P |

Table 11.9.5.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits a PDU SESSION ESTABLISHMENT REQUEST message and sets the request type to "MA PDU request" in the UL NAS TRANSPORT message, include the ATSSS request parameter in the Extended protocol configuration options IE and sets the ATSSS-ST bits in the 5GSM capability IE of the PDU SESSION ESTABLISHMENT REQUEST message according to the ATSSS functionality and steering mode that UE supports on WLAN Cell 27. Note: PDU SESSION ESTABLISHMENT REQUEST is included in UL NAS transport. UL NAS transport message is included in dedicatedNAS-Message of ULInformationTransfer message. | --> | 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST | - | - |
| 2 | The SS transmits an PDU SESSION ESTABLISHMENT ACCEPT on WLAN Cell 27. | <-- | 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT ACCEPT | - | - |

11.9.5.3.3	Specific message contents

Table 11.9.5.3.3-1: REGISTRATION ACCEPT (preamble)

| Derivation path: TS 38.508-1 [4] Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GS network feature support |  |  |  |
| ATS-IND | '1'B | ATSSS supported |  |

Table 11.9.5.3.3-2: PDU SESSION ESTABLISHMENT REQUEST (Step 2, Table 11.9.5.3.2-1 and Step 1, Table 11.9.5.3.2-2)

| Derivation path: TS 38.508-1 [4], Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDU session ID | Any value according to TS 24.501 [25] subclause 9.4 |  | MA PDU Session established on first access |
|  | The value used in the MA PDU Session establish procedure on first access |  | MA PDU Session established on second access |
| 5GSM capability |  |  |  |
| All octets with the exception of octet 3, bits 4 to 7 | Any allowed value |  |  |
| ATSSS-ST (octet 3, bits 4 to 7) | ‘0001’B’ or 0010’B or ‘0011’B | ‘0001’B represents UE supports ATSSS Low-Layer functionality with any steering mode, ’0010’ represents UE supports MPTCP functionality with any steering mode and ATSSS-LL functionality with only active-standby steering mode, ‘0011’ represents UE supports MPTCP functionality with any steering mode and ATSSS-LL functionality with any steering mode |  |
| Extended protocol configuration options |  |  |  |
| Container ID n | ‘0030’H | ATSSS request | UE indicates supporting of establishing a PDN connection as the user plane resource of an MA PDU session during the MA PDU session establishment procedure |
| Length of container ID n contents |  | Length value determined by test implementation |  |
| Container ID n contents | The bit 8 to 5 is set to any allowed values except “0000”, and the bit 4 to 1 is set to the same value of ATSSS-ST in 5GSM capability. |  |  |

Table 11.9.5.3.3-3: UL NAS Transport (Step 2, Table 11.9.5.3.2-1 and Step 1, Table 11.9.5.3.2-2)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Request type | ‘011’B | MA PDU request |  |
| S-NSSAI | Not checked |  |  |

Table 11.9.5.3.3-4: PDU SESSION ESTABLISHMENT ACCEPT (Step 3, Table 11.9.5.3.2-1 and Step 2, Table 11.9.5.3.2-2)

| Derivation path: TS 38.508-1 [4], Table 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ATSSS container | Any allowed value according to TS 24.501 [25] subclause 9.11.4.22 |  |  |

Table 11.9.5.3.3-5: TRACKING AREA UPDATE REQUEST (Step 6, Table 11.9.5.3.2-1)

| Derivation Path: TS 36.508 [2], Table 4.7.2-27, condition NR. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS bearer context status | Present, content not checked | EBI corresponding to active PDN connections (transferred MA PDU Sessions) need to be set to 1 |  |

[End of change]