3GPP TSG-RAN5 Meeting #108	R5-253834

# Bengaluru, India, 25th – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 1177 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of EVM equalizer spectrum flatness for CA (3UL & 4UL CA) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sporton International INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_RF_FR2_req_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Minimum requirement of EVM equalizer spectrum flatness for CA (3UL & 4UL CA) is defined in TS 38.101-2, so test case in RAN5 TS 38.521-2 should be made. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of EVM equalizer spectrum flatness for CA (3UL & 4UL CA) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test Cae 6.4A.2.4.2 & 6.4A.2.4.3 will be still incompleting. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4A.2.4.2, 6.4A.2.4.3 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

6.4A.2.4.2	EVM equalizer spectrum flatness for CA (3UL CA)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainty and Test Tolerance are FFS.

6.4A.2.4.2.1	Test Purpose

The zero-forcing equalizer correction applied in the EVM measurement process (as described in Annex E) must meet a spectral flatness requirement for the EVM measurement to be valid. The EVM equalizer spectrum flatness is defined in terms of the maximum peak-to-peak ripple of the equalizer coefficients (dB) across the allocated uplink block, at which the equalizer coefficients are generated by the EVM measurement process. The basic measurement interval is the same as for EVM.

The EVM equalizer spectrum flatness requirement does not limit the correction applied to the signal in the EVM measurement process but for the EVM result to be valid, the equalizer correction that was applied must meet the EVM equalizer spectrum flatness minimum requirements.

6.4A.2.4.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 3UL CA.

6.4A.2.4.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.4A.2.4.0

6.4A.2.4.2.4	Test description

Same as in clause 6.4A.2.4.1.4 with following exceptions:

-	Instead of Table 6.4A.2.4.1.4.1-1  use Table 6.4A.2.4.2.4.1-1.

Table 6.4A.2.4.2.4.1-1: Test Configuration

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  |  | Low and High range |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  | Lowest aggregated BW of the CA configuration Highest aggregated BW of the CA configuration |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |
| CA Configuration / Aggregated BW |  |  | Downlink Configuration | Uplink Configuration |  |
| Test ID | CC & Mapping (NOTE 3) | CBW (MHz) | RB allocation | Modulation | RB allocation (NOTE 1) |
| 1 | PCC/CC1 | default | - | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC/CC2 |  |  | - | - |
|  | SCC/CC3 |  |  | - | - |
| 2 | PCC/CC1 |  |  | CP-OFDM QPSK | Outer_Full |
|  | SCC/CC2 |  |  | - | - |
|  | SCC/CC3 |  |  | - | - |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: CA Configuration Test cumulative aggregated BW settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 3: PCC/CCi and SCC/CCj means PCC is on component carrier CCi and SCC is on component carrier CCj, with CCi or CCj frequencies defined in TS38.508-1 [10]. NOTE 4: Number of DL CCs shall be configured the same as number of UL CCs. The requirements are appliable as per 5.3A.4: "The requirements are applicable only when Uplink CCs are configured within the frequency range between lower edge of lowest downlink component carrier and upper edge of highest downlink component carrier". |  |  |  |  |  |

6.4A.2.4.2.5	Test requirement

Each of the n spectrum flatness functions, shall derive four ripple results in Annex E.4.4. The derived results shall not exceed the values in Figure 6.4A.2.4.2.5-1: The peak-to-peak variation of the EVM equalizer coefficients contained within the frequency range of the uplink allocation shall not exceed the maximum ripple specified in Table 6.4A.2.4.2.5-1 for normal conditions. For uplink allocations contained within both Range 1 and Range 2, the coefficients evaluated within each of these frequency ranges shall meet the corresponding ripple requirement and the following additional requirements: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 (Table 6.4A.2.4.2.5-1) must not be larger than 7 dB + TT, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 8 dB + TT (see Figure 6.4A.2.4.2.5-1).

The UE passes the test when the derived results for at least one polarization fulfil the test requirements.

Table 6.4A.2.4.2.5-1: Test requirements for EVM equalizer spectrum flatness (normal conditions)

| Frequency range | Maximum ripple (dB) |
| --- | --- |
| ¦FUL_Meas – Fcenter¦ ≤ X MHz (Range 1) | 6 +TT (p-p) |
| ¦FUL_Meas – Fcenter¦ > X MHz (Range 2) | 9 + TT (p-p) |
| NOTE 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated NOTE 2: Fcenter refers to the centre frequency of the CC NOTE 3: X, in MHz, is equal to 30% of the CC bandwidth |  |

Figure 6.4A.2.4.2.5-1: The limits for EVM equalizer spectral flatness with the maximum allowed variation of the coefficients indicated under normal conditions

6.4A.2.4.3	EVM equalizer spectrum flatness for CA (4UL CA)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainty and Test Tolerance are FFS.

6.4A.2.4.3.1	Test Purpose

The zero-forcing equalizer correction applied in the EVM measurement process (as described in Annex E) must meet a spectral flatness requirement for the EVM measurement to be valid. The EVM equalizer spectrum flatness is defined in terms of the maximum peak-to-peak ripple of the equalizer coefficients (dB) across the allocated uplink block, at which the equalizer coefficients are generated by the EVM measurement process. The basic measurement interval is the same as for EVM.

The EVM equalizer spectrum flatness requirement does not limit the correction applied to the signal in the EVM measurement process but for the EVM result to be valid, the equalizer correction that was applied must meet the EVM equalizer spectrum flatness minimum requirements.

6.4A.2.4.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 4UL CA.

6.4A.2.4.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.4A.2.4.0

6.4A.2.4.3.4	Test description

Same as in clause 6.4A.2.4.1.4 with following exceptions:

-	Instead of Table 6.4A.2.4.1.4.1-1  use Table 6.4A.2.4.3.4.1-1.

Table 6.4A.2.4.3.4.1-1: Test Configuration

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  |  | Low and High range |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  | Lowest aggregated BW of the CA configuration Highest aggregated BW of the CA configuration |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |
| CA Configuration / Aggregated BW |  |  | Downlink Configuration | Uplink Configuration |  |
| Test ID | CC & Mapping (NOTE 3) | CBW (MHz) | RB allocation | Modulation | RB allocation (NOTE 1) |
| 1 | PCC/CC1 | default | - | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC/CC2 |  |  | - | - |
|  | SCC/CC3 |  |  | - | - |
|  | SCC/CC4 |  |  | - | - |
| 2 | PCC/CC1 |  |  | CP-OFDM QPSK | Outer_Full |
|  | SCC/CC2 |  |  | - | - |
|  | SCC/CC3 |  |  | - | - |
|  | SCC/CC4 |  |  | - | - |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: CA Configuration Test cumulative aggregated BW settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 3: PCC/CCi and SCC/CCj means PCC is on component carrier CCi and SCC is on component carrier CCj, with CCi or CCj frequencies defined in TS38.508-1 [10]. NOTE 4: Number of DL CCs shall be configured the same as number of UL CCs. The requirements are appliable as per 5.3A.4: "The requirements are applicable only when Uplink CCs are configured within the frequency range between lower edge of lowest downlink component carrier and upper edge of highest downlink component carrier". |  |  |  |  |  |

6.4A.2.4.3.5	Test requirement

Each of the n spectrum flatness functions, shall derive four ripple results in Annex E.4.4. The derived results shall not exceed the values in Figure 6.4A.2.4.3.5-1: The peak-to-peak variation of the EVM equalizer coefficients contained within the frequency range of the uplink allocation shall not exceed the maximum ripple specified in Table 6.4A.2.4.3.5-1 for normal conditions. For uplink allocations contained within both Range 1 and Range 2, the coefficients evaluated within each of these frequency ranges shall meet the corresponding ripple requirement and the following additional requirements: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 (Table 6.4A.2.4.3.5-1) must not be larger than 7 dB + TT, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 8 dB + TT (see Figure 6.4A.2.4.3.5-1).

The UE passes the test when the derived results for at least one polarization fulfil the test requirements.

Table 6.4A.2.4.3.5-1: Test requirements for EVM equalizer spectrum flatness (normal conditions)

| Frequency range | Maximum ripple (dB) |
| --- | --- |
| ¦FUL_Meas – Fcenter¦ ≤ X MHz (Range 1) | 6 +TT (p-p) |
| ¦FUL_Meas – Fcenter¦ > X MHz (Range 2) | 9 + TT (p-p) |
| NOTE 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated NOTE 2: Fcenter refers to the centre frequency of the CC NOTE 3: X, in MHz, is equal to 30% of the CC bandwidth |  |

Figure 6.4A.2.4.3.5-1: The limits for EVM equalizer spectral flatness with the maximum allowed variation of the coefficients indicated under normal conditions

## << End of changes >>