3GPP TSG-RAN5 Meeting #108	R5-254048

# Bengaluru, India, 25th - 29th August, 2025

Agenda item:		5.4.20

Source:				Keysight Technologies UK Ltd

Title:					On Spherical Coverage Test Case for PC6 UEs (HST)

Document for:		Discussion and Endorsement

# Introduction

This contribution discusses the spherical coverage test case for PC6 UEs and what changes in terms of MU, test procedures, etc. have to be taken into account in TS 38.521-2 [1] and TR 38.903 [2]. This contribution is solely focused on the 1 AoA test cases; 2 AoA test cases will be discussed in a future contribution.

# Discussion

The spherical coverage requirements for PC6 UEs (HST) are defined in Clause 6.2.1.1.3.6 of [1].

The minimum EIRP measured over the spherical coverage evaluation areas specified below is defined as the spherical coverage requirement and is found in Table 6.2.1.1.6-3 below. UE spherical coverage evaluation areas are found in Table 6.2.1.1.6-3a below, by consisting of Area-1 and Area-2, in the reference coordinate system in Annex J.1. The requirement is verified with the test metric of EIRP (Link= Spherical coverage grid, Meas=Link angle). Table 6.2.1.1.6-3: UE spherical coverage for power class 6 Operating band; Min EIRP over UE spherical coverage evaluation areas (dBm). n257; 20. n258; 20.4. n261; 20. NOTE 1: Minimum EIRP over UE spherical coverage evaluation areas is defined as the lower limit without tolerance. NOTE 2: The requirements in this table are verified only under normal temperature conditions as defined in TS 38.508-1 [10] subclause 4.1.1. NOTE 3: The requirements in this table are applicable to FR2 PC6 UE with the network signalling highSpeedMeasFlagFR2-r17 configured as set2.. Table 6.2.1.1.6-3a: UE spherical coverage evaluation areas for power class 6 θ range (degree); ϕ range (degree). Area-1; 90 to 60; - 37.5 to + 37.5. Area-2; 90 to 60; 142.5to 217.5. NOTE 1: When testing power class 6 UEs, DUT orientation can be determined according to the UE spherical coverage evaluation areas, not necessarily following default alignment in Figure J.1-2 or positioning guidelines in clause J.3. NOTE 2: High speed train deployment is expected to be w.r.t. the reference coordination system: θ = 90 (degree) corresponds to the ground plane the train is running on, and ϕ= 0 or 180 with θ = 90 are the train track directions..

Unlike for all other power classes, the PC6 spherical coverage does not rely on a CDF of EIRP/EIS measured in all directions over the full sphere, e.g., for PC3 the spherical coverage is defined in Clause 6.2.1.1.3.3 as

The minimum EIRP at the 50th percentile of the distribution of radiated power measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 6.2.1.1.3.3-3 below. The requirement is verified with the test metric of the total component of EIRP, as defined in [5] (Link=Spherical coverage grid, Meas=Link angle).

Instead, the PC6 spherical coverage test assess EIRP/EIS over a very limited range of angles in elevation and azimuth only without postprocessing those measurements using a CDF/CCDF; instead, the EIRP/EIS within the evaluation area are compared against a minimum requirement. The range/evaluation areas over which the PC6 UEs are assessed for spherical coverage performance is visualized in Figure 1. The train track directions are along the ±x axis.

Figure 1: Visualization of PC6 UE and spherical coverage evaluation areas (yellow)

Observation 1: Spherical coverage testing for PC6 UEs does not require a full-sphere assessment and the CDF/CCDF of EIRPs/EISs; instead, the EIRPs/EISs within a partial surface area are compared against a minimum requirement.

Given the lack of CDF/CCDF evaluation for spherical coverage for PC6 UEs, spherical coverage measurement grid analyses applied for all the other power class UEs are not applicable here. It is therefore proposed to leverage the same measurement grid requirements for spherical coverage as for the beam peak search for PC6 UEs only, i.e., the measurement grids defined in Clause M.2.1.6 of [1]

- Constant density grid (using the charged particle implementation) with at least 750 grid points. - Constant step size grid with at least 1106 grid points, corresponding to an angular step size of 7.5º.

apply to Clauses M.3.1.1.6 and M.3.2.1.6 of [1]. Unfortunately, coarser measurement grids were defined for PC6 mistakenly already.

Proposal 1: Discard/correct the spherical coverage grids mistakenly defined for PC6 UEs already in Clauses M.3.1.1.6 and M.3.2.1.6 6 of [1].

Proposal 2: For PC6 UEs only, define the same measurement grid requirements for spherical coverage as for the beam peak search in Clauses M.3.1.1.6 and M.3.2.1.6 of [1].

From a testability perspective, EIRP/EIS has to measured on a very limited set of grid points only for the spherical coverage TC which is further illustrated in Figure 2; those measurements could be leveraged from the beam peak search measurement. The grid points distributed over the whole sphere using a constant step size grid are shown in red while the grid points over which the spherical coverage performance is assessed are shown in green, i.e., they are contained within the yellow spherical coverage evaluation areas. Clearly, the angular step size of 7.5° for the constant step size grid lines up well with the spherical coverage evaluation area as the boundaries are multiple of the 7.5° step size.

Figure 2: Visualization of grid points contributing to spherical coverage performance (green) within evaluation areas (yellow)

In terms of the test procedure for the spherical coverage test case for PC6 UEs, the existing test procedures, e.g., Clause 6.2.1.2.4.2 for TX and Clause 7.3.4.4.2 for RX, need to be augmented to differentiate between PC6 UEs and all other PCs, specifically Steps 4&5 in Clause 6.2.1.2.4.2 for TX and Steps 4 through 6 Clause 7.3.4.4.2 for RX. Alternatively, separate clauses, e.g., 6.2.1.2.4.2a for TX and 7.3.4.4.2a for RX could be added with applicability to PC6 UEs while the existing clauses will be applicable to all PCs other than PC6.

Proposal 3: The test procedures in TS 38.521-2 will have to be augmented in the existing clauses or additional clauses need to be added with specific applicabilities for each clause.

Our suggestion is to augment the existing test procedure to keep things simple. A sample, revised test procedure is shown as below

6.2.1.2.4.2 Test procedure 1. SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2.1.2.4.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. Messages to configure the appropriate uplink modulation in section 6.2.1.2.4.3. 1a. The side conditions for SSB-based and CSI-RS based L1-RSRP measurements are applied as per clause 6.6.1.3.3.1.1 for PC3 and 6.6.1.3.6.1.1 for PC7. 2. Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200 msec to ensure that the UE transmits at its maximum output power. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete. 3. Through its beam correspondence procedure, DUT refines its TX beam toward that direction depending on DUT’s beam correspondence capability which shall match OEM declaration: 3a If the DUT’s beam correspondence capability beamCorrespondenceWithoutUL-BeamSweeping is supported, then DUT autonomously chooses the corresponding TX beam for PUSCH transmission using downlink reference signals to transmit in the direction of the incoming DL signal, which is based on beam correspondence without relying on UL beam sweeping; 3b If the DUT’s beam correspondence capability beamCorrespondenceWithoutUL-BeamSweeping is not present, then DUT chooses the TX beam for PUSCH transmission which is based on beam correspondence with relying on both DL measurements on downlink reference signals and network-assisted uplink beam sweeping: 3b.1) DUT uses downlink reference signals to select proper RX beam and uses autonomous beam correspondence to select the TX beam. 3b.2) SS configures M=8 SRS resources to DUT, with the field spatialRelationInfo omitted and the field usage set as ‘beamManagement’. In case DUT supports less than 8 SRS resources, SS configures the number of SRS resources according to the maximum number of SRS resources indicated by UE capability signalling. Additionally, for codebook based PUSCH transmission, SS configures a semi-persistent SRS resource set with the field usage as 'codebook'. 3b.3) Based on the TX beam autonomously selected by DUT, DUT chooses TX beams to transmit SRS-resources configured by SS. 3b.4) Based on measurement of the received beamManagement SRS, SS chooses the best SRS beam and, if needed, updates the spatial relation information between the semi-persistent codebook SRS resources and the SS selected beamManagement SRS resource in the activation MAC CE of the semi-persistent SRS resource. The SS indicates in the SRS Resource Indicator (SRI) field in the scheduling grant for PUSCH, if present, the SRS resource within the semi-persistent SRS resource set whose spatial relation is linked to the best detected SRS beam. 3b.5) DUT transmits PUSCH corresponding to the SRS resource indicated by the SRI. For power class UEs other than PC6, steps 4 and 5 apply: 4. Measure UE EIRP value for each grid point according to the EIRP spherical coverage procedure defined in Annex K.1.5, and obtain a cumulative distribution function (CDF) of all EIRP dBm values. Alternatively, UE EIRP measurement for each grid point could be done according to Tx Fast spherical coverage procedure defined in Annex K.1.5.1. After a rotation, allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for UE to find the best beam to use. The measuring duration is one active uplink subframe. EIRP is calculated considering both polarizations, theta and phi. 5. Identify the EIRP dBm value corresponding to %-tile (UE power class dependent) value in the applicable test requirement table in section 6.2.1.2.5. For power class 6 UEs, steps 6 and 7 apply: 6. Measure UE EIRP value for each grid point within the UE spherical coverage evaluation areas, Area-1 and Area-2, per Table 6.2.1.1.6-3a. After a rotation, allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for UE to find the best beam to use. The measuring duration is one active uplink subframe. EIRP is calculated considering both polarizations, theta and phi and EIRP needs to be recorded separately for both link polarizations EIRP(PolLink=q), EIRP(PolLink=f). 7. Identify the minimum EIRP dBm value corresponding to value in the applicable test requirement table in clause 6.2.1.2.5. NOTE 1: The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.

7.3.4.4.2 Test procedure 1. SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.3.2.4.1-1. The SS sends downlink MAC padding bits on the DL RMC. 2. SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Tables 7.3.2.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. 3. Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200msec for the UE to reach PUMAX. For power class UEs other than PC6, step 4a applies: 4a. Measure UE EIS value for each grid point according to EIS spherical coverage procedure defined in Annex K.1.6.0, and obtain a Complimentary Cumulative Distribution Function (CCDF) of all EIS dBm values. Alternatively, UE EIS measurement for each grid point could be done according to Rx Fast spherical coverage procedure defined in Annex K.1.6.1. After a rotation, allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for UE to find the best beam to use. EIS is calculated considering both polarizations, theta and phi. For power class 6 UEs, step 4b applies: 4b. Measure UE EIS value for each grid point within the UE spherical coverage evaluation areas, Area-1 and Area-2, per Table 6.2.1.1.6-3a. After a rotation, allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for UE to find the best beam to use. EIS is calculated considering both polarizations, theta and phi. 5. Identify the EIS dBm value corresponding to %-tile (UE power class dependent) value in the applicable test requirement table in section 7.3.4.5. 6. Compare the EIS dBm value identified in step 5, to the limit value in the applicable test requirement table in section 7.3.4.5. If the EIS dBm value is lower or equal to the limit value, pass the UE. Otherwise fail the UE. NOTE 1: The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.2.

The MU for the PC6 spherical coverage test case does not require much additional work. As mentioned above, the traditional spherical coverage MU considerations, used for all power classes other than PC6 previously, do not apply here. Instead, as the spherical coverage test case for PC6 is comparing EIRP/EIS within a limited angular range in EL and AZ against a minimum requirement similar to the MOP-EIRP and Refsens-EIS test cases, it is therefore proposed to leverage the MU analyses from the MOP-EIRP (Min peak EIRP)/Refsens-EIS test cases for PC6 Spherical coverage. The only difference is the assessment of the Influence of Noise MU element for TX which should take the minimum requirement for spherical coverage rather than min peak EIRP requirement of the MOP-EIRP test case into account.

Proposal 4: Leverage the MU analyses from the MOP-EIRP (Min peak EIRP)/Refsens-EIS test cases for PC6 Spherical coverage while re-evaluating the Influence of Noise for TX using the min EIRP requirement of the PC6 spherical coverage requirement rather than the min peak EIRP requirement.

# Conclusion

The following observations and conclusions were made in this contribution.

Observation 1: Spherical coverage testing for PC6 UEs does not require a full-sphere assessment and the CDF/CCDF of EIRPs/EISs; instead, the EIRPs/EISs within a partial surface area are compared against a minimum requirement.

Proposal 1: Discard/correct the spherical coverage grids mistakenly defined for PC6 UEs already in Clauses M.3.1.1.6 and M.3.2.1.6 6 of [1].

Proposal 2: For PC6 UEs only, define the same measurement grid requirements for spherical coverage as for the beam peak search in Clauses M.3.1.1.6 and M.3.2.1.6 of [1].

Proposal 3: The test procedures in TS 38.521-2 will have to be augmented in the existing clauses or additional clauses need to be added with specific applicabilities for each clause.

Proposal 4: Leverage the MU analyses from the MOP-EIRP (Min peak EIRP)/Refsens-EIS test cases for PC6 Spherical coverage while re-evaluating the Influence of Noise for TX using the min EIRP requirement of the PC6 spherical coverage requirement rather than the min peak EIRP requirement.

# References

- TS 38.521-2, User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone

- TR 38.903, Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases