3GPP TSG-RAN5 Meeting #108	R5-254064

# Bengaluru, India, 25th - 29th August, 2025

Agenda item:	5.4.20

Source:	Keysight Technologies UK Ltd

Title:	On Out-of-Band Blocking test optimization

Document for:	Discussion and Endorsement

# Introduction

During RAN5#107 meeting the paper [1] was presented to discuss and propose alternatives in order to reduce testing time in out-of-band blocking (OBB) test case. General concerns were presented and more analysis required. As a result, AP#107.21 was created. This contribution does a counter analysis of proposals in [1] and shares Keysight views.

# Discussion

## Measurement complexity

Both the title and the focus presented in first 2 slides in [1] is not explicitly how to reduce the testing time but how to reduce the measurement complexity of Out-of-Band Blocking into RAN5.

However, the proposals made in [1] really increase the measurement complexity in order to reduce the testing time.

The more complex the interferer frequency selection algorithm, the higher the risk of getting into different test case implementations. In addition, the complexity of the proposals made in [1], where only single CC case has been analysed, will be larger once for example CA is taken into consideration. The frequency tuples will be different for every combo and the complexity of the intermodulation product analysis will increase with the number of CCs part of the CA/DC.

Current test procedure is simpler and aligned with the minimum requirements defined by RAN4. A constant step size, 5MHz in most of the cases, is used to determine the interferer frequency within the 3 defined frequency ranges.

Observation 1. Proposals made in [1] to reduce the testing time increase the test procedure complexity, driving more likely into implementation deviations among test solutions.

Observation 2. Proposals made in [1] to reduce the testing time have only considered the single carrier scenario. Consideration of other scenarios like CA may add extra complexity to the test procedure.

## Test purpose coverage

As mentioned in section 2.1, current test procedure is aligned with the minimum requirements defined by RAN4. A constant step size of  MHz, is used to determine the interferer frequency within the 3 defined frequency ranges. Proposals in [1] essentially propose not to test all the interferer frequencies defined by RAN4. Furthermore, it is not clear what to do with test case 7.7 Spurious Response, as it is not clear if exceptions should be permitted if the interferer test points are significantly reduced and then maybe test 7.7 should be skipped. Therefore, core requirements will not be tested as defined if proposals in [1] are endorsed.

The justifications made in [1] to do such a simplification may be incomplete. When testing Range 3, the difference between interferer power and wanted signal power is really high, greater than 50dB in many cases. That has not been analysed in detail nor the real performance of the band filter, the LNA or the LPF. Indeed, the purpose of the OBB test case is to verify the performance of those filters and the RX chain in general because they cannot be assumed to have a perfect rejection and for that reason the full frequency ranges should be verified and not just a very limited selection of interferer frequencies.

Observation 3. Proposals made in [1] will make the core requirements not to be tested as defined by RAN4 and the justification of such a deviation has not considered key aspects like the big power difference between the interferer and the wanted signal or the none-ideality of the DUT RX chain and filters that are the main purpose of the OBB test.

## Test time duration

The analysis of the test time duration is incorrect in [1]. There it is assumed that the 3 CBWs to be tested, low mid and high, will sweep the 3 frequency ranges. However, according to Note 9 and Note 5 in test configuration Table 7.6.3.5-2 and Table 7.6.3.5-4 respectively in [2], Range 3 shall be tested only with the highest channel bandwidth. Range 3 is the widest frequency range and for the highest channel BW the interferer frequency step will be the widest one, 5MHz. In addition, in [1] it is assuming that 2 test frequencies apply. However, according to test configuration table, only one frequency chosen arbitrarily from low or high range. As a result, the test time will be about ¼ the test time mentioned in [1], that is 1h15mins instead of 5hs for band n3. We can confirm empirically test time duration similar to that value close to 1h. That is a 75% test time reduction vs how analysis had been made in [1]. Later in the document, with all proposals made, the expectation is to have an 80% test time reduction. That is, considering the errors in that analysis, tests durations about 1h would be an acceptable target. Considering such a duration is already the reality then it seems not fully justifiable the deviation of testing purpose captured in the core requirements as explained in previous subsection.

Observation 4. Analysis made in [1], with certain errors, is expecting a test time duration close to 1h with the proposals made there. However, a correct time analysis indicates that current test time duration is already close to 1h. Therefore, the deviation from core requirements testing is not justifiable if such a time duration is considered acceptable.

Proposal 1. Not to deviate OBB test procedure from core requirements while current test time duration is acceptable for most of the bands.

## Regulatory Testing

In European regulatory testing defined in [3], In section 4.1.2.9.1.2.2, essentially the same OBB requirements are defined, using the same frequency ranges and the same frequency step for the interferer signal.

If RAN5 conformance testing defined in [2] is modified as per proposals made in [1], then there will be a deviation between conformance testing and European regulatory testing and it is unclear how and when the regulatory body would react to those changes.

Observation 5. If RAN5 conformance testing defined in [2] is modified as per proposals made in [1], then there will a deviation between conformance testing and European regulatory testing for an undetermined period of time.

# Conclusion

The following observations and proposals were made in this contribution.

Observation 1. Proposals made in [1] to reduce the testing time increase the test procedure complexity, driving more likely into implementation deviations among test solutions.

Observation 2. Proposals made in [1] to reduce the testing time have only considered the single carrier scenario. Consideration of other scenarios like CA may add extra complexity to the test procedure.

Observation 3. Proposals made in [1] will make the core requirements not to be tested as defined by RAN4 and the justification of such a deviation has not considered key aspects like the big power difference between the interferer and the wanted signal or the none-ideality of the DUT RX chain and filters that are the main purpose of the OBB test.

Observation 4. Analysis made in [1], with certain errors, is expecting a test time duration close to 1h with the proposals made there. However, a correct time analysis indicates that current test time duration is already close to 1h. Therefore, the deviation from core requirements testing is not justifiable if such a time duration is considered acceptable.

Observation 5. If RAN5 conformance testing defined in [2] is modified as per proposals made in [1], then there will a deviation between conformance testing and European regulatory testing for an undetermined period of time.

Proposal 1. Not to deviate OBB test procedure from core requirements while current test time duration is acceptable for most of the bands.

# References

- R5-253614, “Discussion on how to reduce the measurement complexity of Out-of-Band Blocking in RAN5”, Huawei, Hisilicon, RAN5#107

- TS 38.521-1, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone”, v19.0.0 (2025-06)

- ETSI EN 301 908-25, “IMT cellular networks; Harmonised Standard for access to radio spectrum; Part 25: New Radio (NR) User Equipment (UE) Release 15”, v15.1.1 (2024-10)