3GPP TSG-RAN5 Meeting #108	R5-254120

# Bengaluru, India, 25th – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0686 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new NR IMS data channel test case 7.45 - MO Video call with precondition and BDC success |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_IMS_dataCH-UEConTest |  |  |  |  |  | Date: |  |  | 2025-07-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This new IMS data channel (IMS DC) test case is added to test: SS send SDP offer with IMS DC and MT UE reply SDP answer with IMS DC. The UE can establish video call with precondition + IMS DC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new NR IMS data channel test case 7.45 IMS Data Channel / DCMTSI MT Video call / Bootstrap Data Channel success / with preconditions at both originating UE and terminating UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NG_RTC_IMS_dataCH-UEConTest is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.45(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 34.229-2 CR 0393 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

7.45	IMS Data Channel / DCMTSI MT Video call / Bootstrap Data Channel success / with preconditions at both originating UE and terminating UE

7.45.1	Test Purpose (TP)

(1)

with { UE supporting IMS data channel and the network supports the IMS data channel }

ensure that {

when { UE receive INVITE message }

then { UE shall include the media feature tags for supported streaming media type with +sip.app-subtype="webrtc-datachannel" in the Contact header field of SIP 18x and 2xx responses to the SIP INVITE request }

}

(2)

with { UE supporting IMS data channel and the network supports the IMS data channel and UE is configured to use IMS data channel }

ensure that {

when { UE receives the INVITE request with an SDP offer, which includes the bootstrap data channel media descriptions }

then { UE generate the SDP answer and shall return a 18x or 2xx response to the INVITE request with the SDP answer }

}

7.45.2	Conformance Requirements

Same as 7.41.2.

7.45.3	Test description

7.45.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured to register for IMS after switch on.

-	The UE is configured to use preconditions.

-	The UE is configured to use IMS data channel.

Preamble:

-	UE is in state 1N-A (TS 38.508-1 [21]) and registered to IMS.

7.45.3.2	Test procedure sequence

Table 7.45.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-8 | Steps 1-8 of generic procedure specified in Table 4.9.16.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| 9 | SS sends INVITE with SDP offer containing the media lines for audio, video and IMS data channel. (Step 1 of Annex A.16.1a) | <-- | INVITE | - | - |
| 10 | UE may send a 100 Trying provisional response. (Step 2 of Annex A.16.1a) | --> | 100 Trying | - | - |
| 11 | UE sends SDP answer in 183 Session Progress. (Step 3 of Annex A.16.1a) | --> | 183 Session Progress | 1,2 | P |
| 12 | SS acks reception of 183 Session Progress. (Step 4 of Annex A.16.1a) | <-- | PRACK | - | - |
| 13 | UE responds to PRACK. (Step 5 of Annex A.16.1a) | --> | 200 OK | - | - |
| 14-16 | Steps 10-12 of generic procedure specified in Table 4.9.16.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| 17 | SS sends a second SDP offer. (Step 6 of Annex A.16.1a) | <-- | UPDATE | - | - |
| 18 | UE responds to UPDATE, including an SDP answer. (Step 7 of Annex A.16.1a) | --> | 200 OK | - | - |
| 19 | UE sends 180 Ringing. (Step 8 of Annex A.16.1a) | --> | 180 Ringing | - | - |
| 20 | SS sends PRACK. (Step 9 of Annex A.16.1a) | <-- | PRACK | - | - |
| 21 | UE sends 200 OK for PRACK. (Step 10 of Annex A.16.1a) | --> | 200 OK | - | - |
| 22 | Make the UE accept the video call. (Note 1) | - | - | - | - |
| 23 | UE accepts the video call. (Step 11 of Annex A.16.1a) | --> | 200 OK | 1,2 | P |
| 24-26 | Steps 1-3 of generic procedure specified in Table 4.9.49.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| 27 | SS acknowledges. (Step 12 of Annex A.16.1a) | <-- | ACK | - | - |
| 28-29 | SS releases the call. (Annex A.8) | - | - | - | - |
| Note 1: This is achieved by MMI or AT command. Note 2: SS ignore the DRB including ClientHello for DTLS handshake sent by UE. |  |  |  |  |  |

7.45.3.3	Specific message contents

Table 7.45.3.3-1: INVITE (step 9, table 7.45.3.2-1)

| Derivation Path: TS 34.229-5, Step 1 of A.16.1a, with following exceptions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Contact feature-param |  | Feature-param is same as TS 34.229-5, Step 1 of A.16.1a except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |  | TS 24.186 [54] |
| Message-body |  | SDP body is same as TS 34.229-5, Step 1 of A.16.1a except adding one Media description for IMS bootstrap data channel as follow: Media description for IMS data channel: m=application (transport port) UDP/DTLS/SCTP webrtc-datachannel [Note 1] b=AS:500 Attributes for media for IMS data channel: a=max-message-size:64000 a=sctp-port:(sctp-port-value) [Note 2] a=fingerprint:SHA-256 (fingerprint) [Note 3] a=setup:actpass a=tls-id:(tls-id-value) a=dcmap:0 subprotocol="http” Note 1: Transport port is the port number of the SS. Note 2: The sctp-port-value is the SCTP port of the SS. Note 3: Fingerprint is calculated by SS. Note 4: The tls-id-value is random number generated by SS. |  | TS 26.114 [33] RFC 8841 [55] RFC 8841 [55] RFC 8122 [56] RFC 4145 [59] RFC 8842 [57] RFC 8864 [58] |

Table 7.45.3.3-2: 183 Session Progress (step 11, table 7.45.3.2-1)

| Derivation Path: TS 34.229-5, Step 3 of A.16.1a, with following exceptions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Contact feature-param |  | Feature-param is same as TS 34.229-5, Step 3 of A.16.1a except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |  | TS 24.186 [54] |
| Message-body |  | SDP body is same as TS 34.229-5, Step 3 of A.16.1a except it may add one Media description as follow: Media description for IMS data channel:[Note 4] m=application (transport port) UDP/DTLS/SCTP webrtc-datachannel Attributes for media for IMS data channel: [Note 4] a=max-message-size: (max-message-size-value)[Note 1] a=sctp-port:(sctp-port-value) [Note 2] a=setup: active a=fingerprint: (hash-func) (fingerprint) [Note 3] a=tls-id: (tls-id-value) a=dcmap:0 subprotocol="http" Note 1: Optional attribute. If the SDP”max-message-size” attribute is not present the default value is 64K. Note 2: SCTP port range is between 0 and 65535(both included). Leading zeroes must not be used. Note 3: SHA-1, SHA-224, SHA-256, SHA-384, or SHA-512 is used as hash-func. Note4: The media description and attributes for media for IMS data channel shall present in 183 Session Progress or 200 OK for INVITE. |  | TS 26.114 [33] |

Table 7.45.3.3-3: UPDATE (step 17, table 7.45.3.2-1)

| Derivation Path: TS 34.229-5, Step 6 of A.16.1a, with following exceptions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Message-body |  | SDP body is same as TS 34.229-5, Step 6 of A.16.1a except adding one Media description as follow: Media description for IMS data channel: Same as it in the INVITE message. Attributes for media for IMS data channel: Same as it in the INVITE message. |  | TS 26.114 [33] |

Table 7.45.3.3-4: 200 OK for UPDATE (step 18, table 7.45.3.2-1)

| Derivation Path: TS 34.229-5, Step 7 of A.16.1a, with following exceptions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Contact feature-param |  | Feature-param is same as TS 34.229-5, Step 7 of A.16.1a except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |  | TS 24.186 [54] |
| Message-body | A1 | SDP body is same as TS 34.229-5, Step 7 of A.16.1a except adding one Media description as follow: Media description for IMS data channel: Same as it in the 183 Session Progress message. Attributes for media for IMS data channel: Same as it in the 183 Session Progress message. |  | TS 26.114 [33] |
|  | A2 | SDP body is same as TS 34.229-5, Step 7 of A.16.1a |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | 183 Session Progress includes SDP offer containing media lines for audio, video and IMS data channel in Step 11. |
| A2 | 183 Session Progress includes SDP offer containing media lines for audio and video in Step 11. |

Table 7.45.3.3-5: 200 OK for INVITE (step 23, table 7.45.3.2-1)

| Derivation Path: TS 34.229-5, Step 11 of A.16.1a, with following exceptions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Contact feature-param |  | Feature-param is same as TS 34.229-5, Step 11 of A.16.1a except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |  | TS 24.186 [54] |
| Message-body | A2 | SDP body is same as TS 34.229-5, Step 6 of A.16.1a (UPDATE) except it add one Media description as follow: Media description for IMS data channel: m=application (transport port) UDP/DTLS/SCTP webrtc-datachannel b=AS:500 Attributes for media for IMS data channel: a=max-message-size: (max-message-size-value) [Note 1] a=sctp-port:(sctp-port-value) [Note 2] a=setup: active a=fingerprint: (hash-func) (fingerprint) [Note 3] a=tls-id: (tls-id-value) a=dcmap:0 subprotocol="http" Note 1: Optional attribute. If the SDP”max-message-size” attribute is not present the default value is 64K. Note 2: SCTP port range is between 0 and 65535(both included). Leading zeroes must not be used. Note3: SHA-1, SHA-224, SHA-256, SHA-384, or SHA-512 is used as hash-func |  | TS 26.114 [33] |
|  | A1 | SDP body is empty. |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | 183 Session Progress includes SDP offer containing media lines for audio, video and IMS data channel in Step 11. |
| A2 | 183 Session Progress includes SDP offer containing media lines for audio and video in Step 11. |

<End of modified section 1>