3GPP TSG-RAN5 Meeting #108	R5-254182

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0692 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrrection to eCALL test case 11.16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Sweden |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCallCEN-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Correction to eCALL initiate method, Invite condition and ACK. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correction is done to handle 34.229-5 Annex A.23 with condition A20 for eCALL initiation for Manual initiation instead automatic. - Table 11.16.3.3-4 is corrected as per manual A11 condition. Table 11.16.3.3-3: Parameter should be A4 instead A1 for ACK for non-2xx response as per TS 34.229-1 [2], Annex A.2.7 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eCallCEN-UEConTest WI will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

## 11.16	eCall capable / Manual initiation / Emergency registration / Abnormal case / IM CN sends a 600 (Busy Everywhere) with positive ACK / PSAP Callback / Success

11.16.1	Test Purpose (TP)

(1)

with { UE is requested to make a manual eCall over IMS }

ensure that {

when { UE receives 600 (Busy Everywhere) with positive ACK }

then { UE considers MSD transmission successful }

}

(2)

with { UE subjected to national regulations in CEN EN 17184 having previously receives 600 (Busy Everywhere) with positive ACK }

ensure that {

when { UE receives INVITE for an incoming IMS call for PSAP callback }

then { UE responds with 200 OK including correctly composed header fields }

}

(3)

with { UE subjected to national regulations in CEN EN 17184 having previously 600 (Busy Everywhere) with positive ACK }

ensure that {

when { SS requests for transfer of MSD during an incoming IMS call }

then { UE responds with 200 OK and performs MSD transfer }

}

11.16.2	Conformance requirement

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-19 requirements.

[TS 24.229, clause 5.1.6.11.1]:

If the upper layers request establishment of an IMS emergency call of the manually initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.manual" as specified in RFC 8147 [244].

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.automatic" as specified in RFC 8147 [244].

NOTE 1:	The manually initiated eCall type of emergency service is used when the eCall IMS emergency session is invoked with user input. The automatically initiated eCall type of emergency service is used if the eCall IMS emergency session is invoked without user input.

NOTE 2:	To ensure the identification and the related routing (e.g. to a test centre) of non-emergency calls initiated for eCall testing or eCall terminal reconfiguration purposes, "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual" are not used for such calls. Additionally the usage of any other service URN to establish non-emergency calls initiated for eCall testing or eCall terminal reconfiguration purposes needs to be in accordance with local operator policy.

[TS 24.229, clause 5.1.6.11.2]:

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service and if allowed by IP-CAN specific annex, the UE shall send an INVITE request as specified in the procedures in subclause 5.1.6.8 with the following additions:

1)	the UE shall set the Request-URI to "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual"; and

2)	if the IP-CAN indicates the eCall over IMS support defined in 3GPP TS 36.331 [19F] or in 3GPP TS 38.331 [19G], the UE shall:

a)	insert a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in CEN EN 15722:2020 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

b)	insert an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

c)	insert a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE 1:	Further content for the INVITE is as defined in RFC 8147 [244].

Then the UE shall proceed as follows:

…

3)	if the UE receives a 486 (Busy Here), 600 (Busy Everywhere) or 603 (Decline) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "true"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as successful and shall act as follows:

iii)	if the UE is in limited service state, perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B]; and

iv)	if the UE is not in limited service state, report the response and successful MSD transfer to upper layers; and

4)	in all other cases, the UE shall perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

NOTE 2:	For case 3-a-iv). upper layers can wait for an incoming call from the PSAP as defined in CEN EN 17184:2024 [300] if incoming IMS calls are possible or if the upper layers can instigate CS fallback to receive incoming calls over CS. If incoming calls cannot be received by the UE, upper layers can perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

Reference(s)

3GPP TS 24.229 [10], clauses 5.1.6.11.1 and 5.1.6.11.2.

Reference(s)

3GPP TS 24.229 [10], clause 5.1.6B.

11.16.3	Test description

11.16.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC with eCall subscription according to TS 38.508-1 [21] table 6.4.1-23.

-	UE is configured to register for IMS after switch on.

Preamble:

-	UE is in state 1N-A (TS 38.508-1 [21]) and registered to IMS.

11.16.3.2	Test procedure sequence

Table 11.16.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is triggered to start an manual eCall | - | - | - | - |
| 2-11 | Steps 1-10 of generic procedure specified in Table 4.9.11.2.2-1 of TS 38.508-1 [21] with condition ‘eCall’ are performed. (Steps 1-4 of Annex A.3) | - | - | - | - |
| 12 | Step 1 of Annex A.23 happens | --> | INVITE | - | - |
| 13 | The SS sends 600 Busy Everywhere response | <-- | 600 Busy Everywhere | - | - |
| 14 | Check: Does UE send ACK ? | --> | ACK | 1 | P |
| 15 | The SS releases the RRC connection. | - | - | - | - |
| 16 | SS waits for 10 seconds | - | - | - | - |
| 17-28 | Steps 1 to 12 of the generic test procedure for IMS MT speech call establishment in 5GC specified in TS 38.508-1 [21] subclause 4.9.16.2.2.1 are performed. (Steps 1-5 of Annex A.5.1a for UE configured with usage of preconditions else A.5.2a) | - | - | - | - |
| - | EXCEPTION: Steps 29a1 to 29b6 describe behaviour that depends on UE configuration; the “lower case letter” identifies a step sequence that takes place if such configuration was conducted. | - | - | - | - |
| 29a1-29a6 | IF the UE is configured to use preconditions THEN steps 6-10A of Annex A.5.1a take place. | - | - | - | - |
| 29a7 | Check: Does the UE send a correctly composed 200 OK for INVITE request? (Step 11 of annex A.5.1a) | --> | 200 OK | 2 | P |
| 29a8 | SS sends ACK | <-- | ACK | - | - |
| 29b1-29b4 | ELSE steps 6-8A of Annex A.5.2a of take place | - | - | - | - |
| 29b5 | Check: Does the UE send a correctly composed 200 OK for INVITE request? (Step 9 of annex A.5.2a) | --> | 200 OK | 2 | P |
| 29b6 | SS send ACK | <-- | ACK | - | - |
| 30-31 | Steps 4-5, as defined in Annex A.23 happens | - | - | - | - |
| 32 | Check: Does UE send SIP INFO message with MSD update? (Step 6 of annex A.23) | --> | SIP INFO | 3 | P |
| 33 | SS responds SIP INFO with 200 OK. | <-- | 200 OK | - | - |
| 34 | Generic test procedure for IMS MT call release in 5GS described in TS 38.508-1 [21] subclause 4.9.18 takes place. | - | - | - | - |

11.16.3.3	Specific message contents

Table 11.16.3.3-1: INVITE (Step 12 (Step 1 of annex A.23), Table 11.16.3.2-1)

Derivation path: Step 1 in Annex A.23 with condition A20

Table 11.16.3.3-2: 600 Busy Everywhere (step 13, table 11.11.3.2-1)

Derivation path: TS 34.229-1 [2], Annex A.2.22 with condition A1.

Table 11.16.3.3-3: ACK (step 14, table 11.11.3.2-1)

Derivation path: TS 34.229-1 [2], Annex A.2.7 with condition A4.

Table 11.16.3.3-4: INVITE (Step 25, Table 11.16.3.2-1)

Derivation path: Step 1 in Annex C.11 of TS 34.229-1[2] with condition A11

Table 11.16.3.3-5: 200 OK (Steps 29a7, 29b5, Table 11.16.3.2-1)

Derivation path: Step 12 in Annex C.11 of TS 34.229-1[2] with condition A24

Table 11.16.3.3-6: BYE (Steps 34, Table 11.16.3.2-1)

Derivation path: Step 1 in Annex A.8 with condition A9

<End of modified section>