3GPP TSG-RAN5 Meeting #108	R5-254232

# Bengaluru, India, 25th - 29th August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0995 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Missing DL RMC for eRedCap FDD test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test case 5.2.1.1.2 test num 1-2 uses R.PDSCH.1-25.1 FDD as reference channel. However, this RMC is defined in TS 38.101-4 v18.8.0, but not defined in TS 38.521-4 Annex A.3.2.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include definition of R.PDSCH.1-25.1 FDD in TS 38.521-4 Annex A.3.2.1, according to definition of R.PDSCH.1-25.1 in TS 38.101-4 v18.8.0. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case RMC configuration will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

### A.3.2.1	FDD

#### A.3.2.1.1	Reference measurement channels for SCS 15 kHz FR1

## <<< Skip unchanged tables >>>

Table A.3.2.1.1-24: PDSCH Reference Channel for FDD PMI reporting requirements

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-X.1 FDD |  |  |  |  |
| Channel bandwidth | MHz | 10 |  |  |  |  |
| Subcarrier spacing | kHz | 15 |  |  |  |  |
| Number of allocated resource blocks | PRBs | 52 |  |  |  |  |
| Number of consecutive PDSCH symbols |  | 12 |  |  |  |  |
| Allocated slots per 2 frames | Slots | 12 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layer |  | 2 |  |  |  |  |
| Number of DMRS REs (Note 3) |  | 24 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For CSI Slots i, if mod (i, 10) = {0,2,4,6}, i={0,…,19} |  | N/A |  |  |  |  |
| For Non CSI-RS Slot i, if mod (i, 10) ={1,3,5,7,8,9}, i={1,..19} | Bits | 24072 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For CSI Slots i, if mod (i, 10) = {0,2,4,6}, i={0,…,19} |  | N/A |  |  |  |  |
| For Non CSI-RS Slot i, if mod (i, 10) ={1,3,5,7,8,9}, i={1,..19} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For CSI Slots i, if mod (i, 10) = {0,2,4,6}, i={0,…,19} |  | N/A |  |  |  |  |
| For Non CSI-RS Slot i, if mod (i, 10) ={1,3,5,7,8,9}, i={1,..19} | CBs | 3 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A |  |  |  |  |
| For CSI Slots i, if mod (i, 10) = {0,2,4,6}, i={0,…,19} |  | N/A |  |  |  |  |
| For Slots i = 11 | Bits | 47424 |  |  |  |  |
| For Non CSI-RS Slot i, if mod (i, 10) = {1,3,5,7,8,9}, i={0,..10,12,…,19} | Bits | 49920 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 14.443 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data |  |  |  |  |  |  |

Table A.3.2.1.1-25: PDSCH Reference Channel for FDD

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-25.1 FDD |  |  |  |  |
| Channel bandwidth | MHz | 10 |  |  |  |  |
| Subcarrier spacing | kHz | 15 |  |  |  |  |
| Number of allocated resource blocks | PRBs | 25 |  |  |  |  |
| Number of consecutive PDSCH symbols |  | 12 |  |  |  |  |
| Allocated slots per 2 frames | Slots | 19 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A |  |  |  |  |
| For Slots i = 1,…, 19 | Bits | 6272 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A |  |  |  |  |
| For Slots i = 1,…, 19 | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A |  |  |  |  |
| For Slots i = 1,…, 19 | CBs | 1 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A |  |  |  |  |
| For Slots i = 10, 11 | Bits | 12600 |  |  |  |  |
| For Slots i =1,…, 9, 12, …, 19 | Bits | 13200 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 5.958 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

## <<< END OF CHANGES >>>