3GPP TSG-RAN WG5 Meeting # 108	R5-254314

# Bengaluru, India, August 25 – 29, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3554 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on test freq for R15 EN-DC configuration with more than three bands |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on the discussion paper in R5-254310, the following guidelines are suggested to be applied to the test frequencies for EN-DC configurations. - For each inter-band EN-DC configuration, remove the duplicated E-UTRA/NR downlink configurations and keep only one entry for E-UTRA/NR downlink configuration. - For E-UTRA/NR downlink or uplink configuration, if inter-band or intra-band CA is applied, the prefix “CA_” should not be omitted. If single band is applied, “xA” or “nxA” should be denoted, where “x”/”nx” is E-UTRA/NR band number. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the test frequency for R15 EN-DC configurations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test frequencies for R15 EN-DC configurations will remain redundant. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.4.1.3, 4.3.1.4.1.4, 4.3.1.4.1.5, 4.3.1.4.1.6, 4.3.1.5.1.3, 4.3.1.5.1.4, 4.3.1.5.1.5, 4.3.1.6.1.2, 4.3.1.6.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

4.3.1.4.1.3	Inter-band EN-DC configurations within FR1 (three bands)

Table 4.3.1.4.1.3-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC Configuration | E-UTRA downlink configuration | NR downlink configuration | E-UTRA uplink configuration | NR uplink configuration | Applicable for protocol testing (Note 1) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_1A-3A_n28A | DC_1A_n28A | CA_1A-3A | n28A | 1A | n28A | No |
|  | DC_3A_n28A |  |  | 3A | n28A |  |
| DC_1A-3A_n41A | DC_1A_n41A | CA_1A-3A | n41A | 1A | n41A | No |
|  | DC_3A_n41A | CA_1A-3A | n41A | 3A | n41A | No |
| DC_1A-3A_n77A | DC_1A_n77A | CA_1A-3A | n77A | 1A | n77A | No |
|  | DC_3A_n77A |  |  | 3A | n77A |  |
| DC_1A-3A_n78A | DC_1A_n78A | CA_1A-3A | n78A | 1A | n78A | No |
|  | DC_3A_n78A |  |  | 3A | n78A |  |
| DC_1A-3C_n77A | DC_1A_n77A | CA_1A-3C | n77A | 1A | n77A | No |
|  | DC_3A_n77A | CA_1A-3C | n77A | 3A | n77A | No |
| DC_1A-3C_n77(2A) | DC_1A_n77A | CA_1A-3C | CA_n77(2A) | 1A | n77A | No |
|  | DC_3A_n77A | CA_1A-3C | CA_n77(2A) | 3A | n77A | No |
| DC_1A-3C_n78A | DC_1A_n78A | CA_1A-3C | n78A | 1A | n78A | No |
|  | DC_3A_n78A |  |  | 3A | n78A |  |
| DC_1A-3C_n78(2A) | DC_1A_n78A | CA_1A-3C | CA_n78(2A) | 1A | n78A | No |
|  | DC_3A_n78A | CA_1A-3C | CA_n78(2A) | 3A | n78A | No |
|  | DC_3C_n78A | CA_1A-3C | CA_n78(2A) | CA_3C | n78A | No |
| DC_1A-1A-3A_n78A | DC_1A_n78A | CA_1A-1A-3A | n78A | 1A | n78A | No |
|  | DC_3A_n78A | CA_1A-1A-3A | n78A | 3A | n78A | No |
| DC_1A-1A-5A_n78A | DC_1A_n78A | CA_1A-1A-5A | n78A | 1A | n78A | No |
|  | DC_5A_n78A | CA_1A-1A-5A | n78A | 5A | n78A | No |
| DC_1A-1A-3C_n78A | DC_1A_n78A | CA_1A-1A-3C | n78A | 1A | n78A | No |
|  | DC_3A_n78A | CA_1A-1A-3C | n78A | 3A | n78A | No |
|  | DC_3C_n78A | CA_1A-1A-3C | n78A | CA_3C | n78A | No |
| DC_1A-3A_n79A | DC_1A_n79A | CA_1A-3A | n79A | 1A | n79A | No |
|  | DC_3A_n79A |  |  | 3A | n79A |  |
| DC_1A-5A_n78C | DC_1A_n78A | CA_1A-5A | CA_n78C | 1A | n78A | No |
|  | DC_5A_n78A | CA_1A-5A | CA_n78C | 5A | n78A | No |
| DC_1A-7A_n3A | DC_1A_n3A | CA_1A-7A | n3A | 1A | n3A | No |
|  | DC_7A_n3A | CA_1A-7A | n3A | 7A | n3A | No |
| DC_1A-7A_n28A | DC_1A_n28A | CA_1A-7A | n28A | 1A | n28A | No |
|  | DC_7A_n28A |  |  | 7A | n28A |  |
| DC_1A-7A_n78A | DC_1A_n78A | CA_1A-7A | n78A | 1A | n78A | No |
|  | DC_7A_n78A |  |  | 7A | n78A |  |
| DC_1A-8A_n3A | DC_1A_n3A | CA_1A-8A | n3A | 1A | n3A | No |
|  | DC_8A_n3A | CA_1A-8A | n3A | 8A | n3A | No |
| DC_1A-8A_n77A | DC_1A_n77A | CA_1A-8A | n77A | 1A | n77A | No |
|  | DC_8A_n77A | CA_1A-8A | n77A | 8A | n77A | No |
| DC_1A-8A_n77(2A) | DC_1A_n77A | CA_1A-8A | CA_n77(2A) | 1A | n77A | No |
|  | DC_8A_n77A | CA_1A-8A | CA_n77(2A) | 8A | n77A | No |
| DC_1A-8A_n78A | DC_1A_n78A | CA_1A-8A | n78A | 1A | n78A | No |
|  | DC_8A_n78A |  |  | 8A | n78A |  |
| DC_1A-8A_n78(2A) | DC_1A_n78A | CA_1A-8A | CA_n78(2A) | 1A | n78A | No |
|  | DC_8A_n78A | CA_1A-8A | CA_n78(2A) | 8A | n78A | No |
| DC_1A-18A_n77A | DC_1A_n77A | CA_1A-18A | n77A | 1A | n77A | No |
|  | DC_18A_n77A |  |  | 18A | n77A |  |
| DC_1A-19A_n77(2A) | DC_1A_n77A | CA_1A-19A | CA_n77(2A) | 1A | n77A | No |
|  | DC_19A_n77A | CA_1A-19A | CA_n77(2A) | 19A | n77A | No |
| DC_1A-19A_n78A | DC_1A_n78A | CA_1A-19A | n78A | 1A | n78A | No |
|  | DC_19A_n78A |  |  | 19A | n78A |  |
| DC_1A-19A_n78(2A) | DC_1A_n78A | CA_1A-19A | CA_n78(2A) | 1A | n78A | No |
|  | DC_19A_n78A | CA_1A-19A | CA_n78(2A) | 19A | n78A | No |
| DC_1A-19A_n79A | DC_1A_n79A | CA_1A-19A | n79A | 1A | n79A | No |
|  | DC_19A_n79A |  |  | 19A | n79A |  |
| DC_1A-20A_n3A | DC_1A_n3A | CA_1A-20A | n3A | 1A | n3A | No |
|  | DC_20A_n3A | CA_1A-20A | n3A | 20A | n3A | No |
| DC_1A-20A_n8A | DC_1A_n8A | CA_1A-20A | n8A | 1A | n8A | No |
|  | DC_20A_n8A | CA_1A-20A | n8A | 20A | n8A | No |
| DC_1A-20A_n28A | DC_1A_n28A | CA_1A-20A | n28A | 1A | n28A | No |
|  | DC_20A_n28A |  |  | 20A | n28A |  |
| DC_1A-20A_n78A | DC_1A_n78A | CA_1A-20A | n78A | 1A | n78A | No |
|  | DC_20A_n78A |  |  | 20A | n78A |  |
| DC_1A-21A_n28A | DC_1A_n28A | CA_1A-21A | n28A | 1A | n28A | No |
|  | DC_21A_n28A | CA_1A-21A | n28A | 21A | n28A | No |
| DC_1A-21A_n77(2A) | DC_1A_n77A | CA_1A-21A | CA_n77(2A) | 1A | n77A | No |
|  | DC_21A_n77A | CA_1A-21A | CA_n77(2A) | 21A | n77A | No |
| DC_1A-21A_n78A | DC_1A_n78A | CA_1A-21A | n78A | 1A | n78A | No |
|  | DC_21A_n78A |  |  | 21A | n78A |  |
| DC_1A-21A_n78(2A) | DC_1A_n78A | CA_1A-21A | CA_n78(2A) | 1A | n78A | No |
|  | DC_21A_n78A | CA_1A-21A | CA_n78(2A) | 21A | n78A | No |
| DC_1A-21A_n79A | DC_1A_n79A | CA_1A-21A | n79A | 1A | n79A | No |
|  | DC_21A_n79A |  |  | 21A | n79A |  |
| DC_1A-28A_n3A | DC_1A_n3A | CA_1A-28A | n3A | 1A | n3A | No |
|  | DC_28A_n3A | CA_1A-28A | n3A | 28A | n3A | No |
| DC_1A-28A_n5A2 | DC_1A_n5A | CA_1A-28A | n5A | 1A | n5A | No |
|  | DC_28A_n5A | CA_1A-28A | n5A | 28A | n5A | No |
| DC_1A-28A_n78C | DC_1A_n78A | CA_1A-28A | CA_n78C | 1A | n78A | No |
|  | DC_28A_n78A |  |  | 28A | n78A |  |
| DC_1A_n28A-n78A | DC_1A_n28A | 1A | CA_n28A-n78A | 1A | n28A | Yes (NR 2CC) |
|  | DC_1A_n78A |  |  | 1A | n78A | No |
| DC_1A_n28A-n79A | DC_1A_n28A | 1A | CA_n28A-n79A | 1A | n28A | Yes (NR 2CC) |
|  | DC_1A_n79A | 1A | CA_n28A-n79A | 1A | n79A | No |
| DC_1A-41A_n28A | DC_1A_n28A | CA_1A-41A | n28A | 1A | n28A | No |
|  | DC_41A_n28A | CA_1A-41A | n28A | 41A | n28A | No |
| DC_1A-41C_n28A | DC_1A_n28A | CA_1A-41C | n28A | 1A | n28A | No |
|  | DC_41A_n28A | CA_1A-41C | n28A | 41A | n28A | No |
|  | DC_41C_n28A | CA_1A-41C | n28A | CA_41C | n28A | No |
| DC_1A-41A_n41A | DC_1A_n41A | CA_1A-41A | n41A | 1A | n41A | No |
| DC_1A-41A_n77A | DC_1A_n77A | CA_1A-41A | n77A | 1A | n77A | No |
|  | DC_41A_n77A |  |  | 41A | n77A |  |
| DC_1A-42A_n78A | DC_1A_n78A | CA_1A-42A | n78A | 1A | n78A | No |
| DC_1A-42C_n78A | DC_1A_n78A | CA_1A-42C | n78A | 1A | n78A | No |
| DC_1A-42D_n78A | DC_1A_n78A | CA_1A-42D | n78A | 1A | n78A | No |
| DC_1A-42E_n78A | DC_1A_n78A | CA_1A-42E | n78A | 1A | n78A | No |
| DC_1A-42A_n79A | DC_1A_n79A | CA_1A-42A | n79A | 1A | n79A | No |
| DC_1A-42C_n79A | DC_1A_n79A | CA_1A-42C | n79A | 1A | n79A | No |
| DC_1A-42D_n79A | DC_1A_n79A | CA_1A-42D | n79A | 1A | n79A | No |
| DC_1A-42E_n79A | DC_1A_n79A | CA_1A-42E | n79A | 1A | n79A | No |
| DC_1A_n78A-n79A | DC_1A_n78A | 1A | CA_n78A-n79A | 1A | n78A | Yes (NR 2CC) |
|  | DC_1A_n79A |  |  | 1A | n79A | No |
| DC_2A-5A_n2A | DC_2A_n2A | CA_2A-5A | n2A | 2A | n2A | No |
|  | DC_5A_n2A | CA_2A-5A | n2A | 5A | n2A | No |
| DC_2A-5A_n66A | DC_2A_n66A | CA_2A-5A | n66A | 2A | n66A | No |
|  | DC_5A_n66A |  |  | 5A | n66A |  |
| DC_2A-2A-5A_n66A | DC_2A_n66A | CA_2A-2A-5A | n66A | 2A | n66A | No |
|  | DC_5A_n66A | CA_2A-2A-5A | n66A | 5A | n66A | No |
| DC_2A-5A_n77A | DC_2A_n77A | CA_2A-5A | n77A | 2A | n77A | No |
|  | DC_5A_n77A | CA_2A-5A | n77A | 5A | n77A | No |
| DC_2A-5A_n77(2A) | DC_2A_n77A | CA_2A-5A | n77(2A) | 2A | n77A | No |
|  | DC_5A_n77A | CA_2A-5A | n77(2A) | 5A | n77A | No |
| DC_2A-2A-5A_n77A | DC_2A_n77A | CA_2A-2A-5A | n77A | 2A | n77A | No |
|  | DC_5A_n77A | CA_2A-2A-5A | n77A | 5A | n77A | No |
| DC_2A-2A-5A_n77(2A) | DC_2A_n77A | CA_2A-2A-5A | n77(2A) | 2A | n77A | No |
|  | DC_5A_n77A | CA_2A-2A-5A | n77(2A) | 5A | n77A | No |
| DC_2A_n5A-n77A | DC_2A_n5A | 2A | CA_n5A-n77A | 2A | n5A | Yes (NR 2CC) |
|  | DC_2A_n77A | 2A | CA_n5A-n77A | 2A | n77A | No |
| DC_2A-13A_n77A | DC_2A_n77A | CA_2A-13A | n77A | 2A | n77A | No |
|  | DC_13A_n77A | CA_2A-13A | n77A | 13A | n77A | No |
| DC_2A-2A-14A_n66A | DC_2A_n66A | CA_2A-2A-14A | n66A | 2A | n66A | No |
|  | DC_14A_n66A | CA_2A-2A-14A | n66A | 14A | n66A | No |
| DC_2A-2A-66A_n66A | DC_2A_n66A | CA_2A-2A-66A | n66A | 2A | n66A | No |
|  | DC_66A_n66A | CA_2A-2A-66A | n66A | 66A | n66A | No |
| DC_2A-14A_n2A | DC_2A_n2A | CA_2A-14A | n2A | 2A | n2A | No |
|  | DC_14A_n2A | CA_2A-14A | n2A | 14A | n2A | No |
| DC_2A-14A_n66A | DC_2A_n66A | CA_2A-14A | n66A | 2A | n66A | No |
|  | DC_14A_n66A | CA_2A-14A | n66A | 14A | n66A | No |
| DC_2A-14A_n77A | DC_2A_n77A | CA_2A-14A | n77A | 2A | n77A | No |
|  | DC_14A_n77A | CA_2A-14A | n77A | 14A | n77A | No |
| DC_2A-14A_n77(2A) | DC_2A_n77A | CA_2A-14A | n77(2A) | 2A | n77A | No |
|  | DC_14A_n77A | CA_2A-14A | n77(2A) | 14A | n77A | No |
| DC_2A-2A-14A_n77A | DC_2A_n77A | CA_2A-2A-14A | n77A | 2A | n77A | No |
|  | DC_14A_n77A | CA_2A-2A-14A | n77A | 14A | n77A | No |
| DC_2A-2A-14A_n77(2A) | DC_2A_n77A | CA_2A-2A-14A | n77(2A) | 2A | n77A | No |
|  | DC_14A_n77A | CA_2A-2A-14A | n77(2A) | 14A | n77A | No |
| DC_2A-66A_n2A | DC_2A_n2A | CA_2A-66A | n2A | 2A | n2A | No |
|  | DC_66A_n2A | CA_2A-66A | n2A | 66A | n2A | No |
| DC_2A-66A_n5A | DC_2A_n5A | CA_2A-66A | n5A | 2A | n5A | No |
|  | DC_66A_n5A | CA_2A-66A | n5A | 66A | n5A | No |
| DC_2A-2A-66A_n5A | DC_2A_n5A | CA_2A-2A-66A | n5A | 2A | n5A | No |
|  | DC_66A_n5A | CA_2A-2A-66A | n5A | 66A | n5A | No |
| DC_2A-66A-66A_n5A | DC_2A_n5A | CA_2A-66A-66A | n5A | 2A | n5A | No |
|  | DC_66A_n5A | CA_2A-66A-66A | n5A | 66A | n5A | No |
| DC_2A-2A-66A 66A_n5A | DC_2A_n5A | CA_2A-2A-66A-66A | n5A | 2A | n5A | No |
|  | DC_66A_n5A | CA_2A-2A-66A-66A | n5A | 66A | n5A | No |
| DC_2A-66A_n41A | DC_2A_n41A | CA_2A-66A | n41A | 2A | n41A | No |
|  | DC_66A_n41A | CA_2A-66A | n41A | 66A | n41A | No |
| DC_2A-66A_n66A | DC_2A_n66A | CA_2A-66A | n66A | 2A | n66A | No |
|  | DC_66A_n66A | CA_2A-66A | n66A | 66A | n66A | No |
| DC_2A-66A-66A_n66A | DC_2A_n66A | CA_2A-66A-66A | n66A | 2A | n66A | No |
|  | DC_66A_n66A | CA_2A-66A-66A | n66A | 66A | n66A | No |
| DC_2A-2A-66A-66A_n66A | DC_2A_n66A | CA_2A-2A-66A | n66A | 2A | n66A | No |
| DC_2A-66A_n71A | DC_2A_n71A | CA_2A-66A | n71A | 2A | n71A | No |
|  | DC_66A_n71A |  |  | 66A | n71A |  |
| DC_2A-66A_n77A | DC_2A_n77A | CA_2A-66A | n77A | 2A | n77A | No |
|  | DC_66A_n77A | CA_2A-66A | n77A | 66A | n77A | No |
| DC_2A-66A-66A_n2A | DC_66A_n2A | CA_2A-66A-66A | n2A | 66A | n2A | No |
| DC_2A-66A_n77(2A) | DC_2A_n77A | CA_2A-66A | n77(2A) | 2A | n77A | No |
|  | DC_66A_n77A | CA_2A-66A | n77(2A) | 66A | n77A | No |
| DC_2A-2A-66A_n77A | DC_2A_n77A | CA_2A-2A-66A | n77A | 2A | n77A | No |
|  | DC_66A_n77A | CA_2A-2A-66A | n77A | 66A | n77A | No |
| DC_2A-2A-66A_n77(2A) | DC_2A_n77A | CA_2A-2A-66A | n77(2A) | 2A | n77A | No |
|  | DC_66A_n77A | CA_2A-2A-66A | n77(2A) | 66A | n77A | No |
| DC_2A-66A-66A_n77A | DC_2A_n77A | CA_2A-66A-66A | n77A | 2A | n77A | No |
|  | DC_66A_n77A | CA_2A-66A-66A | n77A | 66A | n77A | No |
| DC_2A-66A-66A_n77(2A) | DC_2A_n77A | CA_2A-66A-66A | n77(2A) | 2A | n77A | No |
|  | DC_66A_n77A | CA_2A-66A-66A | n77(2A) | 66A | n77A | No |
| DC_2A-2A-66A-66A_n77A | DC_2A_n77A | CA_2A-2A-66A-66A | n77A | 2A | n77A | No |
|  | DC_66A_n77A | CA_2A-2A-66A-66A | n77A | 66A | n77A | No |
| DC_2A-(n)71AA | DC_2A_n71A | 2A | DC_(n)71AA | 2A | n71A | No |
|  | DC_(n)71AA |  |  | - | DC_(n)71AA |  |
| DC_3A-5A_n78C | DC_3A_n78A | CA_3A-5A | CA_n78C | 3A | n78A | No |
|  | DC_5A_n78A | CA_3A-5A | CA_n78C | 5A | n78A | No |
| DC_3A_n1A-n78A | DC_3A_n1A | 3A | CA_n1A-n78A | 3A | n1A | Yes (NR 2CC) |
|  | DC_3A_n78A | 3A | CA_n1A-n78A | 3A | n78A | No |
| DC_3A_n1A-n79A | DC_3A_n1A | 3A | CA_n1A-n79A | 3A | n1A | Yes (NR 2CC) |
|  | DC_3A_n79A | 3A | CA_n1A-n79A | 3A | n79A | No |
| DC_3A-7A_n1A | DC_3A_n1A | CA_3A-7A | n1A | 3A | n1A | No |
|  | DC_7A_n1A | CA_3A-7A | n1A | 7A | n1A | No |
| DC_3A-7A_n5A | DC_3A_n5A | CA_3A-7A | n5A | 3A | n5A | No |
|  | DC_7A_n5A | CA_3A-7A | n5A | 7A | n5A | No |
| DC_3A-7A_n28A | DC_3A_n28A | CA_3A-7A | n28A | 3A | n28A | No |
|  | DC_7A_n28A |  |  | 7A | n28A |  |
| DC_3A-7A_n78A | DC_3A_n78A | CA_3A-7A | n78A | 1A | n78A | No |
|  | DC_7A_n78A |  |  | 7A | n78A |  |
| DC_3A-8A_n1A | DC_3A_n1A | CA_3A-8A | n1A | 3A | n1A | No |
|  | DC_8A_n1A | CA_3A-8A | n1A | 8A | n1A | No |
| DC_3A-8A_n28A | DC_3A_n28A | CA_3A-8A | n28A | 3A | n28A | No |
|  | DC_8A_n28A | CA_3A-8A | n28A | 8A | n28A | No |
| DC_3A-8A_n77A | DC_3A_n77A | CA_3A-8A | n77A | 3A | n77A | No |
|  | DC_8A_n77A | CA_3A-8A | n77A | 8A | n77A | No |
| DC_3A-8A_n77(2A) | DC_3A_n77A | CA_3A-8A | CA_n77(2A) | 3A | n77A | No |
|  | DC_8A_n77A | CA_3A-8A | CA_n77(2A) | 8A | n77A | No |
| DC_3A-8A_n78A | DC_3A_n78A | CA_3A-8A | n78A | 3A | n78A | No |
|  | DC_8A_n78A |  |  | 8A | n78A |  |
| DC_3A-8A_n78(2A) | DC_3A_n78A | CA_3A-8A | CA_n78(2A) | 3A | n78A | No |
|  | DC_8A_n78A | CA_3A-8A | CA_n78(2A) | 8A | n78A | No |
| DC_3A-18A_n77A | DC_3A_n77A | CA_3A-18A | n77A | 3A | n77A | No |
|  | DC_18A_n77A | CA_3A-18A | n77A | 18A | n77A | No |
| DC_3A-18A_n78A | DC_3A_n78A | CA_3A-18A | n78A | 3A | n78A | No |
|  | DC_18A_n78A | CA_3A-18A | n78A | 18A | n78A | No |
| DC_3A-19A_n1A | DC_3A_n1A | CA_3A-19A | n1A | 3A | n1A | No |
|  | DC_19A_n1A | CA_3A-19A | n1A | 19A | n1A | No |
| DC_3A-19A_n77(2A) | DC_3A_n77A | CA_3A-19A | CA_n77(2A) | 3A | n77A | No |
|  | DC_19A_n77A | CA_3A-19A | CA_n77(2A) | 19A | n77A | No |
| DC_3A-19A_n78A | DC_3A_n78A | CA_3A-19A | n78A | 3A | n78A | No |
|  | DC_19A_n78A |  |  | 19A | n78A |  |
| DC_3A-19A_n78(2A) | DC_3A_n78A | CA_3A-19A | CA_n78(2A) | 3A | n78A | No |
|  | DC_19A_n78A | CA_3A-19A | CA_n78(2A) | 19A | n78A | No |
| DC_3A-19A_n79A | DC_3A_n79A | CA_3A-19A | n79A | 3A | n79A | No |
|  | DC_19A_n79A |  |  | 19A | n79A |  |
| DC_3A-20A_n1A | DC_3A_n1A | CA_3A-20A | n1A | 3A | n1A | No |
|  | DC_20A_n1A | CA_3A-20A | n1A | 20A | n1A | No |
| DC_3A-20A_n28A | DC_3A_n28A | CA_3A-20A | n28A | 3A | n28A | No |
|  | DC_20A_n28A |  |  | 20A | n28A |  |
| DC_3A-21A_n1A | DC_3A_n1A | CA_3A-21A | n1A | 3A | n1A | No |
|  | DC_21A_n1A | CA_3A-21A | n1A | 21A | n1A | No |
| DC_3A-21A_n28A | DC_3A_n28A | CA_3A-21A | n28A | 3A | n28A | No |
|  | DC_21A_n28A | CA_3A-21A | n28A | 21A | n28A | No |
| DC_3A-21A_n77(2A) | DC_3A_n77A | CA_3A-21A | CA_n77(2A) | 3A | n77A | No |
|  | DC_21A_n77A | CA_3A-21A | CA_n77(2A) | 21A | n77A | No |
| DC_3A-21A_n78A | DC_3A_n78A | CA_3A-21A | n78A | 3A | n78A | No |
|  | DC_21A_n78A |  |  | 21A | n78A |  |
| DC_3A-21A_n78(2A) | DC_3A_n78A | CA_3A-21A | CA_n78(2A) | 3A | n78A | No |
|  | DC_21A_n78A | CA_3A-21A | CA_n78(2A) | 21A | n78A | No |
| DC_3A-21A_n79A | DC_3A_n79A | CA_3A-21A | n79A | 3A | n79A | No |
|  | DC_21A_n79A |  |  | 21A | n79A |  |
| DC_3A-28A_n78A | DC_3A_n78A | CA_3A-28A | n78A | 3A | n78A | No |
|  | DC_28A_n78A |  |  | 28A | n78A |  |
| DC_3A_n28A-n78A | DC_3A_n28A | 3A | CA_n28A-n78A | 3A | n28A | Yes (NR 2CC) |
|  | DC_3A_n78A |  |  | 3A | n78A | No |
| DC_3A_n28A-n79A | DC_3A_n28A | 3A | CA_n28A-n79A | 3A | n28A | Yes (NR 2CC) |
|  | DC_3A_n79A | 3A | CA_n28A-n79A | 3A | n79A | No |
| DC_3A-40A_n1A | DC_3A_n1A | CA_3A-40A | n1A | 3A | n1A | No |
|  | DC_40A_n1A | CA_3A-40A | n1A | 40A | n1A | No |
| DC_3A-41A_n28A | DC_3A_n28A | CA_3A-41A | n28A | 3A | n28A | No |
|  | DC_41A_n28A | CA_3A-41A | n28A | 41A | n28A | No |
| DC_3A-41C_n28A | DC_3A_n28A | CA_3A-41C | n28A | 3A | n28A | No |
|  | DC_41A_n28A | CA_3A-41C | n28A | 41A | n28A | No |
|  | DC_41C_n28A | CA_3A-41C | n28A | CA_41C | n28A | No |
| DC_3A-41A_n41A | DC_3A_n41A | CA_3A-41A | n41A | 3A | n41A | No |
| DC_3A-41A_n77A | DC_3A_n77A | CA_3A-41A | n77A | 3A | n77A | No |
|  | DC_41A_n77A | CA_3A-41A | n77A | 41A | n77A | No |
| DC_3A-41A_n77(2A) | DC_3A_n77A | CA_3A-41A | CA_n77(2A) | 3A | n77A | No |
|  | DC_41A_n77A | CA_3A-41A | CA_n77(2A) | 41A | n77A | No |
| DC_3A-41C_n77A | DC_3A_n77A | CA_3A-41C | n77A | 3A | n77A | No |
|  | DC_41A_n77A | CA_3A-41C | n77A | CA_41C | n77A | No |
| DC_3A-42A_n1A | DC_3A_n1A | CA_3A-42A | n1A | 3A | n1A | No |
| DC_3A-42C_n1A | DC_3A_n1A | CA_3A-42C | n1A | 3A | n1A | No |
| DC_3A-42A_n78A | DC_3A_n78A | CA_3A-42A | n78A | 3A | n78A | No |
| DC_3A-42C_n78A | DC_3A_n78A | CA_3A-42C | n78A | 3A | n78A | No |
| DC_3A-42D_n78A | DC_3A_n78A | CA_3A-42D | n78A | 3A | n78A | No |
| DC_3A-42E_n78A | DC_3A_n78A | CA_3A-42E | n78A | 3A | n78A | No |
| DC_3A-42A_n79A | DC_3A_n79A | CA_3A-42A | n79A | 3A | n79A | No |
| DC_3A-42C_n79A | DC_3A_n79A | CA_3A-42C | n79A | 3A | n79A | No |
| DC_3A-42D_n79A | DC_3A_n79A | CA_3A-42D | n79A | 3A | n79A | No |
| DC_3A-42E_n79A | DC_3A_n79A | CA_3A-42E | n79A | 3A | n79A | No |
| DC_3A_n78A-n79A | DC_3A_n78A | 3A | CA_n78A-n79A | 3A | n78A | Yes (NR 2CC) |
|  | DC_3A_n79A | 3A | CA_n78A-n79A | 3A | n79A | No |
| DC_3C-8A_n77A | DC_3A_n77A | CA_3C-8A | n77A | 3A | n77A | No |
|  | DC_8A_n77A | CA_3C-8A | n77A | 8A | n77A | No |
| DC_3C-8A_n77(2A) | DC_3A_n77A | CA_3A-8A | CA_n77(2A) | 3A | n77A | No |
|  | DC_8A_n77A | CA_3A-8A | CA_n77(2A) | 8A | n77A | No |
| DC_5A-66A_n2A | DC_5A_n2A | CA_5A-66A | n2A | 5A | n2A | No |
|  | DC_66A_n2A | CA_5A-66A | n2A | 66A | n2A | No |
| DC_5A-66A-66A_n2A | DC_5A_n2A | CA_5A-66A-66A | n2A | 5A | n2A | No |
| r | DC_66A_n2A | CA_5A-66A-66A | n2A | 66A | n2A | No |
| DC_5A-66A_n66A | DC_5A_n66A | CA_5A-66A | n66A | 5A | n66A | No |
| DC_5A-66A_n77A | DC_5A_n77A | CA_5A-66A | n77A | 5A | n77A | No |
|  | DC_66A_n77A | CA_5A-66A | n77A | 66A | n77A | No |
| DC_5A-66A_n77(2A) | DC_5A_n77A | CA_5A-66A | CA_n77(2A) | 5A | n77A | No |
|  | DC_66A_n77A | CA_5A-66A | CA_n77(2A) | 66A | n77A | No |
| DC_5A-66A-66A_n77A | DC_5A_n77A | CA_5A-66A-66A | n77A | 5A | n77A | No |
|  | DC_66A_n77A | CA_5A-66A-66A | n77A | 66A | n77A | No |
| DC_5A-66A-66A_n77(2A) | DC_5A_n77A | CA_5A-66A-66A | CA_n77(2A) | 5A | n77A | No |
|  | DC_66A_n77A | CA_5A-66A-66A | CA_n77(2A) | 66A | n77A | No |
| DC_5A-7A_n78A | DC_5A_n78A | CA_5A-7A | n78A | 5A | n78A | No |
|  | DC_7A_n78A |  |  | 7A | n78A |  |
| DC_7A-8A_n1A | DC_7A_n1A | CA_7A-8A | n1A | 7A | n1A | No |
|  | DC_8A_n1A | CA_7A-8A | n1A | 8A | n1A | No |
| DC_7A-8A_n3A | DC_7A_n3A | CA_7A-8A | n3A | 7A | n3A | No |
|  | DC_8A_n3A | CA_7A-8A | n3A | 8A | n3A | No |
| DC_7A-20A_n1A | DC_7A_n1A | CA_7A-20A | n1A | 7A | n1A | No |
|  | DC_20A_n1A | CA_7A-20A | n1A | 20A | n1A | No |
| DC_7A-20A_n3A | DC_7A_n3A | CA_7A-20A | n3A | 7A | n3A | No |
|  | DC_20A_n3A | CA_7A-20A | n3A | 20A | n3A | No |
| DC_7A-20A_n8A | DC_7A_n8A | CA_7A-20A | n8A | 7A | n8A | No |
|  | DC_20A_n8A | CA_7A-20A | n8A | 20A | n8A | No |
| DC_7A-20A_n28A | DC_7A_n28A | CA_7A-20A | n28A | 7A | n28A | No |
|  | DC_20A_n28A |  |  | 20A | n28A |  |
| DC_7A-20A_n78A | DC_7A_n78A | CA_7A-20A | n78A | 7A | n78A | No |
|  | DC_20A_n78A |  |  | 20A | n78A |  |
| DC_7A-28A_n3A | DC_7A_n3A | CA_7A-28A | n3A | 7A | n3A | No |
|  | DC_28A_n3A | CA_7A-28A | n3A | 28A | n3A | No |
| DC_7A-28A_n5A2 | DC_7A_n5A | CA_7A-28A | n5A | 7A | n5A | No |
|  | DC_28A_n5A | CA_7A-28A | n5A | 28A | n5A | No |
| DC_7A_n5A-n78A | DC_7A_n5A | 7A | CA_n5A-n78A | 7A | n5A | Yes (NR 2CC) |
|  | DC_7A_n78A | 7A | CA_n5A-n78A | 7A | n78A | No |
| DC_7A_n28A-n78A | DC_7A_n28A | 7A | CA_n28A-n78A | 7A | n28A | Yes (NR 2CC) |
|  | DC_7A_n78A |  |  | 7A | n78A | No |
| DC_7C_n5A-n78A | DC_7A_n5A | CA_7C | CA_n5A-n78A | 7A | n5A | No |
|  | DC_7A_n78A | CA_7C | CA_n5A-n78A | 7A | n78A | No |
|  | DC_7C_n5A | CA_7C | CA_n5A-n78A | CA_7C | n5A | No |
|  | DC_7C_n78A | CA_7C | CA_n5A-n78A | CA_7C | n78A | No |
| DC_7C_n28A-n78A | DC_7A_n28A | CA_7C | CA_n28A-n78A | 7A | n28A | No |
|  | DC_7A_n78A | CA_7C | CA_n28A-n78A | 7A | n78A | No |
|  | DC_7C_n28A | CA_7C | CA_n28A-n78A | CA_7C | n28A | No |
|  | DC_7C_n78A | CA_7C | CA_n28A-n78A | CA_7C | n78A | No |
| DC_13A_n2A-n77A | DC_13A_n2A | 13A | CA_n2A-n77A | 13A | n2A | Yes (NR 2CC) |
|  | DC_13A_n77A | 13A | CA_n2A-n77A | 13A | n77A | No |
| DC_13A-66A_n2A | DC_13A_n2A | CA_13A-66A | n2A | 13A | n2A | No |
|  | DC_66A_n2A | CA_13A-66A | n2A | 66A | n2A | No |
| DC_13A-66A_n77A | DC_13A_n77A | CA_13A-66A | n77A | 13A | n77A | No |
|  | DC_66A_n77A | CA_13A-66A | n77A | 66A | n77A | No |
| DC_14A-66A_n2A | DC_14A_n2A | CA_14A-66A | n2A | 14A | n2A | No |
|  | DC_66A_n2A | CA_14A-66A | n2A | 66A | n2A | No |
| DC_14A-66A_n66A | DC_14A_n66A | CA_14A-66A | n66A | 14A | n66A | No |
|  | DC_66A_n66A | CA_14A-66A | n66A | 66A | n66A | No |
| DC_14A-66A-66A_n2A | DC_14A_n2A | CA_14A-66A-66A | n2A | 14A | n2A | No |
|  | DC_66A_n2A | CA_14A-66A-66A | n2A | 66A | n2A | No |
| DC_14A-66A_n77A | DC_14A_n77A | CA_14A-66A | n77A | 14A | n77A | No |
|  | DC_66A_n77A | CA_14A-66A | n77A | 66A | n77A | No |
| DC_14A-66A-66A_n77A | DC_14A_n77A | CA_14A-66A-66A | n77A | 14A | n77A | No |
|  | DC_66A_n77A | CA_14A-66A-66A | n77A | 66A | n77A | No |
| DC_14A-66A_n77(2A) | DC_14A_n77A | CA_14A-66A | n77(2A) | 14A | n77A | No |
|  | DC_66A_n77A | CA_14A-66A | n77(2A) | 66A | n77A | No |
| DC_14A-66A-66A_n77(2A) | DC_14A_n77A | CA_14A-66A-66A | n77(2A) | 14A | n77A | No |
|  | DC_66A_n77A | CA_14A-66A-66A | n77(2A) | 66A | n77A | No |
| DC_18A-41C_n3A | DC_18A_n3A | CA_18A-41C | n3A | 18A | n3A | No |
|  | DC_41A_n3A | CA_18A-41C | n3A | 41A | n3A | No |
|  | DC_41C_n3A | CA_18A-41C | n3A | CA_41C | n3A | No |
| DC_18A-41A_n77A | DC_18A_n77A | CA_18A-41A | n77A | 18A | n77A | No |
|  | DC_41A_n77A | CA_18A-41A | n77A | 41A | n77A | No |
| DC_18A-41C_n77A | DC_18A_n77A | CA_18A-41C | n77A | 18A | n77A | No |
|  | DC_41A_n77A | CA_18A-41C | n77A | 41A | n77A | No |
|  | DC_41C_ n77A | CA_18A-41C | n77A | CA_41C | n77A | No |
| DC_18A-41A_n78A | DC_18A_n78A | CA_18A-41A | n78A | 18A | n78A | No |
|  | DC_41A_n78A | CA_18A-41A | n78A | 41A | n78A | No |
| DC_18A-41C_n78A | DC_18A_n78A | CA_18A-41C | n78A | 18A | n78A | No |
|  | DC_41A_n78A | CA_18A-41C | n78A | 41A | n78A | No |
|  | DC_41C_n78A | CA_18A-41C | n78A | CA_41C | n78A | No |
| DC_19A_n1A-n78A | DC_19A_n1A | 19A | CA_n1A-n78A | 19A | n1A | Yes (NR 2CC) |
|  | DC_19A_n78A | 19A | CA_n1A-n78A | 19A | n78A | No |
| DC_19A_n1A-n79A | DC_19A_n1A | 19A | CA_n1A-n79A | 19A | n1A | Yes (NR 2CC) |
|  | DC_19A_n79A | 19A | CA_n1A-n79A | 19A | n79A | No |
| DC_19A-21A_n1A | DC_19A_n1A | CA_19A-21A | n1A | 19A | n1A | No |
|  | DC_21A_n1A | CA_19A-21A | n1A | 21A | n1A | No |
| DC_19A-21A_n77(2A) | DC_19A_n77A | CA_19A-21A | CA_n77(2A) | 19A | n77A | No |
|  | DC_21A_n77A | CA_19A-21A | CA_n77(2A) | 21A | n77A | No |
| DC_19A-21A_n78A | DC_19A_n78A | CA_19A-21A | n78A | 19A | n78A | No |
|  | DC_21A_n78A |  |  | 21A | n78A |  |
| DC_19A-21A_n78(2A) | DC_19A_n78A | CA_19A-21A | CA_n78(2A) | 19A | n78A | No |
|  | DC_21A_n78A | CA_19A-21A | CA_n78(2A) | 21A | n78A | No |
| DC_19A-21A_n79A | DC_19A_n79A | CA_19A-21A | n79A | 19A | n79A | No |
|  | DC_21A_n79A |  |  | 21A | n79A |  |
| DC_19A-42A_n1A | DC_19A_n1A | CA_19A-42A | n1A | 19A | n1A | No |
| DC_19A-42A_n78A | DC_19A_n78A | CA_19A-42A | n78A | 19A | n78A | No |
| DC_19A-42A_n79A | DC_19A_n79A | CA_19A-42A | n79A | 19A | n79A | No |
| DC_19A-42C_n1A | DC_19A_n1A | CA_19A-42C | n1A | 19A | n1A | No |
| DC_19A-42C_n78A | DC_19A_n78A | CA_19A-42C | n78A | 19A | n78A | No |
| DC_19A-42C_n79A | DC_19A_n79A | CA_19A-42C | n79A | 19A | n79A | No |
| DC_19A_n78A-n79A | DC_19A_n78A | 19A | CA_n78A-n79A | 19A | n78A | Yes (NR 2CC) |
|  | DC_19A_n79A |  |  | 19A | n79A | No |
| DC_20A_n28A-n78A | DC_20A_n28A | 20A | CA_n28A-n78A | 20A | n28A | Yes (NR 2CC) |
|  | DC_20A_n78A |  |  | 20A | n78A | No |
| DC_21A_n1A-n78A | DC_21A_n1A | 21A | CA_n1A-n78A | 21A | n1A | Yes (NR 2CC) |
|  | DC_21A_n78A | 21A | CA_n1A-n78A | 21A | n78A | No |
| DC_21A_n1A-n79A | DC_21A_n1A | 21A | CA_n1A-n79A | 21A | n1A | Yes (NR 2CC) |
|  | DC_21A_n79A | 21A | CA_n1A-n79A | 21A | n79A | No |
| DC_21A_n28A-n77A | DC_21A_n28A | 21A | CA_n28A-n77A | 21A | n28A | Yes (NR 2CC) |
|  | DC_21A_n77A | 21A | CA_n28A-n77A | 21A | n77A | No |
| DC_21A_n28A-n78A | DC_21A_n28A | 21A | CA_n28A-n78A | 21A | n28A | Yes (NR 2CC) |
|  | DC_21A_n78A | 21A | CA_n28A-n78A | 21A | n78A | No |
| DC_21A_n28A-n79A | DC_21A_n28A | 21A | CA_n28A-n79A | 21A | n28A | Yes (NR 2CC) |
|  | DC_21A_n79A | 21A | CA_n28A-n79A | 21A | n79A | No |
| DC_21A-42A_n1A | DC_21A_n1A | CA_21A-42A | n1A | 21A | n1A | No |
| DC_21A-42C_n1A | DC_21A_n1A | CA_21A-42C | n1A | 21A | n1A | No |
| DC_21A-42A_n78A | DC_21A_n78A | CA_21A-42A | n78A | 21A | n78A | No |
| DC_21A-42C_n78A | DC_21A_n78A | CA_21A-42C | n78A | 21A | n78A | No |
| DC_21A-42A_n79A | DC_21A_n79A | CA_21A-42A | n79A | 21A | n79A | No |
| DC_21A-42C_n79A | DC_21A_n79A | CA_21A-42C | n79A | 21A | n79A | No |
| DC_21A_n78A-n79A | DC_21A_n78A | 21A | CA_n78A-n79A | 21A | n78A | Yes (NR 2CC) |
|  | DC_21A_n79A |  |  | 21A | n79A | No |
| DC_28A_n7A-n78A | DC_28A_n7A | 28A | CA_n7A-n78A | 28A | n7A | Yes (NR 2CC) |
|  | DC_28A_n7A | 28A | CA_n7A-n78A | 28A | n78A | No |
| DC_66A_n2A-n77A | DC_66A_n2A | 66A | CA_n2A-n77A | 66A | n2A | Yes (NR 2CC) |
|  | DC_66A_n77A | 66A | CA_n2A-n77A | 66A | n77A | No |
| DC_66A_n5A-n77A | DC_66A_n5A | 66A | CA_n5A-n77A | 66A | n5A | Yes (NR 2CC) |
|  | DC_66A_n77A | 66A | CA_n5A-n77A | 66A | n77A | No |
| DC_66A_(n)71AA | DC_66A_n71A | 66A | DC_(n)71AA | 66A | n71A | No |
|  | DC_(n)71AA |  |  | - | DC_(n)71AA |  |
| Note 1: Protocol testing is limited to EN-DC configurations with 1 CC E-UTRA and 1CC or 2CC NR configurations. Note 2: The frequency range in band 28 is restricted for this band combination to 703-733 MHz for the UL and 758 – 788 MHz for the DL. |  |  |  |  |  |  |

4.3.1.4.1.4	Inter-band EN-DC configurations within FR1 (four bands)

Table 4.3.1.4.1.4-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC Configuration | E-UTRA downlink configuration | NR downlink configuration | E-UTRA uplink configuration | NR uplink configuration | Applicable for protocol testing (Note 1) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_1A-3A-7A_n28A | DC_1A_n28A | CA_1A-3A-7A | n28A | 1A | n28A | No |
|  | DC_3A_n28A |  |  | 3A | n28A |  |
|  | DC_7A_n28A |  |  | 7A | n28A |  |
| DC_1A-3A-7A_n78A | DC_1A_n78A | CA_1A-3A-7A | n78A | 1A | n78A | No |
|  | DC_3A_n78A |  |  | 3A | n78A |  |
|  | DC_7A_n78A |  |  | 7A | n78A |  |
| DC_1A-3A-8A_n77A | DC_1A_n77A | CA_1A-3A-8A | n77A | 1A | n77A | No |
|  | DC_3A_n77A | CA_1A-3A-8A | n77A | 3A | n77A | No |
|  | DC_8A_n77A | CA_1A-3A-8A | n77A | 8A | n77A | No |
| DC_1A-3A-8A_n77(2A) | DC_1A_n77A | CA_1A-3A-8A | CA_n77(2A) | 1A | n77A | No |
|  | DC_3A_n77A | CA_1A-3A-8A | CA_n77(2A) | 3A | n77A | No |
|  | DC_8A_n77A | CA_1A-3A-8A | CA_n77(2A) | 8A | n77A | No |
| DC_1A-3A-8A_n78A | DC_1A_n78A | CA_1A-3A-8A | n78A | 1A | n78A | No |
|  | DC_3A_n78A |  |  | 3A | n78A |  |
|  | DC_8A_n78A |  |  | 8A | n78A |  |
| DC_1A-3A-19A_n77(2A) | DC_1A_n77A | CA_1A-3A-19A | CA_n77A(2A) | 1A | n77A | No |
|  | DC_3A_n77A | CA_1A-3A-19A | CA_n77A(2A) | 3A | n77A | No |
|  | DC_19A_n77A | CA_1A-3A-19A | CA_n77A(2A) | 19A | n77A | No |
| DC_1A-3A-19A_n78A | DC_1A_n78A | CA_1A-3A-19A | n78A | 1A | n78A | No |
|  | DC_3A_n78A |  |  | 3A | n78A |  |
|  | DC_19A_n78A |  |  | 19A | n78A |  |
| DC_1A-3A-19A_n78(2A) | DC_1A_n78A | CA_1A-3A-19A | CA_n78A(2A) | 1A | n78A | No |
|  | DC_3A_n78A | CA_1A-3A-19A | CA_n78A(2A) | 3A | n78A | No |
|  | DC_19A_n78A | CA_1A-3A-19A | CA_n78A(2A) | 19A | n78A | No |
| DC_1A-3A-19A_n79A | DC_1A_n79A | CA_1A-3A-19A | n79A | 1A | n79A | No |
|  | DC_3A_n79A |  |  | 3A | n79A |  |
|  | DC_19A_n79A |  |  | 19A | n79A |  |
| DC_1A-3A-20A_n28A | DC_1A_n28A | CA_1A-3A-20A | n28A | 1A | n28A | No |
|  | DC_3A_n28A |  |  | 3A | n28A |  |
|  | DC_20A_n28A |  |  | 20A | n28A |  |
| DC_1A-3A-20A_n78A | DC_1A_n78A | CA_1A-3A-20A | n78A | 1A | n78A | No |
|  | DC_3A_n78A |  |  | 3A | n78A |  |
|  | DC_20A_n78A |  |  | 20A | n78A |  |
| DC_1A-3A-21A_n77(2A) | DC_1A_n77A | CA_1A-3A-21A | CA_n77A(2A) | 1A | n77A | No |
|  | DC_3A_n77A | CA_1A-3A-21A | CA_n77A(2A) | 3A | n77A | No |
|  | DC_21A_n77A | CA_1A-3A-21A | CA_n77A(2A) | 21A | n77A | No |
| DC_1A-3A-21A_n78A | DC_1A_n78A | CA_1A-3A-21A | n78A | 1A | n78A | No |
|  | DC_3A_n78A |  |  | 3A | n78A |  |
|  | DC_21A_n78A |  |  | 21A | n78A |  |
| DC_1A-3A-21A_n78(2A) | DC_1A_n78A | CA_1A-3A-21A | CA_n78A(2A) | 1A | n78A | No |
|  | DC_3A_n78A | CA_1A-3A-21A | CA_n78A(2A) | 3A | n78A | No |
|  | DC_21A_n78A | CA_1A-3A-21A | CA_n78A(2A) | 21A | n78A | No |
| DC_1A-3A-21A_n79A | DC_1A_n79A | CA_1A-3A-21A | n79A | 1A | n79A | No |
|  | DC_3A_n79A |  |  | 3A | n79A |  |
|  | DC_21A_n79A |  |  | 21A | n79A |  |
| DC_1A-3A-28A_n78A | DC_1A_n78A | CA_1A-3A-28A | n78A | 1A | n78A | No |
|  | DC_3A_n78A |  |  | 3A | n78A |  |
|  | DC_28A_n78A |  |  | 28A | n78A |  |
| DC_1A-3A_n28A-n78A | DC_1A_n28A | CA_1A-3A | CA_n28A-n78A | 1A | n28A | No |
|  | DC_1A_n78A |  |  | 1A | n78A |  |
|  | DC_3A_n28A |  |  | 3A | n28A |  |
|  | DC_3A_n78A |  |  | 3A | n78A |  |
| DC_1A-3A-42A_n78A | DC_1A_n78A | CA_1A-3A-42A | n78A | 1A | n78A | No |
|  | DC_3A_n78A |  |  | 3A | n78A |  |
| DC_1A-3A-42C_n78A | DC_1A_n78A | CA_1A-3A-42C | n78A | 1A | n78A | No |
|  | DC_3A_n78A |  |  | 3A | n78A |  |
| DC_1A-3A-42D_n78A | DC_1A_n78A | CA_1A-3A-42D | n78A | 1A | n78A | No |
|  | DC_3A_n78A | CA_1A-3A-42D | n78A | 3A | n78A | No |
| DC_1A-3A-42A_n79A | DC_1A_n79A | CA_1A-3A-42A | n79A | 1A | n79A | No |
|  | DC_3A_n79A |  |  | 3A | n79A |  |
| DC_1A-3A-42C_n79A | DC_1A_n79A | CA_1A-3A-42C | n79A | 1A | n79A | No |
|  | DC_3A_n79A |  |  | 3A | n79A |  |
| DC_1A-3A-42D_n79A | DC_1A_n79A | CA_1A-3A-42D | n79A | 1A | n79A | No |
|  | DC_3A_n79A | CA_1A-3A-42D | n79A | 3A | n79A | No |
| DC_1A-3C-8A_n77A | DC_1A_n77A | CA_1A-3C-8A | n77A | 1A | n77A | No |
|  | DC_3A_n77A | CA_1A-3C-8A | n77A | 3A | n77A | No |
|  | DC_8A_n77A | CA_1A-3C-8A | n77A | 8A | n77A | No |
| DC_1A-3C-8A_n77(2A) | DC_1A_n77A | CA_1A-3C-8A | CA_n77(2A) | 1A | n77A | No |
|  | DC_3A_n77A | CA_1A-3C-8A | CA_n77(2A) | 3A | n77A | No |
|  | DC_8A_n77A | CA_1A-3C-8A | CA_n77(2A) | 8A | n77A | No |
| DC_1A-7A-20A_n28A | DC_1A_n28A | CA_1A-7A-20A | n28A | 1A | n28A | No |
|  | DC_7A_n28A |  |  | 7A | n28A |  |
|  | DC_20A_n28A |  |  | 20A | n28A |  |
| DC_1A-7A-20A_n78A | DC_1A_n78A | CA_1A-7A-20A | n78A | 1A | n78A | No |
|  | DC_7A_n78A |  |  | 7A | n78A |  |
|  | DC_20A_n78A |  |  | 20A | n78A |  |
| DC_1A-7A-28A_n78A | DC_1A_n78A | CA_1A-7A-28A | n78A | 1A | n78A | No |
|  | DC_7A_n78A | CA_1A-7A-28A | n78A | 7A | n78A | No |
|  | DC_28A_n78A | CA_1A-7A-28A | n78A | 28A | n78A | No |
| DC_1A-7A_n28A-n78A | DC_1A_n28A | CA_1A-7A | CA_n28A-n78A | 1A | n28A | No |
|  | DC_1A_n78A |  |  | 1A | n78A |  |
|  | DC_7A_n28A |  |  | 7A | n28A |  |
|  | DC_7A_n78A |  |  | 7A | n78A |  |
| DC_1A-19A-21A_n77(2A) | DC_1A_n77A | CA_1A-19A-21A | CA_n77(2A) | 1A | n77A | No |
|  | DC_19A_n77A | CA_1A-19A-21A | CA_n77(2A) | 19A | n77A | No |
|  | DC_21A_n77(2A) | CA_1A-19A-21A | CA_n77(2A) | 21A | n77A | No |
| DC_1A-19A-21A_n78A | DC_1A_n78A | CA_1A-19A-21A | n78A | 1A | n78A | No |
|  | DC_19A_n78A |  |  | 19A | n78A |  |
|  | DC_21A_n78A |  |  | 21A | n78A |  |
| DC_1A-19A-21A_n78(2A) | DC_1A_n78A | CA_1A-19A-21A | CA_n78(2A) | 1A | n78A | No |
|  | DC_19A_n78A | CA_1A-19A-21A | CA_n78(2A) | 19A | n78A | No |
|  | DC_21A_n78(2A) | CA_1A-19A-21A | CA_n78(2A) | 21A | n78A | No |
| DC_1A-19A-21A_n79A | DC_1A_n79A | CA_1A-19A-21A | n79A | 1A | n79A | No |
|  | DC_19A_n79A |  |  | 19A | n79A |  |
|  | DC_21A_n79A |  |  | 21A | n79A |  |
| DC_1A-19A-42A_n78A | DC_1A_n78A | CA_1A-19A-42A | n78A | 1A | n78A | No |
|  | DC_19A_n78A |  |  | 19A | n78A |  |
| DC_1A-19A-42C_n78A | DC_1A_n78A | CA_1A-19A-42C | n78A | 1A | n78A | No |
|  | DC_19A_n78A |  |  | 19A | n78A |  |
| DC_1A-19A-42A_n79A | DC_1A_n79A | CA_1A-19A-42A | n79A | 1A | n79A | No |
|  | DC_19A_n79A |  |  | 19A | n79A |  |
| DC_1A-19A-42C_n79A | DC_1A_n79A | CA_1A-19A-42C | n79A | 1A | n79A | No |
|  | DC_19A_n79A |  |  | 19A | n79A |  |
| DC_1A-20A_n28A-n78A | DC_1A_n28A | CA_1A-20A | CA_n28A-n78A | 1A | n28A | No |
|  | DC_1A_n78A |  |  | 1A | n78A |  |
|  | DC_20A_n28A |  |  | 20A | n28A |  |
|  | DC_20A_n78A |  |  | 20A | n78A |  |
| DC_1A-21A-42A_n78A | DC_1A_n78A | CA_1A-21A-42A | n78A | 1A | n78A | No |
|  | DC_21A_n78A |  |  | 21A | n78A |  |
| DC_1A-21A-42C_n78A | DC_1A_n78A | CA_1A-21A-42C | n78A | 1A | n78A | No |
|  | DC_21A_n78A |  |  | 21A | n78A |  |
| DC_1A-21A-42A_n79A | DC_1A_n79A | CA_1A-21A-42A | n79A | 1A | n79A | No |
|  | DC_21A_n79A |  |  | 21A | n79A |  |
| DC_1A-21A-42C_n79A | DC_1A_n79A | CA_1A-21A-42C | n79A | 1A | n79A | No |
|  | DC_21A_n79A |  |  | 21A | n79A |  |
| DC_2A-2A-14A-66A_n66A | DC_2A_n66A | CA_2A-2A-14A-66A | n66A | 2A | n66A | No |
|  | DC_14A_n66A | CA_2A-2A-14A-66A | n66A | 14A | n66A | No |
|  | DC_66A_n66A | CA_2A-2A-14A-66A | n66A | 66A | n66A | No |
| DC_2A-7A-7A-13A_n66A | DC_2A_n66A | CA_2A-7A-7A-13A | n66A | 2A | n66A | No |
|  | DC_7A_n66A | CA_2A-7A-7A-13A | n66A | 7A | n66A | No |
|  | DC_13A_n66A | CA_2A-7A-7A-13A | n66A | 13A | n66A | No |
| DC_2A-7A-7A-66A_n66A | DC_2A_n66A | CA_2A-7A-7A-66A | n66A | 2A | n66A | No |
|  | DC_7A_n66A | CA_2A-7A-7A-66A | n66A | 7A | n66A | No |
|  | DC_66A_n66A | CA_2A-7A-7A-66A | n66A | 66A | n66A | No |
| DC_2A-7A-7A-66A_n78A | DC_2A_n78A | CA_2A-7A-7A-66A | n78A | 2A | n78A | No |
|  | DC_7A_n78A | CA_2A-7A-7A-66A | n78A | 7A | n78A | No |
|  | DC_66A_n78A | CA_2A-7A-7A-66A | n78A | 66A | n78A | No |
| DC_2A-7A-13A_n66A | DC_2A_n66A | CA_2A-7A-13A | n66A | 2A | n66A | No |
|  | DC_7A_n66A | CA_2A-7A-13A | n66A | 7A | n66A | No |
|  | DC_13A_n66A | CA_2A-7A-13A | n66A | 13A | n66A | No |
| DC_2A-7A-66A_n66A | DC_2A_n66A | CA_2A-7A-66A | n66A | 2A | n66A | No |
|  | DC_7A_n66A | CA_2A-7A-66A | n66A | 7A | n66A | No |
|  | DC_66A_n66A | CA_2A-7A-66A | n66A | 66A | n66A | No |
| DC_2A-7C-13A_n66A | DC_2A_n66A | CA_2A-7C-13A | n66A | 2A | n66A | No |
|  | DC_7A_n66A | CA_2A-7C-13A | n66A | 7A | n66A | No |
|  | DC_13A_n66A | CA_2A-7C-13A | n66A | 13A | n66A | No |
| DC_2A-7C-66A_n66A | DC_2A_n66A | CA_2A-7C-66A | n66A | 2A | n66A | No |
|  | DC_7A_n66A | CA_2A-7C-66A | n66A | 7A | n66A | No |
|  | DC_66A_n66A | CA_2A-7C-66A | n66A | 66A | n66A | No |
| DC_2A-7C-66A_n78A | DC_2A_n78A | CA_2A-7C-66A | n78A | 2A | n78A | No |
|  | DC_7A_n78A | CA_2A-7C-66A | n78A | 7A | n78A | No |
|  | DC_66A_n78A | CA_2A-7C-66A | n78A | 66A | n78A | No |
| DC_2A-13A-66A_n77A | DC_2A_n77A | CA_2A-13A-66A | n77A | 2A | n77A | No |
|  | DC_13A_n77A | CA_2A-13A-66A | n77A | 13A | n77A | No |
|  | DC_66A_n77A | CA_2A-13A-66A | n77A | 66A | n77A | No |
| DC_2A-14A-66A_n2A | DC_2A_n2A | CA_2A-14A-66A | n2A | 2A | n2A | No |
|  | DC_14A_n2A | CA_2A-14A-66A | n2A | 14A | n2A | No |
|  | DC_66A_n2A | CA_2A-14A-66A | n2A | 66A | n2A | No |
| DC_2A-14A-66A-66A_n2A | DC_2A_n2A | CA_2A-14A-66A-66A | n2A | 2A | n2A | No |
|  | DC_14A_n2A | CA_2A-14A-66A-66A | n2A | 14A | n2A | No |
|  | DC_66A_n2A | CA_2A-14A-66A-66A | n2A | 66A | n2A | No |
| DC_2A-14A-66A_n66A | DC_2A_n66A | CA_2A-14A-66A | n66A | 2A | n66A | No |
|  | DC_14A_n66A | CA_2A-14A-66A | n66A | 14A | n66A | No |
|  | DC_66A_n66A | CA_2A-14A-66A | n66A | 66A | n66A | No |
| DC_2A-66A-(n)71AA | DC_2A_n71A | CA_2A-66A | DC_(n)71AA | 2A | n71A | No |
|  | DC_66A_n71A |  |  | 66A | n71A |  |
|  | DC_(n)71AA |  |  | - | (n)71AA |  |
|  | DC_2A_n5A | CA_2A-66A | CA_n5A-n77A | 2A | n5A | No |
| DC_2A-66A_n5A-n77A | DC_2A_n77A | CA_2A-66A | CA_n5A-n77A | 2A | n77A | No |
|  | DC_66A_n5A | CA_2A-66A | CA_n5A-n77A | 66A | n5A | No |
|  | DC_66A_n77A | CA_2A-66A | CA_n5A-n77A | 66A | n77A | No |
| DC_3A-7A-20A_n1A | DC_3A_n1A | CA_3A-7A-20A | n1A | 3A | n1A | No |
|  | DC_7A_n1A | CA_3A-7A-20A | n1A | 7A | n1A | No |
|  | DC_20A_n1A | CA_3A-7A-20A | n1A | 20A | n1A | No |
| DC_3A-7A-20A_n8A | DC_3A_n8A | CA_3A-7A-20A | n8A | 3A | n8A | No |
|  | DC_7A_n8A | CA_3A-7A-20A | n8A | 7A | n8A | No |
|  | DC_20A_n8A | CA_3A-7A-20A | n8A | 20A | n8A | No |
| DC_3A-7A-20A_n28A | DC_3A_n28A | CA_3A-7A-20A | n28A | 3A | n28A | No |
|  | DC_7A_n28A |  |  | 7A | n28A |  |
|  | DC_20A_n28A |  |  | 20A | n28A |  |
| DC_3A-7A-20A_n78A | DC_3A_n78A | CA_3A-7A-20A | n78A | 3A | n78A | No |
|  | DC_7A_n78A | CA_3A-7A-20A | n78A | 7A | n78A | No |
|  | DC_20A_n78A | CA_3A-7A-20A | n78A | 20A | n78A | No |
| DC_3A-7A-28A_n78A | DC_3A_n78A | CA_3A-7A-28A | n78A | 3A | n78A | No |
|  | DC_7A_n78A |  |  | 7A | n78A |  |
|  | DC_28A_n78A |  |  | 28A | n78A |  |
| DC_3A-7A_n28A-n78A | DC_3A_n28A | CA_3A-7A | CA_n28A-n78A | 3A | n28A | No |
|  | DC_3A_n78A |  |  | 3A | n78A |  |
|  | DC_7A_n28A |  |  | 7A | n28A |  |
|  | DC_7A_n78A |  |  | 7A | n78A |  |
| DC_3A-19A_n1A-n78A | DC_3A_n1A | CA_3A-19A | CA_n1A-n78A | 3A | n1A | No |
|  | DC_3A_n78A | CA_3A-19A | CA_n1A-n78A | 3A | n78A | No |
|  | DC_19A_n1A | CA_3A-19A | CA_n1A-n78A | 19A | n1A | No |
|  | DC_19A_n78A | CA_3A-19A | CA_n1A-n78A | 19A | n78A | No |
| DC_3A-19A_n1A-n79A | DC_3A_n1A | CA_3A-19A | CA_n1A-n79A | 3A | n1A | No |
|  | DC_3A_n79A | CA_3A-19A | CA_n1A-n79A | 3A | n79A | No |
|  | DC_19A_n1A | CA_3A-19A | CA_n1A-n79A | 19A | n1A | No |
|  | DC_19A_n79A | CA_3A-19A | CA_n1A-n79A | 19A | n79A | No |
| DC_3A-19A-21A_n78A | DC_3A_n78A | CA_3A-19A-21A | n78A | 3A | n78A | No |
|  | DC_19A_n78A |  |  | 19A | n78A |  |
|  | DC_21A_n78A |  |  | 21A | n78A |  |
| DC_3A-19A-21A_n79A | DC_3A_n79A | CA_3A-19A-21A | n79A | 3A | n79A | No |
|  | DC_19A_n79A |  |  | 19A | n79A |  |
|  | DC_21A_n79A |  |  | 21A | n79A |  |
| DC_3A-19A-42A_n1A | DC_3A_n1A | CA_3A-19A-42A | n1A | 3A | n1A | No |
|  | DC_19A_n1A | CA_3A-19A-42A | n1A | 19A | n1A | No |
| DC_3A-19A-42C_n1A | DC_3A_n1A | CA_3A-19A-42C | n1A | 3A | n1A | No |
|  | DC_19A_n1A | CA_3A-19A-42C | n1A | 19A | n1A | No |
| DC_3A-19A-42A_n78A | DC_3A_n78A | CA_3A-19A-42A | n78A | 3A | n78A | No |
|  | DC_19A_n78A |  |  | 19A | n78A |  |
| DC_3A-19A-42C_n78A | DC_3A_n78A | CA_3A-19A-42C | n78A | 3A | n78A | No |
|  | DC_19A_n78A |  |  | 19A | n78A |  |
| DC_3A-19A-42A_n79A | DC_3A_n79A | CA_3A-19A-42A | n79A | 3A | n79A | No |
|  | DC_19A_n79A |  |  | 19A | n79A |  |
| DC_3A-19A-42C_n79A | DC_3A_n79A | CA_3A-19A-42C | n79A | 3A | n79A | No |
|  | DC_19A_n79A |  |  | 19A | n79A |  |
| DC_3A-20A_n28A-n78A | DC_3A_n28A | CA_3A-20A | CA_n28A-n78A | 3A | n28A | No |
|  | DC_3A_n78A |  |  | 3A | n78A |  |
|  | DC_20A_n28A |  |  | 20A | n28A |  |
|  | DC_20A_n78A |  |  | 20A | n78A |  |
| DC_3A-21A_n1A-n78A | DC_3A_n1A | CA_3A-21A | CA_n1A-n78A | 3A | n1A | No |
|  | DC_3A_n78A | CA_3A-21A | CA_n1A-n78A | 3A | n78A | No |
|  | DC_21A_n1A | CA_3A-21A | CA_n1A-n78A | 21A | n1A | No |
|  | DC_21A_n78A | CA_3A-21A | CA_n1A-n78A | 21A | n78A | No |
| DC_3A-21A_n1A-n79A | DC_3A_n1A | CA_3A-21A | CA_n1A-n79A | 3A | n1A | No |
|  | DC_3A_n79A | CA_3A-21A | CA_n1A-n79A | 3A | n79A | No |
|  | DC_21A_n1A | CA_3A-21A | CA_n1A-n79A | 21A | n1A | No |
|  | DC_21A_n79A | CA_3A-21A | CA_n1A-n79A | 21A | n79A | No |
| DC_3A-21A-42A_n78A | DC_3A_n78A | CA_3A-21A-42A | n78A | 3A | n78A | No |
|  | DC_21A_n78A |  |  | 21A | n78A |  |
| DC_3A-21A-42C_n78A | DC_3A_n78A | CA_3A-21A-42C | n78A | 3A | n78A | No |
|  | DC_21A_n78A |  |  | 21A | n78A |  |
| DC_3A-21A-42A_n79A | DC_3A_n79A | CA_3A-21A-42A | n79A | 3A | n79A | No |
|  | DC_21A_n79A |  |  | 21A | n79A |  |
| DC_3A-21A-42C_n79A | DC_3A_n79A | CA_3A-21A-42C | n79A | 3A | n79A | No |
|  | DC_21A_n79A |  |  | 21A | n79A |  |
| DC_3A-42A_n1A-n78A | DC_3A_n1A | CA_3A-42A | n1A | 3A | n1A | No |
|  | DC_3A_n78A | CA_3A-42A | n78A | 3A | n78A | No |
| DC_3A-42C_n1A-n78A | DC_3A_n1A | CA_3A-42C | n1A | 3A | n1A | No |
|  | DC_3A_n78A | CA_3A-42C | n78A | 3A | n78A | No |
| DC_3A-42A_n1A-n79A | DC_3A_n1A | CA_3A-42A | n1A | 3A | n1A | No |
|  | DC_3A_n79A | CA_3A-42A | n79A | 3A | n79A | No |
| DC_3A-42C_n1A-n79A | DC_3A_n1A | CA_3A-42C | n1A | 3A | n1A | No |
|  | DC_3A_n79A | CA_3A-42C | n79A | 3A | n79A | No |
| DC_7A-20A_n28A-n78A | DC_7A_n28A | CA_7A-20A | CA_n28A-n78A | 7A | n28A | No |
|  | DC_7A_n78A |  |  | 7A | n78A |  |
|  | DC_20A_n28A |  |  | 20A | n28A |  |
|  | DC_20A_n78A |  |  | 20A | n78A |  |
| r | DC_13A_n2A | CA_13A-66A | CA_n2A-n77A | 13A | n2A | No |
| DC_13A-66A_n2A-n77A | DC_13A_n77A | CA_13A-66A | CA_n2A-n77A | 13A | n77A | No |
|  | DC_66A_n2A | CA_13A-66A | CA_n2A-n77A | 66A | n2A | No |
|  | DC_66A_n77A | CA_13A-66A | CA_n2A-n77A | 66A | n77A | No |
| DC_19A-21A_n1A-n78A | DC_19A_n1A | CA_19A-21A | CA_n1A-n78A | 19A | n1A | No |
|  | DC_19A_n78A | CA_19A-21A | CA_n1A-n78A | 19A | n78A | No |
|  | DC_21A_n1A | CA_19A-21A | CA_n1A-n78A | 21A | n1A | No |
|  | DC_21A_n78A | CA_19A-21A | CA_n1A-n78A | 21A | n78A | No |
| DC_19A-21A_n1A-n79A | DC_19A_n1A | CA_19A-21A | CA_n1A-n79A | 19A | n1A | No |
|  | DC_19A_n79A | CA_19A-21A | CA_n1A-n79A | 19A | n79A | No |
|  | DC_21A_n1A | CA_19A-21A | CA_n1A-n79A | 21A | n1A | No |
|  | DC_21A_n79A | CA_19A-21A | CA_n1A-n79A | 21A | n79A | No |
| DC_19A-21A-42A_n1A | DC_19A_n1A | CA_19A-21A-42A | n1A | 19A | n1A | No |
|  | DC_21A_n1A | CA_19A-21A-42A | n1A | 21A | n1A | No |
| DC_19A-21A-42C_n1A | DC_19A_n1A | CA_19A-21A-42C | n1A | 19A | n1A | No |
|  | DC_21A_n1A | CA_19A-21A-42C | n1A | 21A | n1A | No |
| DC_19A-21A-42A_n78A | DC_19A_n78A | CA_19A-21A-42A | n78A | 19A | n78A | No |
|  | DC_21A_n78A |  |  | 21A | n78A |  |
| DC_19A-21A-42C_n78A | DC_19A_n78A | CA_19A-21A-42C | n78A | 19A | n78A | No |
|  | DC_21A_n78A |  |  | 21A | n78A |  |
| DC_19A-21A-42A_n79A | DC_19A_n79A | CA_19A-21A-42A | n79A | 19A | n79A | No |
|  | DC_21A_n79A |  |  | 21A | n79A |  |
| DC_19A-21A-42C_n79A | DC_19A_n79A | CA_19A-21A-42C | n79A | 19A | n79A | No |
|  | DC_21A_n79A |  |  | 21A | n79A |  |
| DC_19A-42A_n1A-n78A | DC_19A_n1A | CA_19A-42A | n1A | 19A | n1A | No |
|  | DC_19A_n78A | CA_19A-42A | n78A | 19A | n78A | No |
| DC_19A-42C_n1A-n78A | DC_19A_n1A | CA_19A-42C | n1A | 19A | n1A | No |
|  | DC_19A_n78A | CA_19A-42C | n78A | 19A | n78A | No |
| DC_19A-42A_n1A-n79A | DC_19A_n1A | CA_19A-42A | n1A | 19A | n1A | No |
|  | DC_19A_n79A | CA_19A-42A | n79A | 19A | n79A | No |
| DC_19A-42C_n1A-n79A | DC_19A_n1A | CA_19A-42C | n1A | 19A | n1A | No |
|  | DC_19A_n79A | CA_19A-42C | n79A | 19A | n79A | No |
| DC_21A-42A_n1A-n78A | DC_21A_n1A | CA_21A-42A | n1A | 21A | n1A | No |
|  | DC_21A_n78A | CA_21A-42A | n78A | 21A | n78A | No |
| DC_21A-42C_n1A-n78A | DC_21A_n1A | CA_21A-42C | n1A | 21A | n1A | No |
|  | DC_21A_n78A | CA_21A-42C | n78A | 21A | n78A | No |
| DC_21A-42A_n1A-n79A | DC_21A_n1A | CA_21A-42A | n1A | 21A | n1A | No |
|  | DC_21A_n79A | CA_21A-42A | n79A | 21A | n79A | No |
| DC_21A-42C_n1A-n79A | DC_21A_n1A | CA_21A-42C | n1A | 21A | n1A | No |
|  | DC_21A_n79A | CA_21A-42C | n79A | 21A | n79A | No |
| Note 1: Protocol testing is limited to EN-DC configurations with 1 CC E-UTRA and 1CC or 2CC NR configurations. |  |  |  |  |  |  |

###### 4.3.1.4.1.5	Inter-band EN-DC configurations within FR1 (five bands)

Table 4.3.1.4.1.5-1: Inter-band EN-DC configurations within FR1 (five bands)

| EN-DC configuration | Uplink EN-DC Configuration | E-UTRA downlink configuration | NR downlink configuration | E-UTRA uplink configuration | NR uplink configuration | Applicable for protocol testing (Note 1) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_1A-3A-7A-20A_n28A | DC_1A_n28A | CA_1A-3A-7A-20A | n28A | 1A | n28A | No |
|  | DC_3A_n28A |  |  | 3A | n28A |  |
|  | DC_7A_n28A |  |  | 7A | n28A |  |
|  | DC_20A_n28A |  |  | 20A | n28A |  |
| DC_1A-3A-7A-20A_n78A | DC_1A_n78A | CA_1A-3A-7A-20A | n78A | 1A | n78A | No |
|  | DC_3A_n78A |  |  | 3A | n78A |  |
|  | DC_7A_n78A |  |  | 7A | n78A |  |
|  | DC_20A_n78A |  |  | 20A | n78A |  |
| DC_1A-3A-7A-28A_n78A | DC_1A_n78A | CA_1A-3A-7A-28A | n78A | 1A | n78A | No |
|  | DC_3A_n78A | CA_1A-3A-7A-28A | n78A | 3A | n78A | No |
|  | DC_7A_n78A | CA_1A-3A-7A-28A | n78A | 7A | n78A | No |
|  | DC_28A_n78A | CA_1A-3A-7A-28A | n78A | 28A | n78A | No |
| DC_1A-3A-7A_n28A-n78A | DC_1A_n28A | CA_1A-3A-7A | CA_n28A-n78A | 1A | n28A | No |
|  | DC_1A_n78A |  |  | 1A | n78A |  |
|  | DC_3A_n28A |  |  | 3A | n28A |  |
|  | DC_3A_n78A |  |  | 3A | n78A |  |
|  | DC_7A_n28A |  |  | 7A | n28A |  |
|  | DC_7A_n78A |  |  | 7A | n78A |  |
| DC_1A-3A-19A-42A_n78A | DC_1A_n78A | CA_1A-3A-19A-42A | n78A | 1A | n78A | No |
|  | DC_3A_n78A |  |  | 3A | n78A |  |
|  | DC_19A_n78A |  |  | 19A | n78A |  |
| DC_1A-3A-19A-42C_n78A | DC_1A_n78A | CA_1A-3A-19A-42C | n78A | 1A | n78A | No |
|  | DC_3A_n78A |  |  | 3A | n78A |  |
|  | DC_19A_n78A |  |  | 19A | n78A |  |
| DC_1A-3A-19A-42A_n79A | DC_1A_n79A | CA_1A-3A-19A-42A | n79A | 1A | n79A | No |
|  | DC_3A_n79A |  |  | 3A | n79A |  |
|  | DC_19A_n79A |  |  | 19A | n79A |  |
| DC_1A-3A-19A-42C_n79A | DC_1A_n79A | CA_1A-3A-19A-42C | n79A | 1A | n79A | No |
|  | DC_3A_n79A |  |  | 3A | n79A |  |
|  | DC_19A_n79A |  |  | 19A | n79A |  |
| DC_1A-3A-20A_n28A-n78A | DC_1A_n28A | CA_1A-3A-20A | CA_n28A-n78A | 1A | n28A | No |
|  | DC_1A_n78A |  |  | 1A | n78A |  |
|  | DC_3A_n28A |  |  | 3A | n28A |  |
|  | DC_3A_n78A |  |  | 3A | n78A |  |
|  | DC_20A_n28A |  |  | 20A | n28A |  |
|  | DC_20A_n78A |  |  | 20A | n78A |  |
| DC_1A-3A-21A-42A_n78A | DC_1A_n78A | CA_1A-3A-21A-42A | n78A | 1A | n78A | No |
|  | DC_3A_n78A |  |  | 3A | n78A |  |
|  | DC_21A_n78A |  |  | 21A | n78A |  |
| DC_1A-3A-21A-42C_n78A | DC_1A_n78A | CA_1A-3A-21A-42C | n78A | 1A | n78A | No |
|  | DC_3A_n78A |  |  | 3A | n78A |  |
|  | DC_21A_n78A |  |  | 21A | n78A |  |
| DC_1A-3A-21A-42A_n79A | DC_1A_n79A | CA_1A-3A-21A-42A | n79A | 1A | n79A | No |
|  | DC_3A_n79A |  |  | 3A | n79A |  |
|  | DC_21A_n79A |  |  | 21A | n79A |  |
| DC_1A-3A-21A-42C_n79A | DC_1A_n79A | CA_1A-3A-21A-42C | n79A | 1A | n79A | No |
|  | DC_3A_n79A |  |  | 3A | n79A |  |
|  | DC_21A_n79A |  |  | 21A | n79A |  |
| DC_1A-7A-20A_n28A-n78A | DC_1A_n28A | CA_1A-7A-20A | CA_n28A-n78A | 1A | n28A | No |
|  | DC_1A_n78A |  |  | 1A | n78A |  |
|  | DC_7A_n28A |  |  | 7A | n28A |  |
|  | DC_7A_n78A |  |  | 7A | n78A |  |
|  | DC_20A_n28A |  |  | 20A | n28A |  |
|  | DC_20A_n78A |  |  | 20A | n78A |  |
| DC_1A-19A-21A-42A_n78A | DC_1A_n78A | CA_1A-19A-21A-42A | n78A | 1A | n78A | No |
|  | DC_19A_n78A |  |  | 19A | n78A |  |
|  | DC_21A_n78A |  |  | 21A | n78A |  |
| DC_1A-19A-21A-42C_n78A | DC_1A_n78A | CA_1A-19A-21A-42C | n78A | 1A | n78A | No |
|  | DC_19A_n78A |  |  | 19A | n78A |  |
|  | DC_21A_n78A |  |  | 21A | n78A |  |
| DC_1A-19A-21A-42A_n79A | DC_1A_n79A | CA_1A-19A-21A-42A | n79A | 1A | n79A | No |
|  | DC_19A_n79A |  |  | 19A | n79A |  |
|  | DC_21A_n79A |  |  | 21A | n79A |  |
| DC_1A-19A-21A-42C_n79A | DC_1A_n79A | CA_1A-19A-21A-42C | n79A | 1A | n79A | No |
|  | DC_19A_n79A |  |  | 19A | n79A |  |
|  | DC_21A_n79A |  |  | 21A | n79A |  |
| DC_3A-7A-20A_n28A-n78A | DC_3A_n28A | CA_3A-7A-20A | CA_n28A-n78A | 3A | n28A | No |
|  | DC_3A_n78A |  |  | 3A | n78A |  |
|  | DC_7A_n28A |  |  | 7A | n28A |  |
|  | DC_7A_n78A |  |  | 7A | n78A |  |
|  | DC_20A_n28A |  |  | 20A | n28A |  |
|  | DC_20A_n78A |  |  | 20A | n78A |  |
| DC_3A-19A-21A-42A_n78A | DC_3A_n78A | CA_3A-19A-21A-42A | n78A | 3A | n78A | No |
|  | DC_19A_n78A | CA_3A-19A-21A-42A | n78A | 19A | n78A | No |
|  | DC_21A_n78A | CA_3A-19A-21A-42A | n78A | 21A | n78A | No |
| DC_3A-19A-21A-42C_n78A | DC_3A_n78A | CA_3A-19A-21A-42C | n78A | 3A | n78A | No |
|  | DC_19A_n78A | CA_3A-19A-21A-42C | n78A | 19A | n78A | No |
|  | DC_21A_n78A | CA_3A-19A-21A-42C | n78A | 21A | n78A | No |
| DC_3A-19A-21A-42A_n79A | DC_3A_n79A | CA_3A-19A-21A-42A | n79A | 3A | n79A | No |
|  | DC_19A_n79A | CA_3A-19A-21A-42A | n79A | 19A | n79A | No |
|  | DC_21A_n79A | CA_3A-19A-21A-42A | n79A | 21A | n79A | No |
| DC_3A-19A-21A-42C_n79A | DC_3A_n79A | CA_3A-19A-21A-42C | n79A | 3A | n79A | No |
|  | DC_19A_n79A | CA_3A-19A-21A-42C | n79A | 19A | n79A | No |
|  | DC_21A_n79A | CA_3A-19A-21A-42C | n79A | 21A | n79A | No |
| DC_3A-19A-42A_n1A-n78A | DC_3A_n1A | CA_3A-19A-42A | CA_n1A-n78A | 3A | n1A | No |
|  | DC_3A_n78A | CA_3A-19A-42A | CA_n1A-n78A | 3A | n78A | No |
|  | DC_19A_n1A | CA_3A-19A-42A | CA_n1A-n78A | 19A | n1A | No |
|  | DC_19A_n78A | CA_3A-19A-42A | CA_n1A-n78A | 19A | n78A | No |
| DC_3A-19A-42C_n1A-n78A | DC_3A_n1A | CA_3A-19A-42C | CA_n1A-n78A | 3A | n1A | No |
|  | DC_3A_n78A | CA_3A-19A-42C | CA_n1A-n78A | 3A | n78A | No |
|  | DC_19A_n1A | CA_3A-19A-42C | CA_n1A-n78A | 19A | n1A | No |
|  | DC_19A_n78A | CA_3A-19A-42C | CA_n1A-n78A | 19A | n78A | No |
| DC_3A-19A-42A_n1A-n79A | DC_3A_n1A | CA_3A-19A-42A | CA_n1A-n79A | 3A | n1A | No |
|  | DC_3A_n79A | CA_3A-19A-42A | CA_n1A-n79A | 3A | n79A | No |
|  | DC_19A_n1A | CA_3A-19A-42A | CA_n1A-n79A | 19A | n1A | No |
|  | DC_19A_n79A | CA_3A-19A-42A | CA_n1A-n79A | 19A | n79A | No |
| DC_3A-19A-42C_n1A-n79A | DC_3A_n1A | CA_3A-19A-42C | CA_n1A-n79A | 3A | n1A | No |
|  | DC_3A_n79A | CA_3A-19A-42C | CA_n1A-n79A | 3A | n79A | No |
|  | DC_19A_n1A | CA_3A-19A-42C | CA_n1A-n79A | 19A | n1A | No |
|  | DC_19A_n79A | CA_3A-19A-42C | CA_n1A-n79A | 19A | n79A | No |
| DC_3A-21A-42A_n1A-n78A | DC_3A_n1A | CA_3A-21A-42A | CA_n1A-n78A | 3A | n1A | No |
|  | DC_3A_n78A | CA_3A-21A-42A | CA_n1A-n78A | 3A | n78A | No |
|  | DC_21A_n1A | CA_3A-21A-42A | CA_n1A-n78A | 21A | n1A | No |
|  | DC_21A_n78A | CA_3A-21A-42A | CA_n1A-n78A | 21A | n78A | No |
| DC_3A-21A-42C_n1A-n78A | DC_3A_n1A | CA_3A-21A-42C | CA_n1A-n78A | 3A | n1A | No |
|  | DC_3A_n78A | CA_3A-21A-42C | CA_n1A-n78A | 3A | n78A | No |
|  | DC_21A_n1A | CA_3A-21A-42C | CA_n1A-n78A | 21A | n1A | No |
|  | DC_21A_n78A | CA_3A-21A-42C | CA_n1A-n78A | 21A | n78A | No |
| DC_3A-21A-42A_n1A-n79A | DC_3A_n1A | CA_3A-21A-42A | CA_n1A-n79A | 3A | n1A | No |
|  | DC_3A_n79A | CA_3A-21A-42A | CA_n1A-n79A | 3A | n79A | No |
|  | DC_21A_n1A | CA_3A-21A-42A | CA_n1A-n79A | 21A | n1A | No |
|  | DC_21A_n79A | CA_3A-21A-42A | CA_n1A-n79A | 21A | n79A | No |
| DC_3A-21A-42C_n1A-n79A | DC_3A_n1A | CA_3A-21A-42C | CA_n1A-n79A | 3A | n1A | No |
|  | DC_3A_n79A | CA_3A-21A-42C | CA_n1A-n79A | 3A | n79A | No |
|  | DC_21A_n1A | CA_3A-21A-42C | CA_n1A-n79A | 21A | n1A | No |
|  | DC_21A_n79A | CA_3A-21A-42C | CA_n1A-n79A | 21A | n79A | No |
| DC_19A-21A-42A_n1A-n78A | DC_19A_n1A | CA_19A-21A-42A | CA_n1A-n78A | 19A | n1A | No |
|  | DC_19A_n78A | CA_19A-21A-42A | CA_n1A-n78A | 19A | n78A | No |
|  | DC_21A_n1A | CA_19A-21A-42A | CA_n1A-n78A | 21A | n1A | No |
|  | DC_21A_n78A | CA_19A-21A-42A | CA_n1A-n78A | 21A | n78A | No |
| DC_19A-21A-42C_n1A-n78A | DC_19A_n1A | CA_19A-21A-42C | CA_n1A-n78A | 19A | n1A | No |
|  | DC_19A_n78A | CA_19A-21A-42C | CA_n1A-n78A | 19A | n78A | No |
|  | DC_21A_n1A | CA_19A-21A-42C | CA_n1A-n78A | 21A | n1A | No |
|  | DC_21A_n78A | CA_19A-21A-42C | CA_n1A-n78A | 21A | n78A | No |
| DC_19A-21A-42A_n1A-n79A | DC_19A_n1A | CA_19A-21A-42A | CA_n1A-n79A | 19A | n1A | No |
|  | DC_19A_n79A | CA_19A-21A-42A | CA_n1A-n79A | 19A | n79A | No |
|  | DC_21A_n1A | CA_19A-21A-42A | CA_n1A-n79A | 21A | n1A | No |
|  | DC_21A_n79A | CA_19A-21A-42A | CA_n1A-n79A | 21A | n79A | No |
| Note 1: Protocol testing is limited to EN-DC configurations with 1 CC E-UTRA and 1CC or 2CC NR configurations. |  |  |  |  |  |  |

4.3.1.4.1.6	Inter-band EN-DC configurations within FR1 (six bands)

Table 4.3.1.4.1.6-1: Inter-band EN-DC configurations within FR1 (six bands)

| EN-DC configuration | Uplink EN-DC Configuration | E-UTRA downlink configuration | NR downlink configuration | E-UTRA uplink configuration | NR uplink configuration | Applicable for protocol testing (Note 1) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_1A-3A-7A-20A_n28A-n78A | DC_1A_n28A | CA_1A-3A-7A-20A | CA_n28A-n78A | 1A | n28A | No |
|  | DC_1A_n78A |  |  | 1A | n78A |  |
|  | DC_3A_n28A |  |  | 3A | n28A |  |
|  | DC_3A_n78A |  |  | 3A | n78A |  |
|  | DC_7A_n28A |  |  | 7A | n28A |  |
|  | DC_7A_n78A |  |  | 7A | n78A |  |
|  | DC_20A_n28A |  |  | 20A | n28A |  |
|  | DC_20A_n78A |  |  | 20A | n78A |  |
| Note 1: Protocol testing is limited to EN-DC configurations with 1 CC E-UTRA and 1CC or 2CC NR configurations. |  |  |  |  |  |  |

<< Next changes >>

4.3.1.5.1.3	Inter-band EN-DC configurations including FR2 (three bands)

Table 4.3.1.5.1.3-1: Inter-band EN-DC configurations including FR2 (three bands)

| EN-DC configuration | Uplink EN-DC Configuration | E-UTRA downlink configuration | NR downlink configuration | E-UTRA uplink configuration | NR uplink configuration | Applicable for protocol testing (Note 1) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_1A-3A_n257A | DC_1A_n257A | CA_1A-3A | n257A | 1A | n257A | No |
|  | DC_3A_n257A |  |  | 3A | n257A |  |
| DC_1A-3A_n257G | DC_1A_n257A | CA_1A-3A | CA_n257G | 1A | n257A | No |
|  | DC_1A_n257D | CA_1A-3A | CA_n257G | 1A | CA_n257D | No |
|  | DC_3A_n257A | CA_1A-3A | CA_n257G | 3A | n257A | No |
|  | DC_3A_n257D | CA_1A-3A | CA_n257G | 3A | CA_n257D | No |
|  | DC_3A_n257G | CA_1A-3A | CA_n257G | 3A | CA_n257G | No |
| DC_1A-3A_n257H | DC_1A_n257A | CA_1A-3A | CA_n257H | 1A | n257A | No |
|  | DC_1A_n257D | CA_1A-3A | CA_n257H | 1A | CA_n257D | No |
|  | DC_3A_n257A | CA_1A-3A | CA_n257H | 3A | n257A | No |
|  | DC_3A_n257D | CA_1A-3A | CA_n257H | 3A | CA_n257D | No |
|  | DC_3A_n257G | CA_1A-3A | CA_n257H | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_1A-3A | CA_n257H | 3A | CA_n257H | No |
| DC_1A-3A_n257I | DC_1A_n257A | CA_1A-3A | CA_n257I | 1A | n257A | No |
|  | DC_1A_n257D | CA_1A-3A | CA_n257I | 1A | CA_n257D | No |
|  | DC_3A_n257A | CA_1A-3A | CA_n257I | 3A | n257A | No |
|  | DC_3A_n257D | CA_1A-3A | CA_n257I | 3A | CA_n257D | No |
|  | DC_3A_n257G | CA_1A-3A | CA_n257I | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_1A-3A | CA_n257I | 3A | CA_n257H | No |
|  | DC_3A_n257I | CA_1A-3A | CA_n257I | 3A | CA_n257I | No |
| DC_1A-18A_n257A | DC_1A_n257A | CA_1A-18A | n257A | 1A | n257A | No |
|  | DC_18A_n257A |  |  | 18A | n257A |  |
| DC_1A-18A_n257I | DC_1A_n257I | CA_1A-18A | CA_n257I | 1A | CA_n257I | No |
|  | DC_18A_n257I | CA_1A-18A | CA_n257I | 18A | CA_n257I | No |
| DC_1A-19A_n257A | DC_1A_n257A | CA_1A-19A | n257A | 1A | n257A | No |
|  | DC_19A_n257A |  |  | 19A | n257A |  |
| DC_1A-19A_n257G | DC_1A_n257A | CA_1A-19A | CA_n257G | 1A | n257A | No |
|  | DC_1A_n257D | CA_1A-19A | CA_n257G | 1A | CA_n257D | No |
|  | DC_1A_n257G | CA_1A-19A | CA_n257G | 1A | CA_n257G | No |
|  | DC_19A_n257A | CA_1A-19A | CA_n257G | 19A | n257A | No |
|  | DC_19A_n257D | CA_1A-19A | CA_n257G | 19A | CA_n257D | No |
| DC_1A-19A_n257H | DC_1A_n257A | CA_1A-19A | CA_n257H | 1A | n257A | No |
|  | DC_1A_n257D | CA_1A-19A | CA_n257H | 1A | CA_n257D | No |
|  | DC_1A_n257G | CA_1A-19A | CA_n257H | 1A | CA_n257G | No |
|  | DC_1A_n257H | CA_1A-19A | CA_n257H | 1A | CA_n257H | No |
|  | DC_19A_n257A | CA_1A-19A | CA_n257H | 19A | n257A | No |
|  | DC_19A_n257D | CA_1A-19A | CA_n257H | 19A | CA_n257D | No |
| DC_1A-19A_n257I | DC_1A_n257A | CA_1A-19A | CA_n257I | 1A | n257A | No |
|  | DC_1A_n257D | CA_1A-19A | CA_n257I | 1A | CA_n257D | No |
|  | DC_1A_n257G | CA_1A-19A | CA_n257I | 1A | CA_n257G | No |
|  | DC_1A_n257H | CA_1A-19A | CA_n257I | 1A | CA_n257H | No |
|  | DC_1A_n257I | CA_1A-19A | CA_n257I | 1A | CA_n257I | No |
|  | DC_19A_n257A | CA_1A-19A | CA_n257I | 19A | n257A | No |
|  | DC_19A_n257D | CA_1A-19A | CA_n257I | 19A | CA_n257D | No |
| DC_1A-21A_n257A | DC_1A_n257A | CA_1A-21A | n257A | 1A | n257A | No |
|  | DC_21A_n257A |  |  | 21A | n257A |  |
| DC_1A-21A_n257G | DC_1A_n257A | CA_1A-21A | CA_n257G | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-21A | CA_n257G | 1A | CA_n257G | No |
|  | DC_21A_n257A | CA_1A-21A | CA_n257G | 21A | n257A | No |
|  | DC_21A_n257G | CA_1A-21A | CA_n257G | 21A | CA_n257G | No |
| DC_1A-21A_n257H | DC_1A_n257A | CA_1A-21A | CA_n257H | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-21A | CA_n257H | 1A | CA_n257G | No |
|  | DC_1A_n257H | CA_1A-21A | CA_n257H | 1A | CA_n257H | No |
|  | DC_21A_n257A | CA_1A-21A | CA_n257H | 21A | n257A | No |
|  | DC_21A_n257G | CA_1A-21A | CA_n257H | 21A | CA_n257G | No |
|  | DC_21A_n257H | CA_1A-21A | CA_n257H | 21A | CA_n257H | No |
| DC_1A-21A_n257I | DC_1A_n257A | CA_1A-21A | CA_n257I | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-21A | CA_n257I | 1A | CA_n257G | No |
|  | DC_1A_n257H | CA_1A-21A | CA_n257I | 1A | CA_n257H | No |
|  | DC_1A_n257I | CA_1A-21A | CA_n257I | 1A | CA_n257I | No |
|  | DC_21A_n257A | CA_1A-21A | CA_n257I | 21A | n257A | No |
|  | DC_21A_n257G | CA_1A-21A | CA_n257I | 21A | CA_n257G | No |
|  | DC_21A_n257H | CA_1A-21A | CA_n257I | 21A | CA_n257H | No |
|  | DC_21A_n257I | CA_1A-21A | CA_n257I | 21A | CA_n257I | No |
| DC_1A-41A_n257A | DC_1A_n257A | CA_1A-41A | n257A | 1A | n257A | No |
|  | DC_41A_n257A |  |  | 41A | n257A |  |
| DC_1A-41A_n257I | DC_1A_n257I | CA_1A-41A | CA_n257I | 1A | CA_n257I | No |
|  | DC_41A_n257I | CA_1A-41A | CA_n257I | 41A | CA_n257I | No |
| DC_1A-42A_n257A | DC_1A_n257A | CA_1A-42A | n257A | 1A | n257A | No |
|  | DC_42A_n257A |  |  | 42A | n257A |  |
| DC_1A-42A_n257G | DC_1A_n257A | CA_1A-42A | CA_n257G | 1A | n257A | No |
|  | DC_1A_n257D | CA_1A-42A | CA_n257G | 1A | CA_n257D | No |
|  | DC_1A_n257A | CA_1A-42A | CA_n257G | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-42A | CA_n257G | 1A | CA_n257G | No |
|  | DC_42A_n257A | CA_1A-42A | CA_n257G | 42A | n257A | No |
|  | DC_42A_n257D | CA_1A-42A | CA_n257G | 42A | CA_n257D | No |
| DC_1A-42A_n257H | DC_1A_n257A | CA_1A-42A | CA_n257H | 1A | n257A | No |
|  | DC_1A_n257D | CA_1A-42A | CA_n257H | 1A | CA_n257D | No |
|  | DC_1A_n257A | CA_1A-42A | CA_n257H | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-42A | CA_n257H | 1A | CA_n257G | No |
|  | DC_1A_n257H | CA_1A-42A | CA_n257H | 1A | CA_n257H | No |
|  | DC_42A_n257A | CA_1A-42A | CA_n257H | 42A | n257A | No |
|  | DC_42A_n257D | CA_1A-42A | CA_n257H | 42A | CA_n257D | No |
| DC_1A-42A_n257I | DC_1A_n257A | CA_1A-42A | CA_n257I | 1A | n257A | No |
|  | DC_1A_n257D | CA_1A-42A | CA_n257I | 1A | CA_n257D | No |
|  | DC_1A_n257A | CA_1A-42A | CA_n257I | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-42A | CA_n257I | 1A | CA_n257G | No |
|  | DC_1A_n257H | CA_1A-42A | CA_n257I | 1A | CA_n257H | No |
|  | DC_1A_n257I | CA_1A-42A | CA_n257I | 1A | CA_n257I | No |
|  | DC_42A_n257A | CA_1A-42A | CA_n257I | 42A | n257A | No |
|  | DC_42A_n257D | CA_1A-42A | CA_n257I | 42A | CA_n257D | No |
| DC_1A-42C_n257A | DC_1A_n257A | CA_1A-42C | n257A | 1A | n257A | No |
|  | DC_42A_n257A |  |  | 42A | n257A |  |
| DC_1A-42D_n257A | DC_1A_n257A | CA_1A-42D | n257A | 1A | n257A | No |
|  | DC_42A_n257A |  |  | 42A | n257A |  |
| DC_1A-42E_n257A | DC_1A_n257A | CA_1A-42E | n257A | 1A | n257A | No |
|  | DC_42A_n257A |  |  | 42A | n257A |  |
| DC_1A-42E_n257G | DC_1A_n257A | CA_1A-42E | CA_n257G | 1A | n257A | No |
|  | DC_1A_n257D | CA_1A-42E | CA_n257G | 1A | CA_n257D | No |
|  | DC_1A_n257A | CA_1A-42E | CA_n257G | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-42E | CA_n257G | 1A | CA_n257G | No |
|  | DC_42A_n257A | CA_1A-42E | CA_n257G | 42A | n257A | No |
|  | DC_42A_n257D | CA_1A-42E | CA_n257G | 42A | CA_n257D | No |
| DC_1A-42E_n257H | DC_1A_n257A | CA_1A-42E | CA_n257H | 1A | n257A | No |
|  | DC_1A_n257D | CA_1A-42E | CA_n257H | 1A | CA_n257D | No |
|  | DC_1A_n257A | CA_1A-42E | CA_n257H | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-42E | CA_n257H | 1A | CA_n257G | No |
|  | DC_1A_n257H | CA_1A-42E | CA_n257H | 1A | CA_n257H | No |
|  | DC_42A_n257A | CA_1A-42E | CA_n257H | 42A | n257A | No |
|  | DC_42A_n257D | CA_1A-42E | CA_n257H | 42A | CA_n257D | No |
| DC_1A-42E_n257I | DC_1A_n257A | CA_1A-42E | CA_n257I | 1A | n257A | No |
|  | DC_1A_n257D | CA_1A-42E | CA_n257I | 1A | CA_n257D | No |
|  | DC_1A_n257A | CA_1A-42E | CA_n257I | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-42E | CA_n257I | 1A | CA_n257G | No |
|  | DC_1A_n257H | CA_1A-42E | CA_n257I | 1A | CA_n257H | No |
|  | DC_1A_n257I | CA_1A-42E | CA_n257I | 1A | CA_n257I | No |
|  | DC_42A_n257A | CA_1A-42E | CA_n257I | 42A | n257A | No |
|  | DC_42A_n257D | CA_1A-42E | CA_n257I | 42A | CA_n257D | No |
| DC_2A-5A_n257A | DC_2A_n257A | CA_2A-5A | n257A | 2A | n257A | No |
|  | DC_5A_n257A |  |  | 5A | n257A |  |
| DC_2A-5A_n260A | DC_2A_n260A | CA_2A-5A | n260A | 2A | n260A | No |
|  | DC_5A_n260A |  |  | 5A | n260A |  |
| DC_2A-12A_n260A | DC_2A_n260A | CA_2A-12A | n260A | 2A | n260A | No |
|  | DC_12A_n260A |  |  | 12A | n260A |  |
| DC_2A-14A_n260A | DC_2A_n260A | CA_2A-14A | n260A | 2A | n260A | No |
|  | DC_14A_n260A | CA_2A-14A | n260A | 14A | n260A | No |
| DC_2A-14A_n260G | DC_2A_n260A | CA_2A-14A | CA_n260G | 2A | n260A | No |
|  | DC_2A_n260G | CA_2A-14A | CA_n260G | 2A | CA_n260G | No |
|  | DC_14A_n260A | CA_2A-14A | CA_n260G | 14A | n260A | No |
|  | DC_14A_n260G | CA_2A-14A | CA_n260G | 14A | CA_n260G | No |
| DC_2A-14A_n260H | DC_2A_n260A | CA_2A-14A | CA_n260H | 2A | n260A | No |
|  | DC_2A_n260G | CA_2A-14A | CA_n260H | 2A | CA_n260G | No |
|  | DC_2A_n260H | CA_2A-14A | CA_n260H | 2A | CA_n260H | No |
|  | DC_14A_n260A | CA_2A-14A | CA_n260H | 14A | n260A | No |
|  | DC_14A_n260G | CA_2A-14A | CA_n260H | 14A | CA_n260G | No |
|  | DC_14A_n260H | CA_2A-14A | CA_n260H | 14A | CA_n260H | No |
| DC_2A-14A_n260I | DC_2A_n260A | CA_2A-14A | CA_n260I | 2A | n260A | No |
|  | DC_2A_n260G | CA_2A-14A | CA_n260I | 2A | CA_n260G | No |
|  | DC_2A_n260H | CA_2A-14A | CA_n260I | 2A | CA_n260H | No |
|  | DC_2A_n260I | CA_2A-14A | CA_n260I | 2A | CA_n260I | No |
|  | DC_14A_n260A | CA_2A-14A | CA_n260I | 14A | n260A | No |
|  | DC_14A_n260G | CA_2A-14A | CA_n260I | 14A | CA_n260G | No |
|  | DC_14A_n260H | CA_2A-14A | CA_n260I | 14A | CA_n260H | No |
|  | DC_14A_n260I | CA_2A-14A | CA_n260I | 14A | CA_n260I | No |
| DC_2A-2A-14A_n260A | DC_2A_n260A | CA_2A-2A-14A | n260A | 2A | n260A | No |
|  | DC_14A_n260A | CA_2A-2A-14A | n260A | 14A | n260A | No |
| DC_2A-2A-14A_n260G | DC_2A_n260A | CA_2A-2A-14A | CA_n260G | 2A | n260A | No |
|  | DC_2A_n260G | CA_2A-2A-14A | CA_n260G | 2A | CA_n260G | No |
|  | DC_14A_n260A | CA_2A-2A-14A | CA_n260G | 14A | n260A | No |
|  | DC_14A_n260G | CA_2A-2A-14A | CA_n260G | 14A | CA_n260G | No |
| DC_2A-2A-14A_n260H | DC_2A_n260A | CA_2A-2A-14A | CA_n260H | 2A | n260A | No |
|  | DC_2A_n260G | CA_2A-2A-14A | CA_n260H | 2A | CA_n260G | No |
|  | DC_2A_n260H | CA_2A-2A-14A | CA_n260H | 2A | CA_n260H | No |
|  | DC_14A_n260A | CA_2A-2A-14A | CA_n260H | 14A | n260A | No |
|  | DC_14A_n260G | CA_2A-2A-14A | CA_n260H | 14A | CA_n260G | No |
|  | DC_14A_n260H | CA_2A-2A-14A | CA_n260H | 14A | CA_n260H | No |
| DC_2A-2A-14A_n260I | DC_2A_n260A | CA_2A-2A-14A | CA_n260I | 2A | n260A | No |
|  | DC_2A_n260G | CA_2A-2A-14A | CA_n260I | 2A | CA_n260G | No |
|  | DC_2A_n260H | CA_2A-2A-14A | CA_n260I | 2A | CA_n260H | No |
|  | DC_2A_n260I | CA_2A-2A-14A | CA_n260I | 2A | CA_n260I | No |
|  | DC_14A_n260A | CA_2A-2A-14A | CA_n260I | 14A | n260A | No |
|  | DC_14A_n260G | CA_2A-2A-14A | CA_n260I | 14A | CA_n260G | No |
|  | DC_14A_n260H | CA_2A-2A-14A | CA_n260I | 14A | CA_n260H | No |
|  | DC_14A_n260I | CA_2A-2A-14A | CA_n260I | 14A | CA_n260I | No |
| DC_2A-30A_n260A | DC_2A_n260A | CA_2A-30A | n260A | 2A | n260A | No |
|  | DC_30A_n260A |  |  | 30A | n260A |  |
| DC_2A-66A_n257A | DC_2A_n257A | CA_2A-66A | n257A | 2A | n257A | No |
|  | DC_66A_n257A |  |  | 66A | n257A |  |
| DC_2A-66A_n260A | DC_2A_n260A | CA_2A-66A | n260A | 2A | n260A | No |
|  | DC_66A_n260A |  |  | 66A | n260A |  |
| DC_3A-18A_n257A | DC_3A_n257A | CA_3A-18A | n257A | 3A | n257A | No |
|  | DC_18A_n257A | CA_3A-18A | n257A | 18A | n257A | No |
| DC_3A-18A_n257I | DC_3A_n257I | CA_3A-18A | CA_n257I | 3A | CA_n257I | No |
|  | DC_18A_n257I | CA_3A-18A | CA_n257I | 18A | CA_n257I | No |
| DC_3A-19A_n257A | DC_3A_n257A | CA_3A-19A | n257A | 3A | n257A | No |
|  | DC_19A_n257A |  |  | 19A | n257A |  |
| DC_3A-19A_n257G | DC_3A_n257A | CA_3A-19A | CA_n257G | 3A | n257A | No |
|  | DC_3A_n257D | CA_3A-19A | CA_n257G | 3A | CA_n257D | No |
|  | DC_3A_n257G | CA_3A-19A | CA_n257G | 3A | CA_n257G | No |
|  | DC_19A_n257A | CA_3A-19A | CA_n257G | 19A | n257A | No |
|  | DC_19A_n257D | CA_3A-19A | CA_n257G | 19A | CA_n257D | No |
| DC_3A-19A_n257H | DC_3A_n257A | CA_3A-19A | CA_n257H | 3A | n257A | No |
|  | DC_3A_n257D | CA_3A-19A | CA_n257H | 3A | CA_n257D | No |
|  | DC_3A_n257G | CA_3A-19A | CA_n257H | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_3A-19A | CA_n257H | 3A | CA_n257H | No |
|  | DC_19A_n257A | CA_3A-19A | CA_n257H | 19A | n257A | No |
|  | DC_19A_n257D | CA_3A-19A | CA_n257H | 19A | CA_n257D | No |
| DC_3A-19A_n257I | DC_3A_n257A | CA_3A-19A | CA_n257I | 3A | n257A | No |
|  | DC_3A_n257D | CA_3A-19A | CA_n257I | 3A | CA_n257D | No |
|  | DC_3A_n257G | CA_3A-19A | CA_n257I | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_3A-19A | CA_n257I | 3A | CA_n257H | No |
|  | DC_3A_n257I | CA_3A-19A | CA_n257I | 3A | CA_n257I | No |
|  | DC_19A_n257A | CA_3A-19A | CA_n257I | 19A | n257A | No |
|  | DC_19A_n257D | CA_3A-19A | CA_n257I | 19A | CA_n257D | No |
| DC_3A-21A_n257A | DC_3A_n257A | CA_3A-21A | n257A | 3A | n257A | No |
|  | DC_21A_n257A |  |  | 21A | n257A |  |
| DC_3A-21A_n257G | DC_3A_n257A | CA_3A-21A | CA_n257G | 3A | n257A | No |
|  | DC_3A_n257D | CA_3A-21A | CA_n257G | 3A | CA_n257D | No |
|  | DC_3A_n257G | CA_3A-21A | CA_n257G | 3A | CA_n257G | No |
|  | DC_21A_n257A | CA_3A-21A | CA_n257G | 21A | n257A | No |
|  | DC_21A_n257D | CA_3A-21A | CA_n257G | 21A | CA_n257D | No |
| DC_3A-21A_n257H | DC_3A_n257A | CA_3A-21A | CA_n257H | 3A | n257A | No |
|  | DC_3A_n257D | CA_3A-21A | CA_n257H | 3A | CA_n257D | No |
|  | DC_3A_n257G | CA_3A-21A | CA_n257H | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_3A-21A | CA_n257H | 3A | CA_n257H | No |
|  | DC_21A_n257A | CA_3A-21A | CA_n257H | 21A | n257A | No |
|  | DC_21A_n257D | CA_3A-21A | CA_n257H | 21A | CA_n257D | No |
| DC_3A-21A_n257I | DC_3A_n257A | CA_3A-21A | CA_n257I | 3A | n257A | No |
|  | DC_3A_n257D | CA_3A-21A | CA_n257I | 3A | CA_n257D | No |
|  | DC_3A_n257G | CA_3A-21A | CA_n257I | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_3A-21A | CA_n257I | 3A | CA_n257H | No |
|  | DC_3A_n257I | CA_3A-21A | CA_n257I | 3A | CA_n257I | No |
|  | DC_21A_n257A | CA_3A-21A | CA_n257I | 21A | n257A | No |
|  | DC_21A_n257D | CA_3A-21A | CA_n257I | 21A | CA_n257D | No |
| DC_3A-41A_n257A | DC_3A_n257A | CA_3A-41A | n257A | 3A | n257A | No |
|  | DC_41A_n257A |  |  | 41A | n257A |  |
| DC_3A-41A_n257I | DC_3A_n257I | CA_3A-41A | CA_n257I | 3A | CA_n257I | No |
|  | DC_41A_n257I | CA_3A-41A | CA_n257I | 41A | CA_n257I | No |
| DC_3A-42A_n257A | DC_3A_n257A | CA_3A-42A | n257A | 3A | n257A | No |
|  | DC_42A_n257A |  |  | 42A | n257A |  |
| DC_3A-42A_n257G | DC_3A_n257A | CA_3A-42A | CA_n257G | 3A | n257A | No |
|  | DC_3A_n257D | CA_3A-42A | CA_n257G | 3A | CA_n257D | No |
|  | DC_3A_n257G | CA_3A-42A | CA_n257G | 3A | CA_n257G | No |
|  | DC_42A_n257A | CA_3A-42A | CA_n257G | 42A | n257A | No |
|  | DC_42A_n257D | CA_3A-42A | CA_n257G | 42A | CA_n257D | No |
| DC_3A-42A_n257H | DC_3A_n257A | CA_3A-42A | CA_n257H | 3A | n257A | No |
|  | DC_3A_n257D | CA_3A-42A | CA_n257H | 3A | CA_n257D | No |
|  | DC_3A_n257G | CA_3A-42A | CA_n257H | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_3A-42A | CA_n257H | 3A | CA_n257H | No |
|  | DC_42A_n257A | CA_3A-42A | CA_n257H | 42A | n257A | No |
|  | DC_42A_n257D | CA_3A-42A | CA_n257H | 42A | CA_n257D | No |
| DC_3A-42A_n257I | DC_3A_n257A | CA_3A-42A | CA_n257I | 3A | n257A | No |
|  | DC_3A_n257D | CA_3A-42A | CA_n257I | 3A | CA_n257D | No |
|  | DC_3A_n257G | CA_3A-42A | CA_n257I | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_3A-42A | CA_n257I | 3A | CA_n257H | No |
|  | DC_3A_n257I | CA_3A-42A | CA_n257I | 3A | CA_n257I | No |
|  | DC_42A_n257A | CA_3A-42A | CA_n257I | 42A | n257A | No |
|  | DC_42A_n257D | CA_3A-42A | CA_n257I | 42A | CA_n257D | No |
| DC_3A-42C_n257A | DC_3A_n257A | CA_3A-42C | n257A | 3A | n257A | No |
|  | DC_42A_n257A |  |  | 42A | n257A |  |
| DC_3A-42C_n257G | DC_3A_n257A | CA_3A-42C | CA_n257G | 3A | n257A | No |
|  | DC_3A_n257D | CA_3A-42C | CA_n257G | 3A | CA_n257D | No |
|  | DC_3A_n257G | CA_3A-42C | CA_n257G | 3A | CA_n257G | No |
|  | DC_42A_n257A | CA_3A-42C | CA_n257G | 42A | n257A | No |
|  | DC_42A_n257D | CA_3A-42C | CA_n257G | 42A | CA_n257D | No |
| DC_3A-42C_n257H | DC_3A_n257A | CA_3A-42C | CA_n257H | 3A | n257A | No |
|  | DC_3A_n257D | CA_3A-42C | CA_n257H | 3A | CA_n257D | No |
|  | DC_3A_n257G | CA_3A-42C | CA_n257H | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_3A-42C | CA_n257H | 3A | CA_n257H | No |
|  | DC_42A_n257A | CA_3A-42C | CA_n257H | 42A | n257A | No |
|  | DC_42A_n257D | CA_3A-42C | CA_n257H | 42A | CA_n257D | No |
| DC_3A-42C_n257I | DC_3A_n257A | CA_3A-42C | CA_n257I | 3A | n257A | No |
|  | DC_3A_n257D | CA_3A-42C | CA_n257I | 3A | CA_n257D | No |
|  | DC_3A_n257G | CA_3A-42C | CA_n257I | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_3A-42C | CA_n257I | 3A | CA_n257H | No |
|  | DC_3A_n257I | CA_3A-42C | CA_n257I | 3A | CA_n257I | No |
|  | DC_42A_n257A | CA_3A-42C | CA_n257I | 42A | n257A | No |
|  | DC_42A_n257D | CA_3A-42C | CA_n257I | 42A | CA_n257D | No |
| DC_3A-42D_n257G | DC_3A_n257A | CA_3A-42D | CA_n257G | 3A | n257A | No |
|  | DC_3A_n257D | CA_3A-42D | CA_n257G | 3A | CA_n257D | No |
|  | DC_3A_n257G | CA_3A-42D | CA_n257G | 3A | CA_n257G | No |
|  | DC_42A_n257A | CA_3A-42D | CA_n257G | 42A | n257A | No |
|  | DC_42A_n257D | CA_3A-42D | CA_n257G | 42A | CA_n257D | No |
| DC_3A-42D_n257H | DC_3A_n257A | CA_3A-42D | CA_n257H | 3A | n257A | No |
|  | DC_3A_n257D | CA_3A-42D | CA_n257H | 3A | CA_n257D | No |
|  | DC_3A_n257G | CA_3A-42D | CA_n257H | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_3A-42D | CA_n257H | 3A | CA_n257H | No |
|  | DC_42A_n257A | CA_3A-42D | CA_n257H | 42A | n257A | No |
|  | DC_42A_n257D | CA_3A-42D | CA_n257H | 42A | CA_n257D | No |
| DC_3A-42D_n257I | DC_3A_n257A | CA_3A-42D | CA_n257I | 3A | n257A | No |
|  | DC_3A_n257D | CA_3A-42D | CA_n257I | 3A | CA_n257D | No |
|  | DC_3A_n257G | CA_3A-42D | CA_n257I | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_3A-42D | CA_n257I | 3A | CA_n257H | No |
|  | DC_3A_n257I | CA_3A-42D | CA_n257I | 3A | CA_n257I | No |
|  | DC_42A_n257A | CA_3A-42D | CA_n257I | 42A | n257A | No |
|  | DC_42A_n257D | CA_3A-42D | CA_n257I | 42A | CA_n257D | No |
| DC_3A-42E_n257G | DC_3A_n257A | CA_3A-42E | CA_n257G | 3A | n257A | No |
|  | DC_3A_n257D | CA_3A-42E | CA_n257G | 3A | CA_n257D | No |
|  | DC_3A_n257G | CA_3A-42E | CA_n257G | 3A | CA_n257G | No |
|  | DC_42A_n257A | CA_3A-42E | CA_n257G | 42A | n257A | No |
|  | DC_42A_n257D | CA_3A-42E | CA_n257G | 42A | CA_n257D | No |
| DC_3A-42E_n257H | DC_3A_n257A | CA_3A-42E | CA_n257H | 3A | n257A | No |
|  | DC_3A_n257D | CA_3A-42E | CA_n257H | 3A | CA_n257D | No |
|  | DC_3A_n257G | CA_3A-42E | CA_n257H | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_3A-42E | CA_n257H | 3A | CA_n257H | No |
|  | DC_42A_n257A | CA_3A-42E | CA_n257H | 42A | n257A | No |
|  | DC_42A_n257D | CA_3A-42E | CA_n257H | 42A | CA_n257D | No |
| DC_3A-42E_n257I | DC_3A_n257A | CA_3A-42E | CA_n257I | 3A | n257A | No |
|  | DC_3A_n257D | CA_3A-42E | CA_n257I | 3A | CA_n257D | No |
|  | DC_3A_n257G | CA_3A-42E | CA_n257I | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_3A-42E | CA_n257I | 3A | CA_n257H | No |
|  | DC_3A_n257I | CA_3A-42E | CA_n257I | 3A | CA_n257I | No |
|  | DC_42A_n257A | CA_3A-42E | CA_n257I | 42A | n257A | No |
|  | DC_42A_n257D | CA_3A-42E | CA_n257I | 42A | CA_n257D | No |
| DC_5A-7A_n257A | DC_5A_n257A | CA_5A-7A | n257A | 5A | n257A | No |
|  | DC_7A_n257A |  |  | 7A | n257A |  |
| DC_5A-30A_n260A | DC_5A_n260A | CA_5A-30A | n260A | 5A | n260A | No |
|  | DC_30A_n260A |  |  | 30A | n260A |  |
| DC_5A-66A_n257A | DC_5A_n257A | CA_5A-66A | n257A | 5A | n257A | No |
|  | DC_66A_n257A |  |  | 66A | n257A |  |
| DC_5A-66A_n260A | DC_5A_n260A | CA_5A-66A | n260A | 5A | n260A | No |
|  | DC_66A_n260A |  |  | 66A | n260A |  |
| DC_12A-30A_n260A | DC_12A_n260A | CA_12A-30A | n260A | 12A | n260A | No |
|  | DC_30A_n260A |  |  | 30A | n260A |  |
| DC_12A-66A_n260A | DC_12A_n260A | CA_12A-66A | n260A | 12A | n260A | No |
|  | DC_66A_n260A |  |  | 66A | n260A |  |
| DC_14A-30A_n260A | DC_14A_n260A | CA_14A-30A | n260A | 14A | n260A | No |
|  | DC_30A_n260A | CA_14A-30A | n260A | 30A | n260A | No |
| DC_14A-30A_n260G | DC_14A_n260A | CA_14A-30A | CA_n260G | 14A | n260A | No |
|  | DC_14A_n260G | CA_14A-30A | CA_n260G | 14A | CA_n260G | No |
|  | DC_30A_n260A | CA_14A-30A | CA_n260G | 30A | n260A | No |
|  | DC_30A_n260G | CA_14A-30A | CA_n260G | 30A | CA_n260G | No |
| DC_14A-30A_n260H | DC_14A_n260A | CA_14A-30A | CA_n260H | 14A | n260A | No |
|  | DC_14A_n260G | CA_14A-30A | CA_n260H | 14A | CA_n260G | No |
|  | DC_14A_n260H | CA_14A-30A | CA_n260H | 14A | CA_n260H | No |
|  | DC_30A_n260A | CA_14A-30A | CA_n260H | 30A | n260A | No |
|  | DC_30A_n260G | CA_14A-30A | CA_n260H | 30A | CA_n260G | No |
|  | DC_30A_n260H | CA_14A-30A | CA_n260H | 30A | CA_n260H | No |
| DC_14A-30A_n260I | DC_14A_n260A | CA_14A-30A | CA_n260I | 14A | n260A | No |
|  | DC_14A_n260G | CA_14A-30A | CA_n260I | 14A | CA_n260G | No |
|  | DC_14A_n260H | CA_14A-30A | CA_n260I | 14A | CA_n260H | No |
|  | DC_14A_n260I | CA_14A-30A | CA_n260I | 14A | CA_n260I | No |
|  | DC_30A_n260A | CA_14A-30A | CA_n260I | 30A | n260A | No |
|  | DC_30A_n260G | CA_14A-30A | CA_n260I | 30A | CA_n260G | No |
|  | DC_30A_n260H | CA_14A-30A | CA_n260I | 30A | CA_n260H | No |
|  | DC_30A_n260I | CA_14A-30A | CA_n260I | 30A | CA_n260I | No |
| DC_14A-66A_n260A | DC_14A_n260A | CA_14A-66A | n260A | 14A | n260A | No |
|  | DC_66A_n260A | CA_14A-66A | n260A | 66A | n260A | No |
| DC_14A-66A_n260G | DC_14A_n260A | CA_14A-66A | CA_n260G | 14A | n260A | No |
|  | DC_14A_n260G | CA_14A-66A | CA_n260G | 14A | CA_n260G | No |
|  | DC_66A_n260A | CA_14A-66A | CA_n260G | 66A | n260A | No |
|  | DC_66A_n260G | CA_14A-66A | CA_n260G | 66A | CA_n260G | No |
| DC_14A-66A_n260H | DC_14A_n260A | CA_14A-66A | CA_n260H | 14A | n260A | No |
|  | DC_14A_n260G | CA_14A-66A | CA_n260H | 14A | CA_n260G | No |
|  | DC_14A_n260H | CA_14A-66A | CA_n260H | 14A | CA_n260H | No |
|  | DC_66A_n260A | CA_14A-66A | CA_n260H | 66A | n260A | No |
|  | DC_66A_n260G | CA_14A-66A | CA_n260H | 66A | CA_n260G | No |
|  | DC_66A_n260H | CA_14A-66A | CA_n260H | 66A | CA_n260H | No |
| DC_14A-66A_n260I | DC_14A_n260A | CA_14A-66A | CA_n260I | 14A | n260A | No |
|  | DC_14A_n260G | CA_14A-66A | CA_n260I | 14A | CA_n260G | No |
|  | DC_14A_n260H | CA_14A-66A | CA_n260I | 14A | CA_n260H | No |
|  | DC_14A_n260I | CA_14A-66A | CA_n260I | 14A | CA_n260I | No |
|  | DC_66A_n260A | CA_14A-66A | CA_n260I | 66A | n260A | No |
|  | DC_66A_n260G | CA_14A-66A | CA_n260I | 66A | CA_n260G | No |
|  | DC_66A_n260H | CA_14A-66A | CA_n260I | 66A | CA_n260H | No |
|  | DC_66A_n260I | CA_14A-66A | CA_n260I | 66A | CA_n260I | No |
| DC_14A-66A-66A_n260A | DC_14A_n260A | CA_14A-66A-66A | n260A | 14A | n260A | No |
|  | DC_66A_n260A | CA_14A-66A-66A | n260A | 66A | n260A | No |
| DC_14A-66A-66a_n260G | DC_14A_n260A | CA_14A-66A-66A | CA_n260G | 14A | n260A | No |
|  | DC_14A_n260G | CA_14A-66A-66A | CA_n260G | 14A | CA_n260G | No |
|  | DC_66A_n260A | CA_14A-66A-66A | CA_n260G | 66A | n260A | No |
|  | DC_66A_n260G | CA_14A-66A-66A | CA_n260G | 66A | CA_n260G | No |
| DC_14A-66A-66a_n260H | DC_14A_n260A | CA_14A-66A-66A | CA_n260H | 14A | n260A | No |
|  | DC_14A_n260G | CA_14A-66A-66A | CA_n260H | 14A | CA_n260G | No |
|  | DC_14A_n260H | CA_14A-66A-66A | CA_n260H | 14A | CA_n260H | No |
|  | DC_66A_n260A | CA_14A-66A-66A | CA_n260H | 66A | n260A | No |
|  | DC_66A_n260G | CA_14A-66A-66A | CA_n260H | 66A | CA_n260G | No |
|  | DC_66A_n260H | CA_14A-66A-66A | CA_n260H | 66A | CA_n260H | No |
| DC_14A-66A-66A_n260I | DC_14A_n260A | CA_14A-66A-66A | CA_n260I | 14A | n260A | No |
|  | DC_14A_n260G | CA_14A-66A-66A | CA_n260I | 14A | CA_n260G | No |
|  | DC_14A_n260H | CA_14A-66A-66A | CA_n260I | 14A | CA_n260H | No |
|  | DC_14A_n260I | CA_14A-66A-66A | CA_n260I | 14A | CA_n260I | No |
|  | DC_66A_n260A | CA_14A-66A-66A | CA_n260I | 66A | n260A | No |
|  | DC_66A_n260G | CA_14A-66A-66A | CA_n260I | 66A | CA_n260G | No |
|  | DC_66A_n260H | CA_14A-66A-66A | CA_n260I | 66A | CA_n260H | No |
|  | DC_66A_n260I | CA_14A-66A-66A | CA_n260I | 66A | CA_n260I | No |
| DC_19A-21A_n257A | DC_19A_n257A | CA_19A-21A | n257A | 19A | n257A | No |
|  | DC_21A_n257A |  |  | 21A | n257A |  |
| DC_19A-21A_n257G | DC_19A_n257A | CA_19A-21A | CA_n257G | 19A | n257A | No |
|  | DC_19A_n257D | CA_19A-21A | CA_n257G | 19A | CA_n257D | No |
|  | DC_21A_n257A | CA_19A-21A | CA_n257G | 21A | n257A | No |
|  | DC_21A_n257D | CA_19A-21A | CA_n257G | 21A | CA_n257D | No |
|  | DC_21A_n257G | CA_19A-21A | CA_n257G | 21A | CA_n257G | No |
| DC_19A-21A_n257H | DC_19A_n257A | CA_19A-21A | CA_n257H | 19A | n257A | No |
|  | DC_19A_n257D | CA_19A-21A | CA_n257H | 19A | CA_n257D | No |
|  | DC_21A_n257A | CA_19A-21A | CA_n257H | 21A | n257A | No |
|  | DC_21A_n257D | CA_19A-21A | CA_n257H | 21A | CA_n257D | No |
|  | DC_21A_n257G | CA_19A-21A | CA_n257H | 21A | CA_n257G | No |
|  | DC_21A_n257H | CA_19A-21A | CA_n257H | 21A | CA_n257H | No |
| DC_19A-21A_n257I | DC_19A_n257A | CA_19A-21A | CA_n257I | 19A | n257A | No |
|  | DC_19A_n257D | CA_19A-21A | CA_n257I | 19A | CA_n257D | No |
|  | DC_21A_n257A | CA_19A-21A | CA_n257I | 21A | n257A | No |
|  | DC_21A_n257D | CA_19A-21A | CA_n257I | 21A | CA_n257D | No |
|  | DC_21A_n257G | CA_19A-21A | CA_n257I | 21A | CA_n257G | No |
|  | DC_21A_n257H | CA_19A-21A | CA_n257I | 21A | CA_n257H | No |
|  | DC_21A_n257I | CA_19A-21A | CA_n257I | 21A | CA_n257I | No |
| DC_19A-42A_n257A | DC_19A_n257A | CA_19A-42A | n257A | 19A | n257A | No |
|  | DC_42A_n257A |  |  | 42A | n257A |  |
| DC_19A-42A_n257G | DC_19A_n257A | CA_19A-42A | CA_n257G | 19A | n257A | No |
|  | DC_19A_n257D | CA_19A-42A | CA_n257G | 19A | CA_n257D | No |
|  | DC_19A_n257G | CA_19A-42A | CA_n257G | 19A | CA_n257G | No |
|  | DC_42A_n257A | CA_19A-42A | CA_n257G | 42A | n257A | No |
|  | DC_42A_n257D | CA_19A-42A | CA_n257G | 42A | CA_n257D | No |
|  | DC_42A_n257G | CA_19A-42A | CA_n257G | 42A | CA_n257G | No |
| DC_19A-42A_n257H | DC_19A_n257A | CA_19A-42A | CA_n257H | 19A | n257A | No |
|  | DC_19A_n257D | CA_19A-42A | CA_n257H | 19A | CA_n257D | No |
|  | DC_19A_n257G | CA_19A-42A | CA_n257H | 19A | CA_n257G | No |
|  | DC_19A_n257H | CA_19A-42A | CA_n257H | 19A | CA_n257H | No |
|  | DC_42A_n257A | CA_19A-42A | CA_n257H | 42A | n257A | No |
|  | DC_42A_n257D | CA_19A-42A | CA_n257H | 42A | CA_n257D | No |
|  | DC_42A_n257G | CA_19A-42A | CA_n257H | 42A | CA_n257G | No |
|  | DC_42A_n257H | CA_19A-42A | CA_n257H | 42A | CA_n257H | No |
| DC_19A-42A_n257I | DC_19A_n257A | CA_19A-42A | CA_n257I | 19A | n257A | No |
|  | DC_19A_n257D | CA_19A-42A | CA_n257I | 19A | CA_n257D | No |
|  | DC_19A_n257G | CA_19A-42A | CA_n257I | 19A | CA_n257G | No |
|  | DC_19A_n257H | CA_19A-42A | CA_n257I | 19A | CA_n257H | No |
|  | DC_19A_n257I | CA_19A-42A | CA_n257I | 19A | CA_n257I | No |
|  | DC_42A_n257A | CA_19A-42A | CA_n257I | 42A | n257A | No |
|  | DC_42A_n257D | CA_19A-42A | CA_n257I | 42A | CA_n257D | No |
|  | DC_42A_n257G | CA_19A-42A | CA_n257I | 42A | CA_n257G | No |
|  | DC_42A_n257H | CA_19A-42A | CA_n257I | 42A | CA_n257H | No |
|  | DC_42A_n257I | CA_19A-42A | CA_n257I | 42A | CA_n257I | No |
| DC_19A-42C_n257A | DC_19A_n257A | CA_19A-42C | n257A | 19A | n257A | No |
|  | DC_42A_n257A |  |  | 42A | n257A |  |
| DC_19A-42C_n257G | DC_19A_n257A | CA_19A-42C | CA_n257G | 19A | n257A | No |
|  | DC_19A_n257D | CA_19A-42C | CA_n257G | 19A | CA_n257D | No |
|  | DC_19A_n257G | CA_19A-42C | CA_n257G | 19A | CA_n257G | No |
|  | DC_42A_n257A | CA_19A-42C | CA_n257G | 42A | n257A | No |
|  | DC_42A_n257D | CA_19A-42C | CA_n257G | 42A | CA_n257D | No |
|  | DC_42A_n257G | CA_19A-42C | CA_n257G | 42A | CA_n257G | No |
| DC_19A-42C_n257H | DC_19A_n257A | CA_19A-42C | CA_n257H | 19A | n257A | No |
|  | DC_19A_n257D | CA_19A-42C | CA_n257H | 19A | CA_n257D | No |
|  | DC_19A_n257G | CA_19A-42C | CA_n257H | 19A | CA_n257G | No |
|  | DC_19A_n257H | CA_19A-42C | CA_n257H | 19A | CA_n257H | No |
|  | DC_42A_n257A | CA_19A-42C | CA_n257H | 42A | n257A | No |
|  | DC_42A_n257D | CA_19A-42C | CA_n257H | 42A | CA_n257D | No |
|  | DC_42A_n257G | CA_19A-42C | CA_n257H | 42A | CA_n257G | No |
|  | DC_42A_n257H | CA_19A-42C | CA_n257H | 42A | CA_n257H | No |
| DC_19A-42C_n257I | DC_19A_n257A | CA_19A-42C | CA_n257I | 19A | n257A | No |
|  | DC_19A_n257D | CA_19A-42C | CA_n257I | 19A | CA_n257D | No |
|  | DC_19A_n257G | CA_19A-42C | CA_n257I | 19A | CA_n257G | No |
|  | DC_19A_n257H | CA_19A-42C | CA_n257I | 19A | CA_n257H | No |
|  | DC_19A_n257I | CA_19A-42C | CA_n257I | 19A | CA_n257I | No |
|  | DC_42A_n257A | CA_19A-42C | CA_n257I | 42A | n257A | No |
|  | DC_42A_n257D | CA_19A-42C | CA_n257I | 42A | CA_n257D | No |
|  | DC_42A_n257G | CA_19A-42C | CA_n257I | 42A | CA_n257G | No |
|  | DC_42A_n257H | CA_19A-42C | CA_n257I | 42A | CA_n257H | No |
|  | DC_42A_n257I | CA_19A-42C | CA_n257I | 42A | CA_n257I | No |
| DC_21A-42A_n257A | DC_21A_n257A | CA_21A-42A | n257A | 21A | n257A | No |
|  | DC_42A_n257A |  |  | 42A | n257A |  |
| DC_21A-42A_n257G | DC_21A_n257A | CA_21A-42A | CA_n257G | 21A | n257A | No |
|  | DC_21A_n257D | CA_21A-42A | CA_n257G | 21A | CA_n257D | No |
|  | DC_42A_n257A | CA_21A-42A | CA_n257G | 42A | n257A | No |
|  | DC_42A_n257D | CA_21A-42A | CA_n257G | 42A | CA_n257D | No |
|  | DC_21A_n257G | CA_21A-42A | CA_n257G | 21A | CA_n257G | No |
| DC_21A-42A_n257H | DC_21A_n257A | CA_21A-42A | CA_n257H | 21A | n257A | No |
|  | DC_21A_n257D | CA_21A-42A | CA_n257H | 21A | CA_n257D | No |
|  | DC_42A_n257A | CA_21A-42A | CA_n257H | 42A | n257A | No |
|  | DC_42A_n257D | CA_21A-42A | CA_n257H | 42A | CA_n257D | No |
|  | DC_21A_n257G | CA_21A-42A | CA_n257H | 21A | CA_n257G | No |
|  | DC_21A_n257H | CA_21A-42A | CA_n257H | 21A | CA_n257H | No |
| DC_21A-42A_n257I | DC_21A_n257A | CA_21A-42A | CA_n257I | 21A | n257A | No |
|  | DC_21A_n257D | CA_21A-42A | CA_n257I | 21A | CA_n257D | No |
|  | DC_42A_n257A | CA_21A-42A | CA_n257I | 42A | n257A | No |
|  | DC_42A_n257D | CA_21A-42A | CA_n257I | 42A | CA_n257D | No |
|  | DC_21A_n257G | CA_21A-42A | CA_n257I | 21A | CA_n257G | No |
|  | DC_21A_n257H | CA_21A-42A | CA_n257I | 21A | CA_n257H | No |
|  | DC_21A_n257I | CA_21A-42A | CA_n257I | 21A | CA_n257I | No |
| DC_21A-42C_n257A | DC_21A_n257A | CA_21A-42C | n257A | 21A | n257A | No |
|  | DC_42A_n257A |  |  | 42A | n257A |  |
| DC_21A-42C_n257G | DC_21A_n257A | CA_21A-42C | CA_n257G | 21A | n257A | No |
|  | DC_21A_n257D | CA_21A-42C | CA_n257G | 21A | CA_n257D | No |
|  | DC_42A_n257A | CA_21A-42C | CA_n257G | 42A | n257A | No |
|  | DC_42A_n257D | CA_21A-42C | CA_n257G | 42A | CA_n257D | No |
|  | DC_21A_n257G | CA_21A-42C | CA_n257G | 21A | CA_n257G | No |
| DC_21A-42C_n257H | DC_21A_n257A | CA_21A-42C | CA_n257H | 21A | n257A | No |
|  | DC_21A_n257D | CA_21A-42C | CA_n257H | 21A | CA_n257D | No |
|  | DC_42A_n257A | CA_21A-42C | CA_n257H | 42A | n257A | No |
|  | DC_42A_n257D | CA_21A-42C | CA_n257H | 42A | CA_n257D | No |
|  | DC_21A_n257G | CA_21A-42C | CA_n257H | 21A | CA_n257G | No |
|  | DC_21A_n257H | CA_21A-42C | CA_n257H | 21A | CA_n257H | No |
| DC_21A-42C_n257I | DC_21A_n257A | CA_21A-42C | CA_n257I | 21A | n257A | No |
|  | DC_21A_n257D | CA_21A-42C | CA_n257I | 21A | CA_n257D | No |
|  | DC_42A_n257A | CA_21A-42C | CA_n257I | 42A | n257A | No |
|  | DC_42A_n257D | CA_21A-42C | CA_n257I | 42A | CA_n257D | No |
|  | DC_21A_n257G | CA_21A-42C | CA_n257I | 21A | CA_n257G | No |
|  | DC_21A_n257H | CA_21A-42C | CA_n257I | 21A | CA_n257H | No |
|  | DC_21A_n257I | CA_21A-42C | CA_n257I | 21A | CA_n257I | No |
| Note 1: Protocol testing is limited to EN-DC configurations with 1 CC E-UTRA and 1CC or 2CC NR configurations. |  |  |  |  |  |  |

4.3.1.5.1.4	Inter-band EN-DC configurations including FR2 (four bands)

Table 4.3.1.5.1.4-1: Inter-band EN-DC configurations including FR2 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration | E-UTRA downlink configuration | NR downlink configuration | E-UTRA uplink configuration | NR uplink configuration | Applicable for protocol testing (Note 1) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_1A-3A-18A_n257A | DC_1A_n257A | CA_1A-3A-18A | n257A | 1A | n257A | No |
|  | DC_3A_n257A | CA_1A-3A-18A | n257A | 3A | n257A | No |
|  | DC_18A_n257A | CA_1A-3A-18A | n257A | 18A | n257A | No |
| DC_1A-3A-18A_n257I | DC_1A_n257I | CA_1A-3A-18A | CA_n257I | 1A | CA_n257I | No |
|  | DC_3A_n257I | CA_1A-3A-18A | CA_n257I | 3A | CA_n257I | No |
|  |  | DC_18A_n257I |  | CA_1A-3A-18A |  | CA_n257I |  | 18A |  | CA_n257I |  | No |  |
| DC_1A-3A-19A_n257A |  | DC_1A_n257A |  | CA_1A-3A-19A |  | n257A |  | 1A |  | n257A |  | No |  |
|  |  | DC_3A_n257A |  |  |  |  |  | 3A |  | n257A |  |  |  |
|  |  | DC_19A_n257A |  |  |  |  |  | 19A |  | n257A |  |  |  |
| DC_1A-3A-19A_n257G | DC_1A_n257A | CA_1A-3A-19A | CA_n257G | 1A | n257A | No |
|  | DC_3A_n257A | CA_1A-3A-19A | CA_n257G | 3A | n257A | No |
|  | DC_3A_n257A | CA_1A-3A-19A | CA_n257G | 3A | n257A | No |
|  | DC_3A_n257G | CA_1A-3A-19A | CA_n257G | 3A | CA_n257G | No |
|  | DC_19A_n257A | CA_1A-3A-19A | CA_n257G | 19A | n257A | No |
| DC_1A-3A-19A_n257H | DC_1A_n257A | CA_1A-3A-19A | CA_n257H | 1A | n257A | No |
|  | DC_3A_n257A | CA_1A-3A-19A | CA_n257H | 3A | n257A | No |
|  | DC_3A_n257A | CA_1A-3A-19A | CA_n257H | 3A | n257A | No |
|  | DC_3A_n257G | CA_1A-3A-19A | CA_n257H | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_1A-3A-19A | CA_n257H | 3A | CA_n257H | No |
|  | DC_19A_n257A | CA_1A-3A-19A | CA_n257H | 19A | n257A | No |
| DC_1A-3A-19A_n257I | DC_1A_n257A | CA_1A-3A-19A | CA_n257I | 1A | n257A | No |
|  | DC_3A_n257A | CA_1A-3A-19A | CA_n257I | 3A | n257A | No |
|  | DC_3A_n257A | CA_1A-3A-19A | CA_n257I | 3A | n257A | No |
|  | DC_3A_n257G | CA_1A-3A-19A | CA_n257I | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_1A-3A-19A | CA_n257I | 3A | CA_n257H | No |
|  | DC_3A_n257I | CA_1A-3A-19A | CA_n257I | 3A | CA_n257I | No |
|  |  | DC_19A_n257A |  | CA_1A-3A-19A |  | CA_n257I |  | 19A |  | n257A |  | No |  |
| DC_1A-3A-21A_n257A |  | DC_1A_n257A |  | CA_1A-3A-21A |  | n257A |  | 1A |  | n257A |  | No |  |
|  |  | DC_3A_n257A |  |  |  |  |  | 3A |  | n257A |  |  |  |
|  |  | DC_21A_n257A |  |  |  |  |  | 21A |  | n257A |  |  |  |
| DC_1A-3A-21A_n257G | DC_1A_n257A | CA_1A-3A-21A | CA_n257G | 1A | n257A | No |
|  | DC_3A_n257A | CA_1A-3A-21A | CA_n257G | 3A | n257A | No |
|  | DC_3A_n257G | CA_1A-3A-21A | CA_n257G | 3A | CA_n257G | No |
|  | DC_21A_n257A | CA_1A-3A-21A | CA_n257G | 21A | n257A | No |
| DC_1A-3A-21A_n257H | DC_1A_n257A | CA_1A-3A-21A | CA_n257H | 1A | n257A | No |
|  | DC_3A_n257A | CA_1A-3A-21A | CA_n257H | 3A | n257A | No |
|  | DC_3A_n257G | CA_1A-3A-21A | CA_n257H | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_1A-3A-21A | CA_n257H | 3A | CA_n257H | No |
|  | DC_21A_n257A | CA_1A-3A-21A | CA_n257H | 21A | n257A | No |
| DC_1A-3A-21A_n257I | DC_1A_n257A | CA_1A-3A-21A | CA_n257I | 1A | n257A | No |
|  | DC_3A_n257A | CA_1A-3A-21A | CA_n257I | 3A | n257A | No |
|  | DC_3A_n257G | CA_1A-3A-21A | CA_n257I | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_1A-3A-21A | CA_n257I | 3A | CA_n257H | No |
|  | DC_3A_n257I | CA_1A-3A-21A | CA_n257I | 3A | CA_n257I | No |
|  | DC_21A_n257A | CA_1A-3A-21A | CA_n257I | 21A | n257A | No |
| DC_1A-3A-41A_n257A | DC_1A_n257A | CA_1A-3A-41A | n257A | 1A | n257A | No |
|  | DC_3A_n257A | CA_1A-3A-41A | n257A | 3A | n257A | No |
|  | DC_41A_n257A | CA_1A-3A-41A | n257A | 41A | n257A | No |
| DC_1A-3A-41A_n257I | DC_1A_n257I | CA_1A-3A-41A | CA_n257I | 1A | CA_n257I | No |
|  | DC_3A_n257I | CA_1A-3A-41A | CA_n257I | 3A | CA_n257I | No |
|  |  | DC_41A_n257I |  | CA_1A-3A-41A |  | CA_n257I |  | 41A |  | CA_n257I |  | No |  |
| DC_1A-3A-42A_n257A |  | DC_1A_n257A |  | CA_1A-3A-42A |  | n257A |  | 1A |  | n257A |  | No |  |
|  |  | DC_3A_n257A |  |  |  |  |  | 3A |  | n257A |  |  |  |
|  |  | DC_42A_n257A |  |  |  |  |  | 42A |  | n257A |  |  |  |
| DC_1A-3A-42A_n257G | DC_1A_n257A | CA_1A-3A-42A | CA_n257G | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-3A-42A | CA_n257G | 1A | CA_n257G | No |
|  | DC_3A_n257A | CA_1A-3A-42A | CA_n257G | 3A | n257A | No |
|  | DC_3A_n257G | CA_1A-3A-42A | CA_n257G | 3A | CA_n257G | No |
| DC_1A-3A-42A_n257H | DC_1A_n257A | CA_1A-3A-42A | CA_n257H | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-3A-42A | CA_n257H | 1A | CA_n257G | No |
|  | DC_1A_n257H | CA_1A-3A-42A | CA_n257H | 1A | CA_n257H | No |
|  | DC_3A_n257A | CA_1A-3A-42A | CA_n257H | 3A | n257A | No |
|  | DC_3A_n257G | CA_1A-3A-42A | CA_n257H | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_1A-3A-42A | CA_n257H | 3A | CA_n257H | No |
| DC_1A-3A-42A_n257I | DC_1A_n257A | CA_1A-3A-42A | CA_n257I | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-3A-42A | CA_n257I | 1A | CA_n257G | No |
|  | DC_1A_n257H | CA_1A-3A-42A | CA_n257I | 1A | CA_n257H | No |
|  | DC_1A_n257I | CA_1A-3A-42A | CA_n257I | 1A | CA_n257I | No |
|  | DC_3A_n257A | CA_1A-3A-42A | CA_n257I | 3A | n257A | No |
|  | DC_3A_n257G | CA_1A-3A-42A | CA_n257I | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_1A-3A-42A | CA_n257I | 3A | CA_n257H | No |
|  |  | DC_3A_n257I |  | CA_1A-3A-42A |  | CA_n257I |  | 3A |  | CA_n257I |  | No |  |
| DC_1A-3A-42C_n257A |  | DC_1A_n257A |  | CA_1A-3A-42C |  | n257A |  | 1A |  | n257A |  | No |  |
|  |  | DC_3A_n257A |  |  |  |  |  | 3A |  | n257A |  |  |  |
|  |  | DC_42A_n257A |  |  |  |  |  | 42A |  | n257A |  |  |  |
| DC_1A-3A-42C_n257G | DC_1A_n257A | CA_1A-3A-42C | CA_n257G | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-3A-42C | CA_n257G | 1A | CA_n257G | No |
|  | DC_3A_n257A | CA_1A-3A-42C | CA_n257G | 3A | n257A | No |
|  | DC_3A_n257G | CA_1A-3A-42C | CA_n257G | 3A | CA_n257G | No |
| DC_1A-3A-42C_n257H | DC_1A_n257A | CA_1A-3A-42C | CA_n257H | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-3A-42C | CA_n257H | 1A | CA_n257G | No |
|  | DC_1A_n257H | CA_1A-3A-42C | CA_n257H | 1A | CA_n257H | No |
|  | DC_3A_n257A | CA_1A-3A-42C | CA_n257H | 3A | n257A | No |
|  | DC_3A_n257G | CA_1A-3A-42C | CA_n257H | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_1A-3A-42C | CA_n257H | 3A | CA_n257H | No |
| DC_1A-3A-42C_n257I | DC_1A_n257A | CA_1A-3A-42C | CA_n257I | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-3A-42C | CA_n257I | 1A | CA_n257G | No |
|  | DC_1A_n257H | CA_1A-3A-42C | CA_n257I | 1A | CA_n257H | No |
|  | DC_1A_n257I | CA_1A-3A-42C | CA_n257I | 1A | CA_n257I | No |
|  | DC_3A_n257A | CA_1A-3A-42C | CA_n257I | 3A | n257A | No |
|  | DC_3A_n257G | CA_1A-3A-42C | CA_n257I | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_1A-3A-42C | CA_n257I | 3A | CA_n257H | No |
|  | DC_3A_n257I | CA_1A-3A-42C | CA_n257I | 3A | CA_n257I | No |
| DC_1A-3A-42D_n257G | DC_1A_n257A | CA_1A-3A-42D | CA_n257G | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-3A-42D | CA_n257G | 1A | CA_n257G | No |
|  | DC_3A_n257A | CA_1A-3A-42D | CA_n257G | 3A | n257A | No |
|  | DC_3A_n257G | CA_1A-3A-42D | CA_n257G | 3A | CA_n257G | No |
| DC_1A-3A-42D_n257H | DC_1A_n257A | CA_1A-3A-42D | CA_n257H | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-3A-42D | CA_n257H | 1A | CA_n257G | No |
|  | DC_1A_n257H | CA_1A-3A-42D | CA_n257H | 1A | CA_n257H | No |
|  | DC_3A_n257A | CA_1A-3A-42D | CA_n257H | 3A | n257A | No |
|  | DC_3A_n257G | CA_1A-3A-42D | CA_n257H | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_1A-3A-42D | CA_n257H | 3A | CA_n257H | No |
| DC_1A-3A-42D_n257I | DC_1A_n257A | CA_1A-3A-42D | CA_n257I | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-3A-42D | CA_n257I | 1A | CA_n257G | No |
|  | DC_1A_n257H | CA_1A-3A-42D | CA_n257I | 1A | CA_n257H | No |
|  | DC_1A_n257I | CA_1A-3A-42D | CA_n257I | 1A | CA_n257I | No |
|  | DC_3A_n257A | CA_1A-3A-42D | CA_n257I | 3A | n257A | No |
|  | DC_3A_n257G | CA_1A-3A-42D | CA_n257I | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_1A-3A-42D | CA_n257I | 3A | CA_n257H | No |
|  |  | DC_3A_n257I |  | CA_1A-3A-42D |  | CA_n257I |  | 3A |  | CA_n257I |  | No |  |
| DC_1A-19A-21A_n257A |  | DC_1A_n257A |  | CA_1A-19A-21A |  | n257A |  | 1A |  | n257A |  | No |  |
|  |  | DC_19A_n257A |  |  |  |  |  | 19A |  | n257A |  |  |  |
|  |  | DC_21A_n257A |  |  |  |  |  | 21A |  | n257A |  |  |  |
| DC_1A-19A-21A_n257G | DC_1A_n257A | CA_1A-19A-21A | CA_n257G | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-19A-21A | CA_n257G | 1A | CA_n257G | No |
|  | DC_19A_n257A | CA_1A-19A-21A | CA_n257G | 19A | n257A | No |
|  | DC_21A_n257A | CA_1A-19A-21A | CA_n257G | 21A | n257A | No |
|  | DC_21A_n257G | CA_1A-19A-21A | CA_n257G | 21A | CA_n257G | No |
| DC_1A-19A-21A_n257H | DC_1A_n257A | CA_1A-19A-21A | CA_n257H | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-19A-21A | CA_n257H | 1A | CA_n257G | No |
|  | DC_1A_n257H | CA_1A-19A-21A | CA_n257H | 1A | CA_n257H | No |
|  | DC_19A_n257A | CA_1A-19A-21A | CA_n257H | 19A | n257A | No |
|  | DC_21A_n257A | CA_1A-19A-21A | CA_n257H | 21A | n257A | No |
|  | DC_21A_n257G | CA_1A-19A-21A | CA_n257H | 21A | CA_n257G | No |
|  | DC_21A_n257H | CA_1A-19A-21A | CA_n257H | 21A | CA_n257H | No |
| DC_1A-19A-21A_n257I | DC_1A_n257A | CA_1A-19A-21A | CA_n257I | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-19A-21A | CA_n257I | 1A | CA_n257G | No |
|  | DC_1A_n257H | CA_1A-19A-21A | CA_n257I | 1A | CA_n257H | No |
|  | DC_1A_n257I | CA_1A-19A-21A | CA_n257I | 1A | CA_n257I | No |
|  | DC_19A_n257A | CA_1A-19A-21A | CA_n257I | 19A | n257A | No |
|  | DC_21A_n257A | CA_1A-19A-21A | CA_n257I | 21A | n257A | No |
|  | DC_21A_n257G | CA_1A-19A-21A | CA_n257I | 21A | CA_n257G | No |
|  | DC_21A_n257H | CA_1A-19A-21A | CA_n257I | 21A | CA_n257H | No |
|  |  | DC_21A_n257I |  | CA_1A-19A-21A |  | CA_n257I |  | 21A |  | CA_n257I |  | No |  |
| DC_1A-19A-42A_n257A |  | DC_1A_n257A |  | CA_1A-19A-42A |  | n257A |  | 1A |  | n257A |  | No |  |
|  |  | DC_19A_n257A |  |  |  |  |  | 19A |  | n257A |  |  |  |
|  |  | DC_42A_n257A |  |  |  |  |  | 42A |  | n257A |  |  |  |
| DC_1A-19A-42A_n257G | DC_1A_n257A | CA_1A-19A-42A | CA_n257G | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-19A-42A | CA_n257G | 1A | CA_n257G | No |
|  | DC_19A_n257A | CA_1A-19A-42A | CA_n257G | 19A | n257A | No |
|  | DC_42A_n257A | CA_1A-19A-42A | CA_n257G | 42A | n257A | No |
| DC_1A-19A-42A_n257H | DC_1A_n257A | CA_1A-19A-42A | CA_n257H | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-19A-42A | CA_n257H | 1A | CA_n257G | No |
|  | DC_1A_n257H | CA_1A-19A-42A | CA_n257H | 1A | CA_n257H | No |
|  | DC_19A_n257A | CA_1A-19A-42A | CA_n257H | 19A | n257A | No |
|  | DC_42A_n257A | CA_1A-19A-42A | CA_n257H | 42A | n257A | No |
| DC_1A-19A-42A_n257I | DC_1A_n257A | CA_1A-19A-42A | CA_n257I | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-19A-42A | CA_n257I | 1A | CA_n257G | No |
|  | DC_1A_n257H | CA_1A-19A-42A | CA_n257I | 1A | CA_n257H | No |
|  | DC_1A_n257I | CA_1A-19A-42A | CA_n257I | 1A | CA_n257I | No |
|  | DC_19A_n257A | CA_1A-19A-42A | CA_n257I | 19A | n257A | No |
|  |  | DC_42A_n257A |  | CA_1A-19A-42A |  | CA_n257I |  | 42A |  | n257A |  | No |  |
| DC_1A-19A-42C_n257A |  | DC_1A_n257A |  | CA_1A-19A-42C |  | n257A |  | 1A |  | n257A |  | No |  |
|  |  | DC_19A_n257A |  |  |  |  |  | 19A |  | n257A |  |  |  |
|  |  | DC_42A_n257A |  |  |  |  |  | 42A |  | n257A |  |  |  |
| DC_1A-19A-42C_n257G | DC_1A_n257A | CA_1A-19A-42C | CA_n257G | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-19A-42C | CA_n257G | 1A | CA_n257G | No |
|  | DC_19A_n257A | CA_1A-19A-42C | CA_n257G | 19A | n257A | No |
|  | DC_42A_n257A | CA_1A-19A-42C | CA_n257G | 42A | n257A | No |
| DC_1A-19A-42C_n257H | DC_1A_n257A | CA_1A-19A-42C | CA_n257H | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-19A-42C | CA_n257H | 1A | CA_n257G | No |
|  | DC_1A_n257H | CA_1A-19A-42C | CA_n257H | 1A | CA_n257H | No |
|  | DC_19A_n257A | CA_1A-19A-42C | CA_n257H | 19A | n257A | No |
|  | DC_42A_n257A | CA_1A-19A-42C | CA_n257H | 42A | n257A | No |
| DC_1A-19A-42C_n257I | DC_1A_n257A | CA_1A-19A-42C | CA_n257I | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-19A-42C | CA_n257I | 1A | CA_n257G | No |
|  | DC_1A_n257H | CA_1A-19A-42C | CA_n257I | 1A | CA_n257H | No |
|  | DC_1A_n257I | CA_1A-19A-42C | CA_n257I | 1A | CA_n257I | No |
|  | DC_19A_n257A | CA_1A-19A-42C | CA_n257I | 19A | n257A | No |
|  |  | DC_42A_n257A |  | CA_1A-19A-42C |  | CA_n257I |  | 42A |  | n257A |  | No |  |
| DC_1A-21A-42A_n257A |  | DC_1A_n257A |  | CA_1A-21A-42A |  | n257A |  | 1A |  | n257A |  | No |  |
|  |  | DC_21A_n257A |  |  |  |  |  | 21A |  | n257A |  |  |  |
|  |  | DC_42A_n257A |  |  |  |  |  | 42A |  | n257A |  |  |  |
| DC_1A-21A-42A_n257G | DC_1A_n257A | CA_1A-21A-42A | CA_n257G | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-21A-42A | CA_n257G | 1A | CA_n257G | No |
|  | DC_21A_n257A | CA_1A-21A-42A | CA_n257G | 21A | n257A | No |
|  | DC_21A_n257G | CA_1A-21A-42A | CA_n257G | 21A | CA_n257G | No |
|  | DC_42A_n257A | CA_1A-21A-42A | CA_n257G | 42A | n257A | No |
|  | DC_42A_n257D | CA_1A-21A-42A | CA_n257G | 42A | CA_n257D | No |
| DC_1A-21A-42A_n257H | DC_1A_n257A | CA_1A-21A-42A | CA_n257H | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-21A-42A | CA_n257H | 1A | CA_n257G | No |
|  | DC_1A_n257H | CA_1A-21A-42A | CA_n257H | 1A | CA_n257H | No |
|  | DC_21A_n257A | CA_1A-21A-42A | CA_n257H | 21A | n257A | No |
|  | DC_21A_n257G | CA_1A-21A-42A | CA_n257H | 21A | CA_n257G | No |
|  | DC_21A_n257H | CA_1A-21A-42A | CA_n257H | 21A | CA_n257H | No |
|  | DC_42A_n257A | CA_1A-21A-42A | CA_n257H | 42A | n257A | No |
|  | DC_42A_n257D | CA_1A-21A-42A | CA_n257H | 42A | CA_n257D | No |
| DC_1A-21A-42A_n257I | DC_1A_n257A | CA_1A-21A-42A | CA_n257I | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-21A-42A | CA_n257I | 1A | CA_n257G | No |
|  | DC_1A_n257H | CA_1A-21A-42A | CA_n257I | 1A | CA_n257H | No |
|  | DC_1A_n257I | CA_1A-21A-42A | CA_n257I | 1A | CA_n257I | No |
|  | DC_21A_n257A | CA_1A-21A-42A | CA_n257I | 21A | n257A | No |
|  | DC_21A_n257G | CA_1A-21A-42A | CA_n257I | 21A | CA_n257G | No |
|  | DC_21A_n257H | CA_1A-21A-42A | CA_n257I | 21A | CA_n257H | No |
|  | DC_21A_n257I | CA_1A-21A-42A | CA_n257I | 21A | CA_n257I | No |
|  | DC_42A_n257A | CA_1A-21A-42A | CA_n257I | 42A | n257A | No |
|  |  | DC_42A_n257D |  | CA_1A-21A-42A |  | CA_n257I |  | 42A |  | CA_n257D |  | No |  |
| DC_1A-21A-42C_n257A |  | DC_1A_n257A |  | CA_1A-21A-42C |  | n257A |  | 1A |  | n257A |  | No |  |
|  |  | DC_21A_n257A |  |  |  |  |  | 21A |  | n257A |  |  |  |
|  | DC_42A_n257A | CA_1A-21A-42C | n257A | 42A | n257A | No |
| DC_1A-21A-42C_n257G | DC_1A_n257A | CA_1A-21A-42C | CA_n257G | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-21A-42C | CA_n257G | 1A | CA_n257G | No |
|  | DC_21A_n257A | CA_1A-21A-42C | CA_n257G | 21A | n257A | No |
|  | DC_21A_n257G | CA_1A-21A-42C | CA_n257G | 21A | CA_n257G | No |
|  | DC_42A_n257A | CA_1A-21A-42C | CA_n257G | 42A | n257A | No |
|  | DC_42A_n257D | CA_1A-21A-42C | CA_n257G | 42A | CA_n257D | No |
| DC_1A-21A-42C_n257H | DC_1A_n257A | CA_1A-21A-42C | CA_n257H | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-21A-42C | CA_n257H | 1A | CA_n257G | No |
|  | DC_1A_n257H | CA_1A-21A-42C | CA_n257H | 1A | CA_n257H | No |
|  | DC_21A_n257A | CA_1A-21A-42C | CA_n257H | 21A | n257A | No |
|  | DC_21A_n257G | CA_1A-21A-42C | CA_n257H | 21A | CA_n257G | No |
|  | DC_21A_n257H | CA_1A-21A-42C | CA_n257H | 21A | CA_n257H | No |
|  | DC_42A_n257A | CA_1A-21A-42C | CA_n257H | 42A | n257A | No |
|  | DC_42A_n257D | CA_1A-21A-42C | CA_n257H | 42A | CA_n257D | No |
| DC_1A-21A-42C_n257I | DC_1A_n257A | CA_1A-21A-42C | CA_n257I | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-21A-42C | CA_n257I | 1A | CA_n257G | No |
|  | DC_1A_n257H | CA_1A-21A-42C | CA_n257I | 1A | CA_n257H | No |
|  | DC_1A_n257I | CA_1A-21A-42C | CA_n257I | 1A | CA_n257I | No |
|  | DC_21A_n257A | CA_1A-21A-42C | CA_n257I | 21A | n257A | No |
|  | DC_21A_n257G | CA_1A-21A-42C | CA_n257I | 21A | CA_n257G | No |
|  | DC_21A_n257H | CA_1A-21A-42C | CA_n257I | 21A | CA_n257H | No |
|  | DC_21A_n257I | CA_1A-21A-42C | CA_n257I | 21A | CA_n257I | No |
|  | DC_42A_n257A | CA_1A-21A-42C | CA_n257I | 42A | n257A | No |
|  | DC_42A_n257D | CA_1A-21A-42C | CA_n257I | 42A | CA_n257D | No |
| DC_2A-2A-14A-66A_n260A | DC_2A_n260A | CA_2A-2A-14A-66A | n260A | 2A | n260A | No |
|  | DC_14A_n260A | CA_2A-2A-14A-66A | n260A | 14A | n260A | No |
|  | DC_66A_n260A | CA_2A-2A-14A-66A | n260A | 66A | n260A | No |
| DC_2A-2A-14A-66A_n260G | DC_2A_n260A | CA_2A-2A-14A-66A | CA_n260G | 2A | n260A | No |
|  | DC_2A_n260G | CA_2A-2A-14A-66A | CA_n260G | 2A | CA_n260G | No |
|  | DC_14A_n260A | CA_2A-2A-14A-66A | CA_n260G | 14A | n260A | No |
|  | DC_14A_n260G | CA_2A-2A-14A-66A | CA_n260G | 14A | CA_n260G | No |
|  | DC_66A_n260A | CA_2A-2A-14A-66A | CA_n260G | 66A | n260A | No |
|  | DC_66A_n260G | CA_2A-2A-14A-66A | CA_n260G | 66A | CA_n260G | No |
| DC_2A-2A-14A-66A_n260H | DC_2A_n260A | CA_2A-2A-14A-66A | CA_n260H | 2A | n260A | No |
|  | DC_2A_n260G | CA_2A-2A-14A-66A | CA_n260H | 2A | CA_n260G | No |
|  | DC_2A_n260H | CA_2A-2A-14A-66A | CA_n260H | 2A | CA_n260H | No |
|  | DC_14A_n260A | CA_2A-2A-14A-66A | CA_n260H | 14A | n260A | No |
|  | DC_14A_n260G | CA_2A-2A-14A-66A | CA_n260H | 14A | CA_n260G | No |
|  | DC_14A_n260H | CA_2A-2A-14A-66A | CA_n260H | 14A | CA_n260H | No |
|  | DC_66A_n260A | CA_2A-2A-14A-66A | CA_n260H | 66A | n260A | No |
|  | DC_66A_n260G | CA_2A-2A-14A-66A | CA_n260H | 66A | CA_n260G | No |
|  | DC_66A_n260H | CA_2A-2A-14A-66A | CA_n260H | 66A | CA_n260H | No |
| DC_2A-2A-14A-66A_n260I | DC_2A_n260A | CA_2A-2A-14A-66A | CA_n260I | 2A | n260A | No |
|  | DC_2A_n260G | CA_2A-2A-14A-66A | CA_n260I | 2A | CA_n260G | No |
|  | DC_2A_n260H | CA_2A-2A-14A-66A | CA_n260I | 2A | CA_n260H | No |
|  | DC_2A_n260I | CA_2A-2A-14A-66A | CA_n260I | 2A | CA_n260I | No |
|  | DC_14A_n260A | CA_2A-2A-14A-66A | CA_n260I | 14A | n260A | No |
|  | DC_14A_n260G | CA_2A-2A-14A-66A | CA_n260I | 14A | CA_n260G | No |
|  | DC_14A_n260H | CA_2A-2A-14A-66A | CA_n260I | 14A | CA_n260H | No |
|  | DC_14A_n260I | CA_2A-2A-14A-66A | CA_n260I | 14A | CA_n260I | No |
|  | DC_66A_n260A | CA_2A-2A-14A-66A | CA_n260I | 66A | n260A | No |
|  | DC_66A_n260G | CA_2A-2A-14A-66A | CA_n260I | 66A | CA_n260G | No |
|  | DC_66A_n260H | CA_2A-2A-14A-66A | CA_n260I | 66A | CA_n260H | No |
|  | DC_66A_n260I | CA_2A-2A-14A-66A | CA_n260I | 66A | CA_n260I | No |
| DC_2A-14A-30A_n260A | DC_2A_n260A | CA_2A-14A-30A | n260A | 2A | n260A | No |
|  | DC_14A_n260A | CA_2A-14A-30A | n260A | 14A | n260A | No |
|  | DC_30A_n260A | CA_2A-14A-30A | n260A | 30A | n260A | No |
| DC_2A-14A-30A_n260G | DC_2A_n260A | CA_2A-14A-30A | CA_n260G | 2A | n260A | No |
|  | DC_2A_n260G | CA_2A-14A-30A | CA_n260G | 2A | CA_n260G | No |
|  | DC_14A_n260A | CA_2A-14A-30A | CA_n260G | 14A | n260A | No |
|  | DC_14A_n260G | CA_2A-14A-30A | CA_n260G | 14A | CA_n260G | No |
|  | DC_30A_n260A | CA_2A-14A-30A | CA_n260G | 30A | n260A | No |
|  | DC_30A_n260G | CA_2A-14A-30A | CA_n260G | 30A | CA_n260G | No |
| DC_2A-14A-30A_n260H | DC_2A_n260A | CA_2A-14A-30A | CA_n260H | 2A | n260A | No |
|  | DC_2A_n260G | CA_2A-14A-30A | CA_n260H | 2A | CA_n260G | No |
|  | DC_2A_n260H | CA_2A-14A-30A | CA_n260H | 2A | CA_n260H | No |
|  | DC_14A_n260A | CA_2A-14A-30A | CA_n260H | 14A | n260A | No |
|  | DC_14A_n260G | CA_2A-14A-30A | CA_n260H | 14A | CA_n260G | No |
|  | DC_14A_n260H | CA_2A-14A-30A | CA_n260H | 14A | CA_n260H | No |
|  | DC_30A_n260A | CA_2A-14A-30A | CA_n260H | 30A | n260A | No |
|  | DC_30A_n260G | CA_2A-14A-30A | CA_n260H | 30A | CA_n260G | No |
|  | DC_30A_n260H | CA_2A-14A-30A | CA_n260H | 30A | CA_n260H | No |
| DC_2A-14A-30A_n260I | DC_2A_n260A | CA_2A-14A-30A | CA_n260I | 2A | n260A | No |
|  | DC_2A_n260G | CA_2A-14A-30A | CA_n260I | 2A | CA_n260G | No |
|  | DC_2A_n260H | CA_2A-14A-30A | CA_n260I | 2A | CA_n260H | No |
|  | DC_2A_n260I | CA_2A-14A-30A | CA_n260I | 2A | CA_n260I | No |
|  | DC_14A_n260A | CA_2A-14A-30A | CA_n260I | 14A | n260A | No |
|  | DC_14A_n260G | CA_2A-14A-30A | CA_n260I | 14A | CA_n260G | No |
|  | DC_14A_n260H | CA_2A-14A-30A | CA_n260I | 14A | CA_n260H | No |
|  | DC_14A_n260I | CA_2A-14A-30A | CA_n260I | 14A | CA_n260I | No |
|  | DC_30A_n260A | CA_2A-14A-30A | CA_n260I | 30A | n260A | No |
|  | DC_30A_n260G | CA_2A-14A-30A | CA_n260I | 30A | CA_n260G | No |
|  | DC_30A_n260H | CA_2A-14A-30A | CA_n260I | 30A | CA_n260H | No |
|  | DC_30A_n260I | CA_2A-14A-30A | CA_n260I | 30A | CA_n260I | No |
| DC_2A-14A-66A_n260A | DC_2A_n260A | CA_2A-14A-66A | n260A | 2A | n260A | No |
|  | DC_14A_n260A | CA_2A-14A-66A | n260A | 14A | n260A | No |
|  | DC_66A_n260A | CA_2A-14A-66A | n260A | 66A | n260A | No |
| DC_2A-14A-66A_n260G | DC_2A_n260A | CA_2A-14A-66A | CA_n260G | 2A | n260A | No |
|  | DC_2A_n260G | CA_2A-14A-66A | CA_n260G | 2A | CA_n260G | No |
|  | DC_14A_n260A | CA_2A-14A-66A | CA_n260G | 14A | n260A | No |
|  | DC_14A_n260G | CA_2A-14A-66A | CA_n260G | 14A | CA_n260G | No |
|  | DC_66A_n260A | CA_2A-14A-66A | CA_n260G | 66A | n260A | No |
|  | DC_66A_n260G | CA_2A-14A-66A | CA_n260G | 66A | CA_n260G | No |
| DC_2A-14A-66A_n260H | DC_2A_n260A | CA_2A-14A-66A | CA_n260H | 2A | n260A | No |
|  | DC_2A_n260G | CA_2A-14A-66A | CA_n260H | 2A | CA_n260G | No |
|  | DC_2A_n260H | CA_2A-14A-66A | CA_n260H | 2A | CA_n260H | No |
|  | DC_14A_n260A | CA_2A-14A-66A | CA_n260H | 14A | n260A | No |
|  | DC_14A_n260G | CA_2A-14A-66A | CA_n260H | 14A | CA_n260G | No |
|  | DC_14A_n260H | CA_2A-14A-66A | CA_n260H | 14A | CA_n260H | No |
|  | DC_66A_n260A | CA_2A-14A-66A | CA_n260H | 66A | n260A | No |
|  | DC_66A_n260G | CA_2A-14A-66A | CA_n260H | 66A | CA_n260G | No |
|  | DC_66A_n260H | CA_2A-14A-66A | CA_n260H | 66A | CA_n260H | No |
| DC_2A-14A-66A_n260I | DC_2A_n260A | CA_2A-14A-66A | CA_n260I | 2A | n260A | No |
|  | DC_2A_n260G | CA_2A-14A-66A | CA_n260I | 2A | CA_n260G | No |
|  | DC_2A_n260H | CA_2A-14A-66A | CA_n260I | 2A | CA_n260H | No |
|  | DC_2A_n260I | CA_2A-14A-66A | CA_n260I | 2A | CA_n260I | No |
|  | DC_14A_n260A | CA_2A-14A-66A | CA_n260I | 14A | n260A | No |
|  | DC_14A_n260G | CA_2A-14A-66A | CA_n260I | 14A | CA_n260G | No |
|  | DC_14A_n260H | CA_2A-14A-66A | CA_n260I | 14A | CA_n260H | No |
|  | DC_14A_n260I | CA_2A-14A-66A | CA_n260I | 14A | CA_n260I | No |
|  | DC_66A_n260A | CA_2A-14A-66A | CA_n260I | 66A | n260A | No |
|  | DC_66A_n260G | CA_2A-14A-66A | CA_n260I | 66A | CA_n260G | No |
|  | DC_66A_n260H | CA_2A-14A-66A | CA_n260I | 66A | CA_n260H | No |
|  | DC_66A_n260I | CA_2A-14A-66A | CA_n260I | 66A | CA_n260I | No |
| DC_2A-14A-66A-66A_n260A | DC_2A_n260A | CA_2A-14A-66A-66A | n260A | 2A | n260A | No |
|  | DC_14A_n260A | CA_2A-14A-66A-66A | n260A | 14A | n260A | No |
|  | DC_66A_n260A | CA_2A-14A-66A-66A | n260A | 66A | n260A | No |
| DC_2A-14A-66A-66A_n260G | DC_2A_n260A | CA_2A-14A-66A-66A | CA_n260G | 2A | n260A | No |
|  | DC_2A_n260G | CA_2A-14A-66A-66A | CA_n260G | 2A | CA_n260G | No |
|  | DC_14A_n260A | CA_2A-14A-66A-66A | CA_n260G | 14A | n260A | No |
|  | DC_14A_n260G | CA_2A-14A-66A-66A | CA_n260G | 14A | CA_n260G | No |
|  | DC_66A_n260A | CA_2A-14A-66A-66A | CA_n260G | 66A | n260A | No |
|  | DC_66A_n260G | CA_2A-14A-66A-66A | CA_n260G | 66A | CA_n260G | No |
| DC_2A-14A-66A-66A_n260H | DC_2A_n260A | CA_2A-14A-66A-66A | CA_n260H | 2A | n260A | No |
|  | DC_2A_n260G | CA_2A-14A-66A-66A | CA_n260H | 2A | CA_n260G | No |
|  | DC_2A_n260H | CA_2A-14A-66A-66A | CA_n260H | 2A | CA_n260H | No |
|  | DC_14A_n260A | CA_2A-14A-66A-66A | CA_n260H | 14A | n260A | No |
|  | DC_14A_n260G | CA_2A-14A-66A-66A | CA_n260H | 14A | CA_n260G | No |
|  | DC_14A_n260H | CA_2A-14A-66A-66A | CA_n260H | 14A | CA_n260H | No |
|  | DC_66A_n260A | CA_2A-14A-66A-66A | CA_n260H | 66A | n260A | No |
|  | DC_66A_n260G | CA_2A-14A-66A-66A | CA_n260H | 66A | CA_n260G | No |
|  | DC_66A_n260H | CA_2A-14A-66A-66A | CA_n260H | 66A | CA_n260H | No |
| DC_2A-14A-66A-66A_n260I | DC_2A_n260A | CA_2A-14A-66A-66A | CA_n260I | 2A | n260A | No |
|  | DC_2A_n260G | CA_2A-14A-66A-66A | CA_n260I | 2A | CA_n260G | No |
|  | DC_2A_n260H | CA_2A-14A-66A-66A | CA_n260I | 2A | CA_n260H | No |
|  | DC_2A_n260I | CA_2A-14A-66A-66A | CA_n260I | 2A | CA_n260I | No |
|  | DC_14A_n260A | CA_2A-14A-66A-66A | CA_n260I | 14A | n260A | No |
|  | DC_14A_n260G | CA_2A-14A-66A-66A | CA_n260I | 14A | CA_n260G | No |
|  | DC_14A_n260H | CA_2A-14A-66A-66A | CA_n260I | 14A | CA_n260H | No |
|  | DC_14A_n260I | CA_2A-14A-66A-66A | CA_n260I | 14A | CA_n260I | No |
|  | DC_66A_n260A | CA_2A-14A-66A-66A | CA_n260I | 66A | n260A | No |
|  | DC_66A_n260G | CA_2A-14A-66A-66A | CA_n260I | 66A | CA_n260G | No |
|  | DC_66A_n260H | CA_2A-14A-66A-66A | CA_n260I | 66A | CA_n260H | No |
|  |  | DC_66A_n260I |  | CA_2A-14A-66A-66A |  | CA_n260I |  | 66A |  | CA_n260I |  | No |  |
| DC_3A-19A-21A_n257A |  | DC_3A_n257A |  | CA_3A-19A-21A |  | n257A |  | 3A |  | n257A |  | No |  |
|  |  | DC_19A_n257A |  |  |  |  |  | 19A |  | n257A |  |  |  |
|  |  | DC_21A_n257A |  |  |  |  |  | 21A |  | n257A |  |  |  |
| DC_3A-19A-42A_n257A |  | DC_3A_n257A |  | CA_3A-19A-42A |  | n257A |  | 3A |  | n257A |  | No |  |
|  |  | DC_19A_n257A |  |  |  |  |  | 19A |  | n257A |  |  |  |
|  |  | DC_42A_n257A |  |  |  |  |  | 42A |  | n257A |  |  |  |
| DC_3A-19A-42A_n257G | DC_3A_n257A | CA_3A-19A-42A | CA_n257G | 3A | n257A | No |
|  | DC_3A_n257D | CA_3A-19A-42A | CA_n257G | 3A | CA_n257D | No |
|  | DC_3A_n257G | CA_3A-19A-42A | CA_n257G | 3A | CA_n257G | No |
|  | DC_19A_n257A | CA_3A-19A-42A | CA_n257G | 19A | n257A | No |
|  | DC_19A_n257D | CA_3A-19A-42A | CA_n257G | 19A | CA_n257D | No |
|  | DC_19A_n257G | CA_3A-19A-42A | CA_n257G | 19A | CA_n257G | No |
|  | DC_42A_n257A | CA_3A-19A-42A | CA_n257G | 42A | n257A | No |
|  | DC_42A_n257D | CA_3A-19A-42A | CA_n257G | 42A | CA_n257D | No |
|  | DC_42A_n257G | CA_3A-19A-42A | CA_n257G | 42A | CA_n257G | No |
| DC_3A-19A-42A_n257H | DC_3A_n257A | CA_3A-19A-42A | CA_n257H | 3A | n257A | No |
|  | DC_3A_n257D | CA_3A-19A-42A | CA_n257H | 3A | CA_n257D | No |
|  | DC_3A_n257G | CA_3A-19A-42A | CA_n257H | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_3A-19A-42A | CA_n257H | 3A | CA_n257H | No |
|  | DC_19A_n257A | CA_3A-19A-42A | CA_n257H | 19A | n257A | No |
|  | DC_19A_n257D | CA_3A-19A-42A | CA_n257H | 19A | CA_n257D | No |
|  | DC_19A_n257G | CA_3A-19A-42A | CA_n257H | 19A | CA_n257G | No |
|  | DC_19A_n257H | CA_3A-19A-42A | CA_n257H | 19A | CA_n257H | No |
|  | DC_42A_n257A | CA_3A-19A-42A | CA_n257H | 42A | n257A | No |
|  | DC_42A_n257D | CA_3A-19A-42A | CA_n257H | 42A | CA_n257D | No |
|  | DC_42A_n257G | CA_3A-19A-42A | CA_n257H | 42A | CA_n257G | No |
|  | DC_42A_n257H | CA_3A-19A-42A | CA_n257H | 42A | CA_n257H | No |
| DC_3A-19A-42A_n257I | DC_3A_n257A | CA_3A-19A-42A | CA_n257I | 3A | n257A | No |
|  | DC_3A_n257D | CA_3A-19A-42A | CA_n257I | 3A | CA_n257D | No |
|  | DC_3A_n257G | CA_3A-19A-42A | CA_n257I | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_3A-19A-42A | CA_n257I | 3A | CA_n257H | No |
|  | DC_3A_n257I | CA_3A-19A-42A | CA_n257I | 3A | CA_n257I | No |
|  | DC_19A_n257A | CA_3A-19A-42A | CA_n257I | 19A | n257A | No |
|  | DC_19A_n257D | CA_3A-19A-42A | CA_n257I | 19A | CA_n257D | No |
|  | DC_19A_n257G | CA_3A-19A-42A | CA_n257I | 19A | CA_n257G | No |
|  | DC_19A_n257H | CA_3A-19A-42A | CA_n257I | 19A | CA_n257H | No |
|  | DC_19A_n257I | CA_3A-19A-42A | CA_n257I | 19A | CA_n257I | No |
|  | DC_42A_n257A | CA_3A-19A-42A | CA_n257I | 42A | n257A | No |
|  | DC_42A_n257D | CA_3A-19A-42A | CA_n257I | 42A | CA_n257D | No |
|  | DC_42A_n257G | CA_3A-19A-42A | CA_n257I | 42A | CA_n257G | No |
|  | DC_42A_n257H | CA_3A-19A-42A | CA_n257I | 42A | CA_n257H | No |
|  |  | DC_42A_n257I |  | CA_3A-19A-42A |  | CA_n257I |  | 42A |  | CA_n257I |  | No |  |
| DC_3A-19A-42C_n257A |  | DC_3A_n257A |  | CA_3A-19A-42C |  | n257A |  | 3A |  | n257A |  | No |  |
|  |  | DC_19A_n257A |  |  |  |  |  | 19A |  | n257A |  |  |  |
|  |  | DC_42A_n257A |  |  |  |  |  | 42A |  | n257A |  |  |  |
| DC_3A-19A-42C_n257G | DC_3A_n257A | CA_3A-19A-42C | CA_n257G | 3A | n257A | No |
|  | DC_3A_n257D | CA_3A-19A-42C | CA_n257G | 3A | CA_n257D | No |
|  | DC_3A_n257G | CA_3A-19A-42C | CA_n257G | 3A | CA_n257G | No |
|  | DC_19A_n257A | CA_3A-19A-42C | CA_n257G | 19A | n257A | No |
|  | DC_19A_n257D | CA_3A-19A-42C | CA_n257G | 19A | CA_n257D | No |
|  | DC_19A_n257G | CA_3A-19A-42C | CA_n257G | 19A | CA_n257G | No |
|  | DC_42A_n257A | CA_3A-19A-42C | CA_n257G | 42A | n257A | No |
|  | DC_42A_n257D | CA_3A-19A-42C | CA_n257G | 42A | CA_n257D | No |
|  | DC_42A_n257G | CA_3A-19A-42C | CA_n257G | 42A | CA_n257G | No |
| DC_3A-19A-42C_n257H | DC_3A_n257A | CA_3A-19A-42C | CA_n257H | 3A | n257A | No |
|  | DC_3A_n257D | CA_3A-19A-42C | CA_n257H | 3A | CA_n257D | No |
|  | DC_3A_n257G | CA_3A-19A-42C | CA_n257H | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_3A-19A-42C | CA_n257H | 3A | CA_n257H | No |
|  | DC_19A_n257A | CA_3A-19A-42C | CA_n257H | 19A | n257A | No |
|  | DC_19A_n257D | CA_3A-19A-42C | CA_n257H | 19A | CA_n257D | No |
|  | DC_19A_n257G | CA_3A-19A-42C | CA_n257H | 19A | CA_n257G | No |
|  | DC_19A_n257H | CA_3A-19A-42C | CA_n257H | 19A | CA_n257H | No |
|  | DC_42A_n257A | CA_3A-19A-42C | CA_n257H | 42A | n257A | No |
|  | DC_42A_n257D | CA_3A-19A-42C | CA_n257H | 42A | CA_n257D | No |
|  | DC_42A_n257G | CA_3A-19A-42C | CA_n257H | 42A | CA_n257G | No |
|  | DC_42A_n257H | CA_3A-19A-42C | CA_n257H | 42A | CA_n257H | No |
| DC_3A-19A-42C_n257I | DC_3A_n257A | CA_3A-19A-42C | CA_n257I | 3A | n257A | No |
|  | DC_3A_n257D | CA_3A-19A-42C | CA_n257I | 3A | CA_n257D | No |
|  | DC_3A_n257G | CA_3A-19A-42C | CA_n257I | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_3A-19A-42C | CA_n257I | 3A | CA_n257H | No |
|  | DC_3A_n257I | CA_3A-19A-42C | CA_n257I | 3A | CA_n257I | No |
|  | DC_19A_n257A | CA_3A-19A-42C | CA_n257I | 19A | n257A | No |
|  | DC_19A_n257D | CA_3A-19A-42C | CA_n257I | 19A | CA_n257D | No |
|  | DC_19A_n257G | CA_3A-19A-42C | CA_n257I | 19A | CA_n257G | No |
|  | DC_19A_n257H | CA_3A-19A-42C | CA_n257I | 19A | CA_n257H | No |
|  | DC_19A_n257I | CA_3A-19A-42C | CA_n257I | 19A | CA_n257I | No |
|  | DC_42A_n257A | CA_3A-19A-42C | CA_n257I | 42A | n257A | No |
|  | DC_42A_n257D | CA_3A-19A-42C | CA_n257I | 42A | CA_n257D | No |
|  | DC_42A_n257G | CA_3A-19A-42C | CA_n257I | 42A | CA_n257G | No |
|  | DC_42A_n257H | CA_3A-19A-42C | CA_n257I | 42A | CA_n257H | No |
|  |  | DC_42A_n257I |  | CA_3A-19A-42C |  | CA_n257I |  | 42A |  | CA_n257I |  | No |  |
| DC_3A-21A-42A_n257A |  | DC_3A_n257A |  | CA_3A-21A-42A |  | n257A |  | 3A |  | n257A |  | No |  |
|  |  | DC_21A_n257A |  |  |  |  |  | 21A |  | n257A |  |  |  |
|  |  | DC_42A_n257A |  |  |  |  |  | 42A |  | n257A |  |  |  |
| DC_3A-21A-42A_n257G | DC_3A_n257A | CA_3A-21A-42A | CA_n257G | 3A | n257A | No |
|  | DC_3A_n257D | CA_3A-21A-42A | CA_n257G | 3A | CA_n257D | No |
|  | DC_3A_n257G | CA_3A-21A-42A | CA_n257G | 3A | CA_n257G | No |
|  | DC_21A_n257A | CA_3A-21A-42A | CA_n257G | 21A | n257A | No |
|  | DC_21A_n257D | CA_3A-21A-42A | CA_n257G | 21A | CA_n257D | No |
|  | DC_21A_n257G | CA_3A-21A-42A | CA_n257G | 21A | CA_n257G | No |
|  | DC_42A_n257A | CA_3A-21A-42A | CA_n257G | 42A | n257A | No |
|  | DC_42A_n257D | CA_3A-21A-42A | CA_n257G | 42A | CA_n257D | No |
|  | DC_42A_n257G | CA_3A-21A-42A | CA_n257G | 42A | CA_n257G | No |
| DC_3A-21A-42A_n257H | DC_3A_n257A | CA_3A-21A-42A | CA_n257H | 3A | n257A | No |
|  | DC_3A_n257D | CA_3A-21A-42A | CA_n257H | 3A | CA_n257D | No |
|  | DC_3A_n257G | CA_3A-21A-42A | CA_n257H | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_3A-21A-42A | CA_n257H | 3A | CA_n257H | No |
|  | DC_21A_n257A | CA_3A-21A-42A | CA_n257H | 21A | n257A | No |
|  | DC_21A_n257D | CA_3A-21A-42A | CA_n257H | 21A | CA_n257D | No |
|  | DC_21A_n257G | CA_3A-21A-42A | CA_n257H | 21A | CA_n257G | No |
|  | DC_21A_n257H | CA_3A-21A-42A | CA_n257H | 21A | CA_n257H | No |
|  | DC_42A_n257A | CA_3A-21A-42A | CA_n257H | 42A | n257A | No |
|  | DC_42A_n257D | CA_3A-21A-42A | CA_n257H | 42A | CA_n257D | No |
|  | DC_42A_n257G | CA_3A-21A-42A | CA_n257H | 42A | CA_n257G | No |
|  | DC_42A_n257H | CA_3A-21A-42A | CA_n257H | 42A | CA_n257H | No |
| DC_3A-21A-42A_n257I | DC_3A_n257A | CA_3A-21A-42A | CA_n257I | 3A | n257A | No |
|  | DC_3A_n257D | CA_3A-21A-42A | CA_n257I | 3A | CA_n257D | No |
|  | DC_3A_n257G | CA_3A-21A-42A | CA_n257I | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_3A-21A-42A | CA_n257I | 3A | CA_n257H | No |
|  | DC_3A_n257I | CA_3A-21A-42A | CA_n257I | 3A | CA_n257I | No |
|  | DC_21A_n257A | CA_3A-21A-42A | CA_n257I | 21A | n257A | No |
|  | DC_21A_n257D | CA_3A-21A-42A | CA_n257I | 21A | CA_n257D | No |
|  | DC_21A_n257G | CA_3A-21A-42A | CA_n257I | 21A | CA_n257G | No |
|  | DC_21A_n257H | CA_3A-21A-42A | CA_n257I | 21A | CA_n257H | No |
|  | DC_21A_n257I | CA_3A-21A-42A | CA_n257I | 21A | CA_n257I | No |
|  | DC_42A_n257A | CA_3A-21A-42A | CA_n257I | 42A | n257A | No |
|  | DC_42A_n257D | CA_3A-21A-42A | CA_n257I | 42A | CA_n257D | No |
|  | DC_42A_n257G | CA_3A-21A-42A | CA_n257I | 42A | CA_n257G | No |
|  | DC_42A_n257H | CA_3A-21A-42A | CA_n257I | 42A | CA_n257H | No |
|  |  | DC_42A_n257I |  | CA_3A-21A-42A |  | CA_n257I |  | 42A |  | CA_n257I |  | No |  |
| DC_3A-21A-42C_n257A |  | DC_3A_n257A |  | CA_3A-21A-42C |  | n257A |  | 3A |  | n257A |  | No |  |
|  |  | DC_21A_n257A |  |  |  |  |  | 21A |  | n257A |  |  |  |
|  |  | DC_42A_n257A |  |  |  |  |  | 42A |  | n257A |  |  |  |
| DC_3A-21A-42C_n257G | DC_3A_n257A | CA_3A-21A-42C | CA_n257G | 3A | n257A | No |
|  | DC_3A_n257D | CA_3A-21A-42C | CA_n257G | 3A | CA_n257D | No |
|  | DC_3A_n257G | CA_3A-21A-42C | CA_n257G | 3A | CA_n257G | No |
|  | DC_21A_n257A | CA_3A-21A-42C | CA_n257G | 21A | n257A | No |
|  | DC_21A_n257D | CA_3A-21A-42C | CA_n257G | 21A | CA_n257D | No |
|  | DC_21A_n257G | CA_3A-21A-42C | CA_n257G | 21A | CA_n257G | No |
|  | DC_42A_n257A | CA_3A-21A-42C | CA_n257G | 42A | n257A | No |
|  | DC_42A_n257D | CA_3A-21A-42C | CA_n257G | 42A | CA_n257D | No |
|  | DC_42A_n257G | CA_3A-21A-42C | CA_n257G | 42A | CA_n257G | No |
| DC_3A-21A-42C_n257H | DC_3A_n257A | CA_3A-21A-42C | CA_n257H | 3A | n257A | No |
|  | DC_3A_n257D | CA_3A-21A-42C | CA_n257H | 3A | CA_n257D | No |
|  | DC_3A_n257G | CA_3A-21A-42C | CA_n257H | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_3A-21A-42C | CA_n257H | 3A | CA_n257H | No |
|  | DC_21A_n257A | CA_3A-21A-42C | CA_n257H | 21A | n257A | No |
|  | DC_21A_n257D | CA_3A-21A-42C | CA_n257H | 21A | CA_n257D | No |
|  | DC_21A_n257G | CA_3A-21A-42C | CA_n257H | 21A | CA_n257G | No |
|  | DC_21A_n257H | CA_3A-21A-42C | CA_n257H | 21A | CA_n257H | No |
|  | DC_42A_n257A | CA_3A-21A-42C | CA_n257H | 42A | n257A | No |
|  | DC_42A_n257D | CA_3A-21A-42C | CA_n257H | 42A | CA_n257D | No |
|  | DC_42A_n257G | CA_3A-21A-42C | CA_n257H | 42A | CA_n257G | No |
|  | DC_42A_n257H | CA_3A-21A-42C | CA_n257H | 42A | CA_n257H | No |
| DC_3A-21A-42C_n257I | DC_3A_n257A | CA_3A-21A-42C | CA_n257I | 3A | n257A | No |
|  | DC_3A_n257D | CA_3A-21A-42C | CA_n257I | 3A | CA_n257D | No |
|  | DC_3A_n257G | CA_3A-21A-42C | CA_n257I | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_3A-21A-42C | CA_n257I | 3A | CA_n257H | No |
|  | DC_3A_n257I | CA_3A-21A-42C | CA_n257I | 3A | CA_n257I | No |
|  | DC_21A_n257A | CA_3A-21A-42C | CA_n257I | 21A | n257A | No |
|  | DC_21A_n257D | CA_3A-21A-42C | CA_n257I | 21A | CA_n257D | No |
|  | DC_21A_n257G | CA_3A-21A-42C | CA_n257I | 21A | CA_n257G | No |
|  | DC_21A_n257H | CA_3A-21A-42C | CA_n257I | 21A | CA_n257H | No |
|  | DC_21A_n257I | CA_3A-21A-42C | CA_n257I | 21A | CA_n257I | No |
|  | DC_42A_n257A | CA_3A-21A-42C | CA_n257I | 42A | n257A | No |
|  | DC_42A_n257D | CA_3A-21A-42C | CA_n257I | 42A | CA_n257D | No |
|  | DC_42A_n257G | CA_3A-21A-42C | CA_n257I | 42A | CA_n257G | No |
|  | DC_42A_n257H | CA_3A-21A-42C | CA_n257I | 42A | CA_n257H | No |
|  | DC_42A_n257I | CA_3A-21A-42C | CA_n257I | 42A | CA_n257I | No |
| DC_14A-30A-66A_n260A | DC_14A_n260A | CA_14A-30A-66A | n260A | 14A | n260A | No |
|  | DC_30A_n260A | CA_14A-30A-66A | n260A | 30A | n260A | No |
|  | DC_66A_n260A | CA_14A-30A-66A | n260A | 66A | n260A | No |
| DC_14A-30A-66A_n260G | DC_14A_n260A | CA_14A-30A-66A | CA_n260G | 14A | n260A | No |
|  | DC_14A_n260G | CA_14A-30A-66A | CA_n260G | 14A | CA_n260G | No |
|  | DC_30A_n260A | CA_14A-30A-66A | CA_n260G | 30A | n260A | No |
|  | DC_30A_n260G | CA_14A-30A-66A | CA_n260G | 30A | CA_n260G | No |
|  | DC_66A_n260A | CA_14A-30A-66A | CA_n260G | 66A | n260A | No |
|  | DC_66A_n260G | CA_14A-30A-66A | CA_n260G | 66A | CA_n260G | No |
| DC_14A-30A-66A_n260H | DC_14A_n260A | CA_14A-30A-66A | CA_n260H | 14A | n260A | No |
|  | DC_14A_n260G | CA_14A-30A-66A | CA_n260H | 14A | CA_n260G | No |
|  | DC_14A_n260H | CA_14A-30A-66A | CA_n260H | 14A | CA_n260H | No |
|  | DC_30A_n260A | CA_14A-30A-66A | CA_n260H | 30A | n260A | No |
|  | DC_30A_n260G | CA_14A-30A-66A | CA_n260H | 30A | CA_n260G | No |
|  | DC_30A_n260H | CA_14A-30A-66A | CA_n260H | 30A | CA_n260H | No |
|  | DC_66A_n260A | CA_14A-30A-66A | CA_n260H | 66A | n260A | No |
|  | DC_66A_n260G | CA_14A-30A-66A | CA_n260H | 66A | CA_n260G | No |
|  | DC_66A_n260H | CA_14A-30A-66A | CA_n260H | 66A | CA_n260H | No |
| DC_14A-30A-66A_n260I | DC_14A_n260A | CA_14A-30A-66A | CA_n260I | 14A | n260A | No |
|  | DC_14A_n260G | CA_14A-30A-66A | CA_n260I | 14A | CA_n260G | No |
|  | DC_14A_n260H | CA_14A-30A-66A | CA_n260I | 14A | CA_n260H | No |
|  | DC_14A_n260I | CA_14A-30A-66A | CA_n260I | 14A | CA_n260I | No |
|  | DC_30A_n260A | CA_14A-30A-66A | CA_n260I | 30A | n260A | No |
|  | DC_30A_n260G | CA_14A-30A-66A | CA_n260I | 30A | CA_n260G | No |
|  | DC_30A_n260H | CA_14A-30A-66A | CA_n260I | 30A | CA_n260H | No |
|  | DC_30A_n260I | CA_14A-30A-66A | CA_n260I | 30A | CA_n260I | No |
|  | DC_66A_n260A | CA_14A-30A-66A | CA_n260I | 66A | n260A | No |
|  | DC_66A_n260G | CA_14A-30A-66A | CA_n260I | 66A | CA_n260G | No |
|  | DC_66A_n260H | CA_14A-30A-66A | CA_n260I | 66A | CA_n260H | No |
|  | DC_66A_n260I | CA_14A-30A-66A | CA_n260I | 66A | CA_n260I | No |
| DC_14A-30A-66A-66A_n260A | DC_14A_n260A | CA_14A-30A-66A-66A | n260A | 14A | n260A | No |
|  | DC_30A_n260A | CA_14A-30A-66A-66A | n260A | 30A | n260A | No |
|  | DC_66A_n260A | CA_14A-30A-66A-66A | n260A | 66A | n260A | No |
| DC_14A-30A-66A-66A_n260G | DC_14A_n260A | CA_14A-30A-66A-66A | CA_n260G | 14A | n260A | No |
|  | DC_14A_n260G | CA_14A-30A-66A-66A | CA_n260G | 14A | CA_n260G | No |
|  | DC_30A_n260A | CA_14A-30A-66A-66A | CA_n260G | 30A | n260A | No |
|  | DC_30A_n260G | CA_14A-30A-66A-66A | CA_n260G | 30A | CA_n260G | No |
|  | DC_66A_n260A | CA_14A-30A-66A-66A | CA_n260G | 66A | n260A | No |
|  | DC_66A_n260G | CA_14A-30A-66A-66A | CA_n260G | 66A | CA_n260G | No |
| DC_14A-30A-66A-66A_n260H | DC_14A_n260A | CA_14A-30A-66A-66A | CA_n260H | 14A | n260A | No |
|  | DC_14A_n260G | CA_14A-30A-66A-66A | CA_n260H | 14A | CA_n260G | No |
|  | DC_14A_n260H | CA_14A-30A-66A-66A | CA_n260H | 14A | CA_n260H | No |
|  | DC_30A_n260A | CA_14A-30A-66A-66A | CA_n260H | 30A | n260A | No |
|  | DC_30A_n260G | CA_14A-30A-66A-66A | CA_n260H | 30A | CA_n260G | No |
|  | DC_30A_n260H | CA_14A-30A-66A-66A | CA_n260H | 30A | CA_n260H | No |
|  | DC_66A_n260A | CA_14A-30A-66A-66A | CA_n260H | 66A | n260A | No |
|  | DC_66A_n260G | CA_14A-30A-66A-66A | CA_n260H | 66A | CA_n260G | No |
|  | DC_66A_n260H | CA_14A-30A-66A-66A | CA_n260H | 66A | CA_n260H | No |
| DC_14A-30A-66A-66A_n260I | DC_14A_n260A | CA_14A-30A-66A-66A | CA_n260I | 14A | n260A | No |
|  | DC_14A_n260G | CA_14A-30A-66A-66A | CA_n260I | 14A | CA_n260G | No |
|  | DC_14A_n260H | CA_14A-30A-66A-66A | CA_n260I | 14A | CA_n260H | No |
|  | DC_14A_n260I | CA_14A-30A-66A-66A | CA_n260I | 14A | CA_n260I | No |
|  | DC_30A_n260A | CA_14A-30A-66A-66A | CA_n260I | 30A | n260A | No |
|  | DC_30A_n260G | CA_14A-30A-66A-66A | CA_n260I | 30A | CA_n260G | No |
|  | DC_30A_n260H | CA_14A-30A-66A-66A | CA_n260I | 30A | CA_n260H | No |
|  | DC_30A_n260I | CA_14A-30A-66A-66A | CA_n260I | 30A | CA_n260I | No |
|  | DC_66A_n260A | CA_14A-30A-66A-66A | CA_n260I | 66A | n260A | No |
|  | DC_66A_n260G | CA_14A-30A-66A-66A | CA_n260I | 66A | CA_n260G | No |
|  | DC_66A_n260H | CA_14A-30A-66A-66A | CA_n260I | 66A | CA_n260H | No |
|  |  | DC_66A_n260I |  | CA_14A-30A-66A-66A |  | CA_n260I |  | 66A |  | CA_n260I |  | No |  |
| DC_19A-21A-42A_n257A |  | DC_19A_n257A |  | CA_19A-21A-42A |  | n257A |  | 19A |  | n257A |  | No |  |
|  |  | DC_21A_n257A |  |  |  |  |  | 21A |  | n257A |  |  |  |
|  |  | DC_42A_n257A |  |  |  |  |  | 42A |  | n257A |  |  |  |
| DC_19A-21A-42A_n257G | DC_19A_n257A | CA_19A-21A-42A | CA_n257G | 19A | n257A | No |
|  | DC_19A_n257D | CA_19A-21A-42A | CA_n257G | 19A | CA_n257D | No |
|  | DC_19A_n257G | CA_19A-21A-42A | CA_n257G | 19A | CA_n257G | No |
|  | DC_21A_n257A | CA_19A-21A-42A | CA_n257G | 21A | n257A | No |
|  | DC_21A_n257D | CA_19A-21A-42A | CA_n257G | 21A | CA_n257D | No |
|  | DC_21A_n257G | CA_19A-21A-42A | CA_n257G | 21A | CA_n257G | No |
|  | DC_42A_n257A | CA_19A-21A-42A | CA_n257G | 42A | n257A | No |
|  | DC_42A_n257D | CA_19A-21A-42A | CA_n257G | 42A | CA_n257D | No |
|  | DC_42A_n257G | CA_19A-21A-42A | CA_n257G | 42A | CA_n257G | No |
| DC_19A-21A-42A_n257H | DC_19A_n257A | CA_19A-21A-42A | CA_n257H | 19A | n257A | No |
|  | DC_19A_n257D | CA_19A-21A-42A | CA_n257H | 19A | CA_n257D | No |
|  | DC_19A_n257G | CA_19A-21A-42A | CA_n257H | 19A | CA_n257G | No |
|  | DC_19A_n257H | CA_19A-21A-42A | CA_n257H | 19A | CA_n257H | No |
|  | DC_21A_n257A | CA_19A-21A-42A | CA_n257H | 21A | n257A | No |
|  | DC_21A_n257D | CA_19A-21A-42A | CA_n257H | 21A | CA_n257D | No |
|  | DC_21A_n257G | CA_19A-21A-42A | CA_n257H | 21A | CA_n257G | No |
|  | DC_21A_n257H | CA_19A-21A-42A | CA_n257H | 21A | CA_n257H | No |
|  | DC_42A_n257A | CA_19A-21A-42A | CA_n257H | 42A | n257A | No |
|  | DC_42A_n257D | CA_19A-21A-42A | CA_n257H | 42A | CA_n257D | No |
|  | DC_42A_n257G | CA_19A-21A-42A | CA_n257H | 42A | CA_n257G | No |
|  | DC_42A_n257H | CA_19A-21A-42A | CA_n257H | 42A | CA_n257H | No |
| DC_19A-21A-42A_n257I | DC_19A_n257A | CA_19A-21A-42A | CA_n257I | 19A | n257A | No |
|  | DC_19A_n257D | CA_19A-21A-42A | CA_n257I | 19A | CA_n257D | No |
|  | DC_19A_n257G | CA_19A-21A-42A | CA_n257I | 19A | CA_n257G | No |
|  | DC_19A_n257H | CA_19A-21A-42A | CA_n257I | 19A | CA_n257H | No |
|  | DC_19A_n257I | CA_19A-21A-42A | CA_n257I | 19A | CA_n257I | No |
|  | DC_21A_n257A | CA_19A-21A-42A | CA_n257I | 21A | n257A | No |
|  | DC_21A_n257D | CA_19A-21A-42A | CA_n257I | 21A | CA_n257D | No |
|  | DC_21A_n257G | CA_19A-21A-42A | CA_n257I | 21A | CA_n257G | No |
|  | DC_21A_n257H | CA_19A-21A-42A | CA_n257I | 21A | CA_n257H | No |
|  | DC_21A_n257I | CA_19A-21A-42A | CA_n257I | 21A | CA_n257I | No |
|  | DC_42A_n257A | CA_19A-21A-42A | CA_n257I | 42A | n257A | No |
|  | DC_42A_n257D | CA_19A-21A-42A | CA_n257I | 42A | CA_n257D | No |
|  | DC_42A_n257G | CA_19A-21A-42A | CA_n257I | 42A | CA_n257G | No |
|  | DC_42A_n257H | CA_19A-21A-42A | CA_n257I | 42A | CA_n257H | No |
|  |  | DC_42A_n257I |  | CA_19A-21A-42A |  | CA_n257I |  | 42A |  | CA_n257I |  | No |  |
| DC_19A-21A-42C_n257A |  | DC_19A_n257A |  | CA_19A-21A-42C |  | n257A |  | 19A |  | n257A |  | No |  |
|  |  | DC_21A_n257A |  |  |  |  |  | 21A |  | n257A |  |  |  |
|  |  | DC_42A_n257A |  |  |  |  |  | 42A |  | n257A |  |  |  |
| DC_19A-21A-42C_n257G | DC_19A_n257A | CA_19A-21A-42C | CA_n257G | 19A | n257A | No |
|  | DC_19A_n257D | CA_19A-21A-42C | CA_n257G | 19A | CA_n257D | No |
|  | DC_19A_n257G | CA_19A-21A-42C | CA_n257G | 19A | CA_n257G | No |
|  | DC_21A_n257A | CA_19A-21A-42C | CA_n257G | 21A | n257A | No |
|  | DC_21A_n257D | CA_19A-21A-42C | CA_n257G | 21A | CA_n257D | No |
|  | DC_21A_n257G | CA_19A-21A-42C | CA_n257G | 21A | CA_n257G | No |
|  | DC_42A_n257A | CA_19A-21A-42C | CA_n257G | 42A | n257A | No |
|  | DC_42A_n257D | CA_19A-21A-42C | CA_n257G | 42A | CA_n257D | No |
|  | DC_42A_n257G | CA_19A-21A-42C | CA_n257G | 42A | CA_n257G | No |
| DC_19A-21A-42C_n257H | DC_19A_n257A | CA_19A-21A-42C | CA_n257H | 19A | n257A | No |
|  | DC_19A_n257D | CA_19A-21A-42C | CA_n257H | 19A | CA_n257D | No |
|  | DC_19A_n257G | CA_19A-21A-42C | CA_n257H | 19A | CA_n257G | No |
|  | DC_19A_n257H | CA_19A-21A-42C | CA_n257H | 19A | CA_n257H | No |
|  | DC_21A_n257A | CA_19A-21A-42C | CA_n257H | 21A | n257A | No |
|  | DC_21A_n257D | CA_19A-21A-42C | CA_n257H | 21A | CA_n257D | No |
|  | DC_21A_n257G | CA_19A-21A-42C | CA_n257H | 21A | CA_n257G | No |
|  | DC_21A_n257H | CA_19A-21A-42C | CA_n257H | 21A | CA_n257H | No |
|  | DC_42A_n257A | CA_19A-21A-42C | CA_n257H | 42A | n257A | No |
|  | DC_42A_n257D | CA_19A-21A-42C | CA_n257H | 42A | CA_n257D | No |
|  | DC_42A_n257G | CA_19A-21A-42C | CA_n257H | 42A | CA_n257G | No |
|  | DC_42A_n257H | CA_19A-21A-42C | CA_n257H | 42A | CA_n257H | No |
| DC_19A-21A-42C_n257I | DC_19A_n257A | CA_19A-21A-42C | CA_n257I | 19A | n257A | No |
|  | DC_19A_n257D | CA_19A-21A-42C | CA_n257I | 19A | CA_n257D | No |
|  | DC_19A_n257G | CA_19A-21A-42C | CA_n257I | 19A | CA_n257G | No |
|  | DC_19A_n257H | CA_19A-21A-42C | CA_n257I | 19A | CA_n257H | No |
|  | DC_19A_n257I | CA_19A-21A-42C | CA_n257I | 19A | CA_n257I | No |
|  | DC_21A_n257A | CA_19A-21A-42C | CA_n257I | 21A | n257A | No |
|  | DC_21A_n257D | CA_19A-21A-42C | CA_n257I | 21A | CA_n257D | No |
|  | DC_21A_n257G | CA_19A-21A-42C | CA_n257I | 21A | CA_n257G | No |
|  | DC_21A_n257H | CA_19A-21A-42C | CA_n257I | 21A | CA_n257H | No |
|  | DC_21A_n257I | CA_19A-21A-42C | CA_n257I | 21A | CA_n257I | No |
|  | DC_42A_n257A | CA_19A-21A-42C | CA_n257I | 42A | n257A | No |
|  | DC_42A_n257D | CA_19A-21A-42C | CA_n257I | 42A | CA_n257D | No |
|  | DC_42A_n257G | CA_19A-21A-42C | CA_n257I | 42A | CA_n257G | No |
|  | DC_42A_n257H | CA_19A-21A-42C | CA_n257I | 42A | CA_n257H | No |
|  | DC_42A_n257I | CA_19A-21A-42C | CA_n257I | 42A | CA_n257I | No |
| Note 1: Protocol testing is limited to EN-DC configurations with 1 CC E-UTRA and 1CC or 2CC NR configurations. |  |  |  |  |  |  |

4.3.1.5.1.5	Inter-band EN-DC configurations including FR2 (five bands)

Table 4.3.1.5.1.5-1: Inter-band EN-DC configurations including FR2 (five bands)

| EN-DC Configuration | Uplink EN-DC configuration | E-UTRA downlink configuration | NR downlink configuration | E-UTRA uplink configuration | NR uplink configuration | Applicable for protocol testing (Note 1) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_1A-3A-19A-42A_n257A | DC_1A_n257A | CA_1A-3A-19A-42A | n257A | 1A | n257A | No |
|  | DC_3A_n257A |  |  | 3A | n257A |  |
|  | DC_19A_n257A |  |  | 19A | n257A |  |
|  | DC_42A_n257A |  |  | 42A | n257A |  |
| DC_1A-3A-19A-42A_n257G | DC_1A_n257A | CA_1A-3A-19A-42A | CA_n257G | 1A | n257A | No |
|  | DC_3A_n257A | CA_1A-3A-19A-42A | CA_n257G | 3A | n257A | No |
|  | DC_3A_n257G | CA_1A-3A-19A-42A | CA_n257G | 3A | CA_n257G | No |
|  | DC_19A_n257A | CA_1A-3A-19A-42A | CA_n257G | 19A | n257A | No |
|  | DC_42A_n257A | CA_1A-3A-19A-42A | CA_n257G | 42A | n257A | No |
| DC_1A-3A-19A-42C_n257A | DC_1A_n257A | CA_1A-3A-19A-42C | n257A | 1A | n257A | No |
|  | DC_3A_n257A |  |  | 3A | n257A |  |
|  | DC_19A_n257A |  |  | 19A | n257A |  |
|  | DC_42A_n257A |  |  | 42A | n257A |  |
| DC_1A-3A-19A-42C_n257G | DC_1A_n257A | CA_1A-3A-19A-42C | CA_n257G | 1A | n257A | No |
|  | DC_3A_n257A | CA_1A-3A-19A-42C | CA_n257G | 3A | n257A | No |
|  | DC_3A_n257G | CA_1A-3A-19A-42C | CA_n257G | 3A | CA_n257G | No |
|  | DC_19A_n257A | CA_1A-3A-19A-42C | CA_n257G | 19A | n257A | No |
|  | DC_42A_n257A | CA_1A-3A-19A-42C | CA_n257G | 42A | n257A | No |
| DC_1A-3A-19A-42C_n257H | DC_1A_n257A | CA_1A-3A-19A-42C | CA_n257H | 1A | n257A | No |
|  | DC_3A_n257A | CA_1A-3A-19A-42C | CA_n257H | 3A | n257A | No |
|  | DC_3A_n257G | CA_1A-3A-19A-42C | CA_n257H | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_1A-3A-19A-42C | CA_n257H | 3A | CA_n257H | No |
|  | DC_19A_n257A | CA_1A-3A-19A-42C | CA_n257H | 19A | n257A | No |
|  | DC_42A_n257A | CA_1A-3A-19A-42C | CA_n257H | 42A | n257A | No |
| DC_1A-3A-19A-42C_n257I | DC_1A_n257A | CA_1A-3A-19A-42C | CA_n257I | 1A | n257A | No |
|  | DC_3A_n257A | CA_1A-3A-19A-42C | CA_n257I | 3A | n257A | No |
|  | DC_3A_n257G | CA_1A-3A-19A-42C | CA_n257I | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_1A-3A-19A-42C | CA_n257I | 3A | CA_n257H | No |
|  | DC_3A_n257I | CA_1A-3A-19A-42C | CA_n257I | 3A | CA_n257I | No |
|  | DC_19A_n257A | CA_1A-3A-19A-42C | CA_n257I | 19A | n257A | No |
|  | DC_42A_n257A | CA_1A-3A-19A-42C | CA_n257I | 42A | n257A | No |
| DC_1A-3A-21A-42A_n257A | DC_1A_n257A | CA_1A-3A-21A-42A | n257A | 1A | n257A | No |
|  | DC_3A_n257A |  |  | 3A | n257A |  |
|  | DC_21A_n257A |  |  | 21A | n257A |  |
|  | DC_42A_n257A |  |  | 42A | n257A |  |
| DC_1A-3A-21A-42C_n257A | DC_1A_n257A | CA_1A-3A-21A-42C | n257A | 1A | n257A | No |
|  | DC_3A_n257A |  |  | 3A | n257A |  |
|  | DC_21A_n257A |  |  | 21A | n257A |  |
|  | DC_42A_n257A |  |  | 42A | n257A |  |
| DC_1A-3A-21A-42C_n257G | DC_1A_n257A | CA_1A-3A-21A-42C | CA_n257G | 1A | n257A | No |
|  | DC_3A_n257A | CA_1A-3A-21A-42C | CA_n257G | 3A | n257A | No |
|  | DC_3A_n257G | CA_1A-3A-21A-42C | CA_n257G | 3A | CA_n257G | No |
|  | DC_21A_n257A | CA_1A-3A-21A-42C | CA_n257G | 21A | n257A | No |
|  | DC_42A_n257A | CA_1A-3A-21A-42C | CA_n257G | 42A | n257A | No |
| DC_1A-3A-21A-42C_n257H | DC_1A_n257A | CA_1A-3A-21A-42C | CA_n257H | 1A | n257A | No |
|  | DC_3A_n257A | CA_1A-3A-21A-42C | CA_n257H | 3A | n257A | No |
|  | DC_3A_n257G | CA_1A-3A-21A-42C | CA_n257H | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_1A-3A-21A-42C | CA_n257H | 3A | CA_n257H | No |
|  | DC_21A_n257A | CA_1A-3A-21A-42C | CA_n257H | 21A | n257A | No |
|  | DC_42A_n257A | CA_1A-3A-21A-42C | CA_n257H | 42A | n257A | No |
| DC_1A-3A-21A-42C_n257I | DC_1A_n257A | CA_1A-3A-21A-42C | CA_n257I | 1A | n257A | No |
|  | DC_3A_n257A | CA_1A-3A-21A-42C | CA_n257I | 3A | n257A | No |
|  | DC_3A_n257G | CA_1A-3A-21A-42C | CA_n257I | 3A | CA_n257G | No |
|  | DC_3A_n257H | CA_1A-3A-21A-42C | CA_n257I | 3A | CA_n257H | No |
|  | DC_3A_n257I | CA_1A-3A-21A-42C | CA_n257I | 3A | CA_n257I | No |
|  | DC_21A_n257A | CA_1A-3A-21A-42C | CA_n257I | 21A | n257A | No |
|  | DC_42A_n257A | CA_1A-3A-21A-42C | CA_n257I | 42A | n257A | No |
| DC_1A-19A-21A-42A_n257A | DC_1A_n257A | CA_1A-19A-21A-42A | n257A | 1A | n257A | No |
|  | DC_19A_n257A |  |  | 19A | n257A |  |
|  | DC_21A_n257A |  |  | 21A | n257A |  |
|  | DC_42A_n257A |  |  | 42A | n257A |  |
| DC_1A-19A-21A-42A_n257G | DC_1A_n257A | CA_1A-19A-21A-42A | CA_n257G | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-19A-21A-42A | CA_n257G | 1A | CA_n257G | No |
|  | DC_19A_n257A | CA_1A-19A-21A-42A | CA_n257G | 19A | n257A | No |
|  | DC_21A_n257A | CA_1A-19A-21A-42A | CA_n257G | 21A | n257A | No |
|  | DC_21A_n257G | CA_1A-19A-21A-42A | CA_n257G | 21A | CA_n257G | No |
|  | DC_42A_n257A | CA_1A-19A-21A-42A | CA_n257G | 42A | n257A | No |
| DC_1A-19A-21A-42A_n257H | DC_1A_n257A | CA_1A-19A-21A-42A | CA_n257H | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-19A-21A-42A | CA_n257H | 1A | CA_n257G | No |
|  | DC_1A_n257H | CA_1A-19A-21A-42A | CA_n257H | 1A | CA_n257H | No |
|  | DC_19A_n257A | CA_1A-19A-21A-42A | CA_n257H | 19A | n257A | No |
|  | DC_21A_n257A | CA_1A-19A-21A-42A | CA_n257H | 21A | n257A | No |
|  | DC_21A_n257G | CA_1A-19A-21A-42A | CA_n257H | 21A | CA_n257G | No |
|  | DC_21A_n257H | CA_1A-19A-21A-42A | CA_n257H | 21A | CA_n257H | No |
|  | DC_42A_n257A | CA_1A-19A-21A-42A | CA_n257H | 42A | n257A | No |
| DC_1A-19A-21A-42A_n257I | DC_1A_n257A | CA_1A-19A-21A-42A | CA_n257I | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-19A-21A-42A | CA_n257I | 1A | CA_n257G | No |
|  | DC_1A_n257H | CA_1A-19A-21A-42A | CA_n257I | 1A | CA_n257H | No |
|  | DC_1A_n257I | CA_1A-19A-21A-42A | CA_n257I | 1A | CA_n257I | No |
|  | DC_19A_n257A | CA_1A-19A-21A-42A | CA_n257I | 19A | n257A | No |
|  | DC_21A_n257A | CA_1A-19A-21A-42A | CA_n257I | 21A | n257A | No |
|  | DC_21A_n257G | CA_1A-19A-21A-42A | CA_n257I | 21A | CA_n257G | No |
|  | DC_21A_n257H | CA_1A-19A-21A-42A | CA_n257I | 21A | CA_n257H | No |
|  | DC_21A_n257I | CA_1A-19A-21A-42A | CA_n257I | 21A | CA_n257I | No |
|  | DC_42A_n257A | CA_1A-19A-21A-42A | CA_n257I | 42A | n257A | No |
| DC_1A-19A-21A-42C_n257A | DC_1A_n257A | CA_1A-19A-21A-42C | n257A | 1A | n257A | No |
|  | DC_19A_n257A |  |  | 19A | n257A |  |
|  | DC_21A_n257A |  |  | 21A | n257A |  |
|  | DC_42A_n257A |  |  | 42A | n257A |  |
| DC_1A-19A-21A-42A_n257G | DC_1A_n257A | CA_1A-19A-21A-42A | CA_n257G | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-19A-21A-42A | CA_n257G | 1A | CA_n257G | No |
|  | DC_19A_n257A | CA_1A-19A-21A-42A | CA_n257G | 19A | n257A | No |
|  | DC_21A_n257A | CA_1A-19A-21A-42A | CA_n257G | 21A | n257A | No |
|  | DC_21A_n257G | CA_1A-19A-21A-42A | CA_n257G | 21A | CA_n257G | No |
|  | DC_42A_n257A | CA_1A-19A-21A-42A | CA_n257G | 42A | n257A | No |
| DC_1A-19A-21A-42A_n257H | DC_1A_n257A | CA_1A-19A-21A-42A | CA_n257H | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-19A-21A-42A | CA_n257H | 1A | CA_n257G | No |
|  | DC_1A_n257H | CA_1A-19A-21A-42A | CA_n257H | 1A | CA_n257H | No |
|  | DC_19A_n257A | CA_1A-19A-21A-42A | CA_n257H | 19A | n257A | No |
|  | DC_21A_n257A | CA_1A-19A-21A-42A | CA_n257H | 21A | n257A | No |
|  | DC_21A_n257G | CA_1A-19A-21A-42A | CA_n257H | 21A | CA_n257G | No |
|  | DC_21A_n257H | CA_1A-19A-21A-42A | CA_n257H | 21A | CA_n257H | No |
|  | DC_42A_n257A | CA_1A-19A-21A-42A | CA_n257H | 42A | n257A | No |
| DC_1A-19A-21A-42A_n257I | DC_1A_n257A | CA_1A-19A-21A-42A | CA_n257I | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-19A-21A-42A | CA_n257I | 1A | CA_n257G | No |
|  | DC_1A_n257H | CA_1A-19A-21A-42A | CA_n257I | 1A | CA_n257H | No |
|  | DC_1A_n257I | CA_1A-19A-21A-42A | CA_n257I | 1A | CA_n257I | No |
|  | DC_19A_n257A | CA_1A-19A-21A-42A | CA_n257I | 19A | n257A | No |
|  | DC_21A_n257A | CA_1A-19A-21A-42A | CA_n257I | 21A | n257A | No |
|  | DC_21A_n257G | CA_1A-19A-21A-42A | CA_n257I | 21A | CA_n257G | No |
|  | DC_21A_n257H | CA_1A-19A-21A-42A | CA_n257I | 21A | CA_n257H | No |
|  | DC_21A_n257I | CA_1A-19A-21A-42A | CA_n257I | 21A | CA_n257I | No |
|  | DC_42A_n257A | CA_1A-19A-21A-42A | CA_n257I | 42A | n257A | No |
| DC_1A-19A-21A-42C_n257G | DC_1A_n257A | CA_1A-19A-21A-42C | CA_n257G | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-19A-21A-42C | CA_n257G | 1A | CA_n257G | No |
|  | DC_19A_n257A | CA_1A-19A-21A-42C | CA_n257G | 19A | n257A | No |
|  | DC_21A_n257A | CA_1A-19A-21A-42C | CA_n257G | 21A | n257A | No |
|  | DC_21A_n257G | CA_1A-19A-21A-42C | CA_n257G | 21A | CA_n257G | No |
|  | DC_42A_n257A | CA_1A-19A-21A-42C | CA_n257G | 42A | n257A | No |
| DC_1A-19A-21A-42C_n257H | DC_1A_n257A | CA_1A-19A-21A-42C | CA_n257H | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-19A-21A-42C | CA_n257H | 1A | CA_n257G | No |
|  | DC_1A_n257H | CA_1A-19A-21A-42C | CA_n257H | 1A | CA_n257H | No |
|  | DC_19A_n257A | CA_1A-19A-21A-42C | CA_n257H | 19A | n257A | No |
|  | DC_21A_n257A | CA_1A-19A-21A-42C | CA_n257H | 21A | n257A | No |
|  | DC_21A_n257G | CA_1A-19A-21A-42C | CA_n257H | 21A | CA_n257G | No |
|  | DC_21A_n257H | CA_1A-19A-21A-42C | CA_n257H | 21A | CA_n257H | No |
|  | DC_42A_n257A | CA_1A-19A-21A-42C | CA_n257H | 42A | n257A | No |
| DC_1A-19A-21A-42C_n257I | DC_1A_n257A | CA_1A-19A-21A-42C | CA_n257I | 1A | n257A | No |
|  | DC_1A_n257G | CA_1A-19A-21A-42C | CA_n257I | 1A | CA_n257G | No |
|  | DC_1A_n257H | CA_1A-19A-21A-42C | CA_n257I | 1A | CA_n257H | No |
|  | DC_1A_n257I | CA_1A-19A-21A-42C | CA_n257I | 1A | CA_n257I | No |
|  | DC_19A_n257A | CA_1A-19A-21A-42C | CA_n257I | 19A | n257A | No |
|  | DC_21A_n257A | CA_1A-19A-21A-42C | CA_n257I | 21A | n257A | No |
|  | DC_21A_n257G | CA_1A-19A-21A-42C | CA_n257I | 21A | CA_n257G | No |
|  | DC_21A_n257H | CA_1A-19A-21A-42C | CA_n257I | 21A | CA_n257H | No |
|  | DC_21A_n257I | CA_1A-19A-21A-42C | CA_n257I | 21A | CA_n257I | No |
|  | DC_42A_n257A | CA_1A-19A-21A-42C | CA_n257I | 42A | n257A | No |
| DC_2A-14A-30A-66A_n260A | DC_2A_n260A | CA_2A-14A-30A-66A | n260A | 2A | n260A | No |
|  | DC_14A_n260A | CA_2A-14A-30A-66A | n260A | 14A | n260A | No |
|  | DC_30A_n260A | CA_2A-14A-30A-66A | n260A | 30A | n260A | No |
|  | DC_66A_n260A | CA_2A-14A-30A-66A | n260A | 66A | n260A | No |
| DC_2A-14A-30A-66A_n260G | DC_2A_n260A | CA_2A-14A-30A-66A | CA_n260G | 2A | n260A | No |
|  | DC_2A_n260G | CA_2A-14A-30A-66A | CA_n260G | 2A | CA_n260G | No |
|  | DC_14A_n260A | CA_2A-14A-30A-66A | CA_n260G | 14A | n260A | No |
|  | DC_14A_n260G | CA_2A-14A-30A-66A | CA_n260G | 14A | CA_n260G | No |
|  | DC_30A_n260A | CA_2A-14A-30A-66A | CA_n260G | 30A | n260A | No |
|  | DC_30A_n260G | CA_2A-14A-30A-66A | CA_n260G | 30A | CA_n260G | No |
|  | DC_66A_n260A | CA_2A-14A-30A-66A | CA_n260G | 66A | n260A | No |
|  | DC_66A_n260G | CA_2A-14A-30A-66A | CA_n260G | 66A | CA_n260G | No |
| DC_2A-14A-30A-66A_n260H | DC_2A_n260A | CA_2A-14A-30A-66A | CA_n260H | 2A | n260A | No |
|  | DC_2A_n260G | CA_2A-14A-30A-66A | CA_n260H | 2A | CA_n260G | No |
|  | DC_2A_n260H | CA_2A-14A-30A-66A | CA_n260H | 2A | CA_n260H | No |
|  | DC_14A_n260A | CA_2A-14A-30A-66A | CA_n260H | 14A | n260A | No |
|  | DC_14A_n260G | CA_2A-14A-30A-66A | CA_n260H | 14A | CA_n260G | No |
|  | DC_14A_n260H | CA_2A-14A-30A-66A | CA_n260H | 14A | CA_n260H | No |
|  | DC_30A_n260A | CA_2A-14A-30A-66A | CA_n260H | 30A | n260A | No |
|  | DC_30A_n260G | CA_2A-14A-30A-66A | CA_n260H | 30A | CA_n260G | No |
|  | DC_30A_n260H | CA_2A-14A-30A-66A | CA_n260H | 30A | CA_n260H | No |
|  | DC_66A_n260A | CA_2A-14A-30A-66A | CA_n260H | 66A | n260A | No |
|  | DC_66A_n260G | CA_2A-14A-30A-66A | CA_n260H | 66A | CA_n260G | No |
|  | DC_66A_n260H | CA_2A-14A-30A-66A | CA_n260H | 66A | CA_n260H | No |
| DC_2A-14A-30A-66A_n260I | DC_2A_n260A | CA_2A-14A-30A-66A | CA_n260I | 2A | n260A | No |
|  | DC_2A_n260G | CA_2A-14A-30A-66A | CA_n260I | 2A | CA_n260G | No |
|  | DC_2A_n260H | CA_2A-14A-30A-66A | CA_n260I | 2A | CA_n260H | No |
|  | DC_2A_n260I | CA_2A-14A-30A-66A | CA_n260I | 2A | CA_n260I | No |
|  | DC_14A_n260A | CA_2A-14A-30A-66A | CA_n260I | 14A | n260A | No |
|  | DC_14A_n260G | CA_2A-14A-30A-66A | CA_n260I | 14A | CA_n260G | No |
|  | DC_14A_n260H | CA_2A-14A-30A-66A | CA_n260I | 14A | CA_n260H | No |
|  | DC_14A_n260I | CA_2A-14A-30A-66A | CA_n260I | 14A | CA_n260I | No |
|  | DC_30A_n260A | CA_2A-14A-30A-66A | CA_n260I | 30A | n260A | No |
|  | DC_30A_n260G | CA_2A-14A-30A-66A | CA_n260I | 30A | CA_n260G | No |
|  | DC_30A_n260H | CA_2A-14A-30A-66A | CA_n260I | 30A | CA_n260H | No |
|  | DC_30A_n260I | CA_2A-14A-30A-66A | CA_n260I | 30A | CA_n260I | No |
|  | DC_66A_n260A | CA_2A-14A-30A-66A | CA_n260I | 66A | n260A | No |
|  | DC_66A_n260G | CA_2A-14A-30A-66A | CA_n260I | 66A | CA_n260G | No |
|  | DC_66A_n260H | CA_2A-14A-30A-66A | CA_n260I | 66A | CA_n260H | No |
|  | DC_66A_n260I | CA_2A-14A-30A-66A | CA_n260I | 66A | CA_n260I | No |
| Note 1: Protocol testing is limited to EN-DC configurations with 1 CC E-UTRA and 1CC or 2CC NR configurations. |  |  |  |  |  |  |

<< Next changes >>

#### 4.3.1.6	Test frequencies for EN-DC band combinations including FR1 and FR2

##### 4.3.1.6.1	Inter-band EN-DC configurations including FR1 and FR2

4.3.1.6.1.1	General

For inter-band EN-DC configurations as listed in this clause, the following apply:

For the E-UTRA band and E-UTRA CA configurations, test frequencies as specified in TS 36.508 [2], clause 4.3.1 are used.

For the NR band and NR CA configurations, test frequencies as specified in clause 4.3.1 for FR1 and 4.3.2 for FR2 are used.

4.3.1.6.1.2	Inter-band EN-DC configurations including FR1 and FR2 (three bands)

Table 4.3.1.6.1.2-1: Inter-band EN-DC including FR1 and FR2 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration | E-UTRA downlink configuration | NR downlink configuration | E-UTRA uplink configuration | NR uplink configuration | Applicable for protocol testing (Note 1) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n78A-n257A | DC_1A_n78A | 1A | CA_n78A-n257A | 1A | n78A | Yes (NR 2CC) |
|  | DC_1A_n257A |  |  | 1A | n257A | No |
|  | DC_1A_n78A-n257A |  |  | 1A | CA_n78A-n257A | Yes (NR 2CC) |
| DC_1A_n79A-n257A | DC_1A_n79A | 1A | CA_n79A-n257A | 1A | n79A | Yes (NR 2CC) |
|  | DC_1A_n257A |  |  | 1A | n257A | No |
|  | DC_1A_n79A-n257A |  |  | 1A | CA_n79A-n257A | Yes (NR 2CC) |
| DC_3A_n78A-n257A | DC_3A_n78A | 3A | CA_n78A-n257A | 3A | n78A | Yes (NR 2CC) |
|  | DC_3A_n257A |  |  | 3A | n257A | No |
|  | DC_3A_n78A-n257A |  |  | 3A | CA_n78A-n257A | Yes (NR 2CC) |
| DC_3A_n79A-n257A | DC_3A_n79A | 3A | CA_n79A-n257A | 3A | n79A | Yes (NR 2CC) |
|  | DC_3A_n257A |  |  | 3A | n257A | No |
|  | DC_3A_n79A-n257A |  |  | 3A | CA_n79A-n257A | Yes (NR 2CC) |
| DC_19A_n78A-n257A | DC_19A_n78A | 19A | CA_n78A-n257A | 19A | n78A | Yes (NR 2CC) |
|  | DC_19A_n257A |  |  | 19A | n257A | No |
|  | DC_19A_n78A-n257A |  |  | 19A | CA_n78A-n257A | Yes (NR 2CC) |
| DC_19A_n79A-n257A | DC_19A_n79A | 19A | CA_n79A-n257A | 19A | n79A | Yes (NR 2CC) |
|  | DC_19A_n257A |  |  | 19A | n257A | No |
|  | DC_19A_n79A-n257A |  |  | 19A | CA_n79A-n257A | Yes (NR 2CC) |
| Note 1: Protocol testing is limited to EN-DC configurations with 1 CC E-UTRA and 1CC or 2CC NR configurations. |  |  |  |  |  |  |

4.3.1.6.1.3	Inter-band EN-DC configurations including FR1 and FR2 (four bands)

Table 4.3.1.6.1.3-1: Inter-band EN-DC including FR1 and FR2 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration | E-UTRA downlink configuration | NR downlink configuration | E-UTRA uplink configuration | NR uplink configuration |  | Applicable for protocol testing (Note 1) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-3A_n78A-n257A | DC_1A_n78A | CA_1A-3A | CA_n78A-n257A | 1A | n78A |  | No |  |
|  | DC_1A_n257A |  |  | 1A | n257A |  |  |  |
|  | DC_3A_n78A |  |  | 3A | n78A |  |  |  |
|  | DC_3A_n257A |  |  | 3A | n257A |  |  |  |
| DC_1A-3A_n78A-n257G | DC_1A_n78A | CA_1A-3A | CA_n78A-n257G | 1A | n78A | No |  |
|  | DC_1A_n257A | CA_1A-3A | CA_n78A-n257G | 1A | n257A | No |  |
|  | DC_1A_n257D | CA_1A-3A | CA_n78A-n257G | 1A | CA_n257D | No |  |
|  | DC_1A_n257G | CA_1A-3A | CA_n78A-n257G | 1A | CA_n257G | No |  |
|  | DC_3A_n78A | CA_1A-3A | CA_n78A-n257G | 3A | n78A | No |  |
|  | DC_3A_n257A | CA_1A-3A | CA_n78A-n257G | 3A | n257A | No |  |
|  | DC_3A_n257D | CA_1A-3A | CA_n78A-n257G | 3A | CA_n257D | No |  |
|  | DC_3A_n257G | CA_1A-3A | CA_n78A-n257G | 3A | CA_n257G | No |  |
| DC_1A-3A_n78A-n257H | DC_1A_n78A | CA_1A-3A | CA_n78A-n257H | 1A | n78A | No |  |
|  | DC_1A_n257A | CA_1A-3A | CA_n78A-n257H | 1A | n257A | No |  |
|  | DC_1A_n257D | CA_1A-3A | CA_n78A-n257H | 1A | CA_n257D | No |  |
|  | DC_1A_n257G | CA_1A-3A | CA_n78A-n257H | 1A | CA_n257G | No |  |
|  | DC_1A_n257H | CA_1A-3A | CA_n78A-n257H | 1A | CA_n257H | No |  |
|  | DC_3A_n78A | CA_1A-3A | CA_n78A-n257H | 3A | n78A | No |  |
|  | DC_3A_n257A | CA_1A-3A | CA_n78A-n257H | 3A | n257A | No |  |
|  | DC_3A_n257D | CA_1A-3A | CA_n78A-n257H | 3A | CA_n257D | No |  |
|  | DC_3A_n257G | CA_1A-3A | CA_n78A-n257H | 3A | CA_n257G | No |  |
|  | DC_3A_n257H | CA_1A-3A | CA_n78A-n257H | 3A | CA_n257H | No |  |
| DC_1A-3A_n78A-n257I | DC_1A_n78A | CA_1A-3A | CA_n78A-n257I | 1A | n78A | No |  |
|  | DC_1A_n257A | CA_1A-3A | CA_n78A-n257I | 1A | n257A | No |  |
|  | DC_1A_n257D | CA_1A-3A | CA_n78A-n257I | 1A | CA_n257D | No |  |
|  | DC_1A_n257G | CA_1A-3A | CA_n78A-n257I | 1A | CA_n257G | No |  |
|  | DC_1A_n257H | CA_1A-3A | CA_n78A-n257I | 1A | CA_n257H | No |  |
|  | DC_1A_n257I | CA_1A-3A | CA_n78A-n257I | 1A | CA_n257I | No |  |
|  | DC_3A_n78A | CA_1A-3A | CA_n78A-n257I | 3A | n78A | No |  |
|  | DC_3A_n257A | CA_1A-3A | CA_n78A-n257I | 3A | n257A | No |  |
|  | DC_3A_n257D | CA_1A-3A | CA_n78A-n257I | 3A | CA_n257D | No |  |
|  | DC_3A_n257G | CA_1A-3A | CA_n78A-n257I | 3A | CA_n257G | No |  |
|  | DC_3A_n257H | CA_1A-3A | CA_n78A-n257I | 3A | CA_n257H | No |  |
|  | DC_3A_n257I | CA_1A-3A | CA_n78A-n257I | 3A | CA_n257I | No |  |
| Note 1: Protocol testing is limited to EN-DC configurations with 1 CC E-UTRA and 1CC or 2CC NR configurations. |  |  |  |  |  |  |  |

==============================================================

<< End of changes >>