3GPP TSG-RAN WG5 Meeting # 108	R5-254332

# Bengaluru, India, August 25 – 29, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1980 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of R15 pending configurations for MSD test points for intermodulation interference |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The following R15 EN-DC configurations are pending in PRD21 and should be removed from the MSD table for intermodulation interference due to dual uplink operation. - DC_1A-18A_n78A - DC_1A-18A_n79A - DC_1A-28A_n79A/C - DC_3A-28A_n79A - DC_3A-41A_n78A - DC_3A_SUL_n78A-n82A - DC_7A-46A_n78A - DC_18A-28A_n78A - DC_21A-28A_n77A - DC_21A-28A_n79A - DC_28A-42A_n79A |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the mentioned pending R15 EN-DC configurations from the MSD table for intermodulation interference due to dual uplink operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The mentioned pending R15 EN-DC configurations for MSD talbe will remain in the test specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5B.4.2, 7.3B.2.0.3.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

5.5B.4.2	Inter-band EN-DC configurations within FR1 (three bands)

Table 5.5B.4.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A_n28A | DC_1A_n28A DC_3A_n28A |
| DC_1A-3A_n41A5 | DC_1A_n41A DC_3A_n41A |
| DC_1A-3A_n77A5 | DC_1A_n77A DC_3A_n77A |
| DC_1A-3A_n77(2A)5,14 | DC_1A_n77A14 DC_3A_n77A14 |
| DC_1A-3A_n78A5,14 DC_1A-3C_n78A5 | DC_1A_n78A14 DC_3A_n78A14 DC_3C_n78A |
| DC_1A-1A-3A_n78A DC_1A-1A-3C_n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A |
| DC_1A-3C_n78(2A)5 | DC_1A_n78A DC_3A_n78A DC_3C_n78A |
| DC_1A-3A_n79A5,14 | DC_1A_n79A14 DC_3A_n79A14 |
| DC_1A-5A_n78A5 DC_1A-5A_n78C5 | DC_1A_n78A DC_5A_n78A |
| DC_1A-1A-5A_n78A | DC_1A_n78A DC_5A_n78A |
| DC_1A-7A_n3A | DC_1A_n3A DC_7A_n3A |
| DC_1A-7A_n28A5 | DC_1A_n28A DC_7A_n28A |
| DC_1A-7A_n78A5 | DC_1A_n78A DC_7A_n78A |
| DC_1A-8A_n3A | DC_1A_n3A DC_8A_n3A |
| DC_1A-8A_n77A5 | DC_1A_n77A DC_8A_n77A |
| DC_1A-8A_n77(2A)5,14 | DC_1A_n77A14 DC_8A_n77A14 |
| DC_1A-8A_n78A5 | DC_1A_n78A DC_8A_n78A |
| DC_1A-8A_n78(2A)5 | DC_1A_n78A DC_8A_n78A |
| DC_1A-18A_n77A5 | DC_1A_n77A DC_18A_n77A |
| DC_1A-19A_n77A5 | DC_1A_n77A DC_19A_n77A |
| DC_1A-19A_n78A5,14 DC_1A-19A_n78C5 | DC_1A_n78A14 DC_19A_n78A14 |
| DC_1A-19A_n79A5,14 DC_1A-19A_n79C5 | DC_1A_n79A14 DC_19A_n79A14 |
| DC_1A-20A_n3A | DC_1A_n3A DC_20A_n3A |
| DC_1A-20A_n8A | DC_1A_n8A DC_20A_n8A |
| DC_1A-20A_n28A6,11,12 | DC_1A_n28A DC_20A_n28A |
| DC_1A-20A_n78A5 | DC_1A_n78A DC_20A_n78A |
| DC_1A-21A_n77A5 | DC_1A_n77A DC_21A_n77A |
| DC_1A-21A_n78A5,14 DC_1A-21A_n78C5 | DC_1A_n78A14 DC_21A_n78A14 |
| DC_1A-21A_n79A5,14 DC_1A-21A_n79C5 | DC_1A_n79A14 DC_21A_n79A14 |
| DC_1A-28A_n3A | DC_1A_n3A DC_28A_n3A |
| DC_1A-28A_n5A6 | DC_1A_n5A DC_28A_n5A |
| DC_1A_n28A-n78A5 | DC_1A_n28A DC_1A_n78A |
| DC_1A-41A_n28A5 DC_1A-41C_n28A5 | DC_1A_n28A DC_41A_n28A DC_41C_n28A |
| DC_1A-41A_n41A | DC_1A_n41A |
| DC_1A-41A_n77A | DC_1A_n77A DC_41A_n77A |
| DC_1A-41A_n78A | DC_1A_n78A DC_41A_n78A |
| DC_1A-42A_n77A10,11 DC_1A-42C_n77A10,11 DC_1A-42D_n77A10,11 | DC_1A_n77A |
| DC_1A-42A_n78A10,11,14 DC_1A-42A_n78C10,11 DC_1A-42C_n78A10,11 DC_1A-42C_n78C10,11 DC_1A-42D_n78A10,11 DC_1A-42E_n78A10,11 | DC_1A_n78A14 |
| DC_1A-42A_n79A14 DC_1A-42A_n79C DC_1A-42C_n79A DC_1A-42C_n79C DC_1A-42D_n79A DC_1A-42E_n79A | DC_1A_n79A14 |
| DC_1A_n77A-n79A | DC_1A_n77A DC_1A_n79A |
| DC_1A_n78A-n79A14 | DC_1A_n78A14 DC_1A_n79A14 |
| DC_2A-5A_n2A | DC_5A_n2A DC_2A_n2A2 |
| … | … |

<< Unchanged text omitted >>

7.3B.2.0.3.5.2	MSD test points for intermodulation interference due to dual uplink operation for EN-DC in NR FR1 involving three bands

Table 7.3B.2.0.3.5.2-0: MSD test points for PCell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) |  | IMD order |
| DC_66A_(n)71AA | 66 | 1750 | 5 | 25 | 2150 | 5 |  | IMD4 |
|  | n71 | 678 | 10 | 10 (RBstart =0) | 632 | N/A |  | N/A |
| NOTE 1: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 2: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. |  |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB |  | DL Fc (MHz) |  | MSD (dB) | IMD order |
| DC_1A-3A_n28A | 1 | 1975 | 5 | 25 |  | 2165 |  | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 |  | 765.5 |  | N/A | N/A |
|  | 3 | 1723.5 | 5 | 25 |  | 1818.5 |  | 4.0 | IMD5 |
|  | 3 | 1780 | 5 | 25 |  | 1875 |  | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 |  | 765.5 |  | N/A | N/A |
|  | 1 | 1949 | 5 | 25 | 2139 |  | 11.0 |  | IMD4 |
| DC_1A-7A_n28A | 1 | 1935 | 5 | 25 |  | 2125 |  | N/A | N/A |
|  | n28 | 718 | 5 | 25 |  | 773 |  | N/A | N/A |
|  | 7 | 2533 | 10 | 50 |  | 2653 |  | 30.0 | IMD2 |
| DC_1A-3A_n77A | 1 | 1950 | 5 | 25 |  | 2140 |  | N/A | N/A |
| DC_1A-3A_n77(2A) | 3 | 1712.5 | 5 | 25 |  | 1807.5 |  | 31.5 | IMD2 |
|  | n77 | 3757.5 | 10 | 50 |  | 3757.5 |  | N/A | N/A |
|  | 1 | 1950 | 5 | 25 |  | 2140 |  | N/A | N/A |
|  | 3 | 1775 | 5 | 25 |  | 1870 |  | 8.5 | IMD4 |
|  | n77 | 3980 | 10 | 50 |  | 3980 |  | N/A | N/A |
|  | 1 | 1950 | 5 | 25 |  | 2140 |  | 31.0 | IMD2 |
|  | 3 | 1775 | 5 | 25 |  | 1870 |  | N/A | N/A |
|  | n77 | 3915 | 10 | 50 |  | 3915 |  | N/A | N/A |
| DC_1A-3A_n78A DC_1A-3C_n78A | 1 | 1950 | 5 | 25 |  | 2140 |  | N/A | N/A |
|  | 3 | 1712.5 | 5 | 25 |  | 1807.5 |  | 31.2 | IMD2 |
|  | n78 | 3757.5 | 10 | 50 |  | 3757.5 |  | N/A | N/A |
|  | 1 | 1935 | 5 | 25 |  | 2125 |  | 2.8 | IMD5 |
|  | 3 | 1775 | 5 | 25 |  | 1870 |  | N/A | N/A |
|  | n78 | 3725 | 10 | 50 |  | 3725 |  | N/A | N/A |
| DC_1A_n3A-n78A | 1 | 1950 | 5 | 25 |  | 2140 |  | N/A | N/A |
|  | n3 | 1750 | 5 | 25 |  | 1845 |  | N/A | N/A |
|  | n78 | N/A | 10 | N/A |  | 3700 |  | 28.4 | IMD2 |
|  | 1 | 1950 | 5 | 25 |  | 2140 |  | N/A | N/A |
|  | n3 | N/A | 5 | N/A |  | 1830 |  | 27.9 | IMD2 |
|  | n78 | 3780 | 10 | 50 |  | 3780 |  | N/A | N/A |
| DC_1A-5A_n78A | 1 | 1932 | 5 | 25 |  | 2122 |  | 18.1 | IMD3 |
|  | 5 | 829 | 5 | 25 |  | 874 |  | N/A | N/A |
|  | n78 | 3780 | 10 | 50 |  | 3780 |  | N/A | N/A |
|  | 1 | 1975 | 5 | 25 |  | 2165 |  | N/A | N/A |
|  | 5 | 840 | 5 | 25 |  | 885 |  | 3.1 | IMD5 |
|  | n78 | 3405 | 10 | 50 | 3405 |  | N/A |  | N/A |
| DC_1A-7A_n78A | 1 | 1977.5 | 5 | 25 |  | 2167.5 |  | N/A | N/A |
|  | 7 | 2507.5 | 5 | 25 |  | 2627.5 |  | 9.1 | IMD4 |
|  | n78 | 3305 | 10 | 50 |  | 3305 |  | N/A | N/A |
|  | 1 | 1950 | 5 | 25 |  | 2140 |  | 8.7 | IMD4 |
|  | 7 | 2510 | 10 | 50 |  | 2630 |  | N/A | N/A |
|  | n78 | 3580 | 10 | 50 |  | 3580 |  | N/A | N/A |
| DC_1A_n7A-n78A | 1 | 1977.5 | 5 | 25 |  | 2167.5 |  | N/A | N/A |
|  | n7 | 2507.5 | 5 | 25 |  | 2627.5 |  | 9.1 | IMD4 |
|  | n78 | 3305 | 10 | 50 |  | 3305 |  | N/A | N/A |
|  | 1 | 1970 | 5 | 25 |  | 2160 |  | N/A | N/A |
|  | n7 | 2520 | 5 | 25 |  | 2640 |  | N/A | N/A |
|  | n78 | 3390 | 10 | 50 |  | 3390 |  | 10.1 | IMD4 |
| DC_1A-3A_n79A | 1 | 1950 | 5 | 25 |  | 2140 |  | 3.6 | IMD5 |
|  | 3 | 1750 | 5 | 25 |  | 1845 |  | N/A | N/A |
|  | n79 | 4860 | 40 | 216 |  | 4860 |  | N/A | N/A |
| DC_1A-8A_n77A | 1 | 1955 | 5 | 25 |  | 2145 |  | N/A | N/A |
| DC_1A-8A_n77(2A) | n77 | 3410 | 10 | 50 |  | 2145 |  | N/A | N/A |
|  | 8 | N/A | 5 | N/A |  | 955 |  | 3.3 | IMD5 |
|  | 8 | 910 | 5 | 25 |  | 955 |  | N/A | N/A |
|  | n77 | 3960 | 10 | 50 |  | 3960 |  | N/A | N/A |
|  | 1 | N/A | 5 | N/A |  | 2140 |  | 14.4 | IMD3 |
| DC_1A-8A_n78A | 1 | N/A | N/A | N/A |  | N/A |  | N/A | N/A |
|  | 8 | N/A | N/A | N/A |  | N/A |  | N/A | IMD5 |
|  | n78 | N/A | N/A | N/A |  | N/A |  | N/A | N/A |
|  | 1 | 1945 | 5 | 25 |  | 2135 |  | N/A | N/A |
|  | 8 | 900 | 5 | 25 |  | 945 |  | N/A | N/A |
|  | n78 | 3745 | 10 | 52 |  | 3745 |  | 14.9 | IMD3 |
|  | 1 | 1940 | 5 | 25 |  | 2130 |  | N/A | N/A |
|  | 8 | 895 | 5 | 25 |  | 940 |  | 3.3 | IMD5 |
|  | n78 | 3380 | 10 | 52 |  | 3330 |  | N/A | N/A |
| DC_1A-18A_n77A | 1 | N/A | N/A | N/A |  | N/A |  | N/A | N/A |
|  | 18 | N/A | N/A | N/A |  | N/A |  | N/A | IMD5 |
|  | n77 | N/A | N/A | N/A |  | N/A |  | N/A | N/A |
|  | 1 | 1930 | 5 | 25 |  | 2120 |  | 16.4 | IMD3 |
|  | 18 | 825 | 5 | 25 |  | 870 |  | N/A | N/A |
|  | n77 | 3770 | 10 | 50 |  | 3770 |  | N/A | N/A |
| DC_1A-19A_n77A DC_1A-19A_n78A | 1 | 1940 | 5 | 25 |  | 2130 |  | 17.8 | IMD3 |
|  | 19 | 832.5 | 5 | 25 |  | 877.5 |  | N/A | N/A |
|  | n77, n78 | 3795 | 10 | 50 |  | 3795 |  | N/A | N/A |
|  | 1 | 1940 | 5 | 25 |  | 2130 |  | N/A | N/A |
|  | 19 | N/A | 5 | N/A |  | 880 |  | 5.1 | IMD5 |
|  | n78 | 3350 | 10 | 50 |  | 3350 |  | N/A | N/A |
| DC_1A-19A_n79A | 1 | 1950 | 5 | 25 |  | 2140 |  | N/A | N/A |
|  | 19 | 837.5 | 5 | 25 |  | 882.5 |  | 18.3 | IMD3 |
|  | n79 | 4782.5 | 40 | 216 |  | 4782.5 |  | N/A | N/A |
|  | 1 | 1950 | 5 | 25 |  | 2140 |  | 8.1 | IMD4 |
|  | 19 | 837.5 | 5 | 25 |  | 882.5 |  | N/A | N/A |
|  | n79 | 4652.5 | 40 | 216 |  | 4652.5 |  | N/A | N/A |
| DC_1A-20A_n8A | 1 | 1925 | 5 | 25 |  | 2115 |  | N/A | N/A |
|  | n8 | 910 | 5 | 25 |  | 955 |  | N/A | N/A |
|  | 20 | 846 | 5 | 25 |  | 805 |  | 11.5 | IMD4 |
| DC_1A-20A_n78A | 1 | 1930 | 5 | 25 |  | 2120 |  | 20.3 | IMD3 |
|  | 20 | 835 | 5 | 25 |  | 794 |  | N/A | N/A |
|  | n78 | 3790 | 10 | 50 |  | 3790 |  | N/A | N/A |
|  | 1 | 1950 | 5 | 25 |  | 2140 |  | N/A | N/A |
|  | 20 | 851 | 5 | 25 |  | 810 |  | 3.0 | IMD5 |
|  | n78 | 3330 | 10 | 50 |  | 3330 |  | N/A | N/A |
| DC_1A-21A_n77A DC_1A-21A_n78A | 1 | 1964.6 | 5 | 25 |  | 2154.6 |  | 30.6 | IMD2 |
|  | 21 | 1450.4 | 5 | 25 |  | 1498.4 |  | N/A | N/A |
|  | n77, n78 | 3605 | 10 | 50 |  | 3605 |  | N/A | N/A |
|  | 1 | N/A | 5 | N/A |  | 2154.6 |  | 3.6 | IMD5 |
|  | 21 | 1450.4 | 5 | 25 |  | 1498.4 |  | N/A | N/A |
|  | n78 | 3647 | 10 | 50 |  | 3647 |  | N/A | N/A |
|  | 1 | 1950 | 5 | 25 |  | 2140 |  | N/A | N/A |
|  | 21 | N/A | 5 | N/A |  | 1500 |  | 31.5 | IMD2 |
|  | n78 | 3450 | 10 | 50 |  | 3450 |  | N/A | N/A |
|  | 1 | 1950 | 5 | 25 |  | 2140 |  | N/A | N/A |
|  | 21 | 1452 | 5 | 25 |  | 1500 |  | 2.9 | IMD5 |
|  | n77, n78 | 3675 | 10 | 50 |  | 3675 |  | N/A | N/A |
| DC_1A-21A_n79A | 1 | N/A | N/A | N/A |  | N/A |  | N/A | N/A |
|  | 21 | N/A | N/A | N/A |  | N/A |  | N/A | IMD4 |
|  | n79 | N/A | N/A | N/A |  | N/A |  | N/A | N/A |
| DC_1A-28A_n3A | 28 | 710.5 | 5 | 25 |  | 765.5 |  | N/A | N/A |
|  | n3 | 1780 | 5 | 25 |  | 1875 |  | N/A | N/A |
|  | 1 | 1949 | 5 | 25 |  | 2139 |  | 11.0 | IMD4 |
| DC_1A-28A_n78A | 1 | N/A | 5 | N/A |  | 2150 |  | 15.7 | IMD3 |
|  | 28 | 740 | 5 | 25 |  | 795 |  | N/A | N/A |
|  | n78 | 3630 | 10 | 50 |  | 3630 |  | N/A | N/A |
|  | 1 | 1970 | 5 | 25 |  | 2160 |  | N/A | N/A |
|  | 28 | N/A | 5 | N/A |  | 794 |  | 4.2 | IMD5 |
|  | n78 | 3352 | 10 | 50 |  | 3352 |  | N/A | N/A |
| DC_1A_n28A-n78A | 1 | 1950 | 5 | 25 |  | 2140 |  | N/A | N/A |
|  | n28 | 733 | 5 | 25 |  | 788 |  | N/A | N/A |
|  | n78 | N/A | 10 | N/A |  | 3416 |  | 15.7 | IMD3 |
|  | 1 | 1950 | 5 | 25 |  | 2140 |  | N/A | N/A |
|  | n78 | 3320 | 10 | 50 |  | 3320 |  | N/A | N/A |
|  | n28 | N/A | 5 | N/A |  | 790 |  | 4.2 | IMD5 |
| DC_1A-41A_n28A | 1 | 1935 | 5 | 25 |  | 2125 |  | N/A | N/A |
|  | n28 | 718 | 5 | 25 |  | 773 |  | N/A | N/A |
|  | 41 | 2653 | 10 | 50 |  | 2653 |  | 30 | IMD2 |
| DC_1A-41A_n77A | 1 | 1970 | 5 | 25 |  | 2160 |  | N/A | N/A |
|  | n77 | 3400 | 10 | 50 |  | 3400 |  | N/A |  |
|  | 41 | 2510 | 5 | 25 |  | 2510 |  | 11.0 | IMD4 |
|  | 1 | 1950 | 5 | 25 |  | 2140 |  | 9.3 | IMD4 |
|  | n77 | 3710 | 10 | 50 |  | 3710 |  | N/A | N/A |
|  | 41 | 2640 | 5 | 25 |  | 2640 |  | N/A | N/A |
|  | 1 | 1930 | 5 | 25 |  | 2120 |  | N/A | N/A |
|  | n77 | 4150 | 10 | 50 |  | 4150 |  | N/A |  |
|  | 41 | 2510 | 5 | 25 |  | 2510 |  | 3.6 | IMD5 |
| DC_1A-41A_n78A | 1 | 1950 | 5 | 25 |  | 2140 |  | 9.3 | IMD4 |
|  | 41 | 2640 | 5 | 25 |  | 2640 |  | N/A | N/A |
|  | n78 | 3710 | 10 | 50 |  | 3710 |  | N/A | N/A |
|  | 1 | 1975 | 5 | 25 |  | 2165 |  | N/A | N/A |
|  | 41 | 2515 | 5 | 25 |  | 2515 |  | 12 | IMD4 |
|  | n78 | 3410 | 10 | 50 |  | 3410 |  | N/A | N/A |
|  | 1 | 1955 | 5 | 25 |  | 2145 |  | 8.7 | IMD4 |
|  | 41 | 2507.5 | 10 | 50 |  | 2507.5 |  | N/A | N/A |
|  | n78 | 3580 | 10 | 50 |  | 3580 |  | N/A | N/A |
| DC_1A-42A_n79A | 1 | 1977.5 | 5 | 25 |  | 2167.5 |  | N/A | N/A |
|  | n79 | 4420 | 40 | 216 |  | 4420 |  | N/A | N/A |
|  | 42 | 3490 | 5 | 25 |  | 3490 |  | 4.8 | IMD5 |
|  | 42 | 3402.5 | 5 | 25 |  | 3402.5 |  | N/A | N/A |
|  | n79 | 4640 | 40 | 216 |  | 4640 |  | N/A | N/A |
|  | 1 | 1975 | 5 | 25 |  | 2165 |  | 15.5 | IMD3 |
|  | 42 | 3450 | 5 | 25 |  | 3450 |  | N/A | N/A |
|  | n79 | 4520 | 40 | 216 |  | 4520 |  | N/A | N/A |
|  | 1 | 1950 | 5 | 25 |  | 2140 |  | 9.3 | IMD4 |
| DC_1A_n78A-n79A | 1 | 1950 | 5 | 25 |  | 2140 |  | N/A | N/A |
|  | n78 | 3410 | 10 | 50 |  | 3410 |  | N/A | N/A |
|  | n79 | 4870 | 40 | 216 |  | 4870 |  | 15.9 | IMD3 |
|  | 1 | 1950 | 5 | 25 |  | 2140 |  | N/A | N/A |
|  | n79 | 4670 | 40 | 216 |  | 4670 |  | N/A | N/A |
|  | n78 | 3490 | 10 | 50 |  | 3490 |  | 4.6 | IMD5 |
| DC_2A-5A_n77A11 DC_2A-2A-5A_n77A11 DC_2A-2A-5A_n77(2A)11 | 2 | 1907.5 | 5 | 25 |  | 1987.5 |  | N/A | N/A |
|  | 5 | N/A | 5 | N/A |  | 887.5 |  | 3.8 | IMD5 |
|  | n77 | 3305 | 10 | 50 |  | 3305 |  | N/A | N/A |
|  | 2 | N/A | 5 | N/A |  | 1987 |  | 16.5 | IMD3 |
|  | 5 | 846.5 | 5 | 25 |  | 891.5 |  | N/A | N/A |
|  | n77 | 3680 | 10 | 50 |  | 3680 |  | N/A | N/A |
|  | 2 | 1880 | 5 | 25 |  | 1960 |  | N/A | N/A |
| DC_2A_n5A-n77A | n5 | 830 | 5 | 25 |  | 875 |  | N/A | N/A |
|  | n77 | N/A | 10 | N/A |  | 3540 |  | 16.0 | IMD3 |
| DC_2A-12A_n66A | 2 | N/A | N/A | N/A |  | N/A |  | N/A | IMD4 |
|  | 12 | N/A | N/A | N/A |  | N/A |  | N/A | N/A |
|  | n66 | N/A | N/A | N/A |  | N/A |  | N/A | N/A |
|  | 2 | N/A | 5 | N/A |  | 1944 |  | 16.0 | IMD3 |
| DC_2A-13A_n77A | 13 | 783 | 5 | 25 |  | 752 |  | N/A | N/A |
|  | n77 | 3510 | 10 | 50 |  | 3510 |  | N/A | N/A |
| DC_2A-14A_n66A | 2 | 1874 | 5 | 25 |  | 1954 |  | 7.2 | IMD4 |
|  | 14 | 793 | 5 | 25 |  | 763 |  | N/A | N/A |
|  | 66 | 1770 | 5 | 25 |  | 2170 |  | N/A | N/A |
| DC_2A-14A_n77A DC_2A-14A_n77(2A) DC_2A-2A-14A_n77A DC_2A-2A-14A_n77(2A) | 2 | N/A | 5 | N/A |  | 1954 |  | 16.5 | IMD3 |
|  | 14 | 793 | 5 | 25 |  | 763 |  | N/A | N/A |
|  | n77 | 3540 | 10 | 50 |  | 3540 |  | N/A | N/A |
| DC_2A-66A_n2A | 2 | N/A | 5 | N/A |  | 1980 |  | 20 | IMD3 |
|  | 66 | 1730 | 5 | 25 |  | 2130 |  | N/A | N/A |
|  | n2 | 1855 | 5 | 25 |  | 1935 |  | N/A | N/A |
| DC_2A-66A_n5A, DC_2A-2A-66A_n5A DC_2A-66A-66A_n5A DC_2A-2A-66A-66A_n5A | 2 | 1900 | 5 | 25 |  | 1980 |  | N/A | N/A |
|  | 66 | 1740 | 5 | 25 |  | 2140 |  | 7.2 | IMD4 |
|  | n5 | 830 | 5 | 25 |  | 875 |  | N/A | N/A |
| DC_2A-66A_n41A | 2 | 1860 | 5 | 25 |  | 1940 |  | 22.6 | IMD4 |
|  | 66 | 1715 | 5 | 25 |  | 2115 |  | N/A | N/A |
|  | n41 | 2685 | 5 | 25 |  | 2685 |  | N/A | N/A |
| DC_2A-66A_n77A DC_2A-66A_n77(2A) DC_2A-2A-66A_n77A DC_2A-2A-66A_n77(2A) DC_2A-66A-66A_n77A DC_2A-66A-66A_n77(2A) DC_2A-2A-66A-66A_n77A | 2 | 1855 | 5 | 25 |  | 1935 |  | N/A | N/A |
|  | 66 | N/A | 5 | N/A |  | 2115 |  | 29.2 | IMD2 |
|  | n77 | 3970 | 10 | 50 |  | 3970 |  | N/A | N/A |
|  | 2 | 1880 | 5 | 25 |  | 1960 |  | M/A | N/A |
|  | 66 | N/A | 5 | N/A |  | 2140 |  | 10.4 | IMD4 |
|  | n77 | 3500 | 10 | 50 |  | 3500 |  | N/A | N/A |
|  | 2 | 1885 | 5 | 25 |  | 1965 |  | M/A | N/A |
|  | 66 | N/A | 5 | N/A |  | 2175 |  | 4.0 | IMD5 |
|  | n77 | 3915 | 10 | 50 |  | 3915 |  | N/A | N/A |
|  | 2 | N/A | 5 | N/A |  | 1960 |  | 32.1 | IMD2 |
|  | 66 | 1760 | 5 | 25 |  | 2160 |  | N/A | N/A |
|  | n77 | 3720 | 10 | 50 |  | 3720 |  | N/A | N/A |
| DC_3A-5A_n78A | 3 | N/A | N/A | N/A |  | N/A |  | N/A | IMD3 |
|  | 5 | N/A | N/A | N/A |  | N/A |  | N/A | N/A |
|  | n78 | N/A | N/A | N/A |  | N/A |  | N/A | N/A |
| DC_3A-7A_n5A | 3 | 1780 | 10 | 50 |  | 1875 |  | N/A | N/A |
|  | 7 | 2505 | 10 | 50 |  | 2625 |  | 30.0 | IMD2X |
|  | n5 | 845 | 5 | 25 |  | 890 |  | N/A | N/A |
| DC_3A-7A_n28A DC_3C-7A_n28A | 3 | 1712.5 | 5 | 25 |  | 1807.5 |  | N/A | N/A |
|  | n28 | 743 | 5 | 25 |  | 798 |  | N/A | N/A |
|  | 7 | 2562 | 10 | 50 |  | 2682 |  | 16.9 | IMD3 |
|  | 7 | 2543 | 10 | 50 |  | 2663 |  | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 |  | 765.5 |  | N/A | N/A |
|  | 3 | 1737.5 | 5 | 25 |  | 1832.5 |  | 26.0 | IMD2 |
| DC_3A-7A_n78A DC_3C-7A_n78A | 3 | 1725 | 5 | 25 |  | 1820 |  | 17.6 | IMD3 |
|  | 7 | 2565 | 5 | 25 |  | 2685 |  | N/A | N/A |
|  | n78 | 3310 | 10 | 50 |  | 3310 |  | N/A | N/A |
|  | 3 | 1725 | 5 | 25 |  | 1820 |  | 8.6 | IMD4 |
|  | 7 | 2565 | 5 | 25 |  | 2685 |  | N/A | N/A |
|  | n78 | 3475 | 10 | 50 |  | 3475 |  | N/A | N/A |
| DC_3A-8A_n77A | 3 | 1715 | 5 | 25 |  | 1810 |  | N/A | N/A |
| DC_3A-8A_n77(2A) | n77 | 4190 | 10 | 50 |  | 4190 |  | N/A | N/A |
|  | 8 | N/A | 5 | N/A |  | 955 |  | 9.7 | IMD4 |
|  | 8 | 910 | 5 | 25 |  | 955 |  | N/A | N/A |
|  | n77 | 3640 | 10 | 50 |  | 3640 |  | N/A | N/A |
|  | 3 | N/A | 5 | N/A |  | 1820 |  | 16.5 | IMD3 |
| DC_3A-8A_n78A | 8 | 910 | 5 | 25 |  | 955 |  | N/A | N/A |
|  | n78 | 3640 | 10 | 50 |  | 3640 |  | N/A | N/A |
|  | 3 | 1725 | 5 | 25 |  | 1820 |  | 16.5 | IMD3 |
| DC_3A-18A_n77A DC_3A-18A_n78A | 3 | N/A | N/A | N/A |  | N/A |  | N/A | IMD3 |
|  | 18 | N/A | N/A | N/A |  | N/A |  | N/A | N/A |
|  | n77, n78 | N/A | N/A | N/A |  | N/A |  | N/A | N/A |
| DC_3A-19A_n78A | 3 | N/A | 5 | N/A |  | 1850 |  | 17.3 | IMD3 |
|  | 19 | 835 | 5 | 25 |  | 880 |  | N/A | N/A |
|  | n78 | 3520 | 10 | 50 |  | 3520 |  | N/A | N/A |
| DC_3A-19A_n79A | 3 | 1775 | 5 | 25 |  | 1870 |  | N/A | N/A |
|  | 19 | 840 | 5 | 25 |  | 885 |  | [18.5] | IMD3 |
|  | n79 | 4435 | 40 | 216 |  | 4435 |  | N/A | N/A |
|  | 3 | 1782.5 | 5 | 25 |  | 1877.5 |  | 5.5 | IMD4 |
|  | 19 | 842.5 | 5 | 25 |  | 887.5 |  | N/A | N/A |
|  | n79 | 4420 | 40 | 216 |  | 4420 |  | N/A | N/A |
| DC_3A-20A_n28A DC_3C-20A_n28A | 20 | 852 | 5 | 25 |  | 811 |  | N/A | N/A |
|  | n28 | 728 | 5 | 25 |  | 783 |  | N/A | N/A |
|  | 3 | 1733 | 5 | 25 |  | 1828 |  | 9.4 | IMD4 |
| DC_3A_n20A-n78A | 3 | 1730 | 5 | 25 |  | 1825 |  | N/A | N/A |
|  | n20 | 845 | 5 | 25 |  | 804 |  | N/A | N/A |
|  | n78 | 3420 | 10 | 50 |  | 3420 |  | 16.1 | IMD3 |
| DC_3A-20A_n78A | 3 | 1725 | 5 | 25 |  | 1820 |  | 17.3 | IMD3 |
|  | 20 | 845 | 5 | 25 |  | 804 |  | N/A | N/A |
|  | n78 | 3510 | 10 | 50 |  | 3510 |  | N/A | N/A |
| DC_3A-21A_n77A DC_3A-21A_n78A | 3 | 1767.5 | 5 | 25 |  | 1862.5 |  | N/A | N/A |
|  | 21 | 1459.5 | 5 | 25 |  | 1507.5 |  | 8.8 | IMD4 |
|  | n77, n78 | 3795 | 10 | 50 |  | 3795 |  | N/A | N/A |
|  | 3 | N/A | 5 | N/A |  | 1862.5 |  | 30.8 | IMD2 |
|  | 21 | 1459.5 | 5 | 25 |  | 1507.5 |  | N/A | N/A |
|  | n78 | 3322 | 10 | 50 |  | 3322 |  | N/A | N/A |
| DC_3A-21A_n77A | 3 | 1771.6 | 5 | 25 |  | 1866.6 |  | 3.4 | IMD5 |
|  | 21 | 1450.4 | 5 | 25 |  | 1498.4 |  | N/A | N/A |
|  | n77 | 3935 | 10 | 50 |  | 3935 |  | N/A | N/A |
| DC_3A-21A_n79A | 3 | N/A | N/A | N/A |  | N/A |  | N/A | N/A |
|  | 21 | N/A | N/A | N/A |  | N/A |  | N/A | IMD3 |
|  | n79 | N/A | N/A | N/A |  | N/A |  | N/A | N/A |
|  | 3 | 1774.2 | 5 | 25 |  | 1869.2 |  | 17.8 | IMD3 |
|  | 21 | 1450.4 | 5 | 25 |  | 1498.4 |  | N/A | N/A |
|  | n79 | 4770 | 40 | 216 |  | 4770 |  | N/A | N/A |
| DC_3A-28A_n77A | 3 | 1712.5 | 5 | 25 |  | 1807.5 |  | N/A | N/A |
|  | 28 | 715 | 5 | 25 |  | 770 |  | 15.3 | IMD3 |
|  | n77 | 4195 | 10 | 50 |  | 4195 |  | N/A | N/A |
|  | 3 | 1755 | 5 | 25 |  | 1850 |  | 17.0 | IMD3 |
|  | 28 | 735 | 5 | 25 |  | 790 |  | N/A | N/A |
|  | n77 | 3320 | 10 | 50 |  | 3320 |  | N/A | N/A |
| DC_3A-28A_n78A | 3 | 1775 | 5 | 25 |  | 1870 |  | 17.3 | IMD3 |
|  | 28 | 740 | 5 | 25 |  | 760 |  | N/A | N/A |
|  | n78 | 3350 | 10 | 25 |  | 3350 |  | N/A | N/A |
| DC_3A_n28A-n78A | 3 | 1750 | 5 | 25 |  | 1845 |  | N/A | N/A |
|  | n28 | 743 | 5 | 25 |  | 798 |  | N/A | N/A |
|  | n78 | 3764 | 10 | 50 |  | 3764 |  | 4.5 | IMD5 |
| DC_3A-40A_n1A | n1 | 1950 | 5 | 25 |  | 2140 |  | N/A | N/A |
|  | 3 | 1735 | 5 | 25 |  | 1830 |  | N/A | N/A |
|  | 40 | 2380 | 5 | 25 |  | 2380 |  | 8.0 | IMD5 |
| DC_3A-41A_n28A DC_3A-41C_n28A | 41 | 2543 | 10 | 50 |  | 2543 |  | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 |  | 765.5 |  | N/A | N/A |
|  | 3 | 1737.5 | 5 | 25 |  | 1832.5 |  | 26 | IMD2 |
|  | 3 | 1780 | 5 | 25 |  | 1875 |  | N/A | N/A |
|  | n28 | 738 | 5 | 25 |  | 793 |  | N/A | N/A |
|  | 41 | 2518 | 5 | 25 |  | 2518 |  | 27.4 | IMD2 |
|  | 3 | 1715 | 5 | 25 |  | 1810 |  | N/A | N/A |
|  | n28 | 743 | 5 | 25 |  | 798 |  | N/A | N/A |
|  | 41 | 2687 | 5 | 25 |  | 2687 |  | 15.9 | IMD3 |
| DC_3A-41A_n77A DC_3A-41C_n77A DC_3A-41A_n77(2A) | 3 | 1720 | 5 | 25 |  | 1815 |  | N/A | N/A |
|  | n77 | 3900 | 10 | 50 |  | 3900 |  | N/A | N/A |
|  | 41 | 2640 | 5 | 25 |  | 2640 |  | 5.3 | IMD5 |
|  | 41 | 2620 | 5 | 25 |  | 2620 |  | N/A | N/A |
|  | n77 | 3400 | 10 | 50 |  | 3400 |  | N/A | N/A |
|  | 3 | 1745 | 5 | 25 |  | 1840 |  | 16.4 | IMD3 |
| DC_3A_n78A-n79A | 3 | 1770 | 5 | 25 |  | 1865 |  | N/A | N/A |
|  | n78 | 3340 | 10 | 50 |  | 3340 |  | N/A | N/A |
|  | n79 | 4910 | 40 | 216 |  | 4910 |  | 16.3 | IMD3 |
|  | 3 | 1770 | 5 | 25 |  | 1865 |  | N/A | N/A |
|  | n79 | 4510 | 40 | 216 |  | 4510 |  | N/A | N/A |
|  | n78 | 3710 | 10 | 50 |  | 3710 |  | 4.2 | IMD5 |
| DC_3A-21A_n79A | 3 | 1774.2 | 5 | 25 |  | 1869.2 |  | 17.8 | IMD3 |
|  | 21 | 1450.4 | 5 | 25 |  | 1498.4 |  | N/A | N/A |
|  | n79 | 4770 | 40 | 216 |  | 4770 |  | N/A | N/A |
| DC_5A-7A_n78A | 5 | 844 | 5 | 25 |  | 889 |  | N/A | N/A |
|  | 7 | 2525 | 10 | 50 |  | 2645 |  | 30.1 | IMD2 |
|  | n78 | 3489 | 10 | 50 |  | 3489 |  | N/A | N/A |
|  | 5 | 834 | 5 | 25 |  | 879 |  | 30.2 | IMD2 |
|  | 7 | 2550 | 10 | 50 |  | 2670 |  | N/A | N/A |
|  | n78 | 3429 | 10 | 50 |  | 3429 |  | N/A | N/A |
|  | 5 | 830 | 5 | 25 |  | 875 |  | 3.3 | IMD5 |
|  | 7 | 2525 | 10 | 50 |  | 2645 |  | N/A | N/A |
|  | n78 | 3350 | 10 | 50 |  | 3350 |  | N/A | N/A |
| DC_5A-66A_n77A DC_5A-66A_n77(2A) DC_5A-66A-66A_n77A DC_5A-66A-66A_n77(2A) | 5 | 826.5 | 5 | 25 |  | 871.5 |  | N/A | N/A |
|  | 66 | N/A | 5 | N/A |  | 2142 |  | 13.2 | IMD3 |
|  | n77 | 3795 | 10 | 50 |  | 3795 |  | N/A | N/A |
| DC_5A-66A_n2A DC_5A-66A-66A_n2A | 5 | 834 | 5 | 25 |  | 879 |  | N/A | N/A |
|  | 66 | N/A | 5 | N/A |  | 2132 |  | 7.2 | IMD4 |
|  | n2 | 1900 | 5 | 25 |  | 1980 |  | N/A | N/A |
| DC_7A-8A_n3A | n3 | 1735 | 5 | 25 |  | 1830 |  | N/A | N/A |
|  | 7 | 2530 | 10 | 50 |  | 2650 |  | N/A | N/A |
|  | 8 | 895 | 5 | 25 |  | 940 |  | 18.0 | IMD3 |
|  | n3 | 1780 | 5 | 25 |  | 1875 |  | N/A | N/A |
|  | 8 | 890 | 5 | 25 |  | 935 |  | N/A | N/A |
|  | 7 | 2550 | 10 | 50 |  | 2670 |  | 29.0 | IMD2+IMD34 |
| DC_7A-20A_n1A | 7 | 2510 | 10 | 50 |  | 2630 |  | N/A | N/A |
|  | 20 | 841 | 10 | 50 |  | 800 |  | 4.5 | IMD5 |
|  | n1 | 1940 | 5 | 25 |  | 2130 |  | N/A | N/A |
| DC_7A-20A_n3A | 7 | 2543 | 10 | 50 |  | 2663 |  | N/A | N/A |
|  | 20 | 847 | 10 | 20 |  | 806 |  | 10.5 | IMD2 |
|  | n3 | 1737 | 5 | 25 |  | 1832 |  | N/A | N/A |
|  | 7 | 2510 | 10 | 50 |  | 2630 |  | 26.0 | IMD21 |
|  | 20 | 855 | 5 | 25 |  | 814 |  | N/A | N/A |
|  | n3 | 1775 | 10 | 50 |  | 1870 |  | N/A | N/A |
| DC_7A-20A_n8A | 7 | 2565 | 5 | 25 |  | 2685 |  | N/A | N/A |
|  | n8 | 885 | 5 | 25 |  | 930 |  | N/A | N/A |
|  | 20 | 836 | 5 | 25 |  | 795 |  | 17.4 | IMD3 |
|  | 7 | 2520 | 5 | 25 |  | 2640 |  | 21.1 | IMD3 |
|  | n8 | 900 | 5 | 25 |  | 945 |  | N/A | N/A |
|  | 20 | 840 | 5 | 25 |  | 799 |  | N/A | N/A |
|  | 7 | 2504 | 5 | 25 |  | 2624 |  | 18.8 | IMD3 |
|  | n8 | 910 | 5 | 25 |  | 955 |  | N/A | N/A |
|  | 20 | 857 | 5 | 25 |  | 816 |  | N/A | N/A |
| DC_7A-20A_n28A | 20 | 842 | 5 | 25 |  | 801 |  | N/A | N/A |
|  | n28 | 728 | 5 | 25 |  | 783 |  | N/A | N/A |
|  | 7 | 2520 | 10 | 50 |  | 2640 |  | 5.9 | IMD5 |
| DC_7A-20A_n78A | 7 | 2560 | 5 | 25 |  | 2680 |  | N/A | N/A |
|  | 20 | 851 | 5 | 25 |  | 810 |  | 30.5 | IMD2 |
|  | n78 | 3370 | 10 | 50 |  | 3370 |  | N/A | N/A |
|  | 7 | 2560 | 5 | 25 |  | 2680 |  | N/A | N/A |
|  | 20 | 851 | 5 | 25 |  | 810 |  | 3.0 | IMD5 |
|  | n78 | 3435 | 10 | 50 |  | 3435 |  | N/A | N/A |
|  | 7 | 2555 | 5 | 25 |  | 2675 |  | 30.8 | IMD2 |
|  | 20 | 845 | 5 | 25 |  | 804 |  | N/A | N/A |
|  | n78 | 3520 | 10 | 50 |  | 3520 |  | N/A | N/A |
| DC_7A-28A_n3A | 7 | 2543 | 5 | 25 |  | 2663 |  | N/A | N/A |
|  | 28 | 741 | 5 | 25 |  | 796.0 |  | 20.0 | IMD2 |
|  | n3 | 1747 | 5 | 25 |  | 1842 |  | N/A | N/A |
|  | 7 | 2540 | 5 | 25 |  | 2685 |  | 18 | IMD3 |
|  | 28 | 745 | 5 | 25 |  | 800 |  | N/A | N/A |
|  | n3 | 1715 | 5 | 25 |  | 1810 |  | N/A | N/A |
| DC_7A-28A_n5A DC_7C-28A_n5A | 7 | 2540 | 5 | 25 |  | 2725 |  | N/A | N/A |
|  | 28 | 721 | 5 | 25 |  | 776 |  | 4.4 | IMD5 |
|  | n5 | 829 | 5 | 25 |  | 854 |  | N/A | N/A |
|  | 7 | 2510 | 5 | 25 |  | 2630 |  | 5.9 | IMD5 |
|  | 28 | 730 | 5 | 25 |  | 785 |  | N/A | N/A |
|  | n5 | 840 | 5 | 25 |  | 874 |  | N/A | N/A |
| DC_7A-28A_n78A | 7 | 2570 | 5 | 25 |  | 2670 |  | N/A | N/A |
|  | 28 | 720 | 5 | 25 |  | 780 |  | 8.3 | IMD2 |
|  | n78 | 3350 | 10 | 50 |  | 3421 |  | N/A | N/A |
|  | 7 | 2570 | 5 | 25 |  | 2670 |  | N/A | N/A |
|  | 28 | 720 | 5 | 25 |  | 790 |  | 3.0 | IMD5 |
|  | n78 | 3460 | 10 | 50 |  | 3421 |  | N/A | N/A |
|  | 7 | 2570 | 5 | 25 |  | 2650 |  | 30.5 | IMD2 |
|  | 28 | 740 | 5 | 25 |  | 768 |  | N/A | N/A |
|  | n78 | 3390 | 10 | 50 |  | 3421 |  | N/A | N/A |
| DC_7A_n28A-n78A DC_7C_n28A-n78A | 7 | 2565 | 5 | 25 |  | 2685 |  | N/A | N/A |
|  | n28 | 745 | 5 | 25 |  | 800 |  | N/A | N/A |
|  | n78 | 3310 | 10 | 50 |  | 3310 |  | 29.7 | IMD2 |
|  | 7 | 2565 | 5 | 25 |  | 2685 |  | N/A | N/A |
|  | n78 | 3365 | 10 | 50 |  | 3365 |  | N/A | N/A |
|  | n28 | 745 | 5 | 25 |  | 800 |  | 28.8 | IMD2 |
|  | 13 | 782 | 5 | 25 |  | 751 |  | N/A | N/A |
|  | n2 | 1896 | 5 | 25 |  | 1976 |  | N/A | N/A |
|  | n77 | N/A | 10 | N/A |  | 3460 |  | 17.3 | IMD3 |
| DC_13A_n2A-n77A | 13 | 782 | 5 | 25 |  | 751 |  | N/A | N/A |
|  | n2 | N/A | 5 | N/A |  | 1960 |  | 16.0 | IMD3 |
|  | n77 | 3524 | 10 | 50 |  | 3524 |  | N/A | N/A |
|  | 13 | 782 | 5 | 25 |  | 751 |  | N/A | N/A |
| DC_13A-66A_n2A | 66 | N/A | 5 | N/A |  | 2156 |  | 7..2 | IMD4 |
|  | n2 | 1860 | 5 | 25 |  | 1940 |  | N/A | N/A |
|  | 13 | 782 | 5 | 25 |  | 751 |  | N/A | N/A |
| DC_13A-66A_n77A | 66 | N/A | 5 | N/A |  | 2156 |  | 17.1 | IMD3 |
|  | n77 | 3720 | 10 | 50 |  | 3720 |  | N/A | N/A |
| DC_14A-66A_n2A DC_14A-66A-66A_n2A | 14 | 793 | 5 | 25 |  | 763 |  | N/A | N/A |
|  | 66 | 1762 | 5 | 25 |  | 2162 |  | 7.6 | IMD4 |
|  | n2 | 1874 | 5 | 25 |  | 1954 |  | N/A | N/A |
| DC_14A-66A_n77A DC_14A-66A_n77(2A) DC_14A-66A-66A_n77A DC_14A-66A-66A_n77(2A) | 14 | N/A | 5 | N/A |  | 763 |  | 15.2 | IMD311 |
|  | 66 | 1712.5 | 5 | 25 |  | 2112.5 |  | N/A | N/A |
|  | n77 | 4188 | 10 | 50 |  | 4188 |  | N/A | N/A |
|  | 14 | 793 | 5 | 25 |  | 763 |  | N/A | N/A |
|  | 66 | N/A | 5 | N/A |  | 2155 |  | 13.2 | IMD3 |
|  | n77 | 3741 | 10 | 50 |  | 3741 |  | N/A | N/A |
| DC_18A-41A_n3A DC_18A-41C_n3A | 18 | 820 | 5 | 25 |  | 865 |  | N/A | N/A |
|  | n3 | 1725 | 5 | 25 |  | 1820 |  | N/A | N/A |
|  | 41 | 2630 | 5 | 25 |  | 2630 |  | 16.0 | IMD3 |
|  | 18 | 820 | 5 | 25 |  | 865 |  | 28.9 | IMD2 |
|  | n3 | 1765 | 5 | 25 |  | 1820 |  | N/A | N/A |
|  | 41 | 2630 | 5 | 25 |  | 2630 |  | N/A | N/A |
| DC_18A-41A_n77A DC_18A-41C_n77A | 18 | 820 | 5 | 25 |  | 865 |  | 3.4 | IMD5 |
|  | n77 | 3527.5 | 10 | 50 |  | 3527.5 |  | N/A | N/A |
|  | 41 | 2640 | 5 | 25 |  | 2640 |  | N/A | N/A |
| DC_18A-41A_n78A DC_18A-41C_n78A | 18 | 820 | 5 | 25 |  | 865 |  | 3.4 | IMD5 |
|  | n78 | 3527.5 | 10 | 50 |  | 3527.5 |  | N/A | N/A |
|  | 41 | 2640 | 5 | 25 |  | 2640 |  | N/A | N/A |
| DC_19A-21A_n77A DC_19A-21A_n78A | 19 | 837.5 | 5 | 25 |  | 882.5 |  | 18.7 | IMD3 |
|  | 21 | 1450.4 | 5 | 25 |  | 1498.4 |  | N/A | N/A |
|  | n77, n78 | 3783.3 | 10 | 50 |  | 3783.3 |  | N/A | N/A |
|  | 19 | N/A | 5 | N/A |  | 882.5 |  | 13.2 | IMD4 |
|  | 21 | 1450.4 | 5 | 25 |  | 1498.4 |  | N/A | N/A |
|  | n77, n78 | 3468.7 | 10 | 50 |  | 3468.7 |  | N/A | N/A |
| DC_19A-21A_n77A | 19 | 837.5 | 5 | 25 |  | 882.5 |  | N/A | N/A |
|  | 21 | 1454.5 | 5 | 25 |  | 1502.5 |  | 9.0 | IMD4 |
|  | n77 | 4015 | 10 | 50 |  | 4015 |  | N/A | N/A |
| DC_19A-21A_n79A | 19 | N/A | N/A | N/A |  | N/A |  | N/A | IMD5 |
|  | 21 | N/A | N/A | N/A |  | N/A |  | N/A | N/A |
|  | n79 | N/A | N/A | N/A |  | N/A |  | N/A | N/A |
|  | 19 | 837.5 | 5 | 25 |  | 882.2 |  | N/A | N/A |
|  | 21 | 1452 | 5 | 25 |  | 1500 |  | 3.8 | IMD5 |
|  | n79 | 4850 | 40 | 216 |  | 4850 |  | N/A | N/A |
| DC_20A_n3A-n78A | 20 | 845 | 5 | 25 |  | 804 |  | N/A | N/A |
|  | n3 | 1730 | 5 | 25 |  | 1825 |  | N/A | N/A |
|  | n78 | 3420 | 10 | 50 |  | 3420 |  | 16.1 | IMD3 |
|  | 20 | 845 | 5 | 25 |  | 804 |  | N/A | N/A |
|  | n3 | 1765 | 5 | 25 |  | 1860 |  | 15.7 | IMD3 |
|  | n78 | 3550 | 10 | 50 |  | 3550 |  | N/A | N/A |
| DC_28A_n7A-n78A | 28 | 745 | 5 | 25 |  | 800 |  | N/A | N/A |
|  | n7 | 2565 | 5 | 25 |  | 2685 |  | N/A | N/A |
|  | n78 | 3310 | 10 | 50 |  | 3310 |  | 29.7 | IMD2 |
|  | 28 | 740 | 5 | 25 |  | 795 |  | N/A | N/A |
|  | n7 | 2530 | 5 | 25 |  | 2650 |  | 30.5 | IMD2 |
|  | n78 | 3390 | 10 | 50 |  | 3390 |  | N/A | N/A |
| DC_19A_n78A-n79A | 19 | 835 | 5 | 25 |  | 880 |  | N/A | N/A |
|  | n78 | 3680 | 10 | 50 |  | 3680 |  | N/A | N/A |
|  | n79 | 4515 | 40 | 216 |  | 4515 |  | 29.3 | IMD2 |
|  | 19 | 835 | 5 | 25 |  | 880 |  | N/A | N/A |
|  | n79 | 4550 | 40 | 216 |  | 4550 |  | N/A | N/A |
|  | n78 | 3715 | 10 | 50 |  | 3715 |  | 28.8 | IMD2 |
| DC_20A_n28A-n78A DC_20A_SUL_n78A-n83A | 20 | 857 | 5 | 25 |  | 816 |  | N/A | N/A |
|  | n28, n83 | 743 | 5 | 25 |  | 798 |  | N/A | N/A |
|  | n78 | 3314 | 10 | 50 |  | 3314 |  | 8.7 | IMD4 |
|  | 20 | 837 | 5 | 25 |  | 796 |  | N/A | N/A |
|  | n78 | 3310 | 10 | 50 |  | 3310 |  | N/A | N/A |
|  | n28 | 744 | 5 | 25 |  | 799 |  | 9.4 | IMD4 |
| DC_21A_n78A-n79A | 21 | 1453 | 5 | 25 |  | 1501 |  | N/A | N/A |
|  | n78 | 3420 | 10 | 50 |  | 3420 |  | N/A | N/A |
|  | n79 | 4873 | 40 | 216 |  | 4873 |  | 30.1 | IMD2 |
|  | 21 | 1453 | 5 | 25 |  | 1501 |  | N/A | N/A |
|  | n79 | 4940 | 40 | 216 |  | 4940 |  | N/A | N/A |
|  | n78 | 3487 | 10 | 50 |  | 3487 |  | 29.8 | IMD2 |
|  | n2 | N/A | 5 | N/A |  | 1960 |  | 32.1 | IMD2 |
| DC_66A_n2A-n77A | 66 | 1760 | 5 | 25 |  | 2160 |  | N/A | N/A |
|  | n77 | 3720 | 10 | 50 |  | 3720 |  | N/A | N/A |
|  | 66 | 1770 | 5 | 25 |  | 2170 |  | N/A | N/A |
| DC_66A_n5A-n77A | n5 | 845 | 5 | 25 |  | 890 |  | N/A | N/A |
|  | n77 | N/A | 10 | N/A |  | 3460 |  | 16.6 | IMD39 |
| NOTE 1: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 2: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 3: This band is subject to IMD3 also which MSD is not specified. NOTE 4: This MSD requirement apply with both IMD2 and IMD3 products should be generated. NOTE 5 to NOTE 8: FFS NOTE 9: This band is subject to IMD4 also which MSD is not specified. NOTE 10: FFS NOTE 11: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |

==============================================================

<< End of changes >>