Title:	Discussion on test point selection for NS_18 A-MPR and A-SE in FR1

Document for:	Discussion and approval

# Background

The purpose of this paper is to provide an analysis of the parameters for the A-MPR test configuration table with a selection of test points that minimize testing and still achieve good test coverage for NS_18. The intention with NS_18 is to protect 692-698 MHz (TV bands) as indicated in Table 1 below.

Table 1: Additional requirement for"NS_18"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |  |
| --- | --- | --- | --- |
|  | 3,5, 10, 15, 20, 25, 30 MHz |  |  |
| 692-698 | -26.2 | 6 MHz |  |

The general agreement in RAN5 is that FR1 testing should follow what is being tested for LTE. This is also valid for A-MPR FR1 test point selection.

The selection of test points below is based on approved test point analyzes for SEM [2, 3] for FR1. However, for NS_18 the lower part of operating band n28, n83 and n109 (UL 703 - 733 MHz) has specific A-MPR requirements to protect 692-698 MHz (TV bands), see Table 2. There are no specific A-MPR requirements for upper part of operating band n28, n83 and n109.

Table 2: A-MPR regions for protection of 692-698 MHz (NS_18) (from [4])

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Inner/Outer | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 | N/A | ≤ 5 | 3 | 8 | 3 |
|  | QPSK | ≤ 2 |  | ≤ 5 | 3 | 8 | 3 |
|  | 16 QAM | ≤ 3 |  | ≤ 6 | 3 | 8 | 3 |
|  | 64 QAM | ≤ 4 |  | ≤ 7 | 3 | 8 | 4.5 |
|  | 256 QAM | ≤ 6 |  | ≤ 9 | 3 | 8 | 5.5 |
| CP-OFDM | QPSK | ≤ 5 |  | ≤ 6.5 | 4.5 | 9.5 | 5 |
|  | 16 QAM | ≤ 5 |  | ≤ 7 | 4.5 | 9.5 | 5 |
|  | 64 QAM | ≤ 5.5 |  | ≤ 8.5 | 4.5 | 9.5 | 5.5 |
|  | 256 QAM | ≤ 8.5 |  | ≤ 11.5 | 4.5 | 9.5 | 7.5 |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |

The A-MPR requirements in Table 2 are split into 3 regions, A1 for 3, 5MHz, A2 for 10, 15 and 20MHz and A3, A4 and A5 for 25 ,30 and 40MHz for PC3 and A3,A4,A5 and A6 for 25, 30 and 40MHz for PC2. The UL RB allocation for testing of these regions should be allocated to the lower part of operating band n28, n83 and n109.

## Test Environment

Based on the baseline agreed in [2], the test environment for NR A-MPR measurement for FR1 can be defined as in LTE.

PROPOSAL 1: Select Test Environment as Normal Conditions (NC) for A-MPR.

## Test Frequencies

UL test carrier centre frequencies should be located to the lower part of operating band n28, n83 and n109, since this frequency range is closest to the band to be protected.

PROPOSAL 2: Select Low Range as test frequency for NS_18 A-MPR tests.

## Test Channel Bandwidth

The minimum requirements for NS_18 apply for channel bandwidths 3, 5, 10, 15, 20, 25, 30 and 40MHz. The requirements are different for groups {3, 5MHz}, {10, 15, 20MHz} and {25, 30, 40MHz}. Therefore, it is enough to test with 5MHz, 20MHz and 30MHz channel bandwidth.

PROPOSAL 3: If supported, select 5MHz, 20MHz and 30MHz channel bandwidth.

## Test Subcarrier Spacing

The selection of test subcarrier spacing can be based on the agreed proposals for SEM [2], i.e. Lowest and Highest supported subcarrier spacing.

PROPOSAL 4: Select Lowest and Highest supported subcarrier spacing for NS_18 A-MPR in FR1.

## UL Modulation

The selection of UL modulation can be based on the agreed proposals for SEM [2], i.e. all defined UL waveforms (5 DFT-s OFDM and 4 CP-OFDM waveforms). For 30MHz, only the highest modulation for each A-MPR value is tested.

PROPOSAL 5: Test all UL modulation waveforms supported by the UE for NS_18. For 30MHz, only the highest modulation for each A-MPR value is tested.

## Uplink RB allocation

The UL RB allocation can be based on the agreed proposals for SEM [2], i.e. Edge_1RB_left, Edge_1RB_Right and Outer_Full. These common uplink configurations are listed in [5].

For channel bandwidth other than 30MHz, A-MPR for NS_18 has some region-specific requirements in the lower part of operating band n28, n83 and n109. Edge_1RB_left and Outer_Full can be considered as a worst-case scenario. No need to test with Edge_1RB_Right as it is not close to the lower part of the operating band.

For channel bandwidth 30MHz, some Region-specific RB allocations are specified in terms of RBend × 12 × SCS and LCRB × 12 × SCS. Table 3 shows the corresponding RB ranges for 15/30 SCS and proposed allocation expressed as LCRB@RBstart. Edge_1RB_left, Edge_1RB_Right and Outer_Full are selected to align with SEM. One additional RB allocation for A3 is based on allocating as many RBs as possible located to the lower part of the channel bandwidth, i.e. select lowest RBstart.

Table 3-1: UL RB allocation for NS_18 with 30MHz bandwidth PC3

Table 3-2: UL RB allocation for NS_18 with 40MHz bandwidth for PC3

| Channel Bandwidth, | Carrier Center Frequency, Fc, MHz | SCS | Regions |  |  | Prop. Alloc (DFT-s) |  | Prop. Alloc (CP) |  | A-MPR |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MHz |  |  | RBstart | LCRB* |  |  |  |  |  |  |  |
| 40 MHz | 703~743.04 | 15 | >LCRB/2+47 | ≥Max (0, 206-Rbstart) | 108@108 |  |  | 114@102 |  | A3 |  |
|  |  | 30 | >LCRB/2+23.5 | ≥Max (0, 101-Rbstart) | 50@50 |  |  | 55@51 |  | A3 |  |
|  |  | 15 | ≤LCRB/2+47 | ≥30 | Outer_Full |  |  | Outer_Full |  | A4 |  |
|  |  | 30 | ≤LCRB/2+23.5 | ≥15 | Outer_Full |  |  | Outer_Full |  | A4 |  |
|  |  | 15 | ≤62 | <30 | Edge_1RB_Left |  |  | Edge_1RB_Left |  | A5 |  |
|  |  | 30 | ≤31 | <15 | Edge_1RB_Left |  |  | Edge_1RB_Left |  | A5 |  |

Table 3-3: UL RB allocation for NS_18 for PC2

| Channel Bandwidth, | Carrier Center Frequency, Fc, MHz | SCS |  |  | Regions |  |  | Prop. Alloc (DFT-s) |  |  | Prop. Alloc (CP) |  | A-MPR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MHz |  |  |  | RBstart |  | LCRB* |  |  |  |  |  |  |  |
| 25 MHz | 703~748 |  | 15 | >LCRB/2+20 |  | ≥Max (0, 123-Rbstart) |  |  | 72@54 |  |  | 77@56 | A3 |
|  |  |  | 30 | >LCRB/2+10 |  | ≥Max (0, 60-Rbstart) |  |  | 36@27 |  |  | 38@27 | A3 |
|  |  |  | 15 | ≤LCRB/2+20 |  | ≥30 |  |  | Outer_Full |  |  | Outer_Full | A4 |
|  |  |  | 30 | ≤LCRB/2+10 |  | ≥15 |  |  | Outer_Full |  |  | Outer_Full | A4 |
|  |  |  | 15 | ≤35 |  | <30 |  |  | Edge_1RB_Left |  |  | Edge_1RB_Left | A5 |
|  |  |  | 30 | ≤17.5 |  | <15 |  |  | Edge_1RB_Left |  |  | Edge_1RB_Left | A5 |
|  |  |  | 15 | >LCRB/2+20 ≤ LCRB/2+32 |  |  | <Max (0, 123-Rbstart) ≥30 |  | Outer_Full |  |  | Outer_Full | A6 |
|  |  | 30 | >LCRB/2+10 ≤ LCRB/2+16 | <Max (0, 60-Rbstart) ≥15 | Outer_Full | Outer_Full | A6 |
| 30 MHz | 703~748 |  | 15 | ≥LCRB/2+29 |  | ≥Max (0, 150- RBstart) |  |  | 81@70 |  |  | 87@73 | A3 |
|  |  |  | 30 | ≥LCRB/2+14.5 |  | ≥Max (0, 73- RBstart) |  |  | 40@35 |  |  | 42@36 | A3 |
|  |  |  | 15 | <LCRB/2+29 |  | ≥30 |  |  | Outer_Full |  |  | Outer_Full | A4 |
|  |  |  | 30 | <LCRB/2+14.5 |  | ≥15 |  |  | Outer_Full |  |  | Outer_Full | A4 |
|  |  |  | 15 | <44 |  | <30 |  |  | Edge_1RB_Left |  |  | Edge_1RB_Left | A5 |
|  |  |  | 30 | <22 |  | <15 |  |  | Edge_1RB_Left |  |  | Edge_1RB_Left | A5 |
|  |  |  | 15 | ≥LCRB/2+29 ≤ LCRB/2+41 |  | <Max (0, 150- RBstart) ≥30 |  |  | Outer_Full |  |  | Outer_Full | A6 |
|  |  |  | 30 | ≥LCRB/2+14.5 ≤ LCRB/2+20.5 |  | <Max (0, 73- RBstart) ≥15 |  |  | Outer_Full |  |  | Outer_Full | A6 |
| 40 MHz | 703~743.04 |  | 15 | >LCRB/2+47 |  | ≥Max (0, 206- RBstart) |  |  | 108@108 |  |  | 114@102 | A3 |
|  |  |  | 30 | >LCRB/2+23.5 |  | ≥Max (0, 101- RBstart) |  |  | 50@50 |  |  | 55@51 | A3 |
|  |  |  | 15 | ≤LCRB/2+47 |  | ≥30 |  |  | Outer_Full |  |  | Outer_Full | A4 |
|  |  |  | 30 | ≤LCRB/2+23.5 |  | ≥15 |  |  | Outer_Full |  |  | Outer_Full | A4 |
|  |  |  | 15 | ≤62 |  | <30 |  |  | Edge_1RB_Left |  |  | Edge_1RB_Left | A5 |
|  |  |  | 30 | ≤31 |  | <15 |  |  | Edge_1RB_Left |  |  | Edge_1RB_Left | A5 |
|  |  |  | 15 | ≥LCRB/2+47 ≤ LCRB/2+65 |  | <Max (0, 206- RBstart) ≥30 |  |  | Outer_Full |  |  | Outer_Full | A6 |
|  |  |  | 30 | ≥LCRB/2+23.5 ≤ LCRB/2+32.5 |  | <Max (0, 101- RBstart) ≥15 |  |  | Outer_Full |  |  | Outer_Full | A6 |

PROPOSAL 6: Select Outer_Full and Edge_1RB_Left for testing of channel bandwidth other than 30, 40MHz; Select the RBs as defined in Table 3 for 30MHz Table 3-1 for 40Mhz.

1.7.	Number of Test Points

The maximum number of test points for A-MPR tests for NS_18 (PC3):

| Environment conditions | Maximum Number of Steps (Freq. X Ch BW X RB alloc. X SCS) | Maximum Number of Modulations | Maximum Number of Test Steps |
| --- | --- | --- | --- |
| 1 (NC) | 5MHz and 20MHz: 1 X 2 X 2 X 2 30MHz: 1 x 1 x 1 x 2 40MHz: 1 x 1 x 1 x 2 | 5MHz and 20MHz: 8 30MHz: 6 40MHz: 6 | All: 88 5MHz and 20MHz: 64 30MHz: 12 40Mhz: 12 |

The maximum number of test points for A-MPR tests for NS_18 PC2 is:

| Environment conditions | Maximum Number of Steps (Freq. X Ch BW X RB alloc. X SCS) | Maximum Number of Modulations | Maximum Number of Test Steps |
| --- | --- | --- | --- |
| 1 (NC) | 5MHz and 20MHz: 1 X 2 X 2 X 2 25,30 and 40MHz: 1 x 3 x 1 x 2 | 5MHz and 20MHz: 8 30MHz: 6 | All: 100 5MHz and 20MHz: 64 25,30 and 40MHz: 36 |

# Proposal

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR1 NS_18 A-MPR test case definition:

PROPOSAL 1: Select Test Environment as Normal Conditions (NC) for A-MPR.

PROPOSAL 2: Select Low Range as test frequency for NS_18 A-MPR tests.

PROPOSAL 3: If supported, select 5MHz, 20MHz and 30MHz channel bandwidth.

PROPOSAL 4: Select Lowest and Highest supported subcarrier spacing for NS_18 A-MPR in FR1.

PROPOSAL 5: Test all UL modulation waveforms supported by the UE for NS_18. For 30MHz, only the highest modulation for each A-MPR value is tested.

PROPOSAL 6: Select Outer_Full and Edge_1RB_Left for testing of channel bandwidth other than30,40MHz; Select the max RBs that can be allocated for 30MHz.

# Handling of Additional spurious emission

Given the test point analysis of A-MPR for NS_18 has already taken Additional spurious emission requirement into consideration, the test point of A-SE could reuse that of A-MPR as much as possible. The only difference could be SCS. As per the TP analysis of general spurious emission, higher spectrum utilization would be a more demanding case which can be provided by Lowest SCS.

PROPOSAL 7: A-SE for NS_18 reuses the test points selected for A-MPR for NS_18, except that Lowest SCS is selected.

The maximum number of test points for A-SE tests for NS_18:

| Environment conditions | Maximum Number of Steps (Freq. X Ch BW X RB alloc. X SCS) | Maximum Number of Modulations | Maximum Number of Test Steps |
| --- | --- | --- | --- |
| 1 (NC) | 5MHz and 20MHz: 1 X 2 X 2 X 1 30MHz: 1 x 1 x 1 x 1 | 5MHz and 20MHz: 8 30MHz: 6 | All: 38 5MHz and 20MHz: 32 30MHz: 6 |

# References

- R5-180065, "Discussion about Test parameters for NR TRx TCs " (NTT DOCOMO).

- R5-180886, " Discussion on test point selection for NR SEM in FR1” (Keysight Technologies).

- R5-182019, “Discussion of NR FR1 Test Point for TX Spurious Emission test cases”, (Qualcomm Inc.)

- 3GPP TS 38.101-1, V18.7.0 (2024-09), Table 6.2.3.6-1

- 3GPP TS 38.521-1, V18.4.0 (2024-09), Table 6.1-1