3GPP TSG-RAN5 Meeting #108	R5-254554

# Bengaluru, India, Aug.25th – 29rd 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.903 | CR | 1074 | rev | - | Current version: | 18.7.3 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TT analysis for ATG L1-RSRP accuracy RRM test cases 19.6.4.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Current TT analysis for L1-RSRP relative accuracy TCs (“38.533 4.7.4.1.2+4.7.4.2.2+6.7.4.1.2+6.7.4.2.2 TT v4.zip”) seems are incorrect. The .docx for L3-RSRP relative accuracy TCs (i.e. 4.7.1.1.2/6.7.4.1.2/…) are wrongly included in the .zip file. - To provide TT analysis for following ATG RRM test cases: - 19.6.4.1.1 NR SA FR1 SSB based L1-RSRP absolute measurement accuracy for ATG - 19.6.4.1.2 NR SA FR1 SSB based L1-RSRP relative measurement accuracy for ATG - 19.6.4.2.1 NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy for ATG - 19.6.4.2.2 NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy for ATG |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Grouping Table is updated. - TT analysis for TCs mentioned above are added to attachments of 38.903 - 38.533 4.7.4.1.2+4.7.4.2.2+6.7.4.1.2+6.7.4.2.2 TT.docx is updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8, attachments |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 CR 4087 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RRM TT Associated TC CR: R5-254552 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Following .zip files are removed from the attachments of TR 38.903:

“38.533 4.7.4.1.1+4.7.4.2.1+6.7.4.1.1+6.7.4.2.1 TT v3.zip”

“38.533 4.7.4.1.2+4.7.4.2.2+6.7.4.1.2+6.7.4.2.2 TT v4.zip”

Following .zip files are added to the attachments of TR 38.903:

“38.533 4.7.4.1.1+4.7.4.2.1+6.7.4.1.1+6.7.4.2.1 TT v4.zip”

“38.533 4.7.4.1.2+4.7.4.2.2+6.7.4.1.2+6.7.4.2.2 TT v5.zip”

<Start of Change 1>

Table 8-1: Grouping of FR1 test cases defined in Clauses 4, 4A, 6, 8, 16, 18 and 19 of TS 38.533

| Group | Test Case Numbers | .zip file name | Comments |
| --- | --- | --- | --- |
| SCell_Activation_01 | 4.5.3.1 4.5.3.2 4.5.3.3 6.5.3.1 6.5.3.2 6.5.3.3 10.3.3.1 10.3.3.2 10.3.3.3 11.4.3.1 11.4.3.2 11.4.3.3 13.2.2.1 13.2.2.2 13.2.2.3 | “38.533 4.5.3.1+4.5.3.2+6.5.3.1+6.5.3.2 TT v6.zip” | “2 Inter Frequency NR Cells, 3 time periods, Various number of sub-tests, No fading” |
| <Unchanged contents are omitted> |  |  |  |
| Inter_RAT_SS-SINR_LTE_Serving_01 | 8.5.2.3.1 | “38.533 8.5.2.3.1 TT.zip” | 1 NR Cell, 1 LTE serving cell, periodic SS-SINR reporting, No fading |
| L1-RSRP_Abs_Acc_01 | 4.7.4.1.1 6.7.4.1.1 4.7.4.2.1 6.7.4.2.1 16.7.4.2.1 16.7.4.4.1 19.6.4.1.1 19.6.4.2.1 | “38.533 4.7.4.1.1+4.7.4.2.1+6.7.4.1.1+6.7.4.2.1 TT v4.zip” | 1 NR Cell, periodic L1-RSRP reporting, No fading |
| L1-RSRP_Abs_Acc_01_RedCap_1Rx | 16.7.4.1.1 16.7.4.3.1 | “38.533 16.7.4.1.1 TT.zip” | 1 NR Cell, periodic L1-RSRP reporting, No fading |
| Inter-frequency_L1-RSRP_Abs_Acc | 6.7.17.1.1 | “38.533 6.7.17.1.1 TT.zip” | “1 NR serving cell and 1 NR inter frequency cell, periodic L1-RSRP reporting, no fading” |
| L1-RSRP_Rel_Acc_01 | 4.7.4.1.2 6.7.4.1.2 4.7.4.2.2 6.7.4.2.2 16.7.4.4.2 19.6.4.1.2 19.6.4.2.2 | “38.533 4.7.4.1.2+4.7.4.2.2+6.7.4.1.2+6.7.4.2.2 TT v5.zip” | 1 NR Cell with 2 Beams, periodic L1-RSRP reporting, No fading |
| L1-RSRP_Rel_Acc_01_RedCap_1Rx | 16.7.4.1.2 16.7.4.3.2 | “38.533 16.7.4.1.2 TT.zip” | 1 NR Cell with 2 Beams, periodic L1-RSRP reporting, No fading |
| <Unchanged contents are omitted> |  |  |  |
| SSBless_SCell_Activation_01 | 4.5.3.11 | “38.533 4.5.3.11 TT.zip” | “2 Inter Frequency NR Cells, 2 time periods, No fading” |

<End of Change 1>