3GPP TSG-RAN5 Meeting #108	R5-254569

# Bengaluru, India, Aug.25th – 29rd 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | 3100 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE Cat 1bis RRM test case 5.1.19 and 5.1.20 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, LTE_UE_cat_1RX-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | PCell (Cell 1) Es/Iot level in T2/T3 is only -4.74dB based on TT analysis results given in 36.903 attachments (“5.1.19+5.1.20 TT.zip”), which is too low for PDCCH/PDSCH demodulation reception. As a result, the UE may, - fail to decode PDCCH carrying UL grant, then the UE will fail to send the required measurement report message; - fail to decode the Status report PDU sent by TE on SRB#1. As a result, the UE will trigger a RLF when retransmission number of RLC PDU exceeds the maximum number threshold. either one will lead to failing the test. During RAN5#107 online discussion we have we’d received comments that side condition used in TT analysis shall not be changed. After some checking we realized that this issue is a common issue in LTE intra-frequency measurement test cases 36.521-3 TC 8.1.x. And the existed solution is to increase serving cell Es/Iot by an amount to ensure serving cell Es/Iot is larger than the that PDCCH demodulation specified in 36.101. Take TC 8.1.3 as an example, Cell Es/Iot is increased to -1.69dB (> PDCCH demod requirement -1.7dB) in TT analysis (see “8.1.1+8.1.9 TT_v4”): We suggest do the same updating to TT analysis for 5.1.19 and 5.1.20. Furthermore, we also suggest to increase maximum ReTx number threshold of SRB1 to prevent the unwanted RLF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Updated based on the TT analyis: - Cell 2 Es/Noc in T2 and T3 is decreased by an amount to ensure that Cell 1 Es/Iot is high enough for PDCCH demodulation - a3-offset is decreased by an amount to ensure that entry condition of event A3 is satisfied. 2. Maximum ReTx number threshold of SRB1 is changed to t32 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.19, 5.1.20 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 36.903 CR 0472 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated TT CR: R5-254691 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

5.1.19	E-UTRAN FDD – FDD Intra frequency handover for UE Category 1bis

5.1.19.1	Test purpose

To verify the category 1bis UE's ability to perform handover in RRC_CONNECTED state when an intra-frequency handover is commanded by meeting the UE maximum RRC procedure delay and interruption time requirements.

5.1.19.2	Test applicability

This test applies to all types of E-UTRA FDD UE release 13 and forward of UE category 1bis.

5.1.19.3	Minimum conformance requirements

The handover delay Dhandover shall be less than maximum RRC procedure delay + Tinterrupt in RRC_CONNECTED state.

When the UE receives a RRC message implying handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover seconds from the end of the last TTI containing the RRC command when UE is configured with normal or make-before-break handover.

Where:

Dhandover equals the maximum RRC procedure delay to be defined in clause 11.2 in 3GPP TS 36.331 [5] plus the interruption time stated in 3GPP TS 36.133 [4] clause 5.1.2.1.2.1.

The interruption time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH when UE is configured with normal or make-before-break handover, or the time the UE starts transmission of new PUSCH when UE is configured with RACH-less or combination of make-before-break and RACH-less handover, excluding the RRC procedure delay. This requirement applies when UE is not required to perform any synchronisation procedure before transmitting on the new PRACH or on the new PUSCH.

When intra-frequency or inter-frequency handover is commanded, the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TIU + 20 ms

Where:

Tsearch is the time required to search the target cell when the target cell is not already known when the handover command is received by the UE. If the target cell is known, then Tsearch = 0 ms. If the target cell is unknown and signal quality is sufficient for successful cell detection on the first attempt, then Tsearch = 80 ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to 30 ms.

NOTE:	The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

In the interruption requirement a cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown. Relevant cell identification requirements are described in 3GPP TS 36.133 [4] clause 8.1.2.2.1 for intra-frequency handover.

The normative reference for this requirement is 3GPP TS 36.133 [4] clause 5.1.2.1 and A.5.1.19.

5.1.19.4	Test description

5.1.19.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to Annex E Table E-1 and 3GPP TS 36.508 [7] clauses 4.4.2 and 4.3.1.

Channel Bandwidth to be tested: 10 MHz as defined in 3GPP TS 36.508 [7] clause 4.3.1.

1.	Connect the SS (node B emulator) and AWGN noise sources to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A figure A.40 using only main UE Tx/Rx antenna and without fading.

2.	The general test parameter settings are set up according to Table 5.1.19.4.1-1.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	Message contents are as defined in clause 5.1.19.4.3.

5.	There is one E-UTRA FDD carrier and two cells specified in the test. Cell 1 is the cell used for registration with the power level set according to Annex C.0 and C.1 for this test.

Table 5.1.19.4.1-1: General test parameters for E-UTRAN FDD-FDD intra frequency handover test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| E-UTRA RF Channel Number |  |  | 1 | Only one FDD carrier frequency is used. |
| A3-Offset |  | dB | -4 | Modified by Test tolerance |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  |  | OFF |
| CP length |  |  | Normal |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| PRACH configuration |  |  | 4 | As specified in table 5.7.1-2 in 3GPP TS 36.211 |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

5.1.19.4.2	Test procedure

The test consists of one active cell and one neighbour cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE may not have any timing information of Cell 2. Starting T2, Cell 2 becomes detectable and the UE is expected to detect and send a measurement report. T3 is defined as the end of the last TTI containing the RRC message implying handover.

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 7.2A.3. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 5.1.19.5-1. Propagation conditions are set according to Annex B clause B.1.1. T1 starts.

3.	The SS shall transmit an RRCConnectionReconfiguration message.

4.	The UE shall transmit RRCConnectionReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 5.1.19.5-1.

6.	UE shall transmit a MeasurementReport message triggered by Event A3.

7.	SS shall transmit an RRCConnectionReconfiguration message defined in [Table H.3.2-3] implying handover to Cell 2.

8.	The start of T3 is the instant when the last TTI containing the RRC Connection reconfiguration message implying handover is sent to the UE, at that instant the SS shall switch the power setting from T2 to T3 as specified in Table 5.1.19.5-1.

9.	The UE shall transmit RRCConnectionReconfigurationComplete message.

10.	If the UE transmits the uplink PRACH channel to Cell 2 less than 50 ms from the beginning of time period T3 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

11.	After T3 expires, cause UE handover back to Cell 1 (if the handover fails, switch off the UE) or switch off the UE. Then ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 7.2A.3. Cell 1 is the active cell.

12.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

13.	Repeat step 2-12 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

5.1.19.4.3	Message contents

Same message content as in clause 5.1.1.4.3 with following exception:

-	Table 5.1.1.4.3-2 is replaced by Table 5.1.19.4.3-1.

Table 5.1.19.4.3-1: ReportConfigEUTRA-A3:
Additional E-UTRAN FDD-FDD intra frequency handover test requirement

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-6 ReportConfigEUTRA-A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA-A3 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset | -8 (-4 dB) | -4 is actual value in dB (-8 * 0.5 dB) |  |
| reportOnLeave | FALSE |  |  |
| } |  |  |  |
| } |  |  |  |
| Hysteresis | 0 (0 dB) | 0 is actual value in dB (0 * 0.5 dB) |  |
| timeToTrigger | 0 (0 ms) |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.1.19.4.3-1: SRB-ToAddModList

| Derivation Path: TS 36.508 [7] Table 4.6.3-22A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRB-ToAddModList ::= SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod { | 2 entries |  |  |
| SRB-ToAddMod[1] SEQUENCE { |  | entry 1 SRB1 |  |
| rlc-Config CHOICE { |  |  |  |
| explicitValue SEQUENCE { |  |  |  |
| am SEQUENCE { |  |  |  |
| ul-AM-RLC SEQUENCE { |  |  |  |
| maxRetxThreshold | t32 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

5.1.19.5	Test requirement

Tables 5.1.19.4.1-1 and 5.1.19.5-1 defines the primary level settings including test tolerances for E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis test case.

Table 5.1.19.5-1: Cell specific test parameters for
E-UTRAN FDD-FDD intra frequency handover test case

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 |  | T2 |  |  | T3 |  |
| E-UTRA RF Channel Number |  | 1 |  |  | 1 |  |  |  |  |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |  |  |  |  |
| PDSCH Reference Measurement Channel in clause A.3.1.1.1 |  | R.0 FDD | R.0 FDD | - | - | - |  |  | R.0 FDD |  |  |
| PCFICH/PDCCH/PHICH Reference Channel in clause A.3.1.2.1 |  | R.6 FDD |  |  | R.6 FDD |  |  |  |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 and in A.3.2.1.2 |  | OP.1 FDD | OP.1 FDD | OP.2 FDD | OP.2 FDD | OP.2 FDD |  |  | OP.1 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  |  |  |  |  |  |  |  |  |
|  | dB | 8 | 8 | 8 | - Infinity |  |  | 8.70 |  |  | 8.70 |
|  | dB | 8 | -1.25 | -1.25 | -Infinity |  |  | 0.06 |  |  | 0.06 |
| RSRP Note 3 | dBm/15 KHz | -90 | -90 | -90 | - Infinity |  | -89.30 |  |  | -89.30 |  |
| Io Note 3 | dBm/9MHz | -61.58 | -58.54 | -58.54 | Specified in columns for Cell 1 |  |  |  |  |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |  |  |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  |  |  |  |  |
| Timing offset to Cell 1 Asynchronous cells | ms | - |  |  | 3 |  |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Es/Iot, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |

The handover delay is defined as the time from the beginning of time period T3, to the moment the UE start to transmit the PRACH to Cell 2.

The handover delay Dhandover test requirement in this case is expressed as:

Handover delay Dhandover = maximum RRC procedure delay + Tinterrupt

Tinterrupt = Tsearch + TIU + 20 ms

Tsearch = 0, since Cell 2 is known prior to the test

TIU = 15 ms, since 10 ms due to uncertainty in frame and 5 ms additional delay due to PRACH transmission occasion

Maximum RRC procedure delay = 15 ms as defined in 3GPP TS 36.331 [5] clause 11.2

The handover delay Dhandover shall be less than a total of 50 ms in this test case (note: this gives a total of 15 ms for maximum RRC procedure delay plus 35 ms for Tinterrupt).

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

5.1.20	E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis

5.1.20.1	Test purpose

To verify the category 1bis UE's ability to perform handover in RRC_CONNECTED state when an intra-frequency handover is commanded by meeting the UE maximum RRC procedure delay and interruption time requirements.

5.1.20.2	Test applicability

This test applies to all types of E-UTRA TDD UE release 13 and forward of UE category 1bis.

5.1.20.3	Minimum conformance requirements

The handover delay Dhandover shall be less than maximum RRC procedure delay + Tinterrupt in RRC_CONNECTED state.

When the UE receives a RRC message implying handover, the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover seconds from the end of the last TTI containing the RRC command when UE is configured with normal or make-before-break handover.

Where:

Dhandover equals the maximum RRC procedure delay to be defined in clause 11.2 in 3GPP TS 36.331 [5] plus the interruption time stated in 3GPP TS 36.133 [4] clause 5. 2.2.4.2.1.

The interruption time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH when UE is configured with normal or make-before-break handover, or the time the UE starts transmission of new PUSCH when UE is configured with RACH-less or combination of make-before-break and RACH-less handover, excluding the RRC procedure delay. This requirement applies when UE is not required to perform any synchronisation procedure before transmitting on the new PRACH or on the new PUSCH.

When intra-frequency or inter-frequency handover is commanded, the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TIU + 20 ms

Where:

Tsearch is the time required to search the target cell when the target cell is not already known when the handover command is received by the UE. If the target cell is known, then Tsearch = 0 ms. If the target cell is unknown and signal quality is sufficient for successful cell detection on the first attempt, then Tsearch = 80 ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to 30 ms.

NOTE:	The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

In the interruption requirement a cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown. Relevant cell identification requirements are described in 3GPP TS 36.133 [4] Clause 8.1.2.2.2 for intra-frequency handover.

The normative reference for this requirement is 3GPP TS 36.133 [4] clause 5.2.2.4 and A.5.1.20.

5.1.20.4	Test description

5.1.20.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to Annex E Table E-1 and 3GPP TS 36.508 [7] clauses 4.4.2 and 4.3.1.

Channel Bandwidth to be tested: 10 MHz as defined in 3GPP TS 36.508 [7] clause 4.3.1.

1.	Connect the SS (node B emulator) and AWGN noise sources to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A figure A.40 using only main UE Tx/Rx antenna and without fading.

2.	The general test parameter settings are set up according to Table 5.1.20.4.1-1.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	Message contents are as defined in clause 5.1.20.4.3.

5.	There is one E-UTRA TDD carrier and two cells specified in the test. Cell 1 is the cell used for registration with the power level set according to Annex C.0 and C.1 for this test.

Table 5.1.20.4.1-1: General test parameters for
E-UTRAN TDD-TDD Intra frequency handover test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| E-UTRA RF Channel Number |  |  | 1 | Only one TDD carrier frequency is used. |
| A3-Offset |  | dB | -4 | Modified by test tolerance |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  |  | OFF |
| CP length |  |  | Normal |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in 3GPP TS 36.211 |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in 3GPP TS 36.211 |
| PRACH configuration index |  |  | 53 | As specified in table 5.7.1-3 in 3GPP TS 36.211 |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

5.1.20.4.2	Test procedure

Same test procedure as in clause 5.1.19.4.2 with the following exceptions:

-	Instead of Table 5.1.19.5-1  use Table 5.1.20.5-1.

5.1.20.4.3	Message contents

Same message content as in clause 5.1.2.4.3 with following exceptions:

-	Table 5.1.2.4.3-2 is replaced by Table 5.1.20.4.3-1.

Table 5.1.20.4.3-1: ReportConfigEUTRA-A3: Additional E-UTRAN
TDD-TDD intra frequency handover test requirement

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6, Table 4.6.6-6 ReportConfigEUTRA-A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA-A3 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset | -8 (-4 dB) | -4 is actual value in dB (-8 * 0.5 dB) |  |
| reportOnLeave | FALSE |  |  |
| } |  |  |  |
| } |  |  |  |
| Hysteresis | 0 (0 dB) | 0 is actual value in dB (0 * 0.5 dB) |  |
| timeToTrigger | 0 (0 ms) |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.1.20.4.3-2: SRB-ToAddModList

| Derivation Path: TS 36.508 [7] Table 4.6.3-22A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRB-ToAddModList ::= SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod { | 2 entries |  |  |
| SRB-ToAddMod[1] SEQUENCE { |  | entry 1 SRB1 |  |
| rlc-Config CHOICE { |  |  |  |
| explicitValue SEQUENCE { |  |  |  |
| am SEQUENCE { |  |  |  |
| ul-AM-RLC SEQUENCE { |  |  |  |
| maxRetxThreshold | t32 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

5.1.20.5	Test requirement

Tables 5.1.20.4.1-1 and 5.1.20.5-1 defines the primary level settings including test tolerances for E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis test case.

Table 5.1.20.5-1: Cell specific test parameters for
E-UTRAN TDD-TDD Intra frequency handover test case

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 |  |  | T2 |  | T3 |  |
| E-UTRA RF Channel Number |  | 1 |  |  | 1 |  |  |  |  |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |  |  |  |  |
| PDSCH Reference Measurement Channel in clause A.3.1.1.2 |  | R.0 TDD | R.0 TDD | - | - | - |  |  | R.0 TDD |  |  |
| PCFICH/PDCCH/PHICH Reference Channel in clause A.3.1.2.2 |  | R.6 TDD |  |  | R.6 TDD |  |  |  |  |  |  |
| OCNG Patterns defined in A.3.2.2.1 and in A.3.2.2.2 |  | OP.1 TDD | OP.1 TDD | OP.2 TDD | OP.2 TDD | OP.2 TDD |  |  | OP.1 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  |  |  |  |  |  |  |  |  |
|  | dB | 8 | 8 | 8 | - Infinity |  | 8.70 |  |  |  | 8.70 |
|  | dB | 8 | -1.25 | -1.25 | -Infinity |  | 0.06 |  |  |  | 0.06 |
| RSRP Note 3 | dBm/15 KHz | -90 | -90 | -90 | - Infinity |  |  | -89.30 |  | -89.30 |  |
| Io Note 3 | dBm/9MHz | -61.58 | -58.54 | -58.54 | Specified in columns for Cell 1 |  |  |  |  |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |  |  |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  |  |  |  |  |
| Timing offset to Cell 1 Asynchronous cells | s | - |  |  | 3 |  |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Es/Iot, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |

The handover delay is defined as the time from the beginning of time period T3, to the moment the UE start to transmit the PRACH to Cell 2.

The handover delay Dhandover test requirement in this case is expressed as:

Handover delay Dhandover = maximum RRC procedure delay + Tinterrupt

Tinterrupt = Tsearch + TIU + 20 ms

Tsearch = 0, since Cell 2 is known prior to the test

TIU = 15 ms, since 10 ms due to uncertainty in frame and 5 ms additional delay due to PRACH transmission occasion

Maximum RRC procedure delay = 15 ms as defined in 3GPP TS 36.331 [5] clause 11.2

The handover delay Dhandover shall be less than a total of 50 ms in this test case (note: this gives a total of 15 ms for maximum RRC procedure delay plus 35 ms for Tinterrupt).

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

<End of Change 1>

<Start of Change 2>

Table F.3.2-1: Derivation of Test Requirements (RRM tests)

| Test | Minimum Requirement in 3GPP TS 36.133 | Test Tolerance (TT) | Test Requirement in 3GPP TS 36.521-3 |
| --- | --- | --- | --- |
| 4.2.1 E-UTRA FDD - FDD cell re-selection intra frequency | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13.00dB Ês2 / Noc: +16.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +16.00dB Ês2 / Noc: +13.00dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB +0.45dB During T3: 0dB +0.45dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13.00dB Ês2 / Noc: +16.45dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +16.45dB Ês2 / Noc: +13.00dB |
| 4.2.2 E-UTRA TDD - TDD cell re-selection intra frequency | Same as 4.2.1 | Same as 4.2.1 | Same as 4.2.1 |
| <Unchanged contents are omitted> |  |  |  |
| 5.1.18 E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB | Same as 5.1.16 | Same as 5.1.16 | Same as 5.1.16 |
| 5.1.19 E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +12.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +12.00dB In Signaling A3-offset: 0dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB -3.30dB During T3: 0dB 0dB -3.30dB In Signaling -4dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +8.70dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00 Ês2 / Noc: +8.70dB In Signaling A3-offset: -4dB |
| 5.1.20 E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis | Same as 5.1.19 | Same as 5.1.19 | Same as 5.1.19 |
| 5.1.28 E-UTRAN HD-FDD inter frequency handover for CE UEs in CEModeA | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -3dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -3dB Noc2: -98dBm/15kHz Ês2 / Noc2: +4dB During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: -3dB Noc2: -98dBm/15kHz Ês2 / Noc2: +4dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -3dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -3dB Noc2: -98dBm/15kHz Ês2 / Noc2: +4dB During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: -3dB Noc2: -98dBm/15kHz Ês2 / Noc2: +4dB |
| <Unchanged contents are omitted> |  |  |  |
| 13.6.2.6 HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under enhanced coverage | SNRs as specified | Same as 13.6.2.5 | Same as 13.6.2.5 |

<End of Change 2>