3GPP TSG-RAN5 Meeting #108	R5-254595

# Bengaluru, India, 25th August 2025 – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3582 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of Tables related to SL DRX configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are still FFSs in Tables related to SL DRX configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating Tables Table 4.6.6-9A, 4.6.6-9B, 4.6.6-9C and 4.6.6-9D. Adding Table 4.6.6-9BA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changed Section>

#### –	SL-DRX-Config

Table 4.6.6-9A: SL-DRX-Config

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-DRX-Config-r17 ::= SEQUENCE { |  |  |  |  |
| sl-DRX-ConfigGC-BC-r17 |  | Not Present |  |  |
| sl-DRX-ConfigGC-BC-r17 |  | SL-DRX-ConfigGC-BC with condition SL_DRX_GC_BC |  | SL_DRX_GC_BC |
| sl-DRX-ConfigUC-ToReleaseList-r17 |  | Not Present |  |  |
| sl-DRX-ConfigUC-ToReleaseList-r17 SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-DestinationIndex-r16 { |  | 1 entry |  | SL_DRX_UC_Release |
| SL-DestinationIndex-r16[1] |  | SL-DestinationIdentity | entry 1 |  |
| } |  |  |  |  |
| sl-DRX-ConfigUC-ToAddModList-r17 |  | Not Present |  |  |
| sl-DRX-ConfigUC-ToAddModList-r17 SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-DRX-ConfigUC-Info-r17 { |  | 1 entry |  | SL_DRX_UC_AddMod |
| SL-DRX-ConfigUC-Info-r17[1] SEQUENCE { |  |  | entry 1 |  |
| sl-DestinationIndex-r17 |  | SL-DestinationIdentity |  |  |
| sl-DRX-ConfigUC-r17 |  | sl-DRX-ConfigUC |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SL_DRX_GC_BC | To configure one or multiple sidelink DRX configurations for groupcast and broadcast communication. |
| SL_DRX_UC_Release | To configure the sidelink DRX configurations for corresponding unicast destinations to remove. |
| SL_DRX_UC_AddMod | To configure the sidelink DRX configurations for corresponding unicast destinations to add and/or modify. |

#### –	SL-DRX-ConfigGC-BC

Table 4.6.6-9B: SL-DRX-ConfigGC-BC

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-DRX-ConfigGC-BC-r17 ::= SEQUENCE { |  |  |  |  |
| sl-DRX-GC-BC-PerQoS-List-r17 |  | Not Present |  |  |
| sl-DRX-GC-BC-PerQoS-List-r17 SEQUENCE (SIZE (1..maxSL-GC-BC-DRX-QoS-r17)) OF SL-DRX-GC-BC-QoS-r17 { |  | 1 entry |  | SL_DRX_GC_BC |
| SL-DRX-GC-BC-QoS-r17[1] |  | SL-DRX-GC-BC-QoS | entry 1 |  |
| } |  |  |  |  |
| sl-DRX-GC-generic-r17 |  | Not Present |  |  |
|  |  | FFS |  | SL_DRX_GC |
| sl-DefaultDRX-GC-BC-r17 |  | Not Present |  |  |
|  |  | SL-DRX-GC-BC-QoS |  | SL_DRX_Default |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SL_DRX_GC_BC | To configure one or multiple sidelink DRX configurations for groupcast and broadcast communication. |
| SL_DRX_GC | To configure a sidelink DRX configuration for groupcast communication. |
| SL_DRX_Default | To configure the default sidelink DRX configuration for groupcast and broadcast communications or for the broadcast based or unicast based communication of Direct Link Establishment Request. |

#### –	SL-DRX-GC-BC-QoS

Table 4.6.6-9BA: SL-DRX-GC-BC-QoS

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-DRX-GC-BC-QoS-r17 ::= SEQUENCE { |  |  |  |  |
| sl-DRX-GC-BC-MappedQoS-FlowList-r17 SEQUENCE (SIZE (1..maxNrofSL-QFIs-r16)) OF SL-QoS-Profile-r16 { |  | 1 entry |  |  |
| SL-QoS-Profile-r16[1] |  | SL-QoS-Profile | entry 1 |  |
| } |  |  |  |  |
| sl-DRX-GC-BC-OnDurationTimer-r17 CHOICE { |  |  |  |  |
| milliSeconds |  | ms6 |  |  |
| } |  |  |  |  |
| sl-DRX-GC-InactivityTimer-r17 |  | ms1280 |  |  |
| sl-DRX-GC-BC-Cycle-r17 |  | ms10240 |  |  |
| } |  |  |  |  |

#### –	SL-DRX-ConfigUC

Table 4.6.6-9C: SL-DRX-ConfigUC

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-DRX-ConfigUC-r17 ::= SEQUENCE { |  |  |  |  |
| sl-drx-onDurationTimer-r17 CHOICE { |  |  |  |  |
| milliSeconds |  | ms6 |  |  |
| } |  |  |  |  |
| sl-drx-InactivityTimer-r17 |  | ms1280 |  |  |
| sl-drx-HARQ-RTT-Timer1-r17 |  | Not Present |  |  |
| sl-drx-HARQ-RTT-Timer2-r17 |  | Not Present |  |  |
| sl-drx-RetransmissionTimer-r17 |  |  |  |  |
| sl-drx-CycleStartOffset-r17 CHOICE { |  |  |  |  |
| ms10240 |  | 0 |  |  |
| } |  |  |  |  |
| sl-drx-SlotOffset |  | 0 |  |  |
| } |  |  |  |  |

#### –	SL-DRX-ConfigUC-SemiStatic

Table 4.6.6-9D: SL-DRX-ConfigUC-SemiStatic

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-DRX-ConfigUC-SemiStatic-r17 ::= SEQUENCE { |  |  |  |  |
| sl-drx-onDurationTimer-r17 CHOICE { |  |  |  |  |
| milliSeconds |  | ms6 |  |  |
| } |  |  |  |  |
| sl-drx-CycleStartOffset-r17 CHOICE { |  |  |  |  |
| ms512 |  | 0 |  |  |
| } |  |  |  |  |
| sl-drx-SlotOffset-r17 |  | 0 |  |  |
| } |  |  |  |  |

<End of Changed Section>