3GPP TSG-RAN5 Meeting #108	R5-254600

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3583 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Table 4.6.6-2: SL-BWP-ConfigCommon |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | sl-BWP-DiscPoolConfigCommon-r17 IE for NR sidelink discovery dedicated resource pool configurations was missing in Table 4.6.6-2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding IEs in Table 4.6.6-2 as per TS 38.331 clause 6.3.5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Message content will not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changed Section>

–	SL-BWP-ConfigCommon

Table 4.6.6-2: SL-BWP-ConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-BWP-ConfigCommon-r16 ::= SEQUENCE { |  |  |  |  |
| sl-BWP-Generic-r16 SEQUENCE { |  |  |  |  |
| sl-BWP-r16 |  | BWP |  |  |
| sl-LengthSymbols-r16 |  | sym14 | All symbols in slot are used for SL |  |
|  |  | sym12 | All symbols in slot are used for SL | EXTENDED |
| sl-StartSymbol-r16 |  | sym0 |  |  |
| sl-PSBCH-Config-r16 |  | Not present |  |  |
| sl-TxDirectCurrentLocation-r16 |  | Not present |  |  |
| } |  |  |  |  |
| sl-BWP-PoolConfigCommon-r16 |  | SL-BWP-PoolConfigCommon with RXPOOL and SELECTED |  |  |
| sl-BWP-PoolConfigCommonPS-r17 |  | Not present |  |  |
| sl-BWP-DiscPoolConfigCommon-r17 |  | Not present |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| EXTENDED | When cyclicPrefix is configured in sl-BWP-r16, i.e. extended CP is used |

<End of Changed Section>