3GPP TSG-RAN5 Meeting #108	R5-254612

Bengaluru, India, Aug 25th – 29th 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3474 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case AMPR, ASEM and ASE for 4Tx UL MIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_FR1_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Additional maximum output power reduction for UL MIMO for UE supporting 4Tx test case is absent in 6.2D.3_2 and need to be added. - Additional spectrum emission mask for UL MIMO for UE supporting 4Tx test case is absent in 6.5D.2.3_1 and need to be added. - Additional spurious emissions for UL MIMO for UE supporting 4Tx test case is absent in 6.5D.3_3.3 and need to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Additional maximum output power reduction for UL MIMO for UE supporting 4Tx test case is added. - Additional spectrum emission mask for UL MIMO for UE supporting 4Tx test case is added. - Additional spurious emissions for UL MIMO for UE supporting 4Tx test case is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Additional maximum output power reduction, additional spectrum emission mask and additional spurious emissions for UL MIMO for UE supporting 4Tx test case can not be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2D.3_2(new), 6.5D.2.3_1(new), 6.5D.3.3, 6.5D.3_3.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change1>

6.2D.3_2	UE additional maximum output power reduction for UL MIMO for UE supporting 4Tx

Editor’s Note:

- Tests for network signalling values NS_50 for Power Class 1.5 are FFS.

6.2D.3_2.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated a unique with network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band (the IE freqBandIndicatorNR) and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2D.1.3-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

6.2D.3_2.2	Test applicability

The requirements of this test apply in test case 6.5D.2.3_1 Additional Spectrum Emission mask for UL MIMO for UE supporting 4Tx for network signalling value NS_04 to all types of NR UE release 18 and forward that support 4-layer code book based UL MIMO.

The requirements of this test apply in test case 6.5D.3_3.3 Additional Spurious Emissions for network signalling value NS_04, NS_47 to all types of NR UE release 18 and forward that support 4-layer code book based UL MIMO.

NOTE:	Test execution is not necessary if 6.5D.2.3_1, 6.5D.2.4.2_2 and 6.5D.3_3.3 are executed.

6.2D.3_2.3	Minimum conformance requirements

Same Minimum conformance requirements as 6.2D.3.3.

6.2D.3_2.4	Test description

6.2D.3_2.4.1	Initial conditions

Same initial conditins as 6.2D.3.4.1 with the following exception:

Table 6.2D.3_2.4.1-1: Test Configuration table for NS_04

Same test configuration as listed in Table 6.2D.3.4.1-1 shall be used.

Table 6.2D.3_2.4.1-2: Test Configuration table for NS_47

Same test configuration as listed in Table 6.2D.3.4.1-22 shall be used.

Editor’s note: The following lines belong at the end of subclause 6.2D.3_2.4.1. As new tables are added to this section, these lines should always follow the tables.

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2a for TE diagram and section A.3.2 for UE diagram.

6.2D.3_2.4.2	Test procedure

Same test procedure as specified in 6.2D.3.4.2 with the following exception:

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to applicable table in clause 6.2D.3_2.4.1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with condition 4TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

6.2D.3_2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6.1 ensuring Table 4.6.3-182 with the condition 4TX_UL_MIMO, with the following exceptions for each network signalling value.

6.2D.3_2.4.3.1	Message contents exceptions for network signalling value "NS_04"

Same message contents exceptions as 6.2D.3.4.3.1.

6.2D.3_2.4.3.2	Message contents exceptions for network signalling value "NS_47"

Same message contents exceptions as 6.2D.3.4.3.24.

6.2D.3_2.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from table 6.2D.3_2.5-1 to table 6.2D.3_2.5-2. The allowed A-MPR values specified in table 6.2.3.3.1-1 are in addition to the allowed MPR requirements specified in clause 6.2.2.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2D.1.3-1 apply.

Table 6.2D.3_2.5-0: Test Tolerance (UE additional maximum output power reduction)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |

Table 6.2D.3_2.5-1: UE Power Class 1.5 test requirements (NS_04) for band n41

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR’ (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 9.5 | 9 | 1.5 | 18 | 5 | 3 | 31+TT | 13-TT |
| 2 | 29 | 9.5 | 8 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 3 | 29 | 9.5 | 0 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 4 | 29 | 5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16-TT |
| 5 | 29 | 9.5 | 0 | 1.5 | 18 | 5 | 3 | 31+TT | 13-TT |
| 6 | 29 | 5 | 0 | 0 | 24 | 5 | 3 | 31+TT | 19-TT |
| 7 | 29 | 9.5 | 9 | 1.5 | 18 | 5 | 3 | 31+TT | 13-TT |
| 8 | 29 | 9.5 | 8 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 9 | 29 | 9.5 | 0 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 10 | 29 | 5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16-TT |
| 11 | 29 | 9.5 | 0 | 1.5 | 18 | 5 | 3 | 31+TT | 13-TT |
| 12 | 29 | 5 | 0 | 0 | 24 | 5 | 3 | 31+TT | 19-TT |
| 13 | 29 | 9.5 | 9 | 1.5 | 18 | 5 | 3 | 31+TT | 13-TT |
| 14 | 29 | 9.5 | 8 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 15 | 29 | 9.5 | 0 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 16 | 29 | 7 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16-TT |
| 17 | 29 | 9.5 | 0 | 1.5 | 18 | 5 | 3 | 31+TT | 13-TT |
| 18 | 29 | 7 | 0 | 0 | 22 | 5 | 3 | 31+TT | 17-TT |
| 19 | 29 | 9.5 | 11.5 | 1.5 | 16 | 6 | 3 | 31+TT | 10-TT |
| 20 | 29 | 9.5 | 9 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 21 | 29 | 9.5 | 0 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 22 | 29 | 9.5 | 9 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 23 | 29 | 9.5 | 0 | 1.5 | 18 | 5 | 3 | 31+TT | 13-TT |
| 24 | 29 | 9.5 | 0 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT=0.7 dB for BWchannel ≤ 40 MHz; TT=1.0 dB for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-2: UE Power Class 1.5 test requirements (NS_47) for n41

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 9.5 | 13 | 0 | 16 | 6 | 3 | 31+TT | 10.0-TT |
| 2 | 29 | 3.5 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 3 | 29 | 9.5 | 8 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 4 | 29 | 5 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 5 | 29 | 5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 6 | 29 | 5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 7 | 29 | 5 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 8 | 29 | 5 | 4 | 0 | 24 | 5 | 3 | 31+TT | 19.0-TT |
| 9 | 29 | 9.5 | 13 | 0 | 16 | 6 | 3 | 31+TT | 10.0-TT |
| 10 | 29 | 4 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 11 | 29 | 9.5 | 8 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 12 | 29 | 5 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 13 | 29 | 5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 14 | 29 | 5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 15 | 29 | 5 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 16 | 29 | 5 | 4 | 0 | 24 | 5 | 3 | 31+TT | 19.0-TT |
| 17 | 29 | 9.5 | 13 | 0 | 16 | 6 | 3 | 31+TT | 10.0-TT |
| 18 | 29 | 7 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 19 | 29 | 9.5 | 8 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 20 | 29 | 7 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 21 | 29 | 7 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 22 | 29 | 7 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 23 | 29 | 7 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 24 | 29 | 7 | 0 | 0 | 22 | 5 | 3 | 31+TT | 17.0-TT |
| 25 | 29 | 9.5 | 13 | 0 | 16 | 6 | 3 | 31+TT | 10.0-TT |
| 26 | 29 | 9.5 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 27 | 29 | 9.5 | 8 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 28 | 29 | 9.5 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 29 | 29 | 9.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 30 | 29 | 9.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 31 | 29 | 9.5 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 32 | 29 | 9.5 | 0 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |

Table 6.2D.3.5-3: UE Power Class 1.5 test requirements (NS_47) for n41 (large FWA form factor with 20 dB or above antenna isolation)

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 8 | 13 | 0 | 16 | 6 | 3 | 31+TT | 10.0-TT |
| 2 | 29 | 2 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 3 | 29 | 8 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16.0-TT |
| 4 | 29 | 4.5 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 5 | 29 | 4.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 6 | 29 | 4.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 7 | 29 | 4.5 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 8 | 29 | 4.5 | 4 | 0 | 24.5 | 3 | 3 | 31+TT | 21.5-TT |
| 9 | 29 | 8 | 13 | 0 | 16 | 6 | 3 | 31+TT | 10.0-TT |
| 10 | 29 | 2.5 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 11 | 29 | 8 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16.0-TT |
| 12 | 29 | 4.5 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 13 | 29 | 4.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 14 | 29 | 4.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 15 | 29 | 4.5 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 16 | 29 | 4.5 | 4 | 0 | 24.5 | 3 | 3 | 31+TT | 21.5-TT |
| 17 | 29 | 8 | 13 | 0 | 16 | 6 | 3 | 31+TT | 10.0-TT |
| 18 | 29 | 5 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 19 | 29 | 8 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16.0-TT |
| 20 | 29 | 5 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 21 | 29 | 5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 22 | 29 | 5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 23 | 29 | 5 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 24 | 29 | 5 | 0 | 0 | 24 | 5 | 3 | 31+TT | 19.0-TT |
| 25 | 29 | 8.5 | 13 | 0 | 16 | 6 | 3 | 31+TT | 10.0-TT |
| 26 | 29 | 8.5 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 27 | 29 | 8.5 | 8 | 0 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |
| 28 | 29 | 8.5 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 29 | 29 | 8.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 30 | 29 | 8.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 31 | 29 | 8.5 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 32 | 29 | 8.5 | 0 | 0 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |

< End of Change1 >

### <Start of Change2>

Table 6.5D.2.3.5-5: Additional requirements for "NS_06" or "NS_07"

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- |
|  | 3 | 5 | 10 | 15 |  |
| ± 0 – 0.1 | -13 | -15 | -18 | -20 | 30 kHz |
| ± 0.1 – 1 | -13 | -13 | -13 | -13 | 100 kHz |
| ± 1 – 5 | -13 | -13 | -13 | -13 | 1 MHz |
| ± 5 – 6 | -25 | -13 | -13 | -13 | 1 MHz |
| ± 6 – 10 |  | -25 |  |  |  |
| ± 10 – 15 |  |  | -25 |  |  |
| ± 15 – 20 |  |  |  | -25 |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.5D.2.3_1	Additional spectrum emission mask for UL MIMO for UE supporting 4Tx

Editor’s note:

-Test coverage for the NS_XXs other than NS_04 is FFS

- MU and TT are FFS.

6.5D.2.3_1.1	Test purpose

To verify that the sum of power of any UE emission from all transmit antennas shall not exceed specified lever for the specified channel bandwidth under the deployment scenarios where additional requirements are specified.

6.5D.2.3_1.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE release 18 and forward that supports 4-layer codebook based UL MIMO.

6.5D.2.3_1.3	Minimum conformance requirements

The sum of power of any UE emission shall not exceed the levels specified in Table 6.5.2.3.3.2-1 for NS_04, for the specified channel bandwidth from all transmit antenna connectors.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5D.2 and 6.5.2.3.

6.5D.2.3_1.4	Test description

6.5D.2.3_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, and are shown in clause 6.2D.3_1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2a for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to applicable test configuration table in clause 6.2D.3_2.4.1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5D.2.3_1.4.3.

6.5D.2.3_1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5D.2.2.4.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with condition 4TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2

2.	Send continuously power control “up” commands to the UE until the UE transmits at PUMAX level. Allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in clause 6.2D.3_2.5 as appropriate. The period of the measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots and uplink symbols. For TDD, only slots consisting of only UL symbols are under test.

4.	Measure the sum of power of the transmitted signal from all antenna connectors with a measurement filter of bandwidths according to clause 6.5D.2.3_1.5 and using a rms detector. If the sweep count is higher than one, the trace mode shall be average. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.5D.2.3_1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with condition 4TX_UL_MIMO and exceptions listed in clause 6.2D.3_2.4.3.

6.5D.2.3_1.5	Test requirement

The measured sum of the UE mean power in the channel bandwidth, derived in step 3 shall fulfill the requirements as specified in clause 6.2D.3_2.5 for NS_04 as appropriate, and the sum of power of any UE emission measured from all antennas in step 4 shall fulfill requirements in current subclause.

Table 6.5D.2.3_1.5-1: Test Tolerance (Spectrum Emission Mask)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 100MHz | [1.5 dB] | [1.8 dB] | [1.8 dB] |

Table 6.5D.2.3_1.5-2: Additional requirements for "NS_04"

|  | Spectrum emission limit (dBm) / measurement bandwidth for each channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB MHz | 10 | 15 | 20 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Measurement bandwidth |
| ± 0 - 1 | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT |  |  |  |  |  |  | 2 % channel bandwidth |
|  |  |  |  |  |  | -10 + TT |  |  |  |  |  | 1 MHz |
| ± 1 - 5 | -10 + TT |  |  |  |  |  |  |  |  |  |  | 1 MHz |
| ± 5 - X | -13 + TT |  |  |  |  |  |  |  |  |  |  |  |
| ± X - (BWChannel + 5 MHz) | -25 + TT |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: X is defined in Table 6.5D.2.3_1.5-2a for CP-OFDM NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.5D.2.3_1.5-1. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.5D.2.3_1.5-2a: n41 maximum transmission bandwidths (MHz) for CP-OFDM

| SCS (kHz) | Channel bandwidths (MHz) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 15 | 20 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| 15 | 9.36 | 14.22 | 19.08 | 28.80 | 38.88 | 48.6 | N/A | N/A | N/A | N/A | N/A |
| 30 | 8.64 | 13.68 | 18.36 | 28.08 | 38.16 | 47.88 | 58.32 | 68.04 | 78.12 | 88.02 | 98.28 |
| 60 | 7.92 | 12.96 | 17.28 | 27.36 | 36.72 | 46.8 | 56.88 | 66.96 | 77.04 | 87.12 | 97.20 |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

### <End of Change2>

<Start of Change3>

6.5D.3.3	Additional spurious emissions for UL MIMO

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- There are NS-XXs other than NS_04, NS_21 and NS_47 that are FFS:

6.5D.3.3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions under the deployment scenarios where additional requirements are specified.

6.5D.3.3.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 that support 2-layer codebook based UL MIMO.

6.5D.3.3.3	Minimum conformance requirements

For UEs with two or four transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements specified in subclause 6.5.3 apply. The requirements shall be met with the UL MIMO configurations described in clause 6.2D.1.3. The additional spurious emission requirements specified in 6.5.3.3.3 apply to the sum of the emissions from all UE antenna connectors.

The normative reference for this requirement is TS 38.101-1 [2] clauses 6.5D.3 and 6.5.3.3

6.5D.3.3.4	Test description

6.5D.3.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, and are shown in Table 6.5D.3.3.4.1-1 through Table 6.5D.3.3.4.1-4. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

< End of Change3 >

<Start of Change4>

6.5D.3_3.3	Additional spurious emissions for UL MIMO for UE supporting 4Tx

Editor’s note:

This clause is incomplete. The following aspects are either missing or not yet determined:

- There are NS-XXs other than NS_04 and NS_47 that are FFS.

- MU and TT are FFS.

6.5D.3_3.3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions under the deployment scenarios where additional requirements are specified.

6.5D.3_3.3.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE release 18 that support 4-layer codebook based UL MIMO.

6.5D.3_3.3.3	Minimum conformance requirements

Same minimum conformance requirements as 6.5D.3.3.3.

6.5D.3_3.3.4	Test description

6.5D.3_3.3.4.1	Initial conditions

Same initial conditins as 6.5D.3.3.4.1 with the following exception:

Table 6.5D.3_3.3.4.1-1: Test Configuration Table (network signalling value "NS_04")

Same test configuration as listed in Table 6.5D.3.3.4.1-1 shall be used.

Table 6.5D.3_3.3.4.1-2: Test Configuration Table (network signalling value "NS_47")

Same test configuration as listed in Table 6.5D.3.3.4.1-4a shall be used.

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.2.2a for TE diagram and section A.3.2 for UE diagram.

6.5D.3_3.3.4.2	Test procedure

Same test procedure as specified in 6.5D.3.3.4.2 with the following exception:

1.SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to applicable table in clause 6.2D.3.4.1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with condition 4TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

6.5D.3_3.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with condition 4TX_UL_MIMO and same exceptions listed in clause 6.5.3.3.4.3.

6.5D.3_3.3.5	Test requirement

The measured power derived in step 4 shall meet the requirements for the specified NR band for an additional spurious emission requirement with protected bands as indicated in clause 6.5.3.3.5 for different NS values.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

< End of Change4 >