3GPP TSG-RAN5 Meeting #108	R5-254637

# Bengaluru, India,  August 25 2025 –  August 29 2025

Agenda item:	5.5.1.13

Source: 	Ericsson, Verizon

Title: 	On out of band blocking

Document for:	Discussion and Endorsement

## 1.	Introduction

During RAN5#107 there was a discussion and proposal to limit the number of frequencies for the interfering signal for out of band (OOB) blocking test case 7.6.3. AP#107.21 was created to evaluate the possibility to limit the test time for OOB blocking test case. This paper analyses the core requirement for single carrier test case 7.6.3 and possible impact on network performance.

## 2.	Analysis of the defined requirement in 38.101-1

As per 38.101-1, snippet from 7.6:

Observation 1: Clause 7.6 in 38.101-1 states that blocking performance shall apply at all frequencies, implying measure at each interferer frequency increment.

As per 38.101-1, snippet from 7.6.3:

As per 38.101-1 clause 7.6.3, “measured using a step size” in the NR out-of-band blocking test means that measurements shall be taken at every interferer frequency increment across the defined sweep range.

This sweep must be contiguous — no frequency points may be skipped — to ensure that any potential worst-case blocker condition is detected.

The constant “24” in the formula is a fixed parameter from longstanding 3GPP agreements.

The maximum number of exceptions calculated in the snippet assumes that the full OOB blocking range specified in the standard (up to 12.75 GHz for FR1) is covered in contiguous steps.

Observation 2: The step size of the interferer is based on the channel bandwidth and implies measure at each interferer frequency increment, i.e. clause 7.6.3 in 38.101-1 doesn’t permit partial sweeps.

Observation 3:  The number of allowed exceptions are based on that the whole frequency range up to 12.75 GHz in steps as shown in the snippet above. An approach with fewer interferer steps will not be aligned with the formula in 38.101-1 for calculation of number of exceptions.

## 2.	Analysis of impact on network performance

The blocking characteristic is a measure of the receiver’s ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer. The out of band blocking test case ensures that the NR receiver can maintain performance in a network in respect to re-transmissions, T-put etc. by rejecting strong interfering signals that is not on the same frequency.

The source of the blocker in a real network could be radar systems, Wi-Fi, other cellular bands etc.

To not degrade the performance in the network it’s important that today’s AND future UEs has a good out of band blocking characteristics.

Receiver chain hardware components that are tested in in OOB blocking:

- Band pass filters and the ability to reject out-of-band blocker. Unintended spurious passband  “windows” a.k.a “fly-back” or insufficient roll-off can allow strong blockers to leak into the receiver chain, potentially causing additional issues as listed below.

- AGC/ADC: The AGC may not operate in its optimal range, potentially leading to ADC saturation or distortion of the desired signal.

- LNA, linearity and gain compression when a strong blocker is present.

- Mixer, whether OOB blocker mixes with LO and create spurious response in the band.

Observation 4: If the blocker in the OOB blocking test is limited to Range 1, future band filter designs may be optimized to meet only Range 1 test requirements. As a result, rejection of blockers in Range 2 and/or Range 3 could be inadequate, potentially degrading performance in real network conditions.

Observation 5: Testing out-of-band blocking up to 12.5 GHz, ensures that the UE is robust against real-world high-frequency interferers, validates non-ideal behaviour in RF front-ends, and protects against intermodulation, AGC disruption, and ADC saturation - all critical for robust network performance.

## 4.	Conclusion

Proposal 1: To minimize the risk of negatively impacting network performance, continue to measure at each interferer frequency increment in Range 1, Range 2 and Range 3 in the OOB blocking test case in 3GPP TS 38.521-1 [3].

## 5.		References

[1]	R5-252669

[2]	38.101-1 v.18.9.0

[3]       38.521-1 v19.0.0