3GPP TSG-RAN5 Meeting #108	R5-254699

# Bengaluru, August 25 - 29 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 4110 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Annex F correction for L1-RSRP event-triggered reporting test cases 7.6.22.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | L1-RSRP test case 7.6.22.1 is missing in the Annex F. Event triggered reporting test cases are incomplete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test cases has been added to Annex F. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Applied RRM TT will not be recorded in the Annex F. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

/// Start of changes

Annex F (normative):
Measurement uncertainties and test tolerances

The requirements of this clause apply to all tests in the present document.

# F.1	Measurement uncertainties and test tolerances for FR1 and FR2

/// Unchanged parts omitted

Table F.1.1.2-6: Maximum test system uncertainty for RRM requirements for SA FR2 test cases

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 7.1.1.1 NR SA FR2 cell re-selection | Noc, averaged over BWConfig, ±5.65 dB Noc, averaged over Measurement PRB, ±5.65 dB Es1/Noc averaged over BWConfig ±0.3 dB Es1/Noc averaged over Measurement PRB ±0.3 dB Es2/Noc averaged over BWConfig ±0.3 dB Es2/Noc averaged over Measurement PRB ±0.3 dB | Noc averaged over BWConfig is the AWGN absolute power of RF channel 1 averaged over BWConfig Noc averaged over Measurement PRB is the AWGN absolute power of RF channel 1 averaged over Measurement PRB Es1/Noc averaged over BWConfig is Ratio of cell 1 signal / AWGN averaged over BWConfig Es1/Noc averaged over Measurement PRB is Ratio of cell 1 signal / AWGN averaged over Measurement PRB Es2/Noc averaged over BWConfig is Ratio of cell 2 signal / AWGN averaged over BWConfig Es2/Noc averaged over Measurement PRB is Ratio of cell 2 signal / AWGN averaged over Measurement PRB |
| /// Unchanged rows omitted |  |  |
| 7.6.20.1 NR SA FR2 Intra-frequency SSB based L1-RSRP measurement | Average Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês2 ±5.65 dB, f <= 40.8 GHz | Ês1 is NR Cell 1 signal Ês2 is NR Cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.6.21.1 NR SA FR2 Inter-frequency SSB-based L1-RSRP measurement with measurement gap for LTM | Average Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês2 ±5.65 dB, f <= 40.8 GHz | Ês1 is NR Cell 1 signal Ês2 is NR Cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.6.22.1 NR SA FR2-FR2 SSB based L1-RSRP measurement without measurement gap | Same as in 7.6.21.1 | Same as in 7.6.21.1 |
| 7.7.1.1 NR SA FR2 SS-RSRP measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB Test 2 Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês2 ±5.65 dB, f <= 40.8 GHz | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 1 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Test 2 Ês1 is NR cell 1 signal Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |

/// Unchanged parts omitted

### F.1.3.2	Measurement of RRM requirements

/// Unchanged parts omitted

Table F.1.3.2-4: Derivation of test requirements for NR SA FR2 RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 7.1.1.1 NR SA FR2 cell re-selection | UE PC1 |  |  |
| /// Unchanged rows omitted |  |  |  |
| 7.6.20.1 NR SA FR2 Intra-frequency SSB based L1-RSRP measurement | Signalled A3-Offset: -15dB During T1: Ês/IotBB1: -1.5dB Ês/IotBB2: -1.5dB During T2: Ês/IotBB1: +16.5dB Ês/IotBB2: +16.5dB | 0dB During T1: 1.35dB 1.35dB During T2: 0dB 0dB | Signalled A3-Offset: -15dB During T1: Ês/IotBB1: -0.15dB Ês/IotBB2: -0.15dB During T2: Ês/IotBB1: +16.5dB Ês/IotBB2: +16.5dB |
| 7.6.21.1 NR SA FR2 Inter-frequency SSB-based L1-RSRP measurement with measurement gap for LTM | Signalled A3-Offset: -15dB Signalled Cell 2 offsetMO: 16dB During T1: Ês/IotBB1: 16.5dB Ês/IotBB2: -1.5dB During T2: Ês/IotBB1: +16.5dB Ês/IotBB2: +16.5dB | 0dB 0dB During T1: 0dB 6dB During T2: 0dB 0dB | Signalled A3-Offset: -15dB Signalled Cell 2 offsetMO: 16dB During T1: Ês/IotBB1: 16.5dB Ês/IotBB2: 4.5dB During T2: Ês/IotBB1: +16.5dB Ês/IotBB2: +16.5dB |
| 7.6.22.1 NR SA FR2-FR2 SSB based L1-RSRP measurement without measurement gap | Signalled A3-Offset: -15dB During T1: Ês/IotBB1: -1.5dB Ês/IotBB2: +16.5dB During T2: Ês/IotBB1: -1.5dB Ês/IotBB2: +16.5dB | 0dB During T1: +3.35dB 0dB During T2: +3.35dB 0dB | Signalled A3-Offset: -15dB During T1: Ês/IotBB1: +1.85dB Ês/IotBB2: +16.5dBdB During T2: Ês/IotBB1: +1.85dB Ês/IotBB2: +16.5dB |
| 7.7.1.1 NR SA FR2 SS-RSRP measurement accuracy | UE PC1 |  |  |

/// End of changes