3GPP TSG-RAN5 Meeting #108 	R5-254756

# Bengaluru, India, 25th Aug. 2025 - 29th Aug. 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1994 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of reference sensitivity requirements for Rel-16 FR1 2CC EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The following 2CC EN-DC configurations within FR1 are missing in 7.3B.2 Reference sensitivity. - DC_1A_n71A - DC_28A_n40A |  |  |  |  |  |  |  |  |
| Summary of change: |  | - DC_1A_n71A was added to Tables 7.3B.2.0.3.2-1, 7.3B.2.3.4.2.1-6, and 7.3B.2.3.5-0. - DC_28A_n40A was added to Tables 7.3B.2.0.3.2-1, 7.3B.2.3.4.2.1-6, and 7.3B.2.3.5-0. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The missing EN-DC configurations will not be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.0.3, 7.3B.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

7.3B.2.0.3	Inter-band EN-DC within FR1

Reference sensitivity exceptions are specified for the condition when there is uplink transmission only in the aggressor band.

The reference sensitivity exceptions due to harmonic, harmonic mixing, cross band isolation and power class 2 or power class 3 EN-DC intermodulation interferences are applicable to the UL aggressor band configured with either one Tx antenna connector or two Tx antenna connectors with UL MIMO or Tx diversity operation.

<<Unchanged sections skipped>>

7.3B.2.0.3.2	Reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

Sensitivity degradation is allowed for different combinations of UL configurations and DL channel bandwidths if a band if it is impacted by receiver harmonic mixing due to another band part of the same EN-DC configuration. Reference sensitivity exceptions for the victim band (low) and uplink/downlink configurations due to UL harmonic from a PC3 aggressor UL band (high) for either single band uplink or PC3 or PC2 EN-DC are specified in Table 7.3B.2.0.3.2-1.

Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor UL for PC2 EN-DC are specified in Table 7.3B.2.3.2-1a.

For these exceptions, only the listed test points in Table 7.3B.2.3.2-1 and Table 7.3B.2.3.2-1a need to be tested.

Table 7.3B.2.0.3.2-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| 1 | n71 | 5 | 15 | 25 | 5 | 26.8 | NOTE 4 | UL1/DL3 |
| 1 | n71 | 5 | 15 | 25 | 20 | 15.6 | NOTE 4 | UL1/DL3 |
| 2 | n71 | 5 | 15 | 25 | 5 | 26.8 | NOTE 4 | UL1/DL3 |
| 2 | n71 | 5 | 15 | 25 | 20 | 15.6 | NOTE 4 | UL1/DL3 |
| n2 | 71 | 5 | 15 | 25 | 5 | 26.8 | NOTE 4 | UL1/DL3 |
| n2 | 71 | 5 | 15 | 25 | 20 | 17.1 | NOTE 4 | UL1/DL3 |
| n40 | 28 | 10 | 15 | 25 | 5 | 37.8 | NOTE 4 | UL1/DL3 |
| n40 | 28 | 10 | 15 | 25 | 20 | 30.3 | NOTE 4 | UL1/DL3 |
| n41 | 26 | 10 | 15 | 25 | 5 | 23.8 | NOTE 4 | UL1/DL3 |
| n41 | 26 | 10 | 15 | 25 | 15 | 19 | NOTE 4 | UL1/DL3 |
| n77 | 2 | 10 | 15 | 25 | 5 | 6.7 | NOTE 13 | UL1/DL2 |
| n77 | 2 | 10 | 15 | 25 | 20 | 2.9 | NOTE 13 | UL1/DL2 |
| n77 | 3 | 10 | 15 | 25 | 5 | 5.7 | NOTE 13 | UL1/DL2 |
| n77 | 3 | 10 | 15 | 25 | 20 | 2.3 | NOTE 13 | UL1/DL2 |
| n77 | 12 | 10 | 15 | 25 | 5 | 31 | NOTE 2 | UL1/DL5 |
| n77 | 12 | 10 | 15 | 25 | 10 | 28 | NOTE 2 | UL1/DL5 |
| n77 | 13 | 10 | 15 | 25 | 5 | 31 | NOTE 2 | UL1/DL5 |
| n77 | 13 | 10 | 15 | 25 | 10 | 28 | NOTE 2 | UL1/DL5 |
| n77 | 14 | 10 | 15 | 25 | 5 | 31 | NOTE 2 | UL1/DL5 |
| n77 | 14 | 10 | 15 | 25 | 10 | 28 | NOTE 2 | UL1/DL5 |
| n77 | 28 | 10 | 15 | 25 | 5 | 31 | NOTE 2 | UL1/DL5 |
| n77 | 28 | 10 | 15 | 25 | 20 | 23.5 | NOTE 2 | UL1/DL5 |
| n77 | 41 | 10 | 15 | 12 | 5 | 14.7 | NOTE 8 | UL2/DL3 |
| n77 | 41 | 10 | 15 | 12 | 20 | 9.1 | NOTE 8 | UL2/DL3 |
| n78 | 3 | 10 | 15 | 25 | 5 | 5.7 | NOTE 13 | UL1/DL2 |
| n78 | 3 | 10 | 15 | 25 | 20 | 2.3 | NOTE 13 | UL1/DL2 |
| n78 | 5 | 10 | 15 | 25 | 5 | 5.7 | NOTE 14 | UL1/DL4 |
| n78 | 19 | 10 | 15 | 25 | 5 | 7.3 | NOTE 14 | UL1/DL4 |
| n78 | 19 | 10 | 15 | 25 | 15 | 3.9 | NOTE 14 | UL1/DL4 |
| n78 | 41 | 10 | 15 | 12 | 5 | 14.7 | NOTE 8 | UL2/DL3 |
| n78 | 41 | 10 | 15 | 12 | 20 | 9.1 | NOTE 8 | UL2/DL3 |
| n79 | 11 | 10 | 15 | 25 | 5 | 39.3 | NOTE 4 | UL1/DL3 |
| n79 | 11 | 10 | 15 | 25 | 10 | 36.3 | NOTE 4 | UL1/DL3 |
| n79 | 19 | 10 | 15 | 25 | 5 | 29.5 | NOTE 2 | UL1/DL5 |
| n79 | 19 | 10 | 15 | 25 | 15 | 24.7 | NOTE 2 | UL1/DL5 |
| n79 | 21 | 10 | 15 | 25 | 5 | 39.3 | NOTE 4 | UL1/DL3 |
| n79 | 21 | 10 | 15 | 25 | 15 | 34.5 | NOTE 4 | UL1/DL3 |
| n79 | 26 | 10 | 15 | 25 | 5 | 27 | NOTE 2 | UL1/DL5 |
| n79 | 26 | 10 | 15 | 25 | 15 | 22.2 | NOTE 2 | UL1/DL5 |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band for which the mixing product due to harmonic of victim (lower) band LO with leakage of aggressor (higher) band is within the downlink transmission bandwidth of a victim (lower) band. NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the DL carrier frequency in the lower band and fULHB the UL carrier frequency and BWChannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 3: Void. NOTE 4: The requirements should be verified for DL EARFCN or NR ARFCN of the victim (lower) band (superscript LB) such that and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the DL carrier frequency in the lower band and fULHB the UL carrier frequency and BWChannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 5: Void NOTE 6: Void NOTE 7: Void NOTE 8: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2with the DL carrier frequency in the lower band and fULHB the UL carrier frequency and BWChannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 9: No requirements apply for the case that there is at least one individual RE within the uplink transmission bandwidth of the relative higher band and when the frequency range of relative higher band’s uplink channel bandwidth or uplink 1st adjacent channel bandwidth is fully or partially overlapped with the 3 times of the frequency range of the relative lower band’s downlink channel bandwidth. The reference sensitivity is only verified when this is not the case. NOTE 10: MSD test point can be chosen according to supported BW and lowest SCS supported by the UE. NOTE 11: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. NOTE 12: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band NOTE 13: The requirements should be verified for DL EARFCN or NR ARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.20.1 and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 14: The requirements should be verified for DL EARFCN or NR ARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.40.1 and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 15: The requirements should be verified using RBstart = floor((NRB-LCRB)/2), where floor(x) is the greatest integer less than or equal to x, and where the UL parameters NRB and LCRB are respectively, the transmission bandwidth configuration and the number of RB’s for the specified UL band channel bandwidth and the UL band subcarrier spacing. |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

7.3B.2.3	Reference sensitivity for Inter-band EN-DC within FR1 (2 CCs)

Editor's note:	The minimum conformance requirements for the following test points are missing in 7.3B.2.0.3.

- DC_40A_n78A (REFSENS_ENDC_2) in Table 7.3B.2.3.4.2.1-6

- DC_48A_n66A (REFSENS_ENDC_1) in Table 7.3B.2.3.4.2.1-6

7.3B.2.3.1	Test purpose

Same as in clause 7.3B.2.1.1.

7.3B.2.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting inter-band EN-DC within FR1 with 2 DL CCs.

7.3B.2.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3B.2.0.

LTE anchor agnostic approach is not applied.

7.3B.2.3.4	Test description

7.3B.2.3.4.1	Void

7.3B.2.3.4.2	Test description

7.3B.2.3.4.2.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths based on EN-DC operating bands specified in clause 5.5B, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration, and are shown in Table 7.3B.2.3.4.2.1-0 to Table 7.3B.2.3.4.2.1-6.

The details of the uplink and downlink reference measurement channels (RMCs) are specified in TS 36.521-1 [10] Annex A.2 and A.3 for E-UTRA RMC, and in TS 38.521-1 [8] Annex A.2 and A.3 for NR RMC respectively. The details of the OCNG patterns used are specified in TS 36.521-1 [10] Annex A.5 and in TS 38.521-1 [8] Annex A.5 for E-UTRA CG and NR CG, respectively. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG, respectively.

For configurations without any reference sensitivity exception, in a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. For configurations with reference sensitivity exception, in an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected.

<<Unchanged tables skipped>>

Table 7.3B.2.3.4.2.1-6: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions (two bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |  |
| Test EN-DC channel bandwidth as specified in TS 36.508 [11] clause 4.3.1 and TS 38.508-1 [6] clause 4.3.1. |  |  |  |  |  |  |  |  |  |  | Refer to "NR NRB" and "E-UTRA NRB " columns |  |  |  |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  |  |  |  |  |  | Lowest supported SCS |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  | NS_01 by default Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  |  |  |  |  |  | SCG -NR |  |  |  |  |  |  |  |  |
|  | Band | Range |  |  | NRB |  |  |  |  | Band |  |  | Range |  | NRB |  |  |  |
|  | UL MOD | DL MOD |  |  | CH BW |  |  | DLalloc / UL alloc |  | UL MOD |  |  | DL MOD |  | UL/DL Ch BW | DLalloc / UL alloc |  |  |
| <<Unchanged rows skipped>> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for a DC_1A_n41A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16,19 | 1 | High |  |  |  |  |  |  |  | n41 |  |  | Low |  |  |  |  |  |
|  | QPSK | QPSK |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_3 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 26 | 1 | High |  |  |  |  |  |  |  | n41 |  |  | Low |  |  |  |  |  |
|  | N/A | QPSK |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |  |
| Test Settings for a DC_1A_n71A Configuration (PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 1 | UL 1948.5 |  |  |  |  |  |  |  | n71 |  |  | High |  |  |  |  |  |
|  | QPSK | QPSK |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_2 |  | N/A |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / 0 |  |  |
| 25 | 1 | UL 1926 |  |  |  |  |  |  |  | n71 |  |  | High |  |  |  |  |  |
|  | QPSK | QPSK |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_2 |  | N/A |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / 0 |  |  |
| 39 | 1 | UL Low |  |  |  |  |  |  |  | n71 |  |  | High |  |  |  |  |  |
|  | QPSK | QPSK |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_2 |  | N/A |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / 0 |  |  |
| Test Settings for a DC_1A_n77A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 1 | UL 1950/ DL 2140 |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3900 |  |  |  |  |  |
|  | QPSK | QPSK |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 23 | 1 | UL 1950/ DL 2140 |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3870 |  |  |  |  |  |
|  | QPSK | QPSK |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / 0 |  |  |
| 34 | 1 | UL 1950/ DL 2140 |  |  |  |  |  |  |  | n77 |  |  | UL/DL 4090.005 |  |  |  |  |  |
|  | QPSK | QPSK |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 44 | 1 | UL 1950/ DL 2140 |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3709.995 |  |  |  |  |  |
|  | QPSK | QPSK |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| <<Unchanged rows skipped>> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for a DC_28A_n5A Configuration (PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 28 | High |  |  |  |  |  |  | n5 |  |  |  | Low |  |  |  |  |  |
|  | N/A | QPSK |  | 20MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_3 |
| Test Settings for a DC_28A_n40A Configuration (PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 28 | UL 743.5 / DL 798.5 |  |  |  |  |  |  | n40 |  |  |  | High |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| 25 | 28 | High |  |  |  |  |  |  | n40 |  |  |  | High |  |  |  |  |  |
|  | N/A | QPSK |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| 39 | 28 | Low |  |  |  |  |  |  | n40 |  |  |  | High |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| Test Settings for a DC_28A_n77A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 28 | Low |  |  |  |  |  |  |  | n77 |  | UL/DL 3540 |  |  |  |  |  |  |
|  | QPSK | QPSK | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |  |
| 28 | 28 | Low |  |  |  |  |  |  |  | n77 |  | Low |  |  |  |  |  |  |
|  | QPSK | QPSK | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |  |
| 34 | 28 | UL 705.5 / DL 760.5 |  |  |  |  |  |  |  | n77 |  | UL/DL 3582.495 |  |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |  |
| 45 | 28 | Low |  |  |  |  |  |  |  | n77 |  | UL/DL 3815.01 |  |  |  |  |  |  |
|  | N/A | QPSK | 10 MHz |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_2 |  |
| <<Unchanged rows skipped>> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

4.	NR downlink signals are initially set up according to Annex C.0, C.1, C.2, C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1 of TS 38.521-1 [8].

5.	E-UTRA downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0 of TS 36.521-1 [10].

6.	The DL and UL Reference Measurement channels are set according to Tables 7.3B.2.3.4.2.1-0 to 7.3B.2.3.4.2.1-6 for E-UTRA CG and NR CG.

7.	NR propagation conditions are set according to Annex B.0 of TS 38.521-1 [8]. E-UTRA propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

8.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause7.3B.2.3.4.2.3.

9.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6. Step 9 only applicable to the test configuration in Table 7.3B.2.3.4.2.1-0.

7.3B.2.3.4.2.2	Test procedure

For test points in Table 7.3B.2.3.4.2.1-0:

1.	NR SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.3B.2.3.4.2.1-0 on the NR CC. The NR SS sends downlink MAC padding bits on the DL RMC.

2.	NR SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.3B.2.3.4.2.1-0 Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the appropriate REFSENS value defined in TS 38.521-1 [8] Table 7.3.2.5-1 for NR band. Send continuously uplink power control "up" commands in the uplink scheduling information to both carriers to ensure the UE transmits PUMAX level for at least the duration of the Throughput measurement.

4.	Measure the average throughput of the NR carrier for a duration sufficient to achieve statistical significance according to Annex H.2 of TS 38.521-1 [8] for NR band.

For test points in Table 7.3B.2.3.4.2.1-6:

1.	SS transmits PDSCH via PDCCH DCI format 1A and PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.3B.2.3.4.2.1-6 on the E-UTRA CC and NR CC. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.3B.2.3.4.2.1-6 on the E-UTRA CC and NR CC. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level for the E-UTRA CC and NR CC to the appropriate REFSENS value defined in clause 7.3B.2.3.5. Send continuously uplink power control "up" commands in the uplink scheduling information to both carriers to ensure the UE transmits PUMAX level for at least the duration of the Throughput measurement.

4.	Measure the average throughput of both NR and E-UTRA for a duration sufficient to achieve statistical significance according to Annex H.2 of TS 38.521-1 [8] for NR band, and Annex G.2 of TS 36.521-1 [10] for EUTRA band.

7.3B.2.3.4.2.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED for NR band.

Message contents exceptions for E-UTRA band are according to TS 36.521-1 [10] clause 7.3.4.3 for each network signalling value. Message contents exceptions for NR band are according to TS 38.521-1 [8] clause 7.3.2.4.3 for each network signalling value.

For test points in Table 7.3B.2.3.4.2.1-6, the following message exception applies:

Table 7.3B.2.3.4.2.3-1: RRCConnectionReconfiguration: nr-Config-r15

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 23 |  | Power Class 2 UE |
|  | 20 |  | Power Class 3 UE |

Table 7.3B.2.3.4.2.3-2: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6] Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 2 UE |
|  | 20 |  | Power Class 3 UE |

7.3B.2.3.5	Test requirement

For inter-band EN-DC configurations affected by reference sensitivity exceptions, when test points without NOTE 8, 9 and 11 in Table 7.3B.2.3.4.2.1-6 are tested, the throughput of each CG shall be ≥ 95% of the maximum throughput for the reference receive power level specified in Table 7.3B.2.3.5-0, 7.3B.2.3.5-0a, Table 7.3B.2.3.5-1, Table 7.3B.2.3.5-2, Table 7.3B.2.3.5-3, Table 7.3B.2.3.5-3a, Table 7.3B.2.3.5-4 and Table 7.3B.2.3.5-5 for MSDs due to uplink harmonic, harmonic mixing, cross band isolation for PC3 EN-DC, cross band isolation for PC2 EN-DC, dual uplinks for PC3 EN-DC and dual uplink operation for PC2 EN-DC, respectively. For a given EN-DC combo, if more than one category of MSD applies, UE shall pass all requirement.

For test points with NOTE 8, 9 or 11 in Table 7.3B.2.3.4.2.1-6, reference sensitivity test requirements are specified in Table 7.3.2.5-1 in TS 38.521-1 [8] for the NR CC, and Table 7.3.5-1 in TS 36.521-1 [10] for E-UTRA CC.

Reference sensitivity test requirements for test points in Table 7.3B.2.3.4.2.1-0 are specified in Table 7.3.2.5-1 in TS 38.521-1 [8].

For the UE which supports inter-band EN-DC, the minimum requirement for reference sensitivity in Table 7.3.2.5-1 of TS 38.521-1 [8] for NR band and Table 7.3.5-1 of TS 36.521-1 [10] for EUTRA band, shall be increased by the amount given in ΔRIB,c defined in clause 7.3B.3.3 for the applicable two, three, four and five bands operation.

Table 7.3B.2.3.5-0: Reference sensitivity test requirement for PC3 EN-DC

| EN-DC configuration | Test ID | E-UTRA/NR band | UL/DL CBW (MHz) | REFSENS test requirement (dBm) | Exception type |
| --- | --- | --- | --- | --- | --- |
| DC_1A_n71A | 1 | 1 | 5 | REFSENS | Harmonic mixing |
|  |  | n71 | 5 | REFSENS + 26.8 |  |
|  | 2 | 1 | 5 | REFSENS | Harmonic mixing |
|  |  | n71 | 20 | REFSENS + 15.6 |  |
|  | 3 | 1 | 5 | REFSENS | Harmonic mixing |
|  |  | n71 | 5 | REFSENS | avoided |
| DC_1A_n78A | 1 | 1 | 5 | -91.3 | dual uplink operation |
|  |  | n78 | 10 | REFSENS |  |
| DC_2A_n41A | 1 | 2 | 5 | -96.7 | Cross band isolation |
|  |  | n41 | 100 | REFSENS |  |
| DC_3A_n41A | 1 | 3 | 20 | REFSENS | Cross band isolation |
|  |  | n41 | 10 | -94.1+TT |  |
|  | 2 | 3 | 5 | -95.7 | Cross band isolation |
|  |  | n41 | 100 | REFSENS |  |
|  | 3 | 3 | 5 | -88.1 | dual uplink operation |
|  |  | n41 | 10 | REFSENS |  |
| DC_3A_n77A | 1 | 3 | 5 | REFSENS | UL harmonic |
| DC_3A_n78A |  | n78 | 10 | -71.9+TT | interference |
|  | 2 | 3 | 5 | REFSENS | UL harmonic exception |
|  |  | n78 | 10 | REFSENS | avoided |
|  | 3 | 3 | 10 | REFSENS | UL harmonic |
|  |  | n78 | 100 | -71.7+TT | interference |
|  | 4 | 3 | 10 | REFSENS | UL harmonic exception |
|  |  | n78 | 100 | REFSENS | avoided |
|  | 5 | 3 | 5 | -90.6 | Harmonic mixing |
|  |  | n78 | 10 | REFSENS |  |
|  | 6 | 3 | 20 | -88.0 | Harmonic mixing |
|  |  | n78 | 10 | REFSENS |  |
|  | 7 | 3 | 5 | REFSENS | Harmonic mixing |
|  |  | n78 | 10 | REFSENS | avoided |
|  | 8 | 3 | 20 | REFSENS | Harmonic mixing |
|  |  | n78 | 10 | REFSENS | avoided |
|  | 9 | 3 | 5 | -70.3 | Dual uplink operation |
|  |  | n78 | 10 | REFSENS |  |
|  | 10 | 3 | 5 | -88.8 | Dual uplink operation |
|  |  | n78 | 10 | REFSENS |  |
| DC_5A_n78A | 1 | 5 | 5 | REFSENS | UL harmonic |
|  |  | n78 | 100 | -82.6+TT | Interference |
|  | 2 | 5 | 5 | REFSENS |  |
|  |  | n78 | 10 | -85.3+TT |  |
|  | 3 | 5 | 5 | REFSENS | UL harmonic exception |
|  |  | n78 | 100 | REFSENS | avoided |
|  | 4 | 5 | 5 | -89.0 | Dual uplink operation |
|  |  | n78 | 10 | REFSENS |  |
|  | 5 | 5 | 5 | -91.6 | Harmonic mixing |
|  |  | n78 | 10 | REFSENS |  |
|  | 6 | 5 | 5 | REFSENS | Harmonic mixing |
|  |  | n78 | 10 | REFSENS | avoiding |
| DC_7A_n3A | 1 | 7 | 10 | -81.3 | Dual uplink operation |
|  |  | n3 | 5 | REFSENS |  |
| DC_7A_n78A | 1 | 7 | 5 | -93.5 | Cross band isolation |
|  |  | n78 | 100 | REFSENS |  |
| DC_8A_n77A | 1 | 8 | 5 | REFSENS | UL harmonic |
|  |  | n77 | 10 | -84.5+TT | interference |
|  | 1a | 8 | 5 | REFSENS | UL harmonic |
|  |  | n77 | 100 | -81.94+TT | interference |
|  | 2 | 8 | 5 | REFSENS | UL harmonic exception |
|  |  | n77 | 100 | REFSENS | avoided |
|  | 3 | 8 | 5 | -88.0 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
| DC_8A_n78A | 1 | 8 | 5 | REFSENS | UL harmonic |
|  |  | n78 | 10 | -85+TT | interference |
|  | 1a | 8 | 5 | REFSENS | UL harmonic |
|  |  | n78 | 100 | -82.44+TT | interference |
|  | 2 | 8 | 5 | REFSENS | UL harmonic exception |
|  |  | n78 | 100 | REFSENS | avoided |
|  | 3 | 8 | 5 | -88.0 | Dual uplink operation |
|  |  | n78 | 10 | REFSENS |  |
| DC_20A_n3A | 1 | 20 | 5 | REFSENS | Dual uplink operation |
|  |  | n3 | 5 | -93.0 +TT |  |
|  | 2 | 20 | 5 | -87.3 | Dual uplink operation |
|  |  | n3 | 5 | REFSENS |  |
| DC_20A_n78A | 1 | 20 | 5 | REFSENS | UL harmonic exception |
|  |  | n78 | 100 | REFSENS | avoided |
|  | 2 | 20 | 5 | REFSENS | UL harmonic |
|  |  | n78 | 10 | -85+TT | interference |
|  | 2a | 20 | 5 | REFSENS | UL harmonic |
|  |  | n78 | 100 | -82.44+TT | interference |
|  | 3 | 20 | 5 | - | Dual uplink operation |
|  |  | n78 | 10 | REFSENS |  |
| DC_26A_n41A | 1 | 26 | 5 | REFSENS | UL harmonic |
|  |  | n41 | 100 | -81.8+TT |  |
|  | 2 | 26 | 5 | REFSENS |  |
|  |  | n41 | 10 | -81.1+TT |  |
|  | 3 | 26 | 5 | REFSENS | UL harmonic exception |
|  |  | n41 | 10 | REFSENS | avoided |
|  | 4 | 26 | 5 | -81.2 | dual uplink operation |
|  |  | n41 | 10 | REFSENS |  |
|  | 5 | 26 | 5 | -73 | harmonic mixing |
|  |  | n41 | 10 | REFSENS |  |
|  | 6 | 26 | 15 | -73 |  |
|  |  | n41 | 15 | REFSENS |  |
|  | 7 | 26 | 15 | REFSENS | harmonic mixing |
|  |  | n41 | 15 | REFSENS | avoiding |
| DC_28A_n40A | 1 | 28 | 5 | REFSENS + 37.8 | Harmonic mixing |
|  |  | n40 | 10 | REFSENS |  |
|  | 2 | 28 | 20 | REFSENS + 30.3 | Harmonic mixing |
|  |  | n40 | 10 | REFSENS |  |
|  | 3 | 28 | 5 | REFSENS | Harmonic mixing |
|  |  | n40 | 10 | REFSENS | avoiding |
| DC_39A_n41A | 1 | 39 | 20 | REFSENS | Cross band isolation |
|  |  | n41 | 10 | -91.5+TT |  |
|  | 2 | 39 | 5 | -97.7 | Cross band isolation |
|  |  | n41 | 100 | REFSENS |  |
| DC_40A_n41A | 1 | 40 | 5 | -67.9 | Cross band isolation |
|  |  | n41 | 100 | REFSENS |  |
| DC_41A_n77A | 1 | 41 | 5 | -82.6 | harmonic mixing |
|  |  | n77 | 10 | REFSENS |  |
|  | 1a | 41 | 20 | -82.2 | harmonic mixing |
|  |  | n77 | 10 | REFSENS |  |
|  | 2 | 41 | 5 | -92.8 | Cross band isolation |
|  |  | n77 | 100 | REFSENS |  |
|  | 3 | 41 | 20 | REFSENS | Cross band isolation |
|  |  | n77 | 10 | -87+TT |  |
| DC_41A_n78A | 1 | 41 | 5 | -82.6 | harmonic mixing |
|  |  | n78 | 10 | REFSENS |  |
|  | 1a | 41 | 20 | -82.2 | harmonic mixing |
|  |  | n78 | 10 | REFSENS |  |
|  | 2 | 41 | 5 | -92.8 | Cross band isolation |
|  |  | n78 | 100 | REFSENS |  |
|  | 3 | 41 | 20 | REFSENS | Cross band isolation |
|  |  | n78 | 10 | -87.5+TT |  |
| DC_71A_n2A | 1 | 71 | 5 | REFSENS | UL harmonic |
|  |  | n2 | 5 | -96.3+TT | Interference |
|  | 2 | 71 | 5 | -69.7 | harmonic mixing |
|  |  | n2 | 5 | REFSENS |  |
|  | 3 | 71 | 20 | -69.7 | harmonic mixing |
|  |  | n2 | 5 | REFSENS |  |
|  | 4 | 71 | 20 | REFSENS | harmonic mixing |
|  |  | n2 | 20 | REFSENS | avoiding |
| NOTE 1: The symbol "REFSENS" in this table refers to the reference sensitivity values for single carrier specified in Table 7.3.5-1 of TS 36.521-1 [10] for 2 antenna port E-UTRA band, Table 7.3_1.5-1 of TS 36.521-1 [10] for 4 antenna port E-UTRA band, Table 7.3.2.5-1a and Table 7.3.2.5-1b of TS 38.521-1 [8] for 2 antenna port NR band, Table 7.3.2.5-2a and Table 7.3.2.5-2b of TS 38.521-1 [8] for 4 antenna port NR band. |  |  |  |  |  |

<<End of change>>