3GPP TSG-RAN5 Meeting #108	R5-254766

# Bengaluru, India, 25 – 29 Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3504 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating V2X Tx test cases for intra-band concurrent operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are still open issues in a few V2X test cases, which need to be completed |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding general RB allocation for V2X intra-band concurrent operation in 6.1E. The relevant MPR/SEM/ACLR test cases are updated to refer to 6.1E. - Clarifying MOP doesn’t not need to be tested for intra-band concurrent operation. - Updating test procedures in 6.5E.2 to refer to PC2 configuration and test requiements in MPR and AMPR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1E, 6.2E.1.2, 6.2E.2.2, 6.5E.2.2, 6.5E.2.3, 6.5E.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.1E	General

Sidelink RB allocations given in Table 6.1E-1 are used for V2X test cases, unless otherwise stated by the test case.

Table 6.1E-1: Sidelink configuration for PSSCH/PSCCH

| BWchannel (MHz) | SCS (kHz) | Parameters | RB allocation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Outer_Full | Outer_10RB_Left | Outer_10RB_Right | Inner_Full |
| 10MHz | 15 | PSSCH allocation | 50@0 | 10@0 | 10@42 | 25@12 |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@42 | 10@12 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 42 | 12 |
|  |  | sl-SubchannelSize-r16 | 10 | 10 | 10 | 25 |
|  |  | sl-NumSubchannel-r16 | 5 | 1 | 1 | 1 |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |
|  | 30 | PSSCH allocation | 24@0 | 10@0 | 10@14 | 12@6 |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@14 | 10@6 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 14 | 6 |
|  |  | sl-SubchannelSize-r16 | 12 | 10 | 10 | 12 |
|  |  | sl-NumSubchannel-r16 | 2 | 1 | 1 | 1 |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |
|  | 60 | PSSCH allocation | 10@0 | 10@0 | 10@1 | N/A |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@1 | N/A |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 1 | N/A |
|  |  | sl-SubchannelSize-r16 | 10 | 10 | 10 | N/A |
|  |  | sl-NumSubchannel-r16 | 1 | 1 | 1 | N/A |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |
| 20MHz | 15 | PSSCH allocation | 105@0 | 10@0 | 10@96 | 50@25 |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@96 | 10@25 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 96 | 25 |
|  |  | sl-SubchannelSize-r16 | 15 | 10 | 10 | 10 |
|  |  | sl-NumSubchannel-r16 | 7 | 1 | 1 | 5 |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |
|  | 30 | PSSCH allocation | 50@0 | 10@0 | 10@41 | 25@12 |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@41 | 10@12 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 41 | 12 |
|  |  | sl-SubchannelSize-r16 | 10 | 10 | 10 | 25 |
|  |  | sl-NumSubchannel-r16 | 5 | 1 | 1 | 1 |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |
|  | 60 | PSSCH allocation | 24@0 | 10@0 | 10@14 | 12@6 |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@14 | 10@6 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 14 | 6 |
|  |  | sl-SubchannelSize-r16 | 12 | 10 | 10 | 12 |
|  |  | sl-NumSubchannel-r16 | 2 | 1 | 1 | 1 |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |

| BWchannel (MHz) | SCS (kHz) | Parameters | RB allocation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Outer_Full | Outer_10RB_Left | Outer_10RB_Right | Inner_Full |
| 30MHz | 15 | PSSCH allocation | 160@0 | 10@0 | 10@150 | 80@40 |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@150 | 10@40 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 150 | 40 |
|  |  | sl-SubchannelSize-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-NumSubchannel-r16 | 16 | 1 | 1 | 8 |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |
|  | 30 | PSSCH allocation | 75@0 | 10@0 | 10@68 | 36@18 |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@68 | 10@18 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 68 | 18 |
|  |  | sl-SubchannelSize-r16 | 15 | 10 | 10 | 12 |
|  |  | sl-NumSubchannel-r16 | 5 | 1 | 1 | 3 |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |
|  | 60 | PSSCH allocation | 36@0 | 10@0 | 10@28 | 15@7 |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@28 | 10@7 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 28 | 7 |
|  |  | sl-SubchannelSize-r16 | 12 | 10 | 10 | 15 |
|  |  | sl-NumSubchannel-r16 | 3 | 1 | 1 | 1 |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |
| 40MHz | 15 | PSSCH allocation | 216@0 | 10@0 | 10@206 | 105@52 |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@206 | 10@52 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 206 | 52 |
|  |  | sl-SubchannelSize-r16 | 12 | 10 | 10 | 15 |
|  |  | sl-NumSubchannel-r16 | 18 | 1 | 1 | 7 |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |
|  | 30 | PSSCH allocation | 105@0 | 10@0 | 10@96 | 50@25 |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@96 | 10@25 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 96 | 25 |
|  |  | sl-SubchannelSize-r16 | 15 | 10 | 10 | 10 |
|  |  | sl-NumSubchannel-r16 | 7 | 1 | 1 | 5 |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |
|  | 60 | PSSCH allocation | 50@0 | 10@0 | 10@41 | 25@12 |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@41 | 10@12 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 41 | 12 |
|  |  | sl-SubchannelSize-r16 | 10 | 10 | 10 | 25 |
|  |  | sl-NumSubchannel-r16 | 5 | 1 | 1 | 1 |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |

Table 6.1E-2: Sidelink configuration for PSFCH

| BWchannel (MHz) | SCS (kHz) | Parameters | RB allocation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PSFCH_1RB_Left | PSFCH_1RB_Right | PSFCH_2RB_Left | PSFCH_2RB_Right | PSFCH_Max_Gap |  |
| 10MHz | 15 | PSFCH allocation | 1@0 | 1@51 | 2@0 | 2@50 | 1@0 | 1@49 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 2 | 0 | 2 | 0 |  |
|  |  | sl-SubchannelSize-r16 | 25 | 25 | 25 | 25 | 25 |  |
|  |  | sl-NumSubchannel-r16 | 2 | 2 | 2 | 2 | 2 |  |
|  |  | sl-PSFCH-RB-Set-r16 | Bitstring of length 50, The leftmost 4 bits and rightmost 4 bits are set to "1" and the rest are set to "0" |  |  |  |  |  |
|  | 30 | PSFCH allocation | 1@0 | 1@23 | 2@0 | 2@22 | 1@0 | 1@23 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 0 | 0 | 0 |  |
|  |  | sl-SubchannelSize-r16 | 12 | 12 | 12 | 12 | 12 |  |
|  |  | sl-NumSubchannel-r16 | 2 | 2 | 2 | 2 | 2 |  |
|  |  | sl-PSFCH-RB-Set-r16 | Bitstring of length 24, The leftmost 4 bits and rightmost 4 bits are set to "1" and the rest are set to "0" |  |  |  |  |  |
|  | 60 | PSFCH allocation | 10@0 | 1@10 | 2@0 | 2@9 | 1@0 | 1@9 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 1 | 0 | 1 | 0 |  |
|  |  | sl-SubchannelSize-r16 | 10 | 10 | 10 | 10 | 10 |  |
|  |  | sl-NumSubchannel-r16 | 1 | 1 | 1 | 1 | 1 |  |
|  |  | sl-PSFCH-RB-Set-r16 | Bitstring of length 10, The leftmost 2 bits and rightmost 2 bits are set to "1" and the rest are set to "0" |  |  |  |  |  |
| 20MHz | 15 | PSFCH allocation | 1@0 | 1@105 | 2@0 | 2@104 | 1@0 | 1@104 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 1 | 0 | 1 | 0 |  |
|  |  | sl-SubchannelSize-r16 | 15 | 15 | 15 | 15 | 15 |  |
|  |  | sl-NumSubchannel-r16 | 7 | 7 | 7 | 7 | 7 |  |
|  |  | sl-PSFCH-RB-Set-r16 | Bitstring of length 105, The leftmost 14 bits and rightmost 14 bits are set to "1" and the rest are set to "0" |  |  |  |  |  |
|  | 30 | PSFCH allocation | 1@0 | 1@50 | 2@0 | 2@49 | 1@0 | 1@49 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 1 | 0 | 1 | 0 |  |
|  |  | sl-SubchannelSize-r16 | 25 | 25 | 25 | 25 | 25 |  |
|  |  | sl-NumSubchannel-r16 | 2 | 2 | 2 | 2 | 2 |  |
|  |  | sl-PSFCH-RB-Set-r16 | Bitstring of length 50, The leftmost 4 bits and rightmost 4 bits are set to "1" and the rest are set to "0" |  |  |  |  |  |
|  | 60 | PSFCH allocation | 1@0 | 1@23 | 2@0 | 2@22 | 1@0 | 1@23 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 0 | 0 | 0 |  |
|  |  | sl-SubchannelSize-r16 | 12 | 12 | 12 | 12 | 12 |  |
|  |  | sl-NumSubchannel-r16 | 2 | 2 | 2 | 2 | 2 |  |
|  |  | sl-PSFCH-RB-Set-r16 | Bitstring of length 24, The leftmost 4 bits and rightmost 4 bits are set to "1" and the rest are set to "0" |  |  |  |  |  |
| 30MHz | 15 | PSFCH allocation | 1@0 | 1@159 | 2@0 | 2@158 | 1@0 | 1@159 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 0 | 0 | 0 |  |
|  |  | sl-SubchannelSize-r16 | 20 | 20 | 20 | 20 | 20 |  |
|  |  | sl-NumSubchannel-r16 | 8 | 8 | 8 | 8 | 8 |  |
|  |  | sl-PSFCH-RB-Set-r16 | Bitstring of length 160, The leftmost 16 bits and rightmost 16 bits are set to "1" and the rest are set to "0" |  |  |  |  |  |
|  | 30 | PSFCH allocation | 1@0 | 1@77 | 2@0 | 2@76 | 1@0 | 1@74 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 3 | 0 | 3 | 0 |  |
|  |  | sl-SubchannelSize-r16 | 25 | 25 | 25 | 25 | 25 |  |
|  |  | sl-NumSubchannel-r16 | 3 | 3 | 3 | 3 | 3 |  |
|  |  | sl-PSFCH-RB-Set-r16 | Bitstring of length 75, The leftmost 6 bits and rightmost 6 bits are set to "1" and the rest are set to "0" |  |  |  |  |  |
|  | 60 | PSFCH allocation | 1@0 | 1@37 | 2@0 | 2@36 | 1@0 | 1@35 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 2 | 0 | 2 | 0 |  |
|  |  | sl-SubchannelSize-r16 | 12 | 12 | 12 | 12 | 12 |  |
|  |  | sl-NumSubchannel-r16 | 3 | 3 | 3 | 3 | 3 |  |
|  |  | sl-PSFCH-RB-Set-r16 | Bitstring of length 36, The leftmost 6 bits and rightmost 6 bits are set to "1" and the rest are set to "0" |  |  |  |  |  |

| BWchannel (MHz) | SCS (kHz) | Parameters | RB allocation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PSFCH_1RB_Left | PSFCH_1RB_Right | PSFCH_2RB_Left | PSFCH_2RB_Right | PSFCH_Max_Gap |  |
| 40MHz | 15 | PSFCH allocation | 1@0 | 1@215 | 2@0 | 2@214 | 1@0 | 1@215 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 0 | 0 | 0 |  |
|  |  | sl-SubchannelSize-r16 | 12 | 12 | 12 | 12 | 12 |  |
|  |  | sl-NumSubchannel-r16 | 18 | 18 | 18 | 18 | 18 |  |
|  |  | sl-PSFCH-RB-Set-r16 | Bitstring of length 216, The leftmost 36 bits and rightmost 36 bits are set to "1" and the rest are set to "0" |  |  |  |  |  |
|  | 30 | PSFCH allocation | 1@0 | 1@105 | 2@0 | 2@104 | 1@0 | 1@104 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 1 | 0 | 1 | 0 |  |
|  |  | sl-SubchannelSize-r16 | 15 | 15 | 15 | 15 | 15 |  |
|  |  | sl-NumSubchannel-r16 | 7 | 7 | 7 | 7 | 7 |  |
|  |  | sl-PSFCH-RB-Set-r16 | Bitstring of length 105, The leftmost 14 bits and rightmost 14 bits are set to "1" and the rest are set to "0" |  |  |  |  |  |
|  | 60 | PSFCH allocation | 1@0 | 1@50 | 2@0 | 2@49 | 1@0 | 1@49 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 1 | 0 | 1 | 0 |  |
|  |  | sl-SubchannelSize-r16 | 25 | 25 | 25 | 25 | 25 |  |
|  |  | sl-NumSubchannel-r16 | 2 | 2 | 2 | 2 | 2 |  |
|  |  | sl-PSFCH-RB-Set-r16 | Bitstring of length 50, The leftmost 4 bits and rightmost 4 bits are set to "1" and the rest are set to "0" |  |  |  |  |  |

Table 6.1E-2: Sidelink configuration for S-SSB (PSBCH)

| BWchannel (MHz) | SCS (kHz) | RB allocation |  |  |
| --- | --- | --- | --- | --- |
|  |  | S-SSB_Low | S-SSB_Mid | S-SSB_High |
| 10MHz | 15 | 11@0 | 11@21 | 11@41 |
|  | 30 | 11@0 | 11@7 | 11@13 |
|  | 60 | 11@0 | 11@0 | 11@0 |
| 20MHz | 15 | 11@0 | 11@48 | 11@95 |
|  | 30 | 11@0 | 11@20 | 11@40 |
|  | 60 | 11@0 | 11@7 | 11@13 |
| 30MHz | 15 | 11@0 | 11@75 | 11@149 |
|  | 30 | 11@0 | 11@34 | 11@67 |
|  | 60 | 11@0 | 11@14 | 11@27 |
| 40MHz | 15 | 11@0 | 11@103 | 11@205 |
|  | 30 | 11@0 | 11@48 | 11@95 |
|  | 60 | 11@0 | 11@20 | 11@40 |
| NOTE: sl-AbsoluteFrequencySSB-r16 for each S-SSB allocation is specified in TS 38.508-1 [5] 4.3.1.8. |  |  |  |  |

Table 6.1E-3: Sidelink configuration for PSSCH/PSCCH for intra-band concurrent with contiguous allocation (Inner allocation)

| SCS (kHz) | Aggregated BWchannel (MHz) | UU Carrier |  | PC5 Carrier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | BWchannel (MHz) | RB allocation | BWchannel (MHz) | Parameters | Value |
| 15 | 50MHz | 40 | 105@111 | 10 | PSSCH allocation | N/A |
|  |  |  |  |  | PSCCH allocation |  |
|  |  |  |  |  | sl-StartRB-Subchannel-r16 |  |
|  |  |  |  |  | sl-SubchannelSize-r16 |  |
|  |  |  |  |  | sl-NumSubchannel-r16 |  |
|  |  |  |  |  | sl-FreqResourcePSCCH-r16 |  |
|  |  |  |  |  | sl-TimeResourcePSCCH-r16 |  |
|  | 90MHz | 50 | 149@121 | 40 | PSSCH allocation | 90@0 |
|  |  |  |  |  | PSCCH allocation | 10@0 |
|  |  |  |  |  | sl-StartRB-Subchannel-r16 | 0 |
|  |  |  |  |  | sl-SubchannelSize-r16 | 10 |
|  |  |  |  |  | sl-NumSubchannel-r16 | 10 |
|  |  |  |  |  | sl-FreqResourcePSCCH-r16 | 10 |
|  |  |  |  |  | sl-TimeResourcePSCCH-r16 | 3 |
| 30 | 50MHz | 40 | 48@58 | 10 | PSSCH allocation | N/A |
|  |  |  |  |  | PSCCH allocation |  |
|  |  |  |  |  | sl-StartRB-Subchannel-r16 |  |
|  |  |  |  |  | sl-SubchannelSize-r16 |  |
|  |  |  |  |  | sl-NumSubchannel-r16 |  |
|  |  |  |  |  | sl-FreqResourcePSCCH-r16 |  |
|  |  |  |  |  | sl-TimeResourcePSCCH-r16 |  |
|  | 140MHz | 100 | 178@95 | 40 | PSSCH allocation | 10@0 |
|  |  |  |  |  | PSCCH allocation | 10@0 |
|  |  |  |  |  | sl-StartRB-Subchannel-r16 | 0 |
|  |  |  |  |  | sl-SubchannelSize-r16 | 10 |
|  |  |  |  |  | sl-NumSubchannel-r16 | 1 |
|  |  |  |  |  | sl-FreqResourcePSCCH-r16 | 10 |
|  |  |  |  |  | sl-TimeResourcePSCCH-r16 | 3 |

Table 6.1E-4: Sidelink configuration for PSSCH/PSCCH for intra-band concurrent with contiguous allocation (Outer allocation)

| SCS (kHz) | Aggregated BWchannel (MHz) | UU Carrier |  | PC5 Carrier |  |
| --- | --- | --- | --- | --- | --- |
|  |  | BWchannel (MHz) | RB allocation | BWchannel (MHz) | RB allocation |
| 15 | 50MHz | 40 | Outer_Full in Table 6.1-1 | 10 | Outer_Full in Table Table 6.1E-1 |
|  | 90MHz | 50 |  | 40 |  |
| 30 | 50MHz | 40 |  | 10 |  |
|  | 140MHz | 100 |  | 40 |  |

<Unchanged Text Skipped>

6.2E.1.2	UE maximum output power for V2X / concurrent operation

- No test points are defined for intra-band concurrent operation since there is satisfying MPR=0dB requirements in TS 38.101-1. Testing with minimum MPR has been covered in 6.2E.2.2.

6.2E.1.2.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2E.1.2.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support NR V2X sidelink communication.

6.2E.1.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2E.1.0.

6.2E.1.2.4	Test description

6.2E.1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.2E.2-1 and Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2E.1.2.4.1-1. The details of the V2X reference measurement channels (RMCs) are specified in Annexes A.7.5. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

<Unchanged Text Skipped>

6.2E.2.2	UE maximum output power reduction for V2X / concurrent operation

Editor’s Note:

- MU/TT for aggregated BW > 100MHz is TBD.

6.2E.2.2.1	Test purpose

To verify that the reduction of UE transmitted power due to higher order modulations and transmit bandwidth configurations is within the allowed range. An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2E.2.2.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support NR V2X sidelink communication.

6.2E.2.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2E.2.0.

6.2E.2.2.4	Test description

6.2E.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.2E.2-1 and Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2E.2.2.4.1-1. The details of the V2X reference measurement channels (RMCs) are specified in Annexes A.7.5. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2E.2.2.4.1-1: Test Configuration Table for PC3 inter-band concurrent operation

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Low range for NR Uu and V2X carrier High range for NR Uu and V2X carrier |  |  |  |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Lowest for NR Uu and V2X carrier Highest for NR Uu and V2X carrier |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest for NR Uu and V2X carrier Highest for NR Uu and V2X carrier |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |
|  | NR Uu Configuration |  |  |  | NR V2X Configuration to Transmit |  |  |
|  | Freq | Downlink | Uplink Configuration |  | Freq | Modulation | PSCCH |
|  |  | Configuration | Modulation | RB allocation (NOTE 1) |  |  | and PSSCH RB allocation (Note 2) |
| 1 | Default |  | DFT-s-OFDM QPSK | Inner Full | Default | CP-OFDM QPSK | Inner_Full |
| 2 | Default | N/A | CP-OFDM 256 QAM | Outer Full | Default | CP-OFDM 256QAM | Outer_Full |
| 3 | Default |  | DFT-s-OFDM QPSK | Inner Full | Default | CP-OFDM 256QAM | Outer_Full |
| 4 | Default |  | CP-OFDM 256 QAM | Outer Full | Default | CP-OFDM QPSK | Inner_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1E-1. |  |  |  |  |  |  |  |

Table 6.2E.2.2.4.1-2: Test Configuration Table for PC3 intra-band concurrent operation (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest NRB_agg, Highest NRB_agg |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest for NR Uu and V2X carrier Highest for NR Uu and V2X carrier |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | NR DL Configuration | NR UL & V2X Configuration |  |
|  |  |  | Modulation | RB allocation (NOTE 1) |
| 1 | Default | N/A | CP-OFDM 16QAM | Inner Full |
| 2 | Default |  | CP-OFDM 16QAM | Outer Full |
| 3 | Default |  | CP-OFDM 64QAM | Inner Full |
| 4 | Default |  | CP-OFDM 64QAM | Outer Full |
| 5 | Default |  | CP-OFDM 256QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1E-3 and 6.1E-4. |  |  |  |  |

Table 6.2E.2.2.4.1-3: Test Configuration Table for PC2 intra-band concurrent operation (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest NRB_agg, Highest NRB_agg |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest for NR Uu and V2X carrier Highest for NR Uu and V2X carrier |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | NR DL Configuration | NR UL & V2X Configuration |  |
|  |  |  | Modulation | RB allocation (NOTE 1) |
| 1 | Default | N/A | CP-OFDM QPSK | Inner Full |
| 1 | Default |  | CP-OFDM 16QAM | Inner Full |
| 2 | Default |  | CP-OFDM 16QAM | Outer Full |
| 3 | Default |  | CP-OFDM 64QAM | Inner Full |
| 4 | Default |  | CP-OFDM 64QAM | Outer Full |
| 5 | Default |  | CP-OFDM 256QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1E-3 and 6.1E-4. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.9.3 for TE diagram and clause A.3.2.7 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] clause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The V2X Reference Measurement Channel and NR UL Reference Measurement Channel are set according to Table 6.2E.2.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameter Connectivity NR, Connected without release on, Test Mode On and Sidelink On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2E.2.2.4.3.

6.2E.2.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2E.2.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	SS sends sidelink scheduling information for each SL-HARQ process via PDCCH DCI format 3_0 for C_RNTI to schedule the Sidelink RMC according to Table 6.2E.2.2.4.1-1. UE is configured to transmit at Pcmax on the sidelink carrier. Since the UE has no payload and no loopback data to send the UE sends MAC padding bits on the Sidelink RMC.

4.	Measure the sum of the mean power of the UE at NR Uu carrier and V2X sidelink carrier in the channel bandwidth according to the test configuration from Table 6.2E.2.2.4.1-1. The period of measurement shall be at least continuous duration of one active sub-frame (1ms) excluding guard symbols.

6.2E.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] clause 4.6.

Table 6.2E.2.2.4.3-1: SL-ResourcePool

| Derivation Path: TS 38.508-1 [5], Table 4.6.6-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |
| sl-PSCCH-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sl-TimeResourcePSCCH-r16 | As defined in Table 6.1E-2 |  |  |
| sl-FreqResourcePSCCH-r16 | As defined in Table 6.1E-2 |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-SubchannelSize-r16 | As defined in Table 6.1E-2 |  |  |
| sl-StartRB-Subchannel-r16 | As defined in Table 6.1E-2 |  |  |
| sl-NumSubchannel-r16 | As defined in Table 6.1E-2 |  |  |
| sl-PowerControl-r16 SEQUENCE { |  |  |  |
| sl-MaxTransPower-r16 | 24 |  |  |
| sl-Alpha-PSSCH-PSCCH-r16 | Not present |  |  |
| dl-Alpha-PSSCH-PSCCH-r16 | Not present |  |  |
| sl-P0-PSSCH-PSCCH-r16 | Not present |  |  |
| dl-P0-PSSCH-PSCCH-r16 | Not present |  |  |
| dl-Alpha-PSFCH-r16 | Not present |  |  |
| dl-P0-PSFCH-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

6.2E.2.2.5	Test requirement

The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2E.2.2.5-1.

Table 6.2E.2.2.5-1: UE MPR test requirement for inter-band concurrent NR V2X operation

|  | PPowerClass,NR (dBm) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,c, NR (dBm) | PCMAX_H,c, NR (dB) | PPowerClass,V2X (dBm) | PCMAX_H,f,c,V2X (dBm) | PCMAX_L (dBm) | PCMAX_H (dBm) | T(PCMAX_L) (dB) | T(PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0 | 23 | 23 | 23 | 23 | 23 | 26 | 3 | 2 | 28+TT | 20-TT |
| 2 | 23 | 6.5 | 0 | 16.5 | 23 | 23 | 23 | 16.5 | 26 | 5 | 2 | 28+TT | 11.5-TT |
| 3 | 23 | 0 | 0 | 23 | 23 | 23 | 23 | 23 | 26 | 3 | 2 | 28+TT | 20-TT |
| 4 | 23 | 6.5 | 0 | 16.5 | 23 | 23 | 23 | 16.5 | 26 | 5 | 2 | 28+TT | 11.5-TT |

Table 6.2E.2.2.5-2: Test Tolerance (inter-band concurrent NR V2X operation)

|  | f ≤ 3.0GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB |

Table 6.2E.2.2.5-3: Test Tolerance (intra-band concurrent NR V2X operation)

|  | f ≤ 3.0GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| Aggregated BW ≤ 100MHz | 0.7 dB | 1.0 dB |

Table 6.2E.2.2.5-4: UE MPR test requirement for PC3 intra-band concurrent NR V2X operation

|  | PPowerClass,con-current (dBm) | MPR (dB) | ΔTC,c (dB) | PCMAX_L (dBm) | PCMAX_H (dB) | T(PCMAX_L) (dB) | T(PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 2.5 | 0 | 20.5 | 23 | 2.5 | 2 | 25+TT | 20.5-TT |
| 2 | 23 | 4.5 | 0 | 18.5 | 23 | 4.0 | 2 | 25+TT | 19.0-TT |
| 3 | 23 | 4.5 | 0 | 18.5 | 23 | 4.0 | 2 | 25+TT | 19.0-TT |
| 4 | 23 | 5.0 | 0 | 18.0 | 23 | 4.0 | 2 | 25+TT | 19.0-TT |
| 5 | 23 | 6.0 | 0 | 17.0 | 23 | 5.0 | 2 | 25+TT | 18.0-TT |

Table 6.2E.2.2.5-5: UE MPR test requirement for PC2 intra-band concurrent NR V2X operation

|  | PPowerClass,concurrent (dBm) | MPR (dB) | ΔTC,c (dB) | PCMAX_L (dBm) | PCMAX_H (dB) | T(PCMAX_L) (dB) | T(PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 3.0 | 0 | 23.0 | 26 | 2.0 | 2 | 28+TT | 18.0-TT |
| 2 | 26 | 4.0 | 0 | 22.0 | 26 | 2.0 | 2 | 28+TT | 14.5-TT |
| 3 | 26 | 5.5 | 0 | 20.5 | 26 | 2.5 | 2 | 28+TT | 14.5-TT |
| 4 | 26 | 5.5 | 0 | 20.5 | 26 | 2.5 | 2 | 28+TT | 14.0-TT |
| 5 | 26 | 6.0 | 0 | 20.0 | 26 | 2.5 | 2 | 28+TT | 12.0-TT |
| 6 | 26 | 7.5 | 0 | 18.5 | 26 | 4.0 | 2 | 28+TT | 18.0-TT |

<Unchanged Text Skipped>

6.5E.2.2	Spectrum emission mask for V2X

6.5E.2.2.0	Minimum conformance requirements

6.5E.2.2.0.1	General

For NR V2X UE, the existing NR general spectrum emission mask in subclause 6.5.2.2.3 applies for all supporting NR V2X channel bandwidths. The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the  edge of the assigned NR channel bandwidth. For frequencies greater than (ΔfOOB), the power of any UE emission shall not exceed the levels specified in Table 6.5.2.2.3-1 for the specified channel bandwidth for NR V2X operating bands in Table 5.2E.1-1.

6.5E.2.2.0.2	Spectrum emission mask for V2X concurrent operation

For the inter-band concurrent NR V2X operation, the general/additional SEM requirements specified in clause 6.5.2 shall apply for the uplink in licensed band and the general/additional SEM requirements specified in clause 6.5E.2.2.1 shall apply for the sidelink in licensed band or Band n47.

For intra-band NR V2X transmission with bandwidth class B where Uu and SL overlap in time the specifications in section 6.5A.2.2.1 and 6.5A.2.2.2 apply.

6.5E.2.2.1	Spectrum emission mask for V2X / non-concurrent operation

6.5E.2.2.1.1	Test purpose

To verify that the power of any UE emission shall not exceed specified level for the specified channel bandwidth when UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions.

6.5E.2.2.1.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support NR V2X sidelink communication with non-concurrent operation.

6.5E.2.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5E.2.2.0.

6.5E.2.2.1.4	Test description

6.5E.2.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.2E.1-1 and Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.5E.2.2.1.4.1-1 to 6.5E.2.2.1.4.1-3. The details of the V2X reference measurement channels (RMCs) are specified in Annexe A.7.5 and the GNSS configuration in TS 38.508-1 [5] subclause 4.11.

Table 6.5E.2.2.1.4.1-1: Test Configuration Table for contiguous PSCCH and PSSCH allocation for PC3

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1.8 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | V2X Configuration to Transmit |  |  |
|  |  | Modulation |  | PSSCH RB allocation (Note 1) |
| 1 | Default | 16QAM |  | Outer_Full |
| 2 | Default | 16QAM |  | Inner_Full |
| 3 | Default | 64QAM |  | Outer_Full |
| 4 | Default | 256QAM |  | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1E-1. |  |  |  |  |

Table 6.5E.2.2.1.4.1-1a: Test Configuration Table for contiguous PSCCH and PSSCH allocation for PC2

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1.8 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | V2X Configuration to Transmit |  |  |
|  |  | Modulation |  | PSCCH and PSSCH RB allocation (Note 1) |
| 1 | Default | 16QAM |  | Outer_Full |
| 2 | Default | 16QAM |  | Inner_Full |
| 3 | Default | 64QAM |  | Outer_Full |
| 4 | Default | 64QAM |  | Inner_Full |
| 5 | Default | 256QAM |  | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1E-1. |  |  |  |  |

Table 6.5E.2.2.1.4.1-2: Test Configuration Table for PSFCH for PC2 and PC3

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1.8 |  |  | Low range, High range |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Freq | V2X Configuration to Transmit |  |
|  |  | PSFCH RB allocation (Note 1) |  |
| 1 | Low range | PSFCH_1RB_Left |  |
| 2 | High range | PSFCH_1RB_Right |  |
| 3 | Low range | PSFCH_2RB_Left |  |
| 4 | High range | PSFCH_2RB_Right |  |
| 5 | Default | PSFCH_Max_Gap |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1E-2. |  |  |  |

Table 6.5E.2.2.1.4.1-3: Test Configuration Table for S-SSB for PC2 and PC3

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1.8 |  |  | Low range, High range |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Freq | V2X Configuration to Transmit |  |
|  |  | S-SSB RB allocation (Note 1) |  |
| 1 | Low range | S-SSB_Low |  |
| 2 | High range | S-SSB_High |  |
| 3 | Default | S-SSB_Mid |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1E-3. |  |  |  |

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.9.1 for TE diagram and section A.3.2.7 for UE diagram.

2.	The parameter settings for the NR sidelink transmission over PC5 are pre-configured according to TS 38.508-1 [5] subclause 4.10. Message content exceptions are defined in clause 6.5E.2.2.1.4.3.

3.	The V2X Reference Measurement Channel is set according to Table 6.5E.2.2.1.4.1-1 to Table 6.5E.2.2.1.4.1-3.

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS 38.508-1 [5] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0.

6.5E.2.2.1.4.2	Test procedure

Subtest 1: PSCCH/PSSCH

1.	Ensure the UE is in state Out_of_Coverage with generic procedure parameters Sidelink On, Test Loop Function On with UE test loop mode E closed for Transmit Mode according to TS 38.508-1 [5] clause 4.5.

2.	The UE starts to perform the NR sidelink communication according to SL-PreconfigurationNR. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the NR sidelink RMC.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration from Table 6.5E.2.2.1.4.1-1, which shall meet the requirements described in clause 6.2E.2.1.5. The period of measurement shall be at least continuous duration of one active sub-frame (1ms) excluding guard symbols.

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.5E.2.2.1.5-1 and using a rms detector. If the sweep count is higher than one, the trace mode shall be average. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

Subtest 2: PSFCH

1.	Ensure the UE is in state Out_of_Coverage with generic procedure parameters Sidelink On, Cast type Unicast, Test Loop Function On with UE test loop mode E closed for Receive Mode according to TS 38.508-1 [5] clause 4.5.

2.	The UE starts to perform the NR sidelink reception according to SL-PreconfigurationNR.

3.	The UE’s PSFCH transmission occasion is on slot n according to Table 6.5E.2.2.1.4.1-2. SS transmits PSSCH on combination of slot and subchannel as below:

a)	Test ID 1: slot n-6, Lowest sub-channel

b)	Test ID 2: slot n-3, Highest sub-channel

c)	Test ID 3: slot n-6 and n-5, Lowest sub-channel

d)	Test ID 4: slot n-4 and n-3, Highest sub-channel

e)	Test ID 5: slot n-6, Highest sub-channel and slot n-3, Highest sub-channel

4.	Measure the mean power of the UE on slot n in the channel bandwidth according to the test configuration from Table 6.5E.2.2.1.4.1-2, which shall meet the requirements described in clause 6.2E.2.1.5. The period of measurement is one active sub-frame (1ms) excluding guard symbols.

5.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.5E.2.2.1.5-1 and using a rms detector. If the sweep count is higher than one, the trace mode shall be average. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

Subtest 3: S-SSB

1.	Ensure the UE is in state Out_of_Coverage with generic procedure parameters Sidelink On according to TS 38.508-1 [5] clause 4.5. The UE is synchronized to GNSS,

2.	The UE transmits PSBCH according SL-PreconfigurationNR.

3.	Measure the mean power of the S-SSB in the channel bandwidth according to the test configuration from Table 6.5E.2.2.1.4.1-3, which shall meet the requirements described in clause 6.2E.2.1.5. The period of measurement is one active sub-frame (1ms) excluding guard symbols.

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.5E.2.2.1.5-1 and using a rms detector. If the sweep count is higher than one, the trace mode shall be average. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.5E.2.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.10 with the following exceptions.

Table 6.5E.2.2.1.4.3-1: SL-ResourcePool for PSCCH/PSSCH Testing

| Derivation Path: TS 38.508-1 [5], Table 4.6.6-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |
| sl-PSCCH-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sl-TimeResourcePSCCH-r16 | As defined in Table 6.1E-2 |  |  |
| sl-FreqResourcePSCCH-r16 | As defined in Table 6.1E-2 |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-SubchannelSize-r16 | As defined in Table 6.1E-2 |  |  |
| sl-StartRB-Subchannel-r16 | As defined in Table 6.1E-2 |  |  |
| sl-NumSubchannel-r16 | As defined in Table 6.1E-2 |  |  |
| } |  |  |  |

Table 6.5E.2.2.1.4.3-2: SL-ResourcePool for PSFCH Testing

| Derivation Path: TS 38.508-1 [5], Table 4.6.6-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |
| sl-PSFCH-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sl-PSFCH-Period-r16 | sl4 |  |  |
| sl-PSFCH-RB-Set-r16 | As defined in Table 6.1E-2 |  |  |
| sl-NumMuxCS-Pair-r16 | n1 |  |  |
| sl-MinTimeGapPSFCH-r16 | sl3 |  |  |
| sl-PSFCH-HopID-r16 | Not present | Default frequency hopping ID 0 is used |  |
| sl-PSFCH-CandidateResourceType-r16 | startSubCH |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-SubchannelSize-r16 | As defined in Table 6.1E-2 |  |  |
| sl-StartRB-Subchannel-r16 | As defined in Table 6.1E-2 |  |  |
| sl-NumSubchannel-r16 | As defined in Table 6.1E-2 |  |  |
| } |  |  |  |

Table 6.5E.2.2.1.4.3-3: SL-FreqConfigCommon for S-SSB Testing

| Derivation Path: TS 38.508-1 [5], Table 4.6.6-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-FreqConfigCommon-r16 ::= SEQUENCE { |  |  |  |
| sl-AbsoluteFrequencySSB-r16 | According to section 4.3.1.8 of TS 38.508-1 [5] |  |  |
| } |  |  |  |

6.5E.2.2.1.5	Test requirement

The measured UE mean power in the channel bandwidth, derived in step 3 of Subtest 1, step 4 of Subtest 2 and step 3 of Subtest 3, shall fulfil requirements in clause 6.2E.2.1.5 as appropriate, and the power of any UE emission shall fulfil requirements in Table 6.5E.2.2.1.5-1.

Table 6.5E.2.2.1.5-1: General NR spectrum emission mask for V2X / non-concurrent operation

|  | Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Measurement bandwidth |
| ± 0-1 |  | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT |  |  |  |  |  | 1 % channel bandwidth |
| ± 0-1 |  |  |  |  |  |  |  |  | -24 + TT | -24 + TT | -24 + TT | -24 + TT | -24 + TT | 30 kHz |
| ± 1-5 |  | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | 1 MHz |
| ± 5-6 |  | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT |  |
| ± 6-10 |  | -25 + TT |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 10-15 |  |  | -25 + TT |  |  |  |  |  |  |  |  |  |  |  |
| ± 15-20 |  |  |  | -25 + TT |  |  |  |  |  |  |  |  |  |  |
| ± 20-25 |  |  |  |  | -25 + TT |  |  |  |  |  |  |  |  |  |
| ± 25-30 |  |  |  |  |  | -25 + TT |  |  |  |  |  |  |  |  |
| ± 30-35 |  |  |  |  |  |  | -25 + TT |  |  |  |  |  |  |  |
| ± 35-40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 40-45 |  |  |  |  |  |  |  | -25 + TT |  |  |  |  |  |  |
| ± 45-50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 50-55 |  |  |  |  |  |  |  |  | -25 + TT |  |  |  |  |  |
| ± 55-60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 60-65 |  |  |  |  |  |  |  |  |  | -25 + TT |  |  |  |  |
| ± 65-80 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 80-85 |  |  |  |  |  |  |  |  |  |  | -25 + TT |  |  |  |
| ± 85-90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 90-95 |  |  |  |  |  |  |  |  |  |  |  | -25 + TT |  |  |
| ± 95-100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 100-105 |  |  |  |  |  |  |  |  |  |  |  |  | -25 + TT |  |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel. Note 4: TT for each frequency and channel bandwidth is specified in Table 6.5E.2.2.1.5-2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.5E.2.2.1.5-2: Test Tolerance

|  | f ≤ 3.0GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7dB | 1.0dB |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

6.5E.2.2.1D	Spectrum emission mask for V2X / non-concurrent operation / SL-MIMO

6.5E.2.2.1D.1	Test purpose

Same test purpose as in 6.5E.2.2.1.1.

6.5E.2.2.1D.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support NR V2X sidelink communication and SL-MIMO.

6.5E.2.2.1D.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5E.2.2.0 and 6.5E.2.1.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5E.2.

6.5E.2.2.1D.4	Test description

Same test description as in 6.5E.2.2.1.4 for PSSCH/PSCCH subtest with following exceptions:

For initial conditions:

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.9.2 for TE diagram and section A.3.2.7 for UE diagram.

For test procedures:

Subtest 1 are replaced by:

1.	Ensure the UE is in state 4-A as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), Cast Type (Unicast), GNSS Sync (On) and Transmit Mode with SL-MIMO. The UE is synchronized to GNSS,

2.	The UE starts to perform the NR sidelink communication according to SL-PreconfigurationNR with 2-layer MIMO codebook TPMI 0. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the NR sidelink RMC.

3.	Measure the sum of mean power of the UE at each transmit antenna connector in the channel bandwidth of the radio access mode according to the test configuration from Table 6.5E.2.2.1.4.1-1, which shall meet the requirements described in clause 6.2E.2.1D.5. The period of measurement shall be at least continuous duration of one active sub-frame (1ms) excluding guard symbols.

4.	Measure the power of the transmitted signal at each transmit antenna connector with a measurement filter of bandwidths according to Table 6.5E.2.2.1.5-1 and using a rms detector. If the sweep count is higher than one, the trace mode shall be average. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.5E.2.2.1D.4.1	Void

6.5E.2.2.1D.4.2	Void

6.5E.2.2.1D.4.3	Void

6.5E.2.2.1D.5	Test requirement

The measured sum of mean power of each antenna connector in the channel bandwidth, derived in step 3, shall fulfil requirements in clause 6.2E.2.1D.5 as appropriate, and the power of any UE emission at each antenna connector shall fulfil requirements in Table 6.5E.2.2.1D.5-1.

Table 6.5E.2.2.1D.5-1: General NR spectrum emission mask for V2X / non-concurrent operation / SL-MIMO

|  | Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Measurement bandwidth |
| ± 0-1 |  | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT |  |  |  |  |  | 1 % channel bandwidth |
| ± 0-1 |  |  |  |  |  |  |  |  | -24 + TT | -24 + TT | -24 + TT | -24 + TT | -24 + TT | 30 kHz |
| ± 1-5 |  | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | 1 MHz |
| ± 5-6 |  | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT |  |
| ± 6-10 |  | -25 + TT |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 10-15 |  |  | -25 + TT |  |  |  |  |  |  |  |  |  |  |  |
| ± 15-20 |  |  |  | -25 + TT |  |  |  |  |  |  |  |  |  |  |
| ± 20-25 |  |  |  |  | -25 + TT |  |  |  |  |  |  |  |  |  |
| ± 25-30 |  |  |  |  |  | -25 + TT |  |  |  |  |  |  |  |  |
| ± 30-35 |  |  |  |  |  |  | -25 + TT |  |  |  |  |  |  |  |
| ± 35-40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 40-45 |  |  |  |  |  |  |  | -25 + TT |  |  |  |  |  |  |
| ± 45-50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 50-55 |  |  |  |  |  |  |  |  | -25 + TT |  |  |  |  |  |
| ± 55-60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 60-65 |  |  |  |  |  |  |  |  |  | -25 + TT |  |  |  |  |
| ± 65-80 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 80-85 |  |  |  |  |  |  |  |  |  |  | -25 + TT |  |  |  |
| ± 85-90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 90-95 |  |  |  |  |  |  |  |  |  |  |  | -25 + TT |  |  |
| ± 95-100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 100-105 |  |  |  |  |  |  |  |  |  |  |  |  | -25 + TT |  |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel. Note 4: TT for each frequency and channel bandwidth is specified in Table 6.5E.2.2.1D.5-2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.5E.2.2.1D.5-2: Test Tolerance

|  | f ≤ 3.0GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7dB | 1.0dB |

6.5E.2.2.2	Spectrum emission mask for V2X / concurrent operation

6.5E.2.2.2.1	Test purpose

Same test purpose as in 6.5E.2.2.1.1 for concurrent operation.

6.5E.2.2.2.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support NR V2X sidelink communication.

6.5E.2.2.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5E.2.2.0.

6.5E.2.2.2.4	Test description

6.5E.2.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.2E.2-1 and Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.5E.2.2.2.4.1-1 through Table 6.5E.2.2.2.4.1-3. The details of the V2X reference measurement channels (RMCs) are specified in Annexes A.7.5. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5E.2.2.2.4.1-1: Test Configuration Table for PC3 inter-band concurrent operation

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Low range for NR Uu and V2X carrier High range for NR Uu and V2X carrier |  |  |  |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Lowest for NR Uu and V2X carrier Highest for NR Uu and V2X carrier |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest for NR Uu and V2X carrier Highest for NR Uu and V2X carrier |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |
|  | NR Uu Configuration |  |  |  | NR V2X Configuration to Transmit |  |  |
|  | Freq | Downlink | Uplink Configuration |  | Freq | Modulation | PSCCH |
|  |  | Configuration | Modulation | RB allocation (NOTE 1) |  |  | and PSSCH RB allocation (Note 2) |
| 1 | Default |  | DFT-s-OFDM QPSK | Inner Full | Default | CP-OFDM QPSK | Inner_Full |
| 2 | Default | N/A | CP-OFDM 256 QAM | Outer Full | Default | CP-OFDM 256QAM | Outer_Full |
| 3 | Default |  | DFT-s-OFDM QPSK | Inner Full | Default | CP-OFDM 256QAM | Outer_Full |
| 4 | Default |  | CP-OFDM 256 QAM | Outer Full | Default | CP-OFDM QPSK | Inner_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1E-1. |  |  |  |  |  |  |  |

Table 6.5E.2.2.2.4.1-2: Test Configuration Table for PC3 intra-band concurrent operation (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest NRB_agg, Highest NRB_agg |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest for NR Uu and V2X carrier Highest for NR Uu and V2X carrier |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | NR DL Configuration | NR UL & V2X Configuration |  |
|  |  |  | Modulation | RB allocation (NOTE 1) |
| 1 | Default | N/A | CP-OFDM 16QAM | Inner Full |
| 2 | Default |  | CP-OFDM 16QAM | Outer Full |
| 3 | Default |  | CP-OFDM 64QAM | Inner Full |
| 4 | Default |  | CP-OFDM 64QAM | Outer Full |
| 5 | Default |  | CP-OFDM 256QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1E-3 and 6.1E-4. |  |  |  |  |

Table 6.5E.2.2.2.4.1-3: Test Configuration Table for PC2 intra-band concurrent operation (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest NRB_agg, Highest NRB_agg |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest for NR Uu and V2X carrier Highest for NR Uu and V2X carrier |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | NR DL Configuration | NR UL & V2X Configuration |  |
|  |  |  | Modulation | RB allocation (NOTE 1) |
| 1 | Default | N/A | CP-OFDM QPSK | Inner Full |
| 1 | Default |  | CP-OFDM 16QAM | Inner Full |
| 2 | Default |  | CP-OFDM 16QAM | Outer Full |
| 3 | Default |  | CP-OFDM 64QAM | Inner Full |
| 4 | Default |  | CP-OFDM 64QAM | Outer Full |
| 5 | Default |  | CP-OFDM 256QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1E-3 and 6.1E-4. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.9.3 for TE diagram and clause A.3.2.7 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] clause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The V2X Reference Measurement Channel and NR UL Reference Measurement Channel are set according to Table 6.5E.2.2.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameter Connectivity NR, Connected without release on, Test Mode On and Sidelink On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5E.2.2.2.4.3.

6.5E.2.2.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5E.2.2.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	SS sends sidelink scheduling information for each SL-HARQ process via PDCCH DCI format 3_0 for C_RNTI to schedule the Sidelink RMC according to Table 6.5E.2.2.2.4.1-1. UE is configured to transmit at Pcmax on the sidelink carrier. Since the UE has no payload and no loopback data to send the UE sends MAC padding bits on the Sidelink RMC.

4.	For inter-band concurrent operation: Measure the sum of the mean power of the UE at NR Uu carrier and V2X sidelink carrier in the channel bandwidth according to the test configuration from Table 6.5E.2.2.2.4.1-1, which shall meet the requirements described in Table 6.2E.2.2.5-1 for Power Class 3 UEs. The period of measurement shall be at least continuous duration of one active sub-frame (1ms) excluding guard symbols.

For intra-band concurrent operation: Measure the sum of the mean power of the UE at NR Uu carrier and V2X sidelink carrier in the channel bandwidth according to the test configuration from Table 6.5E.2.2.2.4.1-2 and Table 6.5E.2.2.2.4.1-3 as appropriate, which shall meet the requirements described in Table 6.2E.2.2.5-4 for Power Class 3 UEs and in Table 6.2E.2.2.5-5 for Power Class 2 UEs. The period of measurement shall be at least continuous duration of one active sub-frame (1ms) excluding guard symbols.

5.	For inter-band concurrent operation: Measure the power of the transmitted signal with a measurement filter of bandwidths for NR Uu carrier and V2X sidelink carrier respectively according to Table 6.5E.2.2.2.1.5-1 and using a rms detector. If the sweep count is higher than one, the trace mode shall be average. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

For intra-band concurrent operation: Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.5E.2.2.2.1.5-3 and using a rms detector. If the sweep count is higher than one, the trace mode shall be average. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.5E.2.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] clause 4.6.

Table 6.5E.2.2.2.4.3-1: SL-ResourcePool

| Derivation Path: TS 38.508-1 [5], Table 4.6.6-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |
| sl-PSCCH-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sl-TimeResourcePSCCH-r16 | As defined in Table 6.1E-2 |  |  |
| sl-FreqResourcePSCCH-r16 | As defined in Table 6.1E-2 |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-SubchannelSize-r16 | As defined in Table 6.1E-2 |  |  |
| sl-StartRB-Subchannel-r16 | As defined in Table 6.1E-2 |  |  |
| sl-NumSubchannel-r16 | As defined in Table 6.1E-2 |  |  |
| sl-PowerControl-r16 SEQUENCE { |  |  |  |
| sl-MaxTransPower-r16 | 24 |  |  |
| sl-Alpha-PSSCH-PSCCH-r16 | Not present |  |  |
| dl-Alpha-PSSCH-PSCCH-r16 | Not present |  |  |
| sl-P0-PSSCH-PSCCH-r16 | Not present |  |  |
| dl-P0-PSSCH-PSCCH-r16 | Not present |  |  |
| dl-Alpha-PSFCH-r16 | Not present |  |  |
| dl-P0-PSFCH-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

6.5E.2.2.2.5	Test requirement

The measured UE mean power in the channel bandwidth, derived in step 4 shall fulfil requirements in clause 6.2E.2.2.5 as appropriate, and the power of any UE emission on NR Uu carrier or V2X sidelink carrier derived in step 5 shall fulfil requirements in Table 6.5E.2.2.2.5-1 for inter-band concurrent operation and in Table 6.5E.2.2.2.5-3 for intra-band concurrent operation.

Table 6.5E.2.2.2.5-1: General NR spectrum emission mask for V2X / inter-band concurrent operation

|  | Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Measurement bandwidth |
| ± 0-1 |  | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT |  |  |  |  |  | 1 % channel bandwidth |
| ± 0-1 |  |  |  |  |  |  |  |  | -24 + TT | -24 + TT | -24 + TT | -24 + TT | -24 + TT | 30 kHz |
| ± 1-5 |  | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | 1 MHz |
| ± 5-6 |  | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT |  |
| ± 6-10 |  | -25 + TT |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 10-15 |  |  | -25 + TT |  |  |  |  |  |  |  |  |  |  |  |
| ± 15-20 |  |  |  | -25 + TT |  |  |  |  |  |  |  |  |  |  |
| ± 20-25 |  |  |  |  | -25 + TT |  |  |  |  |  |  |  |  |  |
| ± 25-30 |  |  |  |  |  | -25 + TT |  |  |  |  |  |  |  |  |
| ± 30-35 |  |  |  |  |  |  | -25 + TT |  |  |  |  |  |  |  |
| ± 35-40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 40-45 |  |  |  |  |  |  |  | -25 + TT |  |  |  |  |  |  |
| ± 45-50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 50-55 |  |  |  |  |  |  |  |  | -25 + TT |  |  |  |  |  |
| ± 55-60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 60-65 |  |  |  |  |  |  |  |  |  | -25 + TT |  |  |  |  |
| ± 65-80 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 80-85 |  |  |  |  |  |  |  |  |  |  | -25 + TT |  |  |  |
| ± 85-90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 90-95 |  |  |  |  |  |  |  |  |  |  |  | -25 + TT |  |  |
| ± 95-100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 100-105 |  |  |  |  |  |  |  |  |  |  |  |  | -25 + TT |  |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel. Note 4: TT for each frequency and channel bandwidth is specified in Table 6.5E.2.2.2.5-2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.5E.2.2.2.5-2: Test Tolerance (inter-band concurrent NR V2X operation)

|  | f ≤ 3.0GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB |

Table 6.5E.2.2.2.5-3: General NR spectrum emission mask for V2X / intra-band concurrent operation

| ΔfOOB (MHz) | Spectrum emission limit(dBm) | MBW(MHz) |
| --- | --- | --- |
| ± 0 - 1 | -13+TT | Min(0.01*BWchannel_CA, 0.4) |
| ± 1 - 5 | -10+TT | 1MHz |
| ± 5 – BWchannel_CA | -13+TT | 1MHz |
| ±BWchannel_CA- BWchannel_CA+5 | -25+TT | 1MHz |
| Note 1: TT for each frequency and channel bandwidth is specified in Table 6.5E.2.2.2.5-2. |  |  |

6.5E.2.3	Additional Spectrum emission mask for V2X

6.5E.2.3.0	Minimum conformance requirements

6.5E.2.3.0.1	Requirements for network signalling value "NS_33"

The additional spectrum mask in Table 6.5E.2.3.0.1-1 applies for NR V2X UE within 5 855 MHz to 5 950 MHz according to ETSI EN 302 571. Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_33" is indicated in the cell or pre-configured radio parameters, the power of any V2X UE emission shall not exceed the levels specified in Table 6.5E.2.3.0.1-1.

Table 6.5E.2.3.0.1-1: Additional spectrum mask requirements for 10MHz channel bandwidth

| Spectrum emission limit (dBm EIRP)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [-13-12∆fOOBMHz] | 100 kHz |
| 0.5-5 | [-19-169∆fOOBMHz-0.5] | 100 kHz |
| 5-10 | [-27-2∆fOOBMHz-5.0] | 100 kHz |

NOTE 1:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

NOTE 2:	Additional SEM for NR V2X overrides any other requirements in frequency range 5855-5950MHz.

NOTE 3:	The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I in [11].

6.5E.2.3.0.2	Requirements for network signalling value "NS_52"

The additional spectrum mask in Table 6.5E.2.3.0.2-1 applies for NR V2X UE within 5 765 MHz to 6 005 MHz according to FCC regulation. Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_52" is indicated in the cell or pre-configured radio parameters, the power of any V2X UE emission shall not exceed the levels specified in Table 6.5E.2.3.0.2-1.

Table 6.5E.2.3.0.2-1: Additional spectrum mask requirements for 40MHz channel bandwidth (fc = 5885MHz)

| ΔfOOB (MHz) | Emission Limit (dBm) | Measurement Bandwidth |
| --- | --- | --- |
| 0-2 | -32 | 100kHz |
| 2-10 | -36 | 100kHz |
| 10-20 | -38 | 100kHz |
| 20-40 | -43 | 100kHz |
| 40-100 | -50 | 100kHz |

NOTE:	The ASE requirements for NS_52 will not be verified until the corresponding regulation release a formal rule for C-V2X emission limits.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5E.2.3.

6.5E.2.3.1	Additional Spectrum emission mask for V2X / non-concurrent operation

6.5E.2.3.1.1	Test purpose

To verify that the power of NR V2X UE emission shall not exceed specified level for the specified channel bandwidth under the deployment scenarios where additional requirements are specified.

6.5E.2.3.1.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support NR V2X sidelink communication with non-concurrent operation.

6.5E.2.3.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5E.2.3.0.

6.5E.2.3.1.3.1	Void

6.5E.2.3.1.3.2	Void

6.5E.2.3.1.4	Test description

6.5E.2.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.2E.1-1 and Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in clause 6.2E.3.1.4.1. The details of the V2X reference measurement channels (RMCs) are specified in Annexe A.7.5 and the GNSS configuration in TS 38.508-1 [5] subclause 4.11.

Table 6.5E.2.3.1.4.1-1: Void

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.9.1 for TE diagram and section A.3.2.7 for UE diagram.

2.	The parameter settings for the NR sidelink transmission over PC5 are pre-configured according to TS 38.508-1 [5] subclause 4.10. Message content exceptions are defined in clause 6.5E.2.3.1.4.3.

3.	The V2X Reference Measurement Channel is set according to clause 6.2E.3.1.4.1.

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS 38.508-1 [5] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0.

6.	Void

6.5E.2.3.1.4.2	Test procedure

Subtest 1: PSCCH/PSSCH

1.	Ensure the UE is in state Out_of_Coverage with generic procedure parameters Sidelink On, Test Loop Function On with UE test loop mode E closed for Transmit Mode according to TS 38.508-1 [5] clause 4.5.

2.	The UE starts to perform the NR sidelink communication according to SL-PreconfigurationNR. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the NR sidelink RMC.

3.	Measure the mean power of the UE in the channel bandwidth according to the test configuration from clause 6.2E.3.1.4.1, which shall meet the requirements described in clause 6.2E.3.1.5. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms) excluding guard symbols.

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.5E.2.3.1.5.1-1 or Table 6.5E.2.3.1.5.2-1 and using a rms detector. If the sweep count is higher than one, the trace mode shall be average. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

Subtest 2: PSFCH

1.	Ensure the UE is in state Out_of_Coverage with generic procedure parameters Sidelink On, Cast type Unicast, Test Loop Function On with UE test loop mode E closed for Receive Mode according to TS 38.508-1 [5] clause 4.5.

2.	The UE starts to perform the NR sidelink reception according to SL-PreconfigurationNR.

3.	The UE’s PSFCH transmission occasion is on slot n according to clause 6.2E.3.1.4.1. SS transmits PSSCH on combination of slot and subchannel as below:

a) Test ID 1: slot n-6, Lowest sub-channel

b) Test ID 2, 4, 5 and 6: slot n-6, Lowest sub-channel and slot n-3, Highest sub-channel

c) Test ID 3: slot n-3, Highest sub-channel

4.	Measure the mean power of the UE on slot n in the channel bandwidth according to the test configuration from clause 6.2E.3.1.4.1, which shall meet the requirements described in clause 6.2E.3.1.5. The cumulative measurement period is at least 1ms.

5.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.5E.2.3.1.5.1-2 or Table 6.5E.2.3.1.5.2-2 and using a rms detector. If the sweep count is higher than one, the trace mode shall be average. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

Subtest 3: S-SSB

1.	Ensure the UE is in state Out_of_Coverage with generic procedure parameters Sidelink On according to TS 38.508-1 [5] clause 4.5. The UE is synchronized to GNSS,

2.	The UE transmits PSBCH according SL-PreconfigurationNR.

3.	Measure the mean power of the S-SSB in the channel bandwidth according to the test configuration from clause 6.2E.3.1.4.1, which shall meet the requirements described in clause 6.2E.3.1.5. The cumulative measurement period is at least 1ms.

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.5E.2.3.1.5.1-3 or Table 6.5E.2.3.1.5.2-3 and using a rms detector. If the sweep count is higher than one, the trace mode shall be average. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.5E.2.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.10 with following exceptions.

6.5E.2.3.1.4.3.1	Message contents exceptions (network signalling value “NS_33”)

For "NS_33" see A-MPR test case in clause 6.2E.3.1.4.3.1.

Table 6.5E.2.3.1.4.3.1-1: Void

6.5E.2.3.1.4.3.2	Message contents exceptions (network signalling value “NS_52”)

Table 6.5E.2.3.1.4.3.2-1: Network signalling value "NS_52"

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_52) |  |  |

6.5E.2.3.1.5	Test requirement

Table 6.5E.2.3.1.5-1: Test Tolerance (Additional spectrum emission mask)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.5 dB | 1.8 dB |

6.5E.2.3.1.5.1	Requirements for network signalling value "NS_33"

When "NS_33" is indicated in the cell or pre-configured radio parameters, the power of any NR V2X UE emission shall not exceed the levels specified in Table 6.5E.2.3.1.5.1-1.

Table 6.5E.2.3.1.5.1-1: Additional spectrum mask requirements for 10MHz channel bandwidth

| Spectrum emission limit (dBm EIRP)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [-13-12∆fOOBMHz]+TT | 100 kHz |
| 0.5-5 | [-19-169∆fOOBMHz-0.5]+TT | 100 kHz |
| 5-10 | [-27-2∆fOOBMHz-5.0]+TT | 100 kHz |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.5E.2.3.1.5-1. |  |  |

6.5E.2.3.1.5.2	Requirements for network signalling value "NS_52"

When "NS_52" is indicated in the cell, the power of any NR V2X UE emission shall fulfil requirements in Table 6.5E.2.3.1.5.2-1.

Table 6.5E.2.3.1.5.2-1: Additional spectrum mask requirements for 40MHz channel bandwidth (fc = 5885MHz)

| ΔfOOB (MHz) | Emission Limit (dBm) | Measurement Bandwidth |
| --- | --- | --- |
| 0-2 | -32+TT | 100kHz |
| 2-10 | -36+TT | 100kHz |
| 10-20 | -38+TT | 100kHz |
| 20-40 | -43+TT | 100kHz |
| 40-100 | -50+TT | 100kHz |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.5E.2.3.1.5-1. |  |  |

6.5E.2.3.1D	Additional Spectrum emission mask for V2X / non-concurrent operation / SL-MIMO

6.5E.2.3.1D.1	Test purpose

To verify that the power of NR V2X UE emission shall not exceed specified level for the specified channel bandwidth under the deployment scenarios where additional requirements are specified.

6.5E.2.3.1D.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support NR V2X sidelink communication and SL-MIMO.

6.5E.2.3.1D.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5E.2.1 and 6.5E.2.3.0.

6.5E.2.3.1D.4	Test description

Same test description as in 6.5E.2.3.1.4 for PSSCH/PSCCH subtest with following exceptions:

For initial conditions:

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.9.2 for TE diagram and section A.3.2.7 for UE diagram.

For test procedures:

Subtest 1 are replaced by:

1.	Ensure the UE is in state 4-A as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), Cast Type (Unicast), GNSS Sync (On) and Transmit Mode with SL-MIMO. The UE is synchronized to GNSS,

2.	The UE starts to perform the NR sidelink communication according to SL-PreconfigurationNR with 2-layer MIMO codebook TPMI 0. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the NR sidelink RMC.

3.	Measure the sum of mean power of the UE at each transmit antenna connector in the channel bandwidth according to the test configuration from clause 6.2E.3.1D.4.1, which shall meet the requirements described in clause 6.2E.3.1D.5. The period of measurement shall be at least continuous duration of one active sub-frame (1ms) excluding guard symbols.

4.	Measure the power of the transmitted signal at each transmit antenna connector with a measurement filter of bandwidths according to Table 6.5E.2.3.1D.5.1-1 or Table 6.5E.2.3.1D.5.2-1 and using a rms detector. If the sweep count is higher than one, the trace mode shall be average. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power at each transmit antenna connector shall be recorded for each step. The measurement period shall capture the active TSs.

6.5E.2.3.1D.4.1	Void

6.5E.2.3.1D.4.2	Void

6.5E.2.3.1D.4.3	Void

6.5E.2.3.1D.5	Test requirement

Table 6.5E.2.3.1D.5-1: Test Tolerance

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.5 dB | 1.8 dB |

6.5E.2.3.1D.5.1	Requirements for network signalling value "NS_33"

When "NS_33" is indicated in the cell or pre-configured radio parameter, the power of any NR V2X UE emission at each antenna connector shall not exceed the levels specified in Table 6.5E.2.3.1D.5.1-1.

Table 6.5E.2.3.1D.5.1-1: Additional spectrum mask requirements for 10MHz channel bandwidth

| Spectrum emission limit (dBm EIRP)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [-13-12∆fOOBMHz]+TT | 100 kHz |
| 0.5-5 | [-19-169∆fOOBMHz-0.5]+TT | 100 kHz |
| 5-10 | [-27-2∆fOOBMHz-5.0]+TT | 100 kHz |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.5E.2.3.1D.5-1. |  |  |

6.5E.2.3.1D.5.2	Requirements for network signalling value "NS_52"

When "NS_52" is indicated in the cell, the power of any NR V2X UE emission at each antenna connector shall fulfil requirements in Table 6.5E.2.3.1D.5.2-1.

Table 6.5E.2.3.1D.5.2-1: Additional spectrum mask requirements for 40MHz channel bandwidth (fc = 5885MHz)

| ΔfOOB (MHz) | Emission Limit (dBm) | Measurement Bandwidth |
| --- | --- | --- |
| 0-2 | -32+TT | 100kHz |
| 2-10 | -36+TT | 100kHz |
| 10-20 | -38+TT | 100kHz |
| 20-40 | -43+TT | 100kHz |
| 40-100 | -50+TT | 100kHz |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.5E.2.3.1D.5-1. |  |  |

6.5E.2.3.2	Additional Spectrum emission mask for V2X / concurrent operation

6.5E.2.3.2.1	Test purpose

Same test purpose as in 6.5E.2.3.1 for concurrent operation.

6.5E.2.3.2.2	Test applicability

No test case details are specified for inter-band concurrent operation. The additional spectrum emission requirements for concurrent operation apply and are tested in clauses 6.5.2.3 for NR Uu operation and 6.5E.2.3.1 for NR sidelink option.

No test case details are specified for intra-band concurrent operation since no additional emission requirement is specified in this release.

6.5E.2.4	Adjacent channel leakage ratio for V2X

6.5E.2.4.0	Minimum conformance requirements

6.5E.2.4.0.1	General

Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency.

For NR V2X UE, the existing ACLR requirement for NR uplink transmission in clause 6.5.2.4.3 are applied for NR V2X UE for NR V2X operating bands in 5.2E.1-1.

For NR V2X UE with two transmit antenna connectors, the requirements specified for single carrier shall apply to each transmit antenna connector.

If V2X UE transmits on one antenna connector at a time, the requirements specified for single carrier shall apply to the active antenna connector.

6.5E.2.4.0.2	ACLR for V2X concurrent operation

For the inter-band concurrent NR V2X operation, the ACLR requirement specified in clause 6.5.2.4.3 shall apply for the uplink in licensed band and the ACLR requirement specified in clause 6.5E.2.4.0.1 shall apply for the sidelink in licensed band or Band n47.

For the intra-band NR V2X operation with bandwidth classes B where Uu and SL transmission overlaps in time, the ACLR requirement specified in clause 6.5A.2.4.1 shall apply for the both uplink and sidelink transmission in licensed band.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5E.2.4.

6.5E.2.4.1	Adjacent channel leakage ratio for V2X / non-concurrent operation

The minimum conformance requirements are defined in clause 6.5E.2.4.0.

6.5E.2.4.1.1	Test purpose

To verify that V2X UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage Power Ratio (ACLR) when UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions.

6.5E.2.4.1.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support NR V2X sidelink communication with non-concurrent operation.

6.5E.2.4.1.3	Minimum conformance requirements

Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency.

For NR V2X UE, the existing ACLR requirement for NR uplink transmission in clause 6.5.2.4 are applied for NR V2X UE for NR V2X operating bands in 5.2E.1-1.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5E.2.4.

6.5E.2.4.1.4	Test description

6.5E.2.4.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.2E.1-1 and Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in the test configuration tables in clause 6.2E.2.1.4. The details of the V2X reference measurement channels (RMCs) are specified in Annexe A.7.5 and the GNSS configuration in TS 38.508-1 [5] subclause 4.11.

Table 6.5E.2.4.1.4.1-1: Void

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.9.1 for TE diagram and section A.3.2.7 for UE diagram.

2.	The parameter settings for the NR sidelink transmission over PC5 are pre-configured according to TS 38.508-1 [5] subclause 4.10. Message content exceptions are defined in clause 6.5E.2.4.1.4.3.

3.	The V2X Reference Measurement Channel is set according to the test configuration tables in clause 6.2E.2.1.4.1.

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS 38.508-1 [5] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0.

6.5E.2.4.1.4.2	Test procedure

Subtest 1: PSCCH/PSSCH

1.	Ensure the UE is in state Out_of_Coverage with generic procedure parameters Sidelink On, Test Loop Function On with UE test loop mode E closed for Transmit Mode according to TS 38.508-1 [5] clause 4.5.

2.	The UE starts to perform the NR sidelink communication according to SL-PreconfigurationNR. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the NR sidelink RMC.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration from clause 6.2E.2.1.4.1, which shall meet the requirements described in clause 6.2E.2.1.5. The period of measurement shall be at least continuous duration of one active sub-frame (1ms) excluding guard symbols.

4.	Measure the rectangular filtered mean power for assigned NR V2X sidelink using a rms detector. If the sweep count is higher than one, the trace mode shall be average.

5.	Measure the rectangular filtered mean power of the first NR adjacent channel on both lower and upper side of the assigned NR V2X sidelink channel using a rms detector, respectively. If the sweep count is higher than one, the trace mode shall be average.

6.	Calculate the ratios of the power between the values measured in step 4 over step 5 for lower and upper NRACLR.

Subtest 2: PSFCH

1.	Ensure the UE is in state Out_of_Coverage with generic procedure parameters Sidelink On, Cast type Unicast, Test Loop Function On with UE test loop mode E closed for Receive Mode according to TS 38.508-1 [5] clause 4.5.

2.	The UE starts to perform the NR sidelink reception according to SL-PreconfigurationNR.

3.	The UE’s PSFCH transmission occasion is on slot n according to clause 6.2E.2.1.4.1. SS transmits PSSCH on combination of slot and subchannel as below:

a)	Test ID 1: slot n-6, Lowest sub-channel

b)	Test ID 2: slot n-3, Highest sub-channel

c)	Test ID 3: slot n-6 and n-5, Lowest sub-channel

d)	Test ID 4: slot n-4 and n-3, Highest sub-channel

e)	Test ID 5: slot n-6, Highest sub-channel and slot n-3, Highest sub-channel

4.	Measure the mean power of the UE on slot n in the channel bandwidth according to the test configuration from clause 6.2E.2.1.4.1, which shall meet the requirements described in clause 6.2E.2.1.5. The period of measurement is one active sub-frame (1ms) excluding guard symbols.

5.	Measure the rectangular filtered mean power for assigned NR V2X sidelink using a rms detector. If the sweep count is higher than one, the trace mode shall be average.

6.	Measure the rectangular filtered mean power of the first NR adjacent channel on both lower and upper side of the assigned NR V2X sidelink channel using a rms detector, respectively. If the sweep count is higher than one, the trace mode shall be average.

7.	Calculate the ratios of the power between the values measured in step 5 over step 6 for lower and upper NRACLR.

Subtest 3: S-SSB

1.	Ensure the UE is in state Out_of_Coverage with generic procedure parameters Sidelink On according to TS 38.508-1 [5] clause 4.5. The UE is synchronized to GNSS,

2.	The UE transmits PSBCH according SL-PreconfigurationNR.

3.	Measure the mean power of the S-SSB in the channel bandwidth according to the test configuration from clause 6.2E.2.1.4.1, which shall meet the requirements described in clause 6.2E.2.1.5. The period of measurement is one active sub-frame (1ms) excluding guard symbols.

4.	Measure the rectangular filtered mean power for assigned NR V2X sidelink using a rms detector. If the sweep count is higher than one, the trace mode shall be average.

5.	Measure the rectangular filtered mean power of the first NR adjacent channel on both lower and upper side of the assigned NR V2X sidelink channel using a rms detector, respectively. If the sweep count is higher than one, the trace mode shall be average.

6.	Calculate the ratios of the power between the values measured in step 4 over step 5 for lower and upper NRACLR.

6.5E.2.4.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.10.

6.5E.2.4.1.5	Test requirement

The measured mean power of V2X UE in the channel bandwidth, derived in step 3 of Subtest 1, step 4 of Subtest 2 and step 3 of Subtest 3, shall fulfil requirements in clause 6.2E.2.1.5, and if the measured adjacent channel power is greater than –50 dBm then the measured NR ACLR, derived in step 6 of Subtest 1, step 7 of Subtest 2 and step 6 of Subtest 3, shall be higher than the limits in Table 6.5E.2.4.1.5-2.

Table 6.5E.2.4.1.5-1: Measurement bandwidth

| NR channel bandwidth / NR ACLR measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| NR ACLR measurement bandwidth | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 28.815 | 38.895 | 48.615 | 58.35 | 78.15 | 88.23 | 98.31 |

Table 6.5E.2.4.1.5-2: NR ACLR requirement for V2X / non-concurrent operation

|  | Power class 1 | Power class 2 | Power class 3 |
| --- | --- | --- | --- |
| NR ACLR |  |  | 30 - TT dB |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.5E.2.4.1.5-3. |  |  |  |

Table 6.5E.2.4.1.5-3: Test Tolerance

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 100MHz | 0.8 dB | 0.8 dB | 0.8 dB |

6.5E.2.4.1D	Adjacent channel leakage ratio for V2X / non-concurrent operation / SL-MIMO

6.5E.2.4.1D.1	Test purpose

To verify that V2X UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage Power Ratio (ACLR) when UE is configured for NR V2X sidelink MIMO transmissions non-concurrent with NR uplink transmissions.

6.5E.2.4.1D.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support NR V2X sidelink communication and SL-MIMO.

6.5E.2.4.1D.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5E.1 and 6.5E.2.4.0.

6.5E.2.4.1D.4	Test description

Same test description as in 6.5E.2.4.1.4 for PSSCH/PSCCH subtest with following exceptions:

For initial conditions:

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.9.2 for TE diagram and section A.3.2.7 for UE diagram.

For test procedures:

Subtest 1 are replaced by:

1.	Ensure the UE is in state 4-A as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), Cast Type (Unicast), GNSS Sync (On) and Transmit Mode with SL-MIMO. The UE is synchronized to GNSS,

2.	The UE starts to perform the NR sidelink communication according to SL-PreconfigurationNR with 2-layer MIMO codebook TPMI 0. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the NR sidelink RMC.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration from clause 6.2E.2.1.4.1, which shall meet the requirements described in clause 6.2E.2.1D.5. The period of measurement shall be at least continuous duration of one active sub-frame (1ms) excluding guard symbols.

4.	Measure the rectangular filtered mean power for assigned NR V2X sidelink at each antenna connector using a rms detector. If the sweep count is higher than one, the trace mode shall be average.

5.	Measure the rectangular filtered mean power of the first NR adjacent channel on both lower and upper side of the assigned NR V2X sidelink channel at each antenna connector using a rms detector, respectively. If the sweep count is higher than one, the trace mode shall be average.

6.	Calculate the ratios of the power between the values measured in step 4 over step 5 for lower and upper NRACLR for each antenna connector.

6.5E.2.4.1D.4.1	Void

6.5E.2.4.1D.4.2	Void

6.5E.2.4.1D.4.3	Void

6.5E.2.4.1D.5	Test requirement

The measured sum of mean power of V2X UE at both antenna connector in the channel bandwidth, derived in step 2, shall fulfil requirements in clause 6.2E.2.1D.5, and if the measured adjacent channel power is greater than –50 dBm then the measured NR ACLR for each antenna connector, derived in step 5, shall be higher than the limits in Table 6.5E.2.4.1D.5-2.

Table 6.5E.2.4.1D.5-1: Measurement bandwidth

| NR channel bandwidth / NR ACLR measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| NR ACLR measurement bandwidth | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 28.815 | 38.895 | 48.615 | 58.35 | 78.15 | 88.23 | 98.31 |

Table 6.5E.2.4.1D.5-2: NR ACLR requirement for V2X / non-concurrent operation / SL-MIMO

|  | Power class 1 | Power class 2 | Power class 3 |
| --- | --- | --- | --- |
| NR ACLR |  |  | 30 - TT dB |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.5E.2.4.1D.5-3. |  |  |  |

Table 6.5E.2.4.1D.5-3: Test Tolerance

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 100MHz | 0.8 dB | 0.8 dB | 0.8 dB |

6.5E.2.4.2	Adjacent channel leakage for V2X / concurrent operation

6.5E.2.4.2.1	Test purpose

Same test purpose as in 6.5E.2.4.1.1 for concurrent operation.

6.5E.2.4.2.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support NR V2X sidelink communication.

6.5E.2.4.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5E.2.4.0.

6.5E.2.4.2.4	Test description

6.5E.2.4.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.2E.2-1 and Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in clause 6.2E.2.2.4.1. The details of the V2X reference measurement channels (RMCs) are specified in Annexes A.7.5. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.9.3 for TE diagram and clause A.3.2.7 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] clause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The V2X Reference Measurement Channel and NR UL Reference Measurement Channel are set according to clause 6.2E.2.2.4.1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameter Connectivity NR, Connected without release on, Test Mode On and Sidelink On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5E.2.4.2.4.3.

6.5E.2.4.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to clause 6.2E.2.2.4.1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	SS sends sidelink scheduling information for each SL-HARQ process via PDCCH DCI format 3_0 for C_RNTI to schedule the Sidelink RMC according to clause 6.2E.2.2.4.1. UE is configured to transmit at Pcmax on the sidelink carrier. Since the UE has no payload and no loopback data to send the UE sends MAC padding bits on the Sidelink RMC.

4.	Measure the sum of the mean power of the UE at NR Uu carrier and V2X sidelink carrier in the channel bandwidth according to the test configuration from clause 6.2E.2.2.4.1, which shall meet the requirements described in clause 6.2E.2.2.5. The period of measurement shall be at least continuous duration of one active sub-frame (1ms) excluding guard symbols.

5.	For inter-band concurrent operation, execute 5a) to 5c):

5a)	Measure the rectangular filtered mean power for assigned NR Uu carrier and V2X sidelink carrier respectively using a rms detector. If the sweep count is higher than one, the trace mode shall be average.

5b)	Measure the rectangular filtered mean power of the first NR adjacent channel on both lower and upper side of the assigned NR Uu carrier and V2X sidelink carrier respectively using a rms detector, respectively. If the sweep count is higher than one, the trace mode shall be average.

5c)	Calculate the ratios of the power between the values measured in step 5 over step 6 for lower and upper NRACLR for NR Uu carrier and V2X sidelink carrier respectively.

6.	For intra-band concurrent operation, execute 6a) to 6c):

6a)	Measure the rectangular filtered mean power for assigned aggregated channel bandwidth of NR Uu carrier and V2X sidelink carrier using a rms detector. If the sweep count is higher than one, the trace mode shall be average.

6b)	Measure the rectangular filtered mean power of the first NR adjacent channel on both lower and upper side of the assigned aggregated channel bandwidth using a rms detector, respectively. If the sweep count is higher than one, the trace mode shall be average.

6c)	Calculate the ratios of the power between the values measured in step 6b) over step 6a) for lower and upper NRACLR respectively.

6.5E.2.4.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] clause 4.6.

Table 6.5E.2.4.2.4.3-1: SL-ResourcePool

| Derivation Path: TS 38.508-1 [5], Table 4.6.6-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |
| sl-PSCCH-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sl-TimeResourcePSCCH-r16 | As defined in Table 6.1E-2 |  |  |
| sl-FreqResourcePSCCH-r16 | As defined in Table 6.1E-2 |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-SubchannelSize-r16 | As defined in Table 6.1E-2 |  |  |
| sl-StartRB-Subchannel-r16 | As defined in Table 6.1E-2 |  |  |
| sl-NumSubchannel-r16 | As defined in Table 6.1E-2 |  |  |
| sl-PowerControl-r16 SEQUENCE { |  |  |  |
| sl-MaxTransPower-r16 | 24 |  |  |
| sl-Alpha-PSSCH-PSCCH-r16 | Not present |  |  |
| dl-Alpha-PSSCH-PSCCH-r16 | Not present |  |  |
| sl-P0-PSSCH-PSCCH-r16 | Not present |  |  |
| dl-P0-PSSCH-PSCCH-r16 | Not present |  |  |
| dl-Alpha-PSFCH-r16 | Not present |  |  |
| dl-P0-PSFCH-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

6.5E.2.4.2.5	Test requirement

The measured mean power of V2X UE in the channel bandwidth, derived in step 4 shall fulfil requirements in clause 6.2E.2.2.5, and if the measured adjacent channel power is greater than –50 dBm then the measured NR ACLR, derived in step 7, shall be higher than the limits in Table 6.5E.2.4.2.5-2 for inter-band concurrent operation, and in Table 6.5E.2.4.2.5-4 and in Table 6.5E.2.4.2.5-5 for intra-band concurrent operation.

Table 6.5E.2.4.2.5-1: Measurement bandwidth for inter-band concurrent operation

| NR channel bandwidth / NR ACLR measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| NR ACLR measurement bandwidth | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 28.815 | 38.895 | 48.615 | 58.35 | 78.15 | 88.23 | 98.31 |

Table 6.5E.2.4.2.5-2: NR ACLR requirement for V2X / inter-band concurrent operation

|  | Power class 1 | Power class 2 | Power class 3 |
| --- | --- | --- | --- |
| NR ACLR |  |  | 30 - TT dB |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.5E.2.4.2.5-3. |  |  |  |

Table 6.5E.2.4.2.5-3: Test Tolerance

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 100MHz | 0.8 dB | 0.8 dB | 0.8 dB |

Table 6.5E.2.4.2.5-4: Requirements for intra-band concurrent operation (Power class 3)

|  | ACLR / Measurement bandwidth |
| --- | --- |
| CA ACLR | 30 - TT dB |
| CA Measurement bandwidth (NOTE 1) | Nominal channel space+MBWACLR,low/2+ MBWACLR,high/2 |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA / - BWChannel_CA |
| Difference between ACLR MBW center and Fc,low | MBWshift= (MBWACLR_CA-MBWACLR,low)/2 |
| NOTE 1: MBWACLR,low and MBWACLR,high are the single-channel ACLR measurement bandwidths specified for channel bandwidths BWchannel(low) and BWchannel(high) in 6.5.2.4.1.3, respectively. |  |

Table 6.5E.2.4.2.5-5: Requirements for intra-band concurrent operation (Power class 2)

|  | ACLR / Measurement bandwidth |
| --- | --- |
| CA ACLR | 31 - TT dB |
| CA Measurement bandwidth (NOTE 1) | Nominal channel space+MBWACLR,low/2+ MBWACLR,high/2 |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA / - BWChannel_CA |
| Difference between ACLR MBW center and Fc,low | MBWshift= (MBWACLR_CA-MBWACLR,low)/2 |
| NOTE 1: MBWACLR,low and MBWACLR,high are the single-channel ACLR measurement bandwidths specified for channel bandwidths BWchannel(low) and BWchannel(high) in 6.5.2.4.1.3, respectively. |  |

< End of Changes >