3GPP TSG-RAN5 Meeting #109	R5-255753

# Dallas, US, 17th – 21st November, 2025

Agenda item:	5.4.20

Source:	Keysight Technologies UK Ltd

Title:	On Out-of-Band Blocking test optimization

Document for:	Discussion and Endorsement

# Introduction

This contribution aims to continue the analysis started in [1] under AP#107.21 to evaluate how to optimize testing time in in out-of-band blocking (OBB) test case.

# Discussion

The analysis made in [1] did a counter analysis of the analysis and proposals made in [4] to optimize the OBB testing time. In summary, it was concluded that the actual test time duration is not that long and that skipping interferer frequency points will be against the minimum requirement as defined by RAN4 core specs where the interferer frequency step to use is explicitly defined. Therefore, it was proposed in [1] not to deviate current test definition from the minimum requirements.

During same RAN5#108 meeting, 2 more papers were presented, [2] and [3]. Paper [2] is a continuation of proposals made in [4] that triggered this action point in RAN5#107. Paper [3] is more in line to analysis made in  [1] and so proposing not to deviate current test definition.

During discussions of [1], [2] and [3], we made the suggestion that instead of modifying the test definition, to evaluate if execution of certain bands could be skipped if execution of other bands has already been done. In this paper we are going to explore the feasibility of this alternative based on an analysis of the DL frequencies of all the different bands.

This analysis will focus on single carrier operation in licensed bands. That leaves outside SDL, V2X and NR-U bands.

For the bands under consideration, we have observed 2 candidate types of bands to leverage OOB executions:

- Bands with identical or almost identical DL frequency range.

- Identical

- Almost identical.

- Bands with DL embedded in the DL frequency range of another band.

- If remarkable difference in the band size, otherwise the bands could be considered of Type A.2.

If bands are identical or almost identical in terms of DL frequency range and they are supported by a given device, it is reasonable to assume RX performance of the device in terms of filtering and rejection of OOB signals should be very similar if not the same for both bands. Of course, in case of not identical DL frequency range, a criteria should be defined to determine when 2 bands can be considered almost identical.

For these Type A bands, if eventually agreed as candidate to leverage OOB test execution, it should not matter which band has been executed.

However, for bands of Type B as per definition above, if eventually agreed as candidate to leverage OOB testing execution, it should be determined whether:

- It should not matter which band has been executed to skip execution in the other(s).

- Or, only execution of embedded band(s) can be skipped if larger band has been executed,

- Or, a larger band can be skipped if one of the embedded band has been tested.

In the following tables we have listed groups of bands organized as per definitions above.

| Band | DL Low (MHz) | DL High (MHz) | Duplex Mode |
| --- | --- | --- | --- |
| TDD: n51, n76 (SDL), FDD: n91, n93 | 1427 | 1432 | TDD / FDD |
| TDD:n50, n75 (SDL), FDD: n92, n94, n109 | 1432 | 1517 | TDD / FDD |
| n65, n66 | 2110 | 2200 | FDD |

Table 2-1. Groups of bands of Type A.1: identical DL frequency range.

| Band | DL Low (MHz) | DL High (MHz) | Duplex Mode |
| --- | --- | --- | --- |
| n72 | 461 | 466 | FDD |
| n31 | 462.5 | 467.5 | FDD |
| n105 | 612 | 652 | FDD |
| n71 | 617 | 652 | FDD |
| n85 | 728 | 746 | FDD |
| n12 | 729 | 746 | FDD |
| n26 | 859 | 894 | FDD |
| n5 | 869 | 894 | FDD |

Table 2-2. Groups of bands of Type A.2: almost identical DL frequency range.

| Band | DL Low (MHz) | DL High (MHz) | Duplex Mode |
| --- | --- | --- | --- |
| n8 | 925 | 960.0 | FDD |
| n106 | 935 | 940.0 | FDD |
| n39 | 1880 | 1920.0 | TDD |
| n101 | 1900 | 1910.0 | TDD |
| n40 | 2300 | 2400.0 | TDD |
| n30 | 2350 | 2360.0 | FDD |
| n41 | 2496 | 2690.0 | TDD |
| n38 | 2570 | 2620.0 | TDD |
| n77 | 3300 | 4200.0 | TDD |
| n78 | 3300 | 3800.0 | TDD |

Table 2-3. Groups of bands of Type B: embedded DL frequency range.

With previous information and analysis, we expect RAN5 can better discuss and decide whether OOB testing for certain bands can be leveraged for other bands.

Proposal 1. If there is still interest in optimizing OOB testing tine, RAN5 to further evaluate the alternatives presented in this document regarding leveraging testing among certain bands based on similarities in their DL Frequency range.

# Conclusion

The following observations and proposals were made in this contribution.

Observation 1. Based on the DL frequency range, bands can be set in groups of the following types:

- Type A.1: identical DL frequency range.

- Type A.2: almost identical DL frequency range. Criteria to be considered of Type A.2 is FFS.

- Type B: embedded DL frequency range.

Observation 2. OOB test execution for a band part of a group of bands of Type A.1 [or Type A.2] can be leveraged for the rest of the bands of the group.

Observation 3. If eventually groups of bands of Type B are eligible for leveraging OOB testing, the type of leveraging should be selected between the following alternatives:

- Alternative 1. OOB execution of any band of the group can be leveraged for the rest of the bands in the group.

- Alternative 2. Only execution of embedded band(s) can be skipped if wider band has been executed.

- Alternative 3. Only a wider band can be skipped if (one of) the embedded band(s) has been tested.

Proposal 1. If there is still interest in optimizing OOB testing tine, RAN5 to further evaluate the alternatives presented in this document regarding leveraging testing among certain bands based on similarities in their DL Frequency range.

# References

- R5-254064, “On Out-of-Band Blocking test optimization”, Keysight, RAN5#108

- R5-254633, “Discussion on the optimization of Out-of-band Blocking (OOBB) testing”, Huawei, RAN5#108

- R5-254637, “On out of band blocking”, Ericsson, RAN5#108

- R5-253614, “Discussion on how to reduce the measurement complexity of Out-of-Band Blocking in RAN5”, Huawei, Hisilicon, RAN5#107

- TS 38.521-1, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone”, v19.2.0 (2025-09)