3GPP TSG-RAN5 Meeting #109	R5-256206

DALLAS, USA, Nov 17th – 21st 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3579 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding testing for NS_04 and NS_47 for ASE for UL MIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The testing for NS_04 and NS_47 for additional spurious emissions for UL MIMO is absent. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The test configurations for NS_04 and NS_47 in 6.5D.3_1.3 are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ASE for Band n41 Rel-16 onward cannot be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5D.3_1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

6.5D.3_1.3	Additional spurious emissions for UL MIMO (Rel-16 onward)

6.5D.3_1.3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions under the deployment scenarios where additional requirements are specified.

6.5D.3_1.3.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 16 and forward that support 2-layer codebook based UL MIMO.

6.5D.3_1.3.3 Minimum conformance requirements

For UE supporting UL MIMO, the requirements for Spurious emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products is defined as the sum of the emissions from both UE transmit antenna connectors.

The additional spurious emission requirements specified in clause 6.5.3.3.3 apply. For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements shall be met with the UL MIMO configurations described in clause 6.2D.1. For UEs supporting ULFPTx for UL MIMO, the requirements shall be met with the PUSCH configurations specified in Table 6.2D.1.3-3, based upon UE’s support of uplink full power transmission mode.

The normative reference for this requirement is TS 38.101-1 [2] clauses 6.5D.3 and 6.5.3.3

6.5D.3_1.3.4	Test description

6.5D.3_1.3.4.1	Initial conditions

Same initial conditions as in clause 6.5D.3.3.4.1 together with tables 6.5D.3_1.3.4-1 to 6.5D.3_1.3.4-8.

Table 6.5D.3_1.3.4.1-1: Test Configuration table for NS_05

Same test configuration as listed in Table 6.2D.3.4.1-3 shall be used with the following exceptions:

-	Test SCS shall be: Lowest.

Table 6.5D.3_1.3.4.1-2: Test Configuration table for NS_48

Same test configuration as listed in Table 6.2D.3.4.1-5 shall be used with the following exceptions:

-	Test SCS shall be: Lowest.

Table 6.5D.3_1.3.4.1-3: Test Configuration table for NS_49

Same test configuration as listed in Table 6.2D.3.4.1-6 shall be used with the following exceptions:

-	Test SCS shall be: Lowest.

Table 6.5D.3_1.3.4.1-4: Test Configuration table for NS_46

Same test configuration as listed in Table 6.2D.3.4.1-8 shall be used with the following exceptions:

-	Test SCS shall be: Lowest.

Table 6.5D.3_1.3.4.1-5: Test Configuration table for NS_21

Same test configuration as listed in Table 6.2D.3.4.1-9 shall be used with the following exceptions:

-	Test SCS shall be: Lowest.

Table 6.5D.3_1.3.4.1-6: Test Configuration table for NS_44

Same test configuration as listed in Table 6.2D.3.4.1-10 shall be used with the following exceptions:

-	Test SCS shall be: Lowest.

Table 6.5D.3_1.3.4.1-7: Test Configuration table for NS_27

Same test configuration as listed in Table 6.2D.3.4.1-11 shall be used with the following exceptions:

-	Test SCS shall be: Lowest.

Table 6.5D.3_1.3.4.1-8: Test Configuration table for NS_07 and NS_50

Same test configuration as listed in Table 6.2D.3.4.1-13 shall be used with the following exceptions:

-	Test SCS shall be: Lowest.

Table 6.5D.3_1.3.4.1-9: Test Configuration table for NS_12

Same test configuration as listed in Table 6.2D.3.4.1-18 shall be used with the following exceptions:

-	Test SCS shall be: Lowest.

Table 6.5D.3_1.3.4.1-10: Test Configuration table for NS_13

Same test configuration as listed in Table 6.2D.3.4.1-19 shall be used with the following exceptions:

-	Test SCS shall be: Lowest.

Table 6.5D.3_1.3.4.1-11: Test Configuration table for NS_14

Same test configuration as listed in Table 6.2D.3.4.1-20 shall be used with the following exceptions:

-	Test SCS shall be: Lowest.

Table 6.5D.3_1.3.4.1-12: Test Configuration table for NS_15

Same test configuration as listed in Table 6.2D.3.4.1-21 shall be used with the following exceptions:

-	Test SCS shall be: Lowest.

Table 6.5D.3_1.3.4.1-13: Test Configuration table for NS_18

Same test configuration as listed in Table 6.2D.3.4.1-23 for power class 3 and Table 6.2D.3.4.1-24 for power class 2 shall be used with the following exceptions:

-	Test SCS shall be: Lowest.

Table 6.5D.3_1.3.4.1-14: Test Configuration table for NS_04

Same test configuration as listed in Table 6.2D.3.4.1-1 shall be used with the following exceptions:

-	Test SCS shall be: Lowest.

Table 6.5D.3_1.3.4.1-15: Test Configuration table for NS_47

Same test configuration as listed in Table 6.2D.3.4.1-22 for power class 1.5 and Table 6.2D.3.4.1-23 for power class 2 and power class 3 shall be used with the following exceptions:

-	Test SCS shall be: Lowest.

6.5D.3_1.3.4.2	Test procedure

1	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to tables in clause 6.5D.3_1.3.4.1 as appropriate. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the sum of the mean power at each UE antenna connector in the channel bandwidth of the radio access mode according to the test configurations in clause 6.5D.3_1.3.4.1, which shall meet the requirements described in clauses 6.2D.3.5 as appropriate for each network signalling. The period of measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots and uplink symbols. For TDD, only slots consisting of only UL symbols are under test.

4.	Measure the sum of transmitted power at each UE antenna connector with a measurement filter of bandwidths according to clauses 6.5.3.3.3.1 to 6.5.3.3.3.25 as appropriate. The centre frequency of the filter shall be stepped in contiguous steps according to the same table the measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS. If the sweep count is higher than one, the trace mode shall be average.

5.	Void.

==============End of change==============