3GPP TSG RAN WG5 Meeting #109	R5-256355

Dallas, USA, 17th – 21st November 2025

Agenda Item:	6.6.6.6

Source:	Ericsson

Title:	RFC 4103 and redundancy

Document for:	Endorsement

# 1	Introduction

Test cases for RTT (Real time text) refer to RFC 4103.  For the payload type and particular redundancy there’s some dicussions. This paper provides some RFC information and background.

# 2	Discussion

The RFC 4103 [1] defines a payload type for carrying text conversation session contents in RTP packets. Specific details for Protection against Loss of Data are also described.

According 3.5.  RTP Packet Header:

“Payload Type (PT): The assignment of an RTP payload type is specific

to the RTP profile under which the payload format

is used.  For profiles that use dynamic payload

type number assignment, this payload format can be

identified by the MIME type "text/t140" (see

Section 10).  If redundancy is used per RFC 2198,

another payload type number needs to be provided

for the redundancy format.  The MIME type for

identifying RFC 2198 is available in RFC 4102 [9].”

Observation 2-1:

States “If redundancy is used per RFC 2198,”. Meaning optional.

However, clause 4 Protection against Loss of Data adds additional requirements.

According 4  Protection against Loss of Data:

“Consideration must be devoted to keeping loss of text due to packet

loss within acceptable limits.  (See ITU-T F.703 [17])

The default method that MUST be used, when no other method is

explicitly selected, is redundancy in accordance with RFC 2198 [3].

When this method is used, the original text and two redundant

generations SHOULD be transmitted if the application or end-to-end

conditions do not call for other levels of redundancy to be used.

Forward Error Correction mechanisms, as per RFC 2733 [8], or any

other mechanism with the purpose of increasing the reliability of

text transmission, MAY be used as an alternative or complement to

redundancy.  Text data MAY be sent without additional protection if

end-to-end network conditions allow the text quality requirements,

specified in ITU-T F.703 [17], to be met in all anticipated load

conditions.”

The “Consideration must be devoted to keeping loss of text due to packet loss within acceptable limits” means some action and/or clarification on “Protection against Loss of Data” is mandatory.

Observation 2-2:

For “The default method that MUST be used, when no other method is explicitly selected, is redundancy in accordance with RFC 2198 [3].”. Meaning redundancy could be regarded as mandatory unless any other method is selected.

Proposal 2-1:

Add a capability: For text, the protection against Loss of Data is done with other method than redundancy.

Proposal 2-2:

Align the RTT test cases to conditionally mandate the default method, i.e. redundancy.

# 3	Proposal

It’s proposed to implement the proposals 2-1 and 2-2.

# 4	References

[1] IETF RFC 4103 (2005): "RTP Payload for Text Conversation", G. Hellstrom and P. Jones