3GPP TSG-RAN5 Meeting #109	R5-256406

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1557 | rev | - | Current version: | 18.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to references of 34.229-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is a duplicate reference for TS 34.229-1 in the References section. Also there are a number of incorrect references to TS 34.229-1 where it is referred it with index 35 and 46 instead of 43. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed one of the redundant reference [46] from the reference section and corrected the references to 43. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity in the specification remains |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.5,4.9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document unless the context in which the reference is made suggests a different Release is relevant (information on the applicable release in a particular context can be found in e.g. test case title, description or applicability, message description or content).

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.003: "Numbering, addressing and identification".

[3]	3GPP TS 23.122: "Non-Access-Stratum functions related to Mobile Station (MS) in idle mode".

[4]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[5]	3GPP TS 34.108: "Common Test Environments for User Equipment (UE); Conformance testing".

[6]	3GPP TS 34.109: "Terminal logical test interface; Special conformance testing functions".

[7]	3GPP TS 34.123-1: "User Equipment (UE) conformance specification; Part 1: Protocol conformance specification".

[8]	3GPP TS 34.123-2: "User Equipment (UE) conformance specification; Part 2: Implementation conformance statement (ICS) specification".

[9]	3GPP TS 34.123-3: "User Equipment (UE) conformance specification; Part 3: Abstract test suites (ATSs)".

[10]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[11]	3GPP TS 36.302: "Evolved Universal Terrestrial Radio Access (E-UTRA); Services provided by the physical layer".

[12]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[13]	3GPP TS 36.306: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities".

[14]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification".

[15]	3GPP TS 36.322: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Link Control (RLC) protocol specification".

[16]	3GPP TS 36.323: "Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification".

[17]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol Specification".

[18]	3GPP TS 36.523-1: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification; Part 1: Protocol conformance specification".

[19]	3GPP TS 36.523-2: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification; Part 2: Implementation Conformance Statement (ICS) proforma specification".

[20]	3GPP TS 36.523-3: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification; Part 3: Abstract Test Suites (ATS)".

[21]	3GPP TS 36.521-1: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: conformance testing".

[22]	3GPP TS 36.521-2: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Implementation Conformance Statement (ICS)".

[23]	3GPP TR 24.801: "3GPP System Architecture Evolution; CT WG1 aspects".

[24]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Access Network (E-UTRAN) access".

[25]	3GPP TS 51.010-1: "Mobile Station (MS) conformance specification; Part 1: Conformance specification ".

[26]	ISO/IEC 9646 (all parts): "Information technology - Open Systems Interconnection - Conformance testing methodology and framework".

[27]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[28]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[29]	3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures".

[30]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception".

[31]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[32]	3GPP TS 31.101: "UICC-terminal interface; Physical and logical characteristics".

[33]	3GPP TS 31.102: "Characteristics of the Universal Subscriber Identity Module (USIM) application".

[34]	3GPP TS 36.521-3: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Radio Resource Management conformance testing".

[35]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation".

[36]	3GPP2 TSG-C C.S0038-B v1.0: "Signalling Conformance Specification for High Rate Packet Data Air Interface".

[37]	3GPP2 TSG-C C.S0043-A v1.0: "Signalling Conformance Test Specification for cdma2000 Spread Spectrum Systems".

[38]	3GPP TS 36.509: "Evolved Universal Terrestrial Radio Access (E-UTRA); Special conformance testing functions for User Equipment (UE)"

[39]	3GPP TS 36.133: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management"

[40]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3"

[41]	3GPP TS 23.402: “Architecture enhancements for non-3GPP accesses”

[42]	3GPP2 X.S0057-0-B v1.0 “E-UTRAN - eHRPD Connectivity and Interworking: Core Network Aspects”

[43]	3GPP TS 34.229-1: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 1: Protocol conformance specification ".

[44]	3GPP2 C.S0024-B v3.0: "cdma2000 High Rate Packet Data Air Interface Specification".

[45]	3GPP TS 31.103: "Characteristics of the IP Multimedia Services Identity Module (ISIM) application".

[46]	Void

[47]	3GPP TS 34.229-3: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 3: Abstract Test Suite (ATS)".

[48]	3GPP2 C.S0065-B v2.0: "cdma2000 Application on UICC for Spread Spectrum Systems".

[49]	3GPP2 C.S0005-F v1.0: "Upper Layer (Layer 3) Signalling Standard for cdma2000 Spread Spectrum Systems – Release A, Addendum 2".

[50]	3GPP TS 34.121-1: "User Equipment (UE) conformance specification; Radio transmission and reception (FDD); Part 1: Conformance specification ".

[51]	3GPP TS 34.122: "Terminal conformance specification; Radio transmission and reception (TDD)".

[52]	GSMA PRD IR.92: "IMS Profile for Voice and SMS".

[53]	3GPP TS 24.302: "Access to the 3GPP Evolved Packet Core (EPC)
via non-3GPP access networks; Stage 3".

[54]	Void

[55]	3GPP TS 33.402: "3GPP System Architecture Evolution (SAE);Security aspects of non-3GPP accesses".

[56]	GSMA PRD IR.88: "LTE and EPC Roaming Guidelines".

[57]	IETF RFC 5996: "Internet Key Exchange Protocol Version 2 (IKEv2)".

[58]	3GPP TS 24.334: "Proximity-services (ProSe) User Equipment (UE) to ProSe function protocol aspects; Stage 3".

[59]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2"

[60]	3GPP TS 33.303: "Proximity-based Services (ProSe); Security aspects"

[61]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)"

[62]	3GPP TS 36.212: "Multiplexing and channel coding"

[63]	IETF RFC 4346: "The Transport Layer Security (TLS) Protocol Version 1.1".

[64]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[65]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS);Multimedia Telephony;Media handling and interaction".

[66]	3GPP TS 24.483: "Mission Critical Push To Talk (MCPTT) Management Object (MO)".

[67]	3GPP TS 38.508-1: "5GS; Common test environments for User Equipment (UE) conformance testing"

[68]	3GPP TS 37.571-1: "User Equipment (UE) conformance specification for UE positioning; Part 1: Terminal conformance"

[69]	3GPP TS 37.571-5: "User Equipment (UE) conformance specification for UE positioning; Part 5: Test scenarios and assistance data"

[70]	STANAG 4294: "NATO STANAG 4294. Navstar Global Positioning System (GPS) System Characteristics"

[71]	3GPP TS 38.523-1: "5GS; User Equipment (UE) conformance specification; Part 1: Protocol Conformance Specification"

[72]	3GPP TS 38.133: "New Radio (NR); Requirements for support of radio resource management".

[73]	3GPP TS 36.521-4: "Evolved Universal Terrestrial Radio Access (E-UTRA);User Equipment (UE) conformance specification; Radio transmission and reception; Part 4: Satellite access Radio Frequency (RF) and performance Conformance Testing".

[74]	3GPP TS 36.102: "Evolved Universal Terrestrial Radio Access (E-UTRA);User Equipment (UE) radio transmission and reception for satellite access".

[75]	IEC 60068-2-1: "Environmental testing - Part 2-1: Tests - Test A: Cold"

[76]	IEC 60068-2-2: "Environmental testing - Part 2-2: Tests - Test B: Dry heat"

[77]	3GPP TR 36.905: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Derivation of test points for radio transmission and reception conformance test cases".

SKIPPED TEXT

## 4.5	Generic procedures

This clause describes UE test states which can be used in the initial condition of many test cases defined in TS 36.521-1 [21], TS 36.523-1 [18] and TS 36.523-3 [34] or other procedures defined in this specification. This section also defines a set of procedures to bring the UE into these states.

SKIPPED TEXT

### 4.5A.4	Generic Test Procedure for IMS Emergency call establishment in EUTRA: Normal Service

#### 4.5A.4.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Registered, Idle Mode state (State 2).

#### 4.5A.4.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.4.3	Procedure

The establishment of IMS emergency call is assumed to always be mobile originated.

Table 4.5A.4.3-1: EUTRA/EPS signalling for IMS Emergency Call

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Make the UE attempt an IMS emergency call | - | - |
| 2 | The UE transmits an RRCConnectionRequest message with 'establishmentCause' set to 'emergency'. | --> | RRCConnectionRequest |
| 3 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 4 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 5 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 6 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 7 | The SS configures a new data radio bearer, associated with the default EPS bearer context. The RRCConnectionReconfiguration message is using condition SRB2-DRB(1, 0). The DRB associated with default EPS bearer context obtained during the attach procedure is established | <-- | RRC: RRCConnectionReconfiguration |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer context. | --> | RRC: RRCConnectionReconfigurationComplete |
| 9 | The UE transmits a PDN CONNECTIVITY REQUEST message to request an additional PDN, with 'Request type' set to 'emergency ('0100'B )'. | --> | PDN CONNECTIVITY REQUEST |
| 10 | The SS configures a new data radio bearer, associated with the default EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message. EPS bearer context #2 (QCI 5) according to table 6.6.1-1: Reference default EPS bearer context is used. Note: The APN is set to the test APN defined for emergency bearer services. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST |
| - | EXCEPTION: In parallel to the events described in steps 11 to 15 below, the behaviour in table 4.5A.4.3-2 occurs. (Optional IP address allocation followed by IMS emergency registration and IMS emergency speech call establishment) | - | - |
| 11 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer for emergency call. | --> | RRC: RRCConnectionReconfigurationComplete |
| 12 | The UE transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |
| 13 | The SS configures a new RLC-UM data radio bearer, associated with the dedicated EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message. EPS bearer context #4 (QCI 1) according to table 6.6.2-1: Reference dedicated EPS bearer contexts is used. Note: the same PDN address is applicable because the linked EPS bearer ID refers to the default EBC allocated in step 10 | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| 14 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer for emergency IMS signalling. | --> | RRC: RRCConnectionReconfigurationComplete |
| 15 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |

Table 4.5A.4.3-2: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Step 1 describes behaviour that depends on the UE behaviour. | - | - |
| 1 | If initiated by the UE the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - |
| 2-5 | Steps 1-4 expected sequence defined in annex C.20 of TS 34.229-1 [43]. Emergency registration procedure is performed. | - | - |
| 6-10 | Steps 1-5 defined in annex C.22 of TS 34.229-1 [43]. IMS Emergency call for EPS is established. | - | - |

#### 4.5A.4.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7 with the exceptions below.

Table 4.5A.4.4-1: Message ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST (step 10, Table 4.5A.4.3-1)

| Derivation path: Table 4.7.3-6 and table 4.6.1-8 with condition AM-DRB-ADD(2) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Access point name | sos | APN value as recommended by IR.88 clause 6.4 [56] |  |
| EPS QoS | According to reference default EPS bearer context #2 – in table 6.6.1-1 | SS defines an additional dedicated EPS QoS |  |

Table 4.5A.4.4-3: Message ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST (step 13, Table 4.5A.4.3-1)

| Derivation path: Table 4.7.3-3 and table 4.6.1-8 with condition UM-DRB-ADD(3) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Linked EPS bearer identity | Default EBId-2 (same value like in table 4.5A.4.4-1) |  |  |
| EPS QoS | According to reference dedicated EPS bearer context #1 - in table 6.6.2-1 | SS defines an additional dedicated EPS QoS |  |
| TFT | According to reference dedicated EPS bearer context #1 - in table 6.6.2-1 |  |  |

Table 4.5A.4.4-4: PDN CONNECTIVITY REQUEST (step 9)

| Derivation Path: Table 4.7.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Access point name | Not present or any value |  |  |

### 4.5A.5	Generic Test Procedure for IMS Emergency call establishment in EUTRA: Limited Service

#### 4.5A.5.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE is switched on and camped on the cell in EMM-DEREGISTERED.LIMITED-SERVICE or EMM-REGISTERED.LIMITED-SERVICE state.

#### 4.5A.5.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.5.3	Procedure

The establishment of IMS emergency call is assumed to always be mobile originated.

Table 4.5A.5.3-1: EUTRA/EPS signalling for IMS Emergency Call in limited service

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Make the UE attempt an IMS emergency call. | - | - |
| 2 | The UE transmits an RRCConnectionRequest message with 'establishmentCause' set to 'emergency'. | --> | RRCConnectionRequest |
| 3 | SS transmits an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 4 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the Attach procedure by including the ATTACH REQUEST message, EPS attach type set to “EPS emergency attach" ('0110'B). The PDN CONNECTIVITY REQUEST message is piggybacked in ATTACH REQUEST, with 'Request type' set to 'emergency ('0100'B)'. | --> | RRC: RRCConnectionSetupComplete NAS: ATTACH REQUEST NAS: PDN CONNECTIVITY REQUEST |
| 5a1 – 6b1 | Void | - | - |
| 7 | The SS transmits a NAS SECURITY MODE COMMAND message to activate NAS security. Note: KSI value is set to "000" and EIA0 (NULL integrity), and EEA0 (NULL ciphering) algorithms are used. | <-- | RRC: DLInformationTransfer NAS: SECURITY MODE COMMAND |
| 8 | The UE transmits a NAS SECURITY MODE COMPLETE message and establishes the initial security configuration. | --> | RRC: ULInformationTransfer NAS: SECURITY MODE COMPLETE |
| - | EXCEPTION: Steps 9a1 to 9a2 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that take place if the UE has ESM information which needs to be transferred. | - | - |
| 9a1 | IF the UE sets the ESM information transfer flag in the last PDN CONNECTIVITY REQUEST message THEN the SS transmits an ESM INFORMATION REQUEST message to initiate exchange of protocol configuration options and/or APN. | <-- | RRC: DLInformationTransfer NAS: ESM INFORMATION REQUEST |
| 9a2 | The UE transmits an ESM INFORMATION RESPONSE message to transfer protocol configuration options and/or APN. | --> | RRC: ULInformationTransfer NAS: ESM INFORMATION RESPONSE |
| 10 | The SS transmits a SecurityModeCommand message to activate AS security. Note: eia0 (NULL integrity), and eea0 (NULL ciphering) algorithms are used. | <-- | RRC: SecurityModeCommand |
| 11 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 12 | The SS transmits a UECapabilityEnquiry message to initiate the UE radio access capability transfer procedure. | <-- | RRC: UECapabilityEnquiry |
| 13 | The UE transmits a UECapabilityInformation message to transfer UE radio access capability. | --> | RRC: UECapabilityInformation |
| 14 | The SS transmits an RRCConnectionReconfiguration message to establish the default bearer with condition SRB2-DRB(1, 0). The DRB associated with default EPS bearer context #2 (QCI 5) according to table 6.6.1-1: Reference default EPS bearer contexts, obtained during the attach procedure, is established. This message includes the ATTACH ACCEPT message with EPS attach result set to “EPS only" ('001'B). The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT. Note: The APN is set to the test APN defined for emergency bearer services | <-- | RRC: RRCConnectionReconfiguration NAS: ATTACH ACCEPT NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST |
| - | EXCEPTION: In parallel to the events described in steps 15 to 19 below, the behaviour in table 4.5A.5.3-2 occurs. (Optional IP address allocation followed by IMS emergency speech call establishment) | - | - |
| 15 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of default bearer. | --> | RRC: RRCConnectionReconfigurationComplete |
| 16 | This message includes the ATTACH COMPLETE message. The ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message is piggybacked in ATTACH COMPLETE. | --> | RRC: ULInformationTransfer NAS: ATTACH COMPLETE NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |
| 17 | The SS configures a new RLC-UM data radio bearer, associated with the dedicated EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message. EPS bearer context #4 (QCI 1) according to table 6.6.2-1: Reference dedicated EPS bearer contexts is used. Note: the same PDN address is applicable because the linked EPS bearer ID refers to the default EBC allocated in step 14. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| 18 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer for emergency IMS signalling. | --> | RRC: RRCConnectionReconfigurationComplete |
| 19 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |
| Note 1: Void Note 2: Void |  |  |  |

Table 4.5A.5.3-2: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Step 1 describes behaviour that depends on the UE behaviour. | - | - |
| 1 | If initiated by the UE the generic procedure for IP address allocation in the U-plane specified in subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - |
| 2-6 | Steps 1-5 defined in annex C.22 of TS 34.229-1 [43]. IMS Emergency call for EPS is established. | - | - |

#### 4.5A.5.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7 with the exceptions below.

Table 4.5A.5.4-1: SECURITY MODE COMMAND (step 7, Table 4.5A.5.3-1)

| Derivation Path: Table 4.7.2-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Selected NAS security algorithms |  |  |  |
| Type of integrity protection algorithm | EIA0 |  |  |
| Type of ciphering algorithm | EEA0 |  |  |
| NAS key set identifier |  |  |  |
| NAS key set identifier | '000'B |  |  |
| TSC | '0'B | native security context (for KSIASME) |  |
| Spare half octet | '0000'B |  |  |

Table 4.5A.5.4-2: SecurityModeCommand (step 10, Table 4.5A.5.3-1)

| Derivation Path: Table 4.6.1-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SecurityModeCommand ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| securityModeCommand-r8 SEQUENCE { |  |  |  |
| securityConfigSMC | SecurityConfigSMC-LimitedService | Table 4.5A.5.4-3 |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 4.5A.5.4-3: SecurityConfigSMC-LimitedService (Table 4.5A.5.4-2)

| Derivation Path: Table 4.6.4-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SecurityConfigSMC-LimitedService ::= SEQUENCE { |  |  |  |
| securityAlgorithmConfig SEQUENCE { |  |  |  |
| cipheringAlgorithm | eea0 |  |  |
| integrityProtAlgorithm | eia0-v920 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 4.5A.5.4-4: Message ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST (step 17, Table 4.5A.5.3-1)

| Derivation path: Table 4.7.3-3 and table 4.6.1-8 with condition UM-DRB-ADD(3) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Linked EPS bearer identity | Default EBId-2 (same value as in table 4.5A.5.4-5) |  |  |
| EPS QoS | See Reference dedicated EPS bearer context #4 in table 6.6.2-1 |  |  |
| TFT | See Reference dedicated EPS bearer context #4 in table 6.6.2-1 |  |  |

Table 4.5A.5.4-5: Message ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST (step 14, Table 4.5A.5.3-1)

| Derivation path: Table 4.7.3-6 and table 4.6.1-8 with condition AM-DRB-ADD(2) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Access point name | sos | APN value as recommended by IR.88 clause 6.4 [56] |  |
| EPS QoS | According to reference default EPS bearer context #2 – in table 6.6.1-1 | SS defines an additional dedicated EPS bearer |  |

Table 4.5A.5.4-6: Message ESM INFORMATION RESPONSE (step 9a2, Table 4.5A.5.3-1)

| Derivation Path: Table 4.7.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Access point name | Not present or any value |  |  |

Table 4.5A.5.4-7: INVITE (step 2, Table 4.5A.5.3-2)

Derivation Path: Step 1 of TS 34.229-1 [43] annex C.22 with condition A6 instead of A7

Table 4.5A.5.4-8: 180 Ringing (step 4, Table 4.5A.5.3-2)

Derivation Path: Step 3 of TS 34.229-1 [43] annex C.22 with condition A7 instead of A8

Table 4.5A.5.4-9: 200 OK for INVITE (step 5, Table 4.5A.5.3-2)

Derivation Path: Step 4 of TS 34.229-1 [43] annex C.22 with condition A7 instead of A6

### 4.5A.6	Generic Test Procedure for IMS MO speech call establishment in E-UTRA

#### 4.5A.6.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Registered, Idle Mode state (State 2).

#### 4.5A.6.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.6.3	Procedure

Table 4.5A.6.3-1: EUTRA/EPS signalling for IMS MO speech call

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Make the UE attempt an IMS speech call | - | - |
| 2 | The UE transmits an RRCConnectionRequest message with ' establishmentCause' set to ' mo-Data '. | --> | RRCConnectionRequest |
| 3 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 4 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 5 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 6 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 7 | The SS configures a new data radio bearer, associated with the default EPS bearer context. The RRCConnectionReconfiguration message is using condition SRB2-DRB(1, 0). The DRB associated with default EPS bearer context obtained during the attach procedure is established | <-- | RRC: RRCConnectionReconfiguration |
| - | EXCEPTION: In parallel to the events described in steps 8 below, the behaviour in table 4.5A.6.3-2 occurs. (IMS MTSI MO speech call establishment) | - | - |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer context. | --> | RRC: RRCConnectionReconfigurationComplete |
| - | EXCEPTION: Steps 9a1 to 11b1 describe behaviour that depends on the UE category. | - | - |
| 9a1-11a1 | IF not pc_ue_CategoryDL_M1 THEN steps 3-4 from the expected sequence defined in annex C.21of TS 34.229-1 [43]. MTSI MO speech call for EPS take place. | - | - |
| 9b1-11b1 | IF pc_ue_CategoryDL_M1 THEN steps 3-4 from the expected sequence defined in annex C.21d of TS 34.229-1 [43]. MTSI MO speech call for EPS / UE category M1 take place. | - | - |
| 12 | The SS configures a new RLC-UM data radio bearer with condition DRB (0,1), associated with the dedicated EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message. EPS bearer context #4 (QCI 1) according to table 6.6.2-1: Reference dedicated EPS bearer contexts. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| - | EXCEPTION: In parallel to the events described in steps 13-14 below, the behaviour in table 4.5A.6.3-3 occurs. (IMS MTSI MO speech call establishment) | - | - |
| 13 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the dedicated EPS bearer. | --> | RRC: RRCConnectionReconfigurationComplete |
| 14 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |

Table 4.5A.6.3-2: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | . EXCEPTION: Steps 1a1 to 1b1 describe behaviour that depends on the UE category. |  |  |
| 1a1 | IF not pc_ue_CategoryDL_M1 THEN THEN the step 2 expected sequence defined in annex C.21 of TS 34.229-1 [43]. UE sends INVITE take place. | - | - |
| 1b1 | IF pc_ue_CategoryDL_M1 THEN THEN Step 2 from the expected sequence defined in annex C.21d of TS 34.229-1 [43]. UE sends INVITE take place. | - | - |

Table 4.5A.6.3-3: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | . EXCEPTION: Steps 1a1 to 8b1 describe behaviour that depends on the UE category. |  |  |
| 1a1-8a1 | IF not pc_ue_CategoryDL_M1 THEN HEN the steps from the 5-13 expected sequence defined in annex C.21 of TS 34.229-1 [43]. MTSI MO speech call for EPS take place. | - | - |
| 1b1-8b1 | IF pc_ue_CategoryDL_M1 THEN the steps 5-13 from the expected sequence defined in annex C.21d of TS 34.229-1 [43]. MTSI MO speech call for EPS / UE category M1 take place. | - | - |

#### 4.5A.6.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7.

### 4.5A.7	Generic Test Procedure for IMS MT Speech call establishment in E-UTRA

#### 4.5A.7.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Registered, Idle Mode state (State 2).

#### 4.5A.7.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.7.3	Procedure

Table 4.5A.7.3-1: EUTRA/EPS signalling for IMS MT speech call

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | SS sends a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordLists. | <-- | RRC: Paging (PCCH) |
| 2 | The UE transmits an RRCConnectionRequest message with ' establishmentCause' set to 'mt-Access'. | --> | RRCConnectionRequest |
| 3 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 4 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 5 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 6 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 7 | The SS configures a new data radio bearer, associated with the default EPS bearer context. The RRCConnectionReconfiguration message is using condition SRB2-DRB(1, 0). The DRB associated with default EPS bearer context obtained during the attach procedure is established | <-- | RRC: RRCConnectionReconfiguration |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer context. | --> | RRC: RRCConnectionReconfigurationComplete |
| - | EXCEPTION: Steps 9a1 to 12b3 describe behaviour that depends on the UE category. |  |  |
| 9a1-12a3 | IF not pc_ue_CategoryDL_M1 THEN THEN the steps 1-6 from the expected sequence defined in annex C.11 of TS 34.229-1 [43]. MTSI MT speech call take place. | - | - |
| 9b1-12b3 | IF pc_ue_CategoryDL_M1 THEN steps 1-6 from the expected sequence defined in annex C.11d of TS 34.229-1 [43]. MTSI MT speech call / UE category M1 take place. | - | - |
| 13 | The SS configures a new RLC-UM data radio bearer with condition DRB (0,1), associated with the dedicated EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| 14 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the dedicated EPS bearer. | --> | RRC: RRCConnectionReconfigurationComplete |
| 15 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |
| - | EXCEPTION: Steps 18a1 to 22b1 describe behaviour that depends on the UE category. |  |  |
| 16a1-17 | Void | - | - |
| 18a1-22a1 | IF not pc_ue_CategoryDL_M1 THEN steps 7-11 from the expected sequence defined in annex C.11 of TS 34.229-1 [43] MTSI MT speech call take place. | - | - |
| 18b1-22b1 | IF pc_ue_CategoryDL_M1 THEN the steps 7-11 from the expected sequence defined in annex C.11d of TS 34.229-1 [43] MTSI MT speech call / UE category M1 take place. | - | - |
| - | EXCEPTION: Steps 23Aa1 to 23Ab1 describe behaviour that depends on the UE category. |  |  |
| 23Aa1 | IF not pc_ue_CategoryDL_M1 THEN step 11A from the expected sequence defined in annex C.11 of TS 34.229-1 [43] MTSI MT speech call take place. | - | - |
| 23Ab1 | IF pc_ue_CategoryDL_M1 THEN step 11A from the expected sequence defined in annex C.11d of TS 34.229-1 [43] MTSI MT speech call / UE category M1 take place. | - | - |
| - | EXCEPTION: Steps 23a1 to 24b1 describe behaviour that depends on the UE category. |  |  |
| 23a1-24a1 | IF not pc_ue_CategoryDL_M1 THEN steps 12-13 expected sequence defined in annex C.11 of TS 34.229-1 [43] MTSI MT speech call take place. | - | - |
| 23b1-24b1 | IF pc_ue_CategoryDL_M1 THEN steps 12-13 from the expected sequence defined in annex C.11d of TS 34.229-1 [43] MTSI MT speech call / UE category M1 take place. | - | - |
| - | EXCEPTION: Steps 25a1 to 26b1 describe behaviour that depends on the UE category. |  |  |
| 25a1-26a2 | IF not pc_ue_CategoryDL_M1 THEN steps 14-15 from the expected sequence defined in annex C.11 of TS 34.229-1 [43] MTSI MT speech call take place. | - | - |
| 25b1-26b1 | IF pc_ue_CategoryDL_M1 THEN steps 14-15 from the expected sequence defined in annex C.11d of TS 34.229-1 [43] MTSI MT speech call / UE category M1 take place. | - | - |

#### 4.5A.7.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7.

### 4.5A.8	Generic Test Procedure for IMS MO video call establishment in E-UTRA

#### 4.5A.8.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Registered, Idle Mode state (State 2).

#### 4.5A.8.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.8.3	Procedure

Table 4.5A.8.3-1: EUTRA/EPS signalling for IMS MO video call

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Make the UE attempt an IMS video call | - | - |
| 2 | The UE transmits an RRCConnectionRequest message with ' establishmentCause' set to ' mo-Data '. | --> | RRCConnectionRequest |
| 3 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 4 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 5 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 6 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 7 | The SS configures a new data radio bearer, associated with the default EPS bearer context. The RRCConnectionReconfiguration message is using condition SRB2-DRB(1, 0). The DRB associated with default EPS bearer context obtained during the attach procedure is established | <-- | RRC: RRCConnectionReconfiguration |
| - | EXCEPTION: In parallel to the events described in steps 8 below, the behaviour in table 4.5A.8.3-2 occurs. (IMS MTSI MO video call establishment) | - | - |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer context. | --> | RRC: RRCConnectionReconfigurationComplete |
| 9-11 | Steps 3-4 expected sequence defined in annex C.25 of TS 34.229-1 [43]. MTSI MO video call for EPS. | - | - |
| 12 | The SS configures two new RLC-UM data radio bearers with condition DRB (0,2), associated with the dedicated EPS bearer context. RRCConnectionReconfiguration message contains one ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message per additional data radio bearer. One EPS bearer context #4 (QCI 1) and one EPS bearer context #3 (QCI 2) according table 6.6.2-1: Reference dedicated EPS bearer contexts. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| - | EXCEPTION: In parallel to the events described in steps 13-14 below, the behaviour in table 4.5A.8.3-3 occurs. (IMS MTSI MO video call establishment). | - | - |
| 13 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the dedicated EPS bearer. EPS bearer context #4 (QCI 1) according table to 6.6.2-1: Reference dedicated EPS bearer contexts. | --> | RRC: RRCConnectionReconfigurationComplete |
| 14 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message for the first bearer. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |
| 15 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message for the second bearer. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |

Table 4.5A.8.3-2: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Step 2 expected sequence defined in annex C.25 of TS 34.229-1 [43]. UE sends INVITE. | - | - |

Table 4.5A.8.3-3: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1-8 | Steps 5-13 expected sequence defined in annex C.25 of TS 34.229-1 [43]. MTSI MO video call for EPS. | - | - |

#### 4.5A.8.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7.

### 4.5A.9	Generic Test Procedure for IMS MT video call establishment in E-UTRA

#### 4.5A.9.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Registered, Idle Mode state (State 2).

#### 4.5A.9.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.9.3	Procedure

Table 4.5A.9.3-1: EUTRA/EPS signalling for IMS MT video call

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | SS sends a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordLists. | <-- | RRC: Paging (PCCH) |
| 2 | The UE transmits an RRCConnectionRequest message with ' establishmentCause' set to 'mt-Access'. | --> | RRCConnectionRequest |
| 3 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 4 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 5 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 6 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 7 | The SS configures a new data radio bearer, associated with the default EPS bearer context. The RRCConnectionReconfiguration message is using condition SRB2-DRB(1, 0). The DRB associated with default EPS bearer context obtained during the attach procedure is established | <-- | RRC: RRCConnectionReconfiguration |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer context. | --> | RRC: RRCConnectionReconfigurationComplete |
| 9-12b | Steps 1-6 expected sequence defined in annex C.26 of TS 34.229-1 [43]. MTSI MT video call. | - | - |
| 13 | The SS configures a new RLC-UM data radio bearer with condition DRB (0,2), associated with the dedicated EPS bearer context. RRCConnectionReconfiguration message contains one ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message per additional data radio bearer. One EPS bearer context #4 (QCI 1) and one EPS bearer context #3 (QCI 2) according table 6.6.2-1: Reference dedicated EPS bearer contexts. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| 14 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the dedicated EPS bearer. | --> | RRC: RRCConnectionReconfigurationComplete |
| 15 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message for the first bearer. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |
| 16 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message for the second bearer. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |
| 17-18 | Void | - | - |
| 19-27 | Steps 7-15 expected sequence defined in annex C.26 of TS 34.229-1 [43]. MTSI MT video call for EPS. | - | - |

#### 4.5A.9.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7.

### 4.5A.10	Generic Test Procedure for IMS MO speech and aSRVCC in E-UTRA

#### 4.5A.10.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Registered, Idle Mode state (State 2).

#### 4.5A.10.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.10.3	Procedure

Table 4.5A.10.3-1: EUTRA/EPS signalling for IMS MO speech to alerting state

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Make the UE attempt an IMS speech call | - | - |
| 2 | The UE transmits an RRCConnectionRequest message with ' establishmentCause' set to ' mo-Data '. | --> | RRCConnectionRequest |
| 3 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 4 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 5 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 6 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 7 | The SS configures a new data radio bearer, associated with the default EPS bearer context. The RRCConnectionReconfiguration message is using condition SRB2-DRB(1, 0). The DRB associated with default EPS bearer context obtained during the attach procedure is established | <-- | RRC: RRCConnectionReconfiguration |
| - | EXCEPTION: In parallel to the events described in steps 8 below, the behaviour in table 4.5A.10.3-2 occurs. Initiate MTSI MO speech. | - | - |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer context. | --> | RRC: RRCConnectionReconfigurationComplete |
| 9-11 | Steps 3-4 expected sequence defined in annex C.21 of TS 34.229-1 [43]. MTSI MO speech call for EPS. | - | - |
| 12 | The SS configures a new RLC-UM data radio bearer with condition DRB (0,1), associated with the dedicated EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message. EPS bearer context #4 (QCI 1) according to table 6.6.2-1: Reference dedicated EPS bearer contexts. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
|  | EXCEPTION: In parallel to the events described in steps 13-14 below, the behaviour in table 4.5A.10.3-3 occurs. MTSI MO speech call to alerting state. | - | - |
| 13 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the dedicated EPS bearer. | --> | RRC: RRCConnectionReconfigurationComplete |
| 14 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |

Table 4.5A.10.3-2: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Step 2 expected sequence defined in annex C.21 of TS 34.229-1 [43]. UE sends INVITE. | - | - |

Table 4.5A.10.3-3: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1-7 | Steps 5-11 expected sequence defined in annex C.21 of TS 34.229-1 [43]. MTSI MO speech call to alerting state for EPS. | - | - |

#### 4.5A.10.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7.

### 4.5A.11	Generic Test Procedure for IMS MO add video establishment in E-UTRA

#### 4.5A.11.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall execute 4.5A.6.

#### 4.5A.11.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.11.3	Procedure

Table 4.5A.11.3-1: EUTRA/EPS signalling for IMS MO add video

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Make the UE attempt add IMS video to the voice call. | - | - |
| 2-4 | Steps 2-4 expected sequence defined in expected sequence test case 17.1.4 of TS 34.229-1 [43]. MO Speech, add video remove video. | - | - |
| 5 | The SS configures a new RLC-UM data radio bearer with condition DRB (0,1), associated with the dedicated EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message. EPS bearer context #3 (QCI 2) according to table 6.6.2-1: Reference dedicated EPS bearer contexts. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| - | EXCEPTION: In parallel to the events described in steps 6-7 below, the behaviour in table 4.5A.11.3-2 occurs. (IMS MTSI MO video call establishment). | - | - |
| 6 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the dedicated EPS bearer. EPS bearer context #3 (QCI 2) according table to 6.6.2-1: Reference dedicated EPS bearer contexts. | --> | RRC: RRCConnectionReconfigurationComplete |
| 7 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message for the first bearer. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |

Table 4.5A.11.3-2: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1-6 | Steps 5-10 expected sequence defined in test case 17.1.4 of TS 34.229-1 [43]. MO Speech, add video remove video. | - | - |

#### 4.5A.11.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7.

### 4.5A.12	Generic Test Procedure for IMS MT add video establishment in E-UTRA

#### 4.5A.12.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall execute 4.5A.7.

#### 4.5A.12.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.12.3	Procedure

Table 4.5A.12.3-1: EUTRA/EPS signalling for IMS MT add video

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1-3b | Steps 1-5 expected sequence defined in expected sequence test case 17.2.4 of TS 34.229-1 [43]. MT Speech, add video remove video. | - | - |
| 4 | The SS configures a new RLC-UM data radio bearer with condition DRB (0,1), associated with the dedicated EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message. EPS bearer context #3 (QCI 2) according to table 6.6.2-1: Reference dedicated EPS bearer contexts. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| 5 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the dedicated EPS bearer. EPS bearer context #3 (QCI 2) according table to 6.6.2-1: Reference dedicated EPS bearer contexts. | --> | RRC: RRCConnectionReconfigurationComplete |
| 6 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message for the first bearer. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |
| 7-8 | Void | - | - |
| 9-12 | Steps 6-9 expected sequence defined in test case 17.2.4 of TS 34.229-1 [43], MT Speech, add video remove video. | - | - |

#### 4.5A.12.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7.

4.5A.13	Void

### 4.5A.14	Generic Test Procedure for IMS XCAP establishment in EUTRA

#### 4.5A.14.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Registered, Idle Mode state (State 2).

-	If pc_XCAP_only_APN==true then the UE is connected to the IMS PDN only

-	else the UE is connected to the IMS PDN and the Internet PDN which is used for XCAP signalling.

#### 4.5A.14.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.14.3	Procedure

Table 4.5A.14.3-1: EUTRA/EPS signalling for XCAP

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Make the UE attempt XCAP signalling. | - | - |
| 2 | The UE transmits an RRCConnectionRequest message with ' establishmentCause' set to 'mo-Data '. | --> | RRCConnectionRequest |
| 3 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 4 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 5 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 6 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 7 | The SS configures a new data radio bearer, associated with the default EPS bearer context. The RRCConnectionReconfiguration message is using condition SRB2-DRB(1, 0) in case of pc_XCAP_only_APN==true or using condition SRB2-DRB(2, 0) in case of pc_XCAP_over_Internet_APN==true. The DRBs associated with the respective default EPS bearer context obtained during the attach procedure are established | <-- | RRC: RRCConnectionReconfiguration |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer context. | --> | RRC: RRCConnectionReconfigurationComplete |
|  | EXCEPTION: IF pc_XCAP_only_APN THEN additional PDN connectivity shall be established as specified in TS 36.508 subclause 4.5A.16 (condition: NOT ADD_IMS) |  |  |

#### 4.5A.14.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7.

### 4.5A.15	Generic Test Procedure for EPS Bearer Deactivation

#### 4.5A.15.1	Initial conditions

The EPS bearer to be deactivated has been established before. The EPS bearer has to be specified.

#### 4.5A.15.2	Definition of system information messages

The system information messages applicable in the test case are used.

#### 4.5A.15.3	Procedure

Table 4.5A.15.3-1: Procedure for EPS Bearer Deactivation

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The SS transmits an RRCConnectionReconfiguration message to deactivate an EPS bearer. | <-- | RRC: RRCConnectionReconfiguration NAS: DEACTIVATE EPS BEARER CONTEXT REQUEST |
| - | EXCEPTION: In parallel to the event described in step2 the step specified in Table 4.5A.15.3-2 should take place. | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the deactivation of EPS bearer. | --> | RRC: RRCConnectionReconfigurationComplete |
| 3 | Void | - | - |

Table 4.5A.15.3-2: Procedure for EPS Bearer Deactivation (parallel behaviour)

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The UE transmits an ULInformationTransfer message to accept deactivation of the EPS bearer. | --> | RRC: ULInformationTransfer NAS: DEACTIVATE EPS BEARER CONTEXT ACCEPT |

#### 4.5A.15.4	Specific message contents

Table 4.5A.15.4-1: Message DEACTIVATE EPS BEARER CONTEXT REQUEST (step 1, Table 4.5A.15.3-1)

| Derivation path: Table 4.7.3-12 and table 4.6.1-8 with condition and condition NETWORK-INITIATED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS bearer identity | EPS bearer identity | Same value as in the activation message. |  |
| ESM cause | 00100100 | regular deactivation |  |

Table 4.5A.15.4-2: Message DEACTIVATE EPS BEARER CONTEXT ACCEPT (step 2, Table 4.5A.15.3-1)

| Derivation Path:Table 4.7.3-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS bearer identity | EPS bearer identity | The same value as the value set in DEACTIVATE EPS BEARER CONTEXT REQUEST message. |  |
| Procedure transaction identity | 0 | No procedure transaction identity assigned |  |

### 4.5A.15A	Generic Test Procedure for User or Network Initiated EPS Bearer Deactivation

#### 4.5A.15A.1	Initial conditions

The EPS bearer to be deactivated has been established before. The EPS bearer has to be specified.

#### 4.5A.15A.2	Definition of system information messages

The system information messages applicable in the test case are used.

#### 4.5A.15A.3	Procedure

Table 4.5A.15A.3-1: Procedure for EPS Bearer Deactivation

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Step 1Aa describes behaviour that depends on UE capabilities/configuration; the "lower case letter" identifies a step sequence that takes place if one of those is supported/configured. The SS shall wait up to 5s for the UE to trigger the PDN Disconnect procedure. NOTE: The specified wait period of 5s is a working assumption to facilitate test case implementation. | - | - |
| 1Aa | The UE transmits a PDN DISCONNECT REQUEST | --> | RRC: ULInformationTransfer PDN DISCONNECT REQUEST |
| 1 | The SS transmits an RRCConnectionReconfiguration message to deactivate an EPS bearer. | <-- | RRC: RRCConnectionReconfiguration NAS: DEACTIVATE EPS BEARER CONTEXT REQUEST |
| - | EXCEPTION: In parallel to the event described in step2 the step specified in Table 4.5A.15A.3-2 should take place. | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the deactivation of EPS bearer. | --> | RRC: RRCConnectionReconfigurationComplete |

Table 4.5A.15A.3-2: Procedure for EPS Bearer Deactivation (parallel behaviour)

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The UE transmits an ULInformationTransfer message to accept deactivation of the EPS bearer. | --> | RRC: ULInformationTransfer NAS: DEACTIVATE EPS BEARER CONTEXT ACCEPT |

#### 4.5A.15A.4	Specific message contents

Table 4.5A.15A.4-1: Message DEACTIVATE EPS BEARER CONTEXT REQUEST (step 1, Table 4.5A.15.3-1)

| Derivation path: Table 4.7.3-12 and table 4.6.1-8 with condition and condition NETWORK-INITIATED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS bearer identity | EPS bearer identity | Same value as in the activation message. |  |
| ESM cause | 00100100 | regular deactivation |  |

Table 4.5A.15A.4-2: Message DEACTIVATE EPS BEARER CONTEXT ACCEPT (step 2, Table 4.5A.15.3-1)

| Derivation Path: Table 4.7.3-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS bearer identity | EPS bearer identity | The same value as the value set in DEACTIVATE EPS BEARER CONTEXT REQUEST message. |  |
| Procedure transaction identity | 0 | No procedure transaction identity assigned |  |

### 4.5A.16	Generic Test Procedure to establish additional PDN connectivity

The same assumptions and definitions apply as in clause 4.5.2and in addition:

| Condition | Explanation |
| --- | --- |
| ADD_IMS | true if this procedure is part of the initial registration (PDN2_IMS==true according to definitions in clause 4.5.2); false for all other cases |
| MobileData_OFF | Mobile data associated with Internet PDN is disabled on the DUT. It is configured via setting px_MobileDataOn (TS 36.523-3[20] Table 9.1-1) = false |

#### 4.5A.16.1	Initial conditions

System Simulator:

-	1 cell, default parameters.

-	The procedure shall be performed under ideal radio conditions as defined in clause 5

User Equipment:

- 	The UE is in Registered, RRC_CONNECTED state (State 2).

#### 4.5A.16.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.16.3	Procedure

Table 4.5A.16.3-1: Establishment of additional PDN connectivity

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Depending upon UE configuration Step 0a1 may be performed | - | - |
| 0a1 | If IMS_Internet and MobileData_OFF then the request of connectivity to an additional PDN is performed by MMI or AT command with APN “Internet”. | - | - |
| 1 | The UE transmits a PDN CONNECTIVITY REQUEST message to request an additional PDN. | --> | RRC: ULInformationTransfer NAS: PDN CONNECTIVITY REQUEST |
| 2 | The SS configures a new data radio bearer, associated with the additional default EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST |
| 3 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of additional default bearer. | --> | RRC: RRCConnectionReconfigurationComplete |
| - | EXCEPTION: In parallel to the event described in step 4 below, if initiated by the UE the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - |
| - | EXCEPTION: IF ADD_IMS THEN in parallel to the event described in step 4 below the generic procedure for IMS signalling in the U-plane specified in TS 36.508 subclause 4.5A.3 takes place if requested by the UE | - | - |
| 4 | The UE transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |

#### 4.5A.16.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7 with the exceptions below.

Table 4.5A.16.4-1: PDN CONNECTIVITY REQUEST (step 1)

| Derivation Path: Table 4.7.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Access point name | Any allowed value | The UE includes a new APN. |  |

Table 4.5A.16.4-2: Message ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST (step 2)

| Derivation path: Table 4.7.3-6 with condition IMS_PDN_ConnEstab for ADD_IMS and NOT IMS_PDN_ConnEstab else and table 4.6.1-8 with condition AM-DRB-ADD(1) for ADD_IMS or with condition AM-DRB-ADD(8) else |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS bearer identity | '0101'B | arbitrary value used for PDN connectivity being maintained during the test case | ADD_IMS |
| EPS bearer identity | '1100'B | arbitrary value used for additional non-IMS PDN connectivity | NOT ADD_IMS |
| Procedure transaction identity | PTI-1 | SS re-uses the particular PTI defined by UE for this present additional PDN connectivity request procedure |  |

Table 4.5A.16.4-3: Message ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT (step 4)

| Derivation path: Table 4.7.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS bearer identity | (same value as used in step 2) |  |  |
| Procedure transaction identity | 0 | No procedure transaction identity assigned |  |

### 4.5A.17	Generic Test Procedure for user initiated release of additional PDN connectivity

#### 4.5A.17.1	Initial conditions

System Simulator:

-	1 cell, default parameters.

-	The procedure shall be performed under ideal radio conditions as defined in clause 5

User Equipment:

-	The UE shall be in Registered, Idle Mode state (State 2) with connectivity at least to two PDNs

#### 4.5A.17.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.17.3	Procedure

Table 4.5A.17.3-1: Release of additional PDN connectivity

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Cause the UE to request disconnection from the additional PDN (see Note 1) | - | - |
| 2 | The UE transmits an RRCConnectionRequest | --> | RRCConnectionRequest |
| 3 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| - | EXCEPTION: Steps 4a1 to 4b1 describe behaviour which depends on earlier network indication of supported features. | - | - |
| 4a1 | IF the network indicated support of EXTENDED SERVICE REQUEST for packet services in the preceding ATTACH ACCEPT message, THEN the UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the EXTENDED SERVICE REQUEST message | --> | RRC: RRCConnectionSetupComplete NAS: EXTENDED SERVICE REQUEST |
| 4b1 | ELSE, the UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 5 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 6 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 7 | The SS transmits a RRCConnectionReconfiguration message to establish SRB2 and DRBs associated with all default EPS bearer contexts | <-- | RRC: RRCConnectionReconfiguration |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of SRB2 and DRBs associated with all default EPS bearer contexts. | --> | RRC: RRCConnectionReconfigurationComplete |
| 9 | The UE transmits a PDN DISCONNECT REQUEST | --> | RRC: ULInformationTransfer PDN DISCONNECT REQUEST |
| 10 | The SS transmits a DEACTIVATE EPS BEARER CONTEXT REQUEST message included in an RRCConnectionReconfiguration message. | <-- | RRC: RRCConnectionReconfiguration NAS: DEACTIVATE EPS BEARER CONTEXT REQUEST |
| 11 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the release of the bearer. | --> | RRC: RRCConnectionReconfigurationComplete |
| 12 | The UE transmits a DEACTIVATE EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer DEACTIVATE EPS BEARER CONTEXT ACCEPT |
| 13 | The SS transmits an RRCConnectionRelease message to release RRC connection | <-- | RRC: RRCConnectionRelease |
| NOTE 1: The request to disconnect from a PDN may be performed by MMI or AT command; in any case the EPS bearer identity of the Default EPS Bearer of the PDN to be released needs to be handed over to the UE |  |  |  |

#### 4.5A.17.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7 with the exceptions below.

Table 4.5A.17.4-1: RRCConnectionReconfiguration (step 7)

Derivation path: 36.508 table 4.6.1-8 using condition SRB2-DRB(2, 0) NOTE: The bid for the AM DRBs shall be 1 and 8 instead of 1 and 2.

Table 4.5A.17.4-2: PDN DISCONNECT REQUEST (step 9)

| Derivation Path: TS 36.508 Table 4.7.3-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS bearer identity | ‘0000’ | "no EPS bearer identity assigned" |  |
| Procedure transaction identity | PTI-1 | UE assigns a particular PTI not yet used between 1 and 254 |  |
| Linked EPS bearer identity | (bearer identity as handed over at step 1) |  |  |

Table 4.5A.17.4-3: RRCConnectionReconfiguration (step 10)

Derivation path: 36.508 table 4.6.1-8 using condition DRB-REL(8)

Table 4.5A.17.4-4: DEACTIVATE EPS BEARER CONTEXT REQUEST (step 10)

| Derivation Path: TS 36.508 Table 4.7.3-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS bearer identity | (same as in Linked EPS bearer identity of step 9) |  |  |
| Procedure transaction identity | PTI-1 | SS re-uses the particular PTI defined by UE for this present PDN disconnection procedure. | UE-INITIATED |
| ESM cause | 00100100 | regular deactivation |  |

Table 4.5A.17.4-5: DEACTIVATE EPS BEARER CONTEXT ACCEPT (step 12)

| Derivation Path: TS 36.508 Table 4.7.3-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS bearer identity | (same as in DEACTIVATE EPS BEARER CONTEXT REQUEST of step 10) | The same value as the value set in DEACTIVATE EPS BEARER CONTEXT REQUEST message. |  |
| Procedure transaction identity | 0 | No procedure transaction identity assigned |  |

### 4.5A.18	Generic Test Procedure for network initiated release of additional PDN connectivity

#### 4.5A.18.1	Initial conditions

System Simulator:

-	1 cell, default parameters.

-	The procedure shall be performed under ideal radio conditions as defined in clause 5

User Equipment:

- 	The UE shall be in Registered, Idle Mode state (State 2) with connectivity at least to two PDNs

#### 4.5A.18.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.18.3	Procedure

Table 4.5A.18.3-1: Release of additional PDN connectivity

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1-7 | Steps 1 to 7 of the generic radio bearer establishment procedure (TS 36.508 4.5.3.3-1) |  |  |
| 8 | The SS transmits a RRCConnectionReconfiguration message to establish SRB2 and DRBs associated with all default EPS bearer contexts | <-- | RRC: RRCConnectionReconfiguration |
| 9 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of SRB2 and DRBs associated with all default EPS bearer contexts. | --> | RRC: RRCConnectionReconfigurationComplete |
| 10 | SS releases the PDN connectivity by transmitting a RRCConnectionReconfiguration containing a DEACTIVATE EPS BEARER CONTEXT REQUEST | <-- | RRC: RRCConnectionReconfiguration NAS: DEACTIVATE EPS BEARER CONTEXT REQUEST |
| 11 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the release of the bearer. | --> | RRC: RRCConnectionReconfigurationComplete |
| 12 | The UE transmits a DEACTIVATE EPS BEARER CONTEXT ACCEPT message. | --> | DEACTIVATE EPS BEARER CONTEXT ACCEPT |
| 13 | The SS transmits an RRCConnectionRelease message to release RRC connection | <-- | RRC: RRCConnectionRelease |

#### 4.5A.18.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7 with the exceptions below.

Table 4.5A.18.4-1: RRCConnectionReconfiguration (step 8)

Derivation path: 36.508 table 4.6.1-8 using condition SRB2-DRB(2, 0)

NOTE: The bid for the AM DRBs shall be 1 and 8 instead of 1 and 2.

Table 4.5A.18.4-2: RRCConnectionReconfiguration (step 10)

Derivation path: 36.508 table 4.6.1-8 using condition DRB-REL(8)

Table 4.5A.18.4-3: DEACTIVATE EPS BEARER CONTEXT REQUEST (step 10)

| Derivation Path: TS 36.508 Table 4.7.3-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS bearer identity | (identity of default EPS bearer associated with the PDN to be released) |  |  |
| ESM cause | 00011010 | Insufficient resources |  |

Table 4.5A.18.4-4: DEACTIVATE EPS BEARER CONTEXT ACCEPT (step 12)

| Derivation Path: TS 36.508 Table 4.7.3-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS bearer identity | (same as in DEACTIVATE EPS BEARER CONTEXT REQUEST of step 10) | The same value as the value set in DEACTIVATE EPS BEARER CONTEXT REQUEST message. |  |
| Procedure transaction identity | 0 | No procedure transaction identity assigned |  |

### 4.5A.19	Generic Test Procedure for IMS MO speech call establishment in E-UTRA / EVS

#### 4.5A.19.1	Initial conditions

See clause 4.5A.6.1.

#### 4.5A.19.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.19.3	Procedure

Table 4.5A.19.3-1: EUTRA/EPS signalling for IMS MO speech call / EVS

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U – S | Message |
| 1-7 | Same as table 4.5A.6.3-1, steps 1-7. |  |  |
| - | EXCEPTION: In parallel to the events described in steps 8 below, the behaviour in table 4.5A.19.3-2 occurs. (IMS MTSI MO speech call establishment) | - | - |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer context. | --> | RRC: RRCConnectionReconfigurationComplete |
| 9-11 | Steps 3-4 expected sequence defined in annex C.44 of TS 34.229-1 [43]. MTSI MO speech call for EPS / EVS. | - | - |
| 12 | The SS configures a new RLC-UM data radio bearer with condition DRB (0,1), associated with the dedicated EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message. EPS bearer context #4 (QCI 1) according to table 6.6.2-1: Reference dedicated EPS bearer contexts. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| - | EXCEPTION: In parallel to the events described in steps 13-14 below, the behaviour in table 4.5A.19.3-3 occurs. (IMS MTSI MO speech call establishment) | - | - |
| 13 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the dedicated EPS bearer. | --> | RRC: RRCConnectionReconfigurationComplete |
| 14 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |

Table 4.5A.19.3-2: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Step 2 expected sequence defined in annex C.44 of TS 34.229-1 [43]. UE sends INVITE. | - | - |

Table 4.5A.19.3-3: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1-9 | Steps 5-13 expected sequence defined in annex C.44 of TS 34.229-1 [43]. MTSI MO speech call for EPS / EVS. | - | - |

#### 4.5A.19.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7.

### 4.5A.20	Generic Test Procedure for IMS MT speech call establishment in E-UTRA / EVS

#### 4.5A.20.1	Initial conditions

See clause 4.5A.6.1.

#### 4.5A.20.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.20.3	Procedure

Table 4.5A.20.3-1: EUTRA/EPS signalling for IMS MT speech call / EVS

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U – S | Message |
| 1-8 | Same as table 4.5A.7.3-1, steps 1-8. |  |  |
| 9-11b | Steps 1-6 expected sequence defined in annex C.45 of TS 34.229-1 [43], MTSI MT speech call for EPS / EVS. | - | - |
| 12-14 | Same as table 4.5A.7.3-1, steps 13-15. |  |  |
| 15-16 | Void | - | - |
| 17-25 | Steps 7-15 expected sequence defined in annex C.45 of TS 34.229-1 [43], MTSI MT speech call for EPS / EVS. | - | - |
| 26 | Void | - | - |

#### 4.5A.20.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7.

### 4.5A.21	Generic Test Procedure for IMS MO Customized Alerting Tones and speech establishment in E-UTRA

#### 4.5A.21.1	Initial conditions

See clause 4.5A.6.1

#### 4.5A.21.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.21.3	Procedure

Table 4.5A.21.3-1: EUTRA/EPS signalling for IMS MO CAT and speech call

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1-12 | See table 4.5A.6.3-1, steps 1-12 | - | - |
| - | EXCEPTION: In parallel to the events described in steps 13-14 below, the behaviour in table 4.5A.21.3-2 occurs. (IMS MTSI MO CAT and speech call establishment) | - | - |
| 13-14 | See table 4.5A.6.3-1, steps 13-14 | - | - |

Table 4.5A.21.3-2: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1-14 | Steps 5-18 expected sequence defined in 20.1.4 of TS 34.229-1 [43]. MTSI MO CAT and speech call for EPS. | - | - |

#### 4.5A.21.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7.

### 4.5A.22	Communication with the ProSe Function: Initial Access

#### 4.5A.22.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Registered, Idle Mode state (State 2).

#### 4.5A.22.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.22.3	Procedure

The procedure is only applicable for pc_Provide_ProSe_APN == true.

The HTTP signalling between the UE and the ProSe function is done over TLS i.e. the UE connects to TCP port 443 (HTTPS) and starts TLS handshake.

Table 4.5A.22.3-1: Communication with the ProSe Function

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The UE transmits an RRCConnectionRequest message with ' establishmentCause' set to 'mo-Data '. | --> | RRCConnectionRequest |
| 2 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 3 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 4 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 5 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 6 | The SS configures a new data radio bearer, associated with the default EPS bearer context. The RRCConnectionReconfiguration message is using condition SRB2-DRB(1, 0). The DRB associated with default EPS bearer context obtained during the attach procedure is established | <-- | RRC: RRCConnectionReconfiguration |
| 7 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer context. | --> | RRC: RRCConnectionReconfigurationComplete |
| 8-10 | Steps 1-3 of procedure 4.5A.16.3. | - | - |
| - | EXCEPTION: In parallel to the event described in step 11 below, the behaviour described in Table 4.5A.22.3-2 takes place. | - | - |
| 11 | The UE transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |

Table 4.5A.22.3-2: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that take place if the UE does not have a preconfigured IP address. | - | - |
| 1a1 | If initiated by the UE the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - |
| 2 | The UE transmits a ClientHello message including PSK-based ciphersuites. | --> | TLS: ClientHello |
| 3 | The SS transmits the ServerHello, ServerKeyExchange and ServerHelloDone messages indicating GBA as required. | <-- | TLS: ServerHello TLS: ServerKeyExchange TLS: ServerHelloDone |
| 4-7 | The generic procedure for GBA authentication according TS 34.229-1 [43] subclause C.29.2 (steps 1-4) takes place. | - | - |
| 8 | The UE transmits the ClientKeyExchange including a PSK identity, ChangeCipherSpec and Finished messages. | --> | TLS: ClientKeyExchange TLS: ChangeCipherSpec TLS: Finished |
| 9 | The SS transmits the ChangeCipherSpec and Finished messages. | <-- | TLS: ChangeCipherSpec TLS: Finished |
| - | EXCEPTION: Steps 10a1 to 10d2 describe behaviour which depends on the ProSe Procedure being applied | - | - |
| 10a1 | IF ANNOUNCE/MONITOR REQUEST THEN The UE transmits a DISCOVERY_REQUEST message over the PC3 (UE to ProSe Function) interface. | --> | HTTP Request containing DISCOVERY_REQUEST |
| 10a2 | The SS transmits a DISCOVERY_RESPONSE message over the PC3 (UE to ProSe Function) interface. | <-- | HTTP Response containing DISCOVERY_RESPONSE |
| 10a3 | The UE transmits a SidelinkUEInformation message. | --> | SidelinkUEInformation |
| 10b1 | IF MATCH REPORT THEN The UE transmits a MATCH_REPORT message over the PC3 (UE to ProSe Function) interface. | --> | HTTP Request containing MATCH_REPORT |
| 10b2 | The SS transmits a MATCH_REPORT_ACK message over the PC3 (UE to ProSe Function) interface. | <-- | HTTP Response containing MATCH_REPORT_ACK |
| 10c1 | IF UEREGISTRATION THEN The UE transmits an UE_REGISTRATION_REQUEST message over the PC3 (UE to ProSe Function) interface. | --> | HTTP Request containing UE_REGISTRATION_REQUEST |
| 10c2 | The SS transmits an APPLICATION_REGISTRATION_RESPONSE message with a response-register. | <-- | HTTP Response containing UE_REGISTRATION_RESPONSE |
| 10d1 | IF USAGEINFOREPORT THEN The UE transmits an USAGE_INFORMATION_REPORT_LIST message over the PC3ch (UE to ProSe Function) interface. | --> | HTTP Request containing USAGE_INFORMATION_REPORT_LIST |
| 10d2 | SS sends USAGE_INFORMATION_REPORT_LIST_RESPONSE message. | <-- | HTTP Response containing USAGE_INFORMATION_REPORT_LIST_RESPONSE |

| Condition | Explanation |
| --- | --- |
| ANNOUNCE/MONITOR REQUEST | Steps applicable to Announce request procedure OR Monitor request procedure |
| MATCH REPORT | Steps applicable to Match report procedure |
| UEREGISTRATION | Steps applicable to EPC-level ProSe discovery / UE registration procedure |
| USAGEINFOREPORT | Steps applicable to usage information report list sending procedure. |

#### 4.5A.22.4	Specific message contents

HTTP specific message contents shall be referred to clause 4.7E.

ProSe specific message contents shall be referred to clause 4.7F.

TLS specific message contents shall be referred to clause 4.7H.

Further specific message contents shall be referred to clause 4.6 and 4.7.

### 4.5A.22A	Communication with the ProSe Function: Subsequent Access

#### 4.5A.22A.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Registered Idle Mode state (State 2), or in state Generic RB Established (State 3).

-	The Communication with the ProSe Function: Initial Access in subclause 4.5A22 has taken place since the UE was switched on.

#### 4.5A.22A.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.22A.3	Procedure

The procedure is only applicable for pc_Provide_ProSe_APN == true.

The HTTP signalling between the UE and the ProSe function is done over an already established TLS connection.

Table 4.5A.22A.3-1: Communication with the ProSe Function

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Steps 1a1 to 1a7 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that takes place if the UE is in RRC_IDLE state. | - | - |
| 1a1 | The UE transmits an RRCConnectionRequest message with ' establishmentCause' set to 'mo-Data '. | --> | RRCConnectionRequest |
| 1a 2 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 1a 3 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 1a 4 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 1a 5 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 1a 6 | The SS configures two new data radio bearers, associated with the two default EPS bearer contexts. The RRCConnectionReconfiguration message is using condition SRB2-DRB(2, 0). | <-- | RRC: RRCConnectionReconfiguration |
| 1a 7 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearers, associated with the default EPS bearer contexts. | --> | RRC: RRCConnectionReconfigurationComplete |
| 2-10 | Void |  |  |
| - | EXCEPTION: Steps 10a1 to 10d2 describe behaviour which depends on the ProSe Procedure being applied | - | - |
| 10a1-10d2 | Steps 10a1-10d2 of Table 4.5A.22.3-2. | - | - |

#### 4.5A.22A.4	Specific message contents

HTTP specific message contents shall be referred to clause 4.7E.

ProSe specific message contents shall be referred to clause 4.7F.

Further specific message contents shall be referred to clause 4.6 and 4.7.

### 4.5A.23	Generic Test Procedure for IMS registration in EPC / WLAN

#### 4.5A.23.1	Initial conditions

System Simulator:

-	WLAN Cell 27 according to Table 4.4.8-1 with condition IMSoWLAN.

User Equipment:

-	The UE is in state Switched OFF (state 1).

#### 4.5A.23.2	Definition of system information messages

N/A

#### 4.5A.23.3	Procedure

Table 4.5A.23.3-1: IMS registration in EPC / WLAN

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The UE is switched on | - | - |
| 2 | The UE associates with the WLAN AP and obtains the local IP address | - | - |
| 3 | The UE performs a dynamic selection of ePDG using DNS query | - | - |
| 4 | The UE establishes an IPsec tunnel using the IKEv2 protocol as defined in 3GPP TS 33.402 [55] clause 8.2.2, Figure 8.2.2-1. | - | - |
| 5-13 | The UE performs the IMS registration procedure according TS 34.229-1 [43] subclause C.2c (steps 2-9). | - | - |

#### 4.5A.23.4	Specific message contents

None

### 4.5A.23A	Generic Test Procedure for IPsec Tunnel Disconnection in EPC / WLAN

#### 4.5A.23A.1	Initial conditions

System Simulator:

-	WLAN Cell 27 according to Table 4.4.8-1 with condition IMSoWLAN.

User Equipment:

-	The UE has established an ePDG Tunnel.

#### 4.5A.23A.2	Definition of system information messages

N/A

#### 4.5A.23A.3	Procedure

Table 4.5A.23A.3-1: Generic Test Procedure for IPsec Tunnel Disconnection in EPC / WLAN

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The UE initiates a disconnection from the existing IPsec tunnel as defined in 3GPP TS 24.302 [53] clause 7.2.4.1, which is acknowledged by the SS. |  |  |
| NOTE: It is assumed that the WLAN AP association remains throughout the procedure. |  |  |  |

#### 4.5A.23A.4	Specific message contents

None

### 4.5A.24	Generic Test Procedure for IMS emergency call establishment in EPC / WLAN

#### 4.5A.24.1	Initial conditions

System Simulator:

-	WLAN Cell 27 according to Table 4.4.8-1 with condition IMSoWLAN.

User Equipment:

-	The UE has selected an ePDG and is registered onto the IMS network according to clause 4.5A.23.

#### 4.5A.24.2	Definition of system information messages

N/A

#### 4.5A.24.3	Procedure

Table 4.5A.24.3-1: IMS emergency call establishment in EPC / WLAN

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The UE is triggered to initiate an emergency call | - | - |
| 2 | Void | - | - |
| 3 | The UE performs a dynamic selection of an ePDG that supports emergency services using DNS query | - | - |
| 4 | The UE establishes an additional IPsec tunnel using the IKEv2 protocol as defined in 3GPP TS 33.402 [55] clause 8.2.2, Figure 8.2.2-1. | - | - |
| NOTE 1: It is assumed that the WLAN AP association remains throughout the procedure. NOTE 2: This procedure is based on 3GPP Release 14. |  |  |  |

#### 4.5A.24.4	Specific message contents

None

### 4.5A.25	Generic Test Procedure for XCAP establishment in EPC / WLAN

This procedure is applicable only for the case when pc_WLAN_XCAP_without_PDN is set to false.

#### 4.5A.25.1	Initial conditions

System Simulator:

-	WLAN Cell 27 according to Table 4.4.8-1 with condition IMSoWLAN.

User Equipment:

-	The UE has selected an ePDG and is registered onto the IMS network according to clause 4.5A.23.

#### 4.5A.25.2	Definition of system information messages

N/A

#### 4.5A.25.3	Procedure

Table 4.5A.25.3-1: IMS XCAP establishment in EPC / WLAN

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Make the UE attempt XCAP signalling. | --> | - |
| 2 | The UE establishes an additional PDN and IPsec tunnel using the IKEv2 protocol as defined in 3GPP TS 33.402 [55] clause 8.2.2, Figure 8.2.2-1. | - | - |

#### 4.5A.25.4	Specific message contents

N/A

### 4.5A.26	Generic Test Procedure for eCall over IMS establishment in EUTRA: Normal Service

#### 4.5A.26.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE is in state Registered, Idle Mode (State 2)

#### 4.5A.26.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.26.3	Procedure

The establishment of eCall over IMS is assumed to always be mobile originated.

Note: The trigger to initiate MO call will be part of test case from where the generic procedure is called.

Table 4.5A.26.3-1: EUTRA/EPS signalling for eCall over IMS

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1-8a7 | Steps 3 to 10a7 of the generic radio bearer establishment procedure from TS 36.508 subclause 4.5.3.3-3 are executed | - | - |
| 9 | The UE transmits a PDN CONNECTIVITY REQUEST message to request an additional PDN, with 'Request type' set to 'emergency ('0100'B)'. | --> | NAS: PDN CONNECTIVITY REQUEST |
| 10 | The SS configures a new data radio bearer, associated with the default EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message. EPS bearer context #2 (QCI 5) according to table 6.6.1-1: Reference default EPS bearer context is used. Note: The APN is set to the test APN defined for emergency bearer services. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST |
| - | EXCEPTION: In parallel to the events described in steps 11 to 15 below, the behaviour in table 4.5A.26.3-2 occurs. (Optional IP address allocation followed by IMS emergency registration and eCall over IMS establishment) | - | - |
| 11 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer for emergency call. | --> | RRC: RRCConnectionReconfigurationComplete |
| 12 | The UE transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |
| 13 | The SS configures a new RLC-UM data radio bearer, associated with the dedicated EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message. EPS bearer context #4 (QCI 1) according to table 6.6.2-1: Reference dedicated EPS bearer contexts is used. Note: the same PDN address is applicable because the linked EPS bearer ID refers to the default EBC allocated in step 2 | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| 14 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer for emergency IMS signalling. | --> | RRC: RRCConnectionReconfigurationComplete |
| 15 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |

Table 4.5A.26.3-2: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Step 1 describes behaviour that depends on the UE behaviour. | - | - |
| 1 | If initiated by the UE the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - |
| 2-5 | Steps 1-4 expected sequence defined in annex C.20 of TS 34.229-1 [43]. Emergency registration procedure is performed. | - | - |
| 6-8 | Steps 1-3 defined in annex C.47 of TS 34.229-1 [43] Generic Test Procedure for NG eCall setup. | - | - |

#### 4.5A.26.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7 with the exceptions below.

Table 4.5A.26.4-1: PDN CONNECTIVITY REQUEST (step 9, Table 4.5A.26.3-1)

| Derivation Path: Table 4.7.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Request type | ‘0100’B | Emergency |  |
| Access point name | Not present or any value |  |  |

Table 4.5A.26.4-2: Message ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST (step 10, Table 4.5A.26.3-1)

| Derivation path: Table 4.7.3-6 and table 4.6.1-8 with condition AM-DRB-ADD(2) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Access point name | sos | APN value as recommended by IR.88 clause 6.4 [56] |  |
| EPS QoS | According to reference default EPS bearer context #2 – in table 6.6.1-1 | SS defines an additional dedicated EPS QoS |  |

Table 4.5A.26.4-3: Message ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST (step 13, Table 4.5A.26.3-1)

| Derivation path: Table 4.7.3-3 and table 4.6.1-8 with condition UM-DRB-ADD(3) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Linked EPS bearer identity | Default EBId-2 (same value like in table 4.5A.4.4-1) |  |  |
| EPS QoS | According to reference dedicated EPS bearer context #1 - in table 6.6.2-1 | SS defines an additional dedicated EPS QoS |  |
| TFT | According to reference dedicated EPS bearer context #1 - in table 6.6.2-1 |  |  |

### 4.5A.27	Generic Test Procedure for eCall over IMS establishment in EUTRA: eCall Only Support

#### 4.5A.27.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE is in state Switched OFF (State 1).

#### 4.5A.27.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.27.3	Procedure

The establishment of eCall over IMS is assumed to always be mobile originated.

Note: The generic procedure is defined as per TS 23.401 [24] clause 4.3.12.1.

Table 4.5A.27.3-1: EUTRA/EPS signalling for eCall over IMS (eCall only Support)

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The UE is switched on and SS waits 15 sec to allow the UE to camp on the serving cell and enter and remain in substate EMM-DEREGISTERED.eCALL-INACTIVE. | - | - |
| 2 | Make the UE attempt an eCall (See Note 1) | - | - |
| 3 | Void | 3 | Void |
| 4 | UE transmits an RRCConnectionRequest message with ‘establishmentCause’ set to ‘emergency’. | 4 | UE transmits an RRCConnectionRequest message with ‘establishmentCause’ set to ‘emergency’. |
| 5-17 | Steps 3 to 15 of the generic test procedure for UE registration sub clause 4.5.2.3. | - | - |
| - | EXCEPTION: In parallel to the event described in step 18 below, if initiated by the UE the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - |
| - | EXCEPTION: In parallel to the events described in steps 18-25 below the generic procedure for IMS signalling in the Uplane specified in subclause 4.5A.3 takes place if requested by the UE. | - | - |
| 18 | Step 16 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 is performed. | --> | RRC: ULInformationTransfer NAS: ATTACH COMPLETE NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |
| 19 | The UE transmits a PDN CONNECTIVITY REQUEST message to request an additional PDN, with 'Request type' set to 'emergency ('0100'B)'. | --> | NAS: PDN CONNECTIVITY REQUEST |
| 20 | The SS configures a new data radio bearer, associated with the default EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message. EPS bearer context #2 (QCI 5) according to table 6.6.1-1: Reference default EPS bearer context is used. Note: The APN is set to the test APN defined for emergency bearer services. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST |
| - | EXCEPTION: In parallel to the events described in steps 21 to 25 below, the behaviour in table 4.5A.27.3-2 occurs. (Optional IP address allocation followed by IMS emergency registration and eCall over IMS establishment) | - | - |
| 21 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer for emergency call. | --> | RRC: RRCConnectionReconfigurationComplete |
| 22 | The UE transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |
| 23 | The SS configures a new RLC-UM data radio bearer, associated with the dedicated EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message. EPS bearer context #4 (QCI 1) according to table 6.6.2-1: Reference dedicated EPS bearer contexts is used. Note: the same PDN address is applicable because the linked EPS bearer ID refers to the default EBC allocated in step 2 | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| 24 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer for emergency IMS signalling. | --> | RRC: RRCConnectionReconfigurationComplete |
| 25 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |
| Note 1: The request to originate a manual or Automatic eCall may be performed by MMI or AT command. |  |  |  |

Table 4.5A.27.3-2: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Step 1 describes behaviour that depends on the UE behaviour. | - | - |
| 1 | If initiated by the UE the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - |
| 2-5 | Steps 1-4 expected sequence defined in annex C.20 of TS 34.229-1 [43]. Emergency registration procedure is performed. | - | - |
| 6-8 | Steps 1-3 defined in annex C.47 of TS 34.229-1 [43] Generic procedure for NG eCall Setup. | - | - |

#### 4.5A.27.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7 with the exceptions below.

Table 4.5A.27.4-1: PDN CONNECTIVITY REQUEST (step 19)

| Derivation Path: Table 4.7.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Request type | ‘0100’B | Emergency |  |
| Access point name | Not present or any value |  |  |

Table 4.5A.27.4-2: Message ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST (step 20, Table 4.5A.27.3-1)

| Derivation path: Table 4.7.3-6 and table 4.6.1-8 with condition AM-DRB-ADD(2) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Access point name | sos | APN value as recommended by IR.88 clause 6.4 [56] |  |
| EPS QoS | According to reference default EPS bearer context #2 – in table 6.6.1-1 | SS defines an additional dedicated EPS QoS |  |

Table 4.5A.27.4-3: Message ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST (step 23, Table 4.5A.27.3-1)

| Derivation path: Table 4.7.3-3 and table 4.6.1-8 with condition UM-DRB-ADD(3) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Linked EPS bearer identity | Default EBId-2 (same value like in table 4.5A.4.4-1) |  |  |
| EPS QoS | According to reference dedicated EPS bearer context #1 - in table 6.6.2-1 | SS defines an additional dedicated EPS QoS |  |
| TFT | According to reference dedicated EPS bearer context #1 - in table 6.6.2-1 |  |  |

### 4.5A.28	Generic Test Procedure for Converged IP Communications establishment in EUTRA

#### 4.5A.28.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4.

User Equipment:

-	The UE is in state Switched OFF (state 1) according to [18].

#### 4.5A.28.2	Definition of system information messages

N/A

#### 4.5A.28.3	Procedure

Table 4.5A.28.3-1: Converged IP Communications service establishment in E-UTRA

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The UE is switched on |  |  |
| 2-16 | Steps 2-16 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - |
| 17-20 | Steps 1-4 of the generic procedure to establish an additional PDN Connection specified in TS 36.508 subclause 4.5A.16. |  |  |

#### 4.5A.28.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7 with the exceptions below.

Table 4.5A.26.4-1: PDN CONNECTIVITY REQUEST (step 17)

| Derivation Path: Table 4.7.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Access point name | Any allowed value | The UE includes a new APN. |  |

Table 4.5A.28.4-2: Message ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST (step 18)

| Derivation path: Table 4.6.1-8 with condition AM-DRB-ADD(1) for ADD_IMS or with condition AM-DRB-ADD(8) else |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS bearer identity | '0101'B | arbitrary value used for PDN connectivity being maintained during the test case | ADD_IMS_MultiMedia and SMS |
| EPS bearer identity | '1100'B | arbitrary value used for additional non-IMS PDN connectivity | ADD_IMS_Advanced_Messaging |
| Procedure transaction identity | PTI-1 | SS re-uses the particular PTI defined by UE for this present additional PDN connectivity request procedure |  |
| EPS QoS | See Reference default EPS bearer context #2 in table 6.6.1-1 |  |  |
| Access point name | APN as provided in ESM INFORMATION RESPONSE or PDN CONNECTIVITY REQUEST |  |  |

| Condition | Expression | Explanation |
| --- | --- | --- |
| ADD_IMS_MultiMedia and SMS | pc_IMS AND pc_Provide_IMS_as_second_APN | UE performs IMS registration for multi-media telephony and SMSoIP first, and IMS registration for advanced messaging services second. |
| ADD_IMS_Advanced_Messaging | pc_IMS AND NOT pc_Provide_IMS_as_second_APN | UE performs IMS registration for advanced messaging services first, and IMS registration for multi-media telephony and SMSoIP second. |

Table 4.5A.28.4-3: Message ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT (step 20)

| Derivation path: Table 4.7.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS bearer identity | (same value as used in step 2) |  |  |
| Procedure transaction identity | 0 | No procedure transaction identity assigned |  |

### 4.5A.29	Generic Test Procedure for Converged IP Communications establishment in EPC / WLAN

#### 4.5A.29.1	Initial conditions

System Simulator:

-	WLAN Cell 27 according to Table 4.4.8-1 in [18].

User Equipment:

-	The UE has selected an ePDG and is registered onto the IMS network according to 4.5A.23.

#### 4.5A.29.2	Definition of system information messages

N/A

#### 4.5A.29.3	Procedure

Table 4.5A.29.3-1: Converged IP Communications service establishment in EPC / WLAN

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The UE establishes an IPsec tunnel using the IKEv2 protocol as defined in 3GPP TS 33.402 [55] clause 8.2.2, Figure 8.2.2-1. | - | - |
| 2-9 | The UE performs the IMS registration procedure according TS 34.229-1 [43] subclause C.2c (steps 2-9). |  |  |
| NOTE: It is assumed that the WLAN AP association remains throughout the procedure. |  |  |  |

#### 4.5A.29.4	Specific message contents

FFS

### 4.5A.30	Generic Test Procedure for IMS Re-registration in E-UTRAN

The purpose of this procedure is to allow the successful completion of IMS Re-registration if it is initiated by the UE when moving from HPLMN to VPLMN and vice versa on E-UTRAN.

#### 4.5A.30.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

- 	The UE shall be in Registered, Idle Mode state (State 2).

#### 4.5A.30.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.30.3	Procedure

Table 4.5A.30.3-1: Procedure for IMS Re-registration in E-UTRAN

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Steps 1a1 to 1a2a10/1a2b1 describe behaviour that depends on the UE capability | - | - |
| 1a1 | IF pc_IMS then the SS starts timer Timer_1 = 10s. | - | - |
| - | EXCEPTION: Steps 1a2a1 to 1a2b1 describe a behaviour that depends on the UE implementation | - | - |
| 1a2a1-1a2a7 | Steps 3 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed. | - | - |
| 1a2a8-1a2a9 | The UE performs IMS re-registration using the generic procedure defined in 34.229-1 [43] Annex C.46. Note: SS cancels timer Timer_1 at step 1a2a8. | - | - |
| 1a2a10 | The SS releases the RRC connection. | - | - |
| 1a2b1 | The SS waits for Timer_1 expiry | - | - |

#### 4.5A.30.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7 with exceptions below.

REGISTER (Step 1a2a8)

Use the default message “REGISTER” in TS 34.229-1 [43] annex A.1.1 with conditions A2, A31 and (NOT A34).

### 4.5A.31	Generic Test Procedure for UUAA-SM in EPS

The purpose of this procedure is to allow the successful completion of USS UAV Authorization/Authentication - Session Management in EPS.

#### 4.5A.31.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Registered, Idle Mode state (State 2).

#### 4.5A.31.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.31.3	Procedure

Table 4.5A.31.3-1: Procedure for UUAA-SM in EPS

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U – S | Message |
| 1 | Configure the UE to request transport of UUAA parameters (see Note 1) | - | - |
| 2 | Cause the UE to request connectivity to an additional PDN for UAS services (see Note 2) | - | - |
| 3 | The UE transmits a SERVICE REQUEST message. | --> | SERVICE REQUEST |
| 4 | The SS establishes SRB2 and DRB associated with default EPS bearer context obtained during the attach procedure. | - | - |
| 5 | Check: Does the UE transmit a PDN CONNECTIVITY REQUEST message with service-level-AA container including CAA-level UAV ID requesting additional PDN? | --> | PDN CONNECTIVITY REQUEST |
| 6 | The SS transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message with IE EPS Bearer Identity set to new EPS bearer context. Note: the SS allocates a PDN address of a PDN type which is compliant with the PDN type requested by the UE. | <-- | ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST |
| - | EXCEPTION: In parallel to the event described in step 6 below, the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - |
| 7 | The UE transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message. | --> | ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |
| 8 | The SS transmits a MODIFY EPS BEARER CONTEXT REQUEST message including the EPS bearer identity of the default EPS bearer established at step 1, including service-level-AA container including service-level-AA response parameter with the SLAR field set to "Service level authentication and authorization was successful". This message is included in a DLInformationTransfer message | <-- | MODIFY EPS BEARER CONTEXT REQUEST |
| 9 | Check: Does the UE transmit a MODIFY EPS BEARER CONTEXT ACCEPT message? | --> | MODIFY EPS BEARER CONTEXT ACCEPT |
| 10 | The SS releases the RRC connection. | - | - |
| Note 1: The request for transport of UUAA parameters may be performed by MMI or AT command. Note 2: The request of connectivity to an additional PDN for UAS services may be performed by MMI or AT command. |  |  |  |

#### 4.5A.31.4	Specific message contents

Table 4.5A.31.4-1: Message PDN CONNECTIVITY REQUEST (step 5, Table 4.5A.31.3-1)

| Derivation Path: Table 4.7.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Extended protocol configuration options |  |  |  |
| Service-level-AA container | ‘0041’H |  |  |
| Length of Service-level-AA container contents | ‘00000000 00011101’B | 29 octets |  |
| Service-level-AA container contents |  |  |  |
| Service-level-AA parameter |  | Service-level device ID |  |
| Service-level device ID IEI | ‘10’H |  |  |
| Service-level device ID length | ‘08’H | 8 octets |  |
| Service-level device ID | ‘AABBCCDDEEFFAABB’H | configured using AT command |  |
| Service-level-AA parameter |  | Service-level-AA server address |  |
| Service-level-AA server address IEI | ‘20’H |  |  |
| Service-level-AA server address length | ‘20’H | 3 octets |  |
| Service-level-AA server address type | ‘01’H | IPv4 |  |
| Service-level-AA server address | ‘A B C D’H | 2 octets |  |
| Service-level-AA parameter |  | Service-level-AA payload type |  |
| Service-level-AA payload type IEI | ‘40’H |  |  |
| Service-level-AA payload type length | ‘01’H | 1 octet |  |
| Service-level-AA payload type | ‘01’H | UUAA payload |  |
| Service-level-AA parameter |  | Service-level-AA payload |  |
| Service-level-AA payload IEI | ‘70’H |  |  |
| Service-level-AA payload length | ‘0008’H | 8 octets |  |
| Service-level-AA payload | ‘AABBCCDDEEFFABAB’H | configured using AT command |  |

Table 4.5A.31.4-2: Message ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST (step 6, Table 4.5A.31.3-1)

| Derivation Path: TS 36.508 Table 4.7.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS bearer identity | 7 |  |  |
| Access point name | UAS APN | SS re-uses the particular APN defined by UE for this present additional PDN connectivity request procedure |  |

Table 4.5A.31.4-3: Message MODIFY EPS BEARER CONTEXT REQUEST (step 8, Table 4.5A.31.3-1)

| Derivation Path: Table 4.7.3-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS bearer identity | 7 | SS assigns the current default EPS bearer context. |  |
| Extended protocol configuration options |  |  |  |
| Service-level-AA container | ‘0041’H |  |  |
| Length of Service-level-AA container contents | ‘00000000 00011011’B | 27 octets |  |
| Service-level-AA container contents |  |  |  |
| Service-level-AA parameter |  | Service-level-AA response |  |
| Service-level-AA response IEI | ‘30’H |  |  |
| Service-level-AA response length | ‘01’H | 1 octet |  |
| Service-level-AA response | ‘00000010’B | Service level authentication and authorization was successful |  |
| Service-level-AA parameter |  | Service-level device ID |  |
| Service-level device ID IEI | ‘10’H |  |  |
| Service-level device ID length | ‘08’H | 8 octets |  |
| Service-level device ID | ‘AABBCCDDEEFFAABB’H | SS re-uses the Service-level device ID sent by UE for this present additional PDN connectivity request procedure |  |
| Service-level-AA parameter |  | Service-level-AA payload type |  |
| Service-level-AA payload type IEI | ‘40’H |  |  |
| Service-level-AA payload type length | ‘01’H | 1 octet |  |
| Service-level-AA payload type | ‘01’H | UUAA payload |  |
| Service-level-AA parameter |  | Service-level-AA payload |  |
| Service-level-AA payload IEI | ‘70’H |  |  |
| Service-level-AA payload length | ‘0008’H | 8 octets |  |
| Service-level-AA payload | ‘AABBCCDDEEFFABAB’H | SS re-uses the Service-level-AA payload sent by UE for this present additional PDN connectivity request procedure |  |

Table 4.5A.31.4-4: Message MODIFY EPS BEARER CONTEXT ACCEPT (step 9, Table 4.5A.31.3-1)

| Derivation path: Table 4.7.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS bearer identity | 7 | Same value as in MODIFY EPS BEARER CONTEXT REQUEST |  |
| Extended protocol configuration options |  |  |  |
| Service-level-AA container | ‘0041’H |  |  |
| Length of Service-level-AA container contents | ‘00000000 00001110’B | 14 octets |  |
| Service-level-AA container contents |  |  |  |
| Service-level-AA parameter |  | Service-level-AA payload type |  |
| Service-level-AA payload type IEI | ‘40’H |  |  |
| Service-level-AA payload type length | ‘01’H | 1 octet |  |
| Service-level-AA payload type | ‘01’H | UUAA payload |  |
| Service-level-AA parameter |  | Service-level-AA payload |  |
| Service-level-AA payload IEI | ‘70’H |  |  |
| Service-level-AA payload length | ‘0008’H | 8 octets |  |
| Service-level-AA payload | ‘AABBCCDDEEFFABAB’H |  |  |

### 4.5A.32	Generic Test Procedure for IMS bootstrap data channel establishment in E-UTRA

#### 4.5A.32.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall execute 4.5A.6 or 4.5A.7 or 4.5A.8 or 4.5A.9.

#### 4.5A.32.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.32.3	Procedure

Table 4.5A.32.3-1: EUTRA/EPS signalling for IMS bootstrap data channel establishment

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The SS configures a new RLC-UM data radio bearer with condition DRB (1,0), associated with the dedicated EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message. EPS bearer context #12 (QCI 9) according to table 6.6.2-1B: Reference dedicated EPS bearer contexts. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the dedicated EPS bearer. EPS bearer context #12 (QCI 9) according table to 6.6.2-1B: Reference dedicated EPS bearer contexts. | --> | RRC: RRCConnectionReconfigurationComplete |
| 3 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |

#### 4.5A.32.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7.

### 4.5A.33	Generic Test Procedure for IMS MO add RTT establishment in E-UTRA

#### 4.5A.33.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall execute 4.5A.19.

#### 4.5A.33.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.33.3	Procedure

Table 4.5A.33.3-1: EUTRA/EPS signalling for IMS MO add RTT

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Make the UE attempt add IMS RTT to the voice call. | - | - |
| 2-4 | Steps 2-4 expected sequence defined in expected sequence test case 17.19.4 of TS 34.229-1 [43]. | - | - |
| 5 | The SS configures a new RLC-UM data radio bearer with condition DRB (0,1), associated with the dedicated EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message. EPS bearer context #11 (QCI 1) according to table 6.6.2-1B: Reference dedicated EPS bearer contexts. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| - | EXCEPTION: In parallel to the events described in steps 6-7 below, the behaviour in table 4.5A.nn.3-2 occurs. (IMS MTSI MO RTT call establishment). | - | - |
| 6 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the dedicated EPS bearer. EPS bearer context #11 (QCI 1) according to table 6.6.2-1B: Reference dedicated EPS bearer contexts. | --> | RRC: RRCConnectionReconfigurationComplete |
| 7 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message for the first bearer. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |

Table 4.5A.33.3-2: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1-6 | Steps 5-10 expected sequence defined in test case 17.19.4 of TS 34.229-1 [43]. | - | - |

#### 4.5A.33.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7.

### 4.5A.34	Generic Test Procedure for IMS MT add RTT establishment in E-UTRA

#### 4.5A.34.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall execute 4.5A.7.

#### 4.5A.34.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.34.3	Procedure

Table 4.5A.34.3-1: EUTRA/EPS signalling for IMS MT add RTT

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1-5 | Steps 1-5 expected sequence defined in expected sequence test case 17.20.4 of TS 34.229-1 [43]. | - | - |
| 6 | The SS configures a new RLC-UM data radio bearer with condition DRB (0,1), associated with the dedicated EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message. EPS bearer context #11 (QCI 1) according to table 6.6.2-1b: Reference dedicated EPS bearer contexts. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| 7 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the dedicated EPS bearer. EPS bearer context #11 (QCI 1) according table to 6.6.2-1b: Reference dedicated EPS bearer contexts. | --> | RRC: RRCConnectionReconfigurationComplete |
| 8 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message for the first bearer. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |
| 9-13 | Steps 6-10 expected sequence defined in test case 17.20.4 of TS 34.229-1 [43], | - | - |

#### 4.5A.34.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7.

### 4.5A.35	Generic Test Procedure for Emergency call add RTT establishment in E-UTRA

#### 4.5A.35.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall execute 4.5A.4.

#### 4.5A.35.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.35.3	Procedure

Table 4.5A.35.3-1: EUTRA/EPS signalling for Emergency add RTT

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Make the UE attempt add IMS RTT to the emergency call. | - | - |
| 2-3 | Steps 2-3 expected sequence defined in expected sequence test case 19.1.8 of TS 34.229-1 [43]. | - | - |
| 4 | The SS configures a new RLC-UM data radio bearer with condition DRB (0,1), associated with the dedicated EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message. EPS bearer context #11 (QCI 1) according to table 6.6.2-1B: Reference dedicated EPS bearer contexts. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| - | EXCEPTION: In parallel to the events described in steps 5-6 below, the behaviour in table 4.5A.pp.3-2 occurs. (IMS MTSI MO RTT call establishment). | - | - |
| 5 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the dedicated EPS bearer. EPS bearer context #11 (QCI 1) according to table 6.6.2-1B: Reference dedicated EPS bearer contexts. | --> | RRC: RRCConnectionReconfigurationComplete |
| 6 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message for the first bearer. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |

Table 4.5A.35.3-2: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1-2 | Steps 4-5 expected sequence defined in test case 19.1.8 of TS 34.229-1 [43]. | - | - |

#### 4.5A.35.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7.

### 4.5A.36	Generic Test Procedure for IMS Emergency call and RTT establishment in EUTRA: Normal Service

#### 4.5A.36.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Registered, Idle Mode state (State 2).

#### 4.5A.36.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.36.3	Procedure

The establishment of IMS emergency call is assumed to always be mobile originated.

Table 4.5A.36.3-1: EUTRA/EPS signalling for IMS Emergency Call and RTT

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Make the UE attempt an IMS emergency call and RTT. | - | - |
| 2 | The UE transmits an RRCConnectionRequest message with 'establishmentCause' set to 'emergency'. | --> | RRCConnectionRequest |
| 3 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 4 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 5 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 6 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 7 | The SS configures a new data radio bearer, associated with the default EPS bearer context. The RRCConnectionReconfiguration message is using condition SRB2-DRB(1, 0). The DRB associated with default EPS bearer context obtained during the attach procedure is established | <-- | RRC: RRCConnectionReconfiguration |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer context. | --> | RRC: RRCConnectionReconfigurationComplete |
| 9 | The UE transmits a PDN CONNECTIVITY REQUEST message to request an additional PDN, with 'Request type' set to 'emergency ('0100'B )'. | --> | PDN CONNECTIVITY REQUEST |
| 10 | The SS configures a new data radio bearer, associated with the default EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message. EPS bearer context #2 (QCI 5) according to table 6.6.1-1: Reference default EPS bearer context is used. Note: The APN is set to the test APN defined for emergency bearer services. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST |
| - | EXCEPTION: In parallel to the events described in steps 11 to 15 below, the behaviour in table 4.5A.36.3-2 occurs. (Optional IP address allocation followed by IMS emergency registration and IMS emergency speech call establishment) | - | - |
| 11 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer for emergency call. | --> | RRC: RRCConnectionReconfigurationComplete |
| 12 | The UE transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |
| 13 | The SS configure new RLC-UM data radio bearers with condition DRB (0,2), associated with the dedicated EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST messages. EPS bearer context #4 (QCI 1) according to table 6.6.2-1 and EPS bearer context #11 (QCI 1) according to table 6.6.2-1B: Reference dedicated EPS bearer contexts. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| 14 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer for emergency IMS signalling. | --> | RRC: RRCConnectionReconfigurationComplete |
| 15 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT messages. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT NAS:ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |

Table 4.5A.36.3-2: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Step 1 describes behaviour that depends on the UE behaviour. | - | - |
| 1 | If initiated by the UE the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - |
| 2-5 | Steps 1-4 expected sequence defined in annex C.20 of TS 34.229-1 [43]. Emergency registration procedure is performed. | - | - |
| 6-10 | Steps 1-5 defined in annex C.54 of TS 34.229-1 [43]. IMS Emergency call and RTT for EPS is established. | - | - |

#### 4.5A.36.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7.

SKIPPED TEXT

## 4.9	Common test USIM, CSIM and ISIM parameters

This clause defines default parameters for programming the elementary files of the test UICC when running conformance test cases defined in 3GPP TS 36.523-1[18].

### 4.9.1	General

See clause 8.1 in 3GPP TS 34.108 [5].

#### 4.9.1.1	Definitions

See clause 8.1.1 in 3GPP TS 34.108 [5].

#### 4.9.1.2	Definition of the test algorithm for authentication

Same as clause 8.1.2 in 3GPP TS 34.108[5].

##### 4.9.1.2.1	Authentication and key derivation in the test USIM, CSIM and ISIM and SS

UE and SS calculate Ck, Ik, AUTN, RES[XRES] as in clause 8.1.2.1 in 3GPP TS 34.108 [5]. Derivation of KASME and other E-UTRA Keys shall be as defined in Annex A of 3GPP TS 33.401 [31], using Key derivation function HMAC-SHA-256 algorithm.

##### 4.9.1.2.2	Generation of re-synchronization parameters in the USIM, CSIM and ISIM

Same as clause 8.1.2.1 in 3GPP TS 34.108[5].

##### 4.9.1.2.3	Using the authentication test algorithm for UE conformance testing

See clause 8.1.2.3 in 3GPP TS 34.108 [5].

### 4.9.2	Default parameters for the test USIM, CSIM and ISIM

Same as clause 8.2 in 3GPP TS 34.108 [5].

### 4.9.3	Default settings for the Elementary Files (EFs)

The format and coding of elementary files of the USIM are defined in 3GPP TS 31.101 [32] and 3GPP TS 31.102 [33] . Those of the ISIM are defined in 3GPP TS 31.101 [32] and 3GPP TS 31.103 [45]. Those of the CSIM are defined in 3GPP2 C.S0065-B [48].

The settings of the elementary files at the MF and USIM ADF (Application DF) level resp. the ISIM ADF (Application DF) level are the same as section 8.3 in 3GPP TS 34.108 [5] resp. section 4.2 in 3GPP TS 34.229-1 Annex E [43] and 3GPP TS 34.229-3 Annex B.1 [47] with the exceptions listed below. The settings of the elementary files at the CSIM ADF (Application DF) level as specified below. Note that some files may be updated by the UE based on information received from the SS.