3GPP TSG RAN5 Meeting #109	 	R5-256461

Dallas, Texas, USA, 17th - 21st November 2025

Source:		Rohde & Schwarz

Title:	On the E-UTRA uplink power for test of intra-band EN-DC MSD

Agenda Item:		5.4.20

Document for:	Discussion and Endorsement

# Introduction

In the last meeting, RAN5 received an LS from RAN4 on E-UTRA uplink configurations for intra-band ENDC MSD [1] and the following action point has been created:

| Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| AP#108.22 | RF | In RAN5 test procedure, only PUCCH is transmitted on E-UTRA carrier, figure out methods to avoid the interference from PUCCH on the REFSENS measurement, including the option of configuring the PUCCH transmit power to minimum output power. | Huawei, Keysight, R&S, Anritsu, Apple | R5-253716 | RAN5#109 | Open |

In this contribution, we present the results of our analysis.

# Discussion

In the LS [1], RAN4 states the following on the UL power configuration for E-UTRA:

- E-UTRA transmission power: RAN4 recommends configuring the E-UTRA transmit power as anchor agnostic, or preferably at minimum output power.

Observation 1: RAN4 recommends configuring E-UTRA transmit power as anchor agnostic, or preferably at minimum output power.

The minimum conformance requirement for RefSens MSD is specified in clause 7.3B.2 of TS 38.101-3 [2]:

Table 7.3B.2.1-1: Reference sensitivity (MSD) for intra-band contiguous EN-DC

| EN-DC configuration / channel allocations /MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| DC_(n)3AA | 3 | N/A | 15 | N/A | 1842.5 | 6.2 | FDD |
|  | n3 | 1770.0 | 30 | 10 (RBstart = 150) | 1865.0 | N/A |  |
| DC_(n)5AA | 5 | N/A | 5 | N/A | 871.5 | 5.2 | FDD |
|  | n5 | 839 | 20 | 20 (RBend = 105) | 884 | N/A |  |
| DC_(n)12AA | 12 | N/A | 5 | N/A | 733.5 | 4.5 | FDD |
|  | n12 | 711 | 10 | 20 (RBend = 51) | 741 | N/A |  |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBstart = 0) | 622 | 17.2 | FDD |
|  | n71 | 678 | 10 | 10 (RBend = 51) | 632 | 29.4 |  |
| DC_(n)71AA4 | 71 | 668 | 10 | 10 (RBstart = 0) | 622 | 17.2 |  |
|  | n71 | 678 | 101 | 10 (RBend = 51) | 634.5 | 29.1 |  |
| DC_(n)71AA3 | 71 | N/A | 10 | N/A | 642.0 | 21.3 |  |
|  | n71 | 673.0 | 20 | 5 (RBstart = 0) | 627.0 | N/A |  |
| DC_(n)71AA4 | 71 | N/A | 5 | N/A | 639.5 | 6.8 |  |
|  | n71 | 670.5 | 151 | 5 (RBstart = 2) | 627.0 | N/A |  |
| DC_(n)71AA5 | 71 | N/A | 5 | N/A | 634.5 | 6.4 |  |
|  | n71 | 670.5 | 15 | 5 (RBstart = 0) | 624.5 | N/A |  |
| NOTE 1: In accordance to BCS1, the NR uplink bandwidth is specified as in this table, but the corresponding NR downlink bandwidth is 5 MHz larger. NOTE 2: The transmitters powers shall be set to PUMAX, as defined in TS 38.101-1 [2], TS 38.101-2 [3], and TS 36.101 [4], with additional limits on configured maximum output power for the uplink according to clause 6.2B.4. NOTE 3: Applicable only to BCS 2. NOTE 4: Applicable only to BCS 1. NOTE 5: Applicable only to BCS 0. |  |  |  |  |  |  |  |

Note 2 in Table 7.3B.2.1-1 of TS 38.101-3 [2] requires setting the transmitters powers to PUMAX.

Observation 2: The minimum conformance requirement in Table 7.3B.2.1-1 1 of TS 38.101-3 [2] requires setting the transmitters powers to PUMAX.

The LS from RAN4 contradicts the minimum requirement specified in TS 38.101-3. We therefore suggest that RAN4 should update the requirement in TS 38.101-3.

Proposal 1: We propose to send an LS to RAN4 requesting an update of the specified transmitters power in Table 7.3B.2.1-1 1 of TS 38.101-3 [2].

In order to reduce the transmitter power of E-UTRA, we propose to set P-MaxEUTRA in a message exception to a TBD power level.

Proposal 2: Set P-MaxEUTRA in a message exception to a TBD power level in test case 7.3B.2.1 in TS 38.521-3 [3] when the UL allocation is N/A.

Other methods such as the power control window method we evaluate as not suitable due to the increased test procedure complexity.

# Conclusion

In summary, we have analyzed the E-UTRA uplink power for test of intra-band ENDC MSD and make the following proposals:

Observation 1: RAN4 recommends configuring E-UTRA transmit power as anchor agnostic, or preferably at minimum output power.

Observation 2: The minimum conformance requirement in Table 7.3B.2.1-1 1 of TS 38.101-3 [2] requires setting the transmitters powers to PUMAX.

Proposal 1: We propose to send an LS to RAN4 requesting an update of the specified transmitters power in Table 7.3B.2.1-1 1 of TS 38.101-3 [2].

Proposal 2: Set P-MaxEUTRA in a message exception to a TBD power level in test case 7.3B.2.1 in TS 38.521-3 [3] when the UL allocation is N/A.

# References

- R5-253716, LS on E-UTRA uplink configurations for intra-band ENDC MSD, TSG WG RAN4, 3GPP TSG RAN WG 5 Meeting #108, August 2025

- TS 38.101-3, V19.3.0, 3GPP TSG RAN WG4, September 2025

- TS 38.521-3, V19.2.0, 3GPP TSG RAN WG5, September 2025