3GPP TSG-RAN5 Meeting #108	R5-256535

# Dallas, USA, 17th Nov 2025 - 21st Nov 2025

Agenda item:	5.4.20

Source: 	Apple Inc

Title:       	Evaluation and optimization of OOB measurement points and 	test procedure

Document for:	DISCUSSION AND ENDORSEMENT

# 1	Introduction

The purpose of this paper is to provide some views on the RF Action point #107.21 regarding possible optimization of test points for the FR1 OOB test case. The Out-of-Band Blocking test essentially verifies that a CW interferer outside the receive band does not degrade the UE below the relative-throughput requirement. Test time grows quickly across many bands and CA/DC combinations, largely due to broad far-out frequency coverage that seldom impacts performance. The proposals try to highlight some relevant test points that keep the close-in frequency rigor intact while narrowing far-out testing to physically relevant points. And can potentially be included in a broader framework to optimize the testing of this scenario.

# 2	Discussion

## 2.1 Possible test points of focus

## To start with, we propose that the execution of the contiguous frequency ranges adjacent to the DL band edges continues exactly as currently specified in 38.521-1. No change is proposed for this test item. In fact, we probably only need to test the close-in blocker and spurious response as there should not be concerns about the IMD caused by wanted signal and CW blocker (most of them are not valid test points as they are in-band) nor the harmonics of wanted signal with the CW blocker as the IMD products are expected to be weaker than the wanted signal (no concern for QPSK). However, at this point it might be OK to consider few other test points that are not close-in.

## Observation 1: There are no concerns about the IMD caused by wanted signal and CW blocker (most of them are not valid test points as they are in-band) nor the harmonics of wanted signal with the CW blocker as the IMD products are expected to be weaker than the wanted signal (no concern for QPSK).

## Looking at what we consider the "far-out" or "far-away" ranges,

## • Far-out (|Δf| ≥ 100 MHz from FDL_low/FDL_high): Require testing only at interferer 		frequencies that, by definition of the test configuration, can generate receiver responses 	within the UE operating receive band. Concretely:

## •	IM3 candidates that fall inside the receive band:

## •	IM5 candidates that fall inside the receive band:

## •	Harmonic and fractional-harmonic interferer placements commonly 			used for conformance (execute only those outside the Rx band):

## •	Execution within any qualifying interval: Use the existing step 					size defined for this test item and the existing interferer power levels.

## Proposal 1 — Keep close-in coverage as currently specified; restrict far-out coverage, which is a lesser contributor to test failures, to interferer frequencies that create in-band responses as outlined above.

For CA/DC, a similar analysis as proposal 1 can be performed to identify the explicit interferer-frequency window/points that meet proposal 1. The union of qualifying windows across component carriers whenever an IM3/IM5/harmonic condition yields an in-band response in any CC, can be captured in an informative Annex. If no such windows exist for a combination, it shall be considered that there are no qualifying test points.

## Proposal 2 — Further analyze for CA/DC combinations to identify explicit interferer-frequency window/points that meet proposal 1. Add an informative annex which lists the combos where union of qualifying windows across component carriers, whenever an IM3/IM5/harmonic condition yields an in-band response

# Summary

## Observation 1: There are no concerns about the IMD caused by wanted signal and CW blocker (most of them are not valid test points as they are in-band) nor the harmonics of wanted signal with the CW blocker as the IMD products are expected to be weaker than the wanted signal (no concern for QPSK).

## Proposal 1 — Keep near-edge coverage as specified; restrict far-out coverage, which is a lesser contributor to test failures, to interferer frequencies that create in-band responses as outlined above.

## Proposal 2 — Further analyze for CA/DC combinations to identify explicit interferer-frequency window/points that meet proposal 1. Add an informative annex which lists the combos where union of qualifying windows across component carriers, whenever an IM3/IM5/harmonic condition yields an in-band response

# 3 References

RF AP#107.21 - Further evaluate/optimize OOB measurement points/test procedure in single band and CA/DC configurations considering implications (in test time reduction, Regulatory, implementation consistency etc).

R5-252669 “Discussion on how to reduce the measurement complexity of Out-of-Band Blocking in RAN5”, Huawei, HiSilicon; Malta; May 2025

R5-254633 “Discussion on the optimization of Out-of-band Blocking (OOBB) testing”, Huawei, HiSilicon, Bengaluru, India; August 2025