3GPP TSG-RAN5 Meeting #110	R5-260776

# Gothenburg, Sweden, Feb.09th – Feb.13st 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5297 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR V2X SIG test case 12.2.10.1.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2026-01-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | Update V2X SIG test case 12.2.10.1.3 to capture comments received: - Wording of TP needs to be updated; - Wording of test procedure needs to be updated. - Pre-condition needs to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Following changes are made: - TPs; - test procedure; - Pre-condition is simplified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.2.10.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

12.2.10.1.3	Inter-concurrent operation / MAC / SL-SCH data transmission / Selected sidelink grant

12.2.10.1.3.1	Test Purpose (TP)

(1)

with { UE is configured to perform mode 2 sidelink transmission }

ensure that {

when { UE has data to transmit on a logical channel configured with sl-HARQ-FeedbackEnabled = enabled }

then { UE selects resource pool configured with PSFCH resources and uses it for transmission }

}

(2)

with { UE is configured to perform mode 2 sidelink transmission }

ensure that {

when { UE has data to transmit on a logical channel }

then { UE creates a selected SL grant based on configurations and CBR measurement results and uses it for transmission }

}

(3)

with { UE is configured to perform mode 2 sidelink transmission }

ensure that {

when { UE receives a NACK on PSFCH corresponding to the sent PSSCH }

then { UE re-transmits the SL-SCH MAC PDU }

}

12.2.10.1.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, subclause 5.22.1.1, 5.22.1.2 and 5.22.1.3.1. Unless otherwise stated these are Rel-16 requirements.

[TS 38.321, subclause 5.22.1.1]

…

1>	if the MAC entity has selected to create a selected sidelink grant corresponding to transmission(s) of a single MAC PDU, and if SL data is available in a logical channel, or an SL-CSI reporting is triggered:

2>	if SL data is available in the logical channel:

3>	if sl-HARQ-FeedbackEnabled is set to enabled for the logical channel:

4>	select any pool of resources configured with PSFCH resources among the pools of resources;

3>	else:

4>	select any pool of resources among the pools of resources;

…

2>	perform the TX resource (re-)selection check on the selected pool of resources as specified in clause 5.22.1.2;

2>	if the TX resource (re-)selection is triggered as the result of the TX resource (re-)selection check:

3>	select the number of HARQ retransmissions from the allowed numbers, if configured by RRC, in sl-MaxTxTransNumPSSCH included in sl-PSSCH-TxConfigList and, if configured by RRC, overlapped in sl-MaxTxTransNumPSSCH indicated in sl-CBR-PriorityTxConfigList for the highest priority of the logical channel(s) allowed on the carrier and the CBR measured by lower layers according to clause 5.1.27 of TS 38.215 [24] if CBR measurement results are available or the corresponding sl-defaultTxConfigIndex configured by RRC if CBR measurement results are not available;

3>	select an amount of frequency resources within the range, if configured by RRC, between sl-MinSubChannelNumPSSCH and sl-MaxSubChannelNumPSSCH included in sl-PSSCH-TxConfigList and, if configured by RRC, overlapped between sl-MinSubChannelNumPSSCH and sl-MaxSubChannelNumPSSCH indicated in sl-CBR-PriorityTxConfigList for the highest priority of the logical channel(s) allowed on the carrier and the CBR measured by lower layers according to clause 5.1.27 of TS 38.215 [24] if CBR measurement results are available or the corresponding sl-defaultTxConfigIndex configured by RRC if CBR measurement results are not available;

…

3>	else:

4>	randomly select the time and frequency resources for one transmission opportunity from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7], according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier, and/or the latency requirement of the triggered SL-CSI reporting;

3>	if one or more HARQ retransmissions are selected:

4>	if transmission based on sensing is configured by upper layers and there are available resources left in the resources indicated by the physical layer according to clause 8.1.4 of TS 38.214 [7] for more transmission opportunities; or

…

5>	randomly select the time and frequency resources for one or more transmission opportunities from the available resources, according to the amount of selected frequency resources, the selected number of HARQ retransmissions and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier, and/or the latency requirement of the triggered SL-CSI by ensuring the minimum time gap between any two selected resources in case that PSFCH is configured for this pool of resources, and that a retransmission resource can be indicated by the time resource assignment of a prior SCI according to clause 8.3.1.1 of TS 38.212 [9];

5>	consider a transmission opportunity which comes first in time as the initial transmission opportunity and other transmission opportunities as the retransmission opportunities;

5>	consider all the transmission opportunities as the selected sidelink grant;

…

3>	use the selected sidelink grant to determine PSCCH duration(s) and PSSCH duration(s) according to TS 38.214 [7].

[TS 38.323, subclause 5.22.1.2]

If the TX resource (re-)selection check procedure is triggered on the selected pool of resources for a Sidelink process according to clause 5.22.1.1, the MAC entity shall for the Sidelink process:

…

1>	if there is no selected sidelink grant on the selected pool of resources; or

…

2>	clear the selected sidelink grant associated to the Sidelink process, if available;

2>	trigger the TX resource (re-)selection.

[TS 38.323, subclause 5.22.1.3.1]

For each sidelink grant, the Sidelink HARQ Entity shall:

1>	if the MAC entity determines that the sidelink grant is used for initial transmission as specified in clause 5.22.1.1; or

…

2>	(re-)associate a Sidelink process to this grant, and for the associated Sidelink process:

NOTE 1A:	The Sidelink HARQ Entity will associate the selected sidelink grant to the Sidelink process determined by the MAC entity.

3>	obtain the MAC PDU to transmit from the Multiplexing and assembly entity, if any;

3>	if a MAC PDU to transmit has been obtained:

4>	if a HARQ Process ID has been set for the sidelink grant:

5>	(re-)associate the HARQ Process ID corresponding to the sidelink grant to the Sidelink process;

NOTE 1a:	There is one-to-one mapping between a HARQ Process ID and a Sidelink process in the MAC entity configured with Sidelink resource allocation mode 1.

4>	determines Sidelink transmission information of the TB for the source and destination pair of the MAC PDU as follows:

5>	set the Source Layer-1 ID to the 8 LSB of the Source Layer-2 ID of the MAC PDU;

5>	set the Destination Layer-1 ID to the 16 LSB of the Destination Layer-2 ID of the MAC PDU;

5>	(re-)associate the Sidelink process to a Sidelink process ID;

NOTE 1b:	How UE determine Sidelink process ID in SCI is left to UE implementation for NR sidelink.

5>	consider the NDI to have been toggled compared to the value of the previous transmission corresponding to the Sidelink identification information and the Sidelink process ID of the MAC PDU and set the NDI to the toggled value;

…

5>	else:

6>	set the cast type indicator to one of broadcast, groupcast and unicast as indicated by upper layers.

5>	if HARQ feedback has been enabled for the MAC PDU according to clause 5.22.1.4.2;

6>	set the HARQ feedback enabled/disabled indicator to enabled.

…

5>	set the priority to the value of the highest priority of the logical channel(s), if any, and a MAC CE, if included, in the MAC PDU;

…

5>	set the Redundancy version to the selected value.

4>	deliver the MAC PDU, the sidelink grant and the Sidelink transmission information of the TB to the associated Sidelink process;

4>	instruct the associated Sidelink process to trigger a new transmission.

3>	else:

4>	flush the HARQ buffer of the associated Sidelink process.

1>	else (i.e. retransmission):

…

2>	else:

3>	identify the Sidelink process associated with this grant, and for the associated Sidelink process:

4>	deliver the sidelink grant of the MAC PDU to the associated Sidelink process;

4>	instruct the associated Sidelink process to trigger a retransmission.

12.2.10.1.3.3	Test description

12.2.10.1.3.3.1	Pre-test conditions

System Simulator:

-	SS-NW

-	NR Cell 1

-	System information combination NR-14 as defined in TS 38.508-1 [4] clause 4.4.3.1 is used in NR cell.

-	NR-SS-UE

-	NR-SS-UE1 and NR-SS-UE2 are as defined in TS 38.508-1 [4], configured for and operating as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

-	NR-SS-UE1 and NR-SS-UE2 use GNSS as the synchronization reference source.

-	GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

-	UE is authorised to perform NR sidelink communication.

-	The UE is equipped with a USIM containing default values as per TS 38.508-1 [4] clause 4.8.3.3.3.

-	The UE uses GNSS as the synchronization reference source.

Table 12.2.10.1.3.3.1-1: Void

Preamble:

-	The UE is in state 1N-B as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), Cast Type (Unicast), GNSS Sync (On) using UE initiated unicast mode NR sidelink communication procedure in subclause 4.9.22 with NR-SS-UE1.

12.2.10.1.3.3.2	Test procedure sequence

Table 12.2.10.1.3.3.2-1 illustrates the downlink power levels to be applied for NR-SS-UEs at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while the configuration marked "T1", is applied at the point indicated in the Main behaviour description in Table 12.2.10.1.3.3.2-2.

Table 12.2.10.1.3.3.2-1: Time instances of NR-SS-UE power level and parameter changes for FR1

|  | Parameter | Unit | NR-SS-UE 1 | NR-SS-UE 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | NR-SS-UE power | dBm/ SCS | -85 | OFF | Power level to ensure that transmit power of NR-SS-UE 2 is lower than sl-ThreshS-RSSI-CBR-r16 |
| T1 | NR-SS-UE power | dBm/ SCS | -85 | -85 | Power level to ensure that transmit power of NR-SS-UE 2 is higher than sl-ThreshS-RSSI-CBR-r16 |

Table 12.2.10.1.3.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS triggers UE to close UE test loop mode E (Transmit Mode). NOTE: Closing of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| 2 | Check: Does the UE transmit MAC PDUs in any of the last 5 slots of every 10 slots? | --> | MAC PDU | 1 | F |
| 3 | Check: Does the UE transmit MAC PDU in any of the first 5 slot of every 10 slots in the next 5s using following parameters? - MCS index = 14, - Number of subchannels = 3, - doesn’t do any retransmission. | --> | MAC PDU | 2 | P |
| 4 | The SS triggers UE to open UE test loop mode E. NOTE: Opening of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| 5 | The SS re-adjusts the NR-SS-UEs power levels according to row "T1" in Table 12.2.10.1.3.3.2-1. | - | - | - | - |
| 6 | The NR-SS-UE2 continuously transmits on every subchannels in the first 3 of every 10 slots (i.e. CBR reaches 60%). |  |  |  |  |
| 7 | Wait 1s to let the UE acquire CBR measurement results. | - | - | - | - |
| 8 | The SS triggers UE to close UE test loop mode E (Transmit Mode). NOTE: Closing of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| 9 | Check: Does the UE transmit MAC PDU in any of the first 5 slot of every 10 slots in the next 5s using following parameters? - MCS index = 27, - Number of subchannels = 2, - time-frequency resources reserved for one re-transmission. | --> | MAC PDU | 2 | P |
| 10 | The NR-SS-UE1 sends a negative acknowledgement via PSFCH | <-- | PSFCH(NACK) | - | - |
| 11 | Check: Does the UE re-transmit the MAC PDU using the time-frequency resources reserved for re-transmission, using the same parameters as initial transmission? | --> | MAC PDU | 3 | P |

12.2.10.1.3.3.3	Specific message contents

Table 12.2.10.1.3.3.3-1: SIB12-IEs (Preamble, NR Cell 1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-14A |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SIB12-IEs-r16 ::= SEQUENCE { |  |  |  |  |  |  |  |
| sl-ConfigCommonNR-r16 SEQUENCE { |  |  |  |  |  |  |  |
| sl-FreqInfoList-r16 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfigCommon-r16 { |  | 1 entry |  |  |  |  |  |
| SL-FreqConfigCommon-r16[1] SEQUENCE { |  |  |  | entry 1 |  |  |  |
| sl-BWP-List-r16 SEQUENCE (SIZE (1..maxNrofSL-BWPs-r16)) OF SL-BWP-ConfigCommon-r16 { |  | 1 entry |  |  |  |  |  |
| SL-BWP-ConfigCommon-r16[1] SEQUENCE { |  |  |  | entry 1 |  |  |  |
| sl-BWP-PoolConfigCommon-r16 |  | SL-BWP-PoolConfigCommon |  | Table 12.2.10.1.3.3.3-2 |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| sl-UE-SelectedConfig-r16 SEQUENCE { |  |  |  |  |  |  |  |
| sl-PSSCH-TxConfigList-r16 |  | SL-PSSCH-TxConfigList |  | Table 12.2.10.1.3.3.3-6 |  |  |  |
| sl-CBR-CommonTxConfigList-r16 |  | SL-CBR-CommonTxConfigList |  | Table 12.2.10.1.3.3.3-7 |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 12.2.10.1.3.3.3-2: SL-BWP-PoolConfigCommon (Table 12.2.10.1.3.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.6-4 with condition RXPOOL and SELECTED |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-BWP-PoolConfigCommon-r16 ::= SEQUENCE { |  |  |  |  |
| sl-TxPoolSelectedNormal-r16 SEQUENCE (SIZE (1..maxNrofTXPool-r16)) OF SL-ResourcePoolConfig-r16 { |  | 2 entries |  |  |
| SL-ResourcePoolConfig-r16[1] SEQUENCE { |  |  | entry 1 |  |
| sl-ResourcePoolID-r16 |  | 1 |  |  |
| sl-ResourcePool-r16 |  | SL-ResourcePool-1 | Table 12.2.10.1.3.3.3-3 |  |
| } |  |  |  |  |
| SL-ResourcePoolConfig-r16[2] SEQUENCE { |  |  | entry 2 |  |
| sl-ResourcePoolID-r16 |  | 2 |  |  |
| sl-ResourcePool-r16 |  | SL-ResourcePool-2 | Table 12.2.10.1.3.3.3-5 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 12.2.10.1.3.3.3-3: SL-ResourcePool-1 (Table 12.2.10.1.3.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.6-25 with condition SL_HARQ |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |  |
| sl-UE-SelectedConfigRP-r16 SEQUENCE { |  |  |  |  |
| sl-CBR-PriorityTxConfigList-r16 |  | SL-CBR-PriorityTxConfigList | Table 12.2.10.1.3.3.3-4 |  |
| } |  |  |  |  |
| sl-TimeResource-r16 |  | 1111100000 | First 5 of every 10 logical slots |  |
| } |  |  |  |  |

Table 12.2.10.1.3.3.3-4: SL-CBR-PriorityTxConfigList (Table 12.2.10.1.3.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.6-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-CBR-PriorityTxConfigList -r16 ::= SEQUENCE (SIZE (1..8)) OF SL-PriorityTxConfigIndex-r16 { |  | 1 entry |  |  |
| SL-PriorityTxConfigIndex-r16[1] SEQUENCE { |  |  | entry 1 |  |
| sl-PriorityThreshold-r16 |  | 8 | Apply to all priorities |  |
| sl-DefaultTxConfigIndex-r16 |  | Not present |  |  |
| sl-CBR-ConfigIndex-r16 |  | 0 | 1st entry of SL-CBR-CommonTxConfigList-r16 is used |  |
| sl-Tx-ConfigIndexList-r16 SEQUENCE (SIZE (1.. maxCBR-Level-r16)) OF SL-TxConfigIndex-r16 { |  | 2 entries |  |  |
| SL-TxConfigIndex-r16[1] |  | 0 | entry 1 the first entry in sl-CBR-PSSCH-TxConfigList-r16, i.e. the entry for CBR <50% |  |
| SL-TxConfigIndex-r16[2] |  | 1 | entry 2 the second entry in sl-CBR-PSSCH-TxConfigList-r16, i.e. the entry for CBR >50% |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 12.2.10.1.3.3.3-5: SL-ResourcePool-2 (Table 12.2.10.1.3.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.6-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |  |
| sl-TimeResource-r16 |  | 0000011111 | Last 5 of every 10 logical slots |  |
| } |  |  |  |  |

Table 12.2.10.1.3.3.3-6: SL-PSSCH-TxConfigList (Table 12.2.10.1.3.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.6-19 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment |  | Condition |
| SL-PSSCH-TxConfigList-r16 ::= SEQUENCE (SIZE (1..maxPSSCH-TxConfig-r16)) OF SL-PSSCH-TxConfig-r16 { |  | 1 entry |  |  |  |
| SL-PSSCH-TxConfig-r16[1] SEQUENCE { |  |  | entry 1 |  |  |
| sl-ParametersAboveThres-r16 SEQUENCE { |  |  |  |  |  |  |
| sl-MinMCS-PSSCH-r16 |  |  | 0 |  |  |  |
| sl-MaxMCS-PSSCH-r16 |  |  | 27 |  |  |  |
| sl-MinSubChannelNumPSSCH-r16 |  |  | 1 |  |  |  |
| sl-MaxSubchannelNumPSSCH-r16 |  |  | 27 |  |  |  |
| sl-MaxTxTransNumPSSCH-r16 |  |  | 4 |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |

Table 12.2.10.1.3.3.3-7: SL-CBR-CommonTxConfigList (Table 12.2.10.1.3.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.6-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-CBR-CommonTxConfigList-r16 ::= SEQUENCE { |  |  |  |  |
| sl-CBR-PSSCH-TxConfigList-r16 SEQUENCE (SIZE (1.. maxTxConfig-r16)) OF SL-CBR-PSSCH-TxConfig-r16 { | 2 entries |  |  |
| SL-CBR-PSSCH-TxConfig-r16[1] SEQUENCE { |  | Entry 1 Entry for CBR < 50% |  |
| sl-CR-Limit-r16 | 10000 |  |  |
| sl-TxParameters-r16 SEQUENCE { |  |  |  |
| sl-MinMCS-PSSCH-r16 | 14 |  |  |
| sl-MaxMCS-PSSCH-r16 | 14 |  |  |
| sl-MinSubChannelNumPSSCH-r16 | 3 |  |  |
| sl-MaxSubchannelNumPSSCH-r16 | 3 |  |  |
| sl-MaxTxTransNumPSSCH-r16 | 1 | No re-transmission |  |
| sl-MaxTxPower-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| SL-CBR-PSSCH-TxConfig-r16[2] SEQUENCE { |  | Entry 2 Entry for CBR > 50% |  |
| sl-CR-Limit-r16 | 10000 |  |  |
| sl-TxParameters-r16 SEQUENCE { |  |  |  |
| sl-MinMCS-PSSCH-r16 | 27 |  |  |
| sl-MaxMCS-PSSCH-r16 | 27 |  |  |
| sl-MinSubChannelNumPSSCH-r16 | 2 |  |  |
| sl-MaxSubchannelNumPSSCH-r16 | 2 |  |  |
| sl-MaxTxTransNumPSSCH-r16 | 2 | 1 re-transmission |  |
| sl-MaxTxPower-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |  |

<End of Change 2>