3GPP TSG-RAN5 Meeting #110	R5-260862

# Gothenburg Metropolitan Area, Sweden, 9th Feb 2026 - 13th Feb 2026

# Source:	Qualcomm Incorporated

# Title:	Discussion on Uplink Data Compression Test Cases

# Document for:	Discussion

## 1.	Introduction

3GPP Rel-17 work item NR_UDC-UEConTest introduced nine test cases for verifying PDCP Uplink Data Compression (UDC) functionality in clause 7.1.3.6 of [1]. Across all these test cases, UE TEST LOOP MODE A is configured and primary DRB used for loopback data transfer, according to clause 7.1.3.0 of [1] is:

- "Default DRB of the first PDU session on NR Cell" for Connectivity (NR) and

- "SCG DRB" for Connectivity (NR-DC), Connectivity (NE-DC).

## 2.	Discussion

According to [2], a maximum of two DRBs may be configured with UDC.

TS 38.331 [2]:

Indicates the UDC configuration that the UE shall apply. Network does not configure uplinkDataCompression for a DRB, if headerCompression or ethernetHeaderCompression is already configured or outOfOrderDelivery or DAPS is configured for the DRB. The maximum number of DRBs where uplinkDataCompression can be applied is two.

In addition, as specified in Annex B.1 of [3], when UDC is configured for a DRB, UDC data block contains either compressed blocks generated by UDC protocol or not compressed, indicated by FU field (Table B.2.2.1-1 of [3]) in UDC header.

TS 38.323 [3] Annex B.1:

A UDC packet consists of a UDC header and a UDC data block. A UDC data block contains either DEFLATE compressed blocks generated by UDC protocol or original PDCP SDU for SDU not compressed by UDC protocol; the type is specified in FU field (details see Annex B.2.2.1) in UDC header.

All test cases in [1] verify that the UE sends a UDC compressed data block by checking that FU = 1 in the UDC header.

2.1	Connectivity (NR) test cases 7.1.3.6.1 to 7.1.3.6.7:

Depending on the UE configuration, typically first PDU session established on NR corresponds to "internet" or "ims" dnn. A compliant UE supporting UDC on only one of the internet or IMS PDU sessions, while looping back the data on an unsupported PDU results in the UE setting FU field to 0, resulting in an inconclusive or failed test outcome.

To accurately verify the intended test purposes, loopback data must be transmitted on the DRB/DNN where UDC is applied.

It is proposed to add a new PIXIT parameter say "px_UDC_DNN_Type" and loopback data transfer shall happen on the default bearer of PDU session established on the DNN specified in this PIXIT parameter rather than the default bearer of first PDU session.

2.2 	Connectivity (NR-DC) test case 7.1.3.6.8 and Connectivity (NE-DC) test case 7.1.3.6.9:

For these dual-connectivity scenarios, UDC functionality is exercised on SCG DRBs. Even if the UE supports the UDC feature, it may not necessarily apply it on the configured SCG DRBs.

It is proposed to introduce two PICS parameters pc_NRDC_SCG_udc_r17, pc_NEDC_SCG_udc_r17 for indicating UE support of UDC on SCG DRBs and these PICS shall be referenced in the applicability conditions of test cases 7.1.3.6.8 and 7.1.3.6.9.

## 3.  Proposals/Way Forward

Proposal-1: Introduce a new PIXIT parameter px_UDC_DNN_Type with Supported Values as ims, internet and refer in test cases as needed.

CRs [7] and [8] raised at this meeting handling this proposal.

Proposal-2: Add two new PICS parameters pc_NRDC_SCG_udc_r17, pc_NEDC_SCG_udc_r17 in [5] and update the applicability of test cases 7.1.3.6.8 and 7.1.3.6.9 in [6] accordingly.

CRs [9] and [10] are raised at this meeting handling this proposal.

## 4.	References

[1]	TS 38.523-1	5GS; User Equipment (UE) conformance specification; Part 1: Protocol

[2]	TS 38.331	NR; Radio Resource Control (RRC); Protocol specification

[3]	TS 38.323	NR; Packet Data Convergence Protocol (PDCP) specification

[4]	TS 38.523-3	5GS; User Equipment (UE) conformance specification; Part 3: Protocol Test Suites

[5]	TS 38.508-2	5GS; User Equipment (UE) conformance specification; Part 2: Common Implementation Conformance Statement (ICS) proforma

[6]	TS 38.523-2	5GS; User Equipment (UE) conformance specification; Part 2: Applicability of protocol test cases

[7]	R5-260904	Addition of PIXIT parameter for NR UDC test cases

[8]	R5-260906	Correction of UDC test case 7.1.3.6.1

[9]	R5-260864	Addition of PICS for NR UDC test cases

[10]	R5-260875	Applicability updates for UDC test cases