3GPP TSG-RAN5 Meeting #108	R5-254201

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5481 | rev | 1 | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to EMM test case 9.2.3.3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the test procedure PDP context is gets established before the TAU procedure gets triggered at Step 17 to move from UTRAN to LTE RAT. A UE supporting multi-EPS bearer can have 2 PDP contexts established on UTRA RAT (one for internet and other for IMS) and will indicate them in EPS bearer context status IE in TAU message, this is not currently handled. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified the specific message contents of TRACKING AREA UPDATE REQUEST message in Table 9.2.3.3.1.3.3-3 to allow UE reporting 2 EPS bearer contexts |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN CR R5s250328 is associated with this change |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-254201->R5-254928 |  |  |  |  |  |  |  |  |

##### 9.2.3.3.1	First Iu mode to S1 mode inter-system change after attach

9.2.3.3.1.1	Test Purpose (TP)

(1)

with { the UE is powered on in UTRAN with a USIM containing valid EPS identities and EPS security context and has attached in UTRAN and activated a PDP context  }

ensure that {

when { the UE reselects a E-UTRAN cell }

then { the UE  transmits a TRACKING AREA UPDATE REQUEST message including eKSI stored in the USIM, old GUTI mapped from RAI and P-TMSI, GPRS ciphering sequence number, nonceUE and the UE radio capability information update needed IE, the message is integrity protected using the EPS security context stored in the USIM and encapsulated in an RRCConnectionSetupComplete message with registeredMME set to the MME part of the mapped GUTI }

(2)

with { the UE has performed a TAU procedure in EUTRAN after initial registration at power on in UTRAN  }

ensure that {

when { the UE reselects a UTRAN cell }

then { the UE transmits a ROUTING AREA UPDATE REQUEST message including P-TMSI and P-TMSI signature mapped from GUTI, previously allocated P-TMSI as additional mobile identity, RAI where P-TMSI was allocated as additional old routing area identification, eKSI allocated in UTRAN as GPRS ciphering key sequence number, the message is encapsulated in an INITIAL DIRECT TRANSFER message including IDNSS mapped from P-TMSI }

(3)

with { the UE has performed a TAU procedure in EUTRAN after initial registration at power on in UTRAN and has transmitted a ROUTING AREA UPDATE REQUEST message and received an AUTHENTICATION AND CIPHERING REQUSET message }

ensure that {

when { the UE receives a SECURITY MODE COMMAND message }

then { the UE transmits a SECURITY MODE COMPLETE and starts performing ciphering an integrity protection using new CK and IK }

(4)

with { the UE has performed initial registration at power on in UTRAN and has EPS security context }

ensure that {

when { UE performs a TAU procedure in E-UTRAN }

then { the UE starts performing ciphering an integrity protection using EPS security context }

NOTE:	ISR considered not in the scope of this test case, so it is never activated.

9.2.3.3.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 23.003 clause 2.8.2.1, 2.8.2.2, TS 23.401 clauses 5.3.1.1, 5.3.3.1, 5.3.3.3 and Annex D; TS 25.331 clauses 8.1.8.2 and 10.3.1.6 (on IDNNS); TS 24.008 [NAS message contents]; TS 24.301 clause 5.5.3.2.2 and TS 24.301 V12.2.0 clause 5.5.3.3.2; and TS 36.331 clauses 5.3.3.3 and 5.3.3.4. Unless otherwise stated these are Rel-8 requirements.

[TS 23.003, clause 2.8.2.1]

The mapping of the GUTI shall be done to the combination of RAI of GERAN / UTRAN and the P-TMSI:

E-UTRAN <MCC> maps to GERAN/UTRAN <MCC>

E-UTRAN <MNC> maps to GERAN/UTRAN <MNC>

E-UTRAN <MME Group ID> maps to GERAN/UTRAN <LAC>

E-UTRAN <MME Code> maps to GERAN/UTRAN <RAC> and is also copied into the 8 most significant bits of the NRI field within the P-TMSI;

E-UTRAN <M-TMSI>			maps as follows:

-	6 bits of the E-UTRAN <M-TMSI> starting at bit 29 and down to bit 24 are mapped into bit 29 and down to bit 24 of the GERAN/UTRAN <P-TMSI>;

-	16 bits of the E-UTRAN <M-TMSI> starting at bit 15 and down to bit 0 are mapped into bit 15 and down to bit 0 of the GERAN/UTRAN <P-TMSI>;

-	and the remaining 8 bits of the E-UTRAN <M-TMSI> are mapped into the 8 MBS bits of the <P-TMSI signature> field.

For UTRAN, the 10-bit long NRI bits are masked out from the P-TMSI and also supplied to the RAN node as IDNNS (Intra Domain NAS Node Selector). However, the RAN configured NRI length should not exceed 8 bits.

[TS 23.003, clause 2.8.2.2]

The mapping of P-TMSI (TLLI) and RAI in GERAN/UTRAN to GUTI in E-UTRAN shall be performed as follows:

GERAN/UTRAN <MCC>	maps to E-UTRAN <MCC>

GERAN/UTRAN <MNC>	maps to E-UTRAN <MNC>

GERAN/UTRAN <LAC>	maps to E-UTRAN <MME Group ID>

GERAN/UTRAN <RAC>	maps into bit 23 and down to bit 16 of the M-TMSI

The 8 most significant bits of GERAN/UTRAN <NRI> map to the MME code.

GERAN/UTRAN <P-TMSI> maps as follows:

-	6 bits of the GERAN/UTRAN <P-TMSI> starting at bit 29 and down to bit 24 are mapped into bit 29 and down to bit 24 of the E-UTRAN <M-TMSI>;

-	16 bits of the GERAN/UTRAN <P-TMSI> starting at bit 15 and down to bit 0 are mapped into bit 15 and down to bit 0 of the E-UTRAN <M-TMSI>.

The values of <LAC> and <MME group id> shall be disjoint, so that they can be differentiated. The most significant bit of the <LAC> shall be set to zero; and the most significant bit of <MME group id> shall be set to one. Based on this definition, the most significant bit of the <MME group id> can be used to distinguish the node type, i.e. whether it is an MME or SGSN.

[TS 24.301 clause 5.3.1.1]

When the UE is in EMM-IDLE mode and needs to transmit an initial NAS message, the UE shall request the lower layer to establish a RRC connection. In this request to the lower layer the NAS shall provide to the lower layer the RRC establishment cause and the call type as specified in annex D of this specification.

...

[TS 23.401, clause 5.3.3.1, step 2, “Tracking Area Update procedure with Serving GW change”]

...

If the UE's TIN indicates "P-TMSI" and the UE holds a valid P-TMSI and related RAI then these two elements are indicated as the old GUTI.

...

[TS 23.401, clause 5.3.3.3, step 2, “Routeing Area Update with MME interaction and without S-GW change”]

...

If the UE's internal TIN indicates "GUTI" and the UE holds a valid GUTI then the UE indicates the GUTI as the old P-TMSI and old RAI

...

[TS 24.301, clause 5.5.3.2.2 “Normal and periodic tracking area updating procedure initiation”]

...

If the UE supports A/Gb mode or Iu mode, the UE shall handle the GUTI as follows:

-	if the TIN indicates "P-TMSI" and the UE holds a valid P-TMSI and RAI, the UE shall map the P-TMSI and RAI into the old GUTI IE. If a P-TMSI signature is associated with the P-TMSI, the UE shall include it in the Old P-TMSI signature IE. Additionally, if the UE holds a valid GUTI, the UE shall indicate the GUTI in the Additional GUTI IE.

NOTE:	Mapping the P-TMSI and RAI to the GUTI is specified in Annex H of 3GPP TS 23.401 [10].

-	if the TIN indicates "GUTI" or "RAT-related TMSI" and the UE holds a valid GUTI, the UE shall indicate the GUTI in the Old GUTI IE.

...

If the UE has a current EPS security context, the UE shall include the eKSI (either KSIASME or KSISGSN) in the NAS Key Set Identifier IE in the TRACKING AREA UPDATE REQUEST message. Otherwise, the UE shall set the NAS Key Set Identifier IE to the value "no key is available". If the UE has a current EPS security context, the UE shall integrity protect the TRACKING AREA UPDATE REQUEST message with the current EPS security context. Otherwise the UE shall not integrity protect the TRACKING AREA UPDATE REQUEST message.

...

When the tracking area updating procedure is initiated in EMM-IDLE mode to perform an inter-system change from A/Gb mode or Iu mode to S1 mode and the TIN is set to "P-TMSI", the UE shall include the GPRS ciphering key sequence number applicable for A/Gb mode or Iu mode and a nonceUE in the TRACKING AREA UPDATE REQUEST message.

…

If the UE initiates the first tracking area updating procedure following an attach in A/Gb mode or Iu mode, the UE shall include a UE radio capability information update needed IE in the TRACKING AREA UPDATE REQUEST message.

[Rel-12]

[TS 24.301, clause 5.5.3.3.2]

The UE operating in CS/PS mode 1 or CS/PS mode 2, in state EMM-REGISTERED, shall initiate the combined tracking area updating procedure:

…

d)	when the UE performs an intersystem change from A/Gb or Iu mode to S1 mode, and the UE previously either performed a combined GPRS attach procedure, an IMSI attach procedure, a location area updating procedure or a combined routing area updating procedure, in A/Gb or Iu mode, or moved to A/Gb or Iu mode from S1 mode through an SRVCC handover or moved to Iu mode from S1 mode through an vSRVCC handover. In this case the EPS update type IE shall be set to "combined TA/LA updating with IMSI attach";

[TS 24.301, Annex D]

...

Table D.1.1: Mapping of NAS procedure to establishment cause and call type

| NAS procedure | RRC establishment cause (according 3GPP TS 36.331 [22]) | Call type |
| --- | --- | --- |
| Tracking Area Update | MO signalling (See Note 1) | "originating signalling" |
| Note 1: For these NAS procedures initiated by UEs of access class 12, 13 or 14 in their home country, the RRC establishment cause will be set to "High priority access AC 11 – 15". For this purpose the home country is defined as the country of the MCC part of the IMSI, see 3GPP TS 22.011 [1A]. For these NAS procedures initiated by UE of access class 11 or 15 in their HPLMN or EHPLMN, the RRC establishment cause will be set to "High priority access AC 11 – 15". |  |  |

[TS 36.331, clause 5.3.3.3]

The UE shall set the contents of RRCConnectionRequest message as follows:

1>	set the ue-Identity as follows:

2>	if upper layers provide an S-TMSI:

3>	set the ue-Identity to the value received from upper layers;

2>	else

3>	draw a random value in the range 0 .. 240-1 and set the ue-Identity to this value;

NOTE 1	Upper layers provide the S-TMSI if the UE is registered in the TA of the current cell.

1>	Set the establishmentCause in accordance with the information received from upper layers;

[TS 36.331, clause 5.3.3.4]

...

1>	set the content of RRCConnectionSetupComplete message as follows:

2>	set the selectedPLMN-Identity to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]) from the PLMN(s) included in the plmn-IdentityList in SystemInformationBlockType1;

2>		if upper layers provide the ‘Registered MME’, set the registeredMME as follows:

3>	if the PLMN identity of the ‘Registered MME’ is different from the PLMN selected by the upper layers:

4>	include the plmnIdentity in the registeredMME and set it to the value of the PLMN identity in the ‘Registered MME’ received from upper layers;

3> set the mmegi and the mmec to the value received from upper layers;

2>	set the dedicatedInfoNAS  to include the information received from upper layers;

...

[TS 24.008, clause 4.7.5]

This procedure is used for:

...

-	S1 mode to Iu mode or S1 mode to A/Gb mode intersystem change and ISR is not activated;

...

[TS 24.008, clause 4.7.5.1.1]

To initiate the normal routing area updating procedure, the MS sends the message ROUTING AREA UPDATE REQUEST to the network, starts timer T3330 and changes to state GMM-ROUTING-AREA-UPDATING-INITIATED.

If the MS supports S1 mode, the MS shall handle the P-TMSI IE as follows:

-	If the TIN indicates "GUTI" and the MS holds a valid GUTI, the MS shall map the GUTI into a P-TMSI, P-TMSI signature and RAI as specified in 3GPP TS 23.003 [4]. The MS shall include the mapped RAI in the Old routing area identification IE and the mapped P-TMSI signature in the P-TMSI signature IE. When the routing area updating procedure is initiated in Iu mode, the MS shall also include the mapped P-TMSI in the P-TMSI IE. Additionally, in Iu mode and A/Gb mode, if the MS holds a valid P-TMSI and RAI, the MS shall indicate the P-TMSI in the Additional mobile identity IE and the RAI in the Additional old routing area identification IE.

...

If the routing area updating procedure is initiated by the MS due to an S1 mode to Iu mode or S1 mode to A/Gb mode inter-system change in idle mode and the TIN indicates "GUTI", the MS shall derive a UMTS security context for the PS domain from the current EPS security context as described in the subclause 4.7.7.10. The ROUTING AREA UPDATE REQUEST message shall include a P-TMSI signature filled with a NAS token as specified in 3GPP TS 33.401 [119]. Furthermore, the MS shall indicate the eKSI value, which is associated with the derived UMTS security keys, in the CKSN field of the GPRS GSM ciphering key sequence number IE in the ROUTING AREA UPDATE REQUEST message.

NOTE:	When the MS includes a P-TMSI signature filled with a NAS token, 8 bits of the NAS token will be filled with bits from the M-TMSI (see 3GPP TS 23.003 [4]).

If the routing area updating procedure is initiated by the MS due to the S1 mode to Iu mode or S1 mode to A/Gb mode inter-system change in connected mode, the MS shall derive a UMTS security context for the PS domain from the current EPS security context as described in the subclause 4.7.7.10. Furthermore, the MS shall indicate the eKSI value, which is associated with the derived UMTS security keys, in the CKSN field of the GPRS GSM ciphering key sequence number IE in the ROUTING AREA UPDATE REQUEST message.

[TS 25.331, clause 8.1.8.2]

...

The UE shall, in the INITIAL DIRECT TRANSFER message:

1>	set the IE "NAS message" as received from upper layers; and

1>	set the IE "CN domain identity" as indicated by the upper layers; and

1>	set the IE "Intra Domain NAS Node Selector" as follows:

2>	derive the IE "Intra Domain NAS Node Selector" from TMSI/PMTSI, IMSI, or IMEI; and

2>	provide the coding of the IE "Intra Domain NAS Node Selector" according to the following priorities:

1.	derive the routing parameter for IDNNS from TMSI (CS domain) or PTMSI (PS domain) whenever a valid TMSI/PTMSI is available;

2.	base the routing parameter for IDNNS on IMSI when no valid TMSI/PTMSI is available;

3.	base the routing parameter for IDNNS on IMEI only if no (U)SIM is inserted in the UE.

1>	if the UE, on the existing RRC connection, has received a dedicated RRC message containing the IE "Primary PLMN Identity" in the IE "CN Information Info":

2>	set the IE "PLMN identity" in the INITIAL DIRECT TRANSFER message to the latest PLMN information received via dedicated RRC signalling. If NAS has indicated the PLMN towards which a signalling connection is requested, and this PLMN is not in agreement with the latest PLMN information received via dedicated RRC signalling, then the initial direct transfer procedure shall be aborted, and NAS shall be informed.

1>	if the UE, on the existing RRC connection, has not received a dedicated RRC message containing the IE "CN Information Info" , and if the IE "Multiple PLMN List" was broadcast in the cell where the current RRC connection was established:

2>	set the IE "PLMN identity" in the INITIAL DIRECT TRANSFER message to the PLMN chosen by higher layers [5, 25] amongst the PLMNs in the IE "Multiple PLMN List" broadcast in the cell where the RRC connection was established.

1>	if the IE "Activated service list" within variable MBMS_ACTIVATED_SERVICES includes one or more MBMS services with the IE "Service type" set to "Multicast" and;

1>	if the IE "CN domain identity" as indicated by the upper layers is set to "CS domain" and;

1>	if the variable ESTABLISHED_SIGNALLING_CONNECTIONS does not include the CN domain identity 'PS domain':

2>	include the IE "MBMS joined information";

2>	include the IE "P-TMSI" within the IE "MBMS joined information" if a valid PTMSI is available.

1>	if the UE is in CELL_FACH state and the IE "CN domain identity" as indicated by the upper layers is set to "CS domain":

2>	if the value of the variable ESTABLISHMENT_CAUSE is set to "Originating Conversational Call" or "Emergency Call":

3>	set the value of the IE "Call type" to "speech", "video" or "other" according to the call being initiated.

1>	if the variable ESTABLISHMENT_CAUSE is initialised:

2>	set the IE "Establishment cause" to the value of the variable ESTABLISHMENT_CAUSE;

2>	clear the variable ESTABLISHMENT_CAUSE.

1>	calculate the START according to subclause 8.5.9 for the CN domain as set in the IE "CN Domain Identity"; and

1>	include the calculated START value for that CN domain in the IE "START".

9.2.3.3.1.3	Test description

9.2.3.3.1.3.1	Pre-test conditions

System Simulator:

-	cell A, cell B and cell 5 (UTRA cell in LAI-1/RAI-1);

-	cell 5 is configured as Serving cell, cell A as Non-Suitable cell.

-	System information combination 4 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

NOTE:	For Cell 5, power levels are defined in TS 34.108 subclause 6.1.5 (FDD) or 6.1.6 (TDD).

UE:

-	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell B using default message contents according to TS 36.508 [18].

NOTE:	Cell B belongs to TAI-2 (unlike cell A).

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.3.3.1.3.2	Test procedure sequence

Table 9.2.3.3.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | The UE transmits an ATTACH REQUEST message on Cell 5 | --> | ATTACH REQUEST | - | - |
| 3 | The SS transmits an AUTHENTICATION AND CIPHERING REQUEST message. | <-- | AUTHENTICATION AND CIPHERING REQUEST | - | - |
| 4 | The UE transmits an AUTHENTICATION AND CIPHERING RESPONSE message. | --> | AUTHENTICATION AND CIPHERING RESPONSE | - | - |
| 5 | Void | - | - | - | - |
| 6 | Void | - | - | - | - |
| 7 | Void | - | - | - | - |
| 8 | SS responds with ATTACH ACCEPT message including P-TMSI-1 and RAI-1. | <-- | ATTACH ACCEPT | - | - |
| 9 | The UE transmits an ATTACH COMPLETE message. | --> | ATTACH COMPLETE | - | - |
| 9A | The SS releases the RRC connection. | - | - | - | - |
| 9B | Generic test procedure in TS 36.508 subclause 4.5A.3A.2 is performed | - | - | - | - |
| 10 | The activation of a PDP context is triggered by MMI or AT command. | - | - | - | - |
| 11 | Void | - | - | - | - |
| 12 | The SS establishes a radio bearer associated with the requested PDP context. | - | - | - | - |
| 13 | The SS releases the RRC connection. | - | - | - | - |
| 14 | Cell A is configured as the Serving Cell, Cell 5 is configured as a Suitable Neighbour Cell. | - | - | - | - |
| 15 | Check: Does the UE transmit an RRCConnectionRequest with the InitialUE-Identity set to “randomValue” and the establishmentcause set to MO-signalling? | - | - | 1 | P |
| 16 | The SS responds with RRCConnectionSetup. | - | - |  |  |
| 17 | Check: Does the UE transmit an RRCConnectionSetupComplete message with the mmegi and mmec are set to the values derived from the mapped RAI and P-TMSI, and containing a TRACKING AREA UPDATE REQUEST message as described in the specific message contents? Check2: Does the UE transmit a TRACKING AREA UPDATE REQUEST message integrity protected using IK derived from KASME? | --> | TRACKING AREA UPDATE REQUEST | 1, 4 | P |
| 18 | The SS responds with a TRACKING AREA UPDATE ACCEPT allocating a new GUTI and TAI list. This message is ciphered and integrity protected using CK and IK derived from KASME. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 18A | Check: Does the UE transmit a TRACKING AREA UPDATE COMPLETE message ciphered and integrity protected using CK and IK derived from KASME. | --> | TRACKING AREA UPDATE COMPLETE | 4 | P |
| 19 | The SS releases the RRC connection | - | - | - | - |
| - | Exception: Steps 19A to 19Aa5/19Ab1 describe optional behaviour that depend on the UE capability. | - | - | - |  |
| 19A | If pc_IMS = TRUE SS starts 10s timer | - | - | - |  |
| 19Aa1 | The UE establishes an RRC connection and transmits a SERVICE REQUEST message | - | - | - |  |
| 19Aa2 | The SS establishes SRB2 and DRB | - | - | - |  |
| 19Aa3 | If MULTI-PDN = TRUE the UE executes the generic procedure for establishment of additional PDN connectivity defined in TS 36.508 4.5A.16.3. | - | - | - |  |
| 19Aa4 | The UE performs IMS registration using the generic procedure defined in 34.229-1 [35] Annex C.2 steps 4-11. | - | - | - |  |
| 19Aa5 | The SS releases the RRC connection | - | - | - |  |
| 19Ab1 | The 10s timer expires | - | - | - |  |
| 20 | Void | - | - | - | - |
| 21 | Cell 5 is configured as the Serving Cell and Cell A is configured as a Non-Suitable Cell as defined in table 6.2.2.1-1 of TS 36.508 [18]. | - | - | - | - |
| 22 | Void | - | - | - | - |
| 23 | Check: Does the UE transmit a ROUTING AREA UPDATE REQUEST message as specified in the specific message contents included in an INITIAL DIRECTE TRANSFER message with intraDomainNasNodeSelector mapped from GUTI allocated in step 18? | --> | ROUTING AREA UPDATE REQUEST | 2 | P |
| 23AA | The SS sends an AUTHENTICATION AND CIPHERING REQUEST message to perform a UMTS AKA procedure. | <-- | AUTHENTICATION AND CIPHERING REQUEST | - | - |
| 23AB | The UE sends an AUTHENTICATION AND CIPHERING RESPONSE message. | --> | AUTHENTICATION AND CIPHERING RESPONSE | - | - |
| 23A | The SS sends a SECURITY MODE COMMAND message to activate integrity protection and ciphering. This message is integrity protected using new IK. | - | - | - | - |
| 23B | Check: Does the UE transmit a SECURITY MODE COMPLETE message integrity protected using new IK? | - | - | 3 | P |
| 24 | The SS transmits a ROUTING AREA UPDATE ACCEPT message (no new P-TMSI nor RAI). This message is ciphered and integrity protected using CK and IK derived at step 23AA. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 25 | After the activation time indicated at step 23A, the SS transmits a UE CAPABILITY ENQUIRY message requesting the UE UTRAN capability. This message is ciphered and integrity protected using CK and IK derived at step 23AA. | - | - | - | - |
| 26 | Check: Does the UE transmit a UE CAPABILITY INFORMATION message? Note: the purpose of this message is to show that the UE uses CK and IK derived at step 23AA, and not the CK and IK allocated at step 4. | - | - | 3 | P |
| 27 | The SS transmits a UE CAPABILITY INFORMATION CONFIRM message. | - | - | - | - |
| 28 | The SS releases the RRC connection | - | - | - | - |

9.2.3.3.1.3.3	Specific message contents

Table 9.2.3.3.1.3.3-1: Message RRCConnectionRequest (step 15, Table 9.2.3.3.1.3.2-1)

| Derivation path: 36.508 table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity CHOICE { |  |  |  |
| random-Value | Any allowed value |  |  |
| } |  |  |  |
| establishmentCause | Mo-Signalling |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.3.3.1.3.3-2: Message RRCConnectionSetupComplete (step 17, Table 9.2.3.3.1.3.2-1)

| Derivation path: 36.508 table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |
| selectedPLMN-Identity | 1 |  |  |
| registeredMME { |  |  |  |
| plmn-Identity | Not present |  |  |
| mmegi | LAC sent to the UE in step 8 |  |  |
| mmec | Bit 23 to bit 16 of P-TMSI sent to the UE in step 8 |  |  |
| } |  |  |  |
| dedicatedInfoNAS | See table 9.2.3.3.1.3.3-3 |  |  |
| nonCriticalExtension SEQUENCE {} |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Details to be added |  |  |  |

Table 9.2.3.3.1.3.3-3: Message TRACKING AREA UPDATE REQUEST (step 17, Table 9.2.3.3.1.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 with condition UNCIPHERED (the message is integrity protected using the key stored in the USIM). |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type | 000 or 001 or 010 | 'TA updating' or 'combined TA/LA updating' or ‘combined TA/LA updating with IMSI attach’ | Rel-8 to Rel-11 only |
|  | 000 or 010 | 'TA updating' or ‘combined TA/LA updating with IMSI attach’ | Rel-12 and later releases |
| NAS key set identifier |  |  |  |
| NAS key set identifier | The valid NAS key set identifier stored in the USIM |  |  |
| TSC | '0'B | native security context (for KSIASME) |  |
| Old GUTI or IMSI |  |  |  |
| Type of identity | 110 | GUTI |  |
| MNC/MCC | Mobile Country Code and Mobile Network Code stored in EFIMSI on the test USIM |  |  |
| MME Group ID | LAI allocated at step 8 |  |  |
| MME Code | Bit 23 to bit 16 of P-TMSI allocated at step 8 |  |  |
| M-TMSI | Value is specified bit by bit below: - bit 31 and bit 30: 1 - bit 29 to bit 24: bit 29 to bit 24 of P-TMSI allocated at step 8 - bit 23 to bit 16: RAC allocated at step 8 - bit 15 to bit 0: bit 15 to bit 0 of P-TMSI allocated at step 8 |  |  |
| GPRS ciphering key sequence number | GPRS ciphering key sequence number allocated at step 3 |  |  |
| Old P-TMSI signature | P-TMSI signature allocated at step 8 |  |  |
| Additional GUTI | GUTI-1 |  |  |
| NonceUE | Any allowed value (must be present) |  |  |
| UE network capability | Any allowed value (must be present) |  |  |
| Last visited registered TAI | TAI stored in the test USIM |  |  |
| UE radio capability information update needed | 1 | UE radio capability information update needed |  |
| EPS bearer context status | octet 3 = ('00100000'B or '01100000'B) and octet 4 = '00000000'B) |  |  |
| MS network capability | Any allowed value (must be present) |  |  |
| Old location area identification | Not present if "EPS update type" is 'TA updating', LAI-1 if "EPS update type" is 'combined TA/LA updating' |  |  |
| TMSI status | Not present |  |  |

Table 9.2.3.3.1.3.3-4: INITIAL DIRECT TRANSFER (step 23, Table 9.2.3.3.1.3.2-1)

| Derivation path: 34.108 clause 9.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| InitialDirectTransfer ::= SEQUENCE { |  |  |  |
| cn-DomainIdentity | ps-domain |  |  |
| intraDomainNasNodeSelector SEQUENCE { |  |  |  |
| version CHOICE { |  |  |  |
| release99 SEQUENCE { |  |  |  |
| gsm-Map-IDNNS SEQUENCE { |  |  |  |
| routingbasis CHOICE { |  |  |  |
| tMSIofsamePLMN { |  |  |  |
| routingparameter | bit 23 to bit 14 of P-TMSI mapped from GUTI allocated to UE in step 18 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nas-Message | See table 9.2.3.3.1.3.3-5 |  |  |
| } |  |  |  |

Table 9.2.3.3.1.3.3-5: Message ROUTING AREA UPDATE REQUEST (step 23, Table 9.2.3.3.1.3.2-1)

| Derivation path: 24.008 table 9.4.14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Update type | 000 or 001 | RA updating or Combined RA/LA updating |  |
| GPRS ciphering key sequence number | eKSI stored on the test USIM |  |  |
| Old routing area identification | GUTI-1 right shifted by 32bits |  |  |
| MS Radio Access capability | Not checked |  |  |
| Old P-TMSI signature | Value is specified bit by bit below: - bit 23 to bit 16: bit 23 to bit 16 of M-TMSI allocated in step 18 - bit 15 to bit 0: Truncated NAS-token |  |  |
| Requested READY timer value | Not checked |  |  |
| DRX parameter | Not checked |  |  |
| TMSI status | Not present |  |  |
| P-TMSI | Value is specified bit by bit below: - bit 31 and bit 30: 1 - bit 29 to bit 24: bit 29 to bit 24 of M-TMSI allocated in step 18 - bit 23 to bit 16: MME code allocated in step 18 - bit 15 to bit 0: bit 15 to bit 0 of M-TMSI allocated in step 18 |  |  |
| MS network capability | Not checked |  |  |
| PDP context status | Not checked |  |  |
| PS LCS Capability | Not checked |  |  |
| MBMS context status | Not checked |  |  |
| UE network capability | Any allowed value |  |  |
| Additional mobile identity | P-TMSI allocated in step 8 |  |  |
| Additional old routing area identification | RAI-1 |  |  |
| Mobile station classmark 2 | Not checked |  |  |
| Mobile station classmark 3 | Not checked |  |  |
| Supported Codecs | Not checked |  |  |