3GPP TSG-RAN5 Meeting #108	R5-255040

# Bengaluru, India, 25th Aug 2025 – 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3549 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Common configuration updates related to UAV |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM JAPAN LLC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UAV_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In 38.508-1, Aerial-Config, Altitude, HysteresisAltitude message contents are missing, event H1, H2, AxHy specific contents are missing in ReportConfigNR, aerial-Config-r18 is missing in the RRCReconfiguration message, UAV-specific conditions is missing for Uncrewed Aerial Vehicles |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added aerial-Config-r18 into the RRCReconfiguration message Introduced UAV-specific conditions for Uncrewed Aerial Vehicles Added Aerial-Config, Altitude, HysteresisAltitude message contents Added event H1, H2, AxHy specific contents to ReportConfigNR |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UAV test cases will not be possible to implement |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Table 4.6.1-13, 4.6.3-1B, 4.6.3-2A, 4.6.3-63AAA, 4.6.3-142 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.523-1 CR 5108,5109,5110,5111,5112 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from original R5-254280 |  |  |  |  |  |  |  |  |

### <Unchanged Sections Omitted>

#### –	RRCReconfigurationComplete

Table 4.6.1-13: RRCReconfiguration

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | Not present |  |  |
|  |  | RadioBearerConfig with conditions SRB2 and DRB1 |  | NR, NR-DC |
|  |  | RadioBearerConfig with conditions SRB2 and REEST |  | REEST |
| secondaryCellGroup |  | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC |
|  |  | CellGroupConfig with condition NR-DC_SCG | OCTET STRING (CONTAINING CellGroupConfig) | NR-DC_SCG |
|  |  | CellGroupConfig with conditions EN-DC and PSCell_change | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC_HO |
|  |  | CellGroupConfig with condition MEAS | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC_MEAS |
|  |  | CellGroupConfig with condition SCell_add | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC_SCell_add |
|  |  | Not present |  |  |
| measConfig |  | Not present |  |  |
|  |  | MeasConfig | Measurements configuration | NR_MEAS, EN-DC_MEAS, IRAT_MEAS |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | NR, SCell_add, NR_MEAS, NR-DC, SIDELINK, DAPS_HO_ReleaseSource, CHO, CPA, CPC, REEST, L2RelayUE, L2RemoteUE, SHRT304, SHRT310, SHRT312, SHRRLF, DeactivatedSCG, QoE |
| masterCellGroup |  | CellGroupConfig with condition SRB2_DRB1 | OCTET STRING (CONTAINING CellGroupConfig) | NR, NR-DC |
|  |  | CellGroupConfig with condition SCell_add | OCTET STRING (CONTAINING CellGroupConfig) | SCell_add |
|  |  | CellGroupConfig with condition MEAS | OCTET STRING (CONTAINING CellGroupConfig) | NR_MEAS |
|  |  | CellGroupConfig with condition REEST | OCTET STRING (CONTAINING CellGroupConfig) | REEST |
|  |  | CellGroupConfig with condition L2RelayUE and uuRelaySRB0_SRB1 | OCTET STRING (CONTAINING CellGroupConfig) | L2RelayUE and uuRelaySRB0_SRB1 |
|  |  | CellGroupConfig with condition L2RelayUE and uuRelaySRB2_DRB1 | OCTET STRING (CONTAINING CellGroupConfig) | L2RelayUE and uuRelaySRB2_DRB1 |
|  |  | CellGroupConfig with condition LTM_CSI | OCTET STRING (CONTAINING CellGroupConfig) | LTM_CSI |
| fullConfig |  | Not present |  |  |
| dedicatedNAS-MessageList |  | Not present |  |  |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message { |  | 1 entry |  | NR |
| DedicatedNAS-Message[1] |  | DedicatedNAS-Message | entry 1 A sequence of OCTET STRING (s) containing one or more DedicatedNAS-Message(s) |  |
| } |  |  |  |  |
| masterKeyUpdate |  | Not present |  |  |
| masterKeyUpdate SEQUENCE { |  |  |  | MasterKeyChange |
| keySetChangeIndicator |  | true |  |  |
| nextHopChainingCount |  | NextHopChainingCount |  |  |
| nas-Container |  | Not present |  |  |
|  |  | OCTET STRING including the 10 Octets value generated according to TS 24.501 [28] clause 9.11.2.9 |  | Inter_Sys_HO |
| } |  |  |  |  |
| dedicatedSIB1-Delivery |  | Not present |  |  |
| dedicatedSystemInformationDelivery |  | Not present |  |  |
| otherConfig |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | NR-DC, SIDELINK, DAPS_HO_ReleaseSource, CHO, CPA, CPC, NE-DC, L2RelayUE, L2RemoteUE, SHRT304, SHRT310, SHRT312, SHRRLF, DeactivatedSCG, QoE |
| otherConfig-v1540 |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| mrdc-SecondaryCellGroupConfig |  | Not present |  |  |
| mrdc-SecondaryCellGroupConfig CHOICE { |  |  |  | NR-DC, NE-DC |
| setup SEQUENCE { |  |  |  |  |
| mrdc-ReleaseAndAdd |  | Not present |  |  |
| mrdc-SecondaryCellGroup CHOICE { |  |  |  |  |
| nr-SCG |  | RRCReconfiguration with condition NR-DC_SCG | OCTET STRING (CONTAINING RRCReconfiguration) | NR-DC |
| eutra-SCG |  | RRCConnectionReconfiguration according TS 36.508 [2], table 4.6.1-8 with condition NE-DC | OCTET STRING (CONTAINING RRCConnectionReconfiguration) | NE-DC |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| radioBearerConfig2 |  | Not present |  | NE-DC |
|  |  | RadioBearerConfig with condition DRBn and SecondaryKeys | OCTET STRING (CONTAINING RadioBearerConfig) | NR-DC |
| sk-Counter |  | SK-Counter |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | SIDELINK, DAPS_HO_ReleaseSource, CHO, CPA, CPC, L2RelayUE, L2RemoteUE, SHRT304, SHRT310, SHRT312, SHRRLF, DeactivatedSCG, QoE |
| otherConfig-v1610 |  | Not present |  |  |
| bap-Config-r16 |  | Not present |  |  |
| iab-IP-AddressConfigurationList-r16 |  | Not present |  |  |
| conditionalReconfiguration-r16 |  | Not present |  |  |
|  |  | ConditionalReconfiguration |  | CHO, CPA, CPC |
| daps-SourceRelease-r16 |  | Not present |  |  |
|  |  | true |  | DAPS_HO_ReleaseSource |
| t316-r16 |  | Not present |  |  |
| needForGapsConfigNR-r16 |  | Not present |  |  |
| onDemandSIB-Request-r16 |  | Not present |  |  |
| dedicatedPosSysInfoDelivery-r16 |  | Not present |  |  |
| sl-ConfigDedicatedNR-r16 |  | Not present |  |  |
| sl-ConfigDedicatedNR-r16 CHOICE { |  |  |  | SIDELINK, (L2RelayUEor L2RemoteUE) and (PC5RelaySRB1 or PC5RelaySRB2_DRB1) |
| setup |  | SL-ConfigDedicatedNR-r16 |  | SIDELINK |
|  |  | SL-ConfigDedicatedNR-r16 with condition PC5RelaySRB1 |  | PC5RelaySRB1 |
|  |  | SL-ConfigDedicatedNR-r16 with condition PC5RelaySRB2_DRB1 |  | PC5RelaySRB2_DRB1 |
| } |  |  |  |  |
| sl-ConfigDedicatedEUTRA-Info-r16 |  | Not present |  |  |
| smtc-r16 |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | L2RelayUE, L2RemoteUE, SHRT304, SHRT310, SHRT312, SHRRLF, DeactivatedSCG, QoE |
| otherConfig-v1700 |  | Not present |  |  |
| otherConfig-v1700 SEQUENCE{ |  |  |  | SHRT304, SHRT310, SHRT312, SHRRLF |
| ul-GapFR2-PreferenceConfig-r17 |  | Not present |  |  |
| musim-GapAssistanceConfig-r17 |  | Not present |  |  |
| musim-LeaveAssistanceConfig-r1 |  | Not present |  |  |
| successHO-Config-r17 |  | Not present |  |  |
| successHO-Config-r17 CHOICE { |  |  |  | SHRT304, SHRT310, SHRT312, SHRRLF |
| setup SEQUENCE { |  |  |  |  |
| thresholdPercentageT304-r17 |  | Not present |  |  |
|  |  | p40 |  | SHRT304 |
| thresholdPercentageT310-r17 |  | Not present |  |  |
|  |  | p40 |  | SHRT310 |
| thresholdPercentageT312-r17 |  | Not present |  |  |
|  |  | p20 |  | SHRT312 |
| sourceDAPS-FailureReporting-r17 |  | Not present |  |  |
|  |  | true |  | SHRRLF |
| } |  |  |  |  |
| } |  |  |  |  |
| maxBW-PreferenceConfigFR2-2-r17 |  | Not present |  |  |
| maxMIMO-LayerPreferenceConfigFR2-2-r17 |  | Not present |  |  |
| minSchedulingOffsetPreferenceConfigExt-r17 |  | Not present |  |  |
| rlm-RelaxationReportingConfig-r17 |  | Not present |  |  |
| bfd-RelaxationReportingConfig-r17 |  | Not present |  |  |
| scg-DeactivationPreferenceConfig-r17 |  | Not present |  |  |
| rrm-MeasRelaxationReportingConfig-r17 |  | Not present |  |  |
| propDelayDiffReportConfig-r17 |  | Not present |  |  |
| } |  |  |  |  |
| sl-L2RelayUE-Config-r17 |  | Not present |  |  |
| sl-L2RelayUE-Config-r17 CHOICE { |  |  |  |  |
| setup |  | SL-L2RelayUE-Config with condition SRB0 and SRB1 |  | L2RelayUE and uuRelaySRB0_SRB1 |
|  |  | SL-L2RelayUE-Config with condition SRB2 and DRB1 |  | L2RelayUE and uuRelaySRB2_DRB1 |
| release |  | NULL |  | L2RelayUE_Release |
| } |  |  |  |  |
| sl-L2RemoteUE-Config-r17 |  | Not present |  |  |
| sl-L2RemoteUE-Config-r17 CHOICE { |  |  |  | L2RemoteUE |
| setup |  | SL-L2RemoteUE-Config with condition SRB2 and DRB1 and FirstRRCReconfig |  |  |
| release |  | NULL |  | L2RemoteUE_Release |
| } |  |  |  |  |
| dedicatedPagingDelivery-r17 |  | Not present |  |  |
| needForGapNCSG-ConfigNR-r17 |  | Not present |  |  |
| needForGapNCSG-ConfigEUTRA-r17 |  | Not present |  |  |
| musim-GapConfig-r17 |  | Not present |  |  |
| ul-GapFR2-Config-r17 |  | Not present |  |  |
| ul-GapFR2-Config-r17 CHOICE { |  |  |  | NR-FR2- Uplink-Gaps |
| setup |  | UL-GapFR2-Config-r17 |  |  |
| } |  |  |  |  |
| scg-State-r17 |  | Not present |  |  |
|  |  | deactivated |  | DeactivatedSCG |
| appLayerMeasConfig-r17 |  | Not present |  |  |
|  |  | AppLayerMeasConfig-r17 |  | QoE |
| ue-TxTEG-RequestUL-TDOA-Config-r17 |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | LTM,UAV |
| needForInterruptionConfigNR-r18 |  | Not present |  |  |
| aerial-Config-r18 |  | Not present |  |  |
| aerial-Config-r18 CHOICE { |  |  |  | UAV |
| setup |  | aerial-Config |  |  |
| } |  |  |  |  |
| sl-IndirectPathAddChange-r18 |  | Not present |  |  |
| n3c-IndirectPathAddChange-r18 |  | Not present |  |  |
| n3c-IndirectPathConfigRelay-r18 |  | Not present |  |  |
| otherConfig-v1800 |  | Not present |  |  |
| srs-PosResourceSetAggBW-CombinationList-r18 |  | Not present |  |  |
| ltm-Config-r18 |  | Not present |  |  |
| ltm-Config-r18 CHOICE { |  |  |  | LTM |
| setup |  | LTM-Config |  |  |
| } |  |  |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| EN-DC | E-UTRA-NR Dual Connectivity with E-UTRA connected to EPC |
| EN-DC_MEAS | An EN-DC measurement is configured |
| IRAT_MEAS | An IRAT measurement is configured in the NR PCell |
| NR_MEAS | A NR measurement is configured |
| NR | NR connected to 5GC |
| NR-DC | NR-NR Dual Connectivity is configured |
| NR-DC_SCG | Add SCG side configuration (NR-DC) |
| EN-DC_HO | NR PSCell change (EN-DC) |
| SCell_add | Add SCell |
| EN-DC_SCell_add | Add SCell (EN-DC) |
| MasterKeyChange | MasterKeyUpdate when performing ReconfigurationWithSync and indicating a change of the AS security algorithms associated to the master key |
| Inter_Sys_HO | Used during inter-system handover to NR procedure |
| REEST | The first RRCReconfiguration message after successful completion of the RRC re-establishment procedure |
| SIDELINK | For NR sidelink dedicated configuration |
| DAPS_HO_ReleaseSource | The source cell part of DAPS operation is to be stopped and the source cell part of DAPS configuration is to be released. |
| CHO | Conditional handover |
| CPA | Conditional PSCell addition |
| CPC | Conditional PSCell change |
| NE-DC | NR E-UTRA Dual Connectivity |
| L2RelayUE | Setup L2 U2N Relay UE configuration |
| L2RelayUE_Release | Release L2 U2N Relay UE configuration |
| L2RemoteUE | Setup L2 U2N Remote UE configuration |
| L2RemoteUE_Release | Release L2 U2N Remote UE configuration |
| NR-FR2-Upink-Gaps | Configure Uplink Gaps for NR FR2 |
| SHRT304 | Report the successful handover information to the network for the elapsed time of the timer T304 over the threshold |
| SHRT310 | Report the successful handover information to the network for the elapsed time of the timer T310 over the threshold |
| SHRT312 | Report the successful handover information to the network the ror elapsed time of the timer T312 over the threshold |
| SHRRLF | Report the successful handover information to the network for radio link failure |
| DeactivatedSCG | Indicate that SCG is in deactivated state |
| QoE | Configure Quality of Experience measurement |
| LTM | L1/L2-triggered mobility |
| LTM_CSI | CSI configuration for LTM |
| uuRelaySRB0_SRB1 | Establish uu RelayRLC Channel for SRB0 and SRB1 |
| uuRelaySRB2_DRB1 | Establish uu RelayRLC Channel for SRB2 and DRB1 |
| PC5RelaySRB1 | Establish PC5 RelayRLC Channel for SRB1 |
| PC5RelaySRB2_DRB1 | Establish PC5 RelayRLC Channel for SRB2 and DRB1 |
| UAV | Uncrewed Aerial Vehicles Configurations |

<Unchanged Sections Omitted>

#### –	Aerial-Config

Table 4.6.3-1B: Aerial-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Aerial-Config-r18 ::= SEQUENCE { |  |  |  |
| flightPathUpdateThrConfig-r18 SEQUENCE { |  |  |  |
| flightPathUpdateDistanceThr-r18 CHOICE { |  |  |  |
| setup | FlightPathUpdateDistanceThr-r18 |  |  |
| } |  |  |  |
| flightPathUpdateTimeThr-r18 CHOICE { |  |  |  |
| setup | FlightPathUpdateTimeThr-r18 |  |  |
| } |  |  |  |
| } |  |  |  |

<Unchanged Sections Omitted>

#### –	Altitude

Table 4.6.3-2A: Altitude

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Altitude-r18 | 0 | Value in meters |  |

<Unchanged Sections Omitted>

#### –	HysteresisAltitude

Table 4.6.3-63AAA: HysteresisAltitude

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| HysteresisAltitude-r18 | 0 | Value in meters |  |

<Unchanged Sections Omitted>

#### –	ReportConfigNR

Table 4.6.3-142: ReportConfigNR(Thres1, Thres2)

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| rsType | ssb |  |  |
| reportInterval | ReportInterval |  |  |
| reportAmount | infinity |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | true |  |  |
| sinr | false |  |  |
|  | true |  | pc_ss_SINR_Meas |
| } |  |  |  |
| maxReportCells | 8 |  |  |
| reportQuantityRS-Indexes | Not present |  |  |
| maxNrofRS-IndexesToReport | Not present |  |  |
| includeBeamMeasurements | false |  |  |
| useAllowedCellList | false |  |  |
| measRSSI-ReportConfig-r16 | Not present |  |  |
| includeCommonLocationInfo-r16 | Not present |  |  |
| includeBT-Meas-r16 | Not present |  |  |
| includeBT-Meas-r16 CHOICE { |  |  | MDT_BT |
| setup | BT-NameList |  |  |
| } |  |  |  |
| includeWLAN-Meas-r16 | Not present |  |  |
| includeWLAN-Meas-r16 CHOICE { |  |  | MDT_WLAN |
| setup | WLAN-NameList |  |  |
| } |  |  |  |
| includeSensor-Meas-r16 | Not present |  |  |
| includeSensor-Meas-r16 CHOICE { |  |  | MDT_SENSOR |
| setup | Sensor-NameList |  |  |
| } |  |  |  |
| ul-DelayValueConfig-r16 | Not present |  |  |
| ul-DelayValueConfig-r16 CHOICE { |  |  | MDT_DELAY |
| setup | UL-DelayValueConfig |  |  |
| } |  |  |  |
| reportAddNeighMeas-r16 | Not present |  |  |
| } |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA1 SEQUENCE { |  |  | EVENT_A1 |
| a1-Threshold CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| reportOnLeave | false |  |  |
| hysteresis | Hysteresis |  |  |
| timeToTrigger | TimeToTrigger |  |  |
| } |  |  |  |
| eventA2 SEQUENCE { |  |  | EVENT_A2 |
| a2-Threshold CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| reportOnLeave | false |  |  |
| hysteresis | Hysteresis |  |  |
| timeToTrigger | TimeToTrigger |  |  |
| } |  |  |  |
| eventA3 SEQUENCE { |  |  | EVENT_A3 |
| a3-Offset CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| reportOnLeave | false |  |  |
| hysteresis | Hysteresis |  |  |
| timeToTrigger | TimeToTrigger |  |  |
| useAllowedCellList | false |  |  |
| } |  |  |  |
| eventA4 SEQUENCE { |  |  | EVENT_A4 |
| a4-Threshold CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| reportOnLeave | false |  |  |
| hysteresis | Hysteresis |  |  |
| timeToTrigger | TimeToTrigger |  |  |
| useAllowedCellList | false |  |  |
| } |  |  |  |
| eventA5 SEQUENCE { |  |  | EVENT_A5 |
| a5-Threshold1 CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| a5-Threshold2 CHOICE { |  |  |  |
| rsrp | Thres2 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| reportOnLeave | false |  |  |
| hysteresis | Hysteresis |  |  |
| timeToTrigger | TimeToTrigger |  |  |
| useAllowedCellList | false |  |  |
| } |  |  |  |
| eventA6 SEQUENCE { |  |  | EVENT_A6 |
| a6-Offset CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| reportOnLeave | false |  |  |
| hysteresis | Hysteresis |  |  |
| timeToTrigger | TimeToTrigger |  |  |
| useAllowedCellList | false |  |  |
| } |  |  |  |
| eventX1-r17 SEQUENCE { |  |  | EVENT_X1 |
| x1-Threshold1-Relay-r17 CHOICE { |  |  |  |
| sl-RSRP-r16 | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| x1-Threshold2-r17 CHOICE { |  |  |  |
| rsrp | Thres2 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| reportOnLeave-r17 | false |  |  |
| hysteresis-r17 | 0 |  |  |
| timeToTrigger-r17 | TimeToTrigger |  |  |
| useAllowedCellList-r17 | false |  |  |
| } |  |  |  |
| eventX2-r17 SEQUENCE { |  |  | EVENT_X2 |
| x2-Threshold-Relay-r17 CHOICE { |  |  |  |
| sl-RSRP-r16 | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| reportOnLeave-r17 | false |  |  |
| hysteresis-r17 | 0 |  |  |
| timeToTrigger-r17 | TimeToTrigger |  |  |
| useAllowedCellList-r17 | false |  |  |
| } |  |  |  |
| eventH1-r18 SEQUENCE { |  |  | EVENT_H1 |
| h1-Threshold-r18 | Altitude-r18 |  |  |
| h1-Hysteresis-r18 | HysteresisAltitude-r18 |  |  |
| reportOnLeave-r18 | false |  |  |
| timeToTrigger-r18 | TimeToTrigger |  |  |
| includeAltitudeUE-r18 | true |  |  |
| simulMultiTriggerSingleMeasReport-r18 | false |  |  |
| } |  |  |  |
| eventH2-r18 SEQUENCE { |  |  | EVENT_H2 |
| h2-Threshold-r18 | Altitude-r18 |  |  |
| h2-Hysteresis-r18 | HysteresisAltitude-r18 |  |  |
| reportOnLeave-r18 | false |  |  |
| timeToTrigger-r18 | TimeToTrigger |  |  |
| includeAltitudeUE-r18 | true |  |  |
| simulMultiTriggerSingleMeasReport-r18 | false |  |  |
| } |  |  |  |
| eventA3H1-r18 SEQUENCE { |  |  | EVENT_A3H1 |
| a3-Offset-r18 CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| reportOnLeave-r18 | false |  |  |
| a3-Hysteresis-r18 | Hysteresis |  |  |
| timeToTrigger-r18 | TimeToTrigger |  |  |
| useAllowedCellList-r18 | false |  |  |
| h1-Threshold-r18 | Altitude-r18 |  |  |
| h1-Hysteresis-r18 | HysteresisAltitude-r18 |  |  |
| includeAltitudeUE-r18 | true |  |  |
| simulMultiTriggerSingleMeasReport-r18 | false |  |  |
| } - |  |  |  |
| eventA3H2-r18 SEQUENCE { |  |  | EVENT_A3H2 |
| a3-Offset-r18 CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| reportOnLeave-r18 | false |  |  |
| a3-Hysteresis-r18 | Hysteresis |  |  |
| timeToTrigger-r18 | TimeToTrigger |  |  |
| useAllowedCellList-r18 | false |  |  |
| h2-Threshold-r18 | Altitude-r18 |  |  |
| h2-Hysteresis-r18 | HysteresisAltitude-r18 |  |  |
| includeAltitudeUE-r18 | true |  |  |
| simulMultiTriggerSingleMeasReport-r18 | false |  |  |
| } |  |  |  |
| eventA4H1-r18 SEQUENCE { |  |  | EVENT_A4H1 |
| a4-Threshold-r18 CHOICE { |  |  |  |
| Rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| reportOnLeave-r18 | false |  |  |
| a4-Hysteresis-r18 | Hysteresis |  |  |
| timeToTrigger-r18 | TimeToTrigger |  |  |
| useAllowedCellList-r18 | false |  |  |
| h1-Threshold-r18 | Altitude-r18 |  |  |
| h1-Hysteresis-r18 | HysteresisAltitude-r18 |  |  |
| includeAltitudeUE-r18 | true |  |  |
| simulMultiTriggerSingleMeasReport-r18 | false |  |  |
| } |  |  |  |
| eventA4H2-r18 SEQUENCE { |  |  | EVENT_A4H2 |
| a4-Threshold-r18 CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| reportOnLeave-r18 | false |  |  |
| a4-Hysteresis-r18 | Hysteresis |  |  |
| timeToTrigger-r18 | TimeToTrigger |  |  |
| useAllowedCellList-r18 | false |  |  |
| h2-Threshold-r18 | Altitude-r18 |  |  |
| h2-Hysteresis-r18 | HysteresisAltitude-r18 |  |  |
| includeAltitudeUE-r18 | true |  |  |
| simulMultiTriggerSingleMeasReport-r18 | false |  |  |
| } |  |  |  |
| eventA5H1-r18 SEQUENCE { |  |  | EVENT_A5H1 |
| a5-Threshold1-r18 CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| a5-Threshold2-r18 CHOICE { |  |  |  |
| rsrp | Thres2 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| reportOnLeave-r18 | false |  |  |
| a5-Hysteresis-r18 | Hysteresis |  |  |
| timeToTrigger-r18 | TimeToTrigger |  |  |
| useAllowedCellList-r18 | false |  |  |
| h1-Threshold-r18 | Altitude-r18 |  |  |
| h1-Hysteresis-r18 | HysteresisAltitude-r18 |  |  |
| includeAltitudeUE-r18 | true |  |  |
| simulMultiTriggerSingleMeasReport-r18 | false |  |  |
| } |  |  |  |
| eventA5H2-r18 SEQUENCE { |  |  | EVENT_A5H2 |
| a5-Threshold1-r18 CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| a5-Threshold2-r18 CHOICE { |  |  |  |
| rsrp | Thres2 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| reportOnLeave-r18 | false |  |  |
| a5-Hysteresis-r18 | Hysteresis |  |  |
| timeToTrigger-r18 | TimeToTrigger |  |  |
| useAllowedCellList-r18 | false |  |  |
| h2-Threshold-r18 | Altitude-r18 |  |  |
| h2-Hysteresis-r18 | HysteresisAltitude-r18 |  |  |
| includeAltitudeUE-r18 | true |  |  |
| simulMultiTriggerSingleMeasReport-r18 | false |  |  |
| } |  |  |  |
| } |  |  |  |
| rsType | ssb |  |  |
| reportInterval | ReportInterval |  |  |
| reportAmount | r2 |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | true |  |  |
| sinr | false |  |  |
|  | true |  | pc_ss_SINR_Meas |
| } |  |  |  |
| maxReportCells | 8 |  |  |
| reportQuantityRS-Indexes | Not present |  |  |
| maxNrofRS-IndexesToReport | Not present |  |  |
| includeBeamMeasurements | false |  |  |
| reportAddNeighMeas | Not present |  |  |
| measRSSI-ReportConfig-r16 | Not present |  |  |
|  | MeasRSSI-ReportConfig |  | SharedSpectrum |
| useT312-r16 | Not present |  |  |
| includeCommonLocationInfo-r16 | Not present |  | MDT |
| includeBT-Meas-r16 | Not present |  |  |
| includeBT-Meas-r16 CHOICE { |  |  | MDT_BT |
| setup | BT-NameList |  |  |
| } |  |  |  |
| includeWLAN-Meas-r16 | Not present |  |  |
| includeWLAN-Meas-r16 CHOICE { |  |  | MDT_WLAN |
| setup | WLAN-NameList |  |  |
| } |  |  |  |
| includeSensor-Meas-r16 | Not present |  |  |
| includeSensor-Meas-r16 CHOICE { |  |  | MDT_SENSOR |
| setup | Sensor-NameList |  |  |
| } |  |  |  |
| } |  |  |  |
| reportCGI SEQUENCE { |  |  | CGI OR AUTO_GAPS |
| cellForWhichToReportCGI | PhysCellId |  |  |
| useAutonomousGaps-r16 | Not present |  |  |
|  | setup |  | AUTO_GAPS |
| } |  |  |  |
| reportSFTD SEQUENCE { |  |  | SFTD_NEIGHBOUR or SFTD_PSCELL |
| reportSFTD-Meas | false |  | SFTD_NEIGHBOUR |
|  | true |  | SFTD_PSCELL |
| reportRSRP | false |  |  |
| reportSFTD-NeighMeas | true |  | SFTD_NEIGHBOUR |
|  | Not present |  | SFTD_PSCELL |
| drx-SFTD-NeighMeas | Not present |  |  |
| cellsForWhichToReportSFTD | Not present |  |  |
| } |  |  |  |
| condTriggerConfig-r16 SEQUENCE { |  |  | CHO, CPC |
| condEventId CHOICE { |  |  |  |
| condEventA3 SEQUENCE { |  |  | CHO AND EVENT_A3 |
| a3-Offset CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| hysteresis | Hysteresis |  |  |
| timeToTrigger | TimeToTrigger |  |  |
| } |  |  |  |
| condEventA5 SEQUENCE { |  |  | CHO AND EVENT_A5 |
| a5-Threshold1 CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| a5-Threshold2 CHOICE { |  |  |  |
| rsrp | Thres2 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| hysteresis | Hysteresis |  |  |
| timeToTrigger | TimeToTrigger |  |  |
| } |  |  |  |
| } |  |  |  |
| rsType-r16 | ssb |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| EVENT_A1 | Configuration of Event A1 |
| EVENT_A2 | Configuration of Event A2 |
| EVENT_A3 | Configuration of Event A3 |
| EVENT_A4 | Configuration of Event A4 |
| EVENT_A5 | Configuration of Event A5 |
| EVENT_A6 | Configuration of Event A6 |
| PERIODICAL | Configuration of periodical reporting |
| CGI | Configuration of CGI measurement |
| AUTO_GAPS | Configuration of Autonomous Gaps |
| CHO | Configuration of conditional handover |
| CPC | Conditional PSCell change |
| MDT | Configuration of MDT |
| MDT_BT | Configuration of MDT including Bluetooth measurements |
| MDT_WLAN | Configuration of MDT including WLAN measurements |
| MDT_SENSOR | Configuration of MDT including Sensor measurements |
| MDT_DELAY | Configuration of UL PDCP Packet Delay per DRB |
| SFTD_NEIGHBOUR | Configurations of SFTD measurement on NR neighbour |
| SFTD_PSCELL | Configurations of SFTD measurement on NR PSCell |
| SharedSpectrum | Operation with shared spectrum channel access |
| EVENT_X1 | Configuration of Event X1 |
| EVENT_X2 | Configuration of Event X2 |
| EVENT_H1 | Configuration of Event H1 |
| EVENT_H2 | Configuration of Event H2 |
| EVENT_A3H1 | Configuration of Event A3H1 |
| EVENT_A3H2 | Configuration of Event A3H2 |
| EVENT_A4H1 | Configuration of Event A4H1 |
| EVENT_A4H2 | Configuration of Event A4H2 |
| EVENT_A5H1 | Configuration of Event A5H1 |
| EVENT_A5H2 | Configuration of Event A5H2 |

<Unchanged Sections Omitted>