3GPP TSG-RAN5 Meeting #108	R5-255044

# Bengaluru, India, 25th Aug 2025 – 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5111 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new UAV Test Case 8.1.3.5.4 Test Case for event A3H1 reporting for Inter-Frequency Neighbour |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM JAPAN LLC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UAV_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test case 8.1.3.5.4 needs to be added as per NR_UAV_plus_CT1-UEConTest WP |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case 8.1.3.5.4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This aspect of WP will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3.5.4 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.523-2 CR0616, TS/TR 38.508-1 CR3549 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from Original R5-254277 |  |  |  |  |  |  |  |  |

### <Unchanged Sections Omitted>

##### 8.1.3.5.4		NR UAV / Measurement configuration control and reporting / Event A3H1 / Measurement of Neighbour NR cell / Inter-frequency measurements

8.1.3.5.4.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and inter-frequency measurement configured for event A3H1 }

ensure that {

when { Neighbour cell becomes better than serving cell and the Aerial UE altitude becomes higher than the absolute threshold plus hysteresis }

then { UE sends MeasurementReport with correct measdID for event A3H1}

}

(2)

with { UE in RRC_CONNECTED state and inter-frequency measurement configured for event A3H1 }

ensure that {

when { Neighbour cell becomes better than serving cell and the Aerial UE altitude becomes lower than the absolute threshold minus hysteresis }

then { UE stops sending MeasurementReport }

}

(3)

with { UE in RRC_CONNECTED state and inter-frequency measurement configured for event A3H1 }

ensure that {

when { Neighbour cell becomes lower than serving cell and the Aerial UE altitude becomes lower than the absolute threshold minus hysteresis }

then { UE does not send MeasurementReport }

}

8.1.3.5.4.2	Conformance requirements

Same as 8.1.3.5.2.2 with the following difference:

[TS 38.331, clause 5.5.2.9]

The UE shall:

. . .

1>	if gapUE is set to setup:

2>	if a per UE measurement gap configuration configured by gapUE is already setup, release the per UE measurement gap configuration;

2>	setup the per UE measurement gap configuration indicated by the gapUE in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	apply the specified timing advance mgta to the gap occurrences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurrences);

8.1.3.5.4.3	Test description

8.1.3.5.4.3.1	Pre-test conditions

Same as 8.1.3.5.3.3.1 with the following differences

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

-	System information combination: NR-4 replaces NR-2.

8.1.3.5.4.3.2	Test procedure sequence

Same as test case 8.1.3.5.2 with the following differences:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

8.1.3.5.4.3.3	Specific message contents

Same as test case 8.1.3.5.2 with the following difference:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

Table 8.1.3.5.4.3.3-1: MeasConfig (step 3, Table 8.1.3.5.4.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 2 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-f1 | Table 8.1.3.5.4.3.3-2 |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measObjectId | 2 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-f2 | Table 8.1.3.5.4.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-EventA3H1 | Table 8.1.3.5.2.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.5.4.3.3-2: MeasObjectNR-f1 (Table 8.1.3.5.4.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 1 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

Table 8.1.3.5.4.3.3-3: MeasObjectNR-f1 (Table 8.1.3.5.4.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 3 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

<Unchanged Sections Omitted>