3GPP TSG-RAN5 Meeting #108	R5-255057

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4928 | rev | 1 | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Routine maintenance for TS 36.523-3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2025-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - CR R5-253187, agreed at RAN5#107, has not been implemented fully correctly in the specification. - MCE is not a term used in the 3GPP core specifications. - According to 7A.14.2.4 the RRC connection release sequence is scheduled at time T being at least TNTN,offset in advance. But TS 36.508, cl. 6.3.5.2, defines TNTN,offset value as low as 5ms in case of NGSO scenario, which contradicts with statement “Assuming search space cycles of 64ms as described in clause 7A.3 and with a given search space cycle N starting at time T being at least 100ms in advance” from cl. 7A.7 - PUR services may be required to transport user data |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Missing changes have been re-applied in clauses 4.2.4.2.2 and 7A.2.2.3.2. - The MAC CEs have been renamed to align with their name in the 3GPP core specifications. - “TTCN schedules the RRC connection release sequence at time T being at least TNTN,offset in advance” from cl. 7A.14.2.4 is made exclusive to GSO scenario - The note in the table of MMI commands has been updated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - RAN5 agreements are missing in the specification. - The test specification will remain misaligned with the core specifications. - A conformant NB-IoT NTN UE may fail the NTN test cases on NGSO scenario. - Test case 22.1.5 cannot be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.4.2.2, 5.2, 7.16.1a, 7.19, 7A.2.2.3.2, 7A.14.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Change to the clause 7A.14.2.4 corresponds to R5s250272 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the outcome of 1 revision of R5-254000. |  |  |  |  |  |  |  |  |

##### 4.2.4.2.2	Socket Release

A socket is released:

-	in case of TCP when the remote entity closes the connection;

-	when it is closed explicitly by the IP_PTC (system call 'close').

NOTE 1:	In general, the sockets are independent from the configuration of the DRBs. Especially in case of UDP or ICMP the sockets can exist even without any DRB being configured.

NOTE 2:

For IMS, basically TCP close is expected to be done from the client’s end: 
The SS closes its TCP connections only when the associated security context is released e.g. at IMS de-registration, so the SS keeps the TCP connections even at RRC connection release.
In general, the UE may close its TCP connections at any point in time but especially it may close its unprotected TCP connection after IMS registration. Therefore, the SS shall wait some time before releasing the RRC connection to allow the UE to close a TCP connection. 
In detail
- after initial registration the SS (TTCN) waits T seconds for the UE to close its unprotected TCP connection
- after de-registration the SS (TTCN) waits T seconds for the UE to close its client TCP connections; after this is done the TTCN closes all remaining TCP connection (independently of the server/client role)
- as special case in general, after an emergency call the same procedure is applied as for de-registration
T is 3s.

<SECTIONS SKIPPED>

## 5.2	Upper Tester ASPs

There is one primitive defined carrying either an MMI or an AT command to be sent to the system adaptor and one common confirmation primitive to be sent by the system adaptor.

| TTCN-3 ASP Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name | UT_SYSTEM_REQ |  |  |
| TTCN-3 Type | Record |  |  |
| Cmd |  | TTCN-3 Type | union |
| AT |  | charstring carrying the AT command as defined in TS 27.007 [32], TS 27.005 [31] and TS 27.060 [33] |  |
| MMI |  | - Cmd (charstring) - List of parameters: - Name (charstring) - Value (charstring) |  |
| CnfRequired |  | TTCN-3 Type | Ut_CnfReq_Type |
|  |  | CNF_REQUIRED: SS shall reply with one confirmation (NOTE) NO_CNF_REQUIRED: SS shall swallow any confirmation generated LOCAL_CNF_REQUIRED: SS shall immediately send one confirmation when the command is submitted to the UE i.e. in case of operator interaction when the operator has confirmed the command, but SS shall not wait for the UE responding(NOTE) |  |
| NOTE: In the TTCN, a confirmation shall only be requested in cases when there is no signalling from the UE being triggered by the MMI/AT command |  |  |  |

| TTCN-3 ASP Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name | UT_COMMON_CNF |  |  |
| TTCN-3 Type | Record |  |  |
| Result |  | TTCN-3 Type | boolean |
|  |  | true: success false: failure NOTE: The return value false will result in the test immediately finishing with a fatal error. In case of AT commands, the SS shall return false if and only if the UE comes back with "ERROR" as result code according to ITU-T Rec V.250 clause 5.7 [60]. In case of e.g. "+CME ERROR" the SS shall return true and put the UE's response into the ResultString |  |
| ResultString |  | TTCN-3 Type | charstring |
|  |  | response by the UE for commands which request the UE to return a result, optional In case of AT commands, ResultString shall contain the complete line according to the specification of the AT command. Any carriage return and linefeed characters, defined in TS 27.007 [32] clause 3.1, contained in the response shall be included as the textual representations "<CR>" and "<LF>" respectively. |  |

The following MMI commands are defined.

Table 5.1: MMI commands

| Command | Parameters |  |
| --- | --- | --- |
|  | Name | Value |
| "SWITCH_ON" | (none) |  |
| "SWITCH_OFF" | (none) |  |
| "POWER_ON" | (none) |  |
| "POWER_OFF" | (none) |  |
| "INSERT USIM" | "USIM" | <USIM> |
| "REMOVE_USIM" | "USIM" (optional) | <USIM> |
| "CHECK_PLMN" | "PLMN" | <PLMN ID> |
| "CHECK_ETWS_INDICATION" | “WARNING1” | <WARNING1> |
|  | “WARNING2” | <WARNING2> |
| “CHECK_ETWS_ALERT” | (none) |  |
| “CHECK_ETWS_NO_ALERT” | (none) |  |
| "CHECK_CMAS_INDICATION" | “WARNING1” | <WARNING1> |
|  | “WARNING2” | <WARNING2> |
| “CHECK_CMAS_ALERT” | (none) |  |
| “CHECK_CMAS_NO_ALERT” | (none) |  |
| "HRPD_PDN_CONNECTION” | (none) |  |
| “CHECK_SMS_LENGTH_CONTENTS" | "Length" | <Length> |
|  | "Msg" | <Msg> |
| "CONFIGURE_SMS_ONLY" | (none) |  |
| "DISABLE EPS CAPABILITY" | (none) |  |
| "DETACH_NON_EPS" | (none) |  |
| "CLEAR_STORED_ASSISTANCE_DATA" | (none) |  |
| "CHECK_DTCH_THROUGHCONNECTED" | (none) |  |
| "GERAN_UPLINK_DATA" | (none) |  |
| "SELECT_CSG" | "PLMN" | <PLMN ID> |
|  | "CSG" | <CSG ID> |
| “TRIGGER_USER_RESELECTION” | (none) |  |
| "REQUEST_NON_CALL_RELATED_SS" | (none) |  |
| "MBMS_SERVICE_INTEREST" | "PLMN" | <PLMN ID> |
|  | "Service" | <MBMS Service ID> |
|  | "Interest" | "ON" / "OFF" |
|  | "SAI" | <MBMS SAI> |
| “MBMS_SERVICE_ACTIVE” | "Service" | <MBMS Service ID> |
|  | "SAI" | <MBMS SAI> |
|  | “Active” | "ON" / "OFF" |
| “MBMS_PRIORITY_OVER_UNICAST” | "Priority Over Unicast" | "FALSE" / "TRUE" |
| "CHECK_ETWS_NO_INDICATION" | “WARNING1” | <WARNING1> |
|  | “WARNING2” | <WARNING2> |
| "CHECK_CMAS_NO_INDICATION" | “WARNING1” | <WARNING1> |
|  | “WARNING2” | <WARNING2> |
| "CHECK_CALL_DISPLAY" | "ToDisplay" | <NAME>/<NUMBER> |
| "CHECK_CSG" | "CSG" | < CSG ID > |
|  | "Included" | "FALSE" / "TRUE" |
| "CHECK_NITZ_DST" | "DaylightSavingTime:" | DST |
| "CHECK_RESPONSE_DISPLAY" | "ToDisplay" | <STRING> |
| "CHECK_SS_USER_IND" | "Supplementary Service" | <SUPPLEMENTARY SERVICE> |
|  | "ToDisplay" | <STRING> |
| “CHECK_SS_USER_IND_FAILURE” | (none) |  |
| "CONFIGURE_OPERATIONMODE" | "OPERATION MODE" | <OPERATION MODE> |
| “CONFIGURE_ANDSF_RULE” | “ANDSF Rule” | <ANDSF RULE> |
| “WLAN_ANDSF_CONFIGURED” | (none) |  |
| “WLAN_ANDSF_NOT_CONFIGURED” | (none) |  |
| "CMTLR_DUMMY" (see NOTE 1) | ”Notification Type” | <NOTIFICATION TYPE> |
| "CCFC_DUMMY" (see NOTE 1) | ”Status” | <STATUS> |
| "CNAP_DUMMY" (see NOTE 1) | ”Testname” | <TESTNAME> |
| "CTZE_DUMMY" (see NOTE 1) | "Year | <YEAR> |
|  | "Month" | <MONTH> |
|  | "Day" | <DAY> |
|  | "Hour" | <HOUR> |
|  | "Minutes" | <MINUTES> |
|  | "Seconds" | <SECONDS> |
|  | “Daylight SavingTime” | <DAYLIGHT SAVING TIME> |
|  | "TimeZone" | <TIMEZONE> |
| "CMWN_DUMMY" (see NOTE 1) | (none) |  |
| “D2D_DISCOVERY_CLEAR” | (none) |  |
| “D2D_DISCOVERY_CONFIGURATION” | “ProSeApplicationID” | <charstring> |
|  | "PLMN_List" | <PLMN ID_List> |
|  | T4005 | <T4005> |
| “D2D_DISCOVERY” | “ProSeApplicationID” | <charstring> |
|  | “Announce” | "ON" / "OFF" |
|  | “Monitor” | "ON" / "OFF" |
| “D2D_COMMUNICATION” | “ProSe Layer-2 Group ID” | <charstring> |
|  | “Transmit” | "ON" / "OFF" |
|  | “Receive” | "ON" / "OFF" |
|  | “Max size of UE transmitted Direct Communication data” | <bytes> |
| “D2D_COMMUNICATION_CONFIGURATION” | “ProSe Layer-2 Group ID” | <charstring> |
|  | "PLMN_List" | <PLMN ID_List> |
|  | T4005_Minutes | <T4005> |
|  | “Security Life time timer in minutes” (Default: 0) | <SecurityTimer> |
| "V2X_SIDELINK" | "Data Type" | "IP" / "NON-IP" |
| “INSERT_USIM_OR_PRECONFIGURE” | "USIM" | <USIM> |
| “DELETE_NTN_PLMN_NA” | (none) |  |
| “CONFIGURE_NTN_UE_POSITION” | "SpecReference" | <charstring> |
| “SATELLITE_TRAJECTORY_PREDICTION” | “Prediction” | “DISABLE” / ”ENABLE” |
| “SENSE” | “State” | “CONFIGURED” / “NOT_CONFIGURED” |
| “TRIGGER_EARLYDATA” | "DATA TYPE" | “EDT” / “PUR” / “EDT_PUR” |
|  | "DATA" (see NOTE 2) | <Integer> |
| NOTE 1: These MMI commands are used to indicate to the SS that a UT response is expected by the TTCN in the case when the AT command is not supported. They can be ignored if the UE supports the AT command. The prefix of these commands matches the start of the AT response expected by the TTCN. NOTE 2: Optional. Only included if test case requires service as a transport mechanism for user data. |  |  |

<SECTIONS SKIPPED>

### 7.16.1a	Sequence of inter-cell CA handover (more than one CC before and after handover)

The Inter-Cell handover is done with activation time, i.e. the timing information for configuration of the SS and sending of the RRCConnectionReconfiguration is explicit. Time ‘T’ is set to 700 ms in advance of the handover, time T1 = T + 10 ms and time T2 = T +20 ms.

At Time T, steps 1-3:

1.	Source Pcell: Configure source primary cell for stop of automatic Timing Advance Command MAC CE transmission by SS

2.	Target Pcell: Configure target pcell for no RACH response transmission

3.	Source Pcell: Schedule the transmission of RRC Connection Reconfiguration message to UE requesting Handover to target Pcell and Scell

At time T2, steps 4-5:

4.	Target Pcell: If target Pcell is same as source Scell, configure SS for target Pcell to be converted from a Scell to Pcell

4a.	Configuration of DRBs at the target PCell

5.	Transfer of the PDCP Count for AM DRBs from source to target Pcell:

a)	Source PCell: Get PDCP COUNT.

b)	Target PCell: Set PDCP COUNT.

NOTE 1:	There shall be no further sending/receiving of AM DRB data before the HO has been done.

At time T1, steps 6-12:

6.	Target PCell:	Inform the SS about the HO and about the source Pcell id.

7.	Target PCell:	Configure RACH procedure either dedicated or C-RNTI based.

8.	Target PCell:	Activate security.

NOTE 2:	For AM DRBs the PDCP count is maintained (for SRBs and UM DRBs the PDCP count is reset).

9.	Target PCell & Target Scell:	configure DRX.

NOTE 3:	As long as the DRX configuration is not modified by the RRCConnectionReconfiguration the target cell gets the same DRX configuration as the source cell.

NOTE 4:	According to TS 36.331 clause 5.5.6.1 the measurement gap configuration is released at the UE due to the handover, therefore nothing needs to be configured at the target cell regarding measurement gaps.

10.	Target PCell:	Configure UL grant configuration ("OnSR", periodic TA is not started).

11.	Target PCell:	Configure Target Pcell as Pcell.

12.	Target SCell:	Configure Target Scell for

12.1	Configure RACH procedure C-RNTI based.

12.2	Configure UL grant configuration ("OnSR", periodic TA is not started).

12.3	Configure target SCell as Scell with new Pcell association.

After time T2 (without activation time):

13.	Target PCell:	Receive RRCConnectionReconfigurationComplete.

14.Target PCell:	Start periodic TA.

15.	Target PCell:	Inform the SS about completion of the HO (e.g. to trigger PDCP STATUS PDU).

16.	Target PCell:	Re-configure RACH procedure as for initial access.

16a.	Target Pcell & TargetSCell: Configure measurement gap configuration (if necessary).

17.	Source PCell:	If source Pcell is not target Scell, reset SRBs and release DRBs.

18.	Source Pcell:	If source Pcell is not target Scell, configure from Pcell to normal cell.

19.	Source Pcell:	If source Pcell is not target Scell, Release DRX and MeasGap configuration

20.	Source SCell:	If source Scell is neither target Pcell or Scell, Release DRX and MeasGapConfig configuration.

21.	Source Scell:	If source Scell is neither target Pcell or Scell, configure from Scell to normal cell.

22.	Source Scell:	If source Scell is neither target Pcell or Scell, Configure UL grant configuration ("OnSR", periodic TA is not started).

<SECTIONS SKIPPED>

## 7.19	DL CCCH Message and UE Contention Resolution Identity MAC CE transmission in one MAC PDU or in separate MAC PDUs

When the contention based RACH procedure is being executed (RRC Connection Establishment or RRC Connection Reconfiguration), in general the UE Contention Resolution Identity MAC control element and the DL RRC PDU (RRC Connection Setup/RRC Connection Reject/RRC Connection Re-establishment/RRC Connection Re-establishment Reject) are sent in one MAC PDU. This is achieved by pre-configuring the SS (before the start of the RRC procedure) to send the encoded DL message and the UE Contention Resolution MAC CE in one MAC PDU.

Nevertheless, due to specific test purposes there are still many cases where it is necessary to send the DL CCCH message separately, this is implemented in TTCN using the test case attribute EarlyContentionResolution:

-	RRC connection establishment:

-	When RRC connection establishment is part of the test purpose

-	Special cases: , e.g. when no contention resolution shall be sent according to the test purpose

-	RRC Connection Reestablishment is part of the test purpose

-	RRC Connection Reject is part of the test purpose

-	RRC Connection Reestablishment Reject is part of the test purpose

The way contention resolution is applied has impact on the DCI format being used in a test case: when the DL CCCH message is sent separately DCI combination 1 according to clause 7.3.1 shall be used.

<SECTIONS SKIPPED>

##### 7A.2.2.3.2	Combinations scheduling 2, 3 or 4 SI messages

TS 36.508 [3] clause 8.1.4.3 specifies configuration of SI combination 2 onwards scheduling as shown in table 7A.2.1.3.1-2 with

-	16 repetitions of SIB1-NB  SIB1-NB in every second frame at subframe 4

-	si-WindowLength-r13: 			ms160

-	si-RadioFrameOffset-r13: 		0 (not present in SIB1-NB)

-	2, 3 or 4 SI-messages:
-	SI-Window 1
	si-Periodicity-r13				rf64
	si-RepetitionPattern-r13		every8thRF
	si-TB-r13						552  NSF = 8
-	SI-Window 2
	si-Periodicity-r13				rf64
	si-RepetitionPattern-r13		every16thRF
	si-TB-r13						256  NSF = 8
-	SI-Window 3
	si-Periodicity-r13				rf64
	si-RepetitionPattern-r13		every16thRF
	si-TB-r13						256  NSF = 8
-	SI-Window 4
	si-Periodicity-r13				rf64
	si-RepetitionPattern-r13		every16thRF
	si-TB-r13						256  NSF = 8

-	According to TS 36.331 [19] clause 5.2.3a the SI-windows start at subframe 0 of frame
			(H-SFN * 1024 + SFN) mod TSI1 = 0; 			with TSI1 = 64
			(H-SFN * 1024 + SFN) mod TSI2 = 16; 		with TSI2 = 64
			(H-SFN * 1024 + SFN) mod TSI3 = 32; 		with TSI3 = 64
			(H-SFN * 1024 + SFN) mod TSI4 = 48; 		with TSI3 = 64

Table 7A.2.2.3-2: Combinations scheduling 2, 3 or 4 SI messages

| SI Window | K | SFN = K |  |  |  |  |  |  |  |  |  | SFN = K + 1 |  |  |  |  |  |  |  |  |  | SFN = K + 2 |  |  |  |  |  |  |  |  |  | SFN = K + 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
|  | ... |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SI-1 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SI-2 | 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SI-3 | 32 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 36 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 44 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SI-4 | 48 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 56 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SI-1 | 64 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 68 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 72 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 76 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SI-2 | 80 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 84 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 88 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 92 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SI-3 | 96 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 104 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 108 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SI-4 | 112 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 116 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 124 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SI-1 | 128 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 132 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 136 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 140 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ... |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

With

|  | NPBCH |  | NPSS |  | NSSS |  | SIB1-NB |  | SI-1 |  | SI-2 |  | SI-3 |  | SI-4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

<SECTIONS SKIPPED>

#### 7A.14.2.4	RRC connection release sequence

The sequence specified in clause 7A.7 applies, with the following differences:

For the GSO scenario, TTCN schedules the RRC connection release sequence at time T being at least TNTN,offset in advance:

2.	At T + 1 NPDCCH period bundle:
Scheduling of UL grants for search space cycle starting at (T + 1 NPDCCH period bundle) and four subsequent cycles

3.	At T + 1 NPDCCH period bundle + 16ms

4.	At T + 1 NPDCCH period bundle + 32ms