3GPP TSG-RAN5 Meeting #108 	R5-255228

# Bengaluru, India, 25th Aug. 2025 - 29th Aug. 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3501 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of reference sensitivity requirements for Rel-18 2DL CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu, Huawei, HiSilicon, NTT DOCOMO INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The following 2DL CA configurations are missing in 7.3A Reference sensitivity. - CA_n1A-n26A - CA_n8A-n77A - CA_n26A-n78A |  |  |  |  |  |  |  |  |
| Summary of change: |  | - CA_n1A-n26A was added to Table 7.3A.1.5-1. - CA_n8A-n77A was added to Tables 7.3A.0.4-1, 7.3A.0.4-4b, 7.3A.0.5-1, 7.3A.1.5-1, 7.3A.1_1.4.1-1, and 7.3A.1_1.5-1. - CA_n26A-n78A was added to Tables 7.3A.0.4-1, 7.3A.0.4-4b, 7.3A.0.5-1, 7.3A.1.5-1, 7.3A.1_1.4.1-1, and 7.3A.1_1.5-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The missing CA configurations will not be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.0.4, 7.3A.0.5, 7.3A.1, 7.3A.1_1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TP analysis: TR 38.905 CR 1070 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-254751 and the outcome of 1 revision to this document. r1: Changes to Table 7.3A.0.3.2.1-1 were reverted to solve overlap with R5-254791. Clarified MHz and UL in Table 7.3A.1_1.4.1-1. |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

7.3A.0.4	Reference sensitivity exceptions due to UL harmonic interference for CA

Sensitivity degradation is allowed for different combinations of UL configurations and DL channel bandwidths if a band in frequency range 1 is impacted by UL harmonic interference from another band which belongs to NR band in frequency range 1 of the same downlink CA configuration. Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC3 aggressor NR UL band for either single band uplink or PC3 or PC2 CA are specified in Table 7.3A.0.4-1. For these exceptions, only the listed test points in Table 7.3A.0.4-1 are needed to be tested.

Table 7.3A.0.4-1: Reference sensitivity exceptions due to UL harmonic for NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n1 | n77 | 5 | 15 | 12 | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n1 | n77 | 5 | 15 | 12 | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n2 | n48 | 5 | 15 | 25 | 5 | 8.1 | NOTE 6 | UL2/DL1 near-miss |
| n2 | n77 | 5 | 15 | 12 | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n2 | n77 | 5 | 15 | 12 | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n3 | n77 | 5 | 15 | 12 | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n3 | n77 | 5 | 15 | 12 | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n3 | n78 | 5 | 15 | 12 | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n3 | n78 | 5 | 15 | 12 | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n5 | n778 | 5 | 15 | 6 | 10 | 10.5 | NOTE 4 | UL4/DL1 direct-hit |
| n5 | n778 | 5 | 15 | 6 | 100 | 2.9 | NOTE 4 | UL4/DL1 direct-hit |
| n5 | n778 | 5 | 15 | 6 | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| n5 | n778 | 5 | 15 | 6 | 100 | 2.9 | NOTE 5 | UL5/DL1 direct-hit |
| n5 | n78 | 5 | 15 | 6 | 10 | 10.5 | NOTE 4 | UL4/DL1 direct-hit |
| n5 | n78 | 5 | 15 | 6 | 100 | 2.9 | NOTE 4 | UL4/DL1 direct-hit |
| n8 | n77 | 5 | 15 | 6 | 10 | 10.8 | NOTE 4 | UL4/DL1 direct-hit |
| n8 | n77 | 5 | 15 | 6 | 100 | 3.1 | NOTE 4 | UL4/DL1 direct-hit |
| n8 | n78 | 5 | 15 | 6 | 10 | 10.8 | NOTE 4 | UL4/DL1 direct-hit |
| n8 | n78 | 5 | 15 | 6 | 100 | 3.1 | NOTE 4 | UL4/DL1 direct-hit |
| n14 | n77 | 5 | 15 | 5 | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| n14 | n77 | 5 | 15 | 5 | 100 | 2.9 | NOTE 5 | UL5/DL1 direct-hit |
| n20 | n78 | 5 | 15 | 6 | 10 | 10.8 | NOTE 4 | UL4/DL1 direct-hit |
| n20 | n78 | 5 | 15 | 6 | 100 | 3.1 | NOTE 4 | UL4/DL1 direct-hit |
| n24 | n7712 | 5 | 15 | 12 | 10 | N/A | NOTE 2 | UL2/DL1 direct-hit |
| n25 | n77 | 5 | 15 | 12 | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n25 | n77 | 5 | 15 | 12 | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n25 | n78 | 5 | 15 | 12 | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n25 | n78 | 5 | 15 | 12 | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n26 | n78 | 5 | 15 | 6 | 10 | 10.8 | NOTE 4 | UL4/DL1 direct-hit |
| n26 | n78 | 5 | 15 | 6 | 100 | 3 | NOTE 4 | UL4/DL1 direct-hit |
| n28 | n1 | 5 | 15 | 8 | 5 | 10.2 | NOTE 3 | UL3/DL1 direct-hit |
| n28 | n1 | 5 | 15 | 8 | 50 | 2.6 | NOTE 3 | UL3/DL1 direct-hit |
| n28 | n77 | 5 | 15 | 5 | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| n28 | n77 | 5 | 15 | 5 | 100 | 2.9 | NOTE 5 | UL5/DL1 direct-hit |
| n28 | n78 | 5 | 15 | 5 | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| n28 | n78 | 5 | 15 | 5 | 100 | 2.9 | NOTE 5 | UL5/DL1 direct-hit |
| n66 | n48 | 5 | 15 | 12 | 5 | 27.1 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n48 | 5 | 15 | 12 | 1007 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n77 | 5 | 15 | 12 | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n77 | 5 | 15 | 12 | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n78 | 5 | 15 | 12 | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n78 | 5 | 15 | 12 | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n71 | n41 | 5 | 15 | 6 | 10 | 10.8 | NOTE 4 | UL4/DL1 direct-hit |
| n71 | n41 | 5 | 15 | 6 | 100 | 3.1 | NOTE 4 | UL4/DL1 direct-hit |
| n71 | n70 | 5 | 15 | 8 | 5 | 9.9 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | n70 | 5 | 15 | 8 | 25 | 4.1 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | n77 | 5 | 15 | 5 | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| n71 | n78 | 5 | 15 | 5 | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| NOTE 1: The direct-hit requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd / 3rd / 4th / 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. The requirements should be verified using RBstart = floor((NRB-LCRB)/2), where floor(x) is the greatest integer less than or equal to x, and where the UL parameters NRB and LCRB are respectively, the transmission bandwidth configuration and the number of RB’s for the specified UL band channel bandwidth and the UL band subcarrier spacing. NOTE 2: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 7: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 9: Void. NOTE 10: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band n25 is located with its upper edge at 1995 MHz. NOTE 11: These requirements apply when the lower edge frequency of the uplink channel in Band n71 is located at or below 668 MHz and the downlink channel in Band n25 is located with its upper edge at 1990 MHz. NOTE 12: In the USA, n77 band is restricted to 3450 – 3550 MHz and 3700 – 3980 MHz. There is no UL harmonic due to n24 UL in downlink for n77 operating in 3450 – 3550 MHz and 3700 – 3980 MHz. NOTE 13: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity for all active downlink component carriers is only verified when this is not the case (the requirements specified in clause 7.3.2 apply unless otherwise specified). |  |  |  |  |  |  |  |  |

Table 7.3A.0.4-1a: Void

Table 7.3A.0.4-2: Void

Table 7.3A.0.4-3: Void

Table 7.3A.0.4-3a: Void

Sensitivity degradation is allowed for different combinations of UL configurations and DL channel bandwidths if a band is impacted by receiver harmonic mixing due to another band part which belongs to PC3 NR band or PC2 NR band of the same CA configuration. Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for either PC3 or PC2 CA are specified in Table 7.3A.0.4-4b and from a PC2 aggressor NR UL band for PC2 CA are specified in Table 7.3A.0.4-4c. For these exceptions, only the listed test points in Table 7.3A.0.4-4b, Table 7.3A.0.4-4c and Table 7.3A.0.4-4d are needed to be tested. Sensitivity degradation is not required for receiver even order harmonic mixing with aggressor 3rd order and above harmonic interference.

Table 7.3A.0.4-4: Void

Table 7.3A.0.4-4a: Void

Table 7.3A.0.4-4b: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n3 | n5 | 5 | 15 | 25 | 5 | 4 | NOTE 7 | UL1/DL2 |
| n39 | n41 | 5 | 15 | 6 | 10 | 9.3 | NOTE 11 | UL4/DL3 |
| n39 | n41 | 5 | 15 | 6 | 100 | 2.2 | NOTE 11 | UL4/DL3 |
| n40 | n283 | 10 | 15 | 25 | 5 | 37.8 | NOTE 4 | UL1/DL3 |
| n40 | n283 | 10 | 15 | 25 | 20 | 30.1 | NOTE 4 | UL1/DL3 |
| n41 | n39 | 10 | 15 | 8 | 5 | 4.3 | NOTE 12 | UL3/DL4 |
| n41 | n39 | 10 | 15 | 8 | 40 | 0.8 | NOTE 12 | UL3/DL4 |
| n77 | n2 | 10 | 15 | 25 | 5 | 6.7 | NOTE 7 | UL1/DL2 |
| n77 | n2 | 10 | 15 | 25 | 20 | 2.8 | NOTE 7 | UL1/DL2 |
| n77 | n5 | 10 | 15 | 25 | 5 | 5.7 | NOTE 8 | UL1/DL4 |
| n77 | n5 | 10 | 15 | 25 | 20 | 0.8 | NOTE 8 | UL1/DL4 |
| n77 | n8 | 10 | 15 | 25 | 5 | 5.7 | NOTE 8 | UL1/DL4 |
| n77 | n8 | 10 | 15 | 25 | 20 | 0.8 | NOTE 8 | UL1/DL4 |
| n77 | n14 | 10 | 15 | 25 | 5 | 31 | NOTE 5 | UL1/DL5 |
| n77 | n14 | 10 | 15 | 25 | 10 | 27.8 | NOTE 5 | UL1/DL5 |
| n77 | n25 | 10 | 15 | 25 | 5 | 5.6 | NOTE 7 | UL1/DL2 |
| n77 | n25 | 10 | 15 | 25 | 40 | 0.3 | NOTE 7 | UL1/DL2 |
| n77 | n30 | 10 | 15 | 12 | 5 | 10.4 | NOTE 2 | UL2/DL3 |
| n77 | n30 | 10 | 15 | 12 | 10 | 7.6 | NOTE 2 | UL2/DL3 |
| n77 | n40 | 10 | 15 | 12 | 10 | 11.7 | NOTE 2 | UL2/DL3 |
| n77 | n40 | 10 | 15 | 12 | 100 | 3.6 | NOTE 2 | UL2/DL3 |
| n77 | n41 | 10 | 15 | 12 | 10 | 11.7 | NOTE 2 | UL2/DL3 |
| n77 | n41 | 10 | 15 | 12 | 100 | 3.6 | NOTE 2 | UL2/DL3 |
| n78 | n26 | 10 | 15 | 25 | 5 | 5.4 | NOTE 8 | UL1/DL4 |
| n78 | n28 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 5 | UL1/DL5 |
| NOTE 1: FFS. NOTE 2: The requirements should be verified for DL NR-ARFCN of the Victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with fULHB the UL carrier frequency and BWChannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 3: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel's transmission bandwidth of an aggressor (higher) band. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 5: The requirements should be verified for DL NR-ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 6: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 7: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 8: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 9: FFS. NOTE 10: FFS. NOTE 11: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 12: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 13: The requirements should be verified using RBstart = floor((NRB-LCRB)/2), where floor(x) is the greatest integer less than or equal to x, and where the UL parameters NRB and LCRB are respectively, the transmission bandwidth configuration and the number of RB’s for the specified UL band channel bandwidth and the UL band subcarrier spacing. |  |  |  |  |  |  |  |  |

<<Unchanged tables skipped>>

7.3A.0.5	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA

For inter-band carrier aggregation with uplink assigned to two CCs from up to two UL NR bands and three CCs from two UL NR bands given in Table 7.3A.0.5-1, Table 7.3A.0.5-1a, Table 7.3A.0.5-1b, Table 7.3A.0.5-2 and Table 7.3A.0.5-2a the reference sensitivity is defined only for the specific uplink and downlink test points specified in Table 7.3A.0.5-1, Table 7.3A.0.5-1a, Table 7.3A.0.5-1b, Table 7.3A.0.5-2 and Table 7.3A.0.5-2a. For these test points the reference sensitivity requirement specified in Table 7.3.2.3-1a, Table 7.3.2.3-1b, Table 7.3.2.3-2 and Table 7.3.2.3-2a are relaxed by the amount of the corresponding parameter MSD given in Table 7.3A.0.5-1, Table 7.3A.0.5-1a, Table 7.3A.0.5-1b, Table 7.3A.0.5-2 and Table 7.3A.0.5-2a.

Table 7.3A.0.5-1: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n3 | n1 | 1950 | 5 | 25 | 2140 | 23 | FDD | IMD3 |
|  | n3 | 1760 | 5 | 25 | 1855 | N/A | FDD | N/A |
| CA_n1-n8 | n1 | 1965 | 5 | 25 | 2155 | 6.0 | FDD | IMD4 |
|  | n8 | 887.5 | 5 | 25 | 932.5 | N/A | FDD | N/A |
| CA_n1-n78 | n1 | 1950 | 5 | 25 | 2140 | 8.0 | FDD | IMD4 |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
| CA_n2-n48 | n2 | 1852.5 | 5 | 25 | 1932.5 | 12 | FDD | IMD4 |
|  | n48 | 3625 | 20 | 100 | 3625 | N/A | TDD | N/A |
| CA_n2-n66 | n2 | 1855 | 5 | 25 | 1935 | 20 | FDD | IMD3 |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | FDD | N/A |
|  | n2 | 1883.3 | 5 | 25 | 1963.3 | N/A | FDD | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 4 | FDD | IMD5 |
| CA_n2-n77 | n2 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 | 8.0 | FDD | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
|  | n2 | 1885 | 5 | 25 | 1965 | 5 | FDD | IMD5 |
|  | n77 | 3810 | 10 | 50 | 3810 | N/A | TDD | N/A |
|  | n2 | N/A | 5 | N/A | 1987.5 | 2.7 | FDD | IMD7 |
|  | n7712 | 3455 | 10 | 1 (RBSTART=10) | 3455 | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 (RBSTART=0) | 3945 |  |  |  |
| CA_n3-n5 | n3 | 1771 | 10 | 50 | 1866 | 4 | FDD | IMD4 |
|  | n5 | 838 | 5 | 25 | 883 | N/A | FDD | N/A |
|  | n3 | 1721 | 10 | 50 | 1816 | N/A | FDD | N/A |
|  | n5 | 838 | 5 | 25 | 883 | 24 | FDD | IMD23 |
| CA_n3-n8 | n3 | 1755 | 10 | 50 | 1850 | N/A | FDD | N/A |
|  | n8 | 900 | 5 | 25 | 945 | 8 | FDD | IMD44 |
|  | n3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | FDD | IMD5 |
|  | n8 | 897.5 | 5 | 25 | 942.5 | N/A | FDD | N/A |
| CA_n3-n41 | n3 | 1740 | 5 | 25 | 1835 | 8.2 | FDD | IMD4 |
|  | n41 | 2657.5 | 10 | 50 | 2657.5 | N/A | TDD | N/A |
| CA_n3-n77 | n3 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD24 |
|  | n77 | 3575 | 10 | 50 | 3575 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | IMD44 |
|  | n77 | 3435 | 10 | 50 | 3435 | N/A | TDD | N/A |
| CA_n3-n78 | n3 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD24 |
|  | n78 | 3575 | 10 | 25 | 3575 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | IMD44 |
|  | n78 | 3435 | 10 | 25 | 3435 | N/A | TDD | N/A |
| CA_n5-n66 | n5 | 838 | 5 | 25 | 883 | 30 | FDD | IMD24 |
|  | n66 | 1721 | 5 | 25 | 2121 | N/A | FDD | N/A |
| CA_n5-n77 | n5 | 844 | 5 | 25 | 889 | 8.3 | FDD | IMD413 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |
|  | n5 | 829 | 5 | 25 | 874 | 5.5 | FDD | IMD513 |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | TDD | N/A |
| CA_n5-n78 | n5 | 844 | 5 | 25 | 889 | 8.3 | FDD | IMD4 |
|  | n78 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |
| CA_n8-n77 | n8 | 897.5 | 5 | 25 | 942.5 | 8.3 | FDD | IMD4 |
|  | n77 | 3635 | 10 | 50 | 3635 | N/A | TDD | N/A |
| CA_n8-n78 | n8 | 897.5 | 5 | 25 | 942.5 | 8.3 | FDD | IMD4 |
|  | n78 | 3635 | 10 | 50 | 3635 | N/A | TDD | N/A |
| CA_n14A-n77A | n14 | 793 | 5 | 20 | 763 | 5.5 | FDD | IMD5 |
|  | n77 | 3935 | 10 | 50 | 3935 | N/A | TDD | N/A |
| CA_n20A-n78A | n20 | 850 | 5 | 25 | 809 | 11 | FDD | IMD4 |
|  | n78 | 3359 | 10 | 50 | 3359 | N/A | TDD | N/A |
| CA_n24-n77 | n24 | N/A | N/A | N/A | N/A | N/A | FDD | IMD410 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
| CA_n25-n66 | n25 | 1855 | 5 | 25 | 1935 | 20 | FDD | IMD3 |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | FDD | N/A |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | FDD | N/A |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | 23 | FDD | IMD3 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | FDD | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 4 | FDD | IMD5 |
| CA_n25-n77 | n25 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | 8.0 | FDD | IMD4 |
|  | n77 | 3690 | 10 | 50 | 3690 | N/A | TDD | N/A |
|  | n25 | 1885 | 5 | 25 | 1965 | 5 | FDD | IMD5 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
| CA_n25-n78 | n25 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD24 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
| CA_n26-n66 | n26 | 838 | 5 | 25 | 883 | 30 | FDD | IMD24 |
|  | n66 | 1721 | 5 | 25 | 2121 | N/A | FDD | N/A |
| CA_n26-n70 | n26 | 831 | 5 | 25 | 876 | 30 | FDD | IMD24 |
|  | n70 | 1707.5 | 5 | 25 | 2007.5 | N/A | FDD | N/A |
| CA_n26-n78 | n26 | 836.5 | 5 | 25 | 881.5 | 11.1 | FDD | IMD4 |
|  | n78 | 3391 | 10 | 50 | 3391 | N/A | TDD | N/A |
|  | n26 | 824 | 5 | 12 (RBSTART=0) | 869 | N/A | TDD | N/A |
|  |  | 839 | 5 | 12 (RBSTART=0) | 884 |  |  |  |
|  | n78 | N/A | 10 | N/A | 3336 | 11.1 | FDD | IMD4 |
| CA_n28-n77 | n28 | 705.5 | 5 | 25 | 760.5 | 5.5 | FDD | IMD5 |
|  | n77 | 3582.5 | 10 | 50 | 3582.5 | N/A | TDD | N/A |
| CA_n30-n77 | n30 | 2310 | 5 | 25 | 2355 | 8.0 | FDD | IMD4 |
|  | n77 | 3487.5 | 10 | 50 | 3487.5 | N/A | TDD | N/A |
| CA_n48-n66 | n48 | 3660 | 5 | 25 | 3660 | N/A | TDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 | 5.0 | FDD | IMD5 |
| CA_n48-n70 | n48 | 3695 | 10 | 50 | 3695 | N/A | TDD | N/A |
|  | n70 | 1697.5 | 25/15 | 25 | 1997.5 | 26 | FDD | IMD24 |
| CA_n66-n71 | n66 | 1750 | 5 | 25 | 2150 | 5 | FDD | IMD4 |
|  | n71 | 675 | 5 | 25 | 629 | N/A | FDD | N/A |
| CA_n66-n77 | n66 | 1775 | 5 | 25 | 2175 | 31 | FDD | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | TDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 5.0 | FDD | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
|  | n66 | N/A | 5 | N/A | 2197.5 | 15 | FDD | IMD513 |
|  | n7712 | 3305 | 10 | 1 (RBSTART=0) | 3305 | N/A | TDD | N/A |
|  |  | 3855 | 10 | 1 (RBSTART=8) | 3855 |  |  |  |
|  | n66 | 1730 | 5 | 25 | 2130 | 1.7 | FDD | IMD7 |
|  | n7712 | 3455 | 10 | 1 (RBSTART=10) | 3455 | N/A | TDD | N/A |
|  |  | 3875 | 10 | 1 (RBSTART=0) | 3875 |  |  |  |
| CA_n66-n78 | n66 | 1730 | 5 | 25 | 2130 | 5.0 | FDD | IMD5 |
|  | n78 | 3660 | 10 | 50 | 3660 | N/A | TDD | N/A |
| CA_n70-n71 | n70 | 1697.5 | 5 | 25 | 1997.5 | 5 | FDD | IMD4 |
|  | n71 | 695.5 | 5 | 25 | 649.5 | N/A | FDD | N/A |
| CA_n71-n77 | n71 | 671 | 5 | 25 | 625 | 5.5 | FDD | IMD513 |
|  | n77 | 3309 | 10 | 50 | 3309 | N/A | TDD | N/A |
| CA_n71-n78 | n71 | 681.5 | 5 | 25 | 635.5 | 5.5 | FDD | IMD5 |
|  | n78 | 3361.5 | 10 | 50 | 3361.5 | N/A | TDD | N/A |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,f,c) as defined in subclause 6.2A.4 NOTE 2: RBSTART = 0, 15kHz SCS is assumed. NOTE 3: No requirements apply when there is at least one individual RE within the intermodulation generated by the dual uplink is within the downlink transmission bandwidth of the FDD band. The reference sensitivity should only be verified when this is not the case (the requirements specified in clause 7.3 apply). NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: Void. NOTE 6: TBD NOTE 7: TBD NOTE 8: TBD NOTE 9: TBD NOTE 10: There is no IMD4 product in band n24 downlink for n77 operating in 3450 – 3980 MHz and n24 uplink restricted to between 1627.5 – 1637.5 MHz and between 1646.5 – 1656.5 MHz. NOTE 11: TBD NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE 13: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

7.3A.1	Reference sensitivity power level for 2DL CA without exception

Editor’s Note: Test configuration table and test requirements in clauses 7.3A.1.4.1 and 7.3A.1.5 alignment with TS 38.101-1 (V18.9.0) is FFS.

<<Unchanged sections skipped>>

7.3A.1.5	Test requirement

For 2DL carrier aggregation, test parameters are specified in table 7.3A.1.4.1-1, 7.3A.1.4.1-2 and 7.3A.1.4.1-3. For the CA configurations listed in table 7.3A.1.5-1, the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with reference power level specified in table 7.3.2.5-1a and table 7.3.2.5-1b for non-SDL carrier for 2 Rx antenna port, in table 7.3.2.5-2a and 7.3.2.5-2b for non-SDL carrier for 4 Rx antenna port, in table 7.3.2.5-2e and table 7.3.2.5-2f for non-SDL carrier for 8 Rx antenna port and in table 7.3A.1.5-2 for SDL carrier with following additional requirements:

The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

For the UE which supports inter-band carrier aggregation, the test requirement for reference sensitivity shall be increased by the amount given by ΔRIB,c defined in clause 7.3A.0.3 for the applicable operating bands. Unless otherwise stated, ΔRIB,c is set to zero.

For intra-band non-contiguous 2 DL CA, the test requirement for shall be increased by ΔRIBNC given in Table 7.3A.0.2.2-1 for the SCC. Unless given by Table 7.3.2.3-4, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.3.1-1) configured.

Table 7.3A.1.5-1: Reference sensitivity requirement for 2DL CA

| Carrier aggregation type | DL CA configuration | UL CA configuration |
| --- | --- | --- |
|  | CA_n5B | - |
|  | CA_n41C | - |
|  | CA_n48B | - |
|  | CA_n48C |  |
| Intra-band contiguous 2DL CA | CA_n66B | - |
|  | CA_n77C | - |
|  | CA_n78B | - |
|  | CA_n78C | - |
|  | CA_n79C |  |
|  | CA_n48(2A) | - |
|  | CA_n2(2A) | - |
|  | CA_n66(2A) | - |
| Intra-band non-contiguous 2DL CA | CA_n77(2A) | - |
|  | CA_n78(2A) | - |
|  | CA_n71(2A) | - |
|  | CA_n1A-n3A |  |
|  | CA_n1A-n5A |  |
|  | CA_n1A-n8A | - |
|  | CA_n1A-n26A | - |
|  | CA_n1A-n28A | - |
|  | CA_n1A-n41A |  |
|  | CA_n1A-n77A | - |
|  | CA_n1A-n78A | - |
|  | CA_n1A-n79A |  |
|  | CA_n2A-n5A |  |
|  | CA_n2A-n12A | - |
|  | CA_n2A-n14A |  |
|  | CA_n2A-n30A | - |
|  | CA_n2A-n48A | - |
|  | CA_n2A-n66A | - |
|  | CA_n2A-n77A | - |
|  | CA_n3A-n5A |  |
|  | CA_n3A-n8A | - |
|  | CA_n3A-n28A | - |
|  | CA_n3A-n41A |  |
|  | CA-n3A-n77A | - |
| Inter-band 2DL CA | CA_n3A-n78A | - |
|  | CA_n5A-n30A | - |
|  | CA_n5A-n48A |  |
|  | CA_n5A-n66A | - |
|  | CA_n5A-n77A | - |
|  | CA_n5A-n78A |  |
|  | CA_n8A-n77A | - |
|  | CA_n8A-n78A | - |
|  | CA_n14A-n30A |  |
|  | CA_n14A-n66A |  |
|  | CA_n14A-n77A |  |
|  | CA_n20A-n78A | - |
|  | CA_n24A-n41A |  |
|  | CA_n24A-n48A |  |
|  | CA_n24A-n77A |  |
|  | CA_n25A-n77A | - |
|  | CA_n25A-n78A | - |
|  | CA_n26A-n78A | - |
|  | CA_n28A-n40A | - |
|  | CA_n28A-n41A | - |
|  | CA_n28A-n77A | - |
|  | CA_n28A-n78A | - |
|  | CA_n28A-n79A | - |
|  | CA_n30A-n66A | - |
|  | CA_n30A-n77A | - |
|  | CA_n39A-n41A |  |
|  | CA_n40A-n77A |  |
|  | CA_n41A-n66A (Note 1) | - |
|  | CA_n41A-n71A | - |
|  | CA_n41A-n77A |  |
|  | CA_n41A-n79A | - |
|  | CA_n48A-n77A | - |
|  | CA_n66A-n70A | - |
|  | CA_n66A-n71A | - |
|  | CA_n66A-n77A |  |
|  | CA_n66A-n78A | - |
|  | CA_n70A-n71A | - |
|  | CA_n71A-n77A | - |
|  | CA_n71A-n78A | - |
| SDL configuration | CA_n29A-n66A | - |
|  | CA_n29A-n70A | - |
|  | CA_n29A-n71A (NOTE 1) | - |
| Note 1: Configuration has sensitivity exceptions according to notes in table 7.3A.1.4.1-2. Other Rx cases shall be tested with reference sensitivity levels defined in Clause 7.3A.1_1 Note 2: CA_n41A-66A testpoint 1, reference sensitivity requirement is tested in Clause 7.3A.1_1 due to cross band isolation exception specified in Table 7.3A.0.6-3. Other Rx cases using configuration for testpoint 1 shall be tested with reference sensitivity levels defined in Clause 7.3A.1_1.5. |  |  |

Table 7.3A.1.5-2: Reference sensitivity for SDL bands

| NR band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n29 | 15 | -97.0 +TT | -93.8 +TT |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  | -94.1 +TT |  |  |  |  |  |  |  |  |  |  |
| Note 1: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

For intra-band non-contiguous carrier aggregation with one uplink carrier and two or more downlink sub-blocks, throughput of each downlink component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) and parameters specified in Table 7.3.2.4.1-1, Table 7.3.2.4.1-2, Table 7.3.2.4.1-3, Table 7.3.2.5-1a, Table 7.3.2.5-1b, Table 7.3.2.5-2a, Table 7.3.2.5-2b, Table 7.3.2.5-2c, Table 7.3.2.5-2d, Table 7.3.2.5-2e and Table 7.3.2.5-2f and Table 7.3A.1.4-1   with the reference sensitivity power level increased by ΔRIBNC  given in Table 7.3A.0.2.2-1 for the SCC(s). For aggregation of two downlink FDD carriers with one uplink carrier the reference sensitivity is defined only for the specific uplink and downlink test points which are specified in Table 7.3A.0.2.2-1. The requirements apply with all downlink carriers active. Unless given by Table 7.3.2.3-4, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.3.1-1) configured.

For band combinations including operating bands without uplink band (as noted in Table 5.2-1), the requirements are specified in Table 7.3A.1.5-1 and for any band with uplink the uplink configuration specified in Table 7.3.2.4.1-2. The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels, as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one-sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal, as described in Annex A.5.1.1/A.5.2.1). The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active.

7.3A.1_1	Reference sensitivity power level for 2DL CA exceptions

Editor’s Note: The following aspects are either missing or not yet determined:

- Test point analysis for CA_n3A-n5A harmonic mixing is currently missing in TR 38.905.

Editor’s Note: Test configuration table and test requirements in clauses 7.3A.1_1.4.1 and 7.3A.1_1.5 alignment with TS 38.101-1 (V18.9.0) is FFS.

7.3A.1_1.1	Test purpose

To verify the ability of UE that support CA to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise when CA exceptions are allowed.

A UE unable to meet the throughput requirement under these conditions will decrease the effective coverage area.

7.3A.1_1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support NR 2DL CA

7.3A.1_1.3	Minimum requirements

The minimum conformance requirements are defined in clause 7.3A.0.

7.3A.1_1.4	Test description

7.3A.1_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.2-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 7.3A.1_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.3A.1_1.4.1-1: Test Configuration Table for inter-band 2DL CA exceptions

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |
| Test CC Combination setting (CBW) as specified in subclause Table 5.5A.3.1-1 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  |  |  |  | Refer to “PCC”and “SCC” columns |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / CBW |  |  |  |  |  | DL Allocation |  |  | UL Allocation (Note 2) |  |  |
|  | CA Configuration |  |  |  | PCC | SCC | CC MOD | PCC & SCC RB allocation |  | CC MOD | PCC & SCC RB allocations (LCRB @ RBstart) |  |
|  | PCC |  | SCC |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range | Band | Range |  |  |  |  |  |  |  |  |
| <<Unchanged rows skipped>> |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for CA_n7A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n78 | 3350 MHz | n7 | 2687.5 MHz | 100 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| Test Settings for CA_n8A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n8 | Low | n77 | 3530 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n8 | Low | n77 | 3530 MHz | 5 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n77 | 3770 MHz | n8 | Mid | 10 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 | - |
| 4 | n77 | 3770 MHz | n8 | Mid | 10 MHz | 20 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 | - |
| 5 | n8 | UL 897.5 MHz / DL 942.5 MHz | n77 | 3635 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n8A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n8 | Mid | n78 | 3590 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n8 | Mid | n78 | 3590 MHz | 5 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n8 | 897.5 MHz (UL) | n78 | 3635 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| <<Unchanged rows skipped>> |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for CA_n26A-n70A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n70 | 1707.5 MHz (UL) / 2007.5 MHz (DL) | n26 | 831 MHz (UL) / 876 MHz (DL) | 5 MHz UL / 5 MHz DL | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n26A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n26 | Mid | n78 | 3326 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n26 | 837.5 MHz (UL) | n78 | 3350 MHz | 5 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n78 | 3506 MHz | n26 | Mid | 10 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 | - |
| 4 | n78 | 3391MHz | n26 | 836.5 MHz (UL) / 881.5 MHz (DL) | 10 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n28A-n40A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n40 | 2341.5 MHz | n28 | 780.5 MHz (DL) | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 | - |
| 2 | n40 | 2341.5 MHz | n28 | 780.5 MHz (DL) | 20 MHz | 20 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 | - |
| <<Unchanged rows skipped>> |  |  |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

7.3A.1_1.5	Test requirement

For inter-band carrier aggregation the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.2.2 with parameters specified in Table 7.3A.1_1.5-1 for PC3 2UL CA configuration and CA configuration with single uplink carrier indicating power class 3 in TS 38.508-2 table A.4.3.2A.4.1-3, and in Table 7.3A.1_1.5-1 for CA configuration with single uplink carrier indicating PC2 in TS 38.508-2 table A.4.3.2A.4.1-3 but without note 4 applied in table 5.5A.3.1-1.

For inter-band carrier aggregation, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.2.2 with parameters specified in Table 7.3A.1_1.5-1a for PC2 2UL CA configuration and CA configuration with single uplink carrier with note 4 applied in table 5.5A.3.1-1 and UE indicating power class 2 in TS 38.508-2 Table A.4.3.2A.4.1-3, and Table 7.3A.1_1.5-1b for PC1.5 2UL CA configuration and CA configuration with single uplink carrier indicating power class 1.5 in TS 38.508-2 table A.4.3.2A.4.1-3.

The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

Table 7.3A.1_1.5-1: Reference sensitivity requirement for inter-band CA with PC3 single UL carrier or PC3 2UL CA

| CA configuration | Test ID | NR band | CBW (MHz) | REFSENS test requirement of NR band (dBm) (Note 9, Note 10, Note 11) |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A | 1 | n1 | 5 | REF_victim +23 |
|  |  | n3 | 5 | REF_aggressor |
|  | 2 | n1 | 5 | REF_aggressor |
|  |  | n3 | 5 | REF_victim +3 |
|  | 3 | n1 | 5 | REF_aggressor |
|  |  | n3 | 5 | REF_victim +19.7 |
| CA_n1A-n8A | 1 | n1 | 5 | REF_victim +6 |
|  |  | n8 | 5 | REF_aggressor |
| CA_n1A-n28A | 1 | n1 | 5 | REF_victim +10.2 |
|  |  | n28 | 5 | REF_aggressor |
|  | 2 | n1 | 50 | REF_victim +1.1 |
|  |  | n28 | 5 | REF_aggressor |
| CA_n1A-n41A | 1 | n1 | 50 | REF_aggressor |
|  |  | n41 | 10 | REF_victim +6.1 |
|  | 2 | n1 | 20 | REF_aggressor |
|  |  | n41 | 100 | REF_victim +0.7 |
|  | 3 | n1 | 5 | REF_victim +18.1 |
|  |  | n41 | 100 | REF_aggressor |
| CA_n1A-n77A | 1 | n1 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +27.1 |
|  | 2 | n1 | 20 | REF_aggressor |
|  |  | n77 | 100 | REF_victim +13.8 |
|  | 3 | n1 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +1.9 |
| CA_n1A-n78A | 1 | n1 | 5 | REF_victim +8 |
|  |  | n78 | 10 | REF_aggressor |
| CA_n2A-n48A | 1 | n2 | 5 | REF_aggressor |
|  |  | n48 | 10 | REF_victim +1.9 |
|  | 2 | n2 | 5 | REF_victim +12 |
|  |  | n48 | 20 | REF_aggressor |
| CA_n2A-n66A | 1 | n2 | 5 | REF_victim +20 |
|  |  | n66 | 5 | REF_aggressor |
|  | 2 | n2 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_victim +4 |
| CA_n2A-n77A | 1 | n2 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +23.9 |
|  | 2 | n2 | 10 | REF_aggressor |
|  |  | n77 | 100 | REF_victim +13.8 |
|  | 3 | n2 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +1.1 |
|  | 4 | n2 | 5 | REF_victim +6.7 |
|  |  | n77 | 10 | REF_aggressor |
|  | 5 | n2 | 20 | REF_victim +2.8 |
|  |  | n77 | 10 | REF_aggressor |
|  | 6 | n2 | 5 | REF_victim +26 |
|  |  | n77 | 10 | REF_aggressor |
|  | 7 | n2 | 5 | REF_victim +8 |
|  |  | n77 | 10 | REF_aggressor |
|  | 8 | n2 | 5 | REF_victim +5 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n3A-n5A | 1 | n3 | 10 | REF_victim +4 |
|  |  | n5 | 5 | REF_aggressor |
|  | 2 | n3 | 10 | REF_aggressor |
|  |  | n5 | 5 | REF_victim +24 |
|  | 3 | n3 | 5 | REF_aggressor |
|  |  | n5 | 5 | REF_victim +4 |
| CA_n3A-n8A | 1 | n3 | 10 | REF_aggressor |
|  |  | n8 | 5 | REF_victim +8 |
|  | 2 | n3 | 10 | REF_victim +6.4 |
|  |  | n8 | 5 | REF_aggressor |
| CA_n3A-n41A | 1 | n3 | 5 | REF_victim +8.2 |
|  |  | n41 | 10 | REF_aggressor |
|  | 2 | n3 | 5 | REF_victim +0.6 |
|  |  | n41 | 100 | REF_aggressor |
| CA_n3A-n77A | 1 | n3 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +23.9 |
|  | 2 | n3 | 10 | REF_aggressor |
|  |  | n77 | 100 | REF_victim +13.8 |
|  | 3 | n3 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +1.1 |
|  | 4 | n3 | 5 | REF_victim +26.0 |
|  |  | n77 | 10 | REF_aggressor |
|  | 5 | n3 | 5 | REF_victim +8.0 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n3A-n78A | 1 | n3 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +23.9 |
|  | 2 | n3 | 10 | REF_aggressor |
|  |  | n78 | 100 | REF_victim +13.8 |
|  | 3 | n3 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +1.1 |
|  | 4 | n3 | 5 | REF_victim +26 |
|  |  | n78 | 10 | REF_aggressor |
|  | 5 | n3 | 5 | REF_victim +8 |
|  |  | n78 | 10 | REF_aggressor |
| CA_n5A-n66A | 1 | n5 | 5 | REF_victim +30 |
|  |  | n66 | 5 | REF_aggressor |
| CA_n5A-n77A | 1 | n5 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +10.5 |
|  | 2 | n5 | 5 | REF_aggressor |
|  |  | n77 | 100 | REF_victim +1.4 |
|  | 3 | n5 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +10.4 |
|  | 4 | n5 | 5 | REF_aggressor |
|  |  | n77 | 100 | REF_victim +0.7 |
|  | 5 | n5 | 20 | REF_victim +0.8 |
|  |  | n77 | 10 | REF_aggressor |
|  | 6 | n5 | 5 | REF_victim +5.7 |
|  |  | n77 | 10 | REF_aggressor |
|  | 7 | n5 | 5 | REF_victim +8.3 |
|  |  | n77 | 10 | REF_aggressor |
|  | 8 | n5 | 5 | REF_victim +5.5 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n5A-n78A | 1 | n5 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +10.5 |
|  | 2 | n5 | 5 | REF_aggressor |
|  |  | n78 | 100 | REF_victim +1.4 |
|  | 3 | n5 | 5 | REF_victim +8.3 |
|  |  | n78 | 10 | REF_aggressor |
| CA_n7A-n78A | 1 | n7 | 5 | REF_victim +4.5 |
|  |  | n78 | 100 | REF_aggressor |
| CA_n8A-n77A | 1 | n8 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +10.8 |
|  | 2 | n8 | 5 | REF_aggressor |
|  |  | n77 | 100 | REF_victim +3.1 |
|  | 3 | n8 | 5 | REF_victim +5.7 |
|  |  | n77 | 10 | REF_aggressor |
|  | 4 | n8 | 20 | REF_victim +0.8 |
|  |  | n77 | 10 | REF_aggressor |
|  | 5 | n8 | 5 | REF_victim +8.3 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n8A-n78A | 1 | n8 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +10.8 |
|  | 2 | n8 | 5 | REF_aggressor |
|  |  | n78 | 100 | REF_victim +1.4 |
|  | 3 | n8 | 5 | REF_victim +8.3 |
|  |  | n78 | 10 | REF_aggressor |
| CA_n14A-n77A | 1 | n14 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +10.4 |
|  | 2 | n14 | 5 | REF_aggressor |
|  |  | n77 | 100 | REF_victim +0.7 |
|  | 3 | n14 | 5 | REF_victim + 5.5 |
|  |  | n77 | 10 | REF_aggressor |
|  | 4 | n14 | 5 | REF_victim + 31 |
|  |  | n77 | 10 | REF_aggressor |
|  | 5 | n14 | 10 | REF_victim + 28 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n20A-n78A | 1 | n20 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +10.8 |
|  | 2 | n20 | 5 | REF_aggressor |
|  |  | n78 | 100 | REF_victim +1.4 |
|  | 3 | n20 | 5 | REF_victim + 11 |
|  |  | n78 | 10 | REF_aggressor |
| CA_n25A-n66A | 1 | n66 | 5 | REF_aggressor |
|  |  | n25 | 5 | REF_victim +20 |
|  | 2 | n66 | 5 | REF_victim +23 |
|  |  | n25 | 5 | REF_aggressor |
|  | 3 | n66 | 5 | REF_victim +4 |
|  |  | n25 | 5 | REF_aggressor |
| CA_n25A-n77A | 1 | n25 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +23.9 |
|  | 2 | n25 | 10 | REF_aggressor |
|  |  | n77 | 100 | REF_victim +13.8 |
|  | 3 | n25 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +1.1 |
|  | 4 | n25 | 5 | REF_victim +6.7 |
|  |  | n77 | 10 | REF_aggressor |
|  | 5 | n25 | 40 | REF_victim +1.1 |
|  |  | n77 | 20 | REF_aggressor |
|  | 6 | n25 | 5 | REF_victim +26 |
|  |  | n77 | 10 | REF_aggressor |
|  | 7 | n25 | 5 | REF_victim +8 |
|  |  | n77 | 10 | REF_aggressor |
|  | 8 | n25 | 5 | REF_victim +5 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n25A-n78A | 1 | n25 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +23.9 |
|  | 2 | n25 | 10 | REF_aggressor |
|  |  | n78 | 100 | REF_victim +13.8 |
|  | 3 | n25 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +1.1 |
|  | 4 | n25 | 5 | REF_victim +26 |
|  |  | n78 | 10 | REF_aggressor |
| CA_n26A-n66A | 1 | n26 | 5 | REF_victim + 30 |
|  |  | n66 | 5 | REF_aggressor |
| CA_n26A-n70A | 1 | n26 | 5 | REF_victim + 30 |
|  |  | n70 | 5 | REF_aggressor |
| CA_n26A-n78A | 1 | n26 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim + 10.8 |
|  | 2 | n26 | 5 | REF_aggressor |
|  |  | n78 | 100 | REF_victim + 3 |
|  | 3 | n26 | 5 | REF_victim + 5.4 |
|  |  | n78 | 10 | REF_aggressor |
|  | 4 | n26 | 5 | REF_victim + 11.1 |
|  |  | n78 | 10 | REF_aggressor |
| CA_n28A-n40A | 1 | n28 | 5 | REF_victim +37.8 |
|  |  | n40 | 5 | REF_aggressor |
|  | 2 | n28 | 20 | REF_victim +30.3 |
|  |  | n40 | 20 | REF_aggressor |
| CA_n28A-n77A | 1 | n28 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +10.4 |
|  | 2 | n28 | 5 | REF_aggressor |
|  |  | n77 | 100 | REF_victim +0.7 |
|  | 3 | n28 | 5 | REF_victim +0.7 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n28A-n78A | 1 | n28 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +10.4 |
|  | 2 | n28 | 5 | REF_aggressor |
|  |  | n78 | 100 | REF_victim +0.7 |
|  | 3 | n28 | 5 | REF_victim + 31 |
|  |  | n78 | 10 | REF_aggressor |
| CA_n29A-n71A | 1 | n29 | 5 | REF_victim + 17.5 |
|  |  | n71 | 20 | REF_aggressor |
| CA_n30A-n66A | 1 | n30 | 10 | REF_aggressor |
|  |  | n66 | 15 | REF_victim + 8.3 |
| CA_n30A-n77A | 1 | n30 | 5 | REF_victim + 10.4 |
|  |  | n77 | 10 | REF_aggressor |
|  | 2 | n30 | 10 | REF_victim + 8 |
|  |  | n77 | 10 | REF_aggressor |
|  | 3 | n30 | 5 | REF_victim + 8 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n39A-n41A | 1 | n39 | 5 | REF_aggressor |
|  |  | n41 | 5 | REF_victim +8.1 |
|  | 2 | n39 | 5 | REF_aggressor |
|  |  | n41 | 100 | REF_victim +3.3 |
|  | 3 | n39 | 5 | REF_victim +4.3 |
|  |  | n41 | 10 | REF_aggressor |
|  | 4 | n39 | 40 | REF_victim + 0.8 |
|  |  | n41 | 10 | REF_aggressor |
|  | 5 | n39 | 40 | REF_aggressor |
|  |  | n41 | 10 | REF_victim +3.3 |
|  | 6 | n39 | 5 | REF_victim +1.6 |
|  |  | n41 | 100 | REF_aggressor |
| CA_n40A-n77A | 1 | n40 | 10 | REF_victim +10.4 |
|  |  | n77 | 20 | REF_aggressor |
|  | 2 | n40 | 100 | REF_victim +0.9 |
|  |  | n77 | 20 | REF_aggressor |
|  | 3 | n40 | 10 | REF_victim +4.5 |
|  |  | n77 | 100 | REF_aggressor |
|  | 4 | n40 | 100 | REF_victim +4.5 |
|  |  | n77 | 100 | REF_aggressor |
| CA_n41A-n66A | 1 | n41 | 100 | REF_aggressor |
|  |  | n66 | 5 | REF_victim +10.5 |
| CA_n41A-n71A | 1 | n41 | 10 | REF_victim + 10.8 |
|  |  | n71 | 5 | REF_aggressor |
|  | 2 | n41 | 100 | REF_victim + 1.4 |
|  |  | n71 | 5 | REF_aggressor |
| CA_n41A-n77A | 1 | n41 | 10 | REF_victim +10.4 |
|  |  | n77 | 20 | REF_aggressor |
|  | 2 | n41 | 100 | REF_victim +6.3 |
|  |  | n77 | 20 | REF_aggressor |
|  | 3 | n41 | 100 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +8.3 |
|  | 4 | n41 | 10 | REF_victim +4.5 |
|  |  | n77 | 100 | REF_aggressor |
|  | 5 | n41 | 100 | REF_victim +4.5 |
|  |  | n77 | 100 | REF_aggressor |
| CA_n48A-n66A | 1 | n48 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_victim + 5.0 |
|  | 2 | n48 | 5 | REF_victim +27.1 |
|  |  | n66 | 5 | REF_aggressor |
|  | 3 | n48 | 100 | REF_victim + 13.8 |
|  |  | n66 | 40 | REF_aggressor |
|  | 4 | n48 | 5 | REF_victim + 1.9 |
|  |  | n66 | 5 | REF_aggressor |
| CA_n48A-n70A | 1 | n48 | 10 | REF_aggressor |
|  |  | n70 | 15 MHz UL /25 MHz DL | REF_victim +26 |
| CA_n66A-n71A | 1 | n66 | 5 | REF_victim +5 |
|  |  | n71 | 5 | REF_aggressor |
| CA_n66A-n77A | 1 | n66 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +23.9 |
|  | 2 | n66 | 20 | REF_aggressor |
|  |  | n77 | 100 | REF_victim +13.8 |
|  | 3 | n66 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +1.1 |
|  | 4 | n66 | 5 | REF_victim +31 |
|  |  | n77 | 10 | REF_aggressor |
|  | 5 | n66 | 5 | REF_victim +5 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n66A-n78A | 1 | n66 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +23.9 |
|  | 2 | n66 | 20 | REF_aggressor |
|  |  | n78 | 100 | REF_victim +13.8 |
|  | 3 | n66 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +1.1 |
| CA_n70A-n71A | 1 | n70 | 25 | REF_victim +4.1 |
|  |  | n71 | 5 | REF_aggressor |
|  | 2 | n70 | 5 | REF_victim +9.9 |
|  |  | n71 | 5 | REF_aggressor |
|  | 3 | n70 | 5 | REF_victim +5 |
|  |  | n71 | 5 | REF_aggressor |
| CA_n71A-n77A | 1 | n71 | 5 | REF_victim +5.5 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n71A-n78A | 1 | n71 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +10.4 |
|  | 2 | n71 | 5 | REF_victim +5.5 |
|  |  | n78 | 10 | REF_aggressor |
| CA_n78A-n79A | 1 | n78 | 100 | REF_aggressor |
|  |  | n79 | 40 | REF_victim +2 |
|  | 2 | n78 | 10 | REF_victim +2.6 |
|  |  | n79 | 100 | REF_aggressor |
|  | 3 | n78 | 100 | REF_victim +2.6 |
|  |  | n79 | 100 | REF_aggressor |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.3.5-2) Note 2: The reference measurement channel is specified in Annex A.2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2. Note 3: Void Note 4: Void Note 5: Void Note 6: Void Note 7: Void Note 8: Void Note 9: The symbol “REF_victim” in this table can be used for cases of 2 antenna ports, 4 antenna ports, or 8 antenna ports. The values equal to reference sensitivity values for 2 antenna ports. Refer to Table 7.3.2.5-1a and Table 7.3.2.5-1b for reference sensitivity values for 2 antenna ports. Note 10: The symbol "REF_aggressor" in this table can be used for cases of 2 antenna ports, 4 antenna ports, or 8 antenna ports. The values equal to the corresponding reference sensitivity values. Refer to Table 7.3.2.5-1a and Table 7.3.2.5-1b for reference sensitivity values for 2 antenna ports, Table 7.3.2.5-2a and Table 7.3.2.5-2b for reference sensitivity values for 4 antenna ports, and Table 7.3.2.5-2e and Table 7.3.2.5-2f for reference sensitivity values for 8 antenna ports. Note 11: The test requirement for 2Rx victim band is “REF_victim + MSD”, for 4Rx victim band is “REF_victim + ΔRIB,4R + MSD +¦ΔRIB,4R¦ = REF_victim + MSD”, and for 8Rx victim band is “REF_victim + ΔRIB,8R + MSD +¦ΔRIB,8R ¦ = REF_victim+ MSD”, where the “MSD” refers to the MSD requirements for 2Rx antenna ports. |  |  |  |  |

<<End of change>>