Title:	Discussion on test point selection for additional spurious emission with NS_UAV_46 in FR1

Document for:	Discussion and approval

# Background

When "NS_UAV_46" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 [from 2] from the edge of the channel bandwidth.

Table 1: Additional requirements for "NS_UAV_46"

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10, 15, 20, 25, 30, 35, 40, 50 MHz |  |
| 2690 ≤ f ≤ 2900 | -50 | 1 MHz |

In general, for certain NS values both A-MPR requirements and emissions minimum requirements such as SEM and/or Spurious are defined. The test point analysis of emission requirements would follow that of A-MPR. But for NS_UAV_46 only additional spurious emission requirements apply and no A-MPR is allowed, therefore the test point analysis of general spurious emissions can be reused as much as possible, which reflects the most critical test point configuration for spurious emission measurement.

## Test Environment

The test environment for NR general spurious emission measurement for FR1 can be reused.

PROPOSAL 1: Select Test Environment as Normal Conditions (NC) for additional spurious emission for NS_UAV_46 in FR1.

## Test Frequencies

The additional spurious emission requirements are defined for higher part of band n7, therefore only High range needs to be tested.

PROPOSAL 2: Select 2567.5 MHz for 5MHz channel bandwidth as the test frequencies for NS_UAV_46 in FR1.

## Test Channel Bandwidth

Minimum requirements for NS_UAV_46 apply for channel bandwidths 5MHz, which shall be selected to test additional spurious emission.

PROPOSAL 3: 5MHz need to be selected to test additional spurious emission for NS_UAV_46 in FR1.

## Test Subcarrier Spacing

The selection of test subcarrier spacing can be based on the agreed proposals for general spurious emission, i.e. lowest supported subcarrier spacing.

PROPOSAL 4: Select Lowest supported subcarrier spacing for NS_UAV_46 in FR1.

## UL Modulation

The selection of UL modulation can be based on the agreed proposals for general spurious emission, i.e. CP-OFDM QPSK.

PROPOSAL 5: Test CP-OFDM QPSK for Additional spurious emission for NS_UAV_46 in FR1.

## Uplink RB allocation

Since there is no additional MPR for NS_UAV_46, the selection of UL RB allocation can be based on the agreed proposals for general spurious emission.

For NS_UAV_46, the Edge_1RB_Right and Outer_Full can be considered as a worst-case scenario. No need to test with Edge_1RB_Left as it is not close to the higher part of the operating band.

PROPOSAL 6: Edge_1RB_Right and Outer_Full are tested for additional spurious emission for NS_UAV_46 in FR1.

2.7.	Number of Test Points

The maximum number of test points for Additional spurious emission tests for NS_UAV_46:

| Environment conditions | Maximum Number of Steps (Freq. X Ch BW X RB alloc. X SCS) | Maximum Number of Modulations | Maximum Number of Test Steps |
| --- | --- | --- | --- |
| 1 (NC) | 1 X 1 X 2 X 1 | 1 | 2 |

# Proposal

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR1 NS_UAV_46 A-SE test case definition:

PROPOSAL 1: Select Test Environment as Normal Conditions (NC) for additional spurious emission for NS_UAV_46 in FR1.

PROPOSAL 2: Select 2567.5 MHz for 5MHz channel bandwidth as the test frequencies for NS_UAV_46 in FR1.

PROPOSAL 3: 5MHz need to be selected to test additional spurious emission for NS_UAV_46 in FR1.

PROPOSAL 4: Select Lowest supported subcarrier spacing for NS_UAV_46 in FR1.

PROPOSAL 5: Test CP-OFDM QPSK for additional spurious emission for NS_UAV_46 in FR1.

PROPOSAL 6: Edge_1RB_Right and Outer_Full are tested for additional spurious emission for NS_UAV_46 in FR1.

# References

- R5-182019, “Discussion of NR FR1 Test Point for TX Spurious Emission test cases”, (Qualcomm Inc.)

- 3GPP TS 38.101-1, V18.10.0 (2025-06)