3GPP TSG-RAN5 Meeting #108 	R5-255242

# Bengaluru, India, 25th Aug. 2025 - 29th Aug. 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3598 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of Annex C for valid CORESET 0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current calculation process of test frequencies in Annex C, CORESET#0 may be outside the carrier range. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In order to ensure that the location of CORESET#0 is inside the carrier range, the calculation step 1f of C.3.2 was updated, and the related parameter FcarrierHigh was defined in Table C.1-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cell connection will fail with an invalid CORESET#0. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C.1, C.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

C.1	Definitions and Parameters

Figure C.1-1 shows SSB and CORESET#0 and related parameters. CORESET#0 is required for a carrier to be used as a PCell.

Figure C.1-1: location of SSB and CORESET#0 within a channel

The parameters referenced in figure C.1-1 are defined in Table C.1-1.

Table C.1-1: Definition of parameters in Figure C.1-1 used in Annex C

| Parameter | Description |
| --- | --- |
| FPointA | Reference Point A frequency. |
| Fcarrier | Fcarrier is the centre frequency of a carrier corresponding to its NR-ARFCN value. |
| FcarrierLow | FcarrierLow is the centre frequency of lowest subcarrier of the carrier. FcarrierLow = Fcarrier - 12 * SCSCarrier * (NRB / 2) with NRB according to Table 5.3.2-1 of TS 38.101-1 [7] and TS 38.101-2 [8] for the channel bandwidth of the carrier. |
| FcarrierHigh | FcarrieHigh is the centre frequency of highest subcarrier of the carrier. FcarrieHigh = Fcarrier + 12 * SCSCarrier * (NRB / 2) with NRB according to Table 5.3.2-1 of TS 38.101-1 [7] and TS 38.101-2 [8] for the channel bandwidth of the carrier. |
| ΔFcarrierBandwidth | ΔFcarrierBandwidth is the carrier’s channel bandwidth as provided in carrierBandwidth to the UE (SCS-SpecificCarrier). |
| ΔFoffsetToCarrier | ΔFoffsetToCarrier is the frequency offset between Point A and the centre frequency of lowest subcarrier of the carrier. ΔFoffsetToCarrier = offsetToCarrier * PRBsize, where PRBsize according to the subcarrier spacing of the carrier. offsetToCarrier is signalled to the UE (SCS-SpecificCarrier). |
| FSSref | Centre frequency of SSB. For a cell selectable as PCell the FSSref corresponds to a valid GSCN value according to clause 5.4.3.1 of TS 38.101-1 [7] and TS 38.101-2 [8]. |
| ΔFOffset(RBs) | ΔFOffset(RBs) = 12 * Offset(RBs) * subCarrierSpacingCommon, where Offset(RBs) is given in tables 13-1 to 13-10 of TS 38.213 [22]. |
| ΔFkSSB | ΔFKSSB = kSSB * {15 kHz for FR1, subCarrierSpacingCommon (MIB) for FR2} (TS 38.211 [3], clause 7.4.3.1) |
| ΔFOffsetSSB-CORESET0 | Frequency offset between the lowest subcarrier of the SSB and the lowest subcarrier of CORESET#0. ΔFOffsetSSB-CORESET0 = ΔFOffset(RBs) + ΔFkSSB. |
| ΔFOffsetCORESET-0-Carrier | Frequency offset, FOffsetCORESET#0-Carrier, between the lowest subcarrier of CORESET#0 and the lowest subcarrier of the carrier. |
| ΔFOffsetSSB-Carrier | Frequency offset between the lowest subcarrier of the SSB and the lowest subcarrier of the carrier. |
| FCORESET0Low | Centre frequency of subcarrier 0 of CORESET#0. |
| FOffsetToPointA | Frequency of the lowest subcarrier of the lowest resource block, which has the subcarrier spacing provided by the higher-layer parameter subCarrierSpacingCommon and overlaps with the SS/PBCH block used by the UE for initial cell selection, expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2 (TS 38.211 [29] clause 4.4.4.2). |
| ΔFOffsetToPointA | Frequency offset between FOffsetToPointA and point A. ΔFOffsetToPointA = offsetToPointA * {15 kHz for FR1; 60 kHz for FR2} (TS 38.211 [29] clause 4.4.4.2). |

Additional parameters used in this annex are defined in Table C.1-2.

Table C.1-2: Definition of additional parameters used in Annex C.

| kSSB | As defined in TS 38.211 [29] clause 7.4.3.1 |
| --- | --- |
| SCSCarrier | Subcarrier spacing for the carrier (SCS-SpecificCarrier): FR1: 15kHz, 30kHz or 60kHz according to TS 38.101-1 [7] Table 5.3.5-1 FR2: 60kHz or 120kHz according to TS 38.101-2 [8] Table 5.3.5-1 |
| SCSSSB | SS/PBCH block subcarrier spacing FR1: 15kHz or 30kHz according to TS 38.101-1 [7] Table 5.4.3.3-1 FR2: 120kHz or 240kHz according to TS 38.101-2 [8] Table 5.4.3.3-1 NOTE: According to the tables in clause 13 of TS 38.213 [22] not all combinations of SCSSSB and SCSCarrier are applicable. |
| SCScommon | Subcarrier spacing for SIB1, Msg.2/4 for initial access, paging and broadcast SI-messages. Provided to the UE in the MIB in IE subCarrierSpacingCommon. |
| PRBsize | Physical Resource Block size of the carrier = 12 * SCSCarrier. |
| CRBsize | Common Resource Block size = 12 * SCScommon. |
| FDL_Low, FUL_Low | Lowest frequency of the downlink and uplink frequency range of the band as defined in clause 5.2 of TS 38.101-1 [7] and TS 38.101-2 [8]. |
| FDL_High, FUL_High | Highest frequency of the downlink and uplink frequency range of the band as defined in clause 5.2 of TS 38.101-1 [7] and TS 38.101-2 [8]. |
| ΔFRaster | Frequency raster of the band as defined in clause 5.4.2.3 of TS 38.101-1 [7] and TS 38.101-2 [8]. |
| BWDL | Bandwidth of downlink frequency range of the band. |
| BWUL | Bandwidth of uplink frequency range of the band. |
| CBWDL | Downlink channel bandwidth (MHz) of the carrier according to Table 5.3.2-1 of TS 38.101-1 [7] and TS 38.101-2 [8]. |
| CBWUL | Uplink channel bandwidth (MHz) of the carrier according to Table 5.3.2-1 of TS 38.101-1 [7] and TS 38.101-2 [8]. |
| FTx-Rx_separation | Default Tx – Rx carrier centre frequency separation of the band as defined in clause 5.4.4 of TS 38.101-1 [7]. For TDD bands FTx-Rx_separation = 0. |
| ΔFTx-Rx_separation | ΔFTx-Rx_separation = ¦ (BWDL – BWUL)/2 ¦ is the deviation to the default Tx-Rx carrier centre frequency separation (FTx-Rx_separation) for FDD FR1 bands supporting asymmetric channel bandwidths as defined in clause 5.3.6 of TS 38.101-1 [7]. |
| BWSSB | Bandwidth of the SSB. BWSSB = 12 * SCSSSB * 20. |
| ΔGSCN, GSCNMIN, GSCNMAX | GSCN step size, GSCN minimum and GSCN maximum values for the NR band according to table 5.4.3.3-1 of TS 38.101-1 [7] and TS 38.101-2 [8] |
| OffsetRBs | Offset (RBs) according to the applicable table 13-1 to 13-10 in TS 38.213 [22] for the given band and {SCSSSB, SCSCarrier} combination of the carrier. |
| OffsetRBs,max | Maximum value for Offset (RBs) according to the applicable table 13-1 to 13-1o in TS 38.213 [22] for the given band and {SCSSSB, SCSCarrier} combination of the carrier limited to the table indexes with number of RBs equal to the maximum value of in the table. |
| OffsetRBs,min | Minimum value for Offset (RBs) according to the applicable table 13-1 to 13-1o in TS 38.213 [22] for the given band and {SCSSSB, SCSCarrier} combination of the carrier limited to the table indexes with number of RBs equal to the minimum value of in the table. |

<<Unchanged sections skipped>>

C.3	Determination of SSB and CORESET#0

C.3.1	General

The requirements to be met and the principles used for determining the SSB and CORESET#0 for a PCell are:

1.	The complete SSB and CORESET#0 shall be within the carrier’s channel bandwidth.

2.	The SSB centre frequency (SSref) shall be on the synchronisation raster.

3.	The SSB shall be kept as close as possible to the carrier’s lower edge centre frequency.

4.	CORESET#0 configuration is selected using lowest number of RBs and symbols in applicable table in TS 38.213 [22], clause 13.

5.	The first SSB subcarrier shall be aligned with the defined resource grid given by SCS indicated by subCarrierSpacingCommon in the MIB.

C.3.2	Determination of SSB, CORESET#0 and signalling parameters for a PCell

Calculation of SSB and CORESET#0 parameters is limited to FR1 carriers with SCS=15 kHz or SCS=30 kHz, and to FR2 carriers with SCS=60 kHz or SCS=120 kHz. CORESET#0 is required for a carrier to be used as a PCell.

The following procedure is used to determine an SSB on the synchronisation raster (GSCN) and a CORESET#0 configuration (kSSB, OffsetRBs and OffsetToPointA) as close as possible to the carrier’s lower edge. See figure C1-1 and clause C.1 for definition of parameters referenced in the procedure.

1.	Determine SSB and CORESET#0:

1a.	Calculate the lower of FSSref, FSSref_Min, correspondent to SSB lowest subcarrier being at the same frequency as the carrier’s lowest subcarrier; and the higher limit of FSSref, FSSref_Max, correspondent to SSB highest subcarrier being at the same frequency as the carrier’s highest subcarrier FSSref_Max

FcarrierLow = see formula for FcarrierLow in Table C.1-1

FSSref_Min = FcarrierLow + CRBsize * OffsetRBs,min + BWSSB / 2

FSSref_Max = FcarrierLow + ΔFcarrierBandwidth - BWSSB / 2

1b.	Calculate GSCNMIN correspondent to FSSref_Min in accordance to TS 38.101-1 [7], clause 5.4.3.1 for FR1 and TS 38.101-2 [7], clause 5.4.3.1 for FR2 and select the closest valid GSCN value with GSCN >= GSCNMIN for the carrier in according to the carrier’s synchronisation raster as specified in clause 5.4.3.3 in TS 38.101-1 [7] and TS 38.101-2 [8].

1c.	Calculate the FSSref for the selected GSCN value in step 1b in accordance to TS 38.101-1 [7], clause 5.4.3.1 for FR1 and TS 38.101-2 [7], clause 5.4.3.1 for FR2.

1d.	Calculate the frequency FoffsetToPointA, which is the lowest subcarrier of the lowest resource block with the subcarrier spacing being a multiple of resource blocks expressed in terms of common PRB size and overlaps with the SS/PBCH block subcarrier 0 of the first resource block of the SS/PBCH block, FSSBlow (TS 38.211 [3], clause 7.4.3.1):

FSSBlow = FSSref - BWSSB / 2

FoffsetToPointA = CRBsize * Floor(Round((FSSBlow - FcarrierLow) / CRBsize),5) + FcarrierLow Note 1

Note 1:	The “Round” operation is needed to avoid that platforms using “float” representation will give erroneous results. Hence, the need for the “Round” operation is platform dependant. The “Round” operation is to be done to 5 decimals. It is needed to avoid that a calculated value of for example 5.9999999999 is Floored to the value of 5.

1e.	Calculate the maximum OffsetRBs value with FCORESET0Low >= FcarrierLow:

Max_OffsetRBs = Round((FoffsetToPointA - FcarrierLow) / CRBsize) Note 2

Note 2:	The “Round” operation is needed to avoid that platforms using “float” representation will give erroneous results. Hence, the need for the “Round” operation is platform dependant. It is needed to make sure that the Max_OffsetRBs is an integer. For example a calculated value of Max_OffsetRBs = 5.9999999999 shall result in a Max_OffsetRBs = 6.

1f.	Select the largest valid OffsetRBs value equal or smaller than the calculated max value, Max_OffsetRBs in step 1e within the applicable values for the carrier in TS 38.213 [4], table 13-1 to 13-10 limited to the table indexes with number of RBs and number of symbols  equal to the minimum value of in the table confining CORESET#0 between FcarrierLow and FcarrierHigh and minimum value of  for the selected .If a valid OffsetRBs value is found, then continue from step 1g.

If no valid OffsetRBs value is found, then select the next valid GSCN with FSSref <= FSSref_Max within the valid GSCN range for the carrier and repeat steps 1b to 1f.

If no valid OffsetRBs value found within the valid GSCN range for the currently selected carrier frequency Fcarrier then shift Fcarrier up by ΔFRaster for Low range; or down by ΔFRaster for Mid, Mid-Low, Mid-High and High ranges and repeat steps 1a to 1g. In case of an exception, for Mid, Mid-Low and Mid-High, shifting frequency up can also be considered to achieve the frequency closer to the ideal case.

If no valid OffsetRBs value found within the valid GSCN range then will the carrier not be possible to use as PCell and FSSref, kSSB, FPointA, OffsetToCarrier and OffsetToPointA are calculated as described in clause C.3.3 and the procedure is completed.

1g.	Calculate kSSB

kSSB = (FSSBlow - FoffsetToPointA) / {15 kHz for FR1, subCarrierSpacingCommon (MIB) for FR2} (TS 38.211 [3], clause 7.4.3.1).

N = SCSSSB / {15 kHz for FR1; subCarrierSpacingCommon (MIB) for FR2}. kSSB MOD N <> 0 indicates that the SSB subcarriers are not aligned with the resource grid given by the SCS indicated by subCarrierSpacingCommon in the MIB.

If kSSB is an integer and kSSB MOD N = 0, then continue from step 2.

If kSSB is not an integer value or kSSB MOD N <> 0, then select the next valid GSCN with FSSref < FSSref_Max within the valid GSCN range for the carrier and repeat steps 1b to 1g. 

If N > 1 and no valid kSSB value found within the valid GSCN range for the currently selected carrier frequency Fcarrier then shift Fcarrier up by ΔFRaster for Low range; or down by ΔFRaster for Mid, Mid-Low, Mid-High and High ranges and repeat steps 1a to 1g for a maximum shift of 3 * ΔFRaster (see clause C.3.1, Note 1). Cases requiring more than 3*ΔFRaster shifts shall be analyzed individually and, if eventually agreed, marked in section 4.3 to have been obtained applying certain exceptions over this section C.3.2. In case of an exception, for Mid, Mid-Low and Mid-High, shifting frequency up can also be considered to achieve the frequency closer to the ideal case.

If no valid kSSB value found within the valid GSCN range then will the carrier not be possible to use as PCell and FSSref, kSSB, FPointA, OffsetToCarrier and OffsetToPointA are calculated as described in clause C.3.2 and the procedure is completed.

2.	Determine OffsetToCarrier

Select offsetToCarrier value for the carrier in accordance to Table C.3.2-1.

Table C.3.2-1: Downlink and uplink offsetToCarrier default values for different frequency ranges

| Frequency range | Downlink offsetToCarrier | Uplink offsetToCarrier |
| --- | --- | --- |
| Low range | 0 | 0 |
| Mid range | 102 | 504 |
| High range | 504 | 6 |
| Mid-Low range | 12 | 36 |
| Mid-High range | 24 | 114 |
| Note: Different values of offsetToCarrier have been selected for Low, Mid-Low, Mid, Mid-High and High ranges to achieve enhanced test coverage of the offsetToCarrier range of values. |  |  |

In case low, mid and high range are exactly the same frequency, use offsetToCarrier associated to low range.

2b.	Determine FpointA:

FPointA = FcarrierLow - offsetToCarrier * PRBsize

3.	Calculate ΔFOffsetCORESET-0-Carrier:

The ΔFOffsetCORESET-0-Carrier value is used to calculate the Offset Carrier CORESET#0 parameter included in the test frequency tables in sub-clauses 4.3.1 and 6.2.3.

ΔFOffsetCORESET-0-Carrier = FoffsetToPointA - OffsetRB * CRBsize - FcarrierLow

4.	Calculate signalling parameters:

| IE field | Value |
| --- | --- |
| ssb-SubcarrierOffset | Set to the 4 least significant bits of kSSB. For the case kSSB > 15 the extended by an additional most significant bit encoded within PBCH as specified in TS 38.213 [22]. The IE field ssb-SubcarrierOffset is signalled in the MIB. |
| controlResourceSetZero | Set to the index associated with the selected OffsetRBs value in the applicable table, 13-1 to 13-10, in TS 38.213 [22]. The IE field controlResourceSetZero is signalled in the IE pdcch-ConfigSIB1 in the MIB. |
| absoluteFrequencySSB | Set to FSSref expressed in ARFCN as defined in TS 38.101-1 [15] and TS 38.101-2 [39], clause 5.4.2. |
| absoluteFrequencyPointA | Set to FPointA expressed in ARFCN as defined in TS 38.101-1 [15] and TS 38.101-2 [39], clause 5.4.2. |
| offsetToPointA | (FOffsetToPointA - FPointA) / (12*{15 kHz for FR1; 60 kHz for FR2}). The IE field offsetToPointA is signalled in IE FrequencyInfoDL-SIB. |
| offsetToCarrier | Set to value calculated in step 2a. The IE field offsetToCarrier is signalled in IE SCS-SpecificCarrier. |

C.3.3	Determination of SSB and signalling parameters for a carrier without CORESET#0

The following procedure is used for calculation of SSB and signalling parameters for a carrier without a CORESET#0.

1. 	Calculate FSSref, kSSB and FPointA with the SSB lowest subcarrier at the carrier’s lowest subcarrier (ΔFOffsetSSB-Carrier in Figure C.1-1 = 0):

FSSref = FcarrierLow + BWSSB / 2

kSSB = {31 for FR1; 15 for FR2} indicating that no CORESET#0 is present for the carrier (TS 38.213 [4], clause 13).

offsetToCarrier = target value for offsetToCarrier dependent on frequency range as specified in Table C.3.2-1.

FPointA = FcarrierLow - offsetToCarrier * PRBsize

2.	Calculate signalling parameters:

| IE field | Value |
| --- | --- |
| ssb-SubcarrierOffset | Set to the 4 least significant bits of kSSB. For the case kSSB > 15 the extended by an additional most significant bit encoded within PBCH as specified in TS 38.213 [22]. The IE field ssb-SubcarrierOffset is signalled in the MIB. |
| controlResourceSetZero | Set to 0 indicating that no CORESET#0 exist (TS 38.213 [22], clause 13). The IE field controlResourceSetZero is signalled in the IE pdcch-ConfigSIB1 in the MIB. |
| searchSpaceZero | Set to 0 indicating that no CORESET#0 exist (TS 38.213 [22], clause 13). The IE field searchSpaceZero is signalled in the IE pdcch-ConfigSIB1 in the MIB. |
| absoluteFrequencySSB | Set to FSSref expressed in ARFCN as defined in TS 38.101-1 [15] and TS 38.101-2 [39], clause 5.4.2. |
| absoluteFrequencyPointA | Set to FPointA expressed in ARFCN as defined in TS 38.101-1 [15] and TS 38.101-2 [39], clause 5.4.2. |
| offsetToCarrier | Set to offsetToCarrier target value selected in step 1. |

<<End of change>>