3GPP TSG-RAN5 Meeting #108	R5-255243

# Bengaluru, India, 25th - 29th August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3423 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ATG Tx Tests - Corrections and updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd, CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Core specs have been updated in R4-2507734 with clarifications regarding the requirements applicability in sections 6.3J and sections 6.4J. Those clarifications for section 6.4J need to be propagated into corresponding test cases and section in 38.521-1. MU/TT analysis has progressed in R5-254067 for test cases in section 6.4J.2, even driving into required updates to the test procedures. Subclauses numbering inside test case 6.4J.2.1 were incorrect what has lead into wrong insertion of test cases 6.4J.2.4 and 6.4J.2.5 into test specs version 19.0.0. In addition test definition and subclauses inside 6.4J.1 are also incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated section 6.4J as per indications commented above. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain incomplete, incorrect and misaligned with core specs for ATG test cases in section 6.4J. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4J |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: -Added CMCC as cosourcing company in order to resolve overlap with R5-253881 (CMCC) that has been withdrawn. |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

## 6.4J	Transmit signal quality for ATG

### 6.4J.1	Frequency error for ATG

6.4J.1.1	Test purpose

Same test purpose as in 6.4.1.1

6.4J.1.2	Test applicability

This test case applies to all types of NR UE release 18 and forward that support NR standalone ATG.

6.4J.1.3	Minimum conformance requirements

The ATG UE basic measurement interval of modulated carrier frequency is 1 UL slot. The ATG UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift. The mean value of basic measurements of ATG UE modulated carrier frequency per layer at each transmit antenna connector for UE with omni-directional antenna(s) or at each transceiver array boundary (TAB) connector for UE with antenna array shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency.

UE [shall] rely on the ATG BS location broadcasted by the SIB22 in TS 38.331 [6].

NOTE 1:	The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by ATG BS and UL precompensated doppler frequency shift.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.4J.1.

6.4J.1.4	Test description

Same test description as in clause 6.4.1.4 with the following exceptions:

-	In step 4, measure the Frequency Error for each antenna connector (for ATG UE with omni-directional antennas) or for each TAB connector (for ATG UE with antenna array).

6.4J.1.5	Test requirement

The each of 10 frequency error Δf results, derived in Annex E.3.1 for each antenna connector (for ATG UE with omni-directional antennas) or for each TAB connector (for ATG UE with antenna array) must fulfil the test requirement:

|Δf| ≤ (0.1 PPM + 15 Hz)

### 6.4J.2	Transmit modulation quality for ATG

#### 6.4J.2.1	Error Vector Magnitude for ATG

6.4J.2.1.1	Test purpose

Same test purpose as in 6.4.2.1.1.

6.4J.2.1.2	Test applicability

This test case applies to all types of NR UE release 18 and forward that support NR standalone ATG.

Support of 256QAM is optional, only applicable if indicating support.

6.4J.2.1.3	Minimum conformance requirements

The requirements for transmit modulation quality defined in clause 6.4.2.1.3 shall apply for ATG UE at each transmit antenna connector for UE with omni-directional antenna(s) or at each transceiver array boundary (TAB) connector for UE with antenna array, except for the  requirements for 256QAM modulation are only applicable to ATG UE indicating support of 256QAM.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.4J.2

6.4J.2.1.4	Test description

Same test description as in clause 6.4.2.1.4 with the following exceptions:

-	Pmin is defined in clause 6.3J.1.

-	In steps 1.4 and 2.5 where Uplink power control window is used, power control shall be applied to the sum of power of all antenna connectors (for ATG UE with omni-directional antennas) or of all TAB connectors (for ATG UE with antenna array).

-	In steps 1.3, 1.5, 2.4, 2.6 and 3.7 where EVM is measured, measure the EVM and power for each antenna connector (for ATG UE with omni-directional antennas) or for each TAB connector (for ATG UE with antenna array). For antenna connector or TAB connector with power smaller than Pmin, the EVM measurement is discarded.

6.4J.2.1.5	Test requirement

The PUSCH EVM, derived in Annex E.4.2, for each antenna connector (for ATG UE with omni-directional antennas) or for each TAB connector (for ATG UE with antenna array) with power equal or greater than Pmin, shall not exceed the values in Table 6.4J.4.2.1.5-1.

The PUSCH, derived in Annex E.4.6.2, for each antenna connector (for ATG UE with omni-directional antennas) or for each TAB connector (for ATG UE with antenna array) with power equal or greater than Pmin, shall not exceed the values in Table 6.4J.4.2.1.5-1 when embedded with data symbols of the respective modulation scheme.

Table 6.4J.4.2.1.5-1: Test requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| Pi/2-BPSK | % | 30 + TT |
| QPSK | % | 17.5 + TT |
| 16 QAM | % | 12.5 + TT |
| 64 QAM | % | 8 + TT |
| 256 QAM | % | 3.5 + TT |
| Note 1: TT is defined in Table 6.4.2.1.5-2. |  |  |

Table 6.4J.4.2.1.5-2: Test Tolerance

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| Pi/2-BPSK | % | FFS |
| QPSK | % | FFS |
| 16 QAM | % | FFS |
| 64 QAM | % | FFS |
| 256 QAM | % | FFS |

The PUCCH EVM derived in Annex E.5.9.2 for each antenna connector (for ATG UE with omni-directional antennas) or for each TAB connector (for ATG UE with antenna array) with power equal or greater than Pmin shall not exceed 17.5 %.

The PRACH EVM derived in Annex E.6.9.2 for each antenna connector (for ATG UE with omni-directional antennas) or for each TAB connector (for ATG UE with antenna array) with power equal or greater than Pmin shall not exceed 17.5%.

#### 6.4J.2.1a	Error Vector Magnitude including symbols with transient period for ATG

6.4J.2.1a.1	Test Purpose

Same test purpose as in 6.4.2.1a.1

6.4J.2.1a.2	Test applicability

This test case applies to all types of NR UE release 18 and forward that support NR standalone ATG.

Support of 256QAM is optional, only applicable if indicating support

6.4J.2.1a.3	Minimum conformance requirements

The requirements for transmit modulation quality defined in clause 6.4.2.1a.3 shall apply for ATG UE at each transmit antenna connector for UE with omni-directional antenna(s) or at each transceiver array boundary (TAB) connector for UE with antenna array, except for the requirements for 256QAM modulation are only applicable to ATG UE indicating support of 256QAM.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.4J.2

6.4J.2.1a.4	Test description

Same test description as in clause 6.4.2.1a.4 with the following exceptions:

-	In step 2, instead of using a power control window, send continuously uplink power control "up" commands for both carriers in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

-	In step 4, where EVM is measured, measure the EVM for each antenna connector (for ATG UE with omni-directional antennas) or for each TAB connector (for ATG UE with antenna array).

6.4J.2.1a.5	Test requirement

The EVMbefore and EVMafter derived in Table 6.4.2.1a.3-1 shall not exceed the values in Table 6.4J.2.1a.5-1 for each antenna connector (for ATG UE with omni-directional antennas) or for each TAB connector (for ATG UE with antenna array). Both rising and falling edges of transient occurring at the respective slot boundaries are considered to have the same EVM requirements.

Table 6.4J.2.1a.5-1: Test requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| 64 QAM | % | 10 + TT |
| 256 QAM | % | 8 + TT |
| Note 1: TT is defined in Table 6.4J.2.1a.5-2. |  |  |

Table 6.4J.2.1a.5-2: Test Tolerance

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| 64 QAM | % | 0 |
| 256 QAM | % | 0 |

#### 6.4J.2.2

#### 6.4J.2.3

#### 6.4J.2.4	EVM equalizer spectrum flatness for ATG

6.4J.2.4.1	Test purpose

Same test purpose as in 6.4.2.4.1.

6.4J.2.4.2	Test applicability

This test case applies to all types of NR UE release 18 and forward that support NR standalone ATG.

Support of 256QAM is optional, only applicable if indicating support.

6.4J.2.4.3	Minimum conformance requirements

The requirements for transmit modulation quality defined in clause 6.4.2.4.3 shall apply for ATG UE at each transmit antenna connector for UE with omni-directional antenna(s) or at each transceiver array boundary (TAB) connector for UE with antenna array, except for the requirements for 256QAM modulation are only applicable to ATG UE indicating support of 256QAM.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.4J.2

6.4J.2.4.4	Test description

Same test description as in clause 6.4.2.4.4 with the following exceptions:

-	In step 3, measure spectrum flatness for each antenna connector (for ATG UE with omni-directional antennas) or for each TAB connector (for ATG UE with antenna array).

6.4J.2.4.5	Test requirement

Each of the n spectrum flatness functions, shall derive four ripple results in Annex E.4.4.1. The derived results for each antenna connector (for ATG UE with omni-directional antennas) or for each TAB connector (for ATG UE with antenna array) shall not exceed the values in Figure 6.4J.2.4.5-1:

For normal conditions, the maximum ripple in Range 1 and Range 2 shall not exceed the values specified in Table 6.4.2.4.5-1 and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 6.4 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 8.4 dB (see Figure 6.4J.2.4.5-1).

For normal conditions, the peak-to-peak variation of the EVM equalizer coefficients contained within the frequency range of the uplink allocation shall not exceed the maximum ripple specified in Table 6.4J.2.4.5-1. For uplink allocations contained within both Range 1 and Range 2, the coefficients evaluated within each of these frequency ranges shall meet the corresponding ripple requirement and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 6.4 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 8.4 dB (see Figure 6.4J.2.4.5-1).

For extreme conditions, the EVM equalizer spectral flatness shall not exceed the values specified in Table 6.4J.2.4.5-2. For uplink allocations contained within both Range 1 and Range 2, the coefficients evaluated within each of these frequency ranges shall meet the corresponding ripple requirement and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 7.4 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 11.4 dB (see Figure 6.4J.2.4.5-1).

Table 6.4J.2.4.5-1: Requirements for EVM equalizer spectrum flatness (normal conditions)

| Frequency range | Maximum ripple [dB] |
| --- | --- |
| FUL_Meas – FUL_Low ≥ 3 MHz and FUL_High – FUL_Meas ≥ 3 MHz (Range 1) | 4 + TT (p-p) |
| FUL_Meas – FUL_Low < 3 MHz or FUL_High – FUL_Meas < 3 MHz (Range 2) | 8 + TT (p-p) |
| NOTE 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated NOTE 2: FUL_Low and FUL_High refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 3: Test tolerance TT = 1.4 dB. |  |

Table 6.4J.2.4.5-2: Minimum requirements for EVM equalizer spectrum flatness (extreme conditions)

| Frequency range | Maximum Ripple [dB] |
| --- | --- |
| FUL_Meas – FUL_Low ≥ 5 MHz and FUL_High – FUL_Meas ≥ 5 MHz (Range 1) | 4 + TT (p-p) |
| FUL_Meas – FUL_Low < 5 MHz or FUL_High – FUL_Meas < 5 MHz (Range 2) | 12 + TT (p-p) |
| NOTE 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated NOTE 2: FUL_Low and FUL_High refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 3: Test tolerance TT = 1.4 dB. |  |

Figure 6.4J.2.4.5-1: The test requirements for EVM equalizer spectral flatness with the maximum allowed variation of the coefficients indicated for unshaped modulations (the ETC test requirements are within brackets)

#### 6.4J.2.5	EVM equalizer spectrum flatness for Pi/2 BPSK for ATG

6.4J.2.5.1	Test purpose

Same test purpose as in.6.4.2.5.1

6.4J.2.5.2	Test applicability

This test case applies to all types of power class 3 capable NR UE release 18 and forward that support NR standalone ATG and  that support UE capability powerBoosting-pi2BPSK and operating in TDD bands n41, n78 and n79, and don’t support Tx diversity.

This test case applies to all types of NR UE release 18 and forward that support NR standalone ATG that support UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16, and don’t support Tx diversity.

6.4J.2.5.3	Minimum conformance requirements

The requirements for transmit modulation quality defined in clause 6.4.2.5.3 shall apply for ATG UE at each transmit antenna connector for UE with omni-directional antenna(s) or at each transceiver array boundary (TAB) connector for UE with antenna array.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.4J.2

6.4J.2.5.4	Test description

Same test description as in clause 6.4.2.5.4 with the following exceptions:

-	In step 3, measure spectrum flatness for each antenna connector (for ATG UE with omni-directional antennas) or for each TAB connector (for ATG UE with antenna array).

6.4J.2.5.5	Test requirement

Each of the n spectrum flatness functions, shall derive four ripple results in Annex E.4.4.1 The derived results for each antenna connector (for ATG UE with omni-directional antennas) or for each TAB connector (for ATG UE with antenna array) shall not exceed the values in Figure 6.4J.2.5.5-1:

Table 6.4J.2.5.5-1: Mask for EVM equalizer coefficients for Pi/2 BPSK, normal conditions

| Frequency range | Parameter | Maximum ripple (dB) |
| --- | --- | --- |
| ¦FUL_Meas – F_center¦ ≤ X MHz (Range 1) | X1 | 6 + TT (p-p) |
| ¦FUL_Meas – F_center¦ > X MHz (Range 2) | X2 | 14 + TT (p-p) |
| NOTE 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated NOTE 2: F_center refers to the center frequency of an allocated block of PRBs NOTE 3: X, in MHz, is equal to 25% of the bandwidth of the PRB allocation NOTE 4: See Figure 6.4.2.5.5-1 for description of X1, X2 NOTE 5: Test tolerance TT = 1.4 dB. |  |  |

Figure 6.4J.2.5.5-1: The limits for EVM equalizer spectral flatness with the maximum allowed variation. F_center denotes the center frequency of the allocated block of PRBs. X, in MHz, is equal to 25 % of the bandwidth of the PRB allocation.

Each of the n spectrum flatness functions shall derive an impulse response of the spectral shaping filter in Annex E.4.4.2. The derived results shall fulfill:

where TT = 1.4dB.

6.4J.4.2.1.1	Test purpose

Same test purpose as in 6.4.2.1.1.

6.4J.4.2.1.2	Test applicability

This test case applies to all types of NR UE release 18 and forward that support NR standalone ATG.

Support of 256QAM is optional.only applicable if indicating support.

6.4J.4.2.1.3	Minimum conformance requirements

The requirements for transmit modulation quality defined in clause 6.4.2.1.3 shall apply for ATG UE except for the  requirements for 256QAM modulation are only applicable to ATG UE indicating support of 256QAM.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.4J.2

6.4J.4.2.1.4	Test description

Same test description as in clause 6.4.2.1.4 with the following exceptions:

-	Pmin is defined in clause 6.3J.1.

-	In steps 1.4 and 2.5 where Uplink power control window is used, power control shall be applied to the sum of power of all antenna connectors (for ATG UE with omni-directional antennas) or of all TAB connectors (for ATG UE with antenna array).

-	In steps 1.3, 1.5, 2.4, 2.6 and 3.7 where EVM is measured, measure the EVM and power for each antenna connector (for ATG UE with omni-directional antennas) or for each TAB connector (for ATG UE with antenna array). For antenna connector or TAB connector with power smaller than Pmin, the EVM measurement is discarded.

6.4J.4.2.1.5	Test requirement

The PUSCH EVM, derived in Annex E.4.2, for each antenna connector (for ATG UE with omni-directional antennas) or for each TAB connector (for ATG UE with antenna array) with power equal or greater than Pmin, shall not exceed the values in Table 6.4J.4.2.1.5-1.

The PUSCH, derived in Annex E.4.6.2, for each antenna connector (for ATG UE with omni-directional antennas) or for each TAB connector (for ATG UE with antenna array) with power equal or greater than Pmin, shall not exceed the values in Table 6.4J.4.2.1.5-1 when embedded with data symbols of the respective modulation scheme.

Table 6.4J.4.2.1.5-1: Test requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| Pi/2-BPSK | % | 30 + TT |
| QPSK | % | 17.5 + TT |
| 16 QAM | % | 12.5 + TT |
| 64 QAM | % | 8 + TT |
| 256 QAM | % | 3.5 + TT |
| Note 1: TT is defined in Table 6.4.2.1.5-2. |  |  |

Table 6.4J.4.2.1.5-2: Test Tolerance

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| Pi/2-BPSK | % | FFS |
| QPSK | % | FFS |
| 16 QAM | % | FFS |
| 64 QAM | % | FFS |
| 256 QAM | % | FFS |

The PUCCH EVM derived in Annex E.5.9.2 for each antenna connector (for ATG UE with omni-directional antennas) or for each TAB connector (for ATG UE with antenna array) with power equal or greater than Pmin shall not exceed 17.5 %.

The PRACH EVM derived in Annex E.6.9.2 for each antenna connector (for ATG UE with omni-directional antennas) or for each TAB connector (for ATG UE with antenna array) with power equal or greater than Pmin shall not exceed 17.5%.

## <<< END OF CHANGES >>>