3GPP TSG-RAN5 Meeting #108	R5-255257

# Bengaluru, India, 25 – 29 Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3518 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of AMPR and Spurious emission of CA_n40A-n41A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test aspect of CA_n40A-n41A is still missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the test of AMPR and spurious emissio for CA_n40A-n41A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The configuration of CA_n40A-n41A can’t be tested |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.3, 6.5A.3.1, 6.5A.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.905 CR 1078, 1079 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-254785 |  |  |  |  |  |  |  |  |

<Start of Changes>

6.2A.3.0.3	UE additional maximum output power reduction for inter-band CA

Unless otherwise stated, for inter-band carrier aggregation with uplink assigned to two NR bands, the requirements in subclause 6.2.3 apply for each uplink component carrier.

Unless specified in Table 6.2A.3.0.3-1, for inter-band carrier aggregation with uplink assigned to two NR bands, the requirements in clause 6.2.3 apply only to the indicated carrier. The requirements in Table 6.2A.3.0.3-1 are specified in terms of an additional spectrum emission requirement with their associated network signalling values and the allowed A-MPR. Unless otherwise stated, the combined requirements and allowed A-MPR are applicable on both bands when both component carriers are active. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet the additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. In case of a power class 3 UE, when IE powerBoostPi2BPSK is set to 1, power class 2 A-MPR values apply.

For almost contiguous allocations in CP-OFDM waveforms in power class 3, the allowed A-MPR defined in clause 6.2.3 is increased by CEIL{ 10 log10(1 + NRB_gap / NRB_alloc), 0.5 } dB, where NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs, and the parameter LCRB is replaced by NRB_alloc + NRB_gap in specifying the RB allocation regions.

Unless otherwise specified, pi/2 BPSK in following A-MPR tables refers to both variants of pi/2 BPSK referenced in 6.2.2 tables 6.2.2-1.

The emission requirements specified in Table 6.2A.3.0.3-1 also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.2A.3.0.3-1: Additional Requirements for uplink inter-band carrier aggregation (two-bands)

| NR CA combination | Band | Applied NS | Requirements (clause) | A-MPR (table/clause) | Note |
| --- | --- | --- | --- | --- | --- |
|  | n1 | 05 | 6.5.3.3.3.4 | Clause 6.2.3.3.4 |  |
| CA_n1-n3 |  | 05U | 6.5.3.3.3.4, 6.5.2.4.2 | Clause 6.2.3.3.4 | 1 |
|  | n3 | 100 | 6.5.2.4.2 | Table 6.2.3.3.1-2 |  |
|  | n1 | 05 | 6.5.3.3.3.4 | Clause 6.2.3.3.4 |  |
| CA_n1-n8 |  | 05U | 6.5.3.3.3.4, 6.5.2.4.2 | Clause 6.2.3.3.4 | 1 |
|  | n8 | 43 | 6.5.3.3.3.5 | Clause 6.2.3.3.6 |  |
|  |  | 43U | 6.5.3.3.3.5, 6.5.2.4.2 | Clause 6.2.3.3.6 |  |
| CA_n1-n28 | n1 | 05 | 6.5.3.3.3.4 | Clause 6.2.3.3.4 |  |
|  |  | 05U | 6.5.3.3.3.4, 6.5.2.4.2 | Clause 6.2.3.3.4 | 1,2 |
|  | n28 | 17 | 6.5.3.3.3.2 | N/A |  |
| CA_n1-n41 | n1 | 05 | 6.5.3.3.3.4 | Clause 6.2.3.3.4 |  |
|  |  | 05U | 6.5.3.3.3.4, 6.5.2.4.2 | Clause 6.2.3.3.4 | 1 |
|  | n41 | 47 | 6.5.3.3.3.15 | Table 6.2.3.3.18-2 |  |
| CA_n1-n78 | n1 | 05 | 6.5.3.3.3.4 | Clause 6.2.3.3.4 | 1 |
|  |  | 05U | 6.5.3.3.3.4, 6.5.2.4.2 | Clause 6.2.3.3.4 |  |
| CA_n1-n79 | n1 | 05 | 6.5.3.3.3.4 | Clause 6.2.3.3.4 | 1 |
|  |  | 05U | 6.5.3.3.3.4, 6.5.2.4.2 | Clause 6.2.3.3.4 |  |
| CA_n3-n28 | n3 | 100 | 6.5.2.4.2 | Table 6.2.3.3.1-2 | 1,2 |
|  | n28 | 17 | 6.5.3.3.3.2 | N/A |  |
| CA_n3-n40 | n3 | 100 | 6.5.2.4.2 | Table 6.2.3.3.1-2 | 1 |
| CA_n3-n41 | n3 | 100 | 6.5.2.4.2 | Table 6.2.3.3.1-2 | 1 |
|  | n41 | 47 | 6.5.3.3.3.15 | Table 6.2.3.3.18-2 |  |
| CA_n3-n78 | n3 | 100 | 6.5.2.4.2 | Table 6.2.3.3.1-2 | 1 |
| CA_n5-n77 | n5 | 100 | 6.5.2.4.2 | Table 6.2.3.3.1-2 | 1 |
| CA_n5-n78 | n5 | 100 | 6.5.2.4.2 | Table 6.2.3.3.1-2 | 1 |
| CA_n8-n78 | n8 | 43 | 6.5.3.3.3.5 | Clause 6.2.3.3.6 | 1 |
|  |  | 43U | 6.5.3.3.3.5, 6.5.2.4.2 | Clause 6.2.3.3.6 |  |
| CA_n24-n41 | n24 | 56 | 6.5.3.3.3.27 | Clause 6.2.3.3.30 |  |
|  | n41 | 04 | 6.5.2.3.3.2, 6.5.3.3.3.1 | Clause 6.2.3.3.2 |  |
| CA_n24-n48 | n24 | 56 | 6.5.3.3.3.27 | Clause 6.2.3.3.30 |  |
|  | n48 | 27 | 6.5.2.3.3.8, 6.5.3.3.3.14 | Clause 6.2.3.3.16 |  |
| CA_n24-n77 | n24 | 56 | 6.5.3.3.3.27 | Clause 6.2.3.3.30 |  |
|  | n77 | 55 | NOTE 6 | N/A |  |
| CA_n28-n40 | n28 | 17 | 6.5.3.3.3.2 | N/A | 2 |
| CA_n28-n41 | n28 | 17 | 6.5.3.3.3.2 | N/A | 2 |
|  | n41 | 47 | 6.5.3.3.3.15 | Table 6.2.3.3.18-2 |  |
| CA_n28-n78 | n28 | 17 | 6.5.3.3.3.2 | N/A | 2 |
| CA_n28-n79 | n28 | 17 | 6.5.3.3.3.2 | N/A | 2 |
| CA_n40-n41 | n41 | 47 | 6.5.3.3.15 | Table 6.2.3.18-2 |  |
| CA_n41-n77 | n41 | 47 | 6.5.3.3.3.15 | Table 6.2.3.3.18-2 |  |
| CA_n41-n79 | n41 | 47 | 6.5.3.3.3.15 | Table 6.2.3.3.18-2 |  |
| NOTE 1: NS_05U, NS_43U and NS_100 can be signalled for NR bands that have UTRA services deployed and the requirements in clause 6.5.2.4.2 are only applicable to the signalling carrier. NOTE 2: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 3: TBD NOTE 4: TBD NOTE 5: TBD NOTE 6: This NS value is applicable for cells in the range 3450 – 3550 MHz for operations in the USA. This NS value does not indicate any additional spurious emission and maximum output power reduction requirements. |  |  |  |  |  |

6.2A.3.1	UE additional maximum output power reduction for CA (2UL CA)

Editor’s note: This clause is complete for AMPR testing. But the following aspects are either missing for not yet determined:

-	For intra-band non-contiguous CA, tests for network signalling value other than NS_04 are not complete.

6.2A.3.1.1	Test purpose

Additional emission requirements for CA can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the CA maximum output power as specified in Table 6.2A.1.0.3-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2A.2. Outer and inner allocation notation used in clause 6.2A.3 is defined in clause 6.2.2. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

<Unchanged Text Skipped>

6.2A.3.1.5	Test requirement

The maximum output power, derived in step 6 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2A.3.1.5-1, Table 6.2A.3.1.5-2, Table 6.2A.3.1.5-3 as appropriate. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in 6.2A.4.0 apply. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

Table 6.2A.3.1.5-0: Test Tolerance for Additional Maximum Power Reduction (A-MPR)

| TT for overall output power (dB) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PCell |  |  |  |  |  |
|  |  |  | BW ≤ 40MHz |  |  | 40MHz < BW ≤ 100MHz |  |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| SCell | BW ≤ 40MHz | f ≤ 3.0GHz | 0.7 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  | 40MHz < BW ≤ 100MHz | f ≤ 3.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |

Table 6.2A.3.1.5-1: UE Power Class 3 test requirement (network signalling value NS_100/NS_01) for CA_n3-n78

| Test ID | PPowerclass (dBm) | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 1 | 23 | 0 | 0 | 2 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 6.5 | 6.5 | 6.5 | 0 | 0 | 0 | 3 | 19.5 | 5 | 25+TT | 14.5-TT |
| 3 | 23 | 0 | 6.5 | 2 | 0 | 0 | 0 | 3 | 22.3 | 5 | 25+TT | 17.3-TT |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: Void NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-1a: UE Power Class 2 test requirement with both bands supporting PC3 (network signalling value NS_100) for CA_n3-n78

| Test ID | PPowerclass (dBm) | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 1 | 26 | 0 | 0 | 2 | 0 | 0 | 0 | 3 | 25.1 | 3 | 28+TT | 22.1-TT |
| 2 | 26 | 6.5 | 6.5 | 6.5 | 0 | 0 | 0 | 3 | 19.5 | 5 | 28+TT | 14.5-TT |
| 3 | 26 | 0 | 6.5 | 2 | 0 | 0 | 0 | 3 | 22.3 | 5 | 28+TT | 17.3-TT |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: Void. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-1b: UE Power Class 2 test requirement with PCC supporting PC3 and SCC supporting PC2 (network signalling value NS_100) for CA_n3-n78

| Test ID | PPowerclass (dBm) | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 1 | 26 | 0 | 0 | 2 | 0 | 0 | 0 | 3 | 26 | 3 | 28+TT | 23-TT |
| 2 | 26 | 6.5 | 6.5 | 6.5 | 0 | 0 | 0 | 3 | 21.3 | 5 | 28+TT | 16.3-TT |
| 3 | 26 | 0 | 6.5 | 2 | 0 | 0 | 0 | 3 | 23.3 | 3 | 28+TT | 20.3-TT |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: Void. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-1c: UE Power Class 2 test requirement with supporting higherPowerLimit-r17 (network signalling value NS_100) for CA_n3-n78

| Test ID | PPowerclass (dBm) | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 1 | 27.8 | 0 | 0 | 2 | 0 | 0 | 0 | 3 | 27.2 | 3 | 29.8+TT | 24.2-TT |
| 2 | 27.8 | 6.5 | 6.5 | 6.5 | 0 | 0 | 0 | 3 | 21.3 | 5 | 29.8+TT | 16.3-TT |
| 3 | 27.8 | 0 | 6.5 | 2 | 0 | 0 | 0 | 3 | 23.3 | 3 | 29.8+TT | 20.3-TT |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: Void. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-2: UE Power Class 3 test requirement (network signalling value NS_43/NS_01, NS_43U/NS_01) for CA_n8-n78

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 6.5 | 6.5 | 9.0 | 0 | 0 | 0 | 3 | 18.4 | 5 | 25+TT | 13.4-TT |
| 3 | 23 | 6.5 | 0 | 9.0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-3: UE Power Class 3 test requirement (network signalling value NS_47/NS_01) for CA_n40-n41 and CA_n41-n79

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 1 | 0 | 2 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 1 | 6.5 | 7 | 0 | 0 | 0 | 3 | 19.3 | 5 | 25+TT | 14.3-TT |
| 3 | 23 | 1 | 0 | 7 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-3a: UE Power Class 2 test requirement with both bands supporting PC3 (network signalling value NS_47/NS_01) for CA_n40-n41 and CA_n41-n79

| Test ID | PPowerclass | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 26 | 1 | 0 | 2 | 0 | 0 | 0 | 3 | 25.1 | 3 | 28+TT | 22.1-TT |
| 2 | 26 | 1 | 6.5 | 7 | 0 | 0 | 0 | 3 | 19.3 | 5 | 28+TT | 14.3-TT |
| 3 | 26 | 1 | 0 | 7 | 0 | 0 | 0 | 3 | 23.8 | 3 | 28+TT | 20.8-TT |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-3b: UE Power Class 2 test requirement with PCC supporting PC2 and SCC supporting PC3 (network signalling value NS_47/NS_01) for CA_n40-n41 and CA_n41-n79

| Test ID | PPowerclass | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 26 | 3.5 | 0 | 5 | 0 | 0 | 0 | 3 | 25.1 | 3 | 28+TT | 22.1-TT |
| 2 | 26 | 3.5 | 6.5 | 10 | 0 | 0 | 0 | 3 | 19.3 | 5 | 28+TT | 14.3-TT |
| 3 | 26 | 3.5 | 0 | 10 | 0 | 0 | 0 | 3 | 23.8 | 3 | 28+TT | 20.8-TT |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-3c: UE Power Class 2 test requirement with PCC and SCC supporting PC2 (network signalling value NS_47/NS_01) for CA_n40-n41 and CA_n41-n79

| Test ID | PPowerclass | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 26 | 3.5 | 0 | 5 | 0 | 0 | 0 | 3 | 26 | 3 | 28+TT | 23-TT |
| 2 | 26 | 3.5 | 6.5 | 10 | 0 | 0 | 0 | 3 | 21.1 | 5 | 28+TT | 16.1-TT |
| 3 | 26 | 3.5 | 0 | 10 | 0 | 0 | 0 | 3 | 26 | 3 | 28+TT | 23-TT |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-4: Void

Table 6.2A.3.1.5-5: Void

Table 6.2A.3.1.5-6: Void

Table 6.2A.3.1.5-7: UE Power Class 3 test requirement (network signalling value CA_NS_04) for CA_n41C

| Test ID | PPowerclass | MPR (dB) | AMPRcc (dB) | AMPR(dB) | ΔTC,c (dB) | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) |  |  |  |  |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 8 | - | 8 | 0 | 2 | 15 | 5 | 25+TT | 10-TT |
| 2 | 23 | 8 | - | 8 | 1.5 | 2 | 15 | 5 | 25+TT | 10-TT |
| 3 | 23 | 8 | - | 8 | 0 | 2 | 15 | 5 | 25+TT | 10-TT |
| 4 | 23 | 8 | - | 8 | 1.5 | 2 | 15 | 5 | 25+TT | 10-TT |
| 5 | 23 | 2.5 | 3.5 | 3.5 | 0 | 2 | 19.5 | 3.5 | 25+TT | 16-TT |
| 6 | 23 | 2.5 | 4 | 4 | 0 | 2 | 19 | 3.5 | 25+TT | 15.5-TT |
| 7 | 23 | 2.5 | 4 | 4 | 0 | 2 | 19 | 3.5 | 25+TT | 15.5-TT |
| 8 | 23 | 5 | 4.5 | 5 | 0 | 2 | 18 | 4 | 25+TT | 14-TT |
| 9 | 23 | 7 | 6 | 7 | 0 | 2 | 16 | 5 | 25+TT | 11-TT |
| 10 | 23 | 3.5 | 5.5 | 5.5 | 0 | 2 | 17.5 | 5 | 25+TT | 12.5-TT |
| 11 | 23 | 3.5 | 5.5 | 5.5 | 0 | 2 | 17.5 | 5 | 25+TT | 12.5-TT |
| 12 | 23 | 5 | 5.5 | 5.5 | 0 | 2 | 17.5 | 5 | 25+TT | 12.5-TT |
| 13 | 23 | 7 | 8 | 8 | 0 | 2 | 15 | 5 | 25+TT | 10-TT |

Table 6.2A.3.1.5-8: UE Power Class 2 not supporting TxD test requirement (network signalling value CA_NS_04) for CA_n41C

| Test ID | PPowerclass | MPR (dB) | AMPRcc (dB) | AMPR(dB) | ΔTC,c (dB) | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) |  |  |  |  |  |  | (dB) | (dBm) | (dBm) |
| 1 | 26 | 8 | - | 8 | 0 | 3 | 18 | 4 | 28+TT | 14-TT |
| 2 | 26 | 8 | - | 8 | 1.5 | 3 | 18 | 4 | 28+TT | 1-TT |
| 3 | 26 | 8 | - | 8 | 0 | 3 | 18 | 4 | 28+TT | 14-TT |
| 4 | 26 | 8 | - | 8 | 1.5 | 3 | 18 | 4 | 28+TT | 14-TT |
| 5 | 26 | 2.5 | 5.5 | 5.5 | 0 | 3 | 20.5 | 2.5 | 28+TT | 17.5-TT |
| 6 | 26 | 2.5 | 6 | 6 | 0 | 3 | 20 | 2.5 | 28+TT | 17-TT |
| 7 | 26 | 2.5 | 6 | 6 | 0 | 3 | 20 | 2.5 | 28+TT | 17-TT |
| 8 | 26 | 5 | 6.5 | 6.5 | 0 | 3 | 19.5 | 3.5 | 28+TT | 16-TT |
| 9 | 26 | 7 | 8 | 8 | 0 | 3 | 18 | 4 | 28+TT | 14-TT |
| 10 | 26 | 3.5 | 7.5 | 7.5 | 0 | 3 | 18.5 | 4 | 28+TT | 14.5-TT |
| 11 | 26 | 3.5 | 7.5 | 7.5 | 0 | 3 | 18.5 | 4 | 28+TT | 14.5-TT |
| 12 | 26 | 5 | 7.5 | 7.5 | 0 | 3 | 18.5 | 4 | 28+TT | 14.5-TT |
| 13 | 26 | 7 | 10 | 10 | 0 | 3 | 16 | 5 | 28+TT | 11-TT |

Table 6.2A.3.1.5-8a: UE Power Class 2 supporting TxD test requirement (network signalling value CA_NS_04) for CA_n41C

| Test ID | PPowerclass | MPR (dB) | AMPRcc (dB) | AMPR(dB) | ΔTC,c (dB) | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) |  |  |  |  |  |  | (dB) | (dBm) | (dBm) |
| 1 | 26 | 8.5 | - | 8.5 | 0 | 3 | 17.5 | 5 | 28+TT | 12.5-TT |
| 2 | 26 | 8.5 | - | 8.5 | 1.5 | 3 | 17.5 | 5 | 28+TT | 12.5-TT |
| 3 | 26 | 8.5 | - | 8.5 | 0 | 3 | 17.5 | 5 | 28+TT | 12.5-TT |
| 4 | 26 | 8.5 | - | 8.5 | 1.5 | 3 | 17.5 | 5 | 28+TT | 12.5-TT |
| 5 | 26 | 3.5 | 5.5 | 5.5 | 0 | 3 | 20.5 | 2.5 | 28+TT | 18-TT |
| 6 | 26 | 3.5 | 6 | 6 | 0 | 3 | 20 | 2.5 | 28+TT | 17.5-TT |
| 7 | 26 | 3.5 | 6 | 6 | 0 | 3 | 20 | 2.5 | 28+TT | 17.5-TT |
| 8 | 26 | 6 | 6.5 | 6.5 | 0 | 3 | 19.5 | 3.5 | 28+TT | 16-TT |
| 9 | 26 | 8 | 8 | 8 | 0 | 3 | 18 | 4 | 28+TT | 14-TT |
| 10 | 26 | 4.0 | 7.5 | 7.5 | 0 | 3 | 18.5 | 4 | 28+TT | 14.5-TT |
| 11 | 26 | 4.0 | 7.5 | 7.5 | 0 | 3 | 18.5 | 4 | 28+TT | 14.5-TT |
| 12 | 26 | 5.5 | 7.5 | 7.5 | 0 | 3 | 18.5 | 4 | 28+TT | 14.5-TT |
| 13 | 26 | 7.5 | 10 | 10 | 0 | 3 | 16 | 5 | 28+TT | 11-TT |

Table 6.2A.3.1.5-9: UE Power Class 3 test requirement (network signalling value NS_17/NS_01) for CA_n28-n78 and CA_n28-n79

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 6.5 | 6.5 | 0 | 0 | 0 | 0 | 3 | 19.5 | 5 | 25+TT | 14.5-TT |
| 3 | 23 | 6.5 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 5 | 25+TT | 20-TT |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-10: UE Power Class 2 test requirement (network signalling value NS_17/NS_01) for CA_n28-n78 and CA_n28-n79

| Test ID | PPowerclass | PPowerclass,c |  | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW (PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 26 | 23 | 26 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 26 | 3 | 28+TT | 23-TT |
| 2 | 26 | 23 | 26 | 6.5 | 6.5 | 0 | 0 | 0 | 0 | 3 | 21.3 | 5 | 28+TT | 16.3-TT |
| 3 | 26 | 23 | 26 | 6.5 | 0 | 0 | 0 | 0 | 0 | 3 | 26 | 3 | 28+TT | 23-TT |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-11: UE Power Class 3 test requirement (network signalling value NS_17/NS_47) for CA_n28-n41

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 0 | 1 | 0 | 2 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 6.5 | 1 | 0 | 7 | 0 | 0 | 3 | 19 | 5 | 25+TT | 14-TT |
| 3 | 23 | 0 | 1 | 0 | 7 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-11a: UE Power Class 2 test requirement with both bands supporting PC3 (network signalling value NS_17/NS_47) for CA_n28-n41

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 26 | 0 | 1 | 0 | 2 | 0 | 0 | 3 | 25 | 3 | 28+TT | 22-TT |
| 2 | 26 | 6.5 | 1 | 0 | 7 | 0 | 0 | 3 | 19 | 5 | 28+TT | 14-TT |
| 3 | 26 | 0 | 1 | 0 | 7 | 0 | 0 | 3 | 23.8 | 3 | 28+TT | 20.8-TT |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-11b: UE Power Class 2 test requirement with with PCC supporting PC3 and SCC supporting PC2 (network signalling value NS_17/NS_47) for CA_n28-n41

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 26 | 0 | 3.5 | 0 | 5 | 0 | 0 | 3 | 25 | 3 | 28+TT | 22-TT |
| 2 | 26 | 6.5 | 3.5 | 0 | 10 | 0 | 0 | 3 | 19 | 5 | 28+TT | 14-TT |
| 3 | 26 | 0 | 3.5 | 0 | 10 | 0 | 0 | 3 | 23.8 | 3 | 28+TT | 20.8-TT |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

<Unchanged Text Skipped>

6.5A.3.1	General spurious emissions for CA

6.5A.3.1.0	Minimum conformance requirements

For inter-band carrier aggregation with uplink assigned to two NR bands, the spurious emission requirement Table 6.5.3.1.3-2 apply for the frequency ranges that are more than FOOB as defined in Table 6.5.3.1.3-1 away from edges of the assigned channel bandwidth on a component carrier. If for some frequency a spurious emission requirement of individual component carrier overlaps with the spectrum emission mask or channel bandwidth of another component carrier then it does not apply.

NOTE 1:	For inter-band carrier aggregation with uplink assigned to two NR bands the requirements in Table 6.5.3.1.3-2 could be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur; in that case, the requirements for remaining applicable frequencies in Table 6.5.3.1.3-2 would be considered to be verified by the measurements verifying the one uplink inter-band CA spurious emission requirement.

For intra-band contiguous carrier aggregation the spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) in Table 6.5A.3.1.0-1 from the edge of the aggregated channel bandwidth. For frequencies ΔfOOB greater than FOOB as specified in Table 6.5A.3.1.0-1 the spurious emission requirements in Table 6.5.3.1.3-2 are applicable. For power class 2 intra-band contiguous carrier aggregation, the spurious emissions is measured as the sum from both UE transmit antenna connectors when UE indicates support for dualPA-Architecture IE.

Table 6.5A.3.1.0-1: Boundary between out of band and spurious emission domain for intra-band contiguous carrier aggregation

| Aggregated Channel bandwidth | OOB boundary FOOB (MHz) |
| --- | --- |
| BWChannel_CA | BWChannel_CA + 5 |

For intra-band non-contiguous carrier aggregation transmission the spurious emission requirement is defined as a composite spurious emission requirement. Composite spurious emission requirement applies to frequency ranges that are more than FOOB away from the edges of each carrier in the gap and out of the gap. Composite spurious emission requirement is defined as follows:

a)	Composite spurious emission requirement is a combination of individual sub-block spurious emission requirements

b)	In case the sub-block consist of one component carrier the sub-lock spurious emission requirement and FOOB are defined in subclause 6.5.3.1

c)	If for some frequency an individual sub-block spurious emission requirement overlaps with the general spectrum emission mask or the sub-block bandwidth of another sub-block then it does not apply

For intra-band non-contiguous carrier aggregation, the spurious emissions is measured as the sum from both UE transmit antenna connectors when UE indicates support for dualPA-Architecture IE.

For combinations of intra-band and inter-band carrier aggregation with three uplink component carriers (up to two contiguously aggregated carriers per operating band), the spurious emission requirement is defined as follows. For the NR band supporting one component carrier the requirements in Table 6.5.3.1.3-2 apply for frequency ranges that are more than FOOB (MHz) from the edges of assigned channel bandwidth as defined in Table 6.5.3.1.3-1. For the NR band supporting two contiguous component carriers the requirements in Table 6.5.3.1-2 apply for frequency ranges that are more than FOOB (MHz) from the edges of assigned aggregated channel bandwidth as defined in Table 6.5A.3.1.3-1. If for some frequency a spurious emission requirement of a single component carrier or two contiguous component carriers overlap with the spurious emission requirement or channel bandwidth of another component carrier or two contiguously aggregated carriers then it does not apply.

The normative reference for this requirement is TS 38.101-1 [2] subclause 6.5A.3.1

6.5A.3.1.1	General spurious emissions for CA (2UL CA)

6.5A.3.1.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions.

6.5A.3.1.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 2UL CA.

6.5A.3.1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause 6.5A.3.1.0.

6.5A.3.1.1.4	Test description

6.5A.3.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in Tables 6.5A.3.1.1.4.1-1 to 6.5A.3.1.1.4.1-3. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5A.3.1.1.4.1-1: Inter band CA Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  | Low range for PCC and SCC High range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg for both PCC and SCC Highest NRB_agg for both PCC and SCC |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Modulation | RB allocation (NOTE 1) |  |
|  |  |  | PCC | SCC |
| 1 | N/A | CP-OFDM QPSK | Outer_Full | Outer_Full |
| 2 |  | CP-OFDM QPSK | Edge_1RB_Left | Edge_1RB_Left |
| 3 |  | CP-OFDM QPSK | Edge_1RB_Right | Edge_1RB_Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths is specified in Table 5.5A.3.1-1 and SCS is specified in Table 5.3.5-1 for each NR band. |  |  |  |  |

Table 6.5A.3.1.1.4.1-2: Intra-band contiguous CA Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  | Low range High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg Highest NRB_agg |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Modulation | RB allocation (NOTE 1) |  |
|  |  |  | PCC | SCC |
| 1 | N/A | CP-OFDM QPSK | Outer_Full | N/A |
| 2 |  | CP-OFDM QPSK | Edge_1RB_Left | N/A |
| 3 |  | CP-OFDM QPSK | Edge_1RB_Right | N/A |
| 4 |  | CP-OFDM QPSK | Edge_1RB_Left | Edge_1RB_Right |
| 5 |  | CP-OFDM QPSK | Outer_Full | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths is specified in Table 5.5A.3.1-1 and SCS is specified in Table 5.3.5-1 for each NR band. |  |  |  |  |

Table 6.5A.3.1.1.4.1-3: Intra-band non-contiguous CA Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  | Low range and High range test frequencies as specified in tables for non-contiguous CA configuration with UL CA |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg Highest NRB_agg |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Modulation | RB allocation (NOTE 1) |  |
|  |  |  | PCC | SCC |
| 1 | N/A | CP-OFDM QPSK | Edge_1RB_Left | Edge_1RB_Left |
| 2 |  | CP-OFDM QPSK | Edge_1RB_Right | Edge_1RB_Right |
| 3 |  | CP-OFDM QPSK | Outer_Full | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths is specified in Table 5.5A.3.1-1 and SCS is specified in Table 5.3.5-1 for each NR band. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Tables 6.5A.3.1.1.4.1-1, 6.5A.3.1.1.4.1-2 and 6.5A.3.1.1.4.1-3 as appropriate.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5A.3.1.1.4.3.

6.5A.3.1.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.1.1. Message contents are defined in clause 6.5A.3.1.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause9.3).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Tables 6.5A.3.1.1.4.1-1, 6.5A.3.1.1.4.1-2 and 6.5A.3.1.1.4.1-3 as appropriate on both PCC and SCC. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.5A.3.1.1.5-1 for inter-band CA, table 6.5A.3.1.1.5-3 for intra-band contiguous CA and intra-band non-contiguous CA. The centre frequency of the filter shall be stepped in contiguous steps according to Table 6.5A.3.1.1.5-1 for inter-band CA, table 6.5A.3.1.1.5-3 for intra-band contiguous CA and intra-band non-contiguous CA.

The measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS. If the sweep count is higher than one, the trace mode shall be average. For power class 2 intra-band contiguous carrier aggregation, the spurious emissions is measured as the sum from both UE transmit antenna connectors when UE indicates support for dualPA-Architecture IE.

6.5A.3.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with following exception.

Table 6.5A.3.1.1.4.3-1: FrequencyInfoUL-SIB for inter-band CA

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 20 |  | Power class 3 and Inter-band CA |

Table 6.5A.3.1.1.4.3-2: FrequencyInfoUL-SIB for intra-band contiguous CA

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 10 |  | Power class 3 and Intra-band contiguous CA with Bandwidth class C Test ID 4 |
|  | 5 |  | Power class 3 and Intra-band contiguous CA with Bandwidth class C Test ID 5 |
|  | 8 |  | Power class 3 and Intra-band contiguous CA with Bandwidth class B Test ID 4 |
|  | 3 |  | Power class 3 and Intra-band contiguous CA with Bandwidth class B Test ID 5 |
|  | 11 |  | Power class 2 and Intra-band contiguous CA with Bandwidth class C Test ID 4 |
|  | 5 |  | Power class 3 and Intra-band contiguous CA with Bandwidth class C Test ID 5 |

6.5A.3.1.1.5	Test requirement

This clause specifies the requirements for the specified NR band for Transmitter Spurious emissions requirement with frequency range as indicated in Table 6.5A.3.1.1.5-1 for inter-band UL CA configuration and Table 6.5A.3.1.1.5-3 for intra-band contiguous and non-contiguous UL CA configuration. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

For inter-band CA, the spurious emission limits apply for the frequency ranges that are more than ΔfOOB (MHz) in Table 6.5.3.1.3-1 from the edge of the channel bandwidth. If for some frequency a spurious emission requirement of individual component carrier overlaps with the spectrum emission mask or channel bandwidth of another component carrier, then it does not apply.

For intra-band contiguous CA, the spurious emission limits apply for the frequency ranges that are more than ΔfOOB (MHz) in Table 6.5A.3.1.0-1 from the edge of the aggregated channel bandwidth.

For intra-band non-contiguous CA, the spurious emission limits apply to frequency ranges that are more than FOOB in Table 6.5.3.1.3-1 away from the edges of each carrier in the gap and out of the gap.

The measured average power of spurious emission, derived in step 6, shall not exceed the described value in Table 6.5A.3.1.1.5-1 for inter-band UL CA configuration and Table 6.5A.3.1.1.5-3 for intra-band contiguous UL CA and intra-band non-contiguous configuration.

Table 6.5A.3.1.1.5-1: General spurious emissions test requirements for inter-band UL CA

| Frequency Range |  | Maximum Level |  | Measurement Bandwidth |  | Notes |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test requirements for CA_n1A-n3A Configuration |  |  |  |  |  |  |  |  |
| 135 MHz ≤ f ≤ 270 MHz |  | -36 dBm+TT |  | 100kHz |  |  |  |  |
| 1440 MHz ≤ f ≤ 1650 MHz 2055 MHz ≤ f ≤ 2250 MHz 3630 MHz ≤ f ≤ 3765 MHz 5340 MHz ≤ f ≤ 5550 MHz 5550 MHz ≤ f ≤ 5745 MHz |  | -30 dBm+TT |  | 1MHz |  |  |  |  |
| Test requirements for CA_n1A-n5A Configuration |  |  |  |  |  |  |  |  |
| 222 MHz ≤ f ≤ 332 MHz |  | -36 dBm+TT |  | 100kHz |  |  |  |  |
| 1071 MHz ≤ f ≤ 1156 MHz 2744 MHz ≤ f ≤ 2829 MHz 2991 MHz ≤ f ≤ 3136 MHz 3568 MHz ≤ f ≤ 3678 MHz 4664 MHz ≤ f ≤ 4809 MHz |  | -30 dBm+TT |  | 1MHz |  |  |  |  |
| Test requirements for CA_n1A-n8A Configuration |  |  |  |  |  |  |  |  |
| 90 MHz ≤ f ≤ 220 MHz |  | -36 dBm+TT |  | 100kHz |  |  |  |  |
| 1005 MHz ≤ f ≤ 1100 MHz 2800 MHz ≤ f ≤ 2895 MHz 2925 MHz ≤ f ≤ 3080 MHz 3680 MHz ≤ f ≤ 3810 MHz 4720 MHz ≤ f ≤ 4875 MHz |  | -30 dBm+TT |  | 1MHz |  |  |  |  |
| Test requirements for CA_n1A-n28A Configuration |  |  |  |  |  |  |  |  |
| 424 MHz ≤ f ≤ 574 MHz |  | -36 dBm+TT |  | 100kHz |  |  |  |  |
| 1172 MHz ≤ f ≤ 1277 MHz 2623 MHz ≤ f ≤ 2728 MHz 3092 MHz ≤ f ≤ 3257 MHz 3326 MHz ≤ f ≤ 3476 MHz 4543 MHz ≤ f ≤ 4708 MHz |  | -30 dBm+TT |  | 1MHz |  |  |  |  |
| Test requirements for CA_n1A-n78A Configuration |  |  |  |  |  |  |  |
| 40 MHz ≤ f ≤ 660 MHz |  | -36 dBm+TT |  | 100kHz |  |  |  |
| 1320 MHz ≤ f ≤ 1880 MHz 4620 MHz ≤ f ≤ 5780 MHz 7140 MHz ≤ f ≤ 7760 MHz 8520 MHz ≤ f ≤ 9580 MHz |  | -30 dBm+TT |  | 1MHz |  |  |  |
| Test requirements for CA_n1A-n79A Configuration |  |  |  |  |  |  |  |  |
| 440 MHz ≤ f ≤ 1000 MHz |  | -36 dBm+TT |  | 100kHz |  |  |  |  |
| 1000 MHz ≤ f ≤ 1160 MHz 2420 MHz ≤ f ≤ 3080 MHz 6320 MHz ≤ f ≤ 8080 MHz 8240 MHz ≤ f ≤ 8960 MHz 10720 MHz ≤ f ≤ 11980 MHz |  | -30 dBm+TT |  | 1MHz |  |  |  |  |
| Test requirements for CA_n2A-n5A Configuration |  |  |  |  |  |  |  |  |
| 152 MHz ≤ f ≤ 262 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |  |
| 1001 MHz ≤ f ≤ 1086 MHz 2674 MHz ≤ f ≤ 2759 MHz 2851 MHz ≤ f ≤ 2996 MHz 3498 MHz ≤ f ≤ 3608 MHz 4524 MHz ≤ f ≤ 4669 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |  |
| Test requirements for CA_n2A-n14A Configuration |  |  |  |  |  |  |  |  |
| 254 MHz ≤ f ≤ 334 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |  |
| 1052 MHz ≤ f ≤ 1122 MHz 2638 MHz ≤ f ≤ 2708 MHz 2902 MHz ≤ f ≤ 3032 MHz 3426 MHz ≤ f ≤ 3506 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |  |
| Test requirements for CA_n2A-n30A Configuration |  |  |  |  |  |  |  |  |
| 395 MHz ≤ f ≤ 465 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |  |
| 1385 MHz ≤ f ≤ 1515 MHz 2700 MHz ≤ f ≤ 2780 MHz 4155 MHz ≤ f ≤ 4225 MHz 6005 MHz ≤ f ≤ 6135 MHz 6460 MHz ≤ f ≤ 6540 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |  |
| Test requirements for CA_n2A-n48A Configuration |  |  |  |  |  |  |  |  |
| 9 kHz ≤ f < 150 kHz |  | -36 dBm |  | 1 kHz |  |  |  |  |
| 150 kHz ≤ f < 30 MHz |  | -36 dBm |  | 10 kHz |  |  |  |  |
| 30 MHz ≤ f < 270 MHz |  | -36 dBm |  | 100 kHz |  |  |  |  |
| 1640 MHz ≤ f ≤ 1850 MHz 5190 MHz ≤ f ≤ 5610 MHz 7250 MHz ≤ f ≤ 7520 MHz 8950 MHz ≤ f ≤ 9310 MHz |  | -30 dBm |  | 1 MHz |  |  |  |  |
| Test requirements for CA_n2A-n77A Configuration |  |  |  |  |  |  |  |  |
| 500 MHz ≤ f ≤ 520 MHz |  | -36 dBm +TT |  | 100 kHz |  |  |  |  |
| 1390 MHz ≤ f ≤ 2350 MHz 4690 MHz ≤ f ≤ 6550 MHz 7000 MHz ≤ f ≤ 8020 MHz 8450 MHz ≤ f ≤ 10310 MHz |  | -30 dBm +TT |  | 1 MHz |  |  |  |  |
| Test requirements for CA_n3A-n5A Configuration |  |  |  |  |  |  |  |  |
| 12 MHz ≤ f < 30 MHz |  | -36 dBm+TT |  | 10 kHz |  |  |  |  |
| 30 MHz ≤ f ≤ 137 MHz 861 MHz ≤ f ≤ 961 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |  |
| 2534 MHz ≤ f ≤ 2634 MHz 2571 MHz ≤ f ≤ 2746 MHz 3358 MHz ≤ f ≤ 3483 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |  |
| Test requirements for CA_n3A-n8A Configuration |  |  |  |  |  |  |  |  |
| 25 MHz ≤ f <30 MHz |  | -36 dBm |  | 10kHz |  |  |  |  |
| 30 MHz ≤ f ≤ 120 MHz 795 MHz ≤ f ≤ 905 MHz |  | -36 dBm |  | 100kHz |  |  |  |  |
| 2505 MHz ≤ f ≤ 2700 MHz 3470 MHz ≤ f ≤ 3615 MHz 4300 MHz ≤ f ≤ 4485 MHz |  | -30 dBm |  | 1MHz |  |  |  |  |
| Test requirements for CA_n3A-n28A Configuration |  |  |  |  |  |  |  |
| 214 MHz ≤ f ≤ 379 MHz 962 MHz ≤ f ≤ 1000 MHz |  | -36 dBm +TT |  | 100 kHz |  |  |  |
| 1000 MHz ≤ f ≤ 1082 MHz 2413 MHz ≤ f ≤ 2533 MHz 2672 MHz ≤ f ≤ 2867 MHz 3116 MHz ≤ f ≤ 3281 MHz 4123 MHz ≤ f ≤ 4318 MHz |  | -30 dBm +TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n3A-n40A Configuration |  |  |  |  |  |  |  |
| 515 MHz ≤ f ≤ 690 MHz |  | -36 dBm +TT |  | 100 kHz |  |  |  |
| 1020 MHz ≤ f ≤ 1270 MHz 2815 MHz ≤ f ≤ 3090 MHz 4010 MHz ≤ f ≤ 4185 MHz 5720 MHz ≤ f ≤ 5970 MHz 6310 MHz ≤ f ≤ 6585 MHz |  | -30 dBm +TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n3A-n41A Configuration |  |  |  |  |  |  |  |
| 711 MHz ≤ f ≤ 1000 MHz |  | -36 dBm +TT |  | 100 kHz |  |  |  |
| 1000 MHz ≤ f ≤ 1074 MHz 3207 MHz ≤ f ≤ 3670 MHz 4206 MHz ≤ f ≤ 4475 MHz 5916 MHz ≤ f ≤ 6260 MHz 6702 MHz ≤ f ≤ 7165 MHz |  | -30 dBm |  | 1MHz |  | To be tested with NS_01 signalled for band n41 |  |
|  |  | -25 dBm |  | 1MHz |  | To be tested with NS_04 signalled for band n41 |  |
| Test requirements for CA_n3A-n77A Configuration |  |  |  |  |  |  |  |  |
| 270 MHz ≤ f ≤ 780 MHz |  | -36 dBm +TT |  | 100 kHz |  |  |  |  |
| 1515 MHz ≤ f ≤ 2490 MHz 4815 MHz ≤ f ≤ 6690 MHz 6720 MHz ≤ f ≤ 7770 MHz 8310 MHz ≤ f ≤ 10185 MHz |  | -30 dBm +TT |  | 1 MHz |  |  |  |  |
| Test requirements for CA_n3A-n78A Configuration |  |  |  |  |  |  |  |  |
| 270 MHz ≤ f ≤ 380 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |  |
| 1515MHz ≤ f ≤ 2090 MHz 4815 MHz ≤ f ≤ 5890 MHz 6720 MHz ≤ f ≤ 7370 MHz 8310 MHz ≤ f ≤ 9385 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |  |
| Test requirements for CA_n5A-n30A Configuration |  |  |  |  |  |  |  |
| 607 MHz ≤ f ≤ 667 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 1456 MHz ≤ f ≤ 1491 MHz 3129 MHz ≤ f ≤ 3164 MHz 3761 MHz ≤ f ≤ 3806 MHz 3953 MHz ≤ f ≤ 4013 MHz 5434 MHz ≤ f ≤ 5479 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n5A-n48A Configuration |  |  |  |  |  |  |  |  |
| 1852 MHz ≤ f ≤ 2052 MHz 2701 MHz ≤ f ≤ 2876 MHz 4374 MHz ≤ f ≤ 4549 MHz 5198 MHz ≤ f ≤ 5398 MHz 6251 MHz ≤ f ≤ 6576 MHz 7924 MHz ≤ f ≤ 8249 MHz |  | -30 dBm |  | 1 MHz |  |  |  |  |
| Test requirements for CA_n5A-n66A Configuration |  |  |  |  |  |  |  |  |
| 12 MHz ≤ f ≤ 30 MHz |  | -36 dBm+TT |  | 10 kHz |  |  |  |  |
| 30 MHz ≤ f ≤ 132 MHz 861 MHz ≤ f ≤ 956 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |  |
| 2534 MHz ≤ f ≤ 2736 MHz 3358 MHz ≤ f ≤ 3478 MHz 4244 MHz ≤ f ≤ 4409 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |  |
| Test requirements for CA_n5A-n77A Configuration |  |  |  |  |  |  |  |  |
| 1602 MHz ≤ f ≤ 3376 MHz 4124 MHz ≤ f ≤ 5898 MHz 5751 MHz ≤ f ≤ 9249 MHz |  | -30 dBm +TT |  | 1 MHz |  |  |  |  |
| Test requirements for CA_n5A-n78A Configuration |  |  |  |  |  |  |  |  |
| 1602 MHz ≤ f ≤ 2152 MHz 2451 MHz ≤ f ≤ 2976 MHz 4124 MHz ≤ f ≤ 4649 MHz 4948 MHz ≤ f ≤ 5498 MHz 5751 MHz ≤ f ≤ 6776 MHz 7424 MHz ≤ f ≤ 8449 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |  |
| Test requirements for CA_n8A-n78A Configuration |  |  |  |  |  |  |  |  |
| 1470 MHz ≤ f ≤ 2040 MHz 2385 MHz ≤ f ≤ 2920 MHz 4180 MHz ≤ f ≤4715 MHz 5060 MHz ≤ f ≤5630 MHz 5685 MHz ≤ f ≤6720 MHz 7480 MHz ≤ f ≤8515 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |  |
| Test requirements for CA_n14A-n30A Configuration |  |  |  |  |  |  |  |  |  |
| 709 MHz ≤ f ≤ 739 MHz |  |  | -36 dBm+TT |  | 100 kHz |  |  |  |  |
| 1507 MHz ≤ f ≤ 1527 MHz 2093 MHz ≤ f ≤ 3113 MHz 3812 MHz ≤ f ≤ 3842 MHz 3881 MHz ≤ f ≤ 3911 MHz |  |  | -30 dBm+TT |  | 1 MHz |  |  |  |  |
| Test requirements for CA_n14A-n66A Configuration |  |  |  |  |  |  |  |  |  |
| 114 MHz ≤ f ≤ 204 MHz 912 MHz ≤ f ≤ 992 MHz |  |  | -36 dBm+TT |  | 100 kHz |  |  |  |  |
| 2498 MHz ≤ f ≤ 2578 MHz 2622 MHz ≤ f ≤ 2772 MHz 3286 MHz ≤ f ≤ 3376 MHz 4208 MHz ≤ f ≤ 4358 MHz |  |  | -30 dBm+TT |  | 1 MHz |  |  |  |  |
| Test requirements for CA_n14A-n77A Configuration |  |  |  |  |  |  |  |  |  |
| 1704 MHz ≤ f ≤ 3412 MHz 4088 MHz ≤ f ≤ 5796 MHz 5808 MHz ≤ f ≤ 9198 MHz |  |  | -30 dBm +TT |  | 1 MHz |  |  |  |  |
| Test requirements for CA_n20A-n78A Configuration |  |  |  |  |  |  |  |  |
| 1576 MHz ≤ f ≤ 2136 MHz 2438 MHz ≤ f ≤ 2968 MHz 4132 MHz ≤ f ≤ 4662 MHz 4964 MHz ≤ f ≤ 5524 MHz 5738 MHz ≤ f ≤ 6768 MHz 7432 MHz ≤ f ≤ 8462 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |  |
| Test requirements for CA_n24A-n41A Configuration |  |  |  |  |  |  |  |  |
| 563 MHz ≤ f ≤ 825 MHz 835.5 MHz ≤ f ≤ 1000 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |  |
| 1000 MHz ≤ f ≤ 1063.5 MHz 3331.5 MHz ≤ f ≤ 3753.5 MHz 4122.5 MHz ≤ f ≤ 4350.5 MHz 5749 MHz ≤ f ≤ 6011 MHz 6618.5 MHz ≤ f ≤ 7040.5 MHz |  | -25 dBm+TT |  | 1 MHz |  | NS-01 signalled for n41 |  |  |
|  |  | -30 dBm+TT |  | 1 MHz |  | NS-04 signalled for n41 |  |  |
| Test requirements for CA_n24A-n48A Configuration |  |  |  |  |  |  |  |  |
| 229 MHz ≤ f ≤ 447 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |  |
| 1889.5 MHz ≤ f ≤ 2073.5 MHz 5176.5 MHz ≤ f ≤ 5360.5 MHz 5439.5 MHz ≤ f ≤ 5773.5 MHz 6803 MHz ≤ f ≤ 7021 MHz 8726.5 MHz ≤ f ≤ 9060.5 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |  |
| Test requirements for CA_n24A-n77A Configuration |  |  |  |  |  |  |  |  |
| 21 MHz ≤ f < 30 MHz |  | -36 dBm+TT |  | 10 kHz |  |  |  |  |
| 30 MHz ≤ f < 947 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |  |
| 1639.5 MHz ≤ f ≤ 2573.5 MHz 4926.5 MHz ≤ f ≤ 7521 MHz 8226.5 MHz ≤ f ≤ 10060.5 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |  |
| Test requirements for CA_n25A-n66A Configuration. 70 MHz ≤ f ≤ 205 MHz; -36 dBm+TT; 100 kHz. 1505 MHz ≤ f ≤ 1710 MHz 1920 MHz ≤ f ≤ 2120 MHz 3560 MHz ≤ f ≤ 3695 MHz 5270 MHz ≤ f ≤ 5610 MHz; -30 dBm+TT; 1 MHz. Test requirements for CA_n25A-n77A Configuration. 500 MHz ≤ f ≤ 530 MHz; -36 dBm+TT; 100 kHz. 1385 MHz ≤ f ≤ 2350 MHz 4685 MHz ≤ f ≤ 6550 MHz 7000 MHz ≤ f ≤ 8030 MHz 8450 MHz ≤ f ≤ 10315 MHz; -30 dBm+TT; 1 MHz. Test requirements for CA_n25A-n78A Configuration. 100 MHz ≤ f ≤ 530 MHz; -36 dBm+TT; 100 kHz. 1385 MHz ≤ f ≤ 1950 MHz 4685 MHz ≤ f ≤ 5750 MHz 7000 MHz ≤ f ≤ 7630 MHz 8450 MHz ≤ f ≤ 9515 MHz; -30 dBm+TT; 1 MHz. |
| Test requirements for CA_n26A-n66A Configuration |  |  |  |  |  |  |  |  |
| 12 MHz ≤ f < 30 MHz |  | -36 dBm+TT |  | 10 kHz |  |  |  |  |
| 30 MHz ≤ f ≤ 152 MHz 861 MHz ≤ f ≤ 966 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |  |
| 2524 MHz ≤ f ≤ 2746 MHz 3338 MHz ≤ f ≤ 3478 MHz 4234 MHz ≤ f ≤ 4409 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |  |
| Test requirements for CA_n26A-n70A Configuration |  |  |  |  |  |  |  |  |
| 3 MHz ≤ f < 30 MHz |  | -36 dBm+TT |  | 10 kHz |  |  |  |  |
| 30 MHz ≤ f < 82 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |  |
| 846 MHz ≤ f ≤ 896 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |  |
| 2509 MHz ≤ f ≤ 2606 MHz 3323 MHz ≤ f ≤ 3408 MHz 4204 MHz ≤ f ≤ 4269 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |  |
| Test requirements for CA_n28A-n40A Configuration |  |  |  |  |  |  |  |  |
| 804 MHz ≤ f ≤ 994 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |  |
| 1552 MHz ≤ f ≤ 1697 MHz 3003 MHz ≤ f ≤ 3148 MHz 3706 MHz ≤ f ≤ 4097 MHz 5303 MHz ≤ f ≤ 5548 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |  |
| Test requirements for CA_n28A-n41A Configuration |  |  |  |  |  |  |  |  |
| 1000MHz ≤ f ≤ 1284 MHz 1748MHz ≤ f ≤ 1987 MHz 3199MHz ≤ f ≤ 3438 MHz 3902MHz ≤ f ≤ 4186 MHz 4244MHz ≤ f ≤ 4677 MHz 5695MHz ≤ f ≤ 6128MHz |  | -25 dBm+TT |  | 1MHz |  |  |  |  |
| Test requirements for CA_n28A-n77A Configuration |  |  |  |  |  |  |  |  |
| 1804 MHz ≤ f ≤ 3497 MHz 4003 MHz ≤ f ≤ 5696 MHz 5852 MHz ≤ f ≤ 9148 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |  |
| Test requirements for CA_n28A-n78A Configuration |  |  |  |  |  |  |  |  |
| 1804 MHz ≤ f ≤ 2394 MHz 2552 MHz ≤ f ≤ 3097 MHz 4003 MHz ≤ f ≤ 4548 MHz 4706 MHz ≤ f ≤ 5296 MHz 5852 MHz ≤ f ≤ 6897 MHz 7303 MHz ≤ f ≤ 8348 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |  |
| Test requirements for CA_n28A-n79A Configuration |  |  |  |  |  |  |  |  |
| 2904 MHz ≤ f ≤ 3594 MHz 3652 MHz ≤ f ≤ 4297 MHz 5103 MHz ≤ f ≤ 5748 MHz 5806 MHz ≤ f ≤ 6496 MHz 8052 MHz ≤ f ≤ 9297 MHz 9503 MHz ≤ f ≤ 10748 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |  |
| Test requirements for CA_n30A-n66A Configuration |  |  |  |  |  |  |  |
| 525 MHz ≤ f ≤ 605 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 1105 MHz ≤ f ≤ 1255 MHz 2830 MHz ≤ f ≤ 2920 MHz 4015 MHz ≤ f ≤ 4095 MHz 5725 MHz ≤ f ≤ 5875 MHz 6320 MHz ≤ f ≤ 6410 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n30A-n77A Configuration |  |  |  |  |  |  |  |
| 410 MHz ≤ f ≤ 1000 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 1000 MHz ≤ f ≤ 1895 MHz 4285 MHz ≤ f ≤ 6515 MHz 7910 MHz ≤ f ≤ 8830 MHz 8905 MHz ≤ f ≤ 10715 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n39A-n41A Configuration |  |  |  |  |  |  |  |  |
| 576 MHz ≤ f ≤ 810 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |  |
| 1070 MHz ≤ f ≤ 1344 MHz 3072 MHz ≤ f ≤ 3500 MHz 4376 MHz ≤ f ≤ 4610 MHz 6256 MHz ≤ f ≤ 6530 MHz 6872 MHz ≤f ≤7300 MHz |  | -30 dBm+TT |  | 1 MHz |  | 4 |  |  |
|  |  | -25 dBm +TT |  | 1MHz |  | 3 |  |  |
| Test requirements for CA_n40A-n41A Configuration |  |  |  |  |  |  |  |  |
| 96 MHz ≤ f ≤390 MHz |  | -36 dBm+TT |  | 100kHz |  |  |  |  |
| 1910 MHz <= f <= 2304 MHz 2592 MHz <= f <= 3080 MHz 4796 MHz <= f <= 5090 MHz 7096 MHz <= f <= 7780 MHz |  | -25dBm+TT |  | 1MHz |  |  |  |  |
| Test requirements for CA_n40A-n77A Configuration |  |  |  |  |  |  |  |  |
| 400 MHz ≤ f ≤ 1000 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |  |
| 1000 MHz ≤ f ≤ 1900 MHz 4200 MHz ≤ f ≤ 6600 MHz 7900 MHz ≤ f ≤ 10800 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |  |
| Test requirements for CA_n41A-n77A Configuration |  |  |  |  |  |  |  |  |
| 610 MHz ≤ f ≤ 1000 MHz |  | -36 dBm+TT |  | 100kHz |  |  |  |  |
| 1000 MHz ≤ f ≤ 2080 MHz 3910 MHz ≤ f ≤ 6890 MHz 8292 MHz ≤ f ≤ 11090 MHz |  | -30 dBm+TT |  | 1MHz |  |  |  |  |
| Test requirements for CA_n41A-n79A Configuration |  |  |  |  |  |  |  |  |
| 8 MHz ≤ f ≤30 MHz |  | -36 dBm+TT |  | 10kHz |  |  |  |  |
| 30 MHz ≤ f ≤980 MHz |  | -36 dBm+TT |  | 100kHz |  |  |  |  |
| 1710MHz ≤ f ≤ 2504MHz |  | -30 dBm+TT |  | 1MHz |  |  |  |  |
| 6110MHz ≤ f ≤ 7690MHz |  |  |  |  |  |  |  |  |
| 9392MHz ≤ f ≤ 10380MHz |  |  |  |  |  |  |  |  |
| 11296MHz ≤ f ≤ 12690MHz |  |  |  |  |  |  |  |  |
| Test requirements for CA_n48A-n66A Configuration |  |  |  |  |  |  |  |  |
| 10 MHz ≤ f < 30 MHz |  | -36 dBm+TT |  | 10 kHz |  |  |  |  |
| 30 MHz ≤ f < 280 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |  |
| 1770 MHz ≤ f ≤ 1990 MHz 5260 MHz ≤ f ≤ 5690 MHz 6970 MHz ≤ f ≤ 7260 MHz 8810 MHz ≤ f ≤ 9180 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |  |
| Test requirements for CA_n48A-n70A Configuration |  |  |  |  |  |  |  |  |
| 130 MHz ≤ f ≤ 310 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |  |
| 1840 MHz ≤ f ≤ 2005 MHz 5245 MHz ≤ f ≤ 5705 MHz 6940 MHz ≤ f ≤ 7120 MHz 8795 MHz ≤ f ≤ 9110 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |  |
| Test requirements for CA_n48A-n71A Configuration |  |  |  |  |  |  |  |  |
| 2154 MHz ≤ f ≤ 2374 MHz 2852 MHz ≤ f ≤ 3037 MHz 4213 MHz ≤ f ≤ 4398 MHz 4876 MHz ≤ f ≤ 5096 MHz 6402 MHz ≤ f ≤ 6737 MHz 7763 MHz ≤ f ≤ 8098 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |  |
| Test requirements for CA_n66A-n71A Configuration |  |  |  |  |  |  |  |  |
| 314 MHz ≤ f < 454 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |  |
| 1012 MHz ≤ f ≤ 1117 MHz 2373 MHz ≤ f ≤ 2478 MHz 2722 MHz ≤ f ≤ 2897 MHz 3036 MHz ≤ f ≤ 3176 MHz 4083 MHz ≤ f ≤ 4258 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |  |
| Test requirements for CA_n66A-n77A Configuration |  |  |  |  |  |  |  |  |
| 260 MHz ≤ f ≤ 780 MHz |  | -36 dBm +TT |  | 100 kHz |  |  |  |  |
| 1520 MHz ≤ f ≤ 2490 MHz 4820 MHz ≤ f ≤ 6690 MHz 6720 MHz ≤ f ≤ 7760 MHz 8310 MHz ≤ f ≤ 10180 MHz |  | -30 dBm +TT |  | 1 MHz |  |  |  |  |
| Test requirements for CA_n66A-n78A Configuration |  |  |  |  |  |  |  |
| 260 MHz ≤ f ≤ 380 MHz |  | -36 dBm +TT |  | 100 kHz |  |  |  |
| 1520 MHz ≤ f ≤ 2090 MHz 4820 MHz ≤ f ≤ 5890 MHz 6720 MHz ≤ f ≤ 7360 MHz 8310 MHz ≤ f ≤ 9380 MHz |  | -30 dBm +TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n70A-n71A Configuration |  |  |  |  |  |  |  |  |
| 299 MHz ≤ f < 384 MHz 997 MHz ≤ f < 1000 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |  |
| 1000 MHz ≤ f ≤ 1047 MHz 2358 MHz ≤ f ≤ 2408 MHz 2692 MHz ≤ f ≤ 2757 MHz 3021 MHz ≤ f ≤ 3106 MHz 4053 MHz ≤ f ≤ 4118 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |  |
| Test requirements for CA_n71A-n77A Configuration |  |  |  |  |  |  |  |  |
| 1904 MHz ≤ f ≤ 3537 MHz 3963 MHz ≤ f ≤ 5596 MHz 5902 MHz ≤ f ≤ 9098 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |  |
| Test requirements for CA_n71A-n78A Configuration |  |  |  |  |  |  |  |  |
| 1904 MHz ≤ f ≤ 2474 MHz 2602 MHz ≤ f ≤ 3137 MHz 3963 MHz ≤ f ≤ 4498 MHz 4626 MHz ≤ f ≤ 5196 MHz 5902 MHz ≤ f ≤ 6937 MHz 7263 MHz ≤ f ≤ 8298 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |  |
| NOTE 1: Applies for Band for which the upper frequency edge of the UL Band is greater than 2.55 GHz and less than or equal to 5.2 GHz NOTE 2: Applies for Band that the upper frequency edge of the UL Band more than 5.2 GHz NOTE 3: Applies for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in clause 5.2B of TS 38.101-3 [4] when NS_04 is signalled. NOTE 4: Does not apply for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in subclause 5.2B of TS 38.101-3 [4] when NS_04 is signalled. |  |  |  |  |  |  |  |  |

Table 6.5A.3.1.1.5-2: Test Tolerance for General spurious emissions

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0 | 0 |
| 40MHz < BW ≤ 100MHz | 0 | 0 |

Table 6.5A.3.1.1.5-3: General spurious emissions test requirements for intra-band contiguous and non-contiguous UL CA

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 9 kHz f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz | 4 |
|  | -25 dBm | 1 MHz | 3 |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 dBm | 1 MHz | 1 |
| 12.75 GHz < f < 26 GHz | -30 dBm | 1 MHz | 2 |
| NOTE 1: Applies for Band for which the upper frequency edge of the UL Band is greater than 2.55 GHz and less than or equal to 5.2 GHz NOTE 2: Applies for Band that the upper frequency edge of the UL Band more than 5.2 GHz NOTE 3: Applies for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in sub-clause 5.2B of [4] when NS_04 is signalled. NOTE 4: Does not apply for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in subclause 5.2B of TS 38.101-3 [4] when NS_04 is signalled. |  |  |  |

6.5A.3.2	Spurious emission for UE co-existence

6.5A.3.2.0	Minimum conformance requirements

6.5A.3.2.0.0	General

Unless otherwise stated, the spurious emission for UE co-existence apply for the frequency ranges that are more than FOOB (MHz) in Table 6.5.3.1.3-1 from the edge of the channel bandwidth configured on each component carrier.

6.5A.3.2.0.1	Spurious emissions for UE co-existence for intra-band contiguous CA

This clause specifies the requirements for the specified intra-band contiguous carrier aggregation configurations for coexistence with protected bands, the requirements in Tables 6.5A.3.2.0.1-1 to Table 6.5A.3.2.0.1-3 apply. For power class 2 intra-band contiguous carrier aggregation, the spurious emissions is measured as the sum from both UE transmit antenna connectors when UE indicates support for dualPA-Architecture IE.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

Table 6.5A.3.2.0.1-1: Requirements for uplink intra-band contiguous carrier aggregation (Rel-16)

| NR CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n7 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 20, 22, 26, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 42, 44, 45, 48, 50, 51, 52, 65, 66, 70, 71, 73, 74, 85, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2, 4 |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 6 |
|  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 5, 6 |
| CA_n48 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n77 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 5 |
| CA_n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 5 |
| CA_n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 39, 40, 41, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 5 |
| NOTE 1: Void. NOTE 2: Void. NOTE 3: Void. NOTE 4: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. NOTE 6: This requirement applies when the NR carrier is confined within 2545 – 2575 MHz or 2595 – 2645 MHz and the channel bandwidth is 10 or 20 MHz. |  |  |  |  |  |  |  |

Table 6.5A.3.2.0.1-2: Requirements for uplink intra-band contiguous carrier aggregation (Rel-17)

| NR CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n7 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 20, 22, 26, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85, 103 NR Band n77, n78, n100 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 42, 44, 45, 48, 50, 51, 52, 65, 66, 70, 71, 73, 74, 85, 103 NR Band n77, n78, n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2, 4 |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 6 |
|  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 5, 6 |
| CA_n48 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n77 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 5 |
| CA_n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 5 |
| CA_n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 39, 40, 41, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 5 |
| NOTE 1: Void NOTE 2: Void NOTE 3: Void NOTE 4: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. NOTE 6: This requirement applies when the NR carrier is confined within 2545 – 2575 MHz or 2595 – 2645 MHz and the channel bandwidth is 10 or 20 MHz. NOTE 7: As exceptions, for 90 and 100 MHz aggregated bandwidth, -40 dBm/MHz is applicable in the frequency range of 2496 – 2505 MHz. |  |  |  |  |  |  |  |

Table 6.5A.3.2.0.1-3: Requirements for uplink intra-band contiguous carrier aggregation (Rel-18)

| NR CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n7 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 20, 22, 26, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85, 103 NR Band n77, n78, n100 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 42, 44, 45, 48, 50, 51, 52, 54, 65, 66, 70, 71, 73, 74, 85, 103 NR Band n77, n78, n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2, 4 |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 6 |
|  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 5, 6 |
| CA_n48 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 54, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n77 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 54, 65, n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 5 |
| CA_n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 5 |
| CA_n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 39, 40, 41, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 5 |
| NOTE 1: Void NOTE 2: Void NOTE 3: Void NOTE 4: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. NOTE 6: This requirement applies when the NR carrier is confined within 2545 – 2575 MHz or 2595 – 2645 MHz and the channel bandwidth is 10 or 20 MHz. NOTE 7: As exceptions, for 90 and 100 MHz aggregated bandwidth, -40 dBm/MHz is applicable in the frequency range of 2496 – 2505 MHz. |  |  |  |  |  |  |  |

6.5A.3.2.0.2	Void

6.5A.3.2.0.2a	Spurious emissions for UE co-existence for Intra-band non-contiguous CA

This clause specifies the requirements for the specified intra-band non-contiguous carrier aggregation configurations for coexistence with protected bands, the requirements in Table 6.5A.3.2.0.2a-1 apply.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

Table 6.5A.3.2.0.2a-1: Requirements for uplink intra-band non-contiguous carrier aggregation

| NR CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n77 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 NR Band n101 | FDL_low | - | FDL_high | -50 | 1 |  |
| NOTE 1: FFS NOTE 2: FFS |  |  |  |  |  |  |  |

6.5A.3.2.0.3	Spurious emissions for UE co-existence for Inter-band CA

This clause specifies the additional requirements for inter-band uplink carrier aggregation configurations with the single CC uplink assigned to two NR bands for coexistence with protected bands for the specified uplink carrier aggregation configurations in Table 6.5A.3.2.0.3-1 to Table 6.5A.3.2.0.3-4. The intersection of the requirements for the individual bands specified in clause 6.5.3.2 shall also apply for the specified uplink carrier aggregation configurations. Intersection of a requirement means that both UL constituent bands have the same protected band requirement specified and if one or both protected bands have note(s) associated those note(s) also apply.

For inter-band carrier aggregation with two contiguous carriers assigned to one NR band, the requirements in subclause 6.5A.3.2.0.1 apply for that band.

For inter-band carrier aggregation with two uplink non-contiguous carrier assigned to one NR band, the spurious emissions for UE co-existence requirements in subclause 6.5A.3.2.0.2a apply for that band.

For inter-band carrier aggregation with the uplink assigned to two NR bands, the requirements in Table 6.5A.3.2.0.3-1 to 6.5A.3.2.0.3-4 apply on each component carrier with all component carriers are active.

NOTE:	For inter-band carrier aggregation with single CC uplink assigned to two NR bands, the requirements in Tables 6.5A.3.2.0.3-1 to 6.5A.3.2.0.3-4 could be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur; in that case, the requirements for remaining applicable frequencies in Tables 6.5A.3.2.0.3-1 to Table 6.5A.3.2.0.3-4 and in clause 6.5.3.2 would be considered to be verified by the measurements verifying the one uplink inter-band CA UE to UE co-existence requirements.

Table 6.5A.3.2.0.3-1: Requirements for uplink inter-band carrier aggregation (two bands) Rel-15

| NR CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n3-n78 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n8-n78 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 1: Void NOTE 2: Void NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 5: Void |  |  |  |  |  |  |  |

NOTE:	To simplify Table 6.5A.3.2.0.3-1, E-UTRA band numbers are listed for bands which are specified only for E-UTRA operation or both E-UTRA and NR operation. NR band numbers are listed for bands which are specified only for NR operation.

Table 6.5A.3.2.0.3-2: Requirements for uplink inter-band carrier aggregation (two bands) Rel-16

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (Mhz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n1-n28 | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -30 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
| CA_n3-n28 | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -30 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 | 4 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 11 |
| CA_n3-n40 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n3-n41 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n3-n77 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n3-n78 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n5-n77 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n5-n78 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 | 2 |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| CA_n8-n78 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n28-n40 | Frequency range | 758 | - | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n28-n41 | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 13 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 11 |
| CA_n40-n41 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n41-n79 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 1: Void. NOTE 2: Void. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 from the edge of the channel bandwidth. NOTE 5: Void. NOTE 6: This requirement is applicable for any channel bandwidths within the range 1920 – 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 – 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 9: Void. NOTE 10: Void. NOTE 11: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 12: Void. NOTE 13: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718 - 728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and Rbstart < 48. NOTE 14: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 15: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). |  |  |  |  |  |  |  |

Table 6.5A.3.2.0.3-3: Requirements for uplink inter-band carrier aggregation (two bands) Rel-17

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band |  | Frequency range (Mhz) |  |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n1-n28 | Frequency range |  | 470 |  | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range |  | 470 |  | - | 710 | -26.2 | 6 | 15 |
|  | Frequency range |  | 758 |  | - | 773 | -30 | 1 | 4 |
|  | Frequency range |  | 773 |  | - | 803 | -50 | 1 |  |
|  | Frequency range |  | 662 |  | - | 694 | -26.2 | 6 | 4 |
| CA_n2-n14 | Frequency range |  | 769 |  | - | 775 | -35 | 0.00625 | 4 |
|  | Frequency range |  | 799 |  | - | 805 | -35 | 0.00625 | 4 |
| CA_n3-n5 | Frequency range |  | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n3-n8 | Frequency range |  | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n3-n28 | Frequency range |  | 470 |  | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range |  | 470 |  | - | 710 | -26.2 | 6 | 15 |
|  | Frequency range |  | 758 |  | - | 773 | -30 | 1 | 4 |
|  | Frequency range |  | 773 |  | - | 803 | -50 | 1 |  |
|  | Frequency range |  | 662 |  | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range |  | 1839.9 |  | - | 1879.9 | -50 | 1 | 4 |
|  | Frequency range |  | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3, 11 |
| CA_n3-n40 | Frequency range |  | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n3-n41 | Frequency range |  | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n3-n77 | Frequency range |  | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n3-n78 | Frequency range |  | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n5-n48 | Frequency range |  | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 8 |
| CA_n5-n66 | Frequency range |  | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n5-n77 | Frequency range |  | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n5-n78 | Frequency range |  | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range |  | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range |  | 2545 |  | - | 2575 | -50 | 1 | 2 |
|  | Frequency range |  | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_n8-n78 | Frequency range |  | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n14-n30 | Frequency range |  | 769 |  | - | 775 | -35 | 0.00625 | 4 |
|  | Frequency range |  | 799 |  | - | 805 | -35 | 0.00625 | 4 |
| CA_n14-n66 | Frequency range |  | 769 |  | - | 775 | -35 | 0.00625 | 4 |
|  | Frequency range |  | 799 |  | - | 805 | -35 | 0.00625 | 4 |
| CA_n14-n77 | Frequency range |  | 769 |  | - | 775 | -35 | 0.00625 | 4,20 |
|  | Frequency range |  | 799 |  | - | 805 | -35 | 0.00625 | 4,20 |
| CA_n26-n66 | Frequency range |  | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n26-n70 | Frequency range |  | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n28-n40 | Frequency range |  | 758 |  | - | 773 | -32 | 1 | 4 |
|  | Frequency range |  | 773 |  | - | 803 | -50 | 1 |  |
|  | Frequency range |  | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n28-n41 | Frequency range |  | 470 |  | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range |  | 470 |  | - | 710 | -26.2 | 6 | 13 |
|  | Frequency range |  | 662 |  | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range |  | 758 |  | - | 773 | -32 | 1 | 4 |
|  | Frequency range |  | 773 |  | - | 803 | -50 | 1 |  |
|  | Frequency range |  | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3, 11 |
| CA_n28-n77 | Frequency range |  | 758 |  | - | 773 | -32 | 1 |  |
|  | Frequency range |  | 773 |  | - | 803 | -50 | 1 |  |
|  | Frequency range |  | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3, 11 |
| CA_n28-n78 | Frequency range |  | 758 |  | - | 773 | -32 | 1 |  |
|  | Frequency range |  | 773 |  | - | 803 | -50 | 1 |  |
|  | Frequency range |  | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3, 11 |
| CA_n28-n79 | Frequency range |  | 470 |  | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range |  | 470 |  | - | 710 | -26.2 | 6 | 13 |
|  | Frequency range |  | 662 |  | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range |  | 758 |  | - | 773 | -32 | 1 | 4 |
|  | Frequency range |  | 773 |  | - | 803 | -50 | 1 |  |
|  | Frequency range |  | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3, 10, 11 |
| CA_n40-n41 | Frequency range |  | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n40-n77 | Frequency range |  | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n41-n77 | Frequency range |  | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n41-n79 | Frequency range | 1884.5 |  | - |  | 1915.7 | -41 | 0.3 | 3 |
| NOTE 1: Void. NOTE 2: Void. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 from the edge of the channel bandwidth. NOTE 5: Void. NOTE 6: This requirement is applicable for any channel bandwidths within the range 1920 – 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 – 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 9: Void. NOTE 10: Void. NOTE 11: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 12: Void. NOTE 13: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718 - 728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and Rbstart < 48. NOTE 14: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 15: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 16: Void. NOTE 17: Void NOTE 18: Void NOTE 19: Void NOTE 20: Applicable for cases and when the lower edge of the assigned NR UL channel bandwidth frequency is greater than or equal to 1427 MHz + the channel BW assigned for 5 and 10 MHz bandwidth, and when the lower edge of the assigned NR UL channel bandwidth frequency is greater than or equal to 1440 MHz for 15 and 20 MHz bandwidth. |  |  |  |  |  |  |  |  |  |

Table 6.5A.3.2.0.3-4: Requirements for uplink inter-band carrier aggregation (two bands) Rel-18

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (Mhz) |  |  |  |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n1-n28 | Frequency range | 470 | - |  |  | 694 |  | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - |  |  | 710 |  | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - |  |  | 773 |  | -30 | 1 | 4 |
|  | Frequency range | 773 | - |  |  | 803 |  | -50 | 1 |  |
|  | Frequency range | 662 | - |  |  | 694 |  | -26.2 | 6 | 4 |
| CA_n2-n14 | Frequency range | 769 |  |  | - |  | 775 | -35 | 0.00625 | 4 |
|  | Frequency range | 799 |  |  | - |  | 805 | -35 | 0.00625 | 4 |
| CA_n3-n5 | Frequency range | 1884.5 |  |  | - |  | 1915.7 | -41 | 0.3 | 3 |
| CA_n3-n8 | Frequency range | 1884.5 |  |  | - |  | 1915.7 | -41 | 0.3 | 3 |
| CA_n3-n28 | Frequency range | 470 |  |  | - |  | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 |  |  | - |  | 710 | -26.2 | 6 | 15 |
|  | Frequency range | 758 |  |  | - |  | 773 | -30 | 1 | 4 |
|  | Frequency range | 773 |  |  | - |  | 803 | -50 | 1 |  |
|  | Frequency range | 662 |  |  | - |  | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 1839.9 |  |  | - |  | 1879.9 | -50 | 1 | 4 |
|  | Frequency range | 1884.5 |  |  | - |  | 1915.7 | -41 | 0.3 | 3, 11 |
| CA_n3-n40 | Frequency range | 1884.5 |  |  | - |  | 1915.7 | -41 | 0.3 | 3 |
| CA_n3-n41 | Frequency range | 1884.5 |  |  | - |  | 1915.7 | -41 | 0.3 | 3 |
| CA_n3-n78 | Frequency range | 1884.5 |  |  | - |  | 1915.7 | -41 | 0.3 | 3 |
| CA_n5-n48 | Frequency range | 1884.5 |  |  | - |  | 1915.7 | -41 | 0.3 | 8 |
| CA_n5-n66 | Frequency range | 1884.5 |  |  | - |  | 1915.7 | -41 | 0.3 | 3 |
| CA_n5-n77 | Frequency range | 1884.5 |  |  | - |  | 1915.7 | -41 | 0.3 | 3 |
| CA_n5-n78 | Frequency range | 945 |  |  | - |  | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  |  | - |  | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 |  |  | - |  | 2575 | -50 | 1 | 2 |
|  | Frequency range | 2595 |  |  | - |  | 2645 | -50 | 1 |  |
| CA_n8-n78 | Frequency range | 1884.5 |  |  | - |  | 1915.7 | -41 | 0.3 | 3 |
| CA_n14-n30 | Frequency range | 769 |  |  | - |  | 775 | -35 | 0.00625 | 4 |
|  | Frequency range | 799 |  |  | - |  | 805 | -35 | 0.00625 | 4 |
| CA_n14-n66 | Frequency range | 769 |  |  | - |  | 775 | -35 | 0.00625 | 4 |
|  | Frequency range | 799 |  |  | - |  | 805 | -35 | 0.00625 | 4 |
| CA_n14-n77 | Frequency range | 769 |  |  | - |  | 775 | -35 | 0.00625 | 4,20 |
|  | Frequency range | 799 |  |  | - |  | 805 | -35 | 0.00625 | 4,20 |
| CA_n26-n66 | Frequency range | 1884.5 |  |  | - |  | 1915.7 | -41 | 0.3 | 3 |
| CA_n26-n70 | Frequency range | 1884.5 |  |  | - |  | 1915.7 | -41 | 0.3 | 3 |
| CA_n28-n40 | Frequency range | 758 |  |  | - |  | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 |  |  | - |  | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  |  | - |  | 1915.7 | -41 | 0.3 | 3 |
| CA_n28-n41 | Frequency range | 470 |  |  | - |  | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 |  |  | - |  | 710 | -26.2 | 6 | 13 |
|  | Frequency range | 662 |  |  | - |  | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 758 |  |  | - |  | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 |  |  | - |  | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  |  | - |  | 1915.7 | -41 | 0.3 | 3, 11 |
| CA_n28-n78 | Frequency range | 758 |  |  | - |  | 773 | -32 | 1 |  |
|  | Frequency range | 773 |  |  | - |  | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  |  | - |  | 1915.7 | -41 | 0.3 | 3, 11 |
| CA_n40-n41 | Frequency range | 1884.5 |  | - |  | 1915.7 |  | -41 | 0.3 | 3 |
| CA_n40-n77 | Frequency range | 1884.5 |  | - |  | 1915.7 |  | -41 | 0.3 | 3 |
| CA_n41-n79 | Frequency range | 1884.5 |  | - |  | 1915.7 |  | -41 | 0.3 | 3 |
| NOTE 1: Void NOTE 2: Void NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 from the edge of the channel bandwidth. NOTE 5: Void. NOTE 6: This requirement is applicable for any channel bandwidths within the range 1920 – 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 – 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 9: Void. NOTE 10: Void. NOTE 11: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 12: Void NOTE 13: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718 - 728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and Rbstart < 48. NOTE 14: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 15: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 16: Void NOTE 17: Void NOTE 18: Void NOTE 19: Void NOTE 20: Applicable for cases and when the lower edge of the assigned NR UL channel bandwidth frequency is greater than or equal to 1427 MHz + the channel BW assigned for 5 and 10 MHz bandwidth, and when the lower edge of the assigned NR UL channel bandwidth frequency is greater than or equal to 1440 MHz for 15 and 20 MHz bandwidth. |  |  |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] subclause 6.5A.3.2.

6.5A.3.2.1	Spurious emissions for UE co-existence for CA (2UL CA)

6.5A.3.2.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to co-existing systems for the specified bands which has specific requirements in terms of transmitter spurious emissions for 2UL CA.

6.5A.3.2.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support 2UL CA.

6.5A.3.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause 6.5A.3.2.0.

6.5A.3.2.1.4	Test description

6.5A.3.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in Table 6.5A.3.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5A.3.2.1.4.1-1: Inter band CA Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  | Normal |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  |  |  |  |  | Refer to “PCC NRB@SCS”and “SCC NRB@SCS ” columns |  |  |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  |  |  |  |  | Lowest SCS for default test points. CA configuration specific test points: Refer to “PCC NRB@SCS”and “SCC NRB@SCS ” columns |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID |  | CA Configuration / NRB_agg |  |  |  |  |  |  |  |  |  | DL Allocation |  |  | UL Allocation |  |  |  |  |
|  |  | CA Configuration |  |  |  |  |  |  |  | PCC NRB @SCS | SCC NRB@SCS | CC MOD | PCC & SCC RB allocation |  | CC MOD |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |
|  |  | PCC |  |  |  | SCC |  |  |  |  |  |  | PCC | SCC |  |  |  |  |  |
|  |  | Band |  | Range |  | Band |  | Range |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for CA_n1A-n28A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | n1 |  | Low |  | n28 |  | Low |  | 25@15kHz | 25@15kHz | CP-OFDM QPSK | NA |  | CP-OFDM QPSK |  | 1@0 | 1@0 |  |
| Test Settings for CA_n2A-n14A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 |  | Mid |  | n14 |  | High |  | 106@15kHz |  | 52@15kHz | CP-OFDM QPSK | NA |  |  | CP-OFDM QPSK | 1@0 |  | 1@51 |
| 2 | n2 |  | Mid |  | n14 |  | Low |  | 106@15kHz |  | 52@15kHz | CP-OFDM QPSK | NA |  |  | CP-OFDM QPSK | 1@0 |  | 1@0 |
| Test Settings for CA_n3A-n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | n3 |  | Low |  | n8 |  | Low |  | 100@15kHz | 100@15kHz | CP-OFDM QPSK | NA |  | CP-OFDM QPSK |  | 1@99 | 1@0 |  |
| 2 |  | n3 |  | High |  | n8 |  | High |  | 100@15kHz | 100@15kHz | CP-OFDM QPSK | NA |  | CP-OFDM QPSK |  | 1@99 | 1@100 |  |
| 3 |  | n3 |  | Low |  | n8 |  | High |  | 100@15kHz | 100@15kHz | CP-OFDM QPSK | NA |  | CP-OFDM QPSK |  | 1@0 | 1@100 |  |
| Test Settings for CA_n3A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | n3 |  | Low |  | n77 |  | 3660.15MHz (NOTE 19) |  | 106@15kHz | 273@30kHz | CP-OFDM QPSK | NA |  | CP-OFDM QPSK |  | 1@0 | 1@0 |  |
| Test Settings for CA_n3A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | n3 |  | Low |  | n78 |  | High |  | 160@15kHz | 273@30kHz | CP-OFDM QPSK | NA |  | CP-OFDM QPSK |  | 1@159 | 1@272 |  |
| Test Settings for CA_n5A-n48A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | n5 |  | Low |  | n48 |  | Low |  | 106@15kHz | 106@30kHz | CP-OFDM QPSK | NA |  | CP-OFDM QPSK |  | 1@0 | 1@0 |  |
| Test Settings for CA_n5A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | n5 |  | Low |  | n77 |  | 3538,29 MHz (NOTE 11) |  | 106@15kHz | 52@15kHz | CP-OFDM QPSK | NA |  | CP-OFDM QPSK |  | 1@0 | 1@0 |  |
| Test Settings for CA_n5A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  | n5 |  | Low |  | n78 |  | Low |  | 106@15kHz | 273@30kHz | CP-OFDM QPSK | NA |  | CP-OFDM QPSK |  | 1@0 | 1@272 |  |
| 7 |  | n5 |  | Mid |  | n78 |  | Mid |  | 106@15kHz | 273@30kHz | CP-OFDM QPSK | NA |  | CP-OFDM QPSK |  | 1@100 | 1@272 |  |
| 8 |  | n5 |  | Low |  | n78 |  | 3500.01MHz (NOTE 20) |  | 106@15kHz | 273@30kHz | CP-OFDM QPSK | NA |  | CP-OFDM QPSK |  | 1@0 | 1@0 |  |
| Test Settings for CA_n8A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | n8 |  | High |  | n78 |  | High |  | 106@15kHz | 273@30kHz | CP-OFDM QPSK | NA |  | CP-OFDM QPSK |  | 1@105 | 1@35 |  |
| Test Settings for CA_n28A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | n28 |  | High |  | n41 |  | High |  | 79@15kHz | 217@30kHz | CP-OFDM QPSK | NA |  | CP-OFDM QPSK |  | 1@78 | 1@115 |  |
| Test Settings for CA_n28A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | n28 |  | Low |  | n77 |  | Low |  | 106@15kHz | 273@30kHz | CP-OFDM QPSK | NA |  | CP-OFDM QPSK |  | 1@0 | 1@0 |  |
| Test Settings for CA_n28A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | n28 |  | Low |  | n78 |  | Low |  | 106@15kHz | 273@30kHz | CP-OFDM QPSK | NA |  | CP-OFDM QPSK |  | 1@0 | 1@0 |  |
| Test Settings for CA_n40A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | n40 |  | Low |  | n41 |  | High |  | 273@30kHz | 273@30kHz | CP-OFDM QPSK | NA |  | CP-OFDM QPSK |  | 1@0 | 1@272 |  |
| Test Settings for CA_n40A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | n40 |  | Low |  | n77 |  | High |  | 51@30kHz | 273@30kHz | CP-OFDM QPSK | NA |  | CP-OFDM QPSK |  | 1@0 | 1@272 |  |
| Test Settings for CA_n41A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | n41 |  | Mid |  | n77 |  | Low |  | 273@30kHz | 273@30kHz | CP-OFDM QPSK | NA |  | CP-OFDM QPSK |  | 1@272 | 1@272 |  |
| Test Settings for CA_n41A-n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | n41 |  | High |  | n79 |  | Low |  | 273@30kHz | 273@30kHz | CP-OFDM QPSK | NA |  | CP-OFDM QPSK |  | 1@0 | 1@0 |  |
| NOTE 1: No impact on the protected bands by 2nd and 3rd order IM products caused by the NR CA band combination. NOTE 2: Void NOTE 3: Void NOTE 4: Void NOTE 5: Void NOTE 6: Void NOTE 7: Void NOTE 8: Void NOTE 9: Void NOTE 10: Void NOTE 11: Test Point for CA_n5A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3538.29MHz (NR ARFCN=635867). NOTE 12: Void NOTE 13: Void NOTE 14: Void NOTE 15: Void NOTE 16: Void NOTE 17: Void NOTE 18: Void NOTE 19: Test point for CA_n3A-n77A and Band n77 has the centre frequency at 3660.15MHz (NR ARFCN=644010). NOTE 20: Test point ID 8 for CA_n5A-n78A and Band n78 has the centre frequency at 3500.01MHz (NR ARFCN=633334). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.5A.3.2.1.4.1-2: Intra-band contiguous CA Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  | Low range, High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg, Highest NRB_agg |  |  |
| Test SCS as specified in Table 5.5A.3-1 |  | Lowest |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Modulation | RB allocation (NOTE 1) |  |
|  |  |  | PCC | SCC |
| 1 | N/A | CP-OFDM QPSK | Edge_1RB_Left | Edge_1RB_Right |
| 2 |  | CP-OFDM QPSK | Outer_Full | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths and SCS are specified in Table 5.5A.3-1. |  |  |  |  |

Table 6.5A.3.2.1.4.1-3: Intra-band non-contiguous CA Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1.1.3 for inter band CA in FR1 |  | Low range and High range test frequencies as specified in tables for non-contiguous CA configuration with UL CA |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |
| Test SCS as specified in Table 5.5A.3-1 |  | Lowest |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Modulation | RB allocation (NOTE 1) |  |
|  |  |  | PCC | SCC |
| 1 | N/A | CP-OFDM QPSK | Edge_1RB_Left | Edge_1RB_Right |
| 2 |  | CP-OFDM QPSK | Outer_Full | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths and SCS are specified in Table 5.5A.3-1. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [6] subclause4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Tables 6.5A.3.2.1.4.1-1 to 6.5A.3.2.1.4.1-3.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5A.3.2.1.4.3.

6.5A.3.2.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, and Annex C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.5A.3.2.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause9.3).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5A.3.2.1.4.1-1, Table 6.5A.3.2.1.4.1-2 or Table 6.5A.3.2.1.4.1-3 on both PCC and SCC. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.5A.3.2.1.5-1, 6.5A.3.2.1.5-3 or 6.5A.3.2.1.5-4. The centre frequency of the filter shall be stepped in contiguous steps according to Table 6.5A.3.2.1.5-1, 6.5A.3.2.1.5-3 or 6.5A.3.2.1.5-4. The measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS. If the sweep count is higher than one, the trace mode shall be average. For power class 2 intra-band contiguous carrier aggregation, the spurious emission is measured as the sum from both UE transmit antenna connectors when UE indicates support for dualPA-Architecture IE.

6.5A.3.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with following exception.

Table 6.5A.3.2.1.4.3-1: FrequencyInfoUL-SIB for inter-band CA

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 20 |  | Power class 3 and Inter-band CA |

Table 6.5A.3.2.1.4.3-2: FrequencyInfoUL-SIB for intra-band contiguous CA

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 10 |  | Power class 3 and Intra-band contiguous CA with Bandwidth class C Test ID 1 |
|  | 5 |  | Power class 3 and Intra-band contiguous CA with Bandwidth class C Test ID 2 |
|  | 8 |  | Power class 3 and Intra-band contiguous CA with Bandwidth class B Test ID 1 |
|  | 3 |  | Power class 3 and Intra-band contiguous CA with Bandwidth class B Test ID 2 |
|  | 11 |  | Power class 2 and Intra-band contiguous CA with Bandwidth class C Test ID 1 |
|  | 5 |  | Power class 2 and Intra-band contiguous CA with Bandwidth class C Test ID 2 |

Table 6.5A.3.2.1.4.3-3: FrequencyInfoUL-SIB for intra-band non-contiguous CA

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 6 |  | Power class 3 and Intra-band non-contiguous CA Test ID 1 |
|  | 6 |  | Power class 3 and Intra-band non-contiguous CA Test ID 2 |

6.5A.3.2.1.5	Test requirement

Test requirements for Spurious Emissions UE Co-existence for inter-band CA are release specific and can be found in Table 6.5A.3.2.1.5-1, 6.5A.3.2.1.5-3 and 6.5A.3.2.1.5-4. If the UE support a band combination from a later release, the requirements for this band combination are taken from the table of earliest release where requirements for this band combination are defined. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

For intra-band contiguous CA, the measured average power of spurious emission, derived in step 6, shall not exceed the described value in Tables 6.5A.3.2.1.5-3.

For intra-band non-contiguous CA, the measured average power of spurious emission, derived in step 6, shall not exceed the described value in Tables 6.5A.3.2.1.5-4.

Table 6.5A.3.2.1.5-1: Requirements for uplink inter-band carrier aggregation according to UE NR release (two bands)

| NR CA | Release | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Configuration |  | Protected Band | Frequency range (Mhz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n1-n28 | Rel-16 Rel-17 Rel-18 | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  |  | Frequency range | 470 | - | 710 | -26.2 | 6 | 15 |
| CA_n2-n14 | Rel-17 Rel-18 | Frequency range | 769 | - | 775 | -35 | 0.00625 | 4 |
|  |  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 4 |
| CA_n3-n77 | Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n3-n78 | Rel-15 Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n5-n48 | Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| CA_n5-n77 | Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n5-n78 | Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  |  | Frequency range | 2545 | - | 2575 | -50 | 1 | 2 |
|  |  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| CA_n8-n78 | Rel-15 Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n28-n41 | Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 11 |
| CA_n28-n77 | Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 11 |
| CA_n28-n78 | Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3,11 |
| CA_n40-n41 | Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n40-n77 | Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1900.0 | -41 | 0.3 | 3 |
| CA_n41-n77 | Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n41-n79 | Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n78-n79 | Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 1: Void NOTE 2: Void NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 from the edge of the channel bandwidth. NOTE 5: Void NOTE 6: Void NOTE 7: Void NOTE 8: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 9: Void. NOTE 10: Void. NOTE 11: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 12: Void NOTE 13: Void NOTE 14: Void NOTE 15: Void NOTE 16: Void NOTE 17: Void NOTE 18: Void NOTE 19: Void NOTE 20: Applicable for cases and when the lower edge of the assigned NR UL channel bandwidth frequency is greater than or equal to 1427 MHz + the channel BW assigned for 5 and 10 MHz bandwidth, and when the lower edge of the assigned NR UL channel bandwidth frequency is greater than or equal to 1440 MHz for 15 and 20 MHz bandwidth. |  |  |  |  |  |  |  |  |

Table 6.5A.3.2.1.5-1a: Void

Table 6.5A.3.2.1.5-1b: Void

Table 6.5A.3.2.1.5-2: Void

Unless otherwise stated, the spurious emission limits apply for the frequency ranges that are more than ΔfOOB (MHz) in Table 6.5.3.2.3-1 from the edge of the channel bandwidth. The spurious emission limits in Table 6.5.3.2.3-1 apply for all transmitter band configurations (NRB) and channel bandwidths for all CC combinations.

Table 6.5A.3.2.1.5-3: Requirements for uplink intra-band contiguous carrier aggregation

| NR CA combination | Release | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Protected Band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n41 | Rel-16 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 42, 44, 45, 48, 50, 51, 52, 65, 66, 70, 71, 73, 74, 85, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2, 4 |
|  |  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 6 |
|  |  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  |  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 5, 6 |
|  | Rel-17 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 42, 44, 45, 48, 50, 51, 52, 65, 66, 70, 71, 73, 74, 85, 103 NR Band n77, n78, n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2, 4 |
|  |  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 6 |
|  |  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  |  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 5, 6 |
|  | Rel-18 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 42, 44, 45, 48, 50, 51, 52, 54, 65, 66, 70, 71, 73, 74, 85, 103 NR Band n77, n78, n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2, 4 |
|  |  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 6 |
|  |  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  |  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 5, 6 |
| CA_n48 | Rel-16 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-18 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 54, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n77 | Rel-16 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 5 |
|  | Rel-17 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 5 |
|  | Rel-18 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 54, 65, n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 5 |
| CA_n78 | Rel-16 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 5 |
|  | Rel-17 Rel-18 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 5 |
| CA_n79 | Rel-16 Rel-17 Rel-18 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 39, 40, 41, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 5 |
| NOTE 1: Void. NOTE 2: Void. NOTE 3: Void. NOTE 4: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. NOTE 6: This requirement applies when the NR carrier is confined within 2545 – 2575 MHz or 2595 – 2645 MHz and the channel bandwidth is 10 or 20 MHz NOTE 7: As exceptions, for 90 and 100 MHz aggregated bandwidth, -40 dBm/MHz is applicable in the frequency range of 2496 – 2505 MHz. |  |  |  |  |  |  |  |  |

Table 6.5A.3.2.1.5-4: Requirements for uplink intra-band non-contiguous carrier aggregation

| NR CA combination | Release | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Protected Band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n77 | Rel-16 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 NR Band n100 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n78 | Rel-16 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 Rel-18 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 NR Band n101 | FDL_low | - | FDL_high | -50 | 1 |  |
| NOTE 1: FFS. NOTE 2: FFS. |  |  |  |  |  |  |  |  |

< End of Changes >