3GPP TSG-RAN5 Meeting #109	R5-256667

# Dallas, Texas, USA, 17th - 21st November 2025

Agenda Item:	6.6.6.6

Source:	Orange, MCC TF160

Title:	Discussion on the management of redundant transmission for RTT calls

Document for:	Endorsement

# 1	Context

During RAN5#107 and RAN5#108, divergences were identified regarding the content of the SDP offer for RTT calls, particularly concerning the redundancy aspect that needs to be clarified.

According to RAN5 working assumption red/1000 is optional in the initial SDP offer but Orange and TF160 do not necessarily agree and have an alternative interpretation of the specifications

# 2	Discussion

Based on our analysis and understanding of the relevant section on RTT in the 3GPP specification TS 26.114 [1]:

7.4.4       Real-time text

The following RTP payload format shall be used:

-    T.140 text conversation RTP payload format according to RFC 4103 [31] including the updates from RFC 9071 [185] when the negotiation for support of multiparty real-time text is successful.

Real-time text shall be the only payload type in its RTP stream because the RTP sequence numbers are used for loss detection and recovery. The redundant transmission format shall be used for keeping the effect of packet loss low.

9.4       Text

Redundant transmission provided by the RTP payload format as described in RFC 4103 [31] shall be supported. When the negotiation for support of multiparty real-time text is successful, redundant transmission provided by the RTP payload format as described in IETF RFC 9071 [185] shall also be supported.

Observation 2-1: Section 7.4.4 indicates that the 'redundant transmission format' is mandatory

Some complements are also added in RFC 4103 [2]

4.  Protection against Loss of Data

Consideration must be devoted to keeping loss of text due to packet loss within acceptable limits.

The default method that MUST be used, when no other method is explicitly selected, is redundancy in accordance with RFC 2198 [3]. When this method is used, the original text and two redundant generations SHOULD be transmitted if the application or end-to-end conditions do not call for other levels of redundancy to be used

and in RFC 5194 [4]:

6.2.2.  Transport

The redundancy method of RFC 4103 [2] SHOULD be used to significantly increase the reliability of the real-time text transmission.  A redundancy level using 2 generations gives very reliable results and is therefore strongly RECOMMENDED.

Observation 2-2: This confirms that the 'redundant transmission format' is a mandatory requirement in the absence of any explicitly specified alternative method.

# 3	Proposal

Proposition 3-1 Based on Observations 2-1 and 2-2, we propose to complete the attributes for media text to reflect that redundancy is a mandatory requirement for RTT calls

In TS 34.229-1 : test case 17.19 and clause C54

In TS 34.229-5 : test cases 7.35, 7.36 and clause A29

# 4	Reference

[1] 3GPP TS 26.114: 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; IP Multimedia Subsystem (IMS); Multimedia Telephony;Media handling and interaction (Release 19)

[2] IETF RFC 4103 (2005): "RTP Payload for Text Conversation", G. Hellstrom and P. Jones

[3] IETF RFC 2198 (1997): "RTP Payload for Redundant Audio Data", C. Perkins,I. Kouvelas
O. Hodson,V. Hardman,M. Handley,J.C. Bolot,A. Vega-Garcia and S. Fosse-Parisis

[4] IETF RFC 5194 (2008): "Framework for Real-Time Text over IP Using the Session Initiation Protocol (SIP)", A. van Wijk and  G. Gybels