3GPP TSG-RAN WG5 Meeting #109	R5-256703

DALLAS, USA, 17th – 21st Nov, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3610 | rev | 1 | Current version: | 19.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to SL-ResourcePool default configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In SL resouce pool, symbols available for sidelink are 1 to 12 (i.e. all, except the first and last one). PSCCH is transmitted in symbols 1 and 2. PSSCH DMRS is transmitted in symbols 3 and 10. This leave symbols 4 to 9 (six symbols) to transmit SCI 2-D and SL-PRS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed numSym-SL-PRS-2ndStageSCI-r18 to 2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The release 18 positioning work item will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Table 4.6.6-25 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

–	SL-ResourcePool

Table 4.6.6-25: SL-ResourcePool

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |  |
| sl-PSCCH-Config-r16 CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| sl-TimeResourcePSCCH-r16 |  | n2 |  |  |
| sl-FreqResourcePSCCH-r16 |  | n10 |  |  |
| sl-DMRS-ScrambleID-r16 |  | 0 |  |  |
| sl-NumReservedBits-r16 |  | 2 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-PSSCH-Config-r16 CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| sl-PSSCH-DMRS-TimePatternList-r16 SEQUENCE (SIZE (1..3)) OF INTEGER (2..4) { |  | 2 entries |  |  |
| INTEGER[1] |  | 2 | entry 1 |  |
| INTEGER[2] |  | 4 | entry 2 |  |
| } |  |  |  |  |
| sl-BetaOffsets2ndSCI-r16 SEQUENCE (SIZE (4)) OF SL-BetaOffsets-r16 { |  | 4 entries |  |  |
| SL-BetaOffsets-r16[k, k=1..4] |  | 1 | entry k |  |
| } |  |  |  |  |
| sl-Scaling-r16 |  | f1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-PSFCH-Config-r16 |  | Not present |  |  |
| sl-PSFCH-Config-r16 CHOICE { |  |  |  | SL_HARQ |
| setup SEQUENCE { |  |  |  |  |
| sl-PSFCH-Period-r16 |  | sl4 |  |  |
| sl-PSFCH-RB-Set-r16 |  | bitstring of length n, The leftmost p*floor(m/p)*floor(n/m) bits are set to "1" and the rest are set to "0" | Note 1, Note 2, Note 3 |  |
| sl-NumMuxCS-Pair-r16 |  | n2 |  |  |
| sl-MinTimeGapPSFCH-r16 |  | sl2 |  |  |
| sl-PSFCH-HopID-r16 |  | Not present | Default frequency hopping ID 0 is used |  |
| sl-PSFCH-CandidateResourceType-r16 |  | startSubCH |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-SyncAllowed-r16 SEQUENCE { |  |  |  |  |
| gnss-Sync-r16 |  | true |  |  |
| gnbEnb-Sync-r16 |  | true |  |  |
| ue-Sync-r16 |  | true |  |  |
| } |  |  |  |  |
| sl-SubchannelSize-r16 |  | n10 |  |  |
| dummy |  | Not present |  |  |
| sl-StartRB-Subchannel-r16 |  | 0 | start RB of BWP |  |
| sl-NumSubchannel-r16 |  | floor(n/m) | Note 1, Note 2 |  |
| sl-Additional-MCS-Table-r16 |  | Not present |  |  |
| sl-ThreshS-RSSI-CBR-r16 |  | 0 | actual value is -112 dBm |  |
| sl-TimeWindowSizeCBR-r16 |  | slot100 |  |  |
| sl-TimeWindowSizeCR-r16 |  | slot1000 |  |  |
| sl-PTRS-Config-r16 |  | Not present |  |  |
| sl-UE-SelectedConfigRP-r16 SEQUENCE { |  |  |  |  |
| sl-CBR-PriorityTxConfigList-r16 |  | SL-CBR-PriorityTxConfigList |  |  |
| sl-Thres-RSRP-List-r16 |  | SL-Thres-RSRP-List |  |  |
| sl-MultiReserveResource-r16 |  | Not present |  |  |
| sl-MaxNumPerReserve-r16 |  | n2 |  |  |
| sl-SensingWindow-r16 |  | ms100 |  |  |
| sl-SelectionWindowList-r16 SEQUENCE (SIZE (8)) OF SL-SelectionWindowConfig-r16 { |  | 8 entries |  |  |
| SL-SelectionWindowConfig-r16[k, k=1..8] SEQUENCE { |  |  | entry k |  |
| sl-Priority-r16 |  | k |  |  |
| sl-SelectionWindow-r16 |  | n10 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-ResourceReservePeriodList-r16 |  | Not present |  |  |
| sl-RS-ForSensing-r16 |  | pssch |  |  |
| sl-CBR-PriorityTxConfigList-v1650 |  | Not present |  |  |
| } |  |  |  |  |
| sl-RxParametersNcell-r16 |  | Not present |  |  |
| sl-ZoneConfigMCR-List-r16 |  | Not present |  |  |
| sl-FilterCoefficient-r16 |  | fc4 |  |  |
| sl-RB-Number-r16 |  | 10*floor(n/10) | Note 1 |  |
| sl-PreemptionEnable-r16 |  | Not present |  |  |
| sl-PriorityThreshold-UL-URLLC-r16 |  | Not present |  |  |
| sl-PriorityThreshold-r16 |  | Not present |  |  |
| sl-X-Overhead-r16 |  | Not present | Default value n0 is used |  |
| sl-PowerControl-r16 |  | Not present |  |  |
| sl-TxPercentageList-r16 SEQUENCE (SIZE (8)) OF SL-TxPercentageConfig-r16 { |  | 8 entries |  |  |
| SL-TxPercentageConfig-r16[k, k=1..8] SEQUENCE { |  |  | entry k |  |
| sl-Priority-r16 |  | k |  |  |
| sl-TxPercentage-r16 |  | p50 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-MinMaxMCS-List-r16 |  | Not present |  |  |
| sl-TimeResource-r16 |  | 1111111100 | First 8 of every 10 logical slots |  |
|  |  | 0000000011 | Last 2 of every 10 logical slots | EXCEPTIONAL |
| sl-PBPS-CPS-Config-r17 CHOICE { |  |  |  |  |
| setup |  | SL-PBPS-CPS-Config |  |  |
| } |  |  |  |  |
| sl-InterUE-CoordinationConfig-r17 |  | Not present |  |  |
| sl-CPE-StartingPositionsPSCCH-PSSCH-InitiateCOT-List-r18 |  | Not present |  |  |
| sl-CPE-StartingPositionsPSCCH-PSSCH-InitiateCOT-Default-r18 |  | Not present |  |  |
| sl-CPE-StartingPositionsPSCCH-PSSCH-WithinCOT-List-r18 |  | Not present |  |  |
| sl-CPE-StartingPositionsPSCCH-PSSCH-WithinCOT-Default-r18 |  | Not present |  |  |
| sl-Type1-LBT-BlockingOption1-r18 |  | Not present |  |  |
| sl-Type1-LBT-BlockingOption2-r18 |  | Not present |  |  |
| sl-NumInterlacePerSubchannel-r18 |  | Not present |  |  |
| sl-NumReferencePRBs-OfInterlace-r18 |  | Not present |  |  |
| sl-TransmissionStructureForPSFCH-r18 |  | Not present |  |  |
| sl-NumDedicatedPRBs-ForPSFCH-r18 |  | Not present |  |  |
| sl-NumPSFCH-Occasions-r18 |  | Not present |  |  |
| sl-PSFCH-CommonInterlaceIndex-r18 |  | Not present |  |  |
| sl-CPE-StartingPositionPSFCH-r18 |  | Not present |  |  |
| sl-NumRefSymbolLength-r18 |  | Not present |  |  |
| sl-PSFCH-RB-SetList-r18 |  | Not present |  |  |
| sl-IUC-RB-SetList-r18 |  | Not present |  |  |
| sl-PSFCH-PowerOffset-r18 |  | Not present |  |  |
| sl-RBSetIndexOfResourcePool-r18 |  | Not present |  |  |
| sl-A2X-Service-r18 |  | Not present |  |  |
| sl-PRS-ResourcesSharedSL-PRS-RP-r18 |  | Not present |  |  |
| sl-PRS-ResourcesSharedSL-PRS-RP-r18 SEQUENCE (SIZE (1..17)) OF SL-PRS-ResourceSharedSL-PRS-RP-r18 { |  | 1 entry |  | SLPP |
| SL-PRS-ResourceSharedSL-PRS-RP-r18 [1] SEQUENCE { |  |  | entry 1 |  |
| sl-PRS-ResourceID-r18 |  | 0 |  |  |
| mNumberOfSymbols-r18 |  | 4 |  |  |
| sl-PRS-CombSizeN-AndReOffset-r18 CHOICE { |  |  |  |  |
| n4-r18 |  | 1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| numSym-SL-PRS-2ndStageSCI-r18 |  | Not present |  |  |
|  |  | 2 |  | SLPP |
| sl-SCI-basedSL-PRS-TxTriggerSCI2-D-r18 |  | Not present |  |  |
| } |  |  |  |  |
| Note 1: n is the bandwidth of active SL BWP (in RB). n is determined in Table 4.3.1.0D-1 and Table 4.3.1.0D-2 for FR1 and FR2 respectively. Note 2: m is subchannel size configured by sl-SubchannelSize-r16. Note 3: p is the PSFCH period configured by sl-PSFCH-Period-r16 |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SL_HARQ | To enable SL HARQ feedback |
| EXCEPTIONAL | For exceptional resource pool |
| SLPP | To configure SL-PRS resource pool for SLPP test cases |