3GPP TSG-RAN5 Meeting #109 – TTCN Sidebar	R5w250311

# Dallas, USA, 17th Nov 2025 - 21st Nov 2025

# Agenda Item:       TTCN Sidebar

Source:	MCC TF160

Title:	Rel-17 NR_SL_enh: ASP updates

Document for:	Endorsement

# 1	Introduction

TC 12.1.12.1.NR5GC (PC5-only operation / DRX / Reception), included in the Rel-17 WI NR_SL_enh-UEConTest, is being updated at this RAN5#109 meeting with CR in R5-255918.

With this update, the TC requires the SS to receive positive acknowledgement via PSFCH. This is not supported by the current sidelink test model.

This document proposes an update of the sidelink ASPs to be able to configure the SS to report to TTCN the reception on the PSFCH of an HARQ Ack/Nack feedback.

# 2	ASP updates

# type record NR_SL_SYSTEM_IND {

# /* ASP to report errors or system indications */
  NR_SL_IndAspCommonPart_Type       Common,    /* The NR-SS-UE shall provide TimingInfo (SFN + subframe 
                                                  number) depending on the respective indication: */

# NR_SL_SystemIndication_Type       Indication /* - Error

# *    TimingInfo: related to the error (if available)

# * - SyncRefReporting

# *    TimingInfo: SFN/Subframe/Slot or DFN/DirectSubframe/Slot 
                                                     related to the S-SS/MasterInformationBlockSidelink
                                                     received

# * - PSSCH_SciReporting

# *    TimingInfo: SFN/Subframe/Slot or DFN/DirectSubframe/Slot
                                                     related to the received PSSCH SCI

# * - MAC_CE

# *    TimingInfo: SFN/Subframe/Slot or DFN/DirectSubframe/Slot 
                                                     related to the received MAC_CE
                                                * - PSFCH_HARQ
                                                     TimingInfo: SFN/Subframe/Slot or DFN/DirectSubframe/Slot 
                                                     related to the received PSFCH HARQ

# */
};

# type union NR_SL_SystemIndication_Type {

charstring    Error,    /* indicates an error situation in NR-SS-UE;

* is not explicitly handled in TTCN but causes an INCONC due to default behaviour;

* an additional error code can be signalled in the common part of the ASP;

* NR-SS-UE shall raise an error in case of

* - Invalid TimingInfo

* - Invalid data scheduling

* (NOTE: additional cases may occur) */

NR_SL_SyncRefReporting_Type     SyncRefReporting,   /* Indication for S-SS/MasterInformationBlockSidelink  
                                                         sent by UE */

NR_SL_PSSCH_SciReporting_Type   PSSCH_SciReporting, /* Indication for reception of SCI format 2-A or SCI 
                                                         format 2-B sent by UE */

NR_SL_MAC_ControlElement_Type   MAC_CE,             /* Indication for reception of MAC control element sent 
                                                         by UE */

HARQ_Type                       PSFCH_HARQ        /* Indication for reception of HARQ on PSFCH sent by UE */

};

type enumerated HARQ_Type {ack, nack};                /* ack represents HARQ ACK; nack represents HARQ NACK */

type union NR_SL_System_IndicationControl_Type {

/* Initially all indications are disabled in NR-SS-UE

(i.e. it shall not be necessary in 'normal' test cases to use this primitive but only if a specific
    indication is needed);

omit means indication mode is not changed.

TimingInfo : 'Now' (in general)

*/

IndicationAndControlMode_Type    SyncRefReporting,  /* To enable/disable reporting of start and stop of 
                                                         reception of NR sidelink S-SS/PSBCH.

* NOTE:

* this is applicable when NR-SS-UE is configured with 
                                                         SyncConfigRx

* NR-SS-UE reports an indication every time when
                                                         SLSSID/MasterInformationBlockSidelink reception from
                                                         UE has toggled in the subframes configured for 
                                                         SLSS/MasterInformationBlockSidelink reception

*/

IndicationAndControlMode_Type    PSSCH_SciReporting, /* To enable/disable reporting of reception of NR PSSCH
                                                          SCI-2A or SCI-2B

* NOTE:

* NR-SS-UE reports an indication every time an SCI 
                                                          format 2-A or SCI format 2-B is received

*/

IndicationAndControlMode_Type    SidelinkCSIReportingMAC_CE, /* To enable/disable reporting of Sidelink CSI 
                                                                 Reporting MAC CE received from UE */

IndicationAndControlMode_Type    PSFCH_HARQ          /* To enable/disable reporting of reception of HARQ 
                                                          ACK/NACK */

};