3GPP TSG-RAN5 Meeting #108	R5-254005r2

# Bangalore, India, 25th - 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1619 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of test case 19.1.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - As the emergency call shall be established according to C.22 it is using either AMR/8000 or AMR-WB/16000 The re-INVITE at step 2 and step 7 shall not be restricted to AMR-WB/16000 (or EVS/16000). It is assumed that the same audio codec is used as at emergency call establishment. - o= line in the INVITE at step 2 is identical to previous SDP sent by UE except that sess-version is incremented by one - The 200 OK for INVITE at steps 4 and 10 needs condition A6 (NOTE: default message content of A.3.1 is changed by R5-254001 to allow use of A6 and A23 togehther). - o= line in SDP answer sent to the UE at step 4 needs sess-version being incremented. - An emergency call does not have precondtions. - Note 1 in the specific message contents of the INVITE at step 2 is not precise. - There is a new procedure C.54 (CR 1626) which can be used as reference for the specific message contents. - As emergency call establishment is without precondions the re-INVITE at step 7 does not need to indicate support of preconditions. - There is no need to refer to step 2 in INVITE (Step 7) which refers to step 1 in annex C.54. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specification of the specific message contents done in the same way as in 5G sister test case 10.17 referring to the generic test procedure for setting up emergency speech call and RTT. Supported header field removed from INVITE (step 7) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect test specification, conformat UE may fail. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 19.1.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 34.229-1 CR 1615 TS 34.229-1 CR 1626 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Use of references to message contents of new procedure C.54 Rev2: INVITE (Step 7) simplified |  |  |  |  |  |  |  |  |

### 19.1.8	Emergency call / add RTT and remove RTT

#### 19.1.8.1	Definition

Test to verify that the UE is able to add a bidirectional text (RTT) component to an ongoing IMS emergency call. This process is described in 3GPP TS 24.229 [10], TS 24.173 [65] and TS 26.114 [66].

#### 19.1.8.2	Conformance requirement

[TS 24.229, clause 5.1.2A.1.1]

After the dialog is established the UE may change the dialog capabilities (e.g. add a media or request a supplementary service) if defined for the IMS communication service as identified by the ICSI value using the same dialog. Otherwise, the UE shall initiate a new initial request to the other user.

[TS 26.114, clause 6.2.5]

The SDP shall include bandwidth information for each media stream and also for the session in total. The bandwidth information for each media stream and for the session is defined by the Application Specific (AS) bandwidth modifier as defined in RFC 4566.

[TS 26.114, clause 6.3]:

During session renegotiation for adding or removing media components, the SDP offerer should continue to use the same media (m=) line(s) from the previously negotiated SDP for the media components that are not being added or removed.

[TS 26.114, clause 7.3.1]

The bandwidth for RTCP traffic shall be described using the "RS" and "RR" SDP bandwidth modifiers at media level, as specified by RFC 3556.

#### 19.1.8.3	Test purpose

1)	To verify that when adding an RTT component to an ongoing emergency call the UE performs correct exchange of SIP protocol signalling messages; and

2)	To verify that within SIP signalling the UE performs correct SDP offer/answer exchanges for negotiating media (as described by 3GPP TS 24.229 [10], clause 6.1); and

3)	To verify that when removing the RTT component from theemergency call the UE performs correct exchange of SIP and SDP protocol messages.

#### 19.1.8.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE has established an IMS emergency call according annex C.20 followed by annex C.22.

Test procedure

1)	Adding RTT to the emergency call is initiated on the UE.

2-10)	UE executes the procedure described in TS 36.508 [94] table 4.5A.pp.3-1, steps 2 to 6. In detail the following steps are done in IMS.

2)	UE to sends a re-INVITE request to the SS.

3)	SS responds to the re-INVITE request with a 100 Trying response.

4)	SS responds to the re-INVITE request with a valid 200 OK response.

5)	SS waits for the UE to send an ACK to acknowledge receipt of the 200 OK for INVITE.

6)	The UE is triggered to remove the RTT stream from the emergency call

7)	SS waits the UE to send a re-INVITE request with a SDP offer indicating the removal of the RTT stream.

8)	SS responds to the re-INVITE request with a 100 Trying response.

9)	SS deactivates the EPS bearer corresponding to the RTT stream and releases the associated radio resources by applying the procedure described in TS 36.508 [94] clause 4.5A.15

10)	 SS responds to the re-INVITE request with a valid 200 OK response.

11)	 SS waits for the UE to send an ACK to acknowledge receipt of the 200 OK for re-INVITE.

12-16)	 MO Call release according to procedure C.32.

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | Make UE add RTT to the voice call. |  |
| 2 |  |  | INVITE | UE sends re-INVITE with an SDP offer containing media lines for both voice and RTT |
| 3 |  |  | 100 Trying | The SS responds with a 100 Trying provisional response |
| 4 |  |  | 200 OK | The SS responds re-INVITE with 200 OK |
| 5 |  |  | ACK | The UE acknowledges the receipt of 200 OK for re-INVITE |
| 6 |  |  | Make UE release RTT from the media call |  |
| 7 |  |  | INVITE | UE sends re-INVITE with an SDP offer indicating that the RTT component is removed from the emergency call |
| 8 |  |  | 100 Trying | The SS responds with a 100 Trying provisional response |
| 9 |  |  | SS deactivates the EPS bearer for RTT |  |
| 10 |  |  | 200 OK | The SS responds re-INVITE with 200 OK |
| 11 |  |  | ACK | The UE acknowledges the receipt of 200 OK for re-INVITE |
| 12-16 |  |  | Steps defined in annex C.32 | MO Call release |

Specific Message Contents

INVITE (Step 2)

Use the default message “INVITE for MO Call” in annex A.2.1 with condition A5 (re-INVITE within a dialog) and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Message-body | Same SDP offer as specified for step 1 in annex C.54 with the following modifications: Session description: o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) [Note 1] Note 1: The "o=" line is identical to previous SDP sent by UE except that sess-version is incremented by one. |

200 OK for INVITE (Step 4)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with conditions A6 and A23 and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Content-Type |  |
| media-type | application/sdp |
| Message-body | SDP answer as specified for step 4 in annex C.54 with the following modifications: - "o=" line: o= line of previous SDP sent by SS with sess-version incremented by one. |

ACK (Step 5)

Use the default message “ACK” in annex A.2.7 with condition A5.

INVITE (Step 7)

Use the default message “INVITE for MO Call” in annex A.2.1 with condition A5 (re-INVITE within a dialog) and the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Message-body | Same SDP offer as specified for step 1 in annex C.54 with the following modifications: Session description: o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) [Note 1] Media description (real time text): m=text 0 RTP/AVP (fmt) [Note 2] Note 1: The "o=" line is identical to previous SDP sent by UE except that sess-version is incremented by one. Note 2: There are no requirements on b-, c- or a-lines of the real time text stream. |

200 OK for INVITE (Step 10)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with conditions A6 (Response sent by SS for INVITE for emergency call) and A23 (Response sent for re-INVITE within an established dialog) and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Content-Type |  |
| media-type | application/sdp |
| Message-body | SDP message as received in the re-INVITE at step 7 with the following modifications: - "o=" line: o= line of previous SDP sent by SS with sess-version incremented by one. - "c=" lines: connection-address changed to the IP address of the SS. - "m=" line of the media description for audio: transport port changed to the port number of the audio stream at the SS |

ACK (Step 11)

Use the default message “ACK” in annex A.2.7 with condition A5.

#### 19.1.8.5	Test requirements

The UE shall send requests and responses as described in clause 19.1.8.4