3GPP TSG-RAN5 Meeting #108	R5-254156r1

# Bengaluru, India, 25th - 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3538 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of p-max for FR2 default message contents |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | p-Max is configured in message content exceptions tables for FR2 tests. However, according to 3GPP TS 38.331, the presence of this IE is not required, as it shall be ignored by the UE: According to TS 38.508-2, Table A.4.3.2-1, the correct PICS for pc_dynamicPowerSharing pc_dynamicPowerSharingENDC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The default message contents have been updated for FR2. p-max has been marked as Not present. Exceptions in section 5.4.1 have been updated accordingly PICS for pc_dynamicPowerSharing corrected to pc_dynamicPowerSharingENDC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Configuration of FR2 tests will remain ambiguous with regards to TS 38.331 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3, 5.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1 included correction for pc_dynamicPowerSharingENDC, to solve overlap with R5-254060. Cover sheet has been accordingly updated. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

### 4.6.3	Radio resource control information elements

# < Unchanged sections omitted >

#### P-Max

NOTE:	Exceptions in clause 5.4.1 shall be applied for RF test cases.

Table 4.6.3-89: P-Max

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 23 |  | (NR OR pc_dynamicPowerSharingENDC) AND FR1 |
|  | 20 |  | EN-DC AND (NOT pc_dynamicPowerSharingENDC) AND FR1 |
|  | Not present |  | FR2 |

| Condition | Explanation |
| --- | --- |
| NR | NR connected to 5GC |
| EN-DC | E-UTRA-NR Dual Connectivity with E-UTRA connected to EPC |

# < Unchanged sections omitted >

### 5.4.1	Radio resource control information elements

As defined in clause 4.6.3 with the following exceptions:

For Tx and Rx test cases in which FR1 Power Class 3 UEs verifying Power Class 3 requirements, refer to Table 5.4.1-1; For Tx and Rx test cases in which FR1 Power Class 2 UEs verifying Power Class 2 requirements, refer to Table 5.4.1-2; For Tx and Rx test cases in which FR1 Power Class 1.5 UEs verifying Power Class 1.5 requirements, refer to Table 5.4.1-2A.

Table 5.4.1-1: P-Max-PC3

| Derivation Path: Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | Not present |  | SA AND FR1 |
|  | 23 |  | EN-DC AND FR1 |

Table 5.4.1-2: P-Max-PC2

| Derivation Path: Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | Not present |  | SA AND FR1 |
|  | 26 |  | EN-DC AND FR1 |

| Condition | Explanation |
| --- | --- |
| SA | NR standalone RF tests cases |
| EN-DC | EN-DC RF tests cases specified in 38.521-3 |

Table 5.4.1-2A: P-Max-PC1.5

| Derivation Path: Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | Not present |  | SA AND FR1 |

< End of changes >