3GPP TSG-RAN5 Meeting #108	R5-254208r1

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

Source:	NTTDOCOMO, INC

Title:	Proposal for new test case related to UE mobility in inter-system upon receiving the 5GMM cause value #15 in the REGISTRATION REJECT message

Agenda item:	6.5.6

Document for:	Endorsement

Background

According to TS38.523-1 [1], there is a test case related to UE mobility within NR upon receiving the 5GMM cause value #15 (No suitable cells in tracking area) in the REGISTRATION REJECT message.

==

[TS 38.523-1, clause 9.1.5.1.13]

Initial registration / Rejected / No suitable cells in tracking area

==

However, there is no test case related to UE mobility in inter-system upon receiving the 5GMM cause value #15 in the REGISTRATION REJECT message. According to TS 24.501 [3] and TS 38.304 [4], UE can perform mobility in inter-system upon receiving the 5GMM cause value #15 in the REGISTRATION REJECT message.

==

[TS 24.501, clause 5.5.1.2.5]

If the initial registration request cannot be accepted by the network, the AMF shall send a REGISTRATION REJECT message to the UE including an appropriate 5GMM cause value.

If the initial registration request is rejected due to general NAS level mobility management congestion control, the network shall set the 5GMM cause value to #22 "congestion" and assign a back-off timer T3346.

The UE shall take the following actions depending on the 5GMM cause value received in the REGISTRATION REJECT message.

#3	(Illegal UE); or

#6	(Illegal ME).

….

#15	(No suitable cells in tracking area);

The UE shall set the 5GS update status to 5U3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.2.2) and shall delete any 5G-GUTI, TAI list and ngKSI. Additionally, the UE shall reset the registration attempt counter.

The UE shall store the current TAI in the list of "5GS forbidden tracking areas for roaming" and enter the state 5GMM-DEREGISTERED.LIMITED-SERVICE. The UE shall search for a suitable cell in another tracking area according to 3GPP TS 38.304 [15].

[TS 38.304, clause 5.2.1]

UE shall perform measurements for cell selection and reselection purposes as specified in TS 38.133 [8].

When evaluating Srxlev and Squal of non-serving cells for reselection evaluation purposes, the UE shall use parameters provided by the serving cell and for the final check on cell selection criterion, the UE shall use parameters provided by the target cell for cell reselection.

The NAS can control the RAT(s) in which the cell selection should be performed, for instance by indicating RAT(s) associated with the selected PLMN, and by maintaining a list of forbidden registration area(s) and a list of equivalent PLMNs. The UE shall select a suitable cell based on RRC_IDLE or RRC_INACTIVE state measurements and cell selection criteria.

In order to expedite the cell selection process, stored information for several RATs, if available, may be used by the UE.

When camped on a cell, the UE shall regularly search for a better cell according to the cell reselection criteria. If a better cell is found, that cell is selected. The change of cell may imply a change of RAT. Details on performance requirements for cell reselection can be found in TS 38.133 [8].

The NAS is informed if the cell selection and reselection result in changes in the received system information relevant for NAS.

For normal service, the UE shall camp on a suitable cell, monitor control channel(s) of that cell so that the UE can:

-	receive system information from the PLMN or SNPN; and

-	receive registration area information from the PLMN or SNPN, e.g., tracking area information; and

-	receive other AS and NAS Information; and

-	if registered:

-	receive paging and notification messages from the PLMN or SNPN; and

-	initiate transfer to Connected mode.

For cell selection in multi-beam operations, measurement quantity of a cell is up to UE implementation.

For cell reselection in multi-beam operations, including inter-RAT reselection from E-UTRA to NR, the measurement quantity of this cell is derived amongst the beams corresponding to the same cell based on SS/PBCH block as follows:

-	if nrofSS-BlocksToAverage (maxRS-IndexCellQual in E-UTRA) is not configured in SIB2/SIB4 (SIB24 in E-UTRA); or

-	if absThreshSS-BlocksConsolidation (threshRS-Index in E-UTRA) is not configured in SIB2/SIB4 (SIB24 in E-UTRA); or

-	if the highest beam measurement quantity value is below or equal to absThreshSS-BlocksConsolidation (threshRS-Index in E-UTRA):

-	derive a cell measurement quantity as the highest beam measurement quantity value, where each beam measurement quantity is described in TS 38.215 [11].

-	else:

-	derive a cell measurement quantity as the linear average of the power values of up to nrofSS-BlocksToAverage (maxRS-IndexCellQual in E-UTRA) of highest beam measurement quantity values above absThreshSS-BlocksConsolidation (threshRS-Index in E-UTRA).

==

In the current commercial network, UE often experience mobility in inter-system upon receiving the 5GMM cause value #15 in the REGISTRATION REJECT message.

Therefore, we would like to introduce a test case related to UE mobility in inter-system upon receiving the 5GMM cause value #15 in the REGISTRATION REJECT message to RAN5 for stability of connectivity.

Proposal

I have two proposals to check UE mobility in inter-system upon receiving the 5GMM cause value #15 in the REGISTRATION REJECT message.

The primary proposal is to make a new test case, while an alternative proposal involving updating the current test case has also been prepared.

If one is considered more appropriate and agreed upon, the other will be withdrawn.

Proposal 1: RAN5 make a new test case to check whether UE successfully attach on E-UTRA Cell after receiving the 5GMM cause value #15 in the REGISTRATION REJECT message. This test case is applied to UE supporting Rel.15.

If this proposal is agreed upon, the following new test case has already been proposed.

# test case:

R5-254203: Addition of new test case 9.3.1.7 related to Inter-system mobility registration update / Rejected / No suitable cells in tracking area / 5GC to EPC

# Test applicability:

R5-254204: Applicability update to Inter-system mobility registration update / Rejected / No suitable cells in tracking area / 5GC to EPC test case

Proposal 2: RAN5 updates current test case (9.1.5.1.13/TS38.523-1) ‘s check points to check UE successfully attach on E-UTRA Cell after receiving the 5GMM cause value #15 in the REGISTRATION REJECT message.

The current test case (9.1.5,1,13/TS38.523-1) checks whether UE successfully perform registration on NGC Cell belonging to the other TAI after receiving the 5GMM cause value #15 in the REGISTRATION REJECT message. However, it does not check whether UE successfully attach on E-UTRA Cell after receiving the 5GMM cause value #15 in the REGISTRATION REJECT message.

Therefore, we would like to add a Test Purpose and a Test description regarding UE mobility in inter-system upon receiving the 5GMM cause value #15 in the REGISTRATION REJECT message.

If this proposal is agreed upon, the following update to the current test case has already been proposed.

# test case:

R5-254206: Correction of test case 9.1.5.1.13

Reference

[1] 3GPP TS 38.523-1: “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; 5GS; User Equipment (UE) conformance specification; Part 1: Protocol (Release 19)”.

[2] 3GPP TS 38.523-2: “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; 5GS; User Equipment (UE) conformance specification; Part 2: Applicability of protocol test cases (Release 19)”.

[3] 3GPP TS 24.501: “3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3; (Release 15)”

[4] 3GPP TS 38.304: “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state (Release 15)”