3GPP TSG-RAN WG5 Meeting #109	R5-255696r1

DALLAS, USA, 17th – 21st Nov, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-2 | CR | 0207 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new SLPP position measurement without assistance data test cases for SL-Target UE-based mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SLPP positioning procedure test cases are missing now. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added 9.5.4.7 LMF Initiated Position Measurement without assistance data: SL-Target UE-based - Added 9.5.4.8 SL server UE Initiated Position Measurement without assistance data: SL-Target UE-based (Target UE is in coverage) - Added 9.5.4.9 SL server UE Initiated Position Measurement without assistance data: SL-Target UE-based (Target UE is out of coverage) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The release 18 positioning work item will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.5.4.7(new), 9.5.4.8(new), 9.5.4.9(new), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.571-3 CR 0178 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1:Removed RESET UE POSITIONING STORED INFORMATION message in 9.5.4.8 and changed it to use MMI ot AT command since there is no RRC connection in this test case. |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

9.5.4.7	LMF Initiated Position Measurement without assistance data: SL-Target UE-based

9.5.4.7.1	Test Purpose (TP)

(1)

with { a NAS signalling connection }

ensure that {
  when { UE has no assistance data stored and receives a SLPP location request information message for SL target UE-based mode and the UE requires assistance data in order to fulfill the location request }

then { UE sends a SLPP REQUEST ASSISTANCE DATA message followed by a SLPP PROVIDE LOCATION INFORMATION message containing a location estimate }

}

}

9.5.4.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.355 clauses 5.2.4, 5.2.5, 5.3.5 and 5.3.6. Unless otherwise stated these are Rel-18 requirements.

[TS 38.355, clause 5.2.4]

When triggered to transmit a RequestAssistanceData message, Endpoint A shall:

1>	set the positioning method specific RequestAssistanceData PDUs in accordance with the information received from upper layers.

1>	deliver the message to lower layers for transmission.

[TS 38.355, clause 5.2.5]

Upon receiving a RequestAssistanceData message, Endpoint B shall generate a ProvideAssistanceData message as a response.

Endpoint B shall:

1>	for each positioning method for which a request for assistance data is included in the message:

2>	if Endpoint B supports this positioning method:

3>	include the assistance data for that supported positioning method in the response message;

1>	set the field sessionID in the response message to the same value as the field sessionID in the received message if received;

1>	set the field transactionID in the response message to the same value as the field transactionID in the received message;

- deliver the response message to lower layers for transmission.

[TS 38.355, clause 5.3.5]

Upon receiving a RequestLocationInformation message, Endpoint A shall:

1>	if the requested information is compatible with Endpoint A's capabilities and configuration:

2>	include the requested information in a ProvideLocationInformation message;

2>	set the field sessionID in the response message to the same value as the field sessionID in the received message if received;

2>	set the field transactionID in the response to the same value as the field transactionID in the received message;

2>	deliver the ProvideLocationInformation message to lower layers for transmission.

[TS 38.355, clause 5.3.6]

When triggered to transmit a ProvideLocationInformation message, Endpoint A shall:

1>	for each positioning method contained in the message:

2>	set the corresponding fields to include the available location information;

1>	deliver the response to lower layers for transmission.

9.5.4.7.3	Test description

9.5.4.7.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

-	System information combination NR-1 as defined in TS 38.508-1 [30] clause 4.4.3.1.3 is used in NR cell.

NR-SS-UE

-	Sub-test 33: NR-SS-UE1 and NR-SS-UE2 as specified in clause 8.2.14.

-	Sub-test 34: NR-SS-UE1 and NR-SS-UE2 as specified in clause 8.2.15.

-	Sub-test 35: NR-SS-UE1, NR-SS-UE2 and NR-SS-UE3 as specified in clause 8.2.16.

-	Sub-test 36: NR-SS-UE1 and NR-SS-UE2 as specified in clause 8.2.17.

-	All NR-SS-UEs use GNSS as the synchronization reference source and operate as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

- None.

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [30], subclause 4.4A on NR Cell 1.

9.5.4.7.3.2	Test procedure sequence

This test case includes sub-test cases dependent on the positioning method(s) supported by the UE. Each sub-test case is identified by a sub-test case number as defined in Table 9.5.4.7.3.2-1 below:

Table 9.5.4.7.3.2-1: Sub-test case numbers

| Sub-Test Case Number | Supported Positioning Methods |
| --- | --- |
| 33 | UE supporting SL-RTT (Rel-18 onwards) |
| 34 | UE supporting SL-AoA (Rel-18 onwards) |
| 35 | UE supporting SL-TDOA (Rel-18 onwards) |
| 36 | UE supporting SL-TOA (Rel-18 onwards) |

Table 9.5.4.7.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sends a RESET UE POSITIONING STORED INFORMATION message. | <-- | RESET UE POSITIONING STORED INFORMATION | - | - |
| 2 | The SS sends a SLPP message of type Request Capabilities. | <-- | DLInformationTransfer (SLPP REQUEST CAPABILITIES) | - | - |
| 3 | The UE sends a SLPP message of type Provide Capabilities including the UE positioning capabilities. | --> | ULInformationTransfer (SLPP PROVIDE CAPABILITIES) | - | - |
| 4 | IF the UE SLPP message at step 3 includes an acknowledgment request THEN SS sends a SLPP Acknowledgement response. | <-- | DLInformationTransfer (SLPP ACKNOWLEDGEMENT) | - | - |
| 5 | The SS sends a SLPP message of type Request Location Information including a request for a location estimate. | <-- | DLInformationTransfer (SLPP REQUEST LOCATION INFORMATION) | - |  |
| 6 | The UE sends a SLPP message of type Request Assistance Data including a request for assistance data. NOTE: This requires a method of triggering a SLPP Request Assistance Data message. | --> | ULInformationTransfer (SLPP REQUEST ASSISTANCE DATA) | 1 | P |
| 7 | The SS sends a SLPP message of type Provide Assistance Data. | <-- | DLInformationTransfer (SLPP PROVIDE ASSISTANCE DATA) | - | - |
| 8 | The UE sends a SLPP message of type Provide Location Information including a location estimate. | --> | ULInformationTransfer (SLPP PROVIDE LOCATION INFORMATION) | 1 | P |
| 9 | IF the UE SLPP message at step 8 includes an acknowledgment request THEN SS sends a SLPP Acknowledgement response. | <-- | DLInformationTransfer (SLPP ACKNOWLEDGEMENT) | - | - |

9.5.4.7.3.3	Specific message contents

Table 9.5.4.7.3.3-1: RESET UE POSITIONING STORED INFORMATION (step 1, Table 9.5.4.7.3.2-2)

| Derivation Path: 38.509 clause 6.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE Positioning Technology |  | Sub-test 33: 0 0 0 0 1 0 0 1 Sub-test 34: 0 0 0 0 1 0 1 0 Sub-test 35: 0 0 0 0 1 0 1 1 Sub-test 36: 0 0 0 0 1 1 0 0 | Sub-test 33: SL-RTT Sub-test 34: SL-AoA Sub-test 35: SL-TDOA Sub-test 36: SL-TOA |  |

Table 9.5.4.7.3.3-2: DL InformationTransfer (steps 2, 4, 5, 7 and 9, Table 9.5.4.7.3.2-2)

| Derivation Path: 38.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DLInformationTransfer ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| dlInformationTransfer SEQUENCE { |  |  |  |  |
| dedicatedNAS-Message OCTET STRING |  | Set according to Table 9.5.4.7.3.3-3 | DL NAS TRANSPORT |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.5.4.7.3.3-3: DL NAS TRANSPORT (steps 2, 4, 5, 7 and 9, Table 9.5.4.7.3.2-2)

| Derivation Path: 24.501 Table 8.2.11.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended Protocol discriminator |  | 01111110 | 5GS mobility management messages |  |
| Security header type |  | 0000 | Plain 5GS NAS message |  |
| Spare half octet |  | 0000 | Downlink generic NAS transport |  |
| DL NAS TRANSPORT message identity |  | 01101000 | DL NAS transport |  |
| Payload container type |  | 1100 | SLPP message container |  |
| Spare half octet |  | 0000 |  |  |
| Payload container |  | Step 2: Set according to Table 8.5-1 | SLPP Request Capabilities |  |
|  |  | Steps 4 and 9: Set according to Table 9.5.4.7.3.3-4 | SLPP Acknowledgement |  |
|  |  | Step 5: Set according to Table 8.5-2 | SLPP Provide Assistance Data |  |
|  |  | Step 6: Set according to Table 9.5.4.7.3.3-5 | SLPP Request Location Information |  |
| Additional information |  | Present | Routing Identifier/Correlation ID |  |

Table 9.5.4.7.3.3-4: SLPP Acknowledgement (steps 4 and 9, Table 9.5.4.7.3.2-2)

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| applicationLayerID | Not present |  |  |
| transactionID | Not present |  |  |
| endTransaction | TRUE |  |  |
| sequenceNumber | Not present |  |  |
| sessionID | Not present |  |  |
| acknowledgement SEQUENCE { |  |  |  |
| ackRequested | FALSE |  |  |
| ackIndicator | (0..255) | Contains the same value of the sequenceNumber field in 3 or 8, Table 9.5.4.7.3.2-2. |  |
| } |  |  |  |
| slpp-MessageBody | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |

Table 9.5.4.7.3.3-5: SLPP REQUEST LOCATION INFORMATION (step 5, Table 9.5.4.7.3.2-2)

| Derivation Path: Table 8.5-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| As defined in Table 8.5-3 with the following exceptions: |  |  |  |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| slpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestLocationInformation SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| requestLocationInformation SEQUENCE { |  |  |  |
| commonIEsRequestLocationInformation | Present | OCTET STRING containing CommonIEsRequestLocationInformation as defined in Table 9.5.4.7.3.3-6 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.5.4.7.3.3-6: CommonIEsRequestLocationInformation

| Derivation Path: Table 8.5-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| As defined in Table 8.5-4 with the following exceptions: |  |  |  |
| CommonIEsRequestLocationInformation ::= SEQUENCE { |  |  |  |
| locationInformationType | locationEstimateRequired |  |  |
| } |  |  |  |

Table 9.5.4.7.3.3-7: ULInformationTransfer (steps 3, 6 and 8, Table 9.5.4.7.3.2-2)

| Derivation Path: 38.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| ULInformationTransfer ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| ulInformationTransfer SEQUENCE { |  |  |  |  |
| dedicatedNAS-Message OCTET STRING |  | Set according to Table 9.5.4.7.3.3-8 | UL NAS TRANSPORT |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.5.4.7.3.3-8: UL NAS TRANSPORT (steps 3, 6 and 8, Table 9.5.4.7.3.2-2)

| Derivation Path: 24.501 Table 8.2.10.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended Protocol discriminator |  | 01111110 | 5GS mobility management messages |  |
| Security header type |  | 0000 | Plain 5GS NAS message |  |
| Spare half octet |  | 0000 |  |  |
| UL NAS TRANSPORT message identity |  | 01100111 | UL NAS TRANSPORT |  |
| Payload container type |  | 1100 | SLPP message container |  |
| Spare half octet |  | 0000 |  |  |
| Payload container |  | Step 3: Set according to Table 9.5.1.1.3.3-6 | SLPP Provide Capabilities |  |
|  |  | Step 6: Set according to Table 9.5.4.7.3.3-9 | SLPP Request Assistance Data |  |
|  |  | Step 8: Set according to Table 9.5.4.7.3.3-12 | SLPP Provide Location Information |  |
| Additional information |  | Present | The UE includes the Routing Identifier received in the Additional Information IE of the DOWNLINK GENERIC NAS TRANSPORT message (step 2 or 5 Table 9.5.4.7.3.2-2) |  |

Table 9.5.4.7.3.3-9: SLPP REQUEST ASSISTANCE DATA (step 6, Table 9.5.4.7.3.2-2)

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| applicationLayerID | Not present |  |  |
| transactionID | (0..255) | Contains the same value as the corresponding field in SLPP Request Location Information message in step 5, Table 9.5.4.7.3.2-2 |  |
| endTransaction | FALSE |  |  |
| sequenceNumber | (0..255) |  |  |
| sessionID | Not present |  |  |
| acknowledgement SEQUENCE { | Present, or not present. |  |  |
| ackRequested | TRUE |  |  |
| ackIndicator | Not present |  |  |
| } |  |  |  |
| slpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestAssistanceData SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| requestAssistanceData SEQUENCE { |  |  |  |
| commonIEsRequestAssistanceData | Not checked |  |  |
| commonSL-PRS-MethodsIEsRequestAssistanceData | Present. Any value acceptable |  |  |
| sl-AoA-RequestAssistanceData | Not checked |  |  |
| sl-RTT-RequestAssistanceData | Not checked |  |  |
| sl-TDOA-RequestAssistanceData | Present | OCTET STRING containing SL-TDOA-RequestAssistanceData as defined in Table 9.5.4.7.3.3-10 | Sub-test 35 |
| sl-TOA-RequestAssistanceData | Present | OCTET STRING containing SL-TOA-RequestAssistanceData as defined in Table 9.5.4.7.3.3-11 | Sub-test 36 |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |

Table 9.5.4.7.3.3-10: SL-TDOA-RequestAssistanceData

| Derivation Path: 38.355 clause 6.9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-TDOA-RequestAssistanceData ::= SEQUENCE { |  |  |  |
| sl-RTD-InfoRequest | true |  |  |
| } |  |  |  |

Table 9.5.4.7.3.3-11: SL-TOA-RequestAssistanceData

| Derivation Path: 38.355 clause 6.10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-TOA-RequestAssistanceData ::= SEQUENCE { |  |  |  |
| sl-RTD-InfoRequest | true |  |  |
| } |  |  |  |

Table 9.5.4.7.3.3-12: SLPP PROVIDE LOCATION INFORMATION (step 8, Table 9.5.4.7.3.2-2)

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| applicationLayerID | Not present |  |  |
| transactionID | (0..255) | Contains the same value as the corresponding field in SLPP Request Location Information message in step 5, Table 9.5.4.7.3.2-2 |  |
| endTransaction | TRUE |  |  |
| sequenceNumber | (0..255) |  |  |
| sessionID | Not present |  |  |
| acknowledgement SEQUENCE { | Present, or not present. |  |  |
| ackRequested | TRUE |  |  |
| ackIndicator | Not present |  |  |
| } |  |  |  |
| slpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideLocationInformation SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| provideLocationInformation SEQUENCE { |  |  |  |
| commonIEsProvideLocationInformation | Present | OCTET STRING containing CommonIEsProvideLocationInformation as defined in Table 9.5.4.7.3.3-13 |  |
| commonSL-PRS-MethodsIEsProvideLocationInformation | Not checked |  |  |
| sl-AoA-ProvideLocationInformation | Not present |  |  |
| sl-RTT-ProvideLocationInformation | Not present |  |  |
| sl-TDOA-ProvideLocationInformation | Not present |  |  |
| sl-TOA-ProvideLocationInformation | Not present |  |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |

Table 9.5.4.7.3.3-13: CommonIEsProvideLocationInformation

| Derivation Path: 38.355 clause 6.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CommonIEsProvideLocationInformation ::= SEQUENCE { |  |  |  |
| locationEstimate | Present. Any value acceptable |  |  |
| rangeAndOrDirection | Not present |  |  |
| velocityEstimate | Not present |  |  |
| relativeLocationEstimate | Not present |  |  |
| locationError | Not present |  |  |
| locationTimeStamp | Present. Any value acceptable |  |  |
| } |  |  |  |

9.5.4.8	SL server UE Initiated Position Measurement without assistance data: SL-Target UE-based (Target UE is in coverage)

9.5.4.8.1	Test Purpose (TP)

(1)

with { Target UE is in coverage and having established PC5 RRC connection with peer UE on unicast sidelink }

ensure that {
  when { UE has no assistance data stored and receives a SLPP location request information message for SL target UE-based mode and the UE requires assistance data in order to fulfill the location request }

then { UE sends a SLPP REQUEST ASSISTANCE DATA message followed by a SLPP PROVIDE LOCATION INFORMATION message containing a location estimate }

}

}

9.5.4.8.2	Conformance requirements

As defined in clause 9.5.4.7.2.

9.5.4.8.3	Test description

9.5.4.8.3.1	Pre-test conditions

System Simulator:

SS-NW

-	NR Cell 1.

-	System information combination NR-36 as defined in TS 38.508-1 [30] clause 4.4.3.1.3 is used in NR cell.

NR-SS-UE

-	Sub-test 33: NR-SS-UE1 and NR-SS-UE2 as specified in clause 8.2.14.

-	Sub-test 34: NR-SS-UE1 and NR-SS-UE2 as specified in clause 8.2.15.

-	Sub-test 35: NR-SS-UE1, NR-SS-UE2 and NR-SS-UE3 as specified in clause 8.2.16.

-	Sub-test 36: NR-SS-UE1 and NR-SS-UE2 as specified in clause 8.2.17.

-	All NR-SS-UEs use GNSS as the synchronization reference source and operate as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

-	UE is authorised to perform NR sidelink communication.

-	For V2X capable UE, the UE is equipped with a USIM containing default values as per TS 38.508-1 [30] clause 4.8.3.3.3.

-	For 5G ProSe capable UE, the UE is equipped with a USIM containing default values as per TS 38.508-1 [30] clause 4.8.3.3.4.

-	UE is synchronised on GNSS.

Preamble:

-	The UE is in state 1N-B RRC_IDLE_with_SL and Test Mode (On) with UE test loop mode E as defined in TS 38.508-1 [30], subclause 4.4A using generic procedure parameter Sidelink (On), test mode (On) and GNSS Sync (On) as defined in TS 38.508-1 [30], subclause 4.5.1, using the following procedures:

-	for V2X capable UE, using UE initiated unicast mode NR sidelink communication procedure in subclause 4.9.22;

-	for 5G ProSe capable UE, using UE initiated unicast mode ProSe Direct communication procedure in subclause 4.9.39.

9.5.4.8.3.2	Test procedure sequence

This test case includes sub-test cases dependent on the positioning method(s) supported by the UE. Each sub-test case is identified by a sub-test case number as defined in Table 9.5.4.8.3.2-1 below:

Table 9.5.4.8.3.2-1: Sub-test case numbers

| Sub-Test Case Number | Supported Positioning Methods |
| --- | --- |
| 33 | UE supporting SL-RTT (Rel-18 onwards) |
| 34 | UE supporting SL-AoA (Rel-18 onwards) |
| 35 | UE supporting SL-TDOA (Rel-18 onwards) |
| 36 | UE supporting SL-TOA (Rel-18 onwards) |

Table 9.5.4.8.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS triggers the UE to reset the UE positioning stored information. NOTE: This is performed via MMI or AT command. | - | - | - | - |
| 2 | The NR-SS-UE1 sends a SLPP message of type Request Capabilities. | <-- | SLPP REQUEST CAPABILITIES | - | - |
| 3 | The UE sends a SLPP message of type Provide Capabilities including the UE positioning capabilities. | --> | SLPP PROVIDE CAPABILITIES | - | - |
| 4 | IF the UE SLPP message at step 3 includes an acknowledgment request THEN SS sends a SLPP Acknowledgement response. | <-- | SLPP ACKNOWLEDGEMENT | - | - |
| 5 | The NR-SS-UE1 sends a SLPP message of type Request Location Information including a request for a location estimate. | <-- | SLPP REQUEST LOCATION INFORMATION | - |  |
| 6 | The UE sends a SLPP message of type Request Assistance Data including a request for assistance data. NOTE: This requires a method of triggering a SLPP Request Assistance Data message. | --> | SLPP REQUEST ASSISTANCE DATA | 1 | P |
| 7 | The NR-SS-UE1 sends a SLPP message of type Provide Assistance Data. | <-- | SLPP PROVIDE ASSISTANCE DATA | - | - |
| 8 | The UE sends a SLPP message of type Provide Location Information including a location estimate. | --> | SLPP PROVIDE LOCATION INFORMATION | 1 | P |
| 9 | IF the UE SLPP message at step 8 includes an acknowledgment request THEN NR-SS-UE1 sends a SLPP Acknowledgement response. | <-- | SLPP ACKNOWLEDGEMENT | - | - |

9.5.4.8.3.3	Specific message contents

Table 9.5.4.8.3.3-1: SIB23-IEs (Preamble)

| Derivation Path: TS 38.331 [30], Table 4.6.2-22A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB23-IEs-r18 ::= SEQUENCE { |  |  |  |
| sl-PosConfigCommonNR-r18 SEQUENCE { |  |  |  |
| sl-PosFreqInfoList-r18 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfigCommon-r16 { | 1 entry |  |  |
| SL-FreqConfigCommon-r16[1] | SL-FreqConfigCommon with condition SLPP | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

Table 9.5.4.8.3.3-2: Void

Table 9.5.4.8.3.3-3: SLPP REQUEST CAPABILITIES (step 2, Table 9.5.4.8.3.2-2)

Derivation Path: Table 8.5-1 with UE-only mode condition.

Table 9.5.4.8.3.3-4: SLPP PROVIDE CAPABILITIES (step 3, Table 9.5.4.8.3.2-2)

| Derivation Path: Table 9.5.1.1.3.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| As defined in Table 9.5.1.1.3.3-6 with the following exception: |  |  |  |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| sessionID | Present | Contains the same value of the transactionID field in step 2, Table 9.5.4.8.3.2-2. |  |
| } |  |  |  |

Table 9.5.4.8.3.3-5: SLPP Acknowledgement (steps 4 and 9, Table 9.5.4.8.3.2-2)

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| applicationLayerID | '00 00 01 00'H | Application Layer ID in initiating UE side |  |
| transactionID | Not present |  |  |
| endTransaction | TRUE |  |  |
| sequenceNumber | Not present |  |  |
| sessionID | Present | Contains the same value of the transactionID field in step 2, Table 9.5.4.8.3.2-2. |  |
| acknowledgement SEQUENCE { |  |  |  |
| ackRequested | FALSE |  |  |
| ackIndicator | (0..255) | Contains the same value of the sequenceNumber field in 3 or 8, Table 9.5.4.8.3.2-2. |  |
| } |  |  |  |
| slpp-MessageBody | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |

Table 9.5.4.8.3.3-6: SLPP REQUEST LOCATION INFORMATION (step 5, Table 9.5.4.8.3.2-2)

| Derivation Path: Table 8.5-3 with UE-only mode condition. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| As defined in Table 8.5-3 with the following exceptions: |  |  |  |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| slpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestLocationInformation SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| requestLocationInformation SEQUENCE { |  |  |  |
| commonIEsRequestLocationInformation | Present | OCTET STRING containing CommonIEsRequestLocationInformation as defined in Table 9.5.4.8.3.3-7 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.5.4.8.3.3-7: CommonIEsRequestLocationInformation

| Derivation Path: Table 8.5-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| As defined in Table 8.5-4 with the following exceptions: |  |  |  |
| CommonIEsRequestLocationInformation ::= SEQUENCE { |  |  |  |
| locationInformationType | locationEstimateRequired |  |  |
| } |  |  |  |

Table 9.5.4.8.3.3-8: SLPP REQUEST ASSISTANCE DATA (step 6, Table 9.5.4.8.3.2-2)

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| applicationLayerID | Present. Any value acceptable |  |  |
| transactionID | (0..255) | Contains the same value as the corresponding field in SLPP Request Location Information message in step 5, Table 9.5.4.8.3.2-2 |  |
| endTransaction | FALSE |  |  |
| sequenceNumber | (0..255) |  |  |
| sessionID | Present | Contains the same value of the transactionID field in step 2, Table 9.5.4.8.3.2-2. |  |
| acknowledgement SEQUENCE { | Present, or not present. |  |  |
| ackRequested | TRUE |  |  |
| ackIndicator | Not present |  |  |
| } |  |  |  |
| slpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestAssistanceData SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| requestAssistanceData SEQUENCE { |  |  |  |
| commonIEsRequestAssistanceData | Not checked |  |  |
| commonSL-PRS-MethodsIEsRequestAssistanceData | Present. Any value acceptable |  |  |
| sl-AoA-RequestAssistanceData | Not checked |  |  |
| sl-RTT-RequestAssistanceData | Not checked |  |  |
| sl-TDOA-RequestAssistanceData | Present | OCTET STRING containing SL-TDOA-RequestAssistanceData as defined in Table 9.5.4.8.3.3-9 | Sub-test 35 |
| sl-TOA-RequestAssistanceData | Present | OCTET STRING containing SL-TOA-RequestAssistanceData as defined in Table 9.5.4.8.3.3-10 | Sub-test 36 |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |

Table 9.5.4.8.3.3-9: SL-TDOA-RequestAssistanceData

| Derivation Path: 38.355 clause 6.9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-TDOA-RequestAssistanceData ::= SEQUENCE { |  |  |  |
| sl-RTD-InfoRequest | true |  |  |
| } |  |  |  |

Table 9.5.4.8.3.3-10: SL-TOA-RequestAssistanceData

| Derivation Path: 38.355 clause 6.10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-TOA-RequestAssistanceData ::= SEQUENCE { |  |  |  |
| sl-RTD-InfoRequest | true |  |  |
| } |  |  |  |

Table 9.5.4.8.3.3-11: SLPP PROVIDE ASSISTANCE DATA (step 7, Table 9.5.4.8.3.2-2))

Derivation Path: Table 8.5-2 with UE-only mode condition.

Table 9.5.4.8.3.3-12: SLPP PROVIDE LOCATION INFORMATION (step 8, Table 9.5.4.8.3.2-2)

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| applicationLayerID | Present. Any value acceptable |  |  |
| transactionID | (0..255) | Contains the same value as the corresponding field in SLPP Request Location Information message in step 5, Table 9.5.4.8.3.2-2 |  |
| endTransaction | TRUE |  |  |
| sequenceNumber | (0..255) |  |  |
| sessionID | Present | Contains the same value of the transactionID field in step 2, Table 9.5.4.8.3.2-2. |  |
| acknowledgement SEQUENCE { | Present, or not present. |  |  |
| ackRequested | TRUE |  |  |
| ackIndicator | Not present |  |  |
| } |  |  |  |
| slpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideLocationInformation SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| provideLocationInformation SEQUENCE { |  |  |  |
| commonIEsProvideLocationInformation | Present | OCTET STRING containing CommonIEsProvideLocationInformation as defined in Table 9.5.4.8.3.3-13 |  |
| commonSL-PRS-MethodsIEsProvideLocationInformation | Not checked |  |  |
| sl-AoA-ProvideLocationInformation | Not present |  |  |
| sl-RTT-ProvideLocationInformation | Not present |  |  |
| sl-TDOA-ProvideLocationInformation | Not present |  |  |
| sl-TOA-ProvideLocationInformation | Not present |  |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |

Table 9.5.4.8.3.3-13: CommonIEsProvideLocationInformation

| Derivation Path: 38.355 clause 6.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CommonIEsProvideLocationInformation ::= SEQUENCE { |  |  |  |
| locationEstimate | Present. Any value acceptable |  |  |
| rangeAndOrDirection | Not present |  |  |
| velocityEstimate | Not present |  |  |
| relativeLocationEstimate | Not present |  |  |
| locationError | Not present |  |  |
| locationTimeStamp | Present. Any value acceptable |  |  |
| } |  |  |  |

9.5.4.9	SL server UE Initiated Position Measurement without assistance data: SL-Target UE-based (Target UE is out of coverage)

9.5.4.9.1	Test Purpose (TP)

(1)

with { Target UE is out of coverage and having established PC5 RRC connection with peer UE on unicast sidelink }

ensure that {
  when { UE has no assistance data stored and receives a SLPP location request information message for SL target UE-based mode and the UE requires assistance data in order to fulfill the location request }

then { UE sends a SLPP REQUEST ASSISTANCE DATA message followed by a SLPP PROVIDE LOCATION INFORMATION message containing a location estimate }

}

}

9.5.4.9.2	Conformance requirements

As defined in clause 9.5.4.7.2.

9.5.4.9.3	Test description

9.5.4.9.3.1	Pre-test conditions

System Simulator:

NR-SS-UE

-	Sub-test 33: NR-SS-UE1 and NR-SS-UE2 as specified in clause 8.2.14.

-	Sub-test 34: NR-SS-UE1 and NR-SS-UE2 as specified in clause 8.2.15.

-	Sub-test 35: NR-SS-UE1, NR-SS-UE2 and NR-SS-UE3 as specified in clause 8.2.16.

-	Sub-test 36: NR-SS-UE1 and NR-SS-UE2 as specified in clause 8.2.17.

-	All NR-SS-UEs use GNSS as the synchronization reference source and operate as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

-	UE is authorised to perform NR sidelink communication.

-	For V2X capable UE, the UE is equipped with a USIM containing default values as per TS 38.508-1 [30] clause 4.8.3.3.3.

Table 9.5.4.9.3.1-1: UE/ USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFUST |  | Service n°119 (V2X) supported |  |
| EFVST |  | As per TS 38.508-1 [30] clause 4.8.3.3.3 |  |
| EFV2XP_PC5 |  | SL-PreconfigurationNR field as defined in TS 38.508-1 [30], table 4.10.1-1 with SLPP condition |  |

-	For 5G ProSe capable UE, the UE is equipped with a USIM containing default values as per TS 38.508-1 [30] clause 4.8.3.3.4 except for SL-PreconfigurationNR with SLPP condition.

Table 9.5.4.9.3.1-2: UE / USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFUST |  | Service n°139 5G ProSe is "available" |  |
| EF5G_PROSE_ST |  | Service n°1, n°2 and n°4 are "available" |  |
| EF5G_PROSE_DD |  | SL-PreconfigurationNR field as defined in TS 38.508-1 [30], table 4.10.1-1 with SLPP condition |  |
| EF5G_PROSE_DC |  | SL-PreconfigurationNR field as defined in TS 38.508-1 [30], table 4.10.1-1 with SLPP condition |  |
| EF5G_PROSE_RU |  | SL-PreconfigurationNR field as defined in TS 38.508-1 [30], table 4.10.1-1 with SLPP condition |  |

-	UE is synchronised on GNSS.

Preamble:

-	The UE is in state 4-A and Test Mode (On), Test Loop Function (Off) as defined in TS 38.508-1 [30], Table 4.5.7.2-1 using generic procedure parameter Sidelink (On),Cast Type (Unicast), UE initiating unicast mode NR sidelink communication, GNSS Sync (On), using the following procedures:

-	for V2X capable UE, using UE initiated unicast mode NR sidelink communication procedure in subclause 4.9.22;

-	for 5G ProSe capable UE, using UE initiated unicast mode ProSe Direct communication procedure in subclause 4.9.39.

9.5.4.9.3.2	Test procedure sequence

As defined in clause 9.5.4.8.3.2.

9.5.4.8.3.3	Specific message contents

As defined in clause 9.5.4.8.3.3.