3GPP TSG-RAN5 Meeting #109	R5-255815r1

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 4187 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to representation of UE position in test cases for NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The representation of UE position in test cases for NTN needs to be revised and the definition needs to be clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Revise the representation of UE position from (N,S,H) to (L,B,H). Further clarify the definition of UE position in test cases for NTN in related clauses. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The definition of UE position in test cases for NTN would be not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.2.1.1.4.1, 14.2.1.2.4.1, 14.2.1.3.4.1, 14.2.1.4.4.1, 14.2.1.5.4.1, 14.2.1.6.4.1 and 14.2.1.8.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: removed changes in TC 14.2.2.4. and 14.2.2.5 to CR#4191. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

Table 14.2.1.1.4.1-3: General test parameters for NR SA FR1 Handover for Satellite Access

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1 | One NR NTN satellite RF channel |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| UE position (L,B,H) |  |  | (0, 0, 0) | Set by any preconfigured means. (L,B,H) is Geodetic coordinate, where L is latitude, B is longitude, and H is height. |
| A3-Offset |  | dB | -1 | Modified by Test Tolerance |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

==============Next change==============

Table 14.2.1.2.4.1-3: General test parameters for NR SA FR1-FR1 Handover for Satellite Access

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1, 2 | Two NR NTN satellite RF channel |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| UE position (L,B,H) |  |  | (0, 0, 0) | Set by any preconfigured means. (L,B,H) is Geodetic coordinate, where L is latitude, B is longitude, and H is height. |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

==============Next change==============

Table 14.2.1.3.4.1-3: General test parameters for NR SA FR1 Time-based Conditional Handover for Satellite Access

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1 | One NR NTN satellite RF channel |
| Initial conditions | Active cell |  | Cell 1 | FDD duplex mode cell |
|  | Neighbouring cell |  | Cell 2 | FDD duplex mode cell |
| Final condition | Active cell |  | Cell 2 |  |
| UE position (L,B,H) |  |  | (0, 0, 0) | Set by any preconfigured means. (L,B,H) is Geodetic coordinate, where L is latitude, B is longitude, and H is height. |
| t1-Threshold-r17.condEventT1-r17 |  | s | T1 | Entering condition at start of T2 (end of T1) |
| duration-r17.condEventT1-r17 |  | slot | 1000 | Give 1s search duration |
| A3-Offset in condition |  | dB | -1 | Modified by Test Tolerance |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 2 |  |

==============Next change==============

Table 14.2.1.4.4.1-3: General test parameters for NR SA FR1-FR1 Time-based Conditional Handover for Satellite Access

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1, 2 | Two NR NTN satellite RF channel |
| Initial conditions | Active cell |  | Cell 1 | FDD duplex mode cell |
|  | Neighbouring cell |  | Cell 2 | FDD duplex mode cell |
| Final condition | Active cell |  | Cell 2 |  |
| UE position (L,B,H) |  |  | (0, 0, 0) | Set by any preconfigured means. (L,B,H) is Geodetic coordinate, where L is latitude, B is longitude, and H is height. |
| t1-Threshold-r17.condEventT1-r17 |  | s | T1+1 | Entering condition 1000ms after start of T2 |
| duration-r17.condEventT1-r17 |  | slot | 1000 | Give 1s search duration |
| Gap Pattern Id |  |  | 0 |  |
| Measurement gap offset |  |  | 9 |  |
| A3-Offset in condition |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 2 |  |

==============Next change==============

Table 14.2.1.5.4.1-3: General test parameters for NR SA FR1 Distance-based Conditional Handover for Satellite Access

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1 | One NR NTN satellite RF channel |
| Initial conditions | Active cell |  | Cell 1 | FDD duplex mode cell |
|  | Neighbouring cell |  | Cell 2 | FDD duplex mode cell |
| Final condition | Active cell |  | Cell 2 |  |
| UE position (L,B,H) at T1 start |  |  | (0, 0, 0) | Set by any pre-configured means. (L,B,H) is Geodetic coordinate, where L is latitude, B is longitude, and H is height. |
| UE moving speed |  | km/h | (108, 0, 0) | Set by any pre-configured means |
| referenceLocation1-r17.condEventD1-r17 |  | m | (-700, 0, 0) | Reference location for serving cell |
| referenceLocation2-r17.condEventD1-r17 |  | m | (1300, 0, 0) | Reference location for target cell |
| distanceThreshFromReference1-r17.condEventD1-r17 |  | 50m | 20 | D1-1 Location condition is fulfilled at T2 |
| distanceThreshFromReference2-r17.condEventD1-r17 |  | 50m | 20 | D1-2 Location condition is fulfilled at T2 |
| hysteresis-r17.condEventD1-r17 |  | 10m | 0 |  |
| timeToTrigger-r17.condEventD1-r17 |  | s | 0 |  |
| A3-Offset in condition |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 12 |  |
| T2 |  | s | 6 |  |

==============Next change==============

Table 14.2.1.6.4.1-3: General test parameters for NR SA FR1-FR1 Distance-based Conditional Handover for Satellite Access

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1, 2 | Two NR NTN satellite RF channel |
| Initial conditions | Active cell |  | Cell 1 | FDD duplex mode cell |
|  | Neighbouring cell |  | Cell 2 | FDD duplex mode cell |
| Final condition | Active cell |  | Cell 2 |  |
| UE position (L,B,H) at T1 start |  |  | (0, 0, 0) | Set by any pre-configured means. (L,B,H) is Geodetic coordinate, where L is latitude, B is longitude, and H is height. |
| UE moving speed |  | km/h | (108, 0, 0) | Set by any pre-configured means |
| referenceLocation1-r17.condEventD1-r17 |  | m | (-700, 0, 0) | Reference location for serving cell |
| referenceLocation2-r17.condEventD1-r17 |  | m | (1300, 0, 0) | Reference location for target cell |
| distanceThreshFromReference1-r17.condEventD1-r17 |  | 50m | 20 | D1-1 Location condition is fulfilled at T2 |
| distanceThreshFromReference2-r17.condEventD1-r17 |  | 50m | 20 | D1-2 Location condition is fulfilled at T2 |
| hysteresis-r17.condEventD1-r17 |  | 10m | 0 |  |
| timeToTrigger-r17.condEventD1-r17 |  | s | 0 |  |
| A3-Offset in condition |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 1 |  |
| T2 |  | s | 12 |  |

==============Next change==============

Table 14.2.1.8.4.1-3: General test parameters for NR SA FR1 RACH-less Handover for Satellite Access

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1 | One NR NTN satellite RF channel |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| Satellite configuration | Config 1 |  | Cell 1: SSC.1 Cell 2: NSC.1 | For GSO satellites configuration |
|  | Config 2 |  | Cell 1: SSC.2 Cell 2: NSC.2 | For NGSO inter-satellites configuration |
| UE position (L,B,H) |  |  | [(0, 0, 0)] | Set by any preconfigured means. (L,B,H) is Geodetic coordinate, where L is latitude, B is longitude, and H is height. |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

==============End of change==============