3GPP TSG-RAN5 Meeting #109	R5-255823r1

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3616 | rev | - | Current version: | 19.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to Test loop mode for ATSSS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, ATSSS_Ph2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As some ATSSS Phase 2 test cases (e.g. 11.9.1/3) need to check IP Packet is loop back via the WLAN cell, but currently the generic procedure Ipsec_SA_Released does not specify the requirement of Test Loop Back function. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Generic procedure Ipsec_SA_Released was updated for TLB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test purpose cann`t be checked. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: add the procedure of ACTIVATE TEST MODE in Table 4.5.2.2-3. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

4.5.2.2	Procedures

Table 4.5.2.2-1: E-UTRA RRC_IDLE

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-9a2 | Same as TS 36.508 [2] table 4.5.2.3-1, steps 1-9a2. | - | - | - | - |
| - | EXCEPTION: Steps 10a1 to 10b8 describe behaviour which depends on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 10a1-10a10 | IF Test Mode = On OR Test Loop Function = On THEN steps 10-19 as defined in TS 36.508 [2] table 4.5.2A.3-1, are performed. The ACTIVATE TEST MODE is using the associated condition for the test loop. | - | - | - | - |
| 10b1-10b8 | ELSE steps 10-17 as defined in TS 36.508 [2], table 4.5.2.3-1 are performed. | - | - | - | - |

Table 4.5.2.2-2: NR RRC_IDLE

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | - | <-- | NR RRC: SYSTEM INFORMATION (BCCH) | - | - |
| 2 | The UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest | - | - |
| 3 | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 4 | The UE transmits an RRCSetupComplete message and a REGISTRATION REQUEST message. | --> | NR RRC: RRCSetupComplete 5GMM: REGISTRATION REQUEST | - | - |
| 5 | The SS transmits a DLInformationTransfer message and an AUTHENTICATION REQUEST message. | <-- | NR RRC: DLInformationTransfer 5GMM: AUTHENTICATION REQUEST | - | - |
| 6 | The UE transmits an ULInformationTransfer message and an AUTHENTICATION RESPONSE message. | --> | NR RRC: ULInformationTransfer 5GMM: AUTHENTICATION RESPONSE | - | - |
| 7 | Void | - | - | - | - |
| 8 | The SS transmits a DLInformationTransfer message and a SECURITY MODE COMMAND message. | <-- | NR RRC: DLInformationTransfer 5GMM: SECURITY MODE COMMAND | - | - |
| 9 | The UE transmits an ULInformationTransfer message and a SECURITY MODE COMPLETE message. | --> | NR RRC: ULInformationTransfer 5GMM: SECURITY MODE COMPLETE | - | - |
| - | EXCEPTION: Step 9Aa1 to 9Aa2 describes behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported and the procedure parameter Interworking without N26 interface supported is not present. | - | - | - | - |
| 9Aa1 | IF UE_S1_SUPPORTED the SS transmits a DLInformationTransfer message and a SECURITY MODE COMMAND message. | <-- | NR RRC: DLInformationTransfer 5GMM: SECURITY MODE COMMAND | - | - |
| 9Aa2 | The UE transmits an ULInformationTransfer message and a SECURITY MODE COMPLETE message. | --> | NR RRC: ULInformationTransfer 5GMM: SECURITY MODE COMPLETE | - | - |
| - | EXCEPTION: Steps 9a1 to 9a2 describe the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that takes place if a procedure parameter has a particular value | - | - | - | - |
| 9a1 | IF Test Mode = On OR Test Loop Function = On, the SS transmits an ACTIVATE TEST MODE message to activate UE radio bearer test mode procedure. The ACTIVATE TEST MODE message is using the associated condition for the test loop. | <-- | RRC: DLInformationTransfer TC: ACTIVATE TEST MODE | - | - |
| 9a2 | The UE transmits an ACTIVATE TEST MODE COMPLETE message. | --> | RRC: ULInformationTransfer TC: ACTIVATE TEST MODE COMPLETE | - | - |
| 10 | The SS transmits a SecurityModeCommand message. | <-- | NR RRC: SecurityModeCommand | - | - |
| 11 | The UE transmits a SecurityModeComplete message. | --> | NR RRC: SecurityModeComplete | - | - |
| 12 | The SS transmits a UECapabilityEnquiry message. | <-- | NR RRC: UECapabilityEnquiry | - | - |
| 13 | The UE transmits a UECapabilityInformation message. | --> | NR RRC: UECapabilityInformation | - | - |
| 14 | The SS transmits a DLInformationTransfer message and a REGISTRATION ACCEPT message. | <-- | NR RRC: DLInformationTransfer 5GMM: REGISTRATION ACCEPT | - | - |
| 15 | The UE transmits an ULInformationTransfer message and a REGISTRATION COMPLETE message. | --> | NR RRC: ULInformationTransfer 5GMM: REGISTRATION COMPLETE | - | - |
| 16 | Void | - | - | - | - |
| 17 | Void | - | - | - | - |
| 18 | Void | - | - | - | - |
| - | EXCEPTION: Step 19a1 describes behaviour depending on UE implementation; the "lower case letter" identifies a step sequence that takes place if the UE performs a specific action. | - | - | - | - |
| 19a1 | IF pc_noOf_PDUsSameConnection > 0 THEN the generic procedure for UE-requested PDU session establishment, specified in subclause 4.5A.2, takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsSameConnection. | - | - | - | - |
| 19Aa1 – 19Aa2 | Void | - | - | - | - |
| - | EXCEPTION: Steps 19Ba1 to 19Ba2 describe behaviour depending on UE implementation; the "lower case letter" identifies a step sequence that takes place if the UE performs a specific action. | - | - | - | - |
| 19Ba1 | IF pc_noOf_PDUsSameConnection = 0 OR pc_noOf_PDUsNewConnection > 0 THEN the SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| 19Ba2 | Table 4.5.2.2-4 is performed to establish PDU session(s) on the new connection with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsNewConnection. | - | - | - | - |
| - | EXCEPTION: Steps 19Ca1 to 19Ca2 describe behaviour which depends on the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that takes place if a procedure parameter has a particular value. | - | - | - | - |
| 19Ca1 | IF connected without release is On AND Test Loop Function=On THEN the SS transmits a CLOSE UE TEST LOOP message to enter the UE test loop mode. The CLOSE UE TEST LOOP is using the associated condition for the test loop. | <-- | NR RRC: DLInformationTransfer TC: CLOSE UE TEST LOOP | - | - |
| 19Ca2 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message to confirm that loopback entities for the radio bearer(s) have been created and loop back is activated. | --> | NR RRC: ULInformationTransfer TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| - | EXCEPTION: Step 20a1 depends on the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that takes place if a procedure parameter has a particular value. | - | - | - | - |
| 20a1 | IF connected without release is not present THEN, the SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| - | EXCEPTION: Step 20Aa1 depends on the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that takes place if a procedure parameter has a particular value. | - | - | - | - |
| 20Aa1 | IF GNSS Sync = On THEN trigger UE to reset or clear the current UTC time that has been calculated from GNSS. NOTE: The UTC time can be reset or clear on the UE using AT command (+CUTCR). | - | - | - | - |
| - | EXCEPTION: Step 21a1 and 21b1 depend on the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that takes place if a procedure parameter has a particular value. | - | - | - | - |
| 21a1 | IF Sidelink = On AND Cast type = Unicast AND UE initiated unicast mode NR sidelink comunication THEN generic procedure specified in subclause 4.9.22 is performed to establish unicast mode sidelink communication between the UE and the NR-SS-UE. |  |  |  |  |
| 21b1 | ELSE IF Sidelink = On AND Cast type = Unicast AND NR-SS-UE initiated unicast mode NR sidelink comunication THEN generic procedure specified in subclause 4.9.23 is performed to establish unicast mode sidelink communication between the UE and the NR-SS-UE. |  |  |  |  |
| 22a1 | Void | - | - |  |  |

Table 4.5.2.2-3: WLAN Ipsec_SA_Released

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE associates with the WLAN AP and obtains the local IP address. | - | - | - | - |
| 2 | The UE performs a dynamic selection of N3IWF using DNS query | - | - | - | - |
| - | Exception: The UE establishes an IPsec tunnel in parallel to 5GC registration steps 3 to 7 as per the IKEv2 protocol as defined in TS 23.502 [33] clause 4.12.2.2 figure 4.12.2.2-1. | - | - | - | - |
| 3 | The UE transmits a REGISTRATION REQUEST message. | --> | 5GMM: REGISTRATION REQUEST | - | - |
| 4 | The SS transmits an AUTHENTICATION REQUEST message including EAP-Request/AKA'-Challenge or 5G AKA Challenge. | <-- | 5GMM: AUTHENTICATION REQUEST | - | - |
| 5 | The UE transmits an AUTHENTICATION RESPONSE message including EAP-Response/AKA'-Challenge or 5G AKA Response. | --> | 5GMM: AUTHENTICATION RESPONSE | - | - |
| 6 | The SS transmits a SECURITY MODE COMMAND message including EAP-Success if EAP-AKA' used. | <-- | 5GMM: SECURITY MODE COMMAND | - | - |
| 7 | The UE transmits a SECURITY MODE COMPLETE message. | --> | 5GMM: SECURITY MODE COMPLETE | - | - |
| - | EXCEPTION: Steps 7Aa1 to &Aa2 describe the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that takes place if a procedure parameter has a particular value | - | - | - | - |
| 7Aa1 | IF Test Mode = On OR Test Loop Function = On, the SS transmits an ACTIVATE TEST MODE message to activate UE test mode procedure. The ACTIVATE TEST MODE message is using the associated condition for the test loop. | <-- | TC: ACTIVATE TEST MODE | - | - |
| 7Aa2 | The UE transmits an ACTIVATE TEST MODE COMPLETE message. | --> | TC: ACTIVATE TEST MODE COMPLETE | - | - |
| 8 | The SS transmits a REGISTRATION ACCEPT message. | <-- | 5GMM: REGISTRATION ACCEPT | - | - |
| 9 | The UE transmits a REGISTRATION COMPLETE message. | --> | 5GMM: REGISTRATION COMPLETE | - | - |
| 10 | The generic procedure for UE-requested PDU session establishment, specified in subclause 4.5A.2A, takes place performing establishment of UE-requested PDU session. | - | - | - | - |
| - | EXCEPTION: Steps 10Aa1 to 10Aa2 describe behaviour which depends on the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that takes place if a procedure parameter has a particular value. | - | - | - | - |
| 10Aa1 | IF Test Loop Function=On THEN the SS transmits a CLOSE UE TEST LOOP message to enter the UE test loop mode. The CLOSE UE TEST LOOP is using the associated condition for the test loop. | <-- | TC: CLOSE UE TEST LOOP | - | - |
| 10Aa2 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message to confirm that loopback entities have been created and loop back is activated. | --> | TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| - | EXCEPTION: Step 11a1 depends on the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 11a1 | IF connected without release is not present THEN generic procedure for SS-requested IPsec Secure tunnel disconnection, specified in subclause 4.5A.5, takes place performing disconnection of security association. | - | - | - | - |
| Note: The current procedure assumes UE establishes a single PDU session or a MA PDU session over Non 3GPP Access. |  |  |  |  |  |

==============End of change==============