3GPP TSG-RAN5 Meeting #109	R5-255870r2

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0706 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of eCall test case 11.21 for CEN alignments |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCallCEN-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | New test case 11.21 needs to be added as part of eCallCEN-UEConTest WI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case 11.21 – "eCall capable / eCall to URI for test service / 200 OK without ACK/NACK / PSAP Callback / Success" |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eCallCEN-UEConTest WP will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.21 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 34.229-2 CR 0400 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-255870r1: - Missing Clauses affected included. - TP Updated as per eCALL type - Included Emergency call release - Specific Message contents corrected R5-255870r2: Editorial changes. |  |  |  |  |  |  |  |  |

==============First change==============

## 11.21	eCall capable / eCall to URI for test service / 200 OK without ACK/NACK / PSAP Callback / Success

11.21.1	Test Purpose (TP)

(1)

with { UE having sent an INVITE for eCall to URI for test service initiated }

ensure that {

when { UE receives 200 OK not containing ACK or NACK for MSD transmission }

then { UE continues with the call establishment }

}

(2)

with { UE subjected to national regulations in CEN EN 17184 having previously establishment of an IMS call of the test eCall type to URI for test services received 200 OK not containing ACK or NACK for MSD transmission }

ensure that {

when { UE receives INVITE for an incoming IMS call for PSAP callback }

then { UE responds with 200 OK including correctly composed header fields }

}

(3)

with { UE subjected to national regulations in CEN EN 17184 having previously establishment of an IMS call of the test eCall type to URI for test services received 200 OK not containing ACK or NACK for MSD transmission }

ensure that {

when { SS requests for transfer of MSD during an incoming IMS call }

then { UE responds with 200 OK and performs MSD transfer }

}

11.21.2	Conformance requirement

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-19 requirements.

[Rel-19, TS 24.229, clause 5.1.6A]:

This clause specifies a feature which applies to UEs subject to regulations that mandate support of EN 17184:2024 [300]. If the upper layers request establishment of an IMS call of the test eCall type while in limited service state, the request shall be rejected. Otherwise, the UE shall follow normal voice call setup to attempt the test eCall. If the CS domain is selected, the UE shall attempt a test eCall setup as a non-emergency services call using appropriate access technology specific procedures.

If the IM CN subsystem is selected and the UE is not currently registered, the UE shall perform an initial registration, as described in subclause 5.1.1.2. The UE shall then apply voice domain selection as specified in 3GPP TS 23.221 [6] and 3GPP TS 23.501 [257] and the procedures as specified in subclause 5.1.2A and 5.1.3 to send an INVITE with the following additions:

1)	the UE shall set the Request-URI to a non-emergency service URN (e.g. SIP, Service, Tel URIs) that is defined by regulation, PSAP or eCall test provider for testing purposes; and

NOTE 1:	The Request-URI can be set to "urn:service:test.sos.ecall" as specified in RFC 8147 [244] or the URI can be configured in USIM files; see 3GPP TS 31.102 [15C].

2)	the UE shall insert in the INVITE request:

a)	a To header field with the same service URN as in the Request-URI;

b)	a From header field that includes the public user identity or the tel URI associated with the public user identity, as described in subclause 4.2;

c)	a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in EN 15722:2020 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

d)	an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

e)	a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE 2:	Further content for the INVITE such as set the test bit in the MSD is defined in EN 15722:2020 [245] and others is defined in RFC 8147 [244].

Then the UE shall proceed as follows:

1)	if the UE receives a 200 (OK) response to the INVITE request not containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to either "true" or "false"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then, if the PSAP requests it, the UE shall send the MSD using audio media stream encoded as described in 3GPP TS 26.267 [9C] and in accordance with the audio media procedures in EN 17184:2024 [300] and EN 16062:2023 [304];

[Rel-19, TS 24.229, clause 5.1.6B]:

This clause specifies a feature which applies to UEs subject to national regulations in EN 17184:2024 [300]. The procedures specified in subclause 5.1.6.11.3 for transfer of an updated MSD shall be supported by a UE during an incoming IMS call when the following conditions are fulfilled:

1)	the UE previously established or attempted to establish an emergency call either over IMS or in the CS domain of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service;

2)	the incoming IMS call occurred within an implementation dependent time period of at least one hour;

3)	the INVITE for the incoming IMS call contains a Priority header field with header field value "psap-callback";

4)	the INVITE for the incoming IMS call contains an Accept header field that includes the value “application/EmergencyCallData.Control+xml”; and

5)	the INVITE for the incoming IMS call contains a Recv-Info header field with header field value EmergencyCallData.eCall.MSD.

NOTE 1:	When there are multiple attempts to establish the emergency call, the emergency call in condition 2) refers to the last attempt.

When conditions 1) to 5) are fulfilled, the UE shall include in the 200 OK:

a)	a Recv-Info header field with header field value EmergencyCallData.eCall.MSD; and

b)	an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244].

The UE shall also support the procedures specified in subclause 5.1.6.11.3 for transfer of an updated MSD.

When conditions 1) and 2) and, as an implementation option, condition 3) are fulfilled as well as the PSAP requests it, the UE shall support request and transfer of an updated MSD using audio media stream as described in 3GPP TS 26.267 [9C] and in accordance with the audio media procedures in EN 17184:2024 [300] and EN 16062:2023 [304].

NOTE 2:	A PSAP can request transfer of an updated MSD using audio media stream if the procedures specified in subclause 5.1.6.11.3 cannot be supported by the PSAP, the serving PLMN, the HPLMN or by some other intermediate network.

Reference(s)

3GPP TS 24.229 [10], clauses 5.1.6A and 5.1.6B.

11.21.3	Test description

11.21.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC with eCall subscription according to TS 38.508-1 [21] table 6.4.1-23.

-	UE is configured to register for IMS after switch on.

Preamble:

-	UE is in state 1N-A (TS 38.508-1 [21]) and registered to IMS.

11.21.3.2	Test procedure sequence

Table 11.21.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is triggered to initiate test eCall to uri for test services | - | - | - | - |
| 2-13 | Steps 2-13 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in steps 14 and 16, steps 11 to 13 specified in 38.508-1 [21] Table 4.9.11.2.2-1 takes place. | - | - | - | - |
| 14 | Step 1 of Annex A.23 happens | --> | INVITE | - | - |
| 15 | The SS sends 200 OK without ACK or NACK | <-- | 200 OK |  |  |
| 16 | Step 3 of Annex A.23 happens Check: Does the UE send ACK? | --> | ACK | 1 | P |
| 17 | The Test procedure for IMS MT Emergency call release as specified in TS 38.508-1 [21], subclause 4.9.12B takes place using condition 'keep emergency PDU session'. | - | - | - | - |
| 18 | The SS releases the RRC connection. | - | - | - | - |
| 19 | The SS waits for 5 seconds | - | - | - | - |
| 20-31 | Steps 1 to 12 of the generic test procedure for IMS MT speech call establishment in 5GC specified in TS 38.508-1 [21] Table 4.9.16.2.2-1 are performed. (Steps 1-5 of Annex A.5.1a for UE configured with usage of preconditions else A.5.2a) | - | - | - | - |
| - | EXCEPTION : Steps 32a1 to 32b6 describe behaviour that depends on UE configuration ; the “lower case letter” identifies a step sequence that takes place if such configuration was conducted. | - | - | - | - |
| 32a1-32a6 | IF the UE is configured to use preconditions THEN steps 6-10A of Annex A.5.1a take place. | - | - | - | - |
| 32a7 | Check : Does the UE send a correctly composed 200 OK for INVITE request? (Step 11 of annex A.5.1a) | --> | 200 OK | 2 | P |
| 32a8 | SS sends ACK. | <-- | ACK | - | - |
| 32b1-32b4 | ELSE steps 6-8A of Annex A.5.2a of take place. | - | - | - | - |
| 32b5 | Check : Does the UE send a correctly composed 200 OK for INVITE request? (Step 9 of annex A.5.2a) | --> | 200 OK | 2 | P |
| 32b6 | SS send ACK. | <-- | ACK | - | - |
| 33-34 | Steps 4-5 as defined in Annex A.23 happens. | - | - | - | - |
| 35 | Check : Does UE send SIP INFO message with MSD update? (Step 6 of annex A.23) | --> | SIP INFO | 3 | P |
| 36 | SS responds SIP INFO with 200 OK. | <-- | 200 OK | - | - |
| 37 | Generic test procedure for IMS MT call release in 5GS described in TS 38.508-1 [21] subclause Table 4.9.18 takes place. | - | - | - | - |

11.21.3.3	Specific message contents

Table 11.21.3.3-1: INVITE (step 14, table 11.21.3.2-1)

Derivation path: Step 1 in Annex A.23 with condition A25 instead of A20/A21 and without condition A7

Table 11.21.3.3-2: 200 OK for INVITE (step 15, table 11.21.3.2-1)

Derivation path: Step 2 in Annex A.23 with conditions A6 and without conditions A12, A13.

Table 11.21.3.3-3 INVITE (steps 28,11.21.3.2-1)

Derivation path: Step 1 in Annex A.5.1a or Annex A.5.2a with condition A12

Table 11.21.3.3-4: 200 OK (steps 32a7, 32b5, Table 11.21.3.2-1)

Derivation path: Step 11 in Annex A.5.1a or step 9 in Annex A.5.2a with condition A24

Table 11.21.3.3-5: BYE (step 37, Table 11.21.3.2-1)

Derivation path: Step 1 in Annex A.8 with condition A9

==============End of change==============